| PRR Number | 439PRR | PRR Title | Clarification on defining OOME as instructed deviation |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 6.7.7 Deployment of Out-of-Merit Energy Service |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | This Protocol revision will clarify two exceptions on defining OOME as instructed deviation. Before the balancing energy market clearing, the manual OOME instruction will not be defined as instructed deviation under the following two conditions: 1.) A resource is instructed to operate at or above the instructed output level and the planned output level of that resource is above the instructed output level, or 2.) A resource is instructed to operate at or below an instructed output level and the planned output level of that resource is below the instructed output level. |  |
| ###### Reason for Revision |  | To clarify two exceptions on defining OOME as Instructed Deviation for the implementation of 373PRR. |  |
| ## Timeline |  |  |  |
| Date Received |  | 07/03/03 |  |
| Date Posted |  | 07/08/03 |  |
| Comments Due |  |  |  |
| PRS Review Date |  |  |  |
| PRS Recommendation |  |  |  |

Proposed Protocol Language Revision

### 6.7.7	Deployment of Out-of-Merit Energy Service

Deployment of units for OOME Service will follow Balancing Energy Service deployment guidelines as specified in Section 5, Dispatch of these Protocols.

### [PRR 373:  Replace Subsection 6.7.7, Deployment of Out-of-Merit Energy Service, as follows when the system change is implemented.]

Deployment of units for OOME Service will follow Balancing Energy Service deployment guidelines as specified in Section 5, Dispatch of these Protocols.

      - When a Resource Specific OOME instruction is issued before the Balancing Energy Market clearing, the instructed deviation resulted from the OOME instruction will be defined as the difference between the instructed output level and the Resource Plan output level at the close of the Adjustment Period for the target service interval except: 1.) a Resource is instructed to operate at or above an instructed output level and the planned output level of that resource is above the instructed output level, or 2.) a Resource is instructed to operate at or below an instructed output level and the planned output level of that resource is below the instructed output level. The instructed deviation will be balanced using the balancing energy bid stack.  OOME instructions will be sent to QSEs concurrent with Balancing Energy instructions for the target interval.  The Balancing Energy deployment will be limited by the constraints imposed by the OOME instructions.  The instructed deviations for these OOME instructions will be included in the Balancing Energy instructions for that QSE for the target interval.  ERCOT must honor the unit ramp rates as indicated by the QSE’s Resource Plan.

      - Resource Specific OOME instructions issued after the Balancing Energy Market clearing will not be instructed deviations.  ERCOT will use best efforts to minimize instructions issued after the Balancing Energy Market has cleared.

| ## Sponsor |  |
| --- | --- |
| Name | Larry Gurley (for QSE Managers WG) |
| E-mail Address | Lgurley1@txu.com |
| Company | TXU Energy |
| Company Address | 1717 Main Street Suite 20-014, Dallas, TX 75201 |
| Phone Number | (214) 875-9020 |
| Fax Number | (214) 875-9074 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |