| PRR Number | 448PRR | PRR Title | Changes to Implement Balancing Up Loads (BULs) |
| --- | --- | --- | --- |
| Recommended Action |  | Approve |  |
| Protocol Section Requiring Revision |  | 6.8.1.14, Payments for Capacity for Balancing Energy Up Load Deployed 6.8.1.15.3, Uninstructed Charge Methodology and Equation |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority & Rank Assigned |  | Same as PIP 112 |  |
| Summary of Impact Analysis |  | Impacts ERCOT computer systems, business practices, and staffing. |  |
| Revision Description |  | Clarification of Balancing Up Load (BUL) calculations and inclusion of BUL capacity payment calculations in Section 6.8. |  |
| ###### Benefit |  | ERCOT-wide calculations of BUL payments rely on distinguishing whether Loads are statically or dynamically scheduled. This PRR clarifies how the BUL formulas treat each type of scheduled Load. This PRR also clarifies the BUL capacity payment calculations. |  |
| PRS Recommendation |  | PRS recommends approval of PRR448 as amended by PRS. |  |

Proposed Protocol Language Revision

6.8.1.14   Payments for Capacity for Balancing Energy Up Load Deployed

[PIP112, PRR311, & PRR448: Implement entire new section 6.8.1.14 when BUL is implemented.]

A QSE that is instructed to deploy Balancing Up Load (BUL) shall be paid a capacity payment for the 15-minute interval the instruction is issued and the three subsequent intervals according to the following formulas.  A QSE that is instructed to continue to deploy Balancing Up Load after the first four 15 minute intervals will be paid a capacity payment for each subsequent interval instruction it is given to deploy Balancing Up Load.  The Balancing Up Load capacity payment is calculated as follows:

PCBULiq	=	-1 *(( MBULiq * MCPCNSi )/ 4)

MBULiq	 =	 MAX(BULiq-0, BULiq-1, BULiq-2, BULiq-3)

BULPMiq 	=	BULETRiq –all…zone(BMRDiq + BMRSiq)

BULETRiq	=	[((BRATDiq * AIMLDiq) + (BRATSiq * AIMLSiq)) – ((∑FSBULizq)z + (∑DSBULizq)z)]

Given:

IF Small Customer Load Management THEN

BULiq 	= 	AIMLLMiq – BMRLMiq

ELSE:

BULiq = MIN[MIN(MAX(0,(∑DIBULizq)z),MAX(0,((BRATDiq*AIMLDiq)-BMRDiq))) + MAX(0,((BRATSiq*AIMLSiq)-BMRSiq)), (∑DQiqz)z]

DIBULizq  =  	MAX[0, (∑(MRizq – 	(DSLizq + INSizq + INSewiq + SRSizq))z)]

AIMLDiq=n=1xBMRDqin−MAXn=1,...,xBMRDqi1,⋯,BMRDqix−MINn=1,...,xBMRDqi1,⋯,BMRDqixx−2

AIMLSiq=n=1xBMRSqin−MAXn=1,...,xBMRSqi1,⋯,BMRSqix−MINn=1,...,xBMRSqi1,⋯,BMRSqixx−2AIMLiq=n=110BMRMqin−MAXn=1,...,10BMRMqi1,⋯,BMRMqi10−MINn=1,...,10BMRMqi1,⋯,BMRMqi108

AIMLiq=n=1zBMRMqin−MAXn=1,...,zBMRMqi1,⋯,BMRMqiz−MINn=1,...,zBMRMqi1,⋯,BMRMqizz−2

BMRMqin=m=03BMRqimn

AMLPDiqn=BMRDqi−8n8

AMLPSiqn=BMRSqi−8n8

AMLPBDiq=n=1xAMLPDiqnx−2

AMLPBSiq=n=1xAMLPSiqnx−2

BRATDiq=AMLPDiq0AMLPBDiq

BRATSiq=AMLPSiq0AMLPBSiq

AMLPiqn=BMRMqi−4n+BMRMqi−8n2

AMLPBiq=n=110AMLPiqn10

AMLPBiq=n=1zAMLPiqnz

BRATiq=AMLPiq0AMLPBiq

Where:

i:	Settlement Interval being calculated

q:	QSE

x:	count of proxy days

z:	zone

AIMLDiq       	Representative average interval metered BUL during the last ten (10) proxy days (in the case of weekdays) or six (6) proxy days (if deployment is on a weekend or holiday) for BUL meters that represent dynamic load.  Proxy days cannot be days when BUL was deployed during any interval.

AIMLLMiq 	QSE aggregate baseline load for small customers who participate in load management program assuming the program is not in operation from the appropriate load profile.

AIMLSiq        Representative average interval metered BUL during the last ten (10) proxy days (in the case of weekdays) or six (6) proxy days (if deployment is on a weekend or holiday) for BUL meters that represent static load.  Proxy days cannot be days when BUL was deployed during any interval.

AMLPBDiq    Average interval metered BUL from the prior ten (10) proxy days (in the case of weekday deployment) or six (6) proxy days (for weekend or holiday deployment) for the two (2) hours prior to first BUL instruction for BUL meters that represent dynamic load.  Proxy days can not be days when BUL was deployed during any interval.

AMLPBSiq     Average interval metered BUL from the prior ten (10) proxy days (in the case of weekday deployment) or six (6) proxy days (for weekend or holiday deployment) for the two (2) hours prior to first BUL instruction for BUL meters that represent static load.  Proxy days cannot be days when BUL was deployed during any interval.

AMLPDiqn	Average interval metered BUL during the two (2) hours prior to the hour of BUL notice in the nth prior day for BUL meters that represent dynamic load.  Note, that n=0 refers to day of notice.

AMLPSiqn	Average interval metered BUL during the two (2) hours prior to the hour of BUL notice in the nth prior day for BUL meters that represent static load.  Note, that n=0 refers to day of notice.

BMRDiq  	Aggregate of all actual qualified BUL meter readings per QSE per Settlement Interval per zone for BUL meters that represent dynamic load

BMRDqimn     Aggregate of all qualified dynamic BUL meter readings per QSE q during the mth 15-minute time interval after the Settlement Interval i on the nth day prior to the day of settlement, where the n prior days shall be only weekdays excluding ERCOT-defined holidays if the day of settlement is a weekday, and shall be only weekends and ERCOT-defined holidays if the day of settlement is a weekend.  Days in which BUL instructions occurred are excluded.

BMRLMiq 	QSE aggregate participating customer load during operation of load management program from the appropriate load profile.

BMRSiq  	Aggregate of all actual qualified BUL meter readings per QSE per Settlement Interval per zone for BUL meters that represent static load

BMRSqimn      Aggregate of all qualified static BUL meter readings per QSE q during the mth 15-minute time interval after the Settlement Interval i on the nth day prior to the day of settlement, where the n prior days shall be only weekdays excluding ERCOT-defined holidays if the day of settlement is a weekday, and shall be only weekends and ERCOT-defined holidays if the day of settlement is a weekend.  Days in which BUL instructions occurred are excluded.

BRATDiq	Ratio of average metered dynamic load for 2 hours prior to BUL instruction to the average interval metered BUL from the prior ten (10) days (in the case of weekday deployment) or six (6) days (for weekend or holiday deployment) for the two (2) hours prior to first BUL instruction.

BRATSiq	Ratio of average metered static load for two (2) hours prior to BUL instruction to the average interval metered BUL from the prior ten (10) days (in the case of weekday deployment) or six (6) days (for weekend or holiday deployment) for the two (2) hours prior to first BUL instruction.

BULiq	Quantity in MWhs of BUL deployed per Settlement Interval per QSE

MBULiq	Maximum quantity in MWhs of BUL deployed per Settlement Interval per QSE

BULETRiq	Total BUL expected reduction by QSE per interval ERCOT-wide

BULPMiq	Calculated to determine whether a QSE performed the BUL deployment and should receive a capacity payment

DQiqz	Deployed  BUL quantity in MWs per zone.

DIBULizq	For QSEs with dynamically scheduled Loads, the average power delivered to ERCOT per interval per zone as a result of deploying BUL.

DSBULizq  	For QSEs with dynamically scheduled Loads, the integrated signal for the interval that is an estimate in real-time representing the real power interrupted in response to the deployment of BUL.  DSBULizq = zero (0) if Load is not dynamically scheduled.

DSLizq	The integral of the dynamically scheduled Load telemetered to ERCOT for that QSE per interval per zone

FSBULizq	Integrated signal for the interval sent by static scheduling QSEs to ERCOT and is an estimate in real-time representing the real power interrupted in response to the deployment of BUL

INSizq	Zonal Balancing Energy instructions given to that QSE per zone per interval

INSewiq	ERCOT-wide instructions for that QSE per interval

PCBULiq	Capacity payment for instructed deployment of BUL per Settlement Interval per QSE

MCPCNSi	Highest Non-Spinning Reserve Service Market Clearing Price of Capacity costs ($/MW) for the hour of all procurement processes

MRizq	Metered Resource value for that QSE per interval per zone

SRSizq	Static Resource schedule per interval per zone of that QSE

##### 6.8.1.15.3   Uninstructed Charge Methodology and Equation

[PIP112: Although the formula did not change, the “BUL” language incorporates Load Resources into one of the variables (MR). The current design does not have that capability. Once reworked, the following words in this comment box could be added to the protocols.]

The QSE’s Metered Resource Value will include both Generation and Load Resources. The amount of energy provided by a Load Resource will be obtained from the information submitted on the Resource Plan and the settlement meter of the Load Resource.  Energy from a Load Resource is calculated for each Load Resource that is indicated available on the Resource Plan according to the following formula:

Energy from Load Resource =  MAX (0, MIN ((the integral of the upper operating limit –Load settlement meter), (integral of the upper operating limit -  integral of the lower operating limit)))

At the request of the QSE as specified by Section 6.10.6, ERCOT will disregard energy from a Load Resource for the four consecutive settlement intervals following the time of the request by the QSE as it relates to the following formula.

If the QSE’s Total Uninstructed Deviation is zero, then the zonal uninstructed deviation for every zone is zero.

ZUDizq = 0

If the QSE’s Total Uninstructed Deviation is positive, then the total Uninstructed Deviation will be allocated to those zones in which the value of adding the QSE’s Resource Schedule in that zone and zonal instructions in that zone, and subtracting from the Metered Resource Value in that zone is positive.

ZUDizq=max(0,MRizq−(RSizq+INSizq))all  zonemax(0,MRizq−(RSizq+INSizq))∗TUDiq

[PRR448:  The above equation will be replaced with the following upon the implementation of BUL:]

ZUDizq=max(0,MRizq−(RSizq+INSizq+DSBULizq))all  zonemax(0,MRizq−(RSizq+INSizq+DSBULizq))∗TUDiq

If the QSE’s Total Uninstructed Deviation is negative, then the total Uninstructed Deviation will be allocated to those zones in which the value of adding the QSE’s Resource Schedule in that zone and zonal instructions in that zone, and subtracting from the Metered Resource Value in that zone is negative.

ZUDizq=min(0,MRizq−(RSizq+INSizq))all  zonemin(0,MRizq−(RSizq+INSizq))∗TUDiq

[PRR448:  The above equation will be replaced with the following upon the implementation of BUL:]

ZUDizq=min(0,MRizq−(RSizq+INSizq+DSBULizq))all  zonemin(0,MRizq−(RSizq+INSizq+DSBULizq))∗TUDiq

Where:

i	interval

q	QSE

z	Congestion Zone

TUDiq	Total ERCOT wide uninstructed deviation for that QSE per interval

ZUDizq	Zonal uninstructed deviation for that QSE per interval for that zone

MRizq	Metered Resource Value for that QSE per interval per zone

RSizq	Resource Schedule per interval per zone of that QSE

INSizq	Zonal Balancing Energy instructions given to that QSE per zone per interval

[PIP112 & PRR448: DSBUL will not be factored into the formula until system is reworked to support BUL. Once supported, the definition of variable should be modified above with the following:]

DSBULizq	For QSEs with dynamically scheduled Loads, the integrated signal for the interval that is an estimate in real-time representing the real power interrupted in response to the deployment of BUL.  DSBULizq = zero (0) if Load is not dynamically scheduled.

[PRR403: Replace ZUD formulas and variables above with formulas and variables below upon system implementation and notice to the market.]

If the QSE’s Total Uninstructed Deviation is zero, then the zonal uninstructed deviation for every zone is zero.

ZUDizq = 0

If the QSE’s Total Uninstructed Deviation is positive, then the total Uninstructed Deviation will be allocated to those zones in which the value of adding the QSE’s Resource Schedule in that zone and zonal instructions in that zone, and subtracting from the Metered Resource Value in that zone is positive.

ZUDizq=max(0,MRizq−(SRURCizq+INSizq))all  zonemax(0,MRizq−(SRURCizq+INSizq))∗TUDiq

[PRR448:  The above equation will be replaced with the following upon the implementation of BUL:]

ZUDizq=max(0,MRizq−(SRURCizq+INSizq+DSBULizq))all  zonemax(0,MRizq−(SRURCizq+INSizq+DSBULizq))∗TUDiq

If the QSE’s Total Uninstructed Deviation is negative, then the total Uninstructed Deviation will be allocated to those zones in which the value of adding the QSE’s Resource Schedule in that zone and zonal instructions in that zone, and subtracting from the Metered Resource Value in that zone is negative.

ZUDizq=min(0,MRizq−(SRURCizq+INSizq))all  zonemin(0,MRizq−(SRURCizq+INSizq))∗TUDiq

[PRR448:  The above equation will be replaced with the following upon the implementation of BUL:]

ZUDizq=min(0,MRizq−(SRURCizq+INSizq+DSBULizq))all  zonemin(0,MRizq−(SRURCizq+INSizq+DSBULizq))∗TUDiq

Where:

TUDiq	Total ERCOT wide uninstructed deviation for that QSE per interval

ZUDizq	Zonal uninstructed deviation for that QSE per interval for that zone

MRizq	Metered Resource Value for that QSE per interval per zone

SRURCRSizq	The Resource Schedule (without any DC tie import schedules) smoothed for a 10-minute ramp per interval per zone of that QSE, plus the dynamic schedule per interval per zone of that QSE, plus the DC tie import schedules per interval per zone of that QSE.

INSizq	Zonal Balancing Energy instructions given to that QSE per zone per interval

SRURCizq  =  SRSURCizq + (SRAizq or SRDizq) + DC tie import schedulesizq

SRSURCizq = SRSNDCCURRizq + ((SRSNDCPREVizq - SRSNDCCURRizq) / 12)+((SRSNDCNEXTizq - SRSNDCCURRizq) / 12)

SRSURCizq	The scheduled resource static (without any DC tie import schedules) smoothed for a 10 minute ramp, per interval per zone of the that QSE,

(SRAizq or SRDizq)	The scheduled resource (actual or dynamic, which ever is selected based on the dynamic scheduling logic) per interval per zone of that QSE

DCtie import schedulesizq	The DC tie imports scheduled per interval per zone of that QSE

Where:

SRSURCizq	The scheduled resource static (without any DC tie import schedules) smoothed for a 10 minute ramp, per interval per zone of that QSE.

SRSNDCCURRizq	The scheduled resource static minus any DC tie import schedules, for the current interval, per interval per zone per QSE.

SRSNDCPREVizq	The scheduled resource static minus any DC tie import schedules for the previous interval, per interval per zone perQSE.

SRSNDCNEXTizq	The scheduled resource static minus any DC tie import schedules for the next interval, per interval per zone perQSE.

[PIP112 & PRR448: DSBUL will not be factored into the formula until system is reworked to support BUL. Once supported, the definition of variable should be modified above with the following:]

DSBULizq	For QSEs with dynamically scheduled Loads, the integrated signal for the interval that is an estimate in real-time representing the real power interrupted in response to the deployment of BUL.  DSBULizq = zero (0) if Load is not dynamically scheduled.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted |

| ## Sponsor |  |
| --- | --- |
| Name | Mark Patterson |
| E-mail Address | mpatterson@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive, Taylor, TX 76574 |
| Phone Number | (512) 248-3912 |
| Fax Number | (512) 248-6565 |

| Sponsor's Revision Description | Clarification of Balancing Up Load (BUL) calculations, removal of BUL Resource Plan language, and inclusion of BUL capacity payment calculations in Section 6.8. |
| --- | --- |
| ###### Sponsor's Reason for Revision | ERCOT-wide calculations of BUL payments rely on distinguishing whether Loads are statically or dynamically scheduled. This PRR clarifies how the BUL formulas treat each type of scheduled Load. This PRR also clarifies the BUL capacity payment calculations and removes Resource Plan language since BUL is not a Resource. |