| PRR Number | 478PRR | PRR Title | Use of Lagged Dynamic Samples for New Load Profiles |
| --- | --- | --- | --- |
| Recommended Action |  | Approve as revised. |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 18.2.2, Load Profiles For Non-Interval Metered Loads 18.2.7, Daily Profile Creation Process |  |
| Proposed Effective Date |  | Upon system implementation. |  |
| Priority & Rank Assigned |  | Pending PRS review. |  |
| Summary of Impact Analysis |  | Has minor ERCOT computer systems impact to Lodestar; timeline for completion of this project should not impact timeline for implementing new profiles; business process impacts to test and maintain new Load Profiles; no operating practice impacts. |  |
| Revision Description |  | This is a change to the method for creating Load Profiles allowing for the use of lagged dynamic samples for new profiles adopted subsequent to market open. |  |
| Benefit |  | Allowing the use of lagged dynamic samples for new profiles is expected to provide three important benefits to the market: The time lag between approving a new profile and being able to settle ESI IDs using the new profile will be reduced. New profiles are expected to be subject to significant population change as new ESI IDs are identified for inclusion in the profile; lagged-dynamic samples will accommodate these changes more quickly, accurately and efficiently than is possible with adjusted static models. Use of lagged dynamic samples will allow the market to audit ESI IDs being assigned to new profiles and will create an incentive for CRs using the profile to limit the inclusion of ESI IDs to those appropriate to the profile. |  |
| PRS Recommendation |  | PRS recommends approval of PRR478 as modified by ERCOT comments of 12/16/03 and PRS. |  |
| TAC Recommendation |  | TAC recommends approval of PRR478 as recommended by PRS. |  |

| Sponsor |  |
| --- | --- |
| Name | Ernie Podraza, PWG Chair |
| E-mail Address | epodraza@reliant.com |
| Company | Reliant Energy |
| Company Address | 1111 Louisiana, Houston, Texas 77251 |
| Phone Number | 713-497-5295 |
| Fax Number | 713-497-9878 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Integrate into settlement procedures. | Integrate lagged dynamic into settlement, which can build on current DLC functionality efforts. | Improved accuracy of settlement. |
| MARKET SEGMENT |  |  |  |
| Consumer | N/A | N/A | N/A |
| LSE: General, Including NOIE | Integrate into settlement procedures. | Integrate lagged dynamic into settlement. | Improved accuracy of settlement. |
| LSE: CR & REP | Integrate into settlement procedures. | Integrate lagged dynamic into settlement. | Improved accuracy of settlement. |
| QSE | Integrate into settlement procedures. | N/A | Improved accuracy of settlement. |
| Resource | N/A | N/A | Improved accuracy of settlement. |
| TDSP | Integrate into settlement procedures. | Integrate lagged dynamic into settlement by installing sample sites per ERCOT and passing the data to ERCOT. | N/A |

Proposed Protocol Language Revision

{Reflects ERCOT Comments and PRS changes}

### Load Profiles For Non-Interval Metered Loads

For market open, Load Profiles for non-interval metered loads were created using statistical models developed from appropriate load research sample data.  These models are referred to as “adjusted static.” These model equations relate daily Settlement Interval load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity.  Other daily characteristics such as day-of-the-week and sunrise/sunset times are also employed.

Following market open, new Load Profile segments may be introduced.  After these Load Profile segments receive final approval under the provisions of the Load Profiling Guides, Section 12, Request for Profile Segment Changes, Additions, or Removals, they may be settled by using appropriately sized and representative lagged dynamic samples or adjusted static models.  The decision to use a lagged dynamic sample or adjusted static model shall be based on the judgment of ERCOT’s Load Profiling Department, subject to ERCOT Board approval.

### Daily Profile Creation Process

ERCOT shall maintain adjusted static models for Load Profiles  and any representative samples for lagged dynamic Load Profiles to create Load Profiles for the target settlement day (backcast) and three (3) days following the current day (forecast).  ERCOT will automatically collect actual weather conditions and weather forecasts to enable the creation of the Load Profiles.  ERCOT will maintain sunrise/sunset information for creating Load Profiles that require these parameters.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT | Clarifications |

| Sponsor's Revision Description | This is a change to the method for creating Load Profiles allowing for the use of lagged dynamic samples for new profiles adopted subsequent to market open. |
| --- | --- |
| Sponsor's Reason for Revision | Allowing the use of lagged dynamic samples for new profiles will provide three important benefits to the market: The time lag between approving a new profile and being able to settle ESI IDs using the new profile will be significantly reduced. New profiles are expected to be subject to significant population change as new ESI IDs are identified for inclusion in the profile; lagged-dynamic samples will accommodate these changes more quickly, accurately and efficiently than is possible with adjusted static models. Use of lagged dynamic samples will allow the market to audit ESI IDs being assigned to new profiles and will create an incentive for CRs using the profile to limit the inclusion of ESI IDs to those appropriate to the profile. |