- ERCOT Protocols

- Section 7:  Congestion Management

- December 1, 2003

7	Congestion Management	7-1

7.1	Overview of ERCOT Congestion Management	7-1

7.2	CSC Zone Determination	7-2

7.3	Congestion Management for CSCs/Zonal Congestion	7-6

7.4	Congestion Management for Local Congestion	7-10

7.5	Transmission Congestion Rights	7-15

7.6	Incorporation of Reliability Must-Run and Out of Merit Order Resources	7-24

7.7	Remedies to Congestion Through Transmission Expansion	7-24

7.8	Congestion Management in McCamey Area	7-24

# Congestion Management

## Overview of ERCOT Congestion Management

ERCOT will implement a Zonal Congestion management scheme on February 15, 2002, that is flow-based, whereby the ERCOT Transmission Grid, including attached Generation Resources and Load, will be divided into a predetermined number of Congestion Zones.  Each Congestion Zone is defined such that each Generation Resource or Load within the Congestion Zone boundaries has a similar effect on the loading (Shift Factor) of Transmission Facilities between Congestion Zones.  For purposes of solving Zonal Congestion the Shift Factor will be assumed the same for all Generation Resources and Loads within a Congestion Zone.  Therefore any imbalance between Loads and Generation Resources in a Congestion Zone will be deemed to have the same impact on a given loading between Congestion Zones.

This Congestion management scheme applies zonal Shift Factors, determined by ERCOT, to predict potential Congestion on CSCs under the known topology of the ERCOT System.  The zonal Shift Factors determined by ERCOT should most closely represent the effect of Generation Resources and Loads in the Congestion Zone on the CSCs with the current topology of ERCOT System.  This scheme is used in the Day Ahead and Adjustment Periods to evaluate potential Congestion and notify the market accordingly.  ERCOT also uses this scheme, along with other factors, to determine if Replacement Reserve Service should be purchased in a Congestion Zone to provide additional Balancing Energy Service to provide a solution for expected Congestion.  ERCOT will use monthly zonal Shift Factors to post and analyze the Schedule impact on CSCs until calculations of interval Shift Factors are implemented in the system.

ERCOT will manage transmission Congestion and will categorize the cost of Congestion management as either Zonal Congestion management costs or Local Congestion management Costs.  Zonal Congestion management costs are those costs that are attributable to managing Congestion on CSCs or predefined Closely Related Elements (CRE).  The costs of managing Zonal Congestion will be directly assigned to QSEs based on each QSE’s impact on CSCs.  All other Congestion management costs are considered Local Congestion management costs.

ERCOT will use the Zonal Congestion management model with Shift Factors of Generation Resources and Loads on CSCs and zonal Balancing Energy Service MCPEs to determine the Shadow Prices of energy across corresponding CSCs.   In addition, the costs of any Replacement Reserve Service procured on a zonal basis for congestion management purposes will be directly assigned to QSEs.  These Shadow Prices will be used in settlement to directly assign the cost of managing Zonal Congestion to QSEs.

The Local Congestion management scheme relies on a more detailed operational model to determine how each particular Resource or Load impacts the transmission system.  This model does not use portfolios and it does use the current known topology of the transmission system.

The cost to solve Local Congestion will be uplifted pro rata to each QSE based on the Load Ratio Share by QSE.

ERCOT will manage Congestion by:

- Evaluating the levels of Zonal Congestion and any other Congestion during the Day Ahead, the Adjustment Period and the Operating Period using appropriate models of the ERCOT Transmission Grid;

- Examining the impacts of QSE energy schedules on CSCs;

- Posting on the MIS, the total megawatt quantity impacts on every CSC, and allowing QSEs to adjust schedules to mitigate potential Congestion on the CSCs;

- Procuring during the Adjustment Period, as needed, Replacement Reserve Service (RPRS) to use with other Resources for which QSEs have submitted Balancing Energy bids to provide sufficient capacity for Balancing Energy flows in the Operating Hour while respecting operational limits of the ERCOT Transmission Grid;

- Determining settlement for QSEs providing RPRS procured to manage Congestion; and

- Determining settlement for QSEs providing Balancing Energy associated with resolving Zonal Congestion.

ERCOT will carry out these steps in accordance with this Section and the Scheduling and Ancillary Service Scheduling and Selection requirements in Sections 4 and 6, respectively.

## CSC Zone Determination

### Principles for Determination and Modification and/or Elimination of CSC Congestion Zones

#### Process for Determining CSCs

ERCOT will reassess CSCs annually, based on changes to the ERCOT System topology, and will identify new Congestion Zones based on the reassessed CSC’s by November 1 of each year.  ERCOT will use the following process to identify CSC’s:

- By October 1, of each year, ERCOT will complete an analysis of load flow data and expected system additions and will determine expected operating limits and constraints to be used in the designation of CSCs for the upcoming calendar year.

(2)	Determine candidates for CSC designation:  A directional transmission path or interface will be considered as a candidate for CSC designation if the actual annual Congestion costs from the prior year including the cost of any zonal BES deployment, RMR, OOMC, OOME, or unit specific deployments used to resolve Congestion on that particular transmission path in that particular direction during the prior year and/or projected annual Congestion costs for the CSC designation year exceeds a threshold amount determined by the appropriate subcommittee.  ERCOT will take into account generation and transmission system changes in designating candidates for CSC.

- (3)	Designate CSCs using the following steps:

(a)	Establish Study Zones.  “Study Zones” are groupings of buses used to test transfer limits using power system simulation tools.  These zones should be reasonable approximations of the final Congestion Zones.  The following steps are performed to establish the Study Zones:

(b)	If no new candidate CSCs are considered other than prior year’s CSCs, then use the Congestion Zones from the previous year as Study Zones.

(c)	If new candidate CSCs are considered, then calculate Shift Factors and perform cluster analysis based on the candidate CSCs to create new Study Zones.

(d)	Perform system simulation studies with transfers between the study zones and find constraints on the transmission system to determine if candidate CSCs are appropriate.

(e)	Determine if the candidate CSCs qualify for CSC designation based, in part, on the criterion that there must be a sufficiently competitive market (at a minimum, apparent Market Solutions readily identifiable) to resolve Congestion on the transmission path to be considered for CSC designation.

- (4)	CSC approval process:  The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the list of constraints should be recommended for approval to the TAC and the Board.

- (5)	At the time of the TAC review and approval process, a Non Opt-In Entity may request an exemption to its zonal placement by submitting a request to the TAC for consideration.  TAC will evaluate the merits of the request against the following criteria.  If TAC agrees with the request, TAC will recommend approval to the Board.

(a)	Exemptions shall not cause significant operational impact to the ERCOT System.  This will normally be satisfied if the request will not result in a change in the annual average Shift Factor for the original or requested Congestion Zone by more than five percent (5%).  After the proposed move, the Zonal Shift Factor in each of the original and requested Congestion Zones shall be no less than ninety-five percent (95%), and no more than one hundred and five percent (105%), of its original Zonal Shift Factor before the proposed move.

Since only busses with generation have an impact on the calculation of Zonal Shift Factors, if no generation exists at a bus that is part of an exemption request, a token generator of one (1) MW will be placed at that bus to test the impact of moving that bus from one (1) zone to another.  This will be done to all busses that make up a request for exemption.  For example, if an Entity is requesting an exemption for five (5) busses and only one (1) bus has generation On-line in the Load flow case being used to test the impact, one (1) MW of generation will be placed at the four (4) other busses and the impact of all five (5) busses on Zonal Shift Factors will be determined as a group as opposed to individually.  The Load flow case used to determine this impact will be the same Load flow case used to calculate annual TCR quantities.

(b)	Exemptions shall not have a commercial impact/gain for the requestor.  This will be measured by whether or not the scheduling which would otherwise result would be eligible for Pre-assigned Congestion Rights.

#### Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will determine Congestion Zones and Zonal Shift Factors by:

- Developing Shift Factors using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

- Using statistical clustering to aggregate transmission busses into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); (ii) the R2 (coefficient of determination) from the cluster analysis should be greater than a threshold level set by the appropriate subcommittee; (iii) station IDs as provided by TDSPs in Protocol Section 15.1.2.5, Response from TDSP to Registration Notification Request, can only be assigned into one Congestion zone; and (iv) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone should not exceed a limit set by the same subcommittee.  The limits will be set to balance the competing goals of minimizing the number of Congestion Zones and maximizing the accuracy of the Commercial Model’s representation of the ERCOT System.

- If the clustering process described in (2) results in the splitting of two or more busses within any single station into different zones, then the following process will be used to adjust the zone selection to place all busses within a single station in the same zone:

- For generating plant stations, determine which zone contains the largest amount of generating capacity of the station and assign all busses of that station to that zone;

- For all other stations, determine which zone is assigned the largest total amount of customer load of the busses in the station and assign all busses of that station to that zone;

- If there is no load or generating capacity in the station, or if the amount of generating capacity and/or load assigned to individual station busses are equal, then the load or generating capacity shall be assigned to the zone containing the lowest bus number of the station.

- Creating Zonal Shift Factors for each CSC by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the Generation Resource megawatt levels on the bus in the load flow base case used to determine CSC limits.

- Congestion Zone approval process: The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the Congestion Zones should be recommended for approval to the TAC and the Board.

#### Determining Closely Related Elements (CREs)

The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the CRE identification process used to determine the CREs that should be recommended for approval to the TAC and the Board.  ERCOT may determine CREs beginning with the year 2003 as follows:

(1)	Determine the Zonal Average Shift Factor for a particular CSC (Xz) for each Zone (z).

(2)	Determine the zonal average shift factor for the candidate CRE (Yz) for each Zone z using the same generation weighting as in (1) but ignoring Boundary Generation Resource busses that would cluster into a different Congestion Zone with respect to the CRE.

(3)	Determine positive “a” applying least-square curve fitting to the following equation:

Yz = a (Xz) + bz  	for all Zones z.

(4)	Using “a” from (3), determine the maximum absolute value of bz.

(5)	Also determine the total capacity (MW) of Boundary Generation Resources that would cluster into a different Congestion Zone.

(6)	If the maximum absolute value of bz is less than a threshold set by the appropriate TAC subcommittee, not to exceed 0.2, and the total capacity of Boundary Generation Resources that would cluster into a different Congestion Zone is less than 1,500 MW, then the element is a CRE for the particular CSC.

By November 1 of each year, the appropriate ERCOT subcommittee will report to the TAC and ERCOT Board with recommended CSC designations, resulting Congestion Zone boundaries, CRE designations and associated Boundary Generation Resources for ERCOT Board review and approval.

Immediately following ERCOT Board approval changes in the CSCs and the resulting Congestion Zone boundaries and CRE designations will be posted on the MIS.  This posting will include a bus-by-bus identification of each Congestion Zone.

## Congestion Management for CSCs/Zonal Congestion

### Determination of Zonal Congestion

ERCOT will analyze energy schedules to determine the existence and extent of Zonal Congestion as part of the Day Ahead Scheduling Process.

### Resolution of Zonal Congestion

ERCOT will resolve Zonal Congestion by the following means:

- Using adjusted Balanced Schedules received from QSEs after ERCOT’s posting of CSC impacts greater than the CSC Limit, ERCOT will reassess the resulting level of Zonal Congestion.  ERCOT will take no further Zonal Congestion actions if the adjusted Balanced Schedules resolve initial Zonal Congestion.

- If Zonal Congestion still exists following receipt of adjusted Balanced Schedules, ERCOT may procure RPRS in accordance with Section 6.6.3.2, ERCOT Ancillary Services Procurement during Adjustment Period, provided that sufficient Resources are available to provide Balancing Energy in the Operating Period, in accordance with Section 6.6.3.2.  ERCOT will then balance the energy within the ERCOT System in the Operating Period respecting all operational limitations of the ERCOT System.

- Zonal Balancing Energy can only be deployed for Congestion Management purposes to resolve flow limit violations on CSCs and CREs.

- Congestion is resolved in Real Time as follows:

(a)	Using Zonal Shift Factors, calculate zonal BES deployments needed to maintain flows within CSC Limits.  If the congestion that needs to be resolved is on a CRE, then Boundary Generation Resources unique to that CRE will be instructed to operate at the respective Resource Plan levels (i.e., this is not an instructed deviation from schedule or Resource Plan).

(b)	Based on (a), deploy unit specific instructions to manage Local Congestion.

(c)	In the event of a contingency, deploy BES to maintain flows within pre-second contingency CSC Limits and deploy unit-specific instructions to manage any resulting Local Congestion.

[PRR424: Replace Subsection 7.3.2(4) as follows when the system change is implemented.] - Congestion is resolved in Real Time as follows: - Using Zonal Shift Factors, estimate zonal BES deployments needed to maintain flows within CSC Limits. If the Congestion that needs to be resolved is on a CRE, then Boundary Generation Resources unique to that CRE will be instructed to operate at the respective Resource Plan levels (i.e., this is not an instructed deviation from schedule or Resource Plan). - Based on (a), deploy unit specific instructions to manage Local Congestion. The Resource specific instructions will be defined as instructed deviations using the following criteria: - When a Resource is instructed to operate at or above an instructed output level and the Resource Plan output level is below the instructed output level, the instructed deviation will be defined as the difference between the instructed output level and Resource Plan output level for the target interval. - When a Resource is instructed to operate at or below an instructed output level and Resource Plan output level is above the instructed output level, the instructed deviation will be defined as the difference between the instructed output level and Resource Plan output level for the target interval. - When a Resource is requested to go to an instructed output level, the instructed deviation will be defined as the difference between the instructed output level and the Resource Plan output level. - Otherwise, the Resource specific deployment will not be defined as instructed deviation. - Clear the Balancing Energy Service market to maintain system power balance and the CSC flow within the CSC limit. - In the event of a contingency, estimate BES deployment to maintain flows within pre-second contingency CSC Limits and deploy unit-specific instructions to manage any resulting Local Congestion and clear the BES market to maintain CSC flows within pre-second contingency CSC Limits.

- The Shift Factors for CSCs and CREs shall be at the same value.

The result of Congestion Management should ensure that:

(1)	Balancing Energy MCPEs are the same in all zones if no CSC or CRE is constrained.

(2)	Balancing Energy MCPEs will be different in two or more Congestion Zones only if one or more CSCs/CREs are constrained.

- (3)	The Shadow Price of a constrained CSC will be calculated by taking the difference in Balancing Energy MCPEs between any two zones divided by the difference in Zonal Shift Factors on the corresponding CSC of the two zones even in the event of an outage of the CSC.  If there is no feasible solution, then these criteria may not be met.

### [Expired 2/15/2002]

#### [Expired 2/15/2002]

#### [Expired 2/15/2002]

#### [Expired 2/15/2002]

##### [Expired 2/15/2002]

### Settlement of Zonal Congestion

#### Balancing Energy CSC Congestion Charge

The calculation for Zonal Congestion Management Charge is as follows:

ICSCiq	= Σ ((QSSiqz - SOiqz) * SFzcsc )z

If ICSCiq > 0 then,

CSCBECSCiq	=	SPCSCi	* MAX (0, ICSCiq)

Else (counterflow)

CSCBECSCiq	=	SPCSCi	* ICSCiq

End

CSCBEiq	=	SUM (CSCBECSCiq)CSC

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCBECSCiq 	CSC Energy Related Congestion Charge, per interval, per CSC, per QSE

CSCBEiq 	CSC Energy Related Congestion Charge, per interval of that QSE

SPCSCi    	Shadow Price per CSC, per interval

ICSCiq    	Scheduled MW Impact per CSC, per interval, per QSE

SFzcsc	Commercial Model Shift Factor per CSC, per zone

QSSiqz	QSE Supply Schedule per interval, per zone, per QSE

SOiqz	Scheduled Obligation (prior to real-time) per interval, per zone, per QSE

The Zonal Shift Factors for settlement purposes will be computed using operational Shift Factors of Generators within the zone weighted by the generation level in the monthly base case.  The monthly base case will be the most recent base case used by ERCOT but no less current than the base case used by ERCOT to determine the number of TCRs available for sale for the corresponding month.

#### Replacement Reserve Service Zonal Congestion Charge

Replacement Reserve Zonal Congestion costs will be allocated directly to the QSE impacting the CSC.  The Replacement Reserve Zonal Congestion costs per interval per QSE will be determined by multiplying the Shadow Price of CSC capacity by the QSE’s CSC impact and in that interval.

The CSC impact is determined by taking for each Congestion Zone, the maximum difference, in any of the four intervals in an hour, between each QSE’s Supply schedules and the QSE Obligation schedules at the time of each round of Replacement Reserve capacity procurement multiplied by the Zonal Shift Factor.  The calculation of the Replacement Reserve Service Zonal Congestion Charge will be as follows:

CSCRPhq = SPCCSCh* MAX (0, ICSChq))

ICSChq   = 	MAX [0,MAX [Σ ((QSSTOPiqz – QOSTOPiqz) * SFzcsc)z ]n]

Where:

h	hour being calculated

i	interval within hour h

n	The number of times that RPRS is procured.

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval, per QSE

SPCCSCi    	Capacity Shadow Price per CSC, per interval

ICSCiq    	Scheduled MW Impact per CSC, per interval, at the time of purchase, per QSE

SFzcsc	Zonal Shift Factor per CSC, per zone

QSSTOPiqz	QSE Supply Schedule per interval, per zone, at the time of purchase, per QSE

QOSTOPiqz	QSE Obligation Schedule per interval, per zone, at the time of purchase, per QSE

## Congestion Management for Local Congestion

### Determination of Local Congestion

ERCOT will use the Operational Model of the ERCOT System in combination with QSE energy  schedules and ERCOT forecasted Load to determine the extent of Local Congestion.

### Resolution of Local Congestion

ERCOT will procure RPRS if necessary to provide sufficient capacity to resolve forecast Local Congestion.  ERCOT will purchase RPRS from the Resource(s) having the lowest cost per megawatt based on each Resource’s bid price and its ability to resolve the Congestion given its Shift Factor on the Local Congestion.

ERCOT will instruct and dispatch Resources in Real Time in a manner that does not exceed the operational limits known to ERCOT.  The Resources that have been instructed to operate within a range  to resolve Local Congestion are paid or pay a price, in accordance with Section 6.8, Compensation for Services Provided, that depends on whether a Market Solution exists for resolving the Local Congestion.

The Local Congestion Costs are embedded in each service deployed to resolve Local Congestion such as OOMC, OOME, and unit specific premiums.

The formula to calculate Local Congestion Replacement Reserve payment can be found in Section 6.8.1.11, Local Congestion Replacement Reserve Payment to QSE, of these Protocols.

### Settlement of Local Congestion Costs

Net Local Congestion costs will be settled in the Balancing Energy settlement process.

Effective August 1, 2003 through December 31, 2003, notwithstanding the provisions of any other section of these Protocols, no Market Solution shall be determined to exist for the payment of Resources being deployed in resolving Local Congestion under any circumstances.

Balancing Energy for Local Congestion shall be paid in accordance with Section 6.8.2.3, Energy Payments of Out of Merit Energy Service.

7.4.3.1 Balancing Energy Up from a Specific Resource

In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever an incremental specific Resource is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLquiz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqviz, NETUEQivq)) * LBEAGRivq)

PMqiuz	=	Max (BPMqiuz, BPMqiuz + MCPEiz )

PMqivz	=	Max (BPMqivz, BPMqivz + MCPEiz )

[PRR424:  Replace the formulas above for PMqiuz and PMqivz with the following upon system implementation:]

PMqiuz	=	Max (BPMqiuz, MCPEiz )

PMqivz	=	Max (BPMqivz, MCPEiz )

BPMqivz 	= 	Min (BPMqiuz)

NETUEQivq			=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq 	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 		=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] /

[SUM(LBEUQqui)u + SUM(LBEDQqui)u +

SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSUquiz	Payment to QSE for providing Balancing Energy Up from Specific unit to solve local congestion per QSE per interval per zone.

LPCRSUqviz	Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone.

PMqiuz	Incremental Premium Specified for that specific unit per interval.

PMqivz	Incremental Premium Specified for that specific Aggregated Unit per interval.

BPMqiuz	Incremental bid premium for that specific unit per interval.

BPMqivz	Incremental bid premium for that specific Aggregated Unit per interval.

OLquiz	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval.

OLqviz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval.

IOLquiz	The Instructed output level for the unit given the unit-specific instruction.

IOLqviz	The Instructed output level for the Aggregated Unit containing the unit given a unit-specific instruction.

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

MCPEiz	Market clearing price of Energy per interval per zone.

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq	The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

LBEAGRiv	The percentage of the aggregated instructions to the aggregated unit that represents the OOM Energy market.

7.4.3.2 Balancing Energy Down from a Specific Resource

In accordance with section 6.7.1.2, if a Market Solution exists, whenever a decremental specific Resource is deployed to resolve the Local Congestion, the QSE will be paid the difference of MCPE in the commercial zone, which the specific Resource is in, and the decremental bid premium specified.

LPCRSDquiz 	= 	-1 * Σ((MCPEiz - BPMqiuz ) * Max(0, Min(OLqiuz -MRquiz OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ((MCPEiz - BPMqivz ) * Max(0, Min(OLqivz -MRqviz  NETDEQiv)))v * LBEAGiv

BPMqivz = Max (BPMqiuz)

NETDEQivq   =	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq 	=		Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=		Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq	=		Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDEQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] /

[SUM(LBEUQqui)u + SUM(LBEDQqui)u +

SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSDqiz	Payment to QSE for providing Balancing Energy Down from specific unit to solve local congestion per QSE per interval per zone

BPMqiuz	Decremental bid premium specified for that specific unit per interval

BPMqivz	Decremental bid premium specified for that specific Aggregated Unit per interval

OLquiz	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqviz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

IOLqui	The instructed output level for the unit given a unit-specific instruction

IOOMDNiuq	OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPiuq	OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

MCPEiz	Market Clearing Price of Energy per interval per zone

LBEUQqui	Local Balancing Energy Up instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down instructions for that interval per specific unit

NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivqThe net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

LBEAGRiv	The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market.

7.4.3.3 Balancing Energy Charge to Solve Local Congestion

In accordance with this section, all the cost associated with Balancing Energy from a specific resource to solve Local Congestion will be allocated on a QSE Load Ratio Share.

LCCiq = - 1* Σ ((LPCRSUqiz + LPCRSDqiz) * LRSiq)

Where:

LPCRSUqiz	Payment to QSE for providing Balancing Energy Up from specific Resource to solve local congestion per QSE per interval per zone

LPCRSDqiz	Payment to QSE for providing Balancing Energy Down from specific Resource to solve local congestion per QSE per interval per zone

LCCiq	Local Congestion load allocation charge per QSE per interval

LRSqi	Load Ratio Share

### Direct Assignment of Local Congestion Costs

ERCOT shall directly assign Local Congestion costs beginning six months after ERCOT’s incurred costs of clearing Local Congestion during a rolling twelve month period reaches 20 million dollars.

## Transmission Congestion Rights

Pre-assigned Congestion Rights (PCRs) are priced differently than TCRs, and PCRs are allocated rather than awarded as a result of the annual TCR auction process.  The eligibility, pricing and annual allocation of PCRs are addressed in Section 7.5.6.  For all other purposes, PCRs are functionally and financially equivalent to TCRs.

### Function of Transmission Congestion Rights

TCRs and PCRs function as financial hedges (similar to financial options with zero strike price) against the marginal costs to resolve Zonal Congestion.  The total costs of Zonal Congestion comprise those portions of RPRS costs and the Balancing Energy Service costs associated with managing Zonal Congestion.  The TCR and PCR holder will receive an amount equal to the directly assigned congestion costs for an equivalent quantity of scheduled flow.  TCRs are not deratable once they are sold in the auction(s), and allocated PCRs are not deratable.

A TCR is a financial right on a specified directional CSC for a particular hour that entitles the holder of record to receive remuneration equal to the sum of non-negative BES and RPRS Shadow Prices ($/MW/15-minute) over all 15-minute intervals within the hour for the corresponding CSC multiplied by 1 MW.

### Procedure for Computation of Transmission Congestion Rights Quantities

The ERCOT System will be modeled and the CSC interfaces determined in accordance with Section 7.2 using steady-state and dynamic power system simulation software.    The capacity of the CSCs will be fully used and there will be no reservation of capacity for any particular segment of Load.

ERCOT will determine CSC Limits as follows:

- Determine the pre-contingency flow limit on the directional CSC to withstand any single contingency of any directional CRE corresponding to the CSC, including the CSC, irrespective of the post-contingency limiting element due to the contingency, for all zone-to-zone transfer studies.

- Determine the pre-contingency flow limit on the CSC to withstand any single contingency that results in any CRE corresponding to the CSC to become the post-contingency limiting element for all zone-to-zone transfer studies.

- The lower of the CSC flow limits from (1) and (2) above sets the CSC Limit.

7.5.2.1	Annual TCR Quantities

Annually determined summer ERCOT peak flow limits for CSCs will provide the starting basis for determining the number of TCRs to be auctioned.  The flow limit established for each CSC on an annual basis may vary from time to time based on changes in system conditions.  The ultimate number of TCRs to be auctioned for the CSCs will take into account forecasted changes in system conditions.

ERCOT will determine the total annual quantity of TCRs for the annual distribution by adjusting the annual summer peak base case CSC Limits by the following amount: for each CSC, the sum over all nodes of the product of the expected level of load at each node from the annual summer peak base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and the Zonal Shift Factor.

7.5.2.2	Monthly TCR Quantities

ERCOT will determine the total quantity of TCRs for the monthly distribution by adjusting the corresponding modified seasonal base case CSC Limits by the following amount: for each CSC, the sum over all nodes of the product of the expected level of load at each node from the modified seasonal base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and the Zonal Shift Factor.  The modified seasonal base case for a particular month will be based on the most limiting situation within the corresponding month.  This analysis takes into account all transmission outages of significant duration and impact with the objective of collecting sufficient revenues from direct assigned congestion costs to pay all TCR holders.

ERCOT may distribute TCRs in amounts less than the total quantities based on expectations of the frequency and magnitude of potential CSC de-rates that could occur during any particular month.

7.5.2.3	ERCOT Posting of TCR Quantities

Upon review and approval of the TCR amount, ERCOT will determine and post on the MIS the total quantity, the amount of PCRs, and the remaining available quantity of TCRs for each CSC.  The remaining available quantity of TCRs for auction will be the total amount of TCRs less the amount of PCRs.

### Annual and Monthly Auction of Transmission Congestion Rights

#### 7.5.3.1	Annual and Monthly Auction Summary

Sixty percent (60%) of the total annual quantity of TCRs less PCRs for any given CSC will be awarded to Market Participants based on the results of an initial annual auction.  These TCRs will be made available in the annual auction for each hour of the year.  For the annual auction, the quantity of TCRs used to calculate the 25% limit, pursuant to Section 7.5.9, Limitation on Ownership or Control of TCRs, is based on the total annual quantity of TCRs (which includes PCRs).  Each annual auction will be completed no later than the 15th of December, on a date set by ERCOT.

Prior to each monthly auction, ERCOT will determine the total monthly quantity of TCRs available for that month based upon forecasted system conditions for the month.  The amount of TCRs that will be auctioned in the monthly auction will be the total monthly quantity of TCRs, less any PCRs, and less any TCRs for that month sold in the annual auction.  For that monthly auction, the quantity of TCRs used to calculate the 25% limit, pursuant to Section 7.5.9, Limitation on Ownership or Control of TCRs, is the greater of (A) the total annual quantity of TCRs, or (B) the total monthly quantity of TCRs.  However, in determining the amount of TCRs available in each monthly auction, ERCOT shall seek to achieve revenue neutrality and avoid a revenue shortfall due to payment to TCR holders for the month.  The TCRs auctioned in the monthly auction will be awarded to Market Participants based on the results of monthly auctions.  These TCRs will be made available in the monthly auction for all hours of each day of each month for which the auction is being held. .  Each monthly auction will be completed no later than the 15th of each month prior to the month that the TCRs are effective, on a date set by ERCOT.

#### 7.5.3.2	Auction Procedures

ERCOT will develop a procedure for authenticating ownership for each TCR.

ERCOT, or its designee, will conduct a single-round, simultaneous combinatorial auction for selling the TCRs available for each annual or monthly auction for all CSCs.  ERCOT shall use a standard linear programming model to maximize the auction revenue subject to the constraints that ERCOT and the bidders provide prior to the start of the auction.  The clearing-price for each TCR will be equal to the corresponding shadow price of the marginal TCR awarded on that CSC.

The auction process will be open to any qualified TCR Account Holder.  To participate in the auctions, a bidder must meet financial security requirements established by ERCOT and must have access to the computer hardware, software and communications equipment required to participate.

##### 7.5.3.2.1	TCR Auction Notices

No less than twenty-days (20) prior to the annual auction and ten (10) days prior to the monthly auction, ERCOT will post:

- The Zonal Shift Factor and zone-to-zone impact matrix (to determine the megawatt impact on each CSC);

- The number of TCRs to be issued for each period (annual, monthly) in that auction;

- The number of PCRs that have been allocated for each CSC;

- Deadline for bidders to satisfy financial requirements;

- Specifications for the equipment necessary to participate in the auction;

- The date and time by which bids must be submitted;

- The bid format;

- CSCs and Closely Related Elements (CREs);

- Any other information of commercial significance to bidders; and

- The total number of TCRs available and used to determine the 25% limit, pursuant to Section 7.5.9, Limitation on Ownership or Control of TCRs.

##### 7.5.3.2.2	Auction

The auction will be a single-round, simultaneous combinatorial auction for selling the TCRs available for an annual or monthly auction for CSCs, with the following steps:

- The following constraints shall be entered into ERCOT’s auction engine model:

- The maximum number of TCRs for any given CSC available for auction.

- The maximum number of TCRs for each CSC that a bidder may purchase in the auction that will ensure the bidder will not exceed the lower of the TCR limitation described in Section 7.5.9, Limitation on Ownership or Control of TCRs, or a bidder’s self-imposed TCR limit set by the bidder prior to the auction.

- The bidder’s Credit Limit approved by ERCOT or, if provided, the bidder’s self-imposed maximum credit obligation will be enforced as a constraint such that the summation over all bids of the product of the awarded TCRs and the bidder’s bid price is less than or equal to the lower of the bidder’s self-imposed maximum obligation or the Credit Limit approved by ERCOT.  The bidder’s self-imposed maximum credit obligation cannot exceed the Credit Limit approved by ERCOT.

- Bids may be submitted for TCRs for any individual CSC and for weighted combinations of CSCs.  Each bid must be non-negative and shall contain the maximum quantity of TCRs, the maximum price, and the distribution weights of the TCRs among the CSCs.  The distribution weight shall be submitted with non-negative ratios (with up to three decimal places) that sum to one (1.000), as illustrated in the table below.

- Examples of Allowable Bids

| Bidders and Bids | CSC 1 Weighting | CSC 2 Weighting | CSC 3 Weighting | Bid Price ($ per MWh) | Maximum Quantity (TCRs) |
| --- | --- | --- | --- | --- | --- |
| A1 | 0.2 | 0.3 | 0.5 | $10.00 | 300 |
| A2 | 1.0 | 0.0 | 0.0 | $5.00 | 185 |
| B | 0.2 | 0.5 | 0.3 | $11.25 | 250 |
| C1 | 0.6 | 0.3 | 0.1 | $7.50 | 240 |
| C2 | 1.0 | 0.0 | 0.0 | $1.00 | 100 |
| D1 | 0.0 | 0.5 | 0.5 | $9.50 | 320 |
| D2 | 0.0 | 1.0 | 0.0 | $3.00 | 140 |
| D3 | 0.0 | 0.0 | 1.0 | $2.50 | 170 |

- NOTE: 3 decimal places for weightings and bid price allowable.

- The auction system will award a combination of TCRs that maximizes the sum of bid prices times the awarded MW for the respective bids while simultaneously observing all applicable constraints.  ERCOT shall award TCRs in quantities rounded to three decimal places.

- The TCR Clearing Price for a CSC will be equal to the corresponding Shadow Price of the marginal TCR awarded on that CSC.  The Shadow Price is representative of the decremental bid-based revenue that would be produced from the TCR auction if one less TCR were available to be auctioned on that CSC.

- If all of the annual TCRs available for a CSC in the annual auction are not awarded (excluding fractional rounding), the TCR Clearing Price for that CSC shall be determined to be zero (0.0) dollars per MWh.  The TCRs not awarded/purchased in the annual auction will be included in the monthly auction.

- If all of the TCRs available for a CSC in the monthly auction are not awarded (excluding fractional rounding), the TCR Clearing Price for that CSC shall be determined to be zero (0.0) dollars per MWh.  The TCRs not awarded/purchased in the monthly auction will not be sold.  Any unpaid PCRs that remain outstanding after five (5) days after the issuance of an invoice will be included in the monthly auction.

- The linear programming solution will solve for one optimal solution, which may result in some bids at the clearing price not being awarded a TCR.  This situation may be minimized because bid prices and bid ratios can be submitted with up to three decimal places.

- The auctioneer shall announce the results of the auction simultaneously to all TCR bidders, by posting the results on the MIS.  For each CSC, the total quantity of TCRs awarded, the TCR Clearing Price, and list of all bids will be posted.  The identity of the bidders will not be posted.

- When an Entity is awarded TCRs as a result of an auction, the TCRs will not become the property of the awardees and the TCRs will not be placed in their accounts until the TCR Invoices have been paid in full.  TCRs awarded are considered when determining ownership.  Ownership of TCRs in a secondary transfer does not change until the recipient accepts the transfer.  Until such time, the initiator of the secondary transfer is considered the owner of the TCRs.

- The twenty-five percent (25%) limit shall be applied to secondary transfers.  When a recipient accepts a secondary transfer, the number of TCRs the recipient owns in any hour for a given CSC during the month will be added to the amount of the TCRs they will receive as a result of the secondary transfer.  If the total exceeds a bidder’s twenty-five percent (25%) limit for the CSC, the secondary transfer shall be cancelled.  Electronic notice shall be given to the secondary transfer initiator and recipient of the cancellation.

### Allocation Method and Timing for Distributing TCR Auction Revenues

The TCR auction revenues from each annual auction and PCR revenues will be divided into monthly revenues in the same proportion as ERCOT’s monthly energy forecast.  This amount will be added to the monthly auction revenue.  The total monthly revenues from the auctions will be credited to QSEs once per month on a monthly Load Ratio Share for the ERCOT peak operating interval during the month basis.  These revenues will be paid to QSEs no later than the 15th of the following month.  PCR holders will be issued invoices that are based on 15 percent of the TCR Clearing Price from the Annual Auction.  Payment of these invoices is required prior to the distribution of PCRs.

Any difference between the Directly Assigned Congestion Costs charged by ERCOT and Zonal Congestion Credits paid to TCR holders for any 15-minute interval will be credited or charged to all QSEs on a Load Ratio Share basis for the same 15-minute interval.  ERCOT shall provide a report to be published on its MIS which details by interval, any zonal congestion revenue shortfalls or surpluses.  The report shall be published on a monthly basis within seven (7) Business Days of the end of a month for the prior month’s activity.  The report shall provide a description of the methodology and billing determinants used to determine and assess any revenue shortfalls or surpluses, and shall include any billing determinants necessary to replicate the results in the report, to the extent such billing determinants are not already available.

### TCR Settlement

To participate in ERCOT’s TCR settlements, a TCR holder need not be a QSE nor be represented by a QSE, but must be registered as a TCR account holder with ERCOT and have the ability to make and accept electronic transfer of funds.

TCRULnq=   -1* MAUCRn+AAUCR∗MEnergynAll MonthsMEnergyn∗LSRnq

Where:

n	Month

q	QSE

TCRULnq	TCR Auction Revenue Uplift per QSE per month

MAUCRn	Monthly TCR Auction Revenue

AAUCRn	Annual TCR Auction Revenue plus PCR revenues

Menergyn	Monthly Energy Forecast

LSRnq	QSE’s Load Share Ratio in that month’s peak interval

Congestion Credits, calculated as described below, will be determined based on the record of ownership in ERCOT’s database, at the end of the day for which the TCRs are effective, for the relevant CSC, based on the published Shadow Price(s) for the same.  The TCR holders will be paid Congestion Credits based on their TCR ownership the day after direct assigned invoice payments are due in accordance with the settlement calendar in Section 9.

TCRPAYhp      =	-1 * Σ (TCRCSChp  + PCRCSChp) * (Σ (SPCSCi /4)i + SPCCSCh))CSC

h	Hour being calculated

i	Intervals within the hour

p	TCR owner of record

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval,  per QSE

TCRCSChp    	Total number of TCRs owned per hour, per TCR owner of record

PCRCSChp	Total number of PCRs owned per hour, per PCR owner of record

SPCSCi    	BES Shadow Price per CSC, per interval

SPCCSCh    	RPRS Capacity Shadow Price per CSC, per hour

TCRPAYhp    	TCR payment per hour, per TCR owner of record

### Direct Allocation of Pre-assigned Congestion Rights

Upon the zonal method being implemented and once auctioned TCRs are available for use, MOUs and ECs which own or have a long-term (greater than five years) contractual commitment for annual capacity and energy from a specific remote Generation Resource, and that commitment was entered into prior to September 1, 1999, are eligible for Pre-assigned Congestion Rights (PCRs).  The PCRs will be allocated as follows:

- PCRs are available on an annual basis until the date upon which a MOU or EC implements retail Customer Choice, or, alternatively, until such other date as may be specified by Order of the PUCT.  NOIEs that opt in after the first of the year shall pay a pro rata share of the full value for remaining months:  (Applicable Annual TCR Clearing Price * 85% * Quantity of PCRs * Number of hours remaining in the year).

- The cost of PCRs shall be equal to fifteen percent (15%) of the applicable annual TCR auction clearing price for each CSC for which a PCR is allocated.

PCRINVyp	=	Σ ((ATCRCPCSCyp  * .15 ) * PCRCSCyp * H))CSC

Where:

p		PCR owner of record

y		Year

ATCRCP		Annual TCR Clearing Price ($/MW)

CSC		Commercially Significant Constraint

H		Number of Hours eligible for PCRs (hours/year)

PCRCSCyp	Total number of PCRs allocated for each of the CSCs, per PCR owner of record, for a given year (MW)

PCRINVyp	TCR payment per hour, per TCR owner of record ($/year)

- The allocation of PCRs to any single Entity shall not exceed twenty-five percent (25%) of the total available TCRs at a CSC interface for any single direction.

- The distribution of PCRs will be sufficient to isolate the MOUs and ECs from any direct-assigned costs associated with transporting the output of their remote Generation Resources to their native Load.

- ERCOT shall distribute PCRs and accordingly populate the TCR ownership database for those months where the NOIE meets eligibility requirements.  PCRs will be included in applicable monthly TCR auctions when the NOIE does not meet eligibility requirements across the year.

- PCRs may be traded in the secondary market.

- Holders of PCRs shall not be precluded from participating in the market to purchase additional TCRs.

### Secondary Market Exchange of Transmission Congestion Rights

Market Participants can exchange TCRs and PCRs in any secondary market.  ERCOT will initially populate a database of hourly TCR holders with the annual and monthly first-purchasers of TCRs and hourly PCR holders.  Subsequent holders or owners (other than first-purchasers) of the TCRs and PCRs who want the change in ownership to be recorded in the ERCOT TCR ownership database will be responsible for assuring that the sellers report the transactions to ERCOT’s satisfaction and that the sellers authorize the transaction within the ERCOT database in accordance to ERCOT’s procedures.  A TCR and PCR may change ownership in the ERCOT TCR ownership database through the end of the Adjustment Period of the operating hour for which it is designated to hedge congestion costs.

### Transmission Congestion Right Value Under Physical Curtailment or De-rating

TCRs and PCRs will retain their full value even in the event of physical curtailment or flow limitation below the megawatt quantity for which TCRs were defined on any given CSC path.

### Limitation on Ownership or Control of TCRs

(1)	No Entity combined with its Affiliates may, either directly or indirectly, own, control, or receive the revenue from more than twenty-five percent (25%) of the total available TCRs and/or PCRs at a CSC interface for any single direction for a given hour.  The total available TCRs for the annual auction and each monthly auction are determined pursuant to Section 7.5.3.1, Annual and Monthly Auction Summary, and included in the Auction Notice posted pursuant to Section 7.5.3.2.1, TCR Auction Notices.

(2)	For purposes of this section, the total number of TCRs available at a CSC interface for any single direction for a given hour is the greater of (a) the total quantity of TCRs available for annual distribution for that CSC interface, direction, and hour, as determined under Section 7.5.2.1, Annual TCR Quantities, or (b) the total quantity of TCRs for the monthly distribution for that CSC interface, direction, and hour, as determined under Section 7.5.2.2, Monthly TCR Quantities.

(3)	If an Entity engages in a transaction with a counter-party, whereby the total ownership or control of the combined Entities exceeds the twenty-five percent (25%) limitation, then any TCRs owned by the counter-parties are not, solely by virtue of that transaction, included in the TCRs that the Entity combined with its Affiliates owns, or controls.

## Incorporation of Reliability Must-Run and Out of Merit Order Resources

RMR and OOM Resources will be used for Congestion Management as described in Sections 4, Scheduling, Section 5, Dispatch and Section 6, Ancillary Services.  RMR and OOM Resources will be used for Congestion Management only in the absence of Market Solutions to alleviate Local Congestion.  RMR and OOM Resources will be compensated in accordance with Section 6.8.

## Remedies to Congestion Through Transmission Expansion

ERCOT will review the cost of correcting localized transmission limitations through new construction or other means, and will compare this to the costs incurred to correct those problems.  ERCOT shall publish the average cost of the constraint by period for each transmission limitation subtotaled by TDSP.  This information will be included in ERCOT’s annual transmission planning report.  If the projected or actual cost of ERCOT directed Congestion Management including all energy and capacity tools, is greater than the cost of correcting the transmission limitations through construction of Facilities, then ERCOT may recommend an upgrade of Facilities to the TDSP.

[PRR397: Insert new Section 7.8, as follows, upon system implementation and notice to the Market] ## Congestion Management in McCamey Area The purpose of this section is to minimize uplifted congestion costs while attempting to reliably maximize the use of the transmission system in the McCamey Area. ### 7.8.1 Determination of McCamey Area (1) The McCamey Area is an area of west Texas in which an abundance of wind-powered generation causes Local Congestion. (2) ERCOT shall identify wind-powered Generation Resources (WPGRs) in the McCamey Area that: (a) have significant impact upon the most limiting local Operational Constraint, and (b) cannot operate their facilities at full capacity simultaneously with other WPGRs in the McCamey Area when all local transmission lines are in service without violating ERCOT reliability criteria. (3) ERCOT shall post a current map of the boundaries of the McCamey Area containing those WPGRs on the ERCOT website. ERCOT shall revise the map as necessary to reflect any changes in transmission system configuration or new interconnections of WPGRs in West Texas. (4) TGRs will be allocated only to Wind Powered Generation Resources in the McCamey Area. ### 7.8.2 Determination of Generation Limits ERCOT shall periodically calculate the amount of generation that can be reliably operated within the McCamey Area considering the projected Dispatch of the total generating capacity in the McCamey Area, including any RMR Unit located in the McCamey Area, and any other factors that may affect the amount of local transmission transfer capability from the McCamey Area. ERCOT shall post the results of this calculation for an Operating Day by 0600 of the Day Ahead. ### 7.8.3 WPGR Qualification To be included in the calculation of total generating capacity within the McCamey Area, a WPGR owner must demonstrate, to ERCOT’s satisfaction, that the WPGR will achieve at least a 10% annual capacity factor. This determination may be made at ERCOT’s discretion. After that demonstration has occurred, if the expected operation of a WPGR can reasonably be expected to operate at less than a 10% annual capacity factor, then the WPGR owner shall notify ERCOT as soon as practical. ### 7.8.4 Determination of Tradeable Generation Rights (1) One Tradable Generation Right (TGR) provides a QSE scheduling WPGRs located in the McCamey Area the opportunity to schedule 1 MW of McCamey Area WPGRs in its Resource Plan for each hour of the relevant period (peak or off-peak). (2) ERCOT shall determine and allocate the on-peak and off-peak quantity of TGRs available to QSEs representing WPGRs located in the McCamey Area. The number of TGRs available must be based upon compliance with the ERCOT security criteria. (3) ERCOT shall issue TGRs in on-peak and off-peak blocks as follows: (a) the on-peak block is for the period from hour ending 0600 through the hour ending1800; and (b) the off-peak block is for the period from hour ending1900 through the hour ending 0500. ### 7.8.5 Allocation of Tradable Generation Rights ERCOT shall allocate the total amount of TGRs (expressed in MWs) for each Operating Day to each QSE representing WPGRs in the McCamey Area based on the process as described in Section 7.8.9, ERCOT Day-Ahead and Real-Time Operations. For Wind-Powered Generation Resources, the capacity used for TGR allocation purposes is the capacity of the Wind-Powered Generation Resource established by the PUCT in its certification of the Generation Resource as either a REC generator or as a recipient of REC offsets, with the exception for new or recommissioned Generation Resources as described elsewhere in this Section 7.8, Congestion Management in McCamey Area. ERCOT shall post the results of the allocation of TGRs on the ERCOT website as soon as the allocation is available. The allocation posting must include: WPGR name, its QSE and MWs of TGRs allocated by period, both on-peak and off-peak. ### 7.8.6 Restriction on QSE Scheduling All QSEs scheduling WPGRs located within the McCamey Area are restricted in their Resource Plan submittals and Real Time operation by this Section 7.8, Congestion Management in McCamey Area. A QSE may designate generation levels for an Operating Hour in its Resource Plan from WPGRs located within the McCamey Area equal to the number of TGRs held by the QSE for that Operating Hour. A QSE may not designate generation levels for an Operating Hour in its Resource Plan from WPGRs located within the McCamey Area in excess of the number of TGRs held by the QSE for that Operating Hour. ### 7.8.7 Trading or Other Uses of TGRs (1) QSEs may use the TGRs from a particular WPGR for scheduling other WPGRs or may trade the TGRs to another Market Participant, but only if the WPGR owner, after being given advance notice of the proposed use or trade, does not object to the QSE’s use or trade of the TGRs. If the WPGR owner objects to the QSE’s proposed TGR use or trade, the WPGR shall submit its objection by electronic notification to ERCOT and the appropriate QSE. (2) All TGRs assigned to the unit(s) contained in the WPGR’s objection will be unavailable for use by the QSE. (3) After the QSE representing a WPGR that is allocated TGRs has initially traded a TGR with the permission of the owner of the WPGR, Market Participants may buy, sell, trade, or exchange that TGR. TGR holders shall notify ERCOT of any changes in TGR ownership before the close of the Day Ahead Market for the affected TGRs in accordance with procedures established by ERCOT for such notices. ### Accommodation of New or Recommissioned WPGRs In the case of a new or recommissioned WPGR located in the McCamey Area, the Resource must demonstrate to ERCOT’s satisfaction that the anticipated capacity factor for the first month of commercial operation is greater than or equal to 10% to be included in the determination of total generating capacity in the McCamey Area. 7.8.8.1 New or Recommissioned Unit Startup and Testing For new or recommissioned Generation Resources in the McCamey Area that rely on phased-in facility interconnection, the WPGR owner shall supply ERCOT with a test plan. This plan must indicate how the Resource will increase capacity, along with the expected dates of such capacity becoming available. During the testing period before commercial operation, ERCOT shall allocate to the new WPGR, TGRs equal to the test plan capability if it is less than 5 MW. ERCOT shall allocate to the new WPGR, TGRs proportional to the test plan’s proportion of available capacity if it is more than 5 MW. 7.8.8.2 New or Recommissioned Unit Commercial Operation The owner of a WPGR coming online in the McCamey Area shall notify ERCOT three (3) Business Days before expected commercial operation. The notice must include the MWs of generation capacity expected to become commercial based on the PUCT certification of the Generation Resource as a REC generator, the date of expected commercial operation, and the scheduling QSE(s) with the associated capacity that the QSE will be scheduling from the WPGR. ERCOT shall allocate TGRs to the QSE(s) scheduling the WPGR as of the first day of expected commercial operation in accordance with the methods described in this Section 7.8.9, ERCOT Day Ahead and Real Time Operations. The owner of the WPGR is responsible for notifying ERCOT of any delays in expected commercial operation, as the calculation of the TGR allocation is partially contingent upon this information. 7.8.9 ERCOT Day Ahead and Real Time Operations ERCOT’s Day Ahead and Real Time operations allocate TGRs and manage McCamey Area Congestion as follows: - ERCOT shall perform the Day Ahead calculation of the total amount of TGRs available as described above under the assumption that all of the WPGRs can respond to ERCOT Dispatch Instructions no more frequently than one instruction per three (3) consecutive hours (Initial TGRs). - ERCOT shall post on its website the allocated TGRs for the Day Ahead. - ERCOT shall also perform a Day Ahead calculation of the total amount of TGRs available including the existence of RRWFs, as described in subsection (4) below (Actual TGRs). Rather than performing two separate Load flow analyses for each day, ERCOT may develop and apply a factor that reasonably represents the difference between the Initial TGRs and Actual TGRs. - Any WPGR in the McCamey Area that cannot respond to ERCOT Dispatch Instructions in consecutive 15-minute intervals is designated as a slow response wind farm (SRWF). ERCOT shall allocate TGRs to SRWFs in proportion to their capacity-ratio-share among all McCamey Area WPGRs of the total available Initial TGRs. A SRWF may not generate at a level above its ownership of TGRs at any time. - Any Generation Resource in the McCamey Area that can respond to ERCOT Dispatch Instructions in consecutive 15-minute intervals is called a rapid response wind farm (RRWF). ERCOT shall allocate TGRs to each RRWF in an amount equal to the RRWF’s capacity ratio share among all McCamey Area WPGRs of the total available Initial TGRs, plus the capacity ratio share among all McCamey Area RRWFs of any positive difference between the available Actual TGRs and Initial TGRs. This sum of RRWF TGRs will be equal to the Day Ahead operating limits for RRWFs. A RRWF may generate at a level above its ownership of TGRs as allowed by system conditions and ERCOT Dispatch Instructions with the objective of maximizing the use of available transmission capacity. - If, near Real Time, ERCOT determines that the Total Transmission Capacity in the McCamey Area is greater than the total amount of TGRs allocated to SRWFs and RRWFs, ERCOT may issue maximum output instructions (e.g., every 15-minutes or hour) to RRWFs that increase the allowed operating level to the maximum permitted by system conditions. - If, near Real Time, ERCOT determines that the Total Transmission Capacity in the McCamey Area is less than the total amount of TGRs allocated to SRWFs and RRWFs, all RRWFs will be instructed to operate below their ownership of TGRs based on their pro-rata share of the transmission capacity shortfall. If RRWFs operate below their TGR ownership level for one hour, then each SRWF will be instructed by ERCOT to reduce output by ten (10) MWs (seventy (70) MW total) allowing each RRWF to increase its output until the total available transmission capacity is utilized. If permitted by system conditions, three (3) hours following the implementation of this procedure, ERCOT shall relax the constraints on each SRWF up to its TGR ownership if doing so will result in RRWFs being allowed to operate at or above their TGR ownership. Otherwise, ERCOT shall relax the SRWF constraints and shall issue instructions to RRWFs that will allow an operating level for all SRWFs and RRWFs that is equal on an installed (and available) capacity-ratio-share basis. - ERCOT shall validate compliance with these rules at two (2) levels. First, ERCOT shall ensure that the amount of generation from WPGRs located in the McCamey Area in a QSE’s Resource Plan does not exceed the amount of TGRs allocated to the QSE for that day. Second, if the amount of generation from a WPGR located in the McCamey area shown in a QSE’s Resource Plan exceed its level of TGR ownership in Real Time, that WPGR is not entitled to be compensated for any amount of OOME Down Services provided in excess of the QSE’s TGR ownership. - ERCOT shall post on its website, as close to Real Time as possible, the flows and operating limits on the applicable transmission constraints in the McCamey Area and the aggregate amount of generation from WPGRs located in the McCamey Area. - ERCOT shall develop a procedure for authenticating ownership for each TGR based on certification information provided by the PUCT (i.e., QSE, Resource owner, name of unit, date of commercial operation, output capacity). 7.8.10 No Effect on Loads Acting as Resource These special provisions are designed to limit transmission Congestion costs in the McCamey Area and do not restrict Load acting as a Resource from supplying Ancillary Services in the ERCOT Ancillary Services markets.