- ERCOT Protocols

- Section 11:  Data Acquisition and Aggregation

- February 1, 2004

11	Data Acquisition and Aggregation	11-1

11.1	Data Acquisition And Aggregation From ERCOT Polled Settlement (EPS) Metered Entities	11-1

11.2	Data Acquisition From TDSPs	11-4

11.3	Load Data Aggregation	11-5

11.4	Data Aggregation	11-14

11.5	Unaccounted For Energy (UFE) Analysis	11-16

# Data Acquisition and Aggregation

## Data Acquisition And Aggregation From ERCOT Polled Settlement (EPS) Metered Entities

### Overview

ERCOT will collect interval data from all ERCOT Polled Settlement (EPS) metered Entities according to Section 10, Metering of these Protocols.  Collection of data from EPS metered Entities will be done via the Meter Data Acquisition System (MDAS).  This data will be validated, edited, estimated, adjusted, netted, loss corrected, split, and aggregated as necessary to provide the required settlement inputs.

### EPS Meter Data Collection

ERCOT will perform remote interrogation of EPS metered Entities to provide the necessary data for the Settlement process.  Upon initiation of connection with the meter, the MDAS will verify that the meter’s internal IDR protocol (TIM setting) and the device identifier programmed into the IDR match the master file database stored in the MDAS.  If remote-polling fails for any reason, ERCOT will work closely with the TDSP to resolve data collection problems within the time frame defined in Section 10, Metering.

### EPS Meter Time Synchronization

ERCOT will update the clock of any EPS meter that falls outside the threshold defined in Section 10, Metering of these Protocols.  ERCOT will notify the TDSP regarding any meter that is determined to be inconsistent in its timekeeping function.  The TDSP will facilitate correction of this problem within the time frame detailed in Section 10, Metering.

### EPS Meter Data Validation, Editing, and Estimation (VEE)

After EPS time synchronization has been completed and interval meter data has been retrieved, ERCOT will determine if the data is valid.  The validation process will include, but not be limited to the following tests:

- Flagging of intervals with missing data;

- Exception reporting if the total number of zero values for any channel exceeds the tolerance limit;

- Exception reporting if the total number of power outage intervals exceeds the tolerance limit;

- Channel level exception reporting if any single interval breaches the upper or lower threshold of the limit;

- Channel level validation of the percent change between two consecutive intervals being greater than the established tolerance limit;

- Data overlap validation test, which rejects validations when the current interrogation of data overlaps data previously collected;

- Channel level energy tolerance test, which reports exceptions of total energy accumulated from the interval data not being equivalent to the energy calculated from the meter register’s start and stop readings;

- Validation that the number of expected intervals equals the number of actual intervals collected during the interrogation process; and,

- Validation of data between primary, backup and check meters where available.

ERCOT will perform editing and estimation of EPS meter data according to the Protocols defined in Section 10, Metering.  The Validation process occurs each time data is collected from a meter.

### Loss Compensation of EPS Meter Data

Adjustments will be made to actual metered consumption to accommodate the energy consumption related to line and transformation losses to the point of interconnection with the ERCOT Transmission Grid.   These adjustments are intended specifically to correct the metered consumption when the meter is not located at the point of interconnection with the ERCOT Transmission Grid.

The preferred method for loss compensation and correction is by programming of the meter.  Recognizing that some meters may not have the ability to perform internal compensation computations, ERCOT’s MDAS will have the ability to perform approved loss compensation as necessary.

TDSPs requesting loss compensation for a specific meter will comply with Section 10 of these Protocols and the Operating Guides.  ERCOT will provide a compensation mechanism based upon a single percentage value submitted by the TDSP and approved by ERCOT.  The loss compensation percentage value will remain in place and will be applied to all intervals of data until such time as the TDSP submits, and ERCOT approves, revised loss compensation values.  The loss compensation percentage values should not be changed more than once annually.

### EPS Meter Netting

As allowed by Section 10, Metering of these Protocols, ERCOT will perform the approved netting schemes, which sum the meters at a given Generation Resource site.  Both Load consumption and Generation Resource production meters, will be combined together to obtain a total amount of Load or Resource.

### EPS Generation Meter Splitting

ERCOT will apply any approved splitting schemes to partition generation production and auxiliary Load when the unit is not in operation in accordance with Section 10, Metering of these Protocols.

### Correction of EPS Meter Data for Non Opt-In Transmission Losses

ERCOT will correct the total Load of EPS meters for Non Opt-In Entities that have transmission behind the settlement meters and are connected to the ERCOT Transmission Grid via bi-directional metering for actual Transmission Losses according to Section 13, Transmission and Distribution Losses of these Protocols, ERCOT will populate Settlement Interval Load data for Non Opt-In Entities into a single data set to be used in the Load aggregation process Non Opt-In Entities will be able to extract Load data from the Data Archive via the MIS.

### Treatment of Non Opt-In Radially Connected Entities

At Non Opt-In metering points for which the TDSP is supplying data to ERCOT, the interval Load data that is not bi-directional will have each point of delivery treated as an individual ESI ID.

### Treatment of EPS Load Data

For EPS metering that ERCOT is populating ESI ID Load data, ERCOT will:

(1)	Utilize the data for all settlement calculations and reports;

(2)	Provide the TDSP and LSE with daily KWH consumption information in accordance with SET 867_03 for interval data upon completion of the data aggregation process for the settlement day.  Data changes during settlement runs subsequent to the most current settlement run will result in an additional SET 867_03 being provided to the TDSP and LSE; and,

(3)	Accommodate CR switching via the standard switching process and timelines.

[PIP 204& PRR283: For EPS meter data that will have ESI IDs populated by ERCOT (load resources and generation load), current design does not provide suitable mechanisms for making ESI ID consumption data available to market participants.  Additionally, there is no mechanism for Market Participants to obtain EPS Recorder level data (RIDs).  Implementation of this PIP will provide for access to both types of data sets.]

11.1.10 Treatment of EPS Load Data (ESI IDs)

For EPS metering that ERCOT is populating ESI ID Load data, ERCOT will:

(1)	Be identified as the Meter Reading Entity (MRE).

(2)	Utilize the data for all settlement calculations and reports.

11.1.11 Treatment of EPS Recorder Data (RIDs)

For EPS Meter RID data, ERCOT will:

- Be identified as the Meter Reading Entity (MRE).

- Model and populate data to appropriate channels such that netting and aggregation conform to protocols requirements.

- Make RID interval data available to the QSE, TDSP, LSE and ERCOT via an extract.

The ERCOT Read RID data extract for will:

(a)	Select all ERCOT Read RIDs for the market participant.

(b)	Provide interval data as populated by ERCOT for each channel associated to an RID.

(c)	Allow the user to define start and stop times of the data to be retrieved.

(d)	Allow for retrieval of data based upon a user defined “as-of” date.

## Data Acquisition From TDSPs

### Overview

This section addresses the manner in which ERCOT will receive and validate data from the TDSPs regarding usage for Generation Resources and Load from TDSPs metered Entities as defined in Section 10, Metering, of these Protocols.

### Data provision and verification of Non EPS Metered Points

The TDSP will provide data for TDSP Metered Entities as defined in Section 10, Metering, of these Protocols.

The TDSP will provide data in accordance with the TDSP meter data responsibilities detailed in Section 10, Metering, and will conform to data formats specified in Section 19, Texas Standard Electronic Transaction (SET).

ERCOT will:

- Provide the TDSP a notification of successful/unsuccessful data transfer for the 867 meter data transactions submitted.  At the ESI ID level, the TDSP will be notified of successful and unsuccessful validations;

- Validate that the correct TDSP is submitting meter consumption data on an individual ESI ID basis.  Any rejections from this validation will be returned to the TDSP in the format specified in Section 19, Texas SET;

- Provide a report to the TDSP listing each ESI ID for which ERCOT has not received consumption data for thirty-eight (38) days; and,

- Synchronize the MDAS data with the Customer registration system on a daily basis to ensure the appropriate relationship between the ESI ID, LSE and/or Power Generation Entity, and the meter.  MDAS will provide versioning to ensure ESI ID characteristic changes are time stamped.

## Load Data Aggregation

Data Aggregation is the process of netting, grouping and summing Load consumption data, applying appropriate profiles, Transmission Loss Factors and Distribution Loss Factors and calculating and allocating UFE to determine each QSE and/or Load Serving Entities responsibility by Settlement Interval by Congestion Zone and by other prescribed aggregation determinants.  The process of aggregating Load data provides the determinants that allow the settlement to occur.

### Estimation of Missing Data

The Data Aggregation System will perform estimation of missing interval and non-interval retail Load meter consumption data for use in settlement when actual meter consumption data is unavailable.

### Non-Interval Missing Consumption Data Estimation

The Data Aggregation System will distinguish each ESI ID for which consumption data has not been received for the Operating Day.  Non-Interval ESI ID locations for which no actual consumption exists for the specified Operating Day will be pre-aggregated by like components which may include but are not limited to the following sets:

- (1)	QSE;

- (2)	Load Serving Entity;

- (3)	Congestion Zone;

- (4)	UFE zone;

- (5)	Profile ID;

- (6)	Distribution Loss Factor code;

- (7)	TDSP;

- (8)	Read start date (reading from date); and,

- (9)	Read stop date (reading to date).

Estimates of missing data are based on Profile ID, which includes:

- (1)	Profile Type;

- (2)	Weather Zone;

- (3)	Meter type;

- (4)	Weather sensitivity; and,

- (5)	TOU Schedule.

Profile application will take aggregated non-interval consumption data and apply the Load Profile in order to create interval consumption data.  Profiled non-interval data is calculated by dividing the aggregated ESI ID’s total KWH for a specific time period (usually a month) by the Profile Class’ KWH for the same specific time period and scaling the Load Profile for that same specific Operating Day by the resulting value to provide the profiled non-interval consumption data.

PNDOperating Day  =	(Actual KWHSpecific Time Period / CP KWHSpecific Time Period) * LPOperating Day

Where:

PND	Profiled Non-Interval Data

CP	Class Profile

LP	Load Profile (daily interval data set) in KWh

Any active ESI ID on the Trade Day being settled for which ERCOT does not have a meter read within 6 months of the Trade Day will not have a usage estimate applied to their load profile.  That is, the estimate for these customers will be their assigned profile without any scaling factor applied.

### Interval Consumption Data Estimation

ERCOT will estimate all ESI IDs with IDRs for which consumption data has not been received for the Operating Day.   The method for estimating interval data for ESI IDs with IDRs is a “Weather Response Informed Proxy Day” technique.  This approach seeks to increase estimation accuracy by segmenting ESI IDs with IDRs into two groups based on a known indicator of Load, i.e. weather.  The classification of ESI IDs into a weather-sensitive group and a non-weather-sensitive group determines the proxy day method used for estimation purposes.  The proxy day estimation method for each group captures the factors that best predict the ESI ID-specific Load shape for the Operating Day.

#### Weather Responsiveness Determination

ERCOT will perform the weather responsiveness test for all ESI IDs with IDRs.  For each ESI ID with an IDR, two variables are calculated across all summer weekdays (June-September):

- Daily kWh; and,

- Average daily dry bulb temperature ((MAX + MIN)/2).

An R-square (Pearson Product Moment Correlation) is calculated and all ESI IDs with R-square above 0.6 are classified as Weather Sensitive  IDR (WSIDR) and all below are classified as Non-Weather Sensitive IDR (NWSIDR).

Weather responsiveness determination shall be calculated upon initial data conversion from TDSPs and afterward in November of each year.  If an ESI ID does not have a summer’s worth of interval data, it will be classified as non-weather sensitive.  Any changes to the weather sensitivity portion of the Profile Class resulting from this calculation will be reflected in the registration database.  ERCOT will produce a report that will contain the IDR ESI IDs that have changed from weather sensitive to non-weather sensitive or vice versa.  This report will be published to Market Participants in a data extract via the MIS.

As a part of the Profile Class assignment, TDSPs will assign a non-weather sensitive classification to all newly installed IDRs.  TDSP will also assign a default Profile Type to all IDRs to serve as a final default estimation routine provided all proxy day selection criteria are exhausted.  Use of the default Profile Type is defined further in Section 11.3.3.2, Weather Sensitive (WSIDR) Proxy Day Method, and Section 11.3.3.3, Non-Weather Sensitive (NWSIDR) Proxy Day Method.

#### Weather Sensitive (WSIDR) Proxy Day Method

For ESI IDs designated as WSIDR, ERCOT will use this weather-sensitive proxy day selection method.  ESI IDs within the same Weather Zone will be grouped together.  The proxy days will be the same for all ESI IDs within each of the Weather Zones.  This method incorporates the following:

- To determine eligible proxy days, select all days (of matching weekday/weekend day type and season) within five degrees of the maximum temperature of the target Operating Day based on the previous 365 days and then limit the selection to those days that have their maximum temperatures occurring within two (2) hours of the maximum temperature hour of occurrence of the operating day.  The maximum temperature separation criterion provides initial assurance that the eligible day will have a similar diurnal temperature pattern as the target settlement Operating Day.

- Perform two (2) tests on each potential proxy day identified in Step 1;

  - Temperature magnitude test sums the squared differences between the hourly temperatures of the target Operating Day and the hourly temperatures of the potential proxy day.

  - Temperature shape test calculates the incremental change in temperature from hour to hour during the day and sums the squared differences between the corresponding values of the target Operating Day and the potential proxy day.

- Each potential proxy day for each test described in Step 2 is ranked in ascending order based on the sum of squared differences.

- A final ranking is performed with the temperature magnitude test weighted more heavily than the shape test.  The weighting factors are seventy percent (70%) and thirty percent (30%).

- Select the top three ranked eligible days.

- For each ESI ID, do the following:

  - Use the top ranked proxy day for the target Operating Day, if available.

  - If the top ranked proxy day data is not available, use the second ranked proxy day data as the estimate.

  - If the second ranked proxy day data is not available, use the third proxy day.

  - If no data is available for any of the proxy days selected, then default to the non-weather sensitive proxy day selection list.

  - If still no estimate is generated when the non-weather sensitive method is used, the IDR data will be estimated using the default Profile Class average profile for the trade day.

#### Non-Weather Sensitive (NWSIDR) Proxy Day Method

For ESI IDs designated as NWSIDR, ERCOT will use a method for proxy day determination.  This method incorporates the following:

- Use the most recent proxy day for which data is available as the estimate for the target Operating Day.  From historical ESI ID specific interval data, choose the most recent occurrence of the appropriate day of the week (Su, M, T, W, Th, F, Sa) corresponding to the day of the week of the Operating Day (Holidays are treated as Sundays) within the most recent twelve (12) months of the Operating Day; or

- If there is no historic interval data available according to (1) above, the IDR data will be estimated using the default profile assigned to the ESI ID for the Operating Day.

[PRR471:  Replace Section 11.3.3.3 item (2) above with the following upon system implementation:]

- If there is no historic interval data available according to (1) above, the IDR data will be estimated using the default profile assigned to the ESI ID for the Operating Day.  If non-interval consumption data with a meter read within six (6) months of the trade day is available, and if the ESI ID was profiled with a non-interval meter data type code within ninety (90) days of the trade day, the default profile shall be estimated and/or scaled in accordance with Section 11.3.2, Non-Interval Missing Consumption Data Estimation.

#### IDR Estimation Reporting

ERCOT shall produce a report detailing the proxy day selection list for both NWSIDR and WSIDR methodologies.  This report will be made available to Market Participants on a daily basis.

### Data Aggregation Processing for Actual Data

The Data Aggregation System will apply backcasted profiles to aggregated actual non-interval consumption data for use in settlement when actual meter consumption data is available.  IDR ESI IDs for which actual data exists will be used directly in the Data Aggregation process.

#### Application of Profiles to Non-Interval Data

Non-Interval ESI ID locations for which actual consumption exists for the specified Operating Day will be pre-aggregated by like components which may include but are not limited to the following sets:

- QSE

- Load Serving Entity

- Congestion Zone

- UFE zone

- Profile ID

- Distribution Loss Factor code

- TDSP

- Read start date (reading from date)

- Read stop date (reading to date)

Profile application will take aggregated non-interval consumption data and apply the Load Profile in order to create interval consumption data.  Profiled non-interval data is calculated by dividing the aggregated ESI ID’s total KWH for a specific time period (usually a month) by the Profile Class’ KWH for the same specific time period and scaling the Load Profile for that same specific Operating Day by the resulting value to provide the profiled non-interval consumption data.

PND Operating Day   =  (Actual KWH Specific Time Period/CP KWH Specific Time Period) * LP Operating Day

Where:

PND		Profiled Non-Interval Data

CP		Class Profile

LP		Load Profile (daily interval data set) in KWh

### Adjustment of Consumption Data for Losses

The ERCOT Data Aggregation System shall adjust consumption data for Transmission and Distribution Losses.  The sources of data used in this process are:

- Estimated non-interval data profiled via usage factor routine

- Estimated proxy day interval data

- Profiled actual non-interval data

- Actual interval data

- Distribution Loss Factors

- Transmission Loss Factors (Average ERCOT-Wide)

ERCOT will apply Distribution Loss Factors to aggregate levels of Load data in accordance with Section 13, Transmission and Distribution Losses, of these Protocols.  Aggregated Loads will be adjusted for Distribution Losses based upon Distribution Loss Factor code correlated to the Distribution Loss Factor for each TDSP.  Loads that are transmission connected or that are settled at transmission level will not be allocated distribution level losses.

NDLALi Aggregated Group   =  BLi Aggregated Group * 1 / (1- DLFi Aggregated Group)

Where:

i	Interval

NDLALi	Net distribution loss adjusted Load

BLi	Base Load

DLFi	Distribution Loss Factor (voltage code specific)

ERCOT will apply the ERCOT wide Transmission Loss Factor to the net distribution loss adjusted Loads to produce a net loss adjusted aggregated Load value for each aggregation set.  ERCOT wide Transmission Loss Factors will be developed in accordance with Section 13, Transmission and Distribution Losses, of these Protocols.

NLALi Aggregated Group   =  NDLALi Aggregated Group   *  1/ (1 - TLFi)

Where:

i	Interval

NLALi:	Net loss adjusted Load

NDLALi:	Net distribution loss adjusted Load

TLFi:	Transmission Loss Factor (ERCOT wide factor)

### Unaccounted for Energy Calculation (UFE) and Allocation

The Data Aggregation System shall adjust the net loss adjusted Load for each aggregated group for Unaccounted for Energy (UFE).  The Data Aggregation process will calculate the difference between net loss adjusted Load for the entire ERCOT System, which has been adjusted for Distribution Losses and Transmission Losses, and the total system Load (generation) in order to determine the total UFE.  The calculated UFE for each Settlement Interval is then allocated to Loads.  Net flow out of ERCOT on a DC Tie will be deemed as Load, and net flow into ERCOT on a DC Tie will be deemed as a Resource

#### Calculation of ERCOT-Wide UFE

The Data Aggregation System will calculate ERCOT-wide UFE as the difference between the total generation supplied to a specific physical region (ERCOT) and the total Load, adjusted for losses in that same physical region (ERCOT) during each Settlement Interval.  UFE may be positive or negative in any single Settlement Interval.

UFEi (MWh)   =   ERCOT Generationi Total  – ERCOT Net Loss Adjusted Loadi Total

#### Allocation of UFE

ERCOT will allocate UFE to specific categories based upon adjusted Load Ratio Share.  The adjusted Load Ratio Share will be determined using the following UFE category weighting factors:

- 0.0 - Transmission Voltage level IDR Non Opt-in Entities

- 0.10  - Distribution Voltage level IDR Non Opt-in Entities

- 0.10 - Transmission Voltage level IDR Premises

- 0.50 - Distribution Voltage level IDR Premises

- 1.00 - Distribution Voltage level Profiled Premises

The ERCOT Data Aggregation System shall provide a mechanism to change the UFE category weighting factors for specific transition periods.

#### UFE Allocation to UFE Categories:

For each premise category, and for each settlement interval, the UFE allocated to each UFE category is calculated as follows:

UFE PRiz	= 	UFEiz	X	[(fPRiz	X	LPRiz )  / LUFEiz]

UFE IDRiz	= 	UFEiz	X	[(fIDRiz	X	LIDRiz) / LUFEiz]

UFE TRiz	= 	UFEiz	X	[(fTriz	X	LTRiz) / LUFEiz]

UFE TNOIEiz	= 	UFEiz	X	[(fTNOIEiz	X	LTNOIEiz) / LUFEiz]

UFE DNOIEiz	= 	UFEiz	X	[(fDNOIEiz	X	LDNOIEiz) / LUFEiz]

LUFEiz	=   	fPRiz   X   LPRiz   +   fIDRiz   X   LIDRiz  +fTRiz  X   LTRiz    +   fTNOIEiz   X  LTNOIEiz   +   fDNOIEiz   X   LDNOIEiz

LUFEi	= 	SUM(LUFEiz)z

Where:

i	Interval

z	Zone

UFE PRiz	Amount of UFE allocated to Profile category

UFE IDRiz	Amount of UFE allocated to IDR category

UFE TRiz	Amount of UFE allocated to Transmission category

UFE TNOIEiz	Amount of UFE allocated to Transmission voltage level NOIE category

UFE DNOIEiz	Amount of UFE allocated to Distribution voltage level NOIE category

UFEi	Total ERCOT system UFE

and

LPRiz  	=	Aggregate Load of Profile category – Adjusted for losses

LIDRiz	=	Aggregate Load of all IDR category – Adjusted for losses

LTRiz	=	Aggregate Load of Transmission category – Adjusted for losses

LTNOIEiz	=	Aggregate Load of Transmission level non opt-in category – Adjusted for losses

LDNOIEiz	=	Aggregate Load of Distribution level non opt-in category – Adjusted for losses

and

fPRiz	=	Adjustment Percentage for Profiled premises per interval per zone

fIDRiz	=	Adjustment Percentage for IDR premises per interval per zone

fTRiz	=	Adjustment Percentage for Transmission premises per interval per zone

fTNOIEiz	=	Adjustment Percentage for Transmission voltage level non opt-in premises per interval per zone

fDNOIEiz	=	Adjustment Percentage for  Distribution voltage level non opt-in  premises per interval per zone

LUFEiz:	=	Adjusted total UFE Allocation reference Load

#### UFE Allocation to LSEs within UFE Categories

The UFE allocated to each UFE category type is then allocated to the LSEs within each UFE category based upon each LSEs share of the total Load for the UFE category.

UFE PRiz LSE	=	UFE PRiz	X	(LPRiz LSE/LPRiz)

UFE IDRiz LSE	=	UFE IDRiz	X	(LIDRiz LSE / LIDRiz)

UFE TRiz LSE	=	UFE TRiz	X	(LTRiz LSE / LTRiz)

UFE TNOIEiz LSE	=	UFE TNOIEiz	X	(LTNOIEiz LSE / LTNOIEiz)

UFE DNOIEiz LSE	=	UFE DNOIEiz	X	(LDNOIEiz LSE / LDNOIEiz)

Where:

i	Interval

z	Zone

UFE PRiz LSE	UFE allocated to LSE in UFE profile category

UFE IDRiz LSE	UFE allocated to LSE in UFE IDR category

UFE TRiz LSE	UFE allocated to LSE in UFE Transmission category

UFE TNOIEiz LSE	UFE allocated to LSE in UFE Transmission NOIE category

UFE DNOIEiz LSE	UFE allocated to LSE in UFE Distribution NOIE category

and

UFE PRiz	Amount of UFE allocated to Profile category

UFE IDRiz	Amount of UFE allocated to IDR category

UFE TRiz	Amount of UFE allocated to Transmission category

UFE TNOIEiz	Amount of UFE allocated to Transmission voltage level NOIE category

UFE DNOIEiz	Amount of UFE allocated to Distribution voltage level NOIE category

and

LPRiz LSE	LSE Load in Profile Category – Adjusted for losses

LIDRiz LSE 	LSE Load in IDR Category – Adjusted for losses

LTRiz LSE	LSE Load in Transmission Category – Adjusted for losses

LTNOIEiz LSE	LSE Load in Transmission NOIE Category – Adjusted for losses

LDNOIEiz LSE	LSE Load in Distribution NOIE Category – Adjusted for losses

and

LPRiz  	Aggregate Load of Profile category – Adjusted for losses

LIDRiz	Aggregate Load of all IDR category – Adjusted for losses

LTRiz	Aggregate Load of Transmission category – Adjusted for losses

LTNOIEiz	Aggregate Load of Transmission level non opt-in category – Adjusted for losses

LDNOIEiz	Aggregate Load of Distribution level non opt-in  category – Adjusted for losses

## Data Aggregation

### Aggregate Retail Load Data

Adjusted Metered Load (AML), defined as retail Load data that has been adjusted for UFE and Transmission and/or Distribution Losses, will be aggregated into distinct grouping and segments such as LSE, QSE, and Congestion Zone, and provided to settlement.

#### Aggregated Load Data Posting/Availability

ERCOT will make available to Market Participants the following information on a daily basis:

- LSE Load Ratio Share data by ERCOT total.

- LSE base Load values, by unique combination of QSE, Congestion Zone, UFE zone, Profile Type, Distribution Loss Factor code and TDSP.

- LSE base Load plus allocation of Distribution Losses by unique combination of QSE, Congestion Zone, UFE zone, Profile Type, Distribution Loss Factor code and TDSP.

- LSE base Load plus allocation of Distribution Losses and Transmission Losses by unique combination of QSE, Congestion Zone, UFE zone, Profile Type, Distribution Loss Factor code and TDSP.

- LSE base Load plus allocation of Distribution Losses, Transmission Losses UFE; by unique combination of QSE, Congestion Zone, UFE zone, Profile Type, Distribution Loss Factor code and TDSP allocation of Distribution Losses, Transmission Losses and UFE.

Each Market Participant will have access only to its own information and/or the information of the Entities, which it represents.  ERCOT will make the aforementioned data for each settlement run available to Market Participants via MIS within forty-eight (48) hours of finalizing the data for Settlement Statements.

### Generation Meter Data Aggregation

ERCOT will perform generation aggregation by the following distinct criteria sets:

- By UFE zone: This data set is used in the calculation of Unaccounted for Energy (UFE) in the Load aggregation process.

- By Generation Resource (meter ID), by Generation Entities, by QSE and Congestion Zone: This data set is passed to the settlement process for generation imbalance calculations.

#### Participant Specific Generation Data Posting/Availability

The following market-specific generation information will be made available by ERCOT to each Market Participant:

- Generation Unit Production by Generation Resource Entity

- Generation Resource Entity total generation production by Congestion Zone

- QSE total generation production by Congestion Zone

Each Market Participant will have access only to its own information and/or the information of the Entities, which it represents.

#### General Public Generation Data Posting/Availability

The following general market generation information will be posted to the MIS:

- Total Generation

- Total AML

ERCOT will make the aforementioned data for each settlement run type available to Market Participants via MIS within forty-eight (48) hours of finalizing the data for Settlement Statements.

## Unaccounted For Energy (UFE) Analysis

### Overview

ERCOT will establish a UFE Analysis Team chaired by ERCOT and consisting of Market Participants and ERCOT personnel who report to the Technical Advisory Committee:

The UFE Analysis Team will:

- Analyze UFE data from January 01, 2002 to December 31, 2002

- Post findings to the MIS on a monthly basis

- Provide a recommendation by April 2003 to the ERCOT Board

Prior to any recommendation from the UFE Analysis Team, ERCOT has one (1) UFE Zone, which encompasses all of ERCOT.

### UFE Analysis Process

The UFE analysis team may request additional data from TDSPs to be provided to ERCOT for UFE analysis.  Data that is confidential is to be deemed as such.  ERCOT may request installation of additional meters for analysis, but TDSPs are not required to do so.

The UFE analysis process will:

- Sub-divide ERCOT into a practical granularity of “UFE Analysis Zones,” but no more than ten (10) zones, which can provide reliable and accurate data to be used for Cost/Benefit Analysis and decision making purposes;

- Identify factors that are contributing to UFE and work with the appropriate Entities to rectify problems causing UFE;

- Compare ERCOT-wide UFE to UFE within UFE Analysis Zones and investigate the differences;

- Determine if additional UFE Zones are necessary, and if so, identify the appropriate zones and the associated implementation cost;

- Post UFE analysis findings to Market Participants on a monthly basis;

- Provide a recommendation to the ERCOT Board by April 2003

#### Sub-divide ERCOT into a practical granularity of “UFE Analysis Zones,” but no more than ten (10) zones, which can provide reliable and accurate data to be used for Cost/Benefit Analysis and decision making purposes

ERCOT shall make every effort possible during the design and implementation of market systems to implement mechanisms to capture data considered valuable in the UFE analysis effort.  This will include but not be limited to integration and storage of  “old” Control Area net Real Time Load signals on a Settlement Interval basis.

ERCOT will determine what data exists in the market and its location and attributes including:

- Load data

- Generation data

- Available import and export data

- Sufficiency and relative accuracy of the data that exists for UFE analysis with:

  - Revenue quality data preferred and

  - Acceptable SCADA integrated by ERCOT or the TDSP used on a temporary basis (no more than one month)

  - Minimum installation requirements of new IDR meters for the sole purpose of UFE Analysis

- Zonal criteria (TDSP areas, weather, profiling, etc.)

Based on the existing data, ERCOT will determine all zones that can be defined, and will determine which data is to be stored by ERCOT for UFE analysis purposes.

The UFE analysis team will determine the initial set of UFE Analysis Zones no later than December 01, 2001 but as soon as practical to provide adequate time for installation of any required metering.  The cost of installing any metering solely for UFE analysis purposes will be recovered through the ERCOT System Administration Fee.  A TDSP shall not remove and shall maintain existing metering facilities that have the potential to define UFE zones, as defined by ERCOT, until such time as UFE analysis zones have been defined according to this Protocol section.

#### Identify factors that contribute to UFE and work with the appropriate Entities to rectify problems causing UFE;

The UFE analysis team will perform the following steps to identify factors that are contributing to UFE:

- Calculate UFE for UFE Analysis Zones as described above;

- Compare ERCOT-wide UFE to UFE within UFE Analysis Zones;

- Determine possible causes of abnormal UFE;

- Work with the appropriate Entities to rectify UFE problems when possible;

- Determine if UFE Analysis Zones need to be modified to identify UFE problems;

- Perform analysis going forward that will include the additional UFE Analysis Zones;

#### Determine if UFE Zones are necessary, and if so, identify the appropriate zones and the associated implementation cost;

If UFE problems persist through the analysis phase, the UFE analysis team will make a recommendation of proposed UFE Zones according to the following guidelines:

- Cost-benefit analysis;

- Installation requirements for Revenue Quality Meters;

- Impact on the settlement system;

- Impact on Market Participant systems;

- Cost of UFE to Market Participants;

#### The UFE analysis team will provide an implementation plan for the proposed UFE Zone(s).  Post UFE analysis findings to Market Participants on a monthly basis;

The UFE analysis team will determine the content of the monthly reports.  The reports will be posted monthly for Market Participants.

ERCOT will post the following information on the MIS on the same date that other monthly system information is made available for each Settlement Interval during the month, which may include:

- Total ERCOT-wide UFE MWhs, UFE cost, and percent of total interval MWhs

- ERCOT-wide UFE MWhs, UFE cost, and percent of total interval MWhs allocated to UFE allocation categories

- Each UFE allocation categories’ UFE MWhs, zonal UFE cost, and percent of total interval MWhs for each UFE Analysis Zone

- Results of any monthly UFE Analysis

- Notice of any factors that may be contributing to UFE, by UFE Analysis Zone

- Notice of any plan to rectify factors that may be contributing to UFE, by UFE Analysis Zone

ERCOT will facilitate quarterly Market Participant discussion groups to report findings and gather input for areas of further analysis.

#### Provide a recommendation to the ERCOT Board by April 2003

The UFE analysis team will present a recommendation to the ERCOT Board by April 2003.  By the end of April 2003, ERCOT will provide a report to the ERCOT Board and Market Participants, which provides the following:

- Summaries of all prior UFE analysis;

- Factors that have contributed to UFE, by UFE Analysis Zone;

- Recommendations for UFE Zones (including cost analysis); and,

- Proposed UFE Zone implementation plan (if recommended).

### Data Collection and Metering Requirements For UFE Analysis Zones

TDSP’s have the option of either supplying zonal interval meter data to ERCOT, or providing remote interrogation for ERCOT to directly read the meter on demand.  Temporary telemetry arrangements through SCADA may be integrated by either the TDSP or ERCOT.  SCADA data will not be used for more than one month for purposes of UFE analysis.

If a UFE Analysis Zone can be created with the addition of minimum metering expense, ERCOT will require the TDSP to install the meter.  All meters on the ERCOT Transmission Grid used to define UFE Analysis Zones will be installed, maintained, and read by the associated TDSP.  If the meters have the ability to be remotely interrogated, ERCOT can directly read these meters.

ERCOT will be provided remote access to the UFE Analysis meter in accordance with Section 10, Metering, of these Protocols.  If the TDSP does not provide for remote interrogation, the TDSP will be required to submit meter data to ERCOT in the same manner defined in Section 10, Metering, of these Protocols.  TDSP’s will provide meter data in the same format and method that retail interval meter data is submitted specified in Section 19, Texas SET, of these Protocols.  Furthermore, the TDSP is responsible for meeting the time synchronization requirement in accordance with Section 10, Metering, of these Protocols.