| PRR Number | 490PRR | PRR Title | Loads acting as a Resource (LaaR) Annual Testing Description |
| --- | --- | --- | --- |
| Recommended Action |  | Approve |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 6.10.2, General Capacity Testing Requirements |  |
| Proposed Effective Date |  | May 1, 2004. |  |
| Priority & Rank Assigned |  | N/A |  |
| Summary of Impact Analysis |  | No ERCOT computer system, business function, or grid operations impacts; minor (0.1 FTE already within budget) staffing impacts once LaaRs testing is implemented. |  |
| Revision Description |  | This revision changes LaaR seasonal testing to an annual test consisting of a telemetry check and includes a biennial test of the under frequency relay for LaaRs providing Responsive Reserve Service. |  |
| Benefit |  | Clarifies LaaR annual testing requirements. |  |
| PRS Recommendation |  | PRS recommends approval of PRR490 as submitted. |  |
| TAC Recommendation |  | TAC recommends approval of PRR490 as recommended by PRS. |  |

| Sponsor |  |
| --- | --- |
| Name | Mark Patterson |
| E-mail Address | mpatterson@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive, Taylor, TX 76574 |
| Phone Number | 512-248-3912 |
| Fax Number | 512-248-6560 |

ERCOT/Market Segment Impacts and Benefits

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Reduction of FTE required to process paperwork by changing the testing from seasonal to annual. |  | Clearly define the annual testing requirements for LaaRs. |
| MARKET SEGMENT |  |  |  |
| Consumer | none | none | none |
| LSE: General, Including NOIE | none | none |  |
| LSE: CR & REP | none | none |  |
| QSE | None required but the QSE’s may assist their resources in meeting the annual test requirements. | minimal | Clearly define the annual testing requirements for LaaRs. |
| Resource | Complete telemetry test on each LaaR annually and test the UFR every two years. | minimal | Clearly define the annual testing requirements for LaaRs. |
| TDSP | none | none | none |

Proposed Protocol Language Revision

### 6.10.2 General Capacity Testing Requirements

QSEs shall provide ERCOT a list identifying each Generation Resource unit that is expected to operate more than one hundred sixty eight (168) hours in a Season as a provider of energy and/or Ancillary Services.  ERCOT shall evaluate, during each Season of expected operation, the Net Dependable Capability of each unit expected to operate more than one hundred sixty eight (168) hours during that season, except for any Generation Resources used solely for energy services and whose capacity is less than ten (10) MW.  Prior to the beginning of each Season, QSEs shall identify the Generation Resources to be tested during the Season and the specific week of the test if known.  This schedule may be modified by the QSE (including retests) during the Season.  QSEs not identifying a specific week for a Generation Resource unit test must test the unit within the first one hundred sixty eight (168) hours of run time during the Season or operate with a Net Dependable Capability equal to the highest integrated hourly MWh output demonstrated during the first one hundred sixty eight (168) hours of run time.  QSEs do not have to bring units On-line or shut down solely for the purpose of the seasonal verification.  Any unit for which the QSE desires qualification to provide Ancillary Services shall have its Net Dependable Capability verified prior to providing services using the Generation Resource unit even if it fits the less than one hundred sixty eight (168) hour or small capacity exception.  The capability of hydro units operating in the synchronous condenser fast response mode to provide hydro Responsive Reserve shall be evaluated by Season.

Load acting as a Resource to provide Ancillary Services shall have its telemetry attributes verified by ERCOT annually.  In addition, once every two (2) years, any LaaR providing Responsive Reserve Service shall test the under frequency relay or the output from the solid-state switch, whichever applies, for correct operation.  However, if the Load’s performance has been verified through response to an actual event, the data from the event can be used to meet the annual telemetry verification requirement for that year and/or the biennial relay testing requirement.

Specific Loads to be used for the first time as a Resource to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve must be correctly evaluated (tripped, or simulated trip, if approved by ERCOT) prior to their qualification to provide Ancillary Services.  At a time selected mutually by the Load and ERCOT, ERCOT will notify the QSE representing such Load of the need to confirm that Loads proposed to be used to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve Services can be de-energized, or simulated, if applicable, to ERCOT’s reasonable satisfaction.  ERCOT shall develop a standard test for simulation for Load interruption required under this subsection.  Loads used to provide Responsive Reserve Service shall be qualified for correct operation by their host TDSP.

QSEs shall be responsible for qualifying any Load desiring to have the QSE represent it to provide Balancing Up Load (BUL) Service.

The QSE shall nominate to ERCOT, at least annually, that it is representing an amount of BUL for which it wishes to be qualified to provide.  At a time selected by ERCOT, the ERCOT operator will notify the QSE that it wants to verify the QSE’s ability to provide ERCOT with the appropriate signal simulating that it has initiated an ERCOT requested BUL reduction.  The QSE Operator will immediately simulate the initiation of the reduction and provide ERCOT with the appropriate signal representing some amount of Load to be qualified to provide BUL Resources.  Once ERCOT has verified that it has received an appropriate Load reduction signal from the QSE and has successfully completed the BUL registration process, the QSE will be qualified to provide BUL Resources.  Any changes to the BUL portfolio will require subsequent updates to the registration process.  For NOIEs representing specific Loads qualified as BULs that are located behind the NOIE Settlement Metering points, the NOIE shall provide an alternative unique descriptor of the qualified BUL Load for ERCOT’s records.

Generation Resources and Loads acting as Resources shall be evaluated at least annually by ERCOT for:

- correct operation of telemetry of the breakers controlling the Resource;

- correct mapping of QSE-provided telemetry of Ancillary Service energy to the appropriate energy Settlement Meter,

- data rate update requirements, and

- any other required telemetry attributes.

All Generation Resources and Loads acting as a Resource shall meet all requirements specified in the Operating Guides for proper response to system frequency.  ERCOT may reduce the amount a Resource may contribute toward Ancillary Services if it finds unsatisfactory performance of the Resource as defined in these Protocols and the Operating Guides.

Qualification of a Resource, including a Load acting as a Resource, shall remain valid for such Resource in the event of a change of QSE for the Resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |

| Sponsor's Revision Description | This revision changes LaaR seasonal testing to an annual test consisting of a telemetry check and includes a biennial test of the under frequency relay for LaaRs providing Responsive Reserve Service. |
| --- | --- |
| Sponsor's Reason for Revision | Clarifies LaaR annual testing requirements. |