| PRR Number | 501PRR | PRR Title | A/S Revision to Down Balancing Requirement |
| --- | --- | --- | --- |
| Recommended Action |  | Approve |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 4.5.2, Receipt of QSE’s Balancing Energy Bid Curves 6.5.2 (2), Balancing Energy Service |  |
| Proposed Effective Date |  | Upon updating required documentation. |  |
| Priority & Rank Assigned |  | N/A |  |
| Revision Description |  | Revise the Down Balancing Requirement to account for Regulation Down obligations. |  |
| Benefit |  | Regulation Down requirements aren’t appropriately accounted for in the requirements for Down Balancing. The requirement should be revised to be the lesser of the 12% requirement or the quantity of downward capability less the amount of rewarded Regulation Down for each QSE. |  |
| PRS Recommendation |  | PRS recommended approval of PRR501 as revised by ERCOT Comments dated 061704. |  |
| TAC Recommendation |  | TAC recommends approval of PRR501 as recommended by PRS. |  |

| Sponsor |  |
| --- | --- |
| Name | Clayton Greer |
| E-mail Address | Clayton.greer@constellation.com |
| Company | Constellation |
| Company Address | 8768 Polo Drive, Fort Worth, TX 76123 |
| Phone Number | (817) 938-3692 |
| Fax Number |  |

Proposed Protocol Language Revision

### Receipt of QSE’s Balancing Energy Bid Curves

ERCOT will receive QSEs Balancing Energy bid curves by the end of each Adjustment Period for an Operating Hour.  QSEs may voluntarily submit Balancing Energy Service Up and Down bid curves to ERCOT for use in the Operating Period.

QSEs shall be required to submit mandatory Balancing Energy Service Down bid curves.  The QSEs will submit Balancing Energy Service Down bids based on the specifications set forth below.

1) For each Congestion Zone , the minimum bid shall be the lesser of the following:

(a) ERCOT’s required percentage of Balancing Energy Service Down bids as posted at 0600 of the Day Ahead multiplied by the QSE’s net energy schedule for Balancing Energy Service requirements.

(b) the QSE’s net energy schedule for Balancing Energy Service requirements less the QSE’s minimum sustainable capacity (except for OOMC units and RMR Units) represented in its Resource Plan.

MinimumDownBalanceBidByZone=Min(NetEneSch∗DBESReq%),(NetEneSch−uOLMinCap)

2)If the sum of the Balancing Energy Service Down bids for each Congestion Zone from (1) above  is more than the QSE’s net energy schedule for Balancing Energy Service requirements less the QSE’s minimum sustainable capacity (except for OOMC units and RMR Units) represented in its Resource Plan, and less the QSE’s scheduled Regulation Down Service, then this amount shall be the minimum ERCOT-wide Balancing Energy requirement.  In this case, no zonal requirement shall exist as the requirement will be ERCOT-wide.

MinimumDownBalanceBidERCOTWide=Max0,zNetEneSch−uOLMinCap−SchRegDn

The net energy schedule for Balancing Energy Service requirements is the QSE’s scheduled energy per CM Zone less any QSE trades, energy scheduled by ERCOT from Resources under an RMR Agreement, and energy scheduled from Resources that received an OOMC Dispatch Instruction for the hour.

Where:

NetEneSch = 	the QSE’s scheduled energy per CM zone less any QSE trades, energy scheduled by ERCOT from Resources under an RMR Agreement, and energy scheduled from Resources that received an OOMC Dispatch Instruction for the hour.

OLMinCap = 	Low Sustainable Limit for each On-line and available unit (except for RMR units and OOMC units).

SchRegDn = 	Regulation Service Down scheduled by the QSE

DBESReq% = 	Balancing Energy Service Down percentage posted by ERCOT

z = 	Congestion Management Zone

u = 	unit

Balancing bid curves will have the following components:

- $/MWh

- Quantity (MW)

- Congestion Zone

- Ramp Rate

[PIP 210:  When block deployment for Loads acting as a Resource can be implemented, add the following paragraph:]

A Load acting as a Resource has the option to request a Load bid to be deployed only as a complete block.  The block deployment option shall be selected at the time of bid submittal.

### 6.5.2	Balancing Energy Service

- QSEs shall provide Balancing Energy Down Service bids prior to the close of the Adjustment Period equal to or greater than the requirement according to the amounts set forth for the QSE in

- Section 4.5.2, Receipt of QSE’s Balancing Energy Bid Curves.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT | Revised language |
| ERCOT | Revised language |
| ERCOT | Revised language |
| Constellation | Revised language |

| Sponsor's Revision Description | Revise the Down Balancing Requirement to account for Regulation Down obligations. |
| --- | --- |
| Sponsor's Reason for Revision | Regulation Down requirements aren’t appropriately accounted for in the requirements for Down Balancing. The requirement should be revised to be the lesser of the 12% requirement or the quantity of downward capability less the amount of rewarded Regulation Down for each QSE. |