ERCOT Operating Guides

Section 1:  Introduction

Market Overview, Change Control, 
Relationships and Definitions

October 1, 2003

Contents

1.	Introduction	1-4

1.1	Document Purpose	1-4

1.2	Document Relationship	1-4

1.3	Change Control Process	1-4

1.3.1	Urgent Requests	1-4

1.4	Market Overview	1-4

1.5	Conformance to NERC Policies and Procedures	1-4

1.6	Operating Definitions	1-4

1.7	Entity Definitions and Roles	1-4

1.7.1	ERCOT Control Area Authority	1-4

1.7.2	Qualified Scheduling Entities	1-4

1.7.3	Transmission Operator (TO)	1-4

1.7.4	Resource Entity	1-4

1.7.5	Load Serving Entities (LSE)	1-4

1.8	Operational Training	1-4

1.8.1	Operator Training Objectives	1-4

1.8.2	ERCOT Operations Training Seminar	1-4

1.8.3	ERCOT Black Start Training	1-4

1.8.4	Criteria For The Selection Of Operators	1-4

1.8.5	Training Practices	1-4

1.9	Document Revisions	1-4

Protocol References

Reference:  Protocols Section 5.2.1	1-4

Reference:  Protocols Section 6.10.5.4	1-4

Reference:  Protocols Section 22, Attachment A, item 8(c)	1-4

# Introduction

## Document Purpose

These Electric Reliability Council of Texas (ERCOT) Operating Guides supplement the Protocols and describe the working relationship between the ERCOT Control Area Authority and entities within the ERCOT System that interact with the ERCOT Control Area Authority on a minute-to-minute basis to ensure the reliability and security of the ERCOT System, as shown in the following diagram:

Specific practices described in these Guides for the ERCOT System are consistent with the North American Electric Reliability Council Operating Policies and the ERCOT Protocols and consist of the following Guides:

Section 1 -  Introduction

Section 2 - System Operations

Section 3 - Operational Interfaces

Section 4 - Emergency Operation

Section 5 - Planning

Section 6 - Reports and Forms

Section 7 -  Disturbance Monitoring and System Protection

Section 8 – Operational Metering and Communication

Reference:  Protocols Section 5.2.1

ERCOT and TDSPs shall operate the ERCOT System in compliance with Good Utility Practice and NERC and ERCOT standards, policies, guidelines and operating procedures.  These Protocols shall control to the extent of any inconsistency between the Protocols and any of the following documents:

- Any reliability guides applicable to ERCOT, including the Operating Guides;

- The NERC Operating Manual and ERCOT procedures manual, supplied by NERC and ERCOT, respectively, as references for dispatchers to use during normal and emergency operations of the ERCOT Transmission Grid;

- Specific operating procedures, submitted to ERCOT by individual transmission Facility owners or operators to address operating problems on their respective grids that could affect operation of the interconnected ERCOT Transmission Grid; and

- Guidelines established by the ERCOT Board, which may be more stringent than those established by NERC for the secure operation of the ERCOT System.

## Document Relationship

The relationship of these Operating Guides to other documents is defined in the following diagram:

It is the responsibility of the ERCOT Control Area Authority to develop internal Operating Procedures.  QSEs and TDSPs are required to develop their own internal Operating Procedures with respect to the ERCOT System entities, using their services and the Control Area Authority.  However, in doing so, QSE and TDSP Operating Procedures shall incorporate the relevant requirements of these Operating Guides.

These Operating Guides are derived from the ERCOT Protocols and the NERC Policies and Procedures.  As the ERCOT system is within the State of Texas, the Public Utilities Commission of Texas (PUCT) defines additional requirements for the ERCOT Control Area Authority and connected entities.

PUCT requirements and directives and the ERCOT Protocols supercede these Guides.  NERC Policies and Procedures, with the exception of the specific modifications defined in these Guides will also be followed.

## Change Control Process

The following process has been established to announce, develop, revise, and approve ERCOT Operating Guides:

- Proposed Operating Guides Revision(s) are submitted via email to ERCOT.

- The Reliability and Operations Subcommittee (ROS) reviews and recommends action.

- ERCOT Subcommittees review and comment.

- Comments are submitted via e-mail to ERCOT.

- ERCOT posts and forwards comments to ROS.

- ROS reviews all comments and the decision reached on each comment is posted.

- ROS submits recommended Operating Guides Revision(s) to the TAC for approval.

- ERCOT Board of Directors is advised of the TAC-approved Operating Guides Revision(s).

- Where additional expertise is needed, the ROS may assign responsibility for review of and drafting in response to Operating Guides Revision Requests to working groups or task forces that it creates.  However, the ROS shall retain ultimate responsibility for the processing of all Operating Guides Revision Requests.

- Throughout the Operating Guides, references are made to the Protocols.  When these Protocol references are revised, and they require no other revisions to the Operating Guides other than updating the referenced Protocol language, then the Operating Guides can be changed by ERCOT Staff as directed by ROS rather than using the OGRR process outlined in this section.

- Although Protocol language has been included in the Operating Guides, Protocols are subject to their own revision process, not this Operating Guide revision process.

| CHANGE CONTROL PROCESS |  |
| --- | --- |
| STEP | PROCESS |
| - Request to Revise the Operating Guides | Requests to revise the ERCOT Operating Guides are submitted via e-mail to ERCOT utilizing the Operating Guides Revision Request (OGRR) form, available on the ERCOT website. Any individual or organization with a legitimate interest in the reliability and operation of the ERCOT System may offer a revision for consideration. These individuals and organizations include, but are not limited to: - Transmission Owners - Generation Owners - Load Serving Entities - Power Marketers - Qualified Scheduling Entities - ERCOT Subcommittees, Task Forces, and Working Groups - ERCOT Staff - PUCT Market Oversight Division The ROS shall be responsible for monitoring all Protocol revisions and initiating any OGRRs that may be needed. All OGRRs must include a description of the requested revision, the reason for the suggested revision, a list of any affected ERCOT Operating Guides sections, and the suggested Operating Guides language. ERCOT will verify each OGRR for completeness and shall notify submitter, within five (5) business days of receipt, if the OGRR is incomplete. Complete OGRRs will be posted on the ERCOT website within five business days of ERCOT verification for an initial comment period of twenty-one (21) calendar days. |
| - Reliability and Operations Subcommittee Reviews Request and Recommends Action | ERCOT will distribute the OGRR to the ROS. The ROS will review the proposed OGRR. If it determines that the revision(s) is needed, the ROS will draft proposed Operating Guides language utilizing suggested Operating Guides language from the requester, if appropriate. The proposed Operating Guides language will be posted on the ERCOT website within two (2) business days of the ROS decision. The proposed language will also be forwarded to the other three TAC Subcommittees (PRS, WMS, RMS). If the ROS determines that the revision(s) is not needed, it prepares a response to the requester. The response will be posted on the ERCOT website within two (2) business days of the ROS decision. |
| - ERCOT Subcommittees Review and Comment | All ERCOT subcommittees may comment on the proposed Operating Guides language. The PRS must determine whether the proposed Operating Guides language is consistent with the ERCOT Protocols, and submit its determination to the ROS. If the PRS determines that the proposed Operating Guides language is inconsistent with the current Protocols, the ROS is responsible for submitting a Protocol Revision Request, if it so desires. |
| - Comments submitted to Reliability and Operations Subcommittee | The PRS determination and all comments must be submitted via e-mail to ERCOT within twenty-one (21) calendar days of the proposed Operating Guides language being posted on the ERCOT website. If ERCOT does not receive comments before the deadline specified, it will be assumed that the reviewing subcommittee(s) is in agreement with the proposed revision(s). All comments will be forwarded to the Chair of the ROS. |
| - Reliability and Operations Subcommittee Deliberates on Comments | The ROS will review all comments received no later than its first regularly scheduled meeting following the subcommittee comment period. Based on the comments received and its own review, the ROS will take one of the following action: - Modify the proposed Operating Guides language to address all comments received, or - Modify the proposed Operating Guides language to address some comments received and provide an explanation as to why all comments were not addressed, or - Not modify the proposed Operating Guides language and provide an explanation as to why none of the comments were addressed, or - Terminate the revision process for this OGRR and provide an explanation for this action. The action taken by the ROS on the proposed Operating Guides language will be reported to the reviewing subcommittees. The ROS will document its disposition of all comments received and its decisions, and forward to ERCOT for posting on the ERCOT website. If the ROS modifies the proposed Operating Guides language significantly, it may repeat Steps 3, 4 and 5. Should the ROS not accept some of the comments received, a joint task force may be established between the disagreeing subcommittees to attempt to resolve the disagreement. A subcommittee whose comments are not accepted, or a requester that has an OGRR rejected by the ROS may make a written request to the TAC for further consideration of its comments. |
| - Reliability and Operations Subcommittee Submits Proposed Guides Revision(s) for TAC Vote | The ROS’s final proposed Operating Guides language is posted on the ERCOT website and submitted to the TAC for vote at its next regularly scheduled meeting. All requests for Operating Guides revisions shall be accompanied by a PRS Protocol consistency determination. If no comments opposing the posted language are received within twenty-one (21) calendar days of posting, the TAC may, at the Chair’s discretion, take action via electronic mail rather than at the next regularly scheduled meeting. The TAC will also be provided with the explanation of actions requiring TAC approval. In all cases, electronic mail may be used to expedite the process. If the TAC does not approve the final proposed Operating Guides Revision language, it may return the proposal to the ROS for further work or it may terminate the revision process. |
| - TAC notifies the Board of all votes taken related to Guides Revisions | The Chair of TAC shall report the results of all votes by TAC related to Operating Guides revisions to the Board at their next regularly scheduled meeting. |
| - 8. Effective date for Guides Revisions | The Effective Date for Operating Guides Revisions shall be the first day of the month following TAC approval unless otherwise designated by ROS. |

### Urgent Requests

A proposed Operating Guides Revision may be labeled “urgent” by the submitter if such submitter reasonably believes that failure to rapidly implement an Operating Guides Revision would result in the significant impairment of ERCOT System reliability or market operation.  If a revision request is labeled “urgent” and the ROS determines that such Operating Guides Revision:

    - Requires immediate attention from TAC due to serious concerns about the ERCOT System reliability or market operations under unmodified language; and

    - Is of a nature that allows immediate, interim implementation without negative consequence to the reliability and integrity of the ERCOT System or market operations.

Then ROS shall review the proposed Operating Guides Revision language and either establish an accelerated timeline under which to process the Operating Guides Revision or recommend immediate implementation and approval of the proposed language revision.

Notice of an Operating Guides Revision pursuant to this section, 1.3.1, “Urgent Requests,” shall be posted on the ERCOT website within two (2) business days.  ROS may exercise the option to vote on Urgent Requests via e-mail.  Any revisions to the Operating Guides that take effect pursuant to an Urgent Request shall be subject to PRS review and TAC confirmation.

## Market Overview

The Electric Reliability Council of Texas (ERCOT) is a not-for-profit, regulated Control Area Authority (CAA) and member of the North American Electric Reliability Council (NERC).  The Public Utility Commission of Texas (PUCT) is the principal regulatory authority of ERCOT.  The Federal Energy Regulatory Commission (FERC) has regulatory authority over the DC Tie Line and future interconnections between ERCOT and out of state entities.

The primary responsibility of the ERCOT Control Area Authority is to ensure secure and reliable power grid operation within the ERCOT System in accordance with NERC standards, where applicable and to facilitate the efficient use of the electric transmission system by the market participants.

The ERCOT System market is based on bilateral transactions between buyers and sellers. ERCOT will ensure that the power grid can accommodate market participant schedules.  In addition, ERCOT Control Area Authority will monitor the power grid in real time and provide ancillary services to resolve capacity shortfalls, transmission congestion and maintain reliability.

The ERCOT Control Area Authority only interacts directly with Qualified Scheduling Entities and Transmission and/or Distribution Service Providers to operate the ERCOT System in a reliable and secure manner.

The relationship between the entities is defined in Section 1.7 of these Guides.

## Conformance to NERC Policies and Procedures

The Electric Reliability Council of Texas Operating Guides are for the purpose of outlining specific practices for the ERCOT System.  These practices are consistent with the North American Electric Reliability Council Operating Policies.  For application in ERCOT, some NERC Policies must be adapted to fit the unique characteristics of the ERCOT System.  Specific necessary adaptations are listed below:

| NERC Policy | ERCOT Adaptation |
| --- | --- |
| Time and Frequency Control | Sustained frequency deviations from 60 Hz result in time error. |
| - Time Error Monitoring | ERCOT will monitor accumulated time error and initiate time corrections. The instantaneous time error is available to all ERCOT QSEs in the ERCOT website. When time error is equal to or greater than ±3 seconds, ERCOT may initiate a time correction. The correction will be ended when the error is less than ±0.5 seconds. The time correction may be postponed if it is determined that load patterns in the immediate future will result in the desired time correction; however, at no time should the accumulated time error be allowed to exceed five (5) seconds. |
| - Time Error Correction | When a time correction is necessary, ERCOT will adjust scheduled frequency in the following manner. ERCOT will arrange for more or less resources. Information to be passed along will include the correction frequency (59.98 Hz for fast and 60.02 Hz for slow) and the start time. A time correction may be terminated after five (5) hours, after any hour without a one-half (0.5) second error reduction. The Control Area Authority will provide adequate notice of ending the time correction. |
| Inadvertent Interchange Management | The only Inadvertent Energy will be between ERCOT and SPP and/or CFE. Accounting / payback will be handled according to NERC policy. The hourly difference between a Control Area’s actual net interchange and a Control Area’s scheduled net interchange is classified as inadvertent energy. All inadvertent energy is placed in an Inadvertent Payback Account to be paid back in kind. |
| Control Surveys | Not all of the surveys defined by NERC apply to a system the size of ERCOT and /or a single Control Area interconnection such as ERCOT. |
| Load Shedding and System Restoration | Automatic firm load shedding will be initiated as follows: Frequency % Load Relief 59.3 Hz 5% 58.9 Hz 10% 58.5 Hz 10% Load shedding will be widely dispersed in each TDSP, with no regard for the REP, and will be accomplished by using under-frequency relays. Automatic under-frequency firm load shed relays shall be calibrated to have a breaker trip initiate time delay of 0.6 seconds at rated voltage at a frequency 0.10 Hz above the threshold. (The delay will unavoidably vary with the magnitude of frequency deviation below the threshold and with voltage magnitude due to the nature of electromechanical technology.) The operating time of associated auxiliary tripping relays shall be included as a part of the overall time delay. Circuit breaker operating time shall not be included. If the frequency drops below 58.5 Hz, each TDSP must determine additional steps it will take to survive. Under-frequency relays may be installed on transmission facilities under the direction of ERCOT, provided the relays are set at 58.0 cycles or below, are not directional, and have at least 2.0 seconds time delay. Load restoration will be under the direction of the ERCOT Control Area Authority. A TDSP may by mutual agreement, with the approval of the TAC, arrange to have all or part of its automatic load shedding obligation carried by another TDSP. ERCOT will be notified and provided with the details of any such arrangement prior to implementation |
| Information Exchange – Disturbance Reporting | ERCOT will record the following data from the ERCOT System for frequency deviations of .175 Hz or greater and will use this information to generate the ERCOT initial Disturbance Report: - ERCOT and individual QSE control biases. - Net MW Capability (including any capability lost) at the time of the disturbance. - Net System Load (MW) for the hour ending closest to the time of the disturbance. - Scheduled Net Interchange (MW) at the time of the disturbance. - Amount of interruptible load (MW) tripped. Reference: Protocols Section 6.10.5.4 For all frequency deviations exceeding 0.175 Hz, ERCOT shall measure and record each two (2) second scan rate values of real power output for each QSE Resource providing Responsive Reserve. ERCOT shall measure and record the MW data beginning one (1) minute prior to the start of the frequency excursion event until ten (10) minutes after the start of the frequency excursion event. |
| Reliability Criteria | ERCOT operating reserve requirements are more restrictive than the concepts in the NERC Operating Manual. The ERCOT Responsive Reserve Obligation is 2300 MW. |

## Operating Definitions

A complete list of definitions is contained within the Protocols, Section 2, Definitions and Acronyms.  The following definitions apply specifically to reliability and security operation.

It is essential to the reliability of the ERCOT system that all appropriate personnel use and understand the same terms in their daily operations.  The definitions in this section are intended to enable the ERCOT Control Area Authority, Qualified Scheduling Entities and Transmission and Distribution Service Provider operators to effectively communicate on an ongoing basis.

| Capacitor | Produces reactive power (VAR source) for voltage control and causes the system power factor to move towards a leading condition. |
| --- | --- |

| Designated Agent | Any entity that is authorized to perform actions or functions on behalf of another entity. |
| --- | --- |

| Generator Reactive Power Sign/Direction Terminology | (1) Lagging power factor operating condition is when VAR flow is out of the generating unit (overexcited generator) and into the transmission system and is considered to be positive (+) flow, i.e., in the same direction as MW power flow. The generator is producing MVARs (2) Leading power factor operating condition is when VAR flow is into the generating unit (underexcited generator) and out of the transmission system and is considered to be negative (-) flow, i.e., in the opposite direction as MW power flow. The generator is absorbing MVARs. |
| --- | --- |

| Inadvertent Energy | The difference between a Control Area’s actual net interchange and a Control Area’s scheduled net interchange. |
| --- | --- |

| Interchange | a. Net Interchange The algebraic sum of the power flows of the ERCOT Control Area’s interconnections with other Control Areas. Sign convention is that net interchange out of an area is positive while net interchange into an area is negative. b. Scheduled Net Interchange The mutually prearranged intended net power flow on the ERCOT Control Area’s interconnections with other Control Areas. |
| --- | --- |

| Remedial Action Plan (RAP) | Predetermined operator actions to maintain ERCOT Transmission Grid reliability during a defined adverse operating condition. |
| --- | --- |

| Special Protection System (SPS) | A protective relay system specially designed to detect abnormal system conditions and take pre-planned corrective action (other than the isolation of faulted elements) to provide acceptable system performance. |
| --- | --- |

| Telemetry | Equipment for measuring a quantity (e.g., amps, volts, MW, MVAR, MVA) and transmitting the result to a remote location for indication or recording. |
| --- | --- |

| Time Error | An accumulated time difference between ERCOT system time and the time standard. Time error is caused by a deviation in ERCOT average frequency from 60.0 Hz. |
| --- | --- |

| Transmission Service Provider (TSP) | An Entity that owns or operates for compensation in this state, equipment or Facilities rated at 60kV or higher used to transmit electricity, and whose rates for Transmission Service are set by the PUCT. |
| --- | --- |

| Transmission Line Terminal Sign/Direction Terminology | (1) MW or VAR flow out of the bus and into the line is considered to be positive (+) flow. (2) MW or VAR flow into the bus and out of the line is considered to be negative (-) flow. |
| --- | --- |

## Entity Definitions and Roles

### ERCOT Control Area Authority

The ERCOT Control Area Authority is the regional security coordinator for the ERCOT System and is responsible for all regional security coordination as defined in the NERC Operating Manual and applicable ERCOT Operating Manuals or Guides.

The ERCOT Control Area Authority will deploy all Ancillary Services and dispatch transmission in accordance with the ERCOT Protocols, to ensure operational security and to remedy all Commercially Significant Constraints (CSC) and Operational Constraints.

The ERCOT Control Area Authority shall operate the ERCOT Transmission Grid in compliance with Good Utility Practice, NERC, and ERCOT standards, policies, guidelines and operating procedures, including, but not limited to:

- These ERCOT Operating Guides;

- NERC Operating Manuals as references during normal and emergency operations of the ERCOT System Transmission Grid; and,

- Individual transmission owners Operating Procedures submitted to ERCOT Control Area Authority to identify specific operating problems that could affect operation of the interconnected ERCOT System Transmission Grid.

### Qualified Scheduling Entities

Qualified Scheduling Entities (QSE) will provide the primary information interface for market participants with the ERCOT Control Area Authority.  Market participants wishing to transfer energy or services during the operating hour from one participant to another, or from generation to load, or from the participant to ERCOT, or from ERCOT to the participant will submit Balanced Schedules to the ERCOT Control Area Authority through a Qualified Scheduling Entity.

QSEs are not regulated by the Public Utility Commission of Texas.

### Transmission Operator (TO)

Transmission Operators (TOs) are the subset of Transmission Service Providers (TSPs) or Transmission and/or Distribution Service Providers (TDSPs) that is charged with continuous communication (7x24 basis) with the ERCOT Control Area Authority (CAA) and carrying out Dispatch Instructions directly or on behalf of represented TDSPs.  Each TSP or TDSP will designate either itself or another TSP or TDSP to perform the functions of a TO. TOs must meet all requirements identified in the ERCOT Protocols for TDSPs and TSPs in addition to those requirements stated below for all Transmission Facilities represented:

- Operate and manage the electric Transmission Facilities between energy sources and the point of delivery;

- Coordinate emergency communications between the TDSP or TSP and ERCOT CAA;

- Monitor the loading of the transmission system(s);

- Notify the ERCOT CAA of changes to the status of all Transmission Facilities;

- Act as Single Point of Contact for Transmission Outages;

- Maintain operational metering; and,

- Implement Black Start.

Transmission Operators shall provide the ERCOT CAA their written backup control plans to continue operation in the event the Transmission Operator’s control center becomes inoperable.

Each backup control plan shall be reviewed and updated annually and shall meet the following minimum requirements:

- Include description of actions to be taken by TO personnel to avoid placing a prolonged burden on ERCOT and other Market Participants.

- Include description of specific functions and responsibilities to be performed to continue operations from an alternate location.

- Includes procedures and responsibilities for maintaining basic voice communications capabilities with ERCOT.

- Includes procedures for backup control function testing and the training of personnel.

As an option, the backup plan may include arrangements made with another entity to provide the minimum backup control functions in the event the TO’s primary functions are interrupted.

### Resource Entity

Resource Entities include Power Generation Companies, Opt-in generation, NOIE generation, federal generation facilities and LSEs representing load being used as a resource.

Any Generation Resource that introduces energy into the ERCOT grid must be represented by a Resource Entity.  The sale of the generation produced is negotiated bilaterally with other market participants with the exception of energy produced as Ancillary Services.  The schedule for the power sale is communicated to the QSE that represents the Generation Entity in the ERCOT Market System.  The QSE will communicate an overall Balanced Schedule for the QSE, including the Generation Entity, to ERCOT.

PGCs that offer Regulation Service and Responsive Reserve Services must be directly connected to the QSE for the purpose of receiving a regulation signal in Real Time.

Reference:  Protocols Section 22, Attachment A, item 8(c)

(1)	ERCOT shall notify Participant, through its QSE, when the Black Start Resource must black start.

(2)	If the ERCOT Transmission Grid at the Black Start Resource becomes deenergized and if Participant cannot communicate with either ERCOT or the TDSP serving the Black Start Resource, then Participant shall follow the procedures specified for the Black Start Resource under ERCOT’s Black Start Plan in the Operating Guides, but Participant shall not commence delivering electric energy into the ERCOT System without specific instructions to do so from either ERCOT or the TDSP serving the Black Start Resource.

### Load Serving Entities (LSE)

LSEs include Non Opt-in Entities, the Texas General Land Office and Competitive Retailers.  These are the only organizations authorized to sell electricity directly to retail customers.  An LSE will forecast its contracted load and negotiate bilaterally with Resource Entities to obtain the forecasted energy.  The schedule resulting from this negotiation will be communicated to the QSE, who in turn will communicate an overall Balanced Schedule for the QSE to ERCOT.

## Operational Training

### Operator Training Objectives

A primary objective of each Operating Entity within the ERCOT System should be to train Operators such that they will be educated, motivated, and possess the necessary skills to be able to direct the operation of the power system in accordance with the ERCOT Protocols, ERCOT Operating Guides, ERCOT Control Area Operator Procedures as well as individual entity operating goals, plans and procedures.

These training objectives are:

- Maintain the safety of personnel, even during emergency situations involving complex switching and manipulation of control elements.

- Protect system components, particularly major power system elements from serious life degradation or harm.

- Operate the system in a secure manner to minimize violations of operating limits, avoiding customer outages where reasonably possible, and avoiding unstable situations that might result in widespread outages or blackouts.

- Operate the system as economically as possible within continually changing operating constraints.

- Restore the system to its normal operating state as rapidly as practical after a disturbance.

### ERCOT Operations Training Seminar

ERCOT will host a training seminar that, as a minimum, will be held annually.  The purpose of the training seminar is to provide a forum for system wide problems to be effectively addressed and to maintain an operator training consistency across all ERCOT System Operating Entities.

The seminar provides a forum for all ERCOT System Entity Operators and ERCOT System Operators, to meet and analyze common topics and issues as well as participate in formal training sessions.

### ERCOT Black Start Training

ERCOT shall periodically conduct system restoration seminars for all Transmission and Distribution Service Providers, Qualified Scheduling Entities, Generation Entities and other Market Participants, in accordance with Section 6.5.8 of the Protocols.

### Criteria For The Selection Of Operators

To be selected as Operators, candidates should be capable of directing other personnel in their own organization and, at the same time, work harmoniously with the ERCOT System Operators and other Entities’ Operators.  In addition, they must have a high intellectual ability, above average reasoning, mathematical ability, well-developed communication skills and reasonable mechanical aptitude.  To ensure compliance with these criteria, a screening and selection procedure must be considered for prospective Operators.  This procedure should include the following:

- Evaluation against a detailed job description.

- Analysis of the candidate's past work record to determine character, reputation, and previous experience.

- In-depth interview with each candidate.

- Evaluation of intelligence, logic, mathematical, and communication skills along with psychological fitness.

### Training Practices

Each Operating Entity should establish a clear requirement, define and develop a systematic approach in administering the training, and provide the necessary feedback as a measurement of curriculum suitability and trainee progress.  Each Operating Entity should recognize the importance of training and provide sufficient Operator participation through adequate staffing and work-hour scheduling.

## Document Revisions

Change History

| Issue/Date | Reason for issue |
| --- | --- |
| Version 1.0 February 28, 2001 | Updated from Working Group comments. Submitted to Technical Advisory Committee to approve handover to the Reliability Operating Subcommittee |
| Version 1.1 April 10, 2001 | Reference to Market Oversight Division corrected, added ROS responsibility for submitting PRR, minor formatting changes |
| May 1, 2002 | Formatting |
| July 1, 2002 | OGRR102 |
| October 1, 2002 | OGRR115 |
| December 1, 2002 | Corrected diagrams in Sections 1.2 & 1.4. |
| January 1, 2003 | OGRR120 revised Section 1.7.3 to add a backup plan requirement for TOs. |
| March 1, 2003 | Revised header to state “Section.” |
| May 1, 2003 | Revised diagrams on pages 3 & 5 to state “ERCOT Operating Guides” instead of System Operating Guide. Added language from OGRR128 to allow for administrative update of Protocols references included in the Operating Guides. |
| October 1, 2003 | OGRR132 |