# SPWG Report to ROS

#### August 10th, 2004

SPWG met July 29th and 30th, 2004, at the ERCOT offices in Taylor, Texas.

OGRR 150 – Clarification to UF Relay Trip Time for Load Shedding was discussed at the July SPWG meeting. SPWG members agreed on alternate wording that would facilitate calibration/testing of underfrequency relays and provide a uniform trip initiation time. Revised proposed operating guide language was submitted on behalf of SPWG.

Final future year (2005-2008) short circuit cases were posted by ERCOT.

SPWG member companies are reviewing zone 3 relay settings (i.e. longest reaching phase distance element that will cause tripping in either direction) on lines rated 345 kV and above to determine compliance with NERC recommendations. Review must be completed by September 30, 2004.

SPWG members have agreed that the present work environment better supports compilation of future year short circuit databases starting in April rather than late summer. SPWG Procedures will be modified to reflect the revised schedule, reviewed by SPWG members and should be ready for ROS consideration at the September ROS meeting.

Disturbance monitoring equipment location criteria presently applies to equipment operated at 60 kV or above. This is a change from wording previously approved by the SPWG specifying 100 kV and above. SPWG members agreed reference to 60 kV should be revised to 100 kV. An OGRR will be prepared.

The next SPWG meeting is scheduled for November 18th and 19th, 2004, in Houston.

Jack Dahnke

SPWG Chair