##### ERCOT Deregulation Project

##### Settlement Team – Load Profiling

Load Profiling Methodology

| Document Outline | This document describes a recommended approach towards developing load profiles for Settlement purposes, in ERCOT |
| --- | --- |
| Document Reference |  |
| Version number | 1.04 |
| Date of issue | May 22, 2000 |
| Filename | Load Profiling Methodology, v1.04.doc |
| Authors | ERCOT Settlement Profiling Sub-Group |

###### Table of Contents

Document History & Expected Changes	5

Review and Approval	6

Section 1.	Executive Summary	7

Section 2.	Introduction	9

2.1	Definitions	9

2.2	Objectives	10

Time to implement	10

Cost	10

Non-ERCOT applicability	10

Competitive issues	11

Market bias	11

Metering competition	11

2.3	Goals	11

2.4	Short-term versus Long-term	11

2.5	Application of Methodology	12

Section 3.	Imbalance Market Size	13

3.1	Introduction	13

3.2	Assumptions	13

Section 4.	UFE / Losses	14

4.1	Objective	14

4.2	Definition	14

4.3	Analysis	14

Errors in the Application of a Chosen methodology	14

Errors in Operation of Business Processes	14

5.1	Objectives	16

5.2	Use of Standards	16

5.3	Sample Design	16

5.3.1 Accuracy	16

5.3.2 Design (Auxiliary) Variable	16

5.3.3 Sampling Methodology	17

5.3.4 Determining the Number of Strata and the Strata Boundaries	17

5.3.5 Determining Sample Size	17

5.3.6 Allocation	18

5.3.7 Accounting for Data Loss	18

5.3.8 Selection of Alternate Sample Points	18

5.3.9 Validation	18

5.3.10 Customer Solicitation	19

5.4	Data Processing	19

5.4.1 Verification of collected data	19

5.4.2 Editing	19

5.4.3 Retention of data	19

5.5	Expansion	19

5.5.1 Mean per unit	20

5.5.2 Ratio estimation	20

5.5.3 Difference estimation	20

5.5.4 Regression	20

5.5.5 MBSS	20

5.6	Special Requirement for Interval Metering	20

5.7	Recommendation on Load Shape Determination	20

Section 6.	Profiling Methodologies	21

6.1	Objective	21

6.2	Statement of Load Profiling Methodology Issue	21

6.3	Analysis of Other Markets	21

6.4	Options for Load Profiling Methodologies	21

6.4.1 Option 1 - System Load Shape	22

6.4.2 Option 2 - Net System Load Shape (NSLS)	22

6.4.3 Option 3 - Static Load Profiles	23

6.4.4 Option 4 - Proxy Day Load profiles	23

6.4.5 Option 5 - Adjusted Static Load Profiles	24

6.4.6 Option 6 - True Dynamic Load Profiling	24

6.4.7 Option 7 - Lagged Dynamic Load Profiles	25

6.4.8 Non-metered Load Profiles	25

6.5	Recommendations on Load Profiling Methodologies	27

Section 7.	Maintenance of Samples and Load profiles	28

7.1	Objective	28

7.2	Sample Design	28

7.3	Load Profiles	28

7.4	Recommendation on Maintenance of Samples and Profiles	29

Section 8.	Assessment of ERCOT Load Research Data	30

8.1	Objective	30

8.2	Assessment of Available Data	30

8.3	Evaluation of Interval Metering Costs	31

Section 9.	Number of load profiles	32

9.1	Introduction	32

9.2	Principles	32

9.3	Guidelines	33

9.4	Segmentation Criteria	34

9.5	Cost and Benefits	36

9.5.1 Costs	36

9.5.2 Benefits	36

9.6	Conclusions and Recommendations on Number of Profiles	37

9.6.1 Short Term	37

9.6.2 Long Term	37

Section 10. Overall conclusions and recommendations	39

10.1	Short Term	39

10.2	Longer Term	39

Appendix 1. ERCOT Load Research Data	41

Appendix 2.Non-residential customer segmentation data	45

Appendix 3 Interval Metering Costs	47

Appendix 4 Glossary	48

## Document History & Expected Changes

| Issue # | Date | Reason for issue | Author |
| --- | --- | --- | --- |
| 0.1 | 10/18/99 | Initial draft for review | Profiling Sub-Group |
| 0.2 | 10/19/99 | Ivan Adams’ additions |  |
| 0.3 | 10/22/99 | Darryl Nelson and Robert Amos comments |  |
| 0.4 | 10/29/99 | Includes review comments, key conclusions from data collected, (Section 8), entries in appendices, and sentences from AEIC Manual, (Section 5) |  |
| 0.5 | 01/06/00 | Draft changes to Section 9, following the meetings on 12/01/99 and 12/29/00 and the subsequent comments provided by team members. | Ivan Adams/Darryl Nelson |
| 0.6 | 01/13/00 | Draft changes throughout document following meeting on 01/10/00 | Darryl Nelson/Robert Amos/Ivan Adams |
| 0.7 | 01/17/00 | Incorporating review comments of version 0.6 – teleconference 01/14/00 | Darryl Nelson/Robert Amos/Ivan Adams |
| 0.8 | 02/11/00 | Incorporating additions and changes as discussed at sub-team meeting 02/10/00 | Darryl Nelson/Ivan Adams |
| 0.9 | 02/24/00 | Incorporating minor additional changes following Profiling Group Meeting | Darryl Nelson/Ivan Adams |
| 1.0 | 02/25/00 | Incorporating changes made at Methodology meeting, 2/25/00 | Julia Garcia/Darryl Nelson |
| 1.01 | 03/03/00 | Removed redlining for circulation to Profiling Group; Added data to Appendix 1. Page 45 from CSW | Ken Riordon |
| 1.02 | 03/22/00 | Added comments from Methodology members | Darryl Nelson |
| 1.03 | 03/27/00 | Incorporating changes made at Methodology meeting, 3/24/00 | Darryl Nelson |
| 1.04 | 5/22/2000 | Insert document ownership language | Darryl Nelson |

## Review and Approval

| Name | Role |
| --- | --- |
| Ivan Adams, TXU | Reviewer |
| Robert Amos, TXU | Reviewer |
| Kari Clark, SPS | Reviewer |
| Julia Garcia, TNMP | Reviewer |
| Ted Hailu, Austin Energy | Reviewer |
| Bruce Limke, CSW | Reviewer |
| Ken Monts, Austin Energy | Reviewer |
| Larry Parent, Entergy | Reviewer |
| Ken Riordon, LCRA | Reviewer |
| Brenda Snyder, Entergy | Reviewer |
| Darryl Nelson, TXU | Reviewer |
| Jim Purdue, Reliant Energy | Reviewer |
| Lloyd Young, CSW | Reviewer |
| Kirk Schneider, CSW | Reviewer |
| Mel Schroeder, LCRA | Reviewer |
| James Cohea, TXU | Reviewer |
| Eddie Johnson, Brazos Electric | Reviewer |
| Madjid Zehani, Austin Energy | Reviewer |
| Walt Shumate, Shumate & Assoc. | Reviewer |
| Adrian Marquez, CPS | Reviewer |
| Les Galloway, STEC | Reviewer |
| Ed Lennox, Arthur Andersen | Reviewer |
| Suzi McClennan, OPC | Reviewer |
| Eric Voss, Reliant Energy | Reviewer |
| ERCOT Board | Approval |

# Section 1.	Executive Summary

Purpose and Objectives

The purpose of this paper is to examine options for choice of a suitable methodology for load profiling to be used for settlement in the ERCOT market. Two of the main considerations are the time required for implementation and the costs involved

.

The implementation time is a critical aspect since some form of load profiling is an absolute necessity for retail competition, apart from the use of interval meters. The costs of implementing load profiles for settlement need to be considered in comparison to the benefits of the profile performance accuracy.

Other considerations include:

- applicability both within and outside the ERCOT market area, competitive issues (i.e. ensuring that no REP acquires a particular advantage or disadvantage from the profiling methodology);

- issues of market bias (i.e. to what extent the profiling methodology causes a cross-subsidy between customer groups); and

- any potential impact on metering services competition when this occurs in 2004.

The short-term goal is to have a profiling system in place in time for the customer choice pilot, meaning that the system needs to be ready by around March 1, 2001 to allow for testing.

The following issues influenced the development of the Profiling Methodology document:

- Imbalance market size

- UFE/Losses

- Load shape determination

- Profiling methodologies

- Maintenance of samples  and profiles

- Assessment of ERCOT-wide load research data

- Number of profiles

Conclusions and Recommendations

- In sample design, accuracy shall be defined as industry best practice 90/10, as a minimum.

- All aspects of sampling methodology shall follow the Association of Edison Illuminating Companies (AEIC) load research guidelines.

- Adjusted Static Load Profiling is the recommended methodology for beginning the competitive market within ERCOT.

- In order to determine how many profiles to use at market open:

- existing load research data shall be analyzed,

- a suitable economic model will be developed to provide an indication of the significance of supply cost differences and similarities,

- required analysis shall be performed by the Load Profiling Agent (LPA), and co-participants as necessary,  utilizing the principles and guidelines specified in this document,

- the interactive process between co-participants, including ERCOT stakeholders, shall be defined by ERCOT.

- The primary objectives of the preferred load profiling methodology shall be to ensure fairness to market participants and to be cost effective in terms of accuracy.

- ERCOT Stakeholders shall conduct periodic reviews of load profiles.

Section 2.	Introduction

## Definitions

The following paragraphs present definitions for the purposes of this paper.

A “Load Profile” is an estimate of the average consumption pattern of a group of customers, showing the demand variation on an hourly or sub-hourly basis.

The diagram below shows an example of a typical daily consumption pattern for a residential customer.

| 01:00 | 02:00 | 03:00 | 04:00 | 05:00 | 06:00 | 07:00 | 08:00 | 09:00 | 10:00 | 11:00 | 12:00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.627 | 2.362 | 2.175 | 2.040 | 1.970 | 1.917 | 1.825 | 2.004 | 2.235 | 2.522 | 2.998 | 3.369 |

| 13:00 | 14:00 | 15:00 | 16:00 | 17:00 | 18:00 | 19:00 | 20:00 | 21:00 | 22:00 | 23:00 | 24:00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.674 | 4.013 | 4.140 | 4.343 | 4.460 | 4.455 | 4.456 | 4.286 | 4.019 | 3.707 | 3.433 | 3.008 |

“Load Profiling” is defined as the set of processes, carried out as part of ERCOT settlements, that allows the consumption data in respect of non-interval metered customers, to be separated into hourly or sub-hourly consumption load profiles, by reference to generic hourly or sub-hourly load profiles, representative of those particular customers.

“Load Profiling Methodology” refers to the methodology utilized for:

- the production of the reference hourly or sub-hourly consumption load profiles;

- the statistical analysis of the reference load profiles; and

- maintenance of the reference load profiles.

## Objectives

The purpose of this paper is to examine options for choice of a profiling methodology. Among the criteria to be assessed are:

- time to implement;

- cost involved (start-up and operational);

- applicability within and outside ERCOT;

- competitive issues;

- potential for market bias;

- impact of metering competition;

- privacy/proprietary issues.

### Time to implement

This is the mosta critical aspect since implementation of profiling is essential to enable retail competition at the mass-market level. Ideally, the chosen methodology would be suitable for implementation at market start-up and remain robust against any planned changes (e.g. metering competition). However, it may be necessary to consider an interim methodology for market start-up, with a refined methodology introduced at a later date. This is discussed further in Section 10.

### Cost

While a critical issue is the time to implement, cost is a consideration since it is desirable that the chosen methodology is economically efficient. Since it is not likely that profiling will achieve any market benefits, other than enabling competition, it is important that the overall cost should not impose an undue financial burden on the market. The difficulty is in assessing the level at which the financial burden becomes unacceptable.

However, it may be reasonably safe to assume that the costs of profiling should represent only a small fraction of the overall value of the market involved. We can therefore use the overall value of the market as a guide to the limitation on profiling costs.

A further cost consideration is the incremental cost benefit trade-off obtained from one methodology over another. The additional cost of, say, dynamic profiling versus static profiling, can be compared with the improved accuracy likely to be obtained. There is, however, no deterministic approach to such an evaluation, which is essentially a value judgement process.

### Non-ERCOT applicability

While it would be preferable to have a profiling system that is uniform across ERCOT and non-ERCOT system control areas, this is not considered to be a critical issue. It is also highly dependent on decisions concerning the application of other market principles in non-ERCOT regions.

### Competitive issues

The main concern is that the chosen profiling methodology and choice of number of load profiles should not impede retail competition or result in any advantage or disadvantage to particular Retail Electric Providers (REPs). While profiling may introduce certain competitive issues, it is important that their introduction should not impact unduly on a particular REP. Thus, for example, although the choice of profiling methodology could affect the pricing approach taken by REPs, it should not introduce an unfair advantage for any REP.

### Market bias

The issue of market bias is that profiling may cause a significant cross-subsidy between customer groups. However, unless each individual customer is individually assessed and priced according to their actual pattern of consumption (which might be possible with interval metering but unlikely to be adopted for pricing purposes, except for very large customers) there is always some element of market bias. Assigning customers to any particular load profile will, of necessity, cause some market bias. The methodological approach chosen for profiling should try to limit the amount of bias in the market.

### Metering competition

Since it is intended that competition in metering will be introduced in 2004, it is desirable that the choice of profiling methodology would not impede such competition. It is also desirable to minimize the effect competitive metering will have on the profiling methodology chosen.

Privacy/Proprietary Issues

It is desirable that the choice of profiling methodology not reveal private or proprietary information associated with customers.

## Goals

Some of the goals to be sought in the choice of profiling methodology are:

- a clear identification of the relative costs and benefits of each methodology and the associated implementation difficulties. Some factors are likely to be dependent on other profiling issues such as any mandated level (kW) for interval metering;

- an assessment matrix of each methodology, against each of the above criteria, with a recommendation for a choice of methodology;

- a high-level description of the functionality of the business requirements for the recommended profiling methodology. This would include data acquisition, required statistical analysis, load profile production and implementation within the currently understood settlement processes;

- a plan for design, production and implementation, in conjunction with the overall plan for implementation of the settlements and registration systems.

## Short-term versus Long-term

Even if the recommended methodology is considered to be capable of introduction by market start-up, it is prudent to consider contingency arrangements, based on an alternative, simpler arrangement.

The short-term goal is to have a profiling methodology in place for the opening of the market on January 1, 2002. This means that the methodology will need to be ready for the Customer Choice pilot program on June 1, 2001. Testing for the pilot program may begin on or around March 1, 2001, which then becomes the drop-dead date for the production of load profiles in the short-term.

The preferred or optimal method of profiling methodology in the long term is the one that most closely meets the goals stated above. This method may or may not be the same as the method chosen for market opening. The long-term view is detailed in Section 10.

## Application of Methodology

This Load Profiling Methodology has been developed specifically for ERCOT. Non-ERCOT utilities may, however, choose to utilize the same methodology.

In addition to the use of load profiles for settlement purposes, it is recognized that ERCOT-ISO and market participants may wish to utilize load profiles for scheduling purposes. It will therefore be necessary for the LPA to create load profiles both for previous days and future days.

# Section 3.	Imbalance Market Size

This section examines the size of the market that is affected by the use of load profiles. The purpose is to relate the costs and benefits of any improvement in load profile accuracy obtained from a particular methodology to the overall market size.

## Introduction

There is no inherent value in profiling per se – it is merely a mechanism for achieving settlement of the market. Therefore, any costs associated with profiling must be accommodated within the overall costs of administering the market. The ERCOT market is concerned with the settlement of imbalances between the scheduled loads and actual loads incurred by REPs. While it is likely that REPs would use the load profiles as a basis for scheduling and possibly for concluding bilateral contracts, these are outside the scope of the ERCOT market. It is therefore the size of the imbalance market that is the significant factor.

## Assumptions

As a basis for estimating the size of the imbalance market (including the ancillary services) we can use figures from Cal-ISO for 1998. These show that the total value of the ISO market in California for that year was $1.4 billion, compared with a total market value in California of $28 billion, i.e. 5% of the total market. Using this proportion and taking $20 billion as an estimate of the total annual value of the Texas market, an estimate of the imbalance market value is therefore $1 billion.

The relevance of this figure is that the market costs should be small by comparison. There is no straightforward method of determining how much value to place on load profile accuracy. There are three components of accuracy value:

- the value of certainty – the extent to which REPs can predetermine what hourly or sub-hourly load shapes will be used as a basis for their settlement charges; this is presumed to add value to them in managing the volume risk of trading. However, it is not necessarily accurate load shapes that are most valuable in managing risk but shapes that can be predetermined. Arguably, it is static load profiles that are most determinable;

- the time value of settlement - the extent to which there is a difference between initial and final settlement and the associated time value of money;

- the value of reduced cross-subsidy between load profile groups. More accurate profiling methods will reflect costs between groups more closely and therefore reduce the cross-subsidy between them. There are consequent societal costs involved but it is necessary to bear in mind that there are already extensive cross-subsidies inherent in current rate structures. This issue is closely related to the number of load profiles to be adopted, as discussed in a later section.

# Section 4.	UFE / Losses

## Objective

This section is included to explain what Unaccounted for Energy (UFE) is, and to put in context how the choice of profiling methodology affects the accuracy of the calculation of UFE.

## Definition

Unaccounted for Energy, or UFE, is the difference between the total generation supplied to a specific physical region and the total load (corrected for losses) in that same physical region during each settlement interval. UFE may be positive or negative in any single settlement interval.

## Analysis

Given the above definition, UFE therefore represents the proportion of ‘missing’ energy that is composed of:

- errors in the application of a chosen methodology;

- errors in operation of business processes, underpinning the chosen methodology, above.

### 4.3.1	Errors in the Application of a Chosen methodology

In this category are:

- inaccuracy of method used to calculate distribution losses;

- inaccuracy of method used to calculate transmission losses;

- inaccuracy of load profiles on a settlement interval basis.

The first two items in the above list are outside of the scope of this document. The remaining item represents a significant proportion of methodology errors, and is discussed in this paper. The options for the design and use of the profiling methodology are presented in Sections 5 & 6, and the options for segmentation of the population into load profile classes are presented in Section 9.

### 4.3.2	Errors in Operation of Business Processes

To complete the picture, however, it is important to appreciate that no matter how satisfactory a methodology, the continued incidence of theft, plus the likely incidence of a number of errors in the operation of key Settlement processes, especially at market start-up, is likely to diminish its effectiveness:

- theft of service, and unrecorded service (e.g. service provided before meter installation);

- error in the application of load profiles for non-interval metered data, (for example, wrong load profile class recorded for a customer);

- error in estimation of missing interval metered data;

- incorrect meter readings;

- error in operation of Settlement processes, particularly incorrect aggregation and reporting of metered data;

- errors in reporting of non-metered accounts;

- incorrect assignment of customers to UFE zones.

Whereas market rules can be created such that obligations are placed on market participants to operate processes to a threshold level of performance, experience of other markets would suggest that process errors, and the incidence of theft, can contribute to UFE. The need for a cost-effective solution is discussed in Section 6 – Profiling Methodologies, and is set in the context of the overall dollar value of the size of the Imbalance Market in the previous section

(Section 3).

# Section 5.	Load Shape Determination

## Objectives

The objective of this section is to explain that as standards for industry best practice already exist, then those procedures need to be adopted, where appropriate.

This section will also recommend a standardized methodology needed to produce the sample used for the profiling methodology that accurately reflects the hourly or sub-hourly consumption of the population.

## Use of Standards

The remainder of this section is based on guidelines recommended by the AEIC Load Research Manual.

## Sample Design

There are several steps that are required in the sample design of a load profile. These steps are outlined and briefly described below.  Please refer to the AEIC Load Research Manual Chapter 4 for more details.

### 5.3.3	Accuracy

- The accuracy of the load profile is a function of sample size and population variance.  A design accuracy of 10% at the 90% confidence level at the system and class peak time was specified in 1978 by PURPA for all major rate classes. This is the recommended minimum requirement specified for any load profile and applies to the data used to develop the load profile (sample level or strata level).

### 5.3.2	Design (Auxiliary) Variable

The design variable must be known for each unit of the entire population. Possible choices for the auxiliary variable in load profile applications are:

- Annual energy usage

- Monthly energy usage

- Winter peak month energy usage

- Summer peak month energy usage

- Annual peak demand

- Monthly peak demand

- Summer peak demand

- Winter peak demand

- Type of appliances (electric heating or electric cooling)

- Type of residence or business

- Load Factor

### 5.3.3	Sampling Methodology

Several types of sampling procedures are commonly used to select individual sampling locations representative of a target population. The method chosen depends on the load characteristics to be estimated, the information available about the population, the uniformity of the population, and other sampling and budget constraints.

- Simple random sampling

- Selects individual units throughout the population at random; each customer has an equal chance of being included in the sample.

- Stratified random sampling

- Divides the population into mutually exclusive, non-overlapping groups called strata. This type of sampling can involve stratifying in one or more dimensions.

- Systematic sampling

- Involves selecting every kth point from a sample population. This selection can begin at a random start or a centered start.

- Cluster sampling

Groups containing smaller units, such as neighborhoods, apartment complexes, joint poles, and transformers are randomly sampled, and the selected sampling units within the clusters are individually metered.

- Two-phase sampling

A large sample is first taken to obtain information on a design, or auxiliary, variable. The results of the first sample are then used to design a smaller load study stratified by that auxiliary variable.

- Model-based statistical sampling (MBSS)

Based on two equations that describe how the mean and standard deviation of the variable of interest vary from customer to customer within the population.

- Census (100%) sampling

A stratum that contains a few customers that account for a large amount of load is a viable candidate for census sampling.

- Judgmental sampling

Units considered to be representative of the population are selected or certain types of customers are included in the sample based on the analyst’s judgement.

### 5.3.4	Determining the Number of Strata and the Strata Boundaries

- The number of strata can often be determined for stratified random designs by calculating the coefficient of variation of the sampling distribution for test designs with different numbers of strata. The strata boundaries can be calculated by using the Dalenius-Hodges procedure or MBSS “strong stratification” as well as the analyst’s judgement.

5.3.5	Determining Sample Size

- The sampling method, estimation technique and required accuracy all influence sample size requirements. Sample size equations require estimates of the mean and variance of the variable of interest or an auxiliary variable. The preferred method of obtaining estimates for sample design is to derive them from prior load research data. When prior load research data are not available, a common procedure is to use an auxiliary variable (such as consumption) to estimate the mean and variance of the variable of interest.

- Minimum requirements for sample size should be determined by the sampling method, estimation technique, and the chosen accuracy level plus a percentage margin to compensate for missing data. Sample sizes should be determined in such a way as to ensure accuracy for each month, not just the peak periods.

5.3.6	Allocation

- There are two types of allocation techniques commonly used in load profiling.  Proportional allocation assigns sampling points to each stratum based upon the number of population units represented in the stratum. Neyman allocation assigns sampling points to each stratum based upon the percentage of the total population standard deviation represented by the stratum. A finite population correction (fpc) factor should be incorporated if the percentage of the sample to the population is substantial (greater than 5% is widely used).

### 5.3.7	Accounting for Data Loss

- Since data will not be available for every customer during every time period in the study due to events such as equipment malfunction, human errors, customers moving, and customers requesting removal of recording equipment, techniques should be considered to account for data loss in the sample. There are two methods of accomplishing this. One technique is to apply a data loss factor which increases the overall sample size by a fixed percentage. Another technique is to set a minimum number of sample points per stratum and increase the sample size accordingly after allocation. The minimum number is a function of the data loss factor and research judgement.

### 5.3.8	Selection of Alternate Sample Points

- Once all efforts to install a particular metering site have failed, selecting sample replacements becomes necessary. Replacements are chosen in one of three ways: (1) replacements randomly chosen at the time of sample selection, (2) replacements systematically chosen at time of sample selection, or (3) customers with similar information (usage, demographic, geographical location) chosen as backups. Any replacement method chosen should minimize sample bias. (See also Section 5.6).

5.3.9	Validation

- After a prospective sample is chosen, it should be compared to the population to determine how well it represents the population, thus ensuring that the sample selection procedure has been performed correctly and that the appropriate sampling frame has been used. There are several methods of performing this validation. The simplest is to compare the sample mean usage to the population mean usage (annual or monthly). A more complex procedure involves setting specific tolerance ranges, which the sample must meet, for specific variables. Samples are then selected until a suitable sample is found.   A third method is to select numerous samples and choose the sample that best matches the population for specific variables.

### 5.3.10	Customer Solicitation

Many utilities seek the customer’s agreement to participate even though, legally, such an agreement may not be necessary. Therefore, there are three alternatives to customer solicitation: customer approval required, customers notified but approval not required, or customers not notified. Since customer rejection could introduce sampling bias, care should be taken when choosing the method of customer solicitation. Bias is the difference between the mean value of the estimate and the true value being estimated. Sampling bias occurs when some members of the population about which inferences are to be made are accidentally or purposely excluded from the population frame.

It is realized that all of the above requirements might not be attainable at market open, but should be accomplished in the mature market.

## Data Processing

There are several steps that are required in the data processing of load profile data. These steps are outlined and briefly described below. Please refer to the AEIC Load Research Manual Chapter 6 for more details.

### 5.4.1	Verification of collected data

A validation of the collected sample data should be performed and should include a comparison of the interval kWh and the metered kWh, translation period start and end dates, and clock times of the interval recorder.

### 5.4.2	Editing

Editing of data should be used sparingly.  Missing and incorrect data should not be estimated and therefore, should not be used.  A complete trail must be maintained for all editing.

### 5.4.3	Retention of data

Data retention should follow proper standard research needs, e.g. ten years. This issue is subject to any ERCOT or PUC data retention policy.

  - Expansion

There are several estimation techniques that can expand load profiling sample data to total system parameters. These methods are briefly described below. Please refer to the AEIC Load Research Manual Chapter 7 for more details.

5.5.1	Mean per unit

The mean-per-unit technique calculates an estimated demand per customer and expands this estimate to the target population by multiplying by the number of customers in the population.

### 5.5.2	Ratio estimation

Ratio estimation is a technique that can take advantage of the correlation of the x and y variables to obtain increased precision.  The ratio estimation technique can be applied if the relationship of x and y is approximately linear and passes through the origin.  Class demand estimates for rate classes and other populations with “known” total energy use (X) are adjusted by the ratio of demand (y) to energy use (x) for the sample.

### 5.5.3	Difference estimation

As in the ratio technique, the difference estimation technique recognizes that energy use per customer from either stratified or non-stratified random samples often differs from the true mean use per customer for the target population.  The difference technique should be used rather than the ratio technique when the linear relationship of x and y does not pass through the origin.

5.5.4	Regression

Regression models are most commonly used to describe a relationship and to predict future values.  They often provide more reliable estimates, particularly for small samples.  Regression methods are adaptable to analyses and extrapolation of a sub-population drawn from a primary sample.

### 5.5.5	MBSS

Model-based statistical sampling and its expansion techniques are refinements of design-based sampling and regression modeling.  The most unique aspect of MBSS is its use of models to describe the variances of the variables being estimated.  The MBSS variance models are applied directly to the total population, e.g. to the frequency distribution of annual kWh.

- Special Requirement for Interval Metering

When a customer has an interval data recorder (IDR) installed as part of a load research sample used for profiling, and that customer or that customer’s REP elects to use interval data for settlement purposes, it will be necessary to replace that customer in the sample.

## Recommendation on Load Shape Determination

In sample design, accuracy shall be defined as industry best practice 90/10, as a minimum.

All aspects of sampling methodology, through to customer solicitation within Sample Design shall follow the AEIC guidelines, as shall the approach adopted towards Data Processing and Expansion.

# Section 6.	Profiling Methodologies

## Objective

The objective of this section is to identify the options available for load profiling methodologies and to recommend an approach. The recommendation shall include a view for the short-term, i.e. market opening, and a view for the long term.

## Statement of Load Profiling Methodology Issue

In general terms, load profiling is the estimation of hourly or sub-hourly loads at the delivery point for customers who do not have IDRs. The methodologies proposed for load profiling vary greatly. Some use a system load shape (generation) as an estimate for all customers, while others employ a statistical sampling approach that produces estimates of customer segments, usually rate classes.

There is even a wide variety within the sampling methods. Some use only historical sample load data, others take a modeling approach, while others use sample load data but in a timely manner for settlement purposes. Two of the primary concerns that should be addressed when determining the proper load profiling methodology are the accuracy of the estimates and the costs of improving that accuracy.

## Analysis of Other Markets

Rochester Gas & Electric (NY) uses the Net System Load Shape method for load profiling because no load research sample data exists, neither current or historical, for residential or small commercial customers. California used static load profiles for the first three months of open-access market operation then switched to dynamic load profiling. Eastern Utilities Associates (New England) employs a proxy day methodology that utilizes an extensive historical load research database. ConEd (NY) uses a modeling approach that includes temperature and day type.

## Options for Load Profiling Methodologies

A few of the methodologies used for load profiling are summarized below in order of simplest to most complex, generally speaking. The criteria that should be used to select the proper load profiling methodology include:

- current state of load research samples;

- method of load data collection;

- amount of historical load research sample data;

- resources available to facilitate the methodology.

As with most cost-benefit analyses pertaining to accuracy versus cost, there is usually a direct relationship: to achieve more accurate results requires more money.

### 6.4.1	Option 1 - System Load Shape

The System Load Shape method uses the actual system load shape (e.g., ERCOT control area, Distribution Utility (DU), substation, and feeder) for all customers. This method could be used if no customers have IDRs, and no load research sample data is available.

| Advantages | Disadvantages |
| --- | --- |
| Simple to develop | Many customers will not “fit” the load shape; too generic |
| Easy to understand | Low class-level accuracy |
| Low or no cost |  |
| No sample design or modeling error |  |

### 6.4.2	Option 2 - Net System Load Shape (NSLS)

The NSLS method uses the same load shape for all non-interval-metered customers. The NSLS load shape is determined by calculating the hourly or sub-hourly differences between the actual system load shape (e.g., ERCOT control area, Distribution Utility (DU), substation, and feeder) and the loss-adjusted aggregate load for all interval-metered customers.

| Advantages | Disadvantages |
| --- | --- |
| Relatively simple to develop | Many customers will not “fit” the load shape, but improves as the number of IDRs increases; too generic |
| Easy to understand | Low class-level accuracy |
| Low cost | Timing of the collection of interval-metered customer data may affect settlement |
| No sample design or modeling error |  |

### 6.4.3	Option 3 - Static Load Profiles

Static load profiles are created by averaging historical data from load research samples by class, and generating load shapes that approximate customer segment usage for the given day. Static load shapes are fixed in advance. They can be differentiated by season, month, and day.  Examples are: Typical days of the week by month or season, i.e., typical April Monday, typical spring Tuesday; typical weekday/weekend by month or season, e.g. typical April weekday, typical spring weekend.

| Advantages | Disadvantages |
| --- | --- |
| Relatively easy to produce and administer | May not capture other important load profile traits that influence usage, e.g. weather |
| Calculation is straightforward for all market participants to understand | Could be considered too simplistic because it is “over-averaged” |
| High class-level accuracy | Requires historical load research data (2 years minimum) |
| Most large utilities within ERCOT currently have experience with this methodology | Costs of implementing load research samples |
| Potential for easy regulatory approval |  |
| Load profile data used for initial settlement would be available to market participants prior to settlement day |  |

### 6.4.4	Option 4 - Proxy Day Load profiles

Proxy day profiling is the simplest form of modeling. Sample load research data is used from a day that is similar to the target day in terms of metered system total or class hourly or sub-hourly load profile shape, and/or day type and weather.

| Advantages | Disadvantages |
| --- | --- |
| Moderately easy to produce and administer | Introduces modeling error |
| Captures influence of weather and other load profile traits on usage | Methodology may not be easy for all market participants to understand |
| High class-level accuracy | Requires historical load research data (12 months minimum) |
| Potential consistency in estimation procedures for interval-metered customers, if necessary | Costs of implementing load research samples and proxy day selection process |

### 6.4.5	Option 5 - Adjusted Static Load Profiles

(also Modeled, Modeled Dynamic, Dynamic Modeling, Statistical Modeling)

Adjusted static load profiles are created by analyzing historical data taken from load research samples representing customer class or segment, and generating load shapes that approximate the customer class or segment usage for the target day. The load profiles can be differentiated by season, month, or day. These load profiles are then adjusted to reflect weather changes and/or other specific characteristics of the target day. Examples of adjusted static approaches are regression analysis and neural networks.

| Advantages | Disadvantages |
| --- | --- |
| Moderately easy to produce and administer | Introduces modeling error |
| Captures influence of weather and other load profile traits on usage | Methodology may not be easy for all market participants to understand |
| High class-level accuracy* | Requires some historical load research data (12 months minimum) |
|  | Costs of implementing load research samples and modeling process |

### 6.4.6	Option 6 - True Dynamic Load Profiling

True dynamic load profiling is defined as the daily interrogation of load research sample IDRs for the purpose of generating load profiles to reflect actual usage for non-interval metered customers for the target day.

| Advantages | Disadvantages |
| --- | --- |
| Most class-level accurate and timely of all load profiling methods* | Probably most costly of all profiling methods due to need for telemetry on all sampled points |
| Reflects current conditions that affect usage such as weather, etc. | Must be able to collect and analyze load research data daily |
| No modeling needed which eliminates modeling error and improves load profile accuracy | Potential for increased sample size to compensate for missing data |
| No need for historical load research data prior to implementation | Cost of implementing load research samples |
| No modeling approach should be easier for all market participants to understand |  |
| Initial Imbalance Settlement process (e.g., day after) would have most accurate load profiles* |  |

### 6.4.7	Option 7 - Lagged Dynamic Load Profiles

Lagged dynamic load profiling is the same as true dynamic load profiling except instead of daily collection of load research data, a weekly or monthly interrogation of load research sample recorders is performed. This methodology would be used in final settlement only, e.g. 45 days after settlement day.  If an initial settlement were performed, then lagged dynamic would be used in addition to static, proxy day or adjusted static.

| Advantages | Disadvantages |
| --- | --- |
| Same level of accuracy as true dynamic | Probably most costly of all profiling methods except for true dynamic |
| Reflects current conditions that affect usage such as weather, etc. | Must be able to collect and analyze load research data weekly if required |
| No modeling needed which eliminates modeling error and improves load profile accuracy | Since limited to final settlement, another profiling methodology is required for initial settlement |
| No need for historical load research data prior to implementation |  |
| No modeling approach should be easier for all market participants to understand |  |

### 6.4.8	Non-metered Load Profiles

Certain types of customers have no electrical consumption metering equipment installed at their location, such as street lighting and traffic lights. Typically, these customers have known hours of operation and consistent electrical usage during those hours. Using engineering estimates of their hourly or sub-hourly usage can produce load profiles for these customers. The Load Profiling Agent shall produce suitable profiles to allow for appropriate variation in sunrise and sunset times.

Load Profiling Methodologies – Assessment Matrix

| Option | $ Cost | Time Requirement | Impact from Competitive Metering | Class-level Accuracy |
| --- | --- | --- | --- | --- |
| System Load Shape | Lowest | Lowest | None | Poor |
| Net System Load Shape | Low | Dependent on kW threshold level; Low if no level | May change interval-metered customers data flows/collection | Poor |
| Static Load Profiles | Implementation of load research samples & 2 years historical data | Minimum 2 years historical load research sample data needed | May change sample data flows/collection; may necessitate telemetry | High |
| Proxy Day Load profiles | Implementation of load research samples, 12 months historical data, & proxy day selection process | Minimum 12 months historical load research sample data & proxy day selection process | May change sample data flows/collection; may necessitate telemetry | High |
| Adjusted Static Load Profiles | Implementation of load research samples, some historical data, & modeling process | Some historical load research sample data & modeling process | May change sample data flows/collection; may necessitate telemetry | High |
| True Dynamic Load Profiling | Probably most costly | Installation of telemetry on all sampled customers | May change sample data flows/collection | High |
| Lagged Dynamic Load Profiling* | Implementation of load research samples | Dependent on initial settlement profiling methodology | May change sample data flows/collection; may necessitate telemetry | High |

*  Would be used in final settlement, e.g. 45 days after settlement day, only.  If an initial settlement were performed, then lagged dynamic would be used in addition to static, proxy day or adjusted static.

Note:  The assessment criteria ERCOT versus Non-ERCOT Issues, Competitive Impact, and Bias in Market were deemed to be non-issues and thus removed from this table in the interest of space.

## Recommendations on Load Profiling Methodologies

Given the time restrictions for market opening, a pragmatic approach should be used to determine the proper load profiling methodology. Currently, some sort of load research sample data exists for most of the large utilities within ERCOT. This load data could, with a detailed review, allow a reasonable estimate of hourly or sub-hourly consumption to be made for every customer within ERCOT. Since these load profiles are customer segmented (rate class), they are more accurate at the class level, i.e. for those customers in that class, than System Load Shape and Net System Load Shape.

Also, most of the load research programs do not have the capability currently to produce Dynamic Load Profiles. The major issues involved with Dynamic Load Profiling are the costs and time involved to install telemetry equipment on all sampled customers’ IDRs.

Static Load Profiles could be used at market opening, but there is no allowance for the effect of actual weather on the load profile estimates. Proxy Day and Adjusted Static Load Profiling are accurate at the class level, would require little start-up costs, and do allow for the effects of weather.  Proxy Day, however, takes a simplistic approach to modeling, whereas, Adjusted Static can account for multiple variables that have an effect on load profiles. Therefore, Adjusted Static Load Profiling is the recommended methodology for beginning the competitive market within ERCOT.

The long-range view on a preferred load profiling methodology should be to ensure fairness to market participants and to be cost effective in terms of accuracy.

# Section 7.	Maintenance of Samples and Load profiles

## Objective

This section covers the requirements for maintaining the samples used for profiling and the maintenance of the load profiles to ensure that they continue to represent their intended population, and that they are consistent with achieving the overall objectives of the chosen methodology.

## Sample Design

Samples need to be designed to achieve the required level of accuracy (e.g. 90/10 standard) in the expected results. In addition to the initial design considerations, however, there is a need to maintain the sample, so as to continue to attain the desired standard. Two main considerations are:

- Drift – i.e. a situation where the sample no longer reflects the population parameters to the required standards. A particular example of this phenomenon is when customers move to interval metering. This problem will affect adjusted static load profiles since a standing sample is used to represent a defined population. The decision on a kW threshold for interval metering would also have an impact on this requirement. In order to address load profile drift, it would be necessary to monitor the position on a routine basis and to take appropriate steps to change the sample if appropriate;

- Mortality – i.e. a situation where membership of the sample drops off for various reasons – (e.g. a change of use at the premises). This requires replacement of the sample membership as required, utilizing reserve lists from which to draw a suitable replacement. (See also Section 5.6).

## Load Profiles

In addition to maintenance of the sample, there is a need to maintain the validity of the load profile data with regard to current usage of the population. The two main considerations are:

- there is a need to monitor any historical data to ensure that it continues to reflect current usage. Customer usage patterns may change over time, associated with lifestyle changes, electrical appliance technology, economic or other extraneous variables. Also, the period over which the sample data has been collected may not be representative of future temperature conditions. These considerations require changes to be made to the sample data (i.e. the load profile) but not necessarily to the sample on a long-term routine basis;

- in relation to modeled load profiles, there is also a need to monitor and update where appropriate any modeling parameters used to calculate the daily load profiles.

Methodologies used to monitor the validity of load profile data might include:

- Mean Absolute Deviation (MAD);

- Mean Absolute Percentage Error (MAPE).

## Recommendation on Maintenance of Samples and Profiles

The recommendation is to conduct periodic reviews, (for example quarterly) using MAD or MAPE methodologies.  The specific actions depend on the incidence of Drift and Mortality. (See also Section 5.7).

###### Section 8.	Assessment of ERCOT Load Research Data

  - Objective

The objective of this section is to indicate what load profile data exists currently within the ERCOT region.

## Assessment of Available Data

A survey of ERCOT utilities was conducted to determine the following, which will be used to address two issues:

- how to meet the 2002 retail competition opening with existing load research data;

- how to provide an optimal longer-term solution for load profile development.

The data collected is also useful in addressing methodology issues related to sample design and extrapolation to population methodologies. It includes the following:

- number of rate classes with IDRs used for load research samples and billing;

- installation date of sample;

- number of IDRs (load research sample and billing) the utility is collecting data from;

- frequency of IDR data retrieval;

- geographic area covered by load research samples;

- stratification methodology of load research samples;

- statistical confidence of load research samples;

- population size represented by load research samples;

- date load research sample last extrapolated to population;

- extrapolation methodology;

- annual peak kW by class derived from load research sample;

- annual kWh for classes with load research samples;

- estimate of UFE.

This data is presented in Appendix 1, and shows that:

- For ERCOT utilities reporting data, 15 residential and 56 non-residential rate classes have load shapes developed (excluding non-metered services).

- Rate classes are not defined in a consistent manner across utilities.

- Estimated load profiles are prepared for non-metered services.

- Existing load research samples covers approximately 64% of the total number of ERCOT customers.

- Cellular technology for data retrieval is used by one utility; other utilities use a combination of probe/landlines.

- Expansion methodologies employed include mean-per-unit and ratio estimation.

- Samples are most generally designed to meet a 90% confidence, +/-10 relative accuracy.

A second survey of ERCOT utilities was conducted to acquire information to develop:

- a frequency distribution of kWh;

- number of customers and median load factors by kW ranges such as 0-20 kW, 21-50 kW, 51-100 kW, etc.

This data will be used in an evaluation of establishing kW requirements for interval data recorders; e.g. California has requirement for customers to have IDR if kW is >50.

This data is presented in Appendix 2, and shows that:

- Over 60% of non-residential customers have a demand less than 20 kW and many do not have a demand meter installed. However, these customers account for less than 5% of the non-residential energy.

- For the ERCOT utilities reporting kWh information, 50% of non-residential kWh is consumed by customers with a kW of 1500 or greater.

- Large customers with a kW of 2500 or greater account for approximately 40% of non-residential energy consumed.

Surveys still to be defined, include:

- Regionalization, to determine the amount of IDR data available by geographic area. The information will be a factor in making a recommendation of the number of load profiles required due to differences in weather zones;

- Gaps, to determine geographic areas in ERCOT that currently do not have load research samples in place. Recommendations will be made on how best to fill in the gaps (provide load profiles for the areas with no samples for profiling).

## Evaluation of Interval Metering Costs

An informal survey was conducted among the members of the Profiling Methodology work group to examine IDR costs. The survey collected information on the cost of IDR equipment, installation, maintenance, communications equipment, monthly communications charges, and data retrieval and translation. The results are shown in Appendix 3.

As an example, the average cost of a telephone landline IDR over a five-year life is:

Average cost for IDR and installation			$     776

Average cost for maintenance and testing			    1,200

Average cost for landline communications equipment	       123

Average cost for landline communications			    1,660

Average costs for data retrieval & translation		       960

Total 5 year cost						$  4,718

# Section 9.	Number of load profiles

  - Introduction

The objective of this section is to analyze the criteria that could be used for segmentation of the non-interval metered population, and to consider the overall number of load profiles that might be generated as a consequence.

These choices hinge on the costs and benefits associated with each, the extent to which a solution is timely and pragmatic in the period prior to June 1, 2001, but at the same time provides a platform from which a longer term solution can be devised with minimal change.

This section outlines a series of principles and guidelines that have been produced to provide the Load Profiling Agent with guidance when determining the numbers of load profiles to use. This section does not specify what the numbers of load profiles should be. Rather, it recognizes that further analyses will be required before this determination can be made, and the principles and guidelines below should be adhered to during the ongoing analysis work.

## Principles

The following principles are recommended as a basis for decision on the number of load profiles to be adopted:

- Load profiles used in the Market shall not give an unfair advantage to any party.

- Load profiles used in the Market shall minimize their contribution to UFE over all settlement time periods.

- Load profile classes shall reflect reasonably homogenous groups, with respect to load shape and likely supply costs.

- The total number of load profiles used in the Market shall be as few as possible.

- Each load profile shall be distinctly different from each other.

- Due to the lack of complete load profile data for some areas, it will be necessary to assign load profile classes to these areas based on load profiles developed for other areas, taking into account similarities and differences in load usage. When necessary to assign load profiles, this shall be done in a fair and equitable manner.

- Load profile classes shall be based on readily identifiable parameters.

- Load profiles shall be based on the most accurate load research data available.

- It shall be possible to accommodate Time of Use (TOU) rate classes, controlled load classes and other similar pricing schemes in the load profiles.

- The form in which the load profiles are expressed shall be easily understood and readily available.

- The profiling methodology shall be clearly expressed and available to all interested parties.

  - Guidelines

The following guidelines are to be used by the Load Profiling Agent (LPA) in developing load profiles to be used in the Market:

- In order to minimize the total number of load profiles to be used in the Market, the LPA shall review the existing load research data available for each geographic/climatological area and analyze opportunities for using one load profile to represent more than one existing class load shape.

- A relatively simple economic model shall be developed, in conjunction with ERCOT, to enable the LPA to analyze existing load data, together with representative generation price data, so as to provide ERCOT with information on the appropriate number of load profiles to adopt for the Market. In particular, this would allow the following questions to be addressed:

      - To what extent do the existing load profiles represent homogeneous groups with respect to load shape and supply costs?

      - To what extent do the existing load shapes for similar customer groups (e.g. residential) show distinct differences from each other, especially during periods of high generation cost volatility?

- The assignment of load profiles to areas that do not currently have load research data available shall be based on the following approach issues:

- What separate customer groups are currently recognized for the area requiring a load profile (e.g. rate classes)?

- What load shapes are available from other areas for each of these customer groups?

- Where possible, examine broad measures of similarity between the load research data that is available and the customer group requiring a load profile. These measures might include:

        - Average kWh consumption per year or month from billing records.

        - For customer groups with demand metering, the annual average load factor.

        - Other specific data that may be available for the customer group requiring a load profile (e.g. where the type of electrical use is considered to be similar to that of another area with a similar usage pattern).

- The geographic proximity of the areas for which load research data is available.

- In adopting load profiles for those areas where load research data already exists and in assigning load profiles to those areas that do not currently have load research data, there shall be readily identifiable parameters, for each customer, to enable load profiles to be assigned to each customer. Ideally, the customer parameters that determine which load profile that customer is assigned shall be based upon existing data. Some examples of readily identifiable parameters are: 1) Type of customer (residential, small commercial, large commercial, etc.); 2) Peak demand; and 3) Load factor. Other parameters, such as those relating to geographic location, shall be unambiguous and straightforward.

- Where alternative load research data may exist, the most accurate data shall be used. This accuracy shall be based on load research data on all customers from all distribution utilities in that region. Generally, the most recent data is preferred but other factors such as the sample size and customer coverage shall be considered.

- In order to accommodate Time of Use (TOU) rate classes, controlled load and other similar pricing schemes, the LPA shall consider the following possibilities:

- Where specific load research data exists for a particular rate group, utilize that data.

- When appropriate, generic load profiles can be modified to approximate the consumption patterns of multi-rate tariffs.

- Where specific load research data does not exist for a particular rate group, appropriate load profiles could be used from other areas, based on the relevant guideline above (paragraph 3).

- Load profiles shall be clearly expressed and readily available. A standard form to represent all load profiles is desirable for consistency and ease of understanding.

- The methodology used to create load profiles shall be fully defined. Any mathematical or statistical equations used shall be unambiguously defined.

## Segmentation Criteria

In the choice of segmentation parameters, for the identification of load profile classes, the following criteria are appropriate:

- the parameter shall identify significantly different load profile shapes and therefore energy costs;

- the parameter shall be clearly and easily measurable;

- the parameter shall be easily identifiable;

- the parameter shall be robust, i.e. not subject to frequent change.

Identified below are some possible parameters.  Certain of these parameters contain privacy and proprietary issues.  For example:

- existing rate class or combination of rate classes. This approach meets all of the above selection criteria, and has the additional advantage of reflecting the cost of supply;

- usage based on load factor, (used in the UK for non-residential less than 100kW). This approach meets all of the above selection criteria, with the possible exception of robustness, but the disadvantage is that it requires kW, or kVA meters, which may or may not already be installed;

- usage based on kW. This approach meets all of the above selection criteria, with the possible exception of robustness. It has the added advantage that it is used commonly now for cost allocation purposes, but requires kW metering or some means of assessing the kW demand of each customer;

- usage based on kWh. This approach meets all of the above selection criteria, with the possible exception of robustness;

- premise type, such as use of SIC Code. This approach fails on some of the above selection criteria, in particular, the code is not easily identifiable initially, nor is it easy to maintain its accuracy on an on-going basis;

- location, by service territory or weather zone. This approach meets all of the above selection criteria, with the possible exception of not reflecting significantly different shapes, other than caused by weather;

- use of demographic factors, reflecting a customer’s behavior pattern, relating indirectly to electricity usage due to correlation with time of use. This approach fails on some of the above selection criteria, particularly, it is not easily identifiable initially, nor is it easy to maintain its accuracy on an on-going basis. It suffers from an additional disadvantage in that there are likely to be privacy of information issues to address.

A summary of the relative impact, in terms of cost, time to implement, and the criteria considered above is summarized in the table below:

| Option for segmentation parameter | $ cost | Time requirement | Privacy/ Proprietary Issues | Availability and robustness |
| --- | --- | --- | --- | --- |
| Existing rate class, or combination | No additional cost involved | Low | Low | Parameter exists and is robust |
| Usage – load factor (n/a to residential and non-demand metered commercial customers) | Additional cost of identifying the load factor of each customer from billing records3 | Relatively low | High | Parameter needs to be created from billing records |
| Usage – kW (n/a to residential and non-demand metered commercial customers) | Additional cost of identifying the kW maximum demand level of each customer from billing records | Relatively low | High | Parameter needs to be created from billing records |
| Usage – kWh | Additional cost of identifying the kWh consumption over a specified time period of each customer from billing records | Relatively low | High | Parameter needs to be created from billing records |
| Premise type | No additional cost involved, assuming that SIC exists on all billing records | Relatively low (assuming parameter exists on all billing records) | Medium | Not likely to entirely robust |
| Location | No additional cost involved, assuming that zip code, or other geographic parameter, exists on all billing records. | Relatively low (assuming parameter exists on all billing records) | Low | Parameter needs to be created |
| Demographic factor | Additional cost involved of identifying the demographic parameter for each customer | Relatively high | High | Not likely to be robust |

Note: Other assessment criteria (ERCOT versus Non-ERCOT, competitive impact, competitive metering and bias in the market) were deemed not to be affected by the segmentation options.

  - Cost and Benefits

### 9.5.1	Costs

Costs include start-up costs and on-going costs in consideration of the cost of ownership of any one solution. From this perspective, the following three aspects of the solution will have a direct impact on costs (generally in proportion to the number of load profiles chosen):

- maintenance of load profiles in terms of data processing and storage facilities, since more load profiles will increase the number of sample points needed, for accuracy reasons;

- set-up costs for meter installation and location and definition of sample sites, since more load profiles will increase the number of sample points, as above;

- administration costs in terms of people and execution of business processes (note though that it is only the on-going costs that will vary with number of load profiles, and this will likely to be a lesser effect than the above two aspects).

9.5.2	Benefits

The following types of benefit, or objectives, are desirable when choosing the number of load profiles to use:

- improved accuracy and a reduction in cross-subsidy between and within load profile classes;

- minimize the risk for market start-up. One way of achieving this is to make minimal change to current practice;

- simplicity of understanding for the less experienced players in the market, and for customers;

- from a REP’s perspective, customer market niches (probably a premise type), should fall within one load profile class, otherwise this makes market analysis and price setting more difficult (note that it is not necessary that each market niche has its own load profile class);

- provide equity across ERCOT; achieved by a rationalization of what exists presently.

Clearly, the above benefits may each be achieved by using different numbers of load profiles. In some cases (e.g. improved accuracy) it would be desirable to have a higher number of load profiles and in other cases (e.g. simplicity) fewer load profiles would be advantageous.

## Conclusions and Recommendations on Number of Profiles

### 9.6.1	Short Term

Based upon the above principles and associated guidelines, some initial analysis of the existing load research data will need to be undertaken. This analysis should aim to provide insight into the following questions:

- To what extent do the existing load shapes represent homogeneous groups?

- To what extent do the existing load shapes for similar customer groups (e.g. residential) from different utilities show significant differences between them?

- To what extent do existing load shapes from the same utility differ between customer groups, especially where a large number of such load shapes exist?

- What is the relationship between load shape and weather?

- To what extent can existing load shapes be segmented?

- To what extent will the existing load shapes facilitate a transition from market open to mature market?

In order to provide answers to some of the above, it is likely that the underlying sample data will require to be analyzed, in addition to the load shapes themselves. To provide an indication of the significance of supply cost differences and similarities; it would also be necessary to develop a suitable economic model.

It is recommended that consideration be given to the most suitable method of carrying out the required analysis, possibly in conjunction with the Load Profiling Agent.

For market opening, the Load Profiling Agent shall:

            - Collect load research data from existing utilities.

            - Qualify the data.

            - Search the data for:

- homogeneous samples;

- geographic zones that might be relevant, e.g.

          - weather zones

          - former control areas

          - UFE zones

            - Create load profiles for areas where data is available.

            - Identify regions without data and develop a methodology to assign load profiles to those areas or create hybrid load profiles from more than one utility’s data set.

### 9.6.2	Long Term

There is a good argument for defining a solution to be used on market start-up that minimizes risk. This solution is likely to be a pragmatic approach that can be delivered in time for the Customer Choice Pilot, due by June 1, 2001. A subsidiary argument is that whatever solution is selected, it should be close to the final solution (i.e. that used beyond market open). The disturbance of migrating all load profile class data on the Customer Registration database to a new regime may be unacceptable, particularly during a period of some incidence of resolving errors in other key components of the operation of the new market. There may also be an issue of customer cost/price disturbance caused by re-assigning customers to different load profile classes.

For the long-term market, the Load Profiling Agent shall:

            - Conduct load research.

            - Create load profiles for all areas open to competition.

            - When new areas open to competition (when munis or coops opt in) assign load profiles to those areas or create hybrid load profiles from the available data.

            - When new areas open to competition, expand load research to include those areas.

# Section 10.	Overall conclusions and recommendations

## Short Term

In summary, the short-term recommendations for Profiling Methodology are as follows:

- In sample design, accuracy shall be defined as industry best practice 90/10, as a minimum (Section 5).  It is recognized that this may not be achievable for market open, but shall be the target.

- All aspects of sampling methodology shall follow the Association of Edison Illuminating Companies (AEIC) load research guidelines (Section 5).

- Adjusted Static Load Profiling is the recommended methodology for beginning the competitive market within ERCOT (Section 6).

Currently, some sort of load research sample data exists for most of the large utilities within ERCOT. This load data could, with a detailed review, allow a reasonable estimate of hourly or sub-hourly consumption to be made for every customer within ERCOT.

Most of the load research programs do not have the capability currently to produce Dynamic Load Profiles. The major issues involved with Dynamic Load Profiling are the costs and time involved to install telemetry equipment on all sampled customers’ interval meters.

Static Load Profiles could be used at market opening, but there is no allowance for the effect of actual weather on the load profile estimates. Proxy Day and Adjusted Static Load Profiling are accurate at the class level, would require little start-up costs, and do allow for the effects of weather. Proxy Day, however, takes a simplistic approach to modeling, whereas, Adjusted Static can account for multiple variables that have an effect on load profiles.

- In order to determine how many load profiles to use at market open, existing load research will need to be analyzed (Section 9).

- To provide an indication of the significance of supply cost differences and similarities; it would also be necessary to develop a suitable economic model (Section 9).

- Consideration needs to be given to the most suitable method of carrying out the required data analysis, possibly in conjunction with the Load Profiling Agent, utilizing the principles and guidelines specified within this document (Section 9).

## Longer Term

In summary, the long-term recommendations for Profiling Methodology are as follows:

- The primary objectives for the preferred load profiling methodology shall be to ensure fairness to market participants and to be cost effective in terms of accuracy (Section 6).

- Periodic reviews of load profiles, (for example quarterly) using Mean Absolute Deviation (MAD) or Mean Absolute Percentage Error (MAPE) methodologies, shall be conducted (Section 7).  The specific actions depend on the incidence of Drift and Mortality.

# Appendix 1.	ERCOT Load Research Data

| ERCOT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Residential |  |  |  |  |  |  |  |  |
| # of Sample Interval Meters & Population Represented |  |  |  |  |  |  |  |  |
|  |  |  | Population | # Samples |  |  |  |  |
| CSWS Central Power & Light |  |  |  |  |  |  |  |  |  |
| CSWS Central Power & Light |  |  | 526,510 | 303 |  |  |  |  |
|  | Res Regular |  | 300,914 | 166 |  |  |  |  |
|  | Res Economy |  | 41,493 | 78 |  |  |  |  |
|  | Res Total Electric |  | 184,103 | 59 |  |  |  |  |
| Austin Energy Residential |  |  | 250,454 | 470 |  |  |  |  |
|  | Residential/Low Income |  | 250,000 | 410 |  |  |  |  |
|  | Residential – Churches |  | 454 | 60 |  |  |  |  |
| CSW-WTU Residential |  |  | 146,612 | 269 |  |  |  |  |
|  | Basic |  | 98,537 | 65 |  |  |  |  |
|  | Water Heat |  | 10,183 | 65 |  |  |  |  |
|  | Space Heat |  | 6,913 | 74 |  |  |  |  |
|  | Total Electric |  | 30,979 | 65 |  |  |  |  |
| TXU Residential |  |  | 2,100,153 | 300 |  |  |  |  |
|  | Residential |  | 2,100,000 | 250 |  |  |  |  |
|  | Time-of-Use |  | 153 | 50 |  |  |  |  |
| CPS Residential |  |  | 440,000 | 270 |  |  |  |  |
|  | Residential |  | 350,000 | 150 |  |  |  |  |
|  | Residential All Electric |  | 90,000 | 120 |  |  |  |  |
| Reliant Residential |  |  | 1,417,206 | 352 |  |  |  |  |
| TNMP Residential |  |  | 144,803 | 245 |  |  |  |  |
| TOTAL ERCOT RESIDENTIAL |  |  |  |  |  |  |  |  |
|  | REPRESENTED BY |  |  |  |  |  |  |  |
|  | PROFILING |  | 5,025,738 | 2,131 |  |  |  |  |
| ERCOT |  |  |  |  |  |  |  |  |
| Non-Residential |  |  |  |  |  |  |  |  |
| # of Sample Recorders & Interval Meters with Population Represented |  |  |  |  |  |  |  |  |
| (Does not include Engineering-Estimated Load profiles such as Street Lighting) |  |  |  |  |  |  |  |  |
| UTILITY |  | Rate Class |  |  | Population |  | # Samples |  |
| CSWS (Central P&L) |  | General Service |  |  | 76,270 |  | 114 |  |
| CSWS (Central P&L) |  | Light & Power Sec'ry |  |  | 5,495 |  | SNK* |  |
| CSWS (Central P&L) |  | Light & Power Primary |  |  | 122 |  | SNK* |  |
| CSWS (Central P&L) |  | Industrial Power Primary |  |  | 235 |  | 64 |  |
| CSWS (Central P&L) |  | Commercial Heat |  |  | 3,524 |  | SNK* |  |
| CSWS (Central P&L) |  | Commercial Water Heat |  |  | 316 |  | SNK* |  |
| CSWS (Central P&L) |  | Small Irrigation Sec'y |  |  | 606 |  | 148 |  |
| CSWS (Central P&L) |  | Large Irrigation Sec'y |  |  | 140 |  | 148 |  |
| CSWS (Central P&L) |  | Large Irrigation Primary |  |  | 18 |  | 148 |  |
| CSWS (Central P&L) |  | Cotton Gin |  |  | 47 |  | 47 |  |
| CSWS (Central P&L) |  | Petroleum Service Sec'y |  |  | 3721 |  | SNK* |  |
| CSWS (Central P&L) |  | Petroleum Service Primary |  |  | 94 |  | SNK* |  |
| CSWS (Central P&L) |  | Large Industrial ISB |  |  | 16 |  | 16 |  |
| CSWS (Central P&L) |  | Large Industrial 68 |  |  | 24 |  | 24 |  |
| CSWS-CPL Total |  |  |  |  | 90,628 |  | 709 |  |
| Austin Energy |  | Commercial non-demand |  |  | 22,000 |  | 175 |  |
| Austin Energy |  | Water & Waste Water |  |  | 150 |  | 30 |  |
| Austin Energy |  | Commercial Demand |  |  | 6,000 |  | 220 |  |
| Austin Energy |  | Primary Service |  |  | 23 |  | 23 |  |
| Austin Energy |  | Lg Primary Svs & TOU |  |  | 12 |  | 12 |  |
| Austin Energy |  | State of TX Non-Demand |  |  | 85 |  | - |  |
| Austin Energy |  | State of Texas Demand |  |  | 165 |  | - |  |
| Austin Energy |  | St of TX Large Primary & TOU |  |  | 8 |  | 8 |  |
| Austin Energy |  | Schools |  |  | 203 |  | 40 |  |
| Austin Energy Total |  |  |  |  | 28,646 |  | 508 |  |
| CSWS-WTU |  | Gen Serv. Space Heat |  |  | 301 |  | 40 |  |
| CSWS-WTU |  | Comm. Service Space Heat |  |  | 748 |  | 63 |  |
| CSWS-WTU |  | Comm. Service TCM |  |  | 32,008 |  | 115 |  |
| CSWS-WTU |  | Cotton Gin |  |  | 61 |  | 54 |  |
| CSWS-WTU |  | Gen Serv Secondary |  |  | 4,288 |  | 99 |  |
| CSWS-WTU |  | Gen Service Primary |  |  | 318 |  | 94 |  |
| CSWS-WTU |  | Municipal Water Pumping |  |  | 426 |  | 36 |  |
| CSWS-WTU Total |  |  |  |  | 38,150 |  | 501 |  |

| TXU | Gen Service Secondary | 326,115 | 410 |
| --- | --- | --- | --- |
| TXU | Gen Service Primary | 6,300 | 302 |
| TXU | Municipal Service Sec'y | 15,843 | 190 |
| TXU Total |  | 348,258 | 902 |
| CPS | General Service (C<1000) | 54,000 | 100 |
| CPS | Large Light & Power (C 50-3000 kW) | 1,900 | 140 |
| CPS | Extra Large Power (I – 2000-7000 kW) | 100 | 70 |
| CPS | Super Large Power (I > 4500 kW) | 30 | 30 |
| CPS Total |  | 56,030 | 340 |
| Reliant | Econ. Improvement Rate | 10 | 10 |
| Reliant | Hourly Variable Pricing | 3 | 3 |
| Reliant | HLP Own Use/Large Buildings | 6 | 6 |
| Reliant | Interruptible Svcs. 10 minute notice | 34 | 34 |
| Reliant | Interruptible Svcs. 30 minute notice | 24 | 24 |
| Reliant | Interruptible Svcs. Instantaneous | 4 | 4 |
| Reliant | Interruptible Svcs.- Supplemental | 8 | 8 |
| Reliant | Large Overhead Service (A) | 77 | 77 |
| Reliant | Large Overhead Service (B) | 19 | 19 |
| Reliant | Large General Svcs | 1,633 | 1,633 |
| Reliant | Misc. Gen. Svcs. | 191,565 | 732 |
| Reliant | Standby Interruptible Service | 10 | 10 |
| Reliant | Special Contract Pricing | 12 | 12 |
| Reliant | State-owned Educational Institutions | 5 | 5 |
| Reliant | Standby Electric Service | 18 | 18 |
| Reliant | TX-NM Power Company Contract | 2 | 2 |
| Reliant Total |  | 193,430 | 2,597 |
| TNMP | Gen Service (<100 kW) | 12,214 | 140 |
| TNMP | LGS (>100 kw) | 433 | 61 |
| TNMP | LGS (>100 kW) | 433 | 61 |
| TNMP | IPS (>2000 kva) | 7 | 7 |
| TNMP | IPS (>2000 kVa) | 7 | 7 |
| TNMP Total |  | 12,654 | 208 |
|  | Total, All Companies | 767,796 | 5,765 |
| *SNK - Sample size not known. |  |  |  |

| ERCOT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MISCELLANEOUS INFORMATION |  |  |  |  |  |  |  |
| CURRENT ASSESSMENT OF LOAD PROFILING INFORMATION AVAILABLE |  |  |  |  |  |  |  |
|  | CSWS CPL | Reliant | CPS | Austin Energy | TXU | CSW-WTU | TNMP |
| When was sample last used (mmyy)? | Sep99 | Sep-99 | Aug-98 | Aug-97 | Sep-99 | Sep99 | Sep-93 |
| kW above which interval recorder required for billing | 500 | 400 | None | 3000 | 2500 | 500 | 2000 kVa |
| Expansion Methodology | Ratio | Ratio | MPU | MPU | Ratio | MPU | MPU |
| Stratification Variable or Methodology | kWh | Various; Random, Dalenius-Hodges-Neyman | kWh, kW | Various: Random, Dalenius-Hodges-Neyman | Simple random; kW | kWh | kWh |
| % of ann'l system peak covered by all interval metering | 39 | 34.5 | N/A* | 9.1 | 11% | 46 | N/A* |
| % of peak month (mWh) covered by all interval metering | 33 | 40.5 | N/A* | 12.6 | 12% | 39 | N/A* |
| Sample Removal Date | N/A* | Still in Place | Still in Place | Still in Place | Still in Place | N/A* | N/A* |
| Data Retrieval Method | Probe | MV90, ITRON | Probe | MV90, Hand-held; Probe | Cell Phone | Probe | Probe |
| Statistical Confidence of Sample | N/A* | 90% +/-10% | 95% +/-5% | 90% +/-10% | 90%-95% +/-10% | 90% +/-10% | 90% +/-10% |
| *N/A – Data not provided. |  |  |  |  |  |  |  |

# Appendix 2.	Non-residential customer segmentation data

# Appendix 3.	Interval Metering Costs

| ERCOT Profiling Methodology Work Group |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Estimated costs to purchase and install interval data recorders |  |  |  |  |  |  |  |
|  | for load research related to profiling |  |  |  |  |  |  |
| Sample | Meter & | Maintenance | Communications (Cellular) |  | Communications (Land Line) |  | Data Retrieval |
|  | Installation | & Testing | Equip & Install | Monthly | Equip & Install | Monthly | & Translation |
| 1 | $ 753 | $23/meter/mo | $ 850 |  | $ 120 |  | $11/meter/mo |
| 2 | $ 730 | no bid | no bid |  | no bid |  | $14/meter/mo |
| 3 | $ 721 | $19/meter/mo | 820 |  | 100 |  | $25/meter/mo |
| 4 | <------------ | $ 1,450 | ----------> | $12 -15 |  |  |  |
| 5 | $ 500 | Mnt. =$33/hr | $850 | contract | $ 100 | $ 25 |  |
|  |  | Tstng=$50/mtr |  |  |  |  |  |
| 6 | $ 735 | $19/meter/mo | $ 850 |  | $ 112 |  | $25/meter/mo |
| 7 |  |  |  |  |  |  | $11/meter/mo |
| 8 |  |  |  |  |  |  | $ 8/meter/mo |
| 9 | $ 1,130 | $20/meter/mo | no bid |  | $ 155 |  | $14/meter/mo |
| 10 | $ 1,174 | $25/meter/mo | no bid |  | $ 125 |  | $19/meter/mo |
| 11 | $ 669 |  |  |  | $ 94 | $ 40 |  |
| 12 | $ 570 | $14/meter/mo | 1000 |  | $ 175 | $ 18 |  |
| Note: Prices increase as distance from major metropolitan area increases. |  |  |  |  |  |  |  |
| High | $ 1,174 | $25/meter/mo | $ 1,000 |  | $ 175 | $ 40 | $ 8/meter/mo |
| Low | $ 500 | $19/meter/mo | $ 820 |  | $ 94 | $ 18 | $25/meter/mo |
| Average | $ 776 | $20/meter/mo | $ 874 |  | $ 123 | $ 28 | $16/meter/mo |
| Five Year life costs per meter |  |  |  |  |  |  |  |
| Assumptions: |  |  |  |  |  |  |  |
|  | Average cost for meter and installation |  |  |  |  | $ 776 |  |
|  | Average cost for meter maintenance and testing * 60 months |  |  |  |  | 1,200 |  |
|  | Average cost for land line communications equipment & installation |  |  |  |  | 123 |  |
|  | Average cost for monthly land line communications cost * 60 months |  |  |  |  | 1,660 |  |
|  | Average cost for translation * 60 months |  |  |  |  | 960 |  |
|  |  |  |  |  |  | $ 4,718 |  |

# Appendix 4.	Glossary

This Glossary is intended as a reference to terms and definitions used specifically in relation to load profiling, as used in settlement. It does not include other terms and definitions that are used in the ERCOT Deregulation Project.

Customer – In general, customer equates to customer premise and will conform to the definition of the ERCOT Customer Registration taskforce document ‘Customer Data Recommendations.

Data processing – The set of processes carried out on the raw data collected from the IDRs. These processes include verification, editing and data retention.

Design (auxiliary) variable – The variable chosen, as part of the sample design, which is used to determine the statistical requirements for the sample size.

Expansion – The process of estimating a particular attribute of a population from the relevant attribute of a sample of that population.

Interval Data Recorder (IDR) - Metering that is capable of recording load usage in relatively short time intervals (e.g. hourly, 15-minute). For purposes of this document, such devices are installed on a sample of customers for a given class to measure the customers’ load usage for the development of load profiles.

Load Profile - A representation of the consumption pattern of a group of customers, showing the demand variation on an hourly or sub-hourly basis.

Load Profile Class - A group of customers having similar energy usage patterns and that are assigned the same load profile.

Load Profiling - The set of processes, carried out as part of ERCOT settlements, that allows the consumption data in respect of non-interval metered customers, to be separated into hourly or sub-hourly consumption load profiles, by reference to generic hourly or sub-hourly load profiles, representative of those particular customers.

Load Profiling Methodology - The methodology utilized for:

- the production of the reference hourly or sub-hourly consumption load profiles;

- the statistical analysis of the reference load profiles; and

- maintenance of the reference load profiles.

Mean Absolute Deviation (MAD) – The mean of the absolute value of the differences between the supplied values (sample) and the arithmetic mean of all values (population).

Mandated level for interval metering – The level of maximum demand of an individual customer, specified in kW, above which it is required that an IDR is installed for the purpose of settlement.

Mean Absolute Percentage Error (MAPE) – The mean of the absolute value of the percentage errors of all the projected data in the forecasting model versus the actual data.

Sampling – The process of selecting a subset of a population of customers that statistically represent the population. For load profiling purposes, the samples have IDR installed.

Sample design – A set of processes used to determine the appropriate requirements for a sample of customer premises to be used for the purpose of creating a load profile.

Sample size – The number of data points (i.e. customer premises) in a particular sample.

Segmentation – The process of dividing a population into a number of sub-sets, according to certain parameters, for the purpose of creating load profiles for sub-sets of the population.

Segmentation parameter – The parameter chosen as the basis for segmentation.

Association of Edison Illuminating Companies Load Research Manual

* Some studies have shown adjusted static load profiling could be more accurate than true dynamic.

Relevant Co-op data may also be available.

Assumes that load factor/kW basis is used only for customer groups that already have maximum demand metering installed.

1