| PRR Number | 532PRR | PRR Title | Implementation of Non-Transmission Alternatives to RMR and OOM Services |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 2.1, Definitions 6.1.9, Reliability Must-Run Service 6.3.1, ERCOT Responsibilities 6.5.9.2, Exit Strategy from an RMR Agreement [new] 6.7.8, Deployment of RMR Service 6.8.3, Capacity and Energy Payments for RMR and Synchronous Service 6.9.4.2, Settlement Obligations for RMR Service 7.4.5, Plan to Alleviate Chronic Local Congestion Charges |  |
| Requested Resolution (Normal or Urgent) |  | Urgent WMS is requested urgent status based on the fact that RMR contracts will need to be renewed by years end for 2005. Therefore, any non-transmission exit strategies for RMR for 2005 will need to proceed immediately for consideration. Without passage of this PRR, those alternatives could not be considered. |  |
| Revision Description |  | Revisions to allow ERCOT to implement non-transmission alternatives to Reliability Must-Run (“RMR”) and Out-Of-Merit (“OOM”) Services to reduce uneconomic uplift. |  |
| Reason for Revision |  | Current Protocols require ERCOT to identify non-transmission Resources that could result in lower local congestion costs. This Protocol change would allow ERCOT to implement Agreements with these non-transmission Resources and settle the resulting payments. |  |
| Timeline |  |  |  |
| Date Posted |  | 7/20/04 |  |
| Please see the Master List on the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Robert Helton, on behalf of WMS |
| E-mail Address | bhelton@anpower.com |
| Company | American National Power, Inc. |
| Company Address | 18509 Champions Circle, Point Venture, TX 78645 |
| Phone Number | (512)-267-6986 |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | More contracting, contract administration, proposal analysis, settlement according to non-standard contract | None |  |
| MARKET SEGMENT |  |  |  |
| Consumer | More active input into planning process | None | Lower local congestion costs |
| LSE: General, Including NOIE | More active input into planning process | None | Lower local congestion costs |
| LSE: CR & REP | More active input into planning process | None | Lower local congestion costs |
| QSE |  | None |  |
| Resource | More active input into planning process | None |  |
| TDSP |  | None |  |

Proposed Protocol Language Revision

## 2.1	Definitions

Must-Run Alternative (MRA) Agreement

The contractual arrangement between ERCOT and a MRA Resource.

Must-Run Alternative (MRA) Resource

A Resource that was selected through the planning process pursuant to Section 6.5.9.2, Exit Strategy from an RMR Agreement, or 7.4.5, Plan to Alleviate Chronic Local Congestion Charges, to provide steady-state or dynamic voltage support, stability or management of localized transmission constraints under first contingency criteria, as described in the Operating Guides, at a lower cost than an RMR Agreement or chronic OOM deployment.

[Add to Acronyms in Section 2]

MRA	Must-Run Alternative

SPS	Special Protection Systems

### Reliability Must-Run Service

As provided by ERCOT to QSEs: Agreements for capacity and energy from Resources which otherwise would not operate and which are necessary to provide voltage support, stability or management of localized transmission constraints under first contingency criteria, as described in the Operating Guides, where Market Solutions do not exist.  This includes service provided by RMR Units and MRA Resources.

As provided by a QSE to ERCOT: The provision of Generation Resource capacity and/or energy Resources under a Reliability Must-Run (RMR) Agreement or a Must Run Alternative (MRA) Agreement, including Agreements with Synchronous Condenser Units, whose operation is directed by ERCOT.

### ERCOT Responsibilities

- ERCOT shall make reasonable efforts to minimize the use of OOMC, Zonal OOME, or contracted RMR Facilities.  This includes entering into MRA Agreements with Resources selected through the planning process, pursuant to Sections 6.5.9.2 and 7.4.5, to provide services to meet reliability requirements at lower total expected costs than would otherwise be provided by RMR Agreements or through OOM Service deployments.

6.5.9	Reliability Must-Run Service

#### 6.5.9.2 Exit Strategy from an RMR Agreement

(1)	{Moved this paragraph to this new subsection from 6.5.9.1(11)} No later than ninety (90) days following the execution of an RMR Agreement, ERCOT shall report to the Board and post on the MIS a list of feasible alternatives that may, at a future time, be more cost-effective than the continued renewal of the existing RMR Agreement.  Through the normal ERCOT System planning process, ERCOT shall develop a list of potential alternatives to the service provided by the RMR Unit.  At a minimum, the list of potential alternatives that ERCOT shall consider include, but are not limited to, construction of new or expansion of existing Transmission Facilities, installation of voltage control devices, solicitation or auctions for interruptible Load from Retail Electric Providers, or extension of the existing RMR Agreement on an annual basis.  If a cost-effective alternative to the service provided by the RMR Unit is identified, ERCOT shall provide a proposed timeline to study and/or implement the alternative.

(2)	ERCOT shall provide available information that would enable potential MRA Resources to assess the feasibility of submitting a proposal to provide a more cost-effective alternative to an RMR Unit through the regional planning process, including any known minimum technical requirements and/or operational characteristics required to eliminate the need for the RMR Unit.

(3)	ERCOT may negotiate a contract for an MRA Resource identified by Market Participants through the regional planning process that:

(a)	technically provides the level of reliability that would otherwise continue to be provided by an RMR Unit;

(b)	clearly provides a more cost-effective alternative to continued service by the RMR Unit or the most cost-effective transmission alternative; and

(c)	has been posted for a twenty-eight (28) day comment period.

(4)	If the resulting MRA Agreement would result in lower total costs than continued service by the RMR Agreement or the most cost-effective transmission alternative, ERCOT shall recommend approval of the MRA Agreement to the ERCOT Board of Directors.

(5)	If  the MRA Agreement is more expensive than the preferred transmission alternative, but less expensive than continuation of the RMR Agreement, ERCOT shall recommend approval of the MRA Agreement to the ERCOT Board of Directors with  the term of the MRA Agreement limited to the time period until the cost-effective transmission alternative can be implemented.

(6)	If the execution of an MRA Agreement would result in the foreclosure of other technically viable solutions (e.g., the RMR Resource that is being replaced by the MRA Agreement retires and is no longer available as an alternative to the MRA Agreement), the MRA Agreement shall include terms and conditions that limit the MRA Resource owner’s ability to withdraw or raise the price of the MRA Agreement in future years until a transmission solution can be implemented.

### Deployment of RMR Service

- If Market Solutions are not available, and in Emergency Conditions, ERCOT shall have the option to Dispatch a contracted RMR Unit at any time for voltage support or localized transmission limitations, but it must Dispatch the unit as early as possible (if conditions merit) once conditions are identified that require the use of the RMR Unit and only to the extent of megawatt loading necessary to correct the voltage support or localized transmission limitation.

- ERCOT must elect to use units under an RMR Agreement or MRA Agreement before issuing an OOME or Zonal OOME Dispatch Instruction subject to the terms of the Agreement, if practical.

- ERCOT will deploy RMR Units in accordance with the RMR Agreement and MRA Resources in accordance with the MRA Agreement.  RMR Agreements with ERCOT are expected to include limitations on the total service hours, megawatt-hour output, and the number of starts available to ERCOT for each RMR Unit.

- ERCOT shall issue Dispatch Instructions via the Messaging System for any RMR Unit deployment or MRA Resource deployment.  Any revisions to those instructions must be communicated via revised Dispatch Instructions.

- Note:  Language added by PRR515 approved by the Board on July 20, 2004 has been added below as (5) since it will go into the Protocols on August 1.

- In the event that RMR is ordered to sustain reliable ERCOT System operation in any Operating Day, ERCOT will post on the MIS as soon as possible, but no later than the next Business Day, for such Operating Day:

- (a)	each unit receiving an RMR Dispatch Instruction for each interval;

(b)	the amount of RMR energy provided by each unit for each interval; and

(c)	the binding transmission constraint (contingency and/or overloaded element(s)) causing RMR deployments.

- ERCOT shall publicly post an annual forecast of the Dispatch pattern it expects for each contracted RMR Unit as well as monthly and week-ahead forecasts regarding its use of such units.

- ERCOT will adjust the amount of Balancing Energy acquired due to the impact of RMR Energy deployed and energy deployed from MRA Resources.  If adjustments made by ERCOT would result in the QSE exceeding its scheduled amount of generation, then the affected QSE must not accommodate these changes by adjusting other Resources such that the Schedule Control Error is minimized.  URC charges will not be assessed to the QSE as a result of these adjustments for the interval.  The RMR may implement a Responsibility Transfer between its QSE and ERCOT for energy delivered under an RMR Agreement to minimize the impact of RMR scheduling on its QSE.

### 6.8.3	Capacity and Energy Payments for RMR and Synchronous Service

RMR Units and MRA Resources selected through the planning process, pursuant to Sections 6.5.9.2, Exit Strategy from an RMR Agreement, and 7.4.5, Plan to Alleviate Chronic Local Congestion Charges, providing services will be paid according to their Agreement.

#### 6.9.4.2	Settlement Obligations for RMR Service

(1)	Reliability Must Run costs, separated by category and available to QSEs via data extract (i.e., Standby Price and energy dollar amounts) will be allocated on a Load Ratio Share per QSE in the following manner:

LARMRiq 	=  -1 *{(Σ (ERMRiq)+ Σ (SBRMRiq) + Σ (UMRMRiq) + ΣERRMRiq)+ Σ(CRMRiq)} * LRSiq

Where:

i	Interval

q	QSE

LARMRiq 	RMR Load Allocation for that interval of that QSE

ERMRiq	RMR Energy Dollar Amount per interval of that QSE

SBRMRiq	RMR Standby Price in that interval of the QSE

UMRMRiq	RMR Unexcused Misconduct Fee in that interval of that QSE

LRSiq	Load Share Ratio for that interval of the QSE

ERRMRiq	RMR Energy Rebate Amount for that interval for that QSE

CRMRiq	Payment in that interval to MRA Resources reflecting the equivalent hourly capacity payment* and the actual hourly energy payment to each QSE

*	The equivalent hourly capacity payment for each hour will be determined by dividing each capacity payment by the number of hours covered by the term of such capacity payment for the term that includes the hour.

[PRR 492: Add Section 7.4.5 upon system implementation and filling staffing requirements:] 7.4.5 Plan to Alleviate Chronic Local Congestion Charges (1) ERCOT shall monitor Local Congestion Area costs and post a report on the MIS ten (10) days following the end of the month. The report shall include the amount of Local Congestion Area costs by type of payment to Resources (e.g., OOME Up, OOME Down, and OOMC) including discussion of the limiting transmission element(s) or other significant events that may have contributed to Local Congestion as available. This information will also be reported monthly to the ERCOT Board of Directors, PUCT Market Oversight Division, and appropriate ERCOT subcommittees. (2) For the purposes of this section, “Local Congestion Area” is defined as a sub-region of a Congestion Zone in which ERCOT dispatches OOME, OOMC, or local Balancing Energy Service on a regular basis as described in the ERCOT monthly report of Local Congestion Cost. (3) If the amount of OOME, OOMC, or local Balancing Energy Service in a Local Congestion Area exceeds a threshold amount of either ten million dollars ($10 million) during the most current twelve (12) consecutive months or five million dollars ($5 million) in the most current three (3) consecutive months, ERCOT shall report such to the ERCOT Board, PUC Market Oversight Division, and appropriate ERCOT subcommittees and shall initiate a study to identify feasible alternatives that may, at a future time, be more economically efficient than the continued use of OOME, OOMC, or local Balancing Energy Service. (4) ERCOT shall develop feasible alternatives and their associated costs to reduce or replace the use of OOME, OOMC, or local Balancing Energy Service. At a minimum, the list of feasible alternatives that ERCOT shall include consideration of a new CSC for the following year under Section 7.2.1.1, Process for Determining CSCs, item (2), a change in ERCOT operations affecting the Local Congestion, construction of new or expansion of existing Transmission Facilities, installation of voltage control devices, more accurate modeling of transmission line or transformer MVA Real Time limits, solicitation or auctions for interruptible Load from Retail Electric Providers when such Protocols are available, or continued use of OOME, OOMC, or local Balancing Energy Service. (5) ERCOT shall prepare a report, no later than forty-five (45) days after the threshold is exceeded, on the feasible alternatives available to reduce Local Congestion below the threshold amount. The report shall include: (a) The root cause of the Local Congestion; (b) Current actions of the TDSP and/or ERCOT to relieve such Local Congestion; (c) A review of ERCOT’s operations (e.g., Dispatch Instructions, transmission line limits, etc.); (d) A determination of short term mitigation measures that could reduce the use of OOME, OOMC or local Balancing Energy Service; and (e) The amount of market impact caused by the Local Congestion. The report shall also include an implementation plan, timeline, and recommendations regarding short-and long-term solutions to the Local Congestion as appropriate. The implementation plan may identify studies to be conducted prior to recommendations for short- and long-term solutions to the Local Congestion. These additional studies will be conducted through the regional planning process, as appropriate. (6) This report shall be posted on the MIS and provided to the PUCT Market Oversight Division, the appropriate ERCOT subcommittee(s), and ERCOT Board of Directors. (7) Subsequent to the initial report, ERCOT staff shall provide monthly updates on the schedule and planning status to the PUCT Market Oversight Division, appropriate ERCOT subcommittee(s), and the Board. ERCOT will use its best efforts to implement short- and long-term solutions as quickly as practical. (8) ERCOT shall provide available information that would allow potential MRA Resources to assess the feasibility of submitting a proposal to provide a more cost-effective alternative to OOME, OOMC, or local Balancing Energy Service through the regional planning process, including any known minimum technical requirements and/or operational characteristics required to eliminate or reduce the OOME, OOMC, or local Balancing Energy Service. (9) ERCOT may negotiate a contract for an MRA Resource identified by Market Participants through the regional planning process that: (a) technically provides the level of reliability that would otherwise continue to be provided by OOME, OOMC, or local Balancing Energy Service; (b) clearly provides a more cost-effective alternative to continued service by the RMR Unit or the most cost-effective transmission alternative; and (c) has been posted for a twenty-eight (28) day comment period. (10) If the resulting MRA Agreement would result in lower total costs than continued service from the OOME, OOMC, or local Balancing Energy Service or the most cost-effective transmission alternative, ERCOT shall recommend approval of the MRA Agreement to the ERCOT Board of Directors. (11) If the MRA Agreement is more expensive than the preferred transmission alternative, but can provide relief until the transmission alternative can be implemented, ERCOT shall recommend approval of the MRA Agreement to the ERCOT Board of Directors with the term of the MRA Agreement limited to the period until the transmission alternative can be implemented. (12) If the execution of an MRA Agreement would result in the foreclosure of other technically viable solutions (e.g., the RMR Resource that is being replaced by the MRA Agreement retires and is no longer available as an alternative to the MRA Agreement), the MRA Agreement shall include terms and conditions that limit the MRA Resource owner’s ability to withdraw or raise the price of the MRA Agreement in future years until a transmission solution can be implemented.