| PRR Number | 476PRR | PRR Title | Ramp Rate Adherence During Local Congestion |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs 6.10.7, Individual Resource Dispatch Performance |  |
| Requested Resolution (Normal or Urgent) |  | Urgent |  |
| Revision Description |  | Incorporates language in the Protocols to cover a unit’s ramp rate to be included in unit specific deployments |  |
| Reason for Revision |  | Currently, the ramp rates on individual resources are ignored during Local Congestion. This leads to Dispatch Instructions that are impossible to adhere to, especially when every unit running for a QSE is getting a unit specific instruction. Although a QSE may ignore category 4 instructions, they must still follow the category 1. In the event that every unit running for a QSE in a zone is getting a category 4, they all make up the category 1. As a result of the ramp rates being violated, SCE cannot be followed, which puts a strain on frequency which leads to regulation and balancing instructions. Further, the QSE can potentially be penalized with imbalance charges for not keeping SCE at 0. To illustrate this here is an example: QSE 1 has 2 units both with a 200 MW capability in the North Zone – Unit A with a 15 MW/min ramp rate and Unit B with a 1 MW/min ramp rate – both units are scheduled at min load of 50 MW each. QSE 1 receives a category 1 UBES deployment to go up 150 MW. In addition, they receive a category 3 for Unit B to go to 200 MW. Although the category 1 can be followed if both units move, the category 3 cannot be met as Unit B can only go a max of 10 up. Therefore, the QSE will not be paid for meeting unit specific deployment since the unit can’t respond and they will have to move Unit A to make up for it in order to maintain SCE – Essentially collecting MCPE which may be much lower than the cost of the unit resulting in financial loss for the QSE This PRR recommends provisions be added to the Protocols requiring ERCOT Dispatch Instructions include and adhere to unit Resource Plan ramp rates. |  |
| Timeline |  |  |  |
| Date Posted |  | 11/13/03 |  |
| Please see the Master List on the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name |  | Kristy Ashley |  |  |
| E-mail Address |  | kashley@pwrteam.com |  |  |
| Company |  | Exelon Power Team |  |  |
| Company Address |  | 300 Exelon Way Kennett Square, PA |  |  |
| Phone Number |  | 610-765-6631 |  |  |
| Fax Number |  | 610-765-6595 |  |  |
| ERCOT/Market Segment Impacts and Benefits |  |  |  |  |
|  | Impact |  |  | Benefit |
|  | Business |  | Computer Systems |  |
| ERCOT | Yes |  | Yes | Better Frequency Control |
| MARKET SEGMENT |  |  |  |  |
| Consumer | Not known |  | Not known | Not Known |
| LSE: General, Including NOIE |  |  |  |  |
| LSE: CR & REP |  |  |  |  |
| QSE | Yes |  | No | Better SCE Control/management and fewer imbalance charges |
| Resource | Yes |  | No |  |
| TDSP |  |  |  |  |

Proposed Protocol Language Revision

#### Deployment of Balancing Energy when Congestion Occurs

- If the Operational Model indicates there is Zonal Congestion, ERCOT will separate the Balancing Energy Service bids into a Bid Stack for each Congestion Zone.

- ERCOT will use the Operational Model to determine the amount and location of Balancing Energy deployment for clearing Zonal Congestion as well as balancing the system.

- Except as stated in item (4) below, ERCOT will deploy Balancing Energy bids within a zone in bid price Merit Order.

- ERCOT may form specific Resource prices for both incrementing and decrementing a specific Resource to resolve Local Congestion.

- As part of the submittal of the Resource Plan, QSEs may specify bid premiums by Resource that would be used to determine Resource-specific prices.  Resource-specific incremental prices will be formed by ERCOT by adding the incremental bid premium to the MCPE of the Congestion Zone.  The Resource-specific premium price will not be less than the bid premium.  Resource-specific decremental prices will be the decremental premium specified.

- The actual Shift Factors with respect to the Local Congestion of Resources’ individual incremental and decremental prices from above are used to determine the most economical deployment of individual Resources to solve Local Congestion.

- ERCOT will instruct QSEs to deploy Balancing Energy Service from a specific Resource through the issuance of a Dispatch Instruction to each Resource according to the most economical solution to resolve the Local Congestion.  The deployment of Resource specific Balancing Energy Service must be in equal incremental and decremental amounts.  The decremental amount will be selected such that it does not adversely affect any other constraint.

- The Dispatch Instruction will specify the current output level, the amount of Balancing Energy Service, and the range of acceptable operation of the specific Resource.  ERCOT’s determination of Dispatch Instruction will also adhere to, and not exceed unit ramp rates in the QSE’s Resource Plan.

- If a Market Solution exists for an incremental Resource-specific instruction, the QSE will be paid the incremental premium specified, times the difference in megawatts between the Resource’s current output level and the minimum of the allowed range specified in the Dispatch Instruction at the time of receipt of the Dispatch Instruction for all hours or portions of an hour the Resource is individually dispatched.

- If a Market Solution exists for a decremental unit-specific Dispatch Instruction, the QSE will be paid the product of:  1) the difference in MCPE minus the decremental premium specified, times 2) the Instruction for all hours or portions of an hour the Resource is individually dispatched.

- QSEs shall first meet the specific Resource deployment performance requirements of Section 6.10.7, Individual Resource Dispatch Performance and then provide the Balancing Energy Service deployment instructed pursuant to Section 6.7.1, Deployment of Balancing Energy Service.  In the event that a QSE is unable to provide the Balancing Energy Service due to a specific Resource deployment then the QSE will follow the notification procedures established in Section 5, Dispatch.

- If a Market Solution does not exist, then ERCOT will use OOME to price the Resources dispatch to resolve the constraint.

- The QSEs providing Balancing Energy Service shall meet the deployment performance requirements specified in Section 6.10.4, Ancillary Services Deployment Measures.

[PRR359 & PRR424: Replace Subsection 6.7.1.2(5)-(12) as follows when the system change is implemented.]

- As part of the submittal of the Resource Plan, QSEs may specify bid premiums by Resource that would be used to determine Resource-specific prices.  Resource-specific incremental prices will be the incremental bid premium specified by the QSE and the Resource-specific decremental prices will be the decremental premium specified by the QSE.

- The actual Shift Factors with respect to the Local Congestion of Resources’ individual incremental and decremental prices from above are used to determine the most economical deployment of individual Resources to solve Local Congestion.

- ERCOT will instruct QSEs to deploy Balancing Energy Service from a specific Resource through the issuance of a Dispatch Instruction for each Resource using the most economical solution to resolve the Local Congestion.

- The Dispatch Instruction will specify the instructed output level, the amount of Balancing Energy Service, and the range of acceptable operation of the specific Resource.  ERCOT’s determination of Dispatch Instruction will also adhere to, and not exceed unit ramp rates in the QSE’s Resource Plan.

- If a Market Solution exists for an incremental Resource-specific instruction, the QSE will be paid in accordance with Section 7.4.3.1, Balancing Energy Up from a Specific Resource.

- If a Market Solution exists for a decremental unit-specific Dispatch Instruction, the QSE will be paid in accordance with Section 7.4.3.2, Balancing Energy Down from a Specific Resource.

- QSEs shall first meet the specific Resource deployment performance requirements of Section 6.10.7, Individual Resource Dispatch Performance and then provide the Balancing Energy Service deployment instructed pursuant to Section 6.7.1, Deployment of Balancing Energy Service.  In the event that a QSE is unable to provide the Balancing Energy Service due to a specific Resource deployment then the QSE will follow the notification procedures established in Section 5, Dispatch.

- If a Market Solution does not exist, then ERCOT will use OOME to determine the amount paid to the QSE in connection with the Resources dispatched to resolve the constraint

## Ancillary Service Qualification, Testing and Performance Standards

### Individual Resource Dispatch Performance

QSEs may receive Dispatch Instructions to operate a specific Resource within limitations specified for an individual Resource.  The QSE shall conform to the limitations restricting Resource loading for the effective time of the Dispatch Instruction to within two (2) MW of the requested limitation.  ERCOT’s Dispatch Instruction will adhere to, and not exceed the QSE’s unit specific Resource Plan ramp rates.  QSE’s conformance to ERCOT Dispatch Instructions will be within its unit ramp rates in its Resource Plan.