Texas

Standard

Electronic

Transaction

814_20:

##### Create/Maintain/Retire ESI ID Request

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_20:

CREATE/MAINTAIN/RETIRE ESI ID REQUEST

This transaction set...

… from the TDSP to ERCOT is used to initially populate the registration system for conversion/opt-in.

...  from the TDSP to ERCOT is used to communicate the addition of a new ESI ID, changes to information associated with an existing ESI ID, or retirement of an existing ESI ID.

... from ERCOT to the current CR, is used to communicate the TDSP’s changes to an existing ESI ID.

... from ERCOT to the new CR, is used to communicate the TDSP’s changes to an existing ESI ID.

Document Flow:

- TDSP to ERCOT

- ERCOT to Current CR

- ERCOT to New CR

Note:  When the Current CR and New CR are the same ERCOT will only send one 814_20 to that CR.

|  |  | # Summary of Changes |
| --- | --- | --- |
| July 3, 2000 |  | Initial Release |
| August 23, 2000 |  | The following changes were made: |
|  |  | - Added LIN code value “IN” for Initial Conversion/Opt In |
|  |  | - Added Voltage Code (REF~AQ) with possible codes for Primary, Secondary and Transmission and associated Reason for Change Code (REF~TD~REFAQ) |
|  |  | - Clarified Standard Interconnection Point (REF~SPL) |
|  |  | - Removed Settlement Zone (REF~S1) and associated Reason for Change Code (REF~TD~REFS1) |
|  |  | - Changed ERCOT/Non-ERCOT to Power Region and added the codes: ERCOT, WSCC, SERC and SPP |
|  |  | - Added NM101 codes for Meter Addition (MA), Meter Removal (MR) and Meter Exchange (MX) along with their associated Reason for Change Codes (REF~TD) |
|  |  | - Added Old Meter Number for Meter Exchange |
|  |  | - Added Meter information – Meter Multiplier (REF~4P), Number of Dials (REF~IX) |
|  |  | - Removed UFE Zone (REF~D8) and associated Reason for Change Code (REF~TD~REFD8) |
|  |  | - Changed Load Profile (REF~LO), TDSP Rate Class (REF~NH), TDSP Rate Subclass (REF~PR), Meter Cycle (REF~TZ or DTM~313) to Required on Conversion/Create ESI ID. |
|  |  | - Added Start and End Meter Read for Meter Additions, Removals and Exchanges. |
|  |  | - Removed Congestion Zone (REF~ZW) and associated Reason for Change Code (REF~TD~REFZW) |
|  |  | - Change transaction description page |
|  |  | - Remove REF Reference Identification (New Premise Indicator) segment. |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add BGN06 Element to BGN segment per Change Control #2000-037. |
|  |  | - Add BGN08 Element to indicate the TX SET Transaction Number per Change Control #2000-015. |
|  |  | - Add examples to end of transaction. |
|  |  | - Add REF~PTC Premise Type to LIN loop per Change Control #2000-006. |
|  |  | - Change REF~AQ Distribution Loss Factor to include 5 TDSP defined codes and the Code “T” Transmission Only per Change Control #2000-035. |
|  |  | - Change REF~SPL from Standard Interconnection Point to Station ID in the LIN loop per Change Control #2000-004. |
|  |  | - Add Time of Use code 71-- Summer Super On-Peak to REF04 of REF~4P, REF~IX, Q2 and Q3 Segments in the NM1 loop per Change Control #2000-18. |
|  |  | - Change the gray box Level 1 note to read “Create ESI ID Request: Not Used” instead of “Create ESI ID Request: Required” in the REF~NH TDSP Rate Class segment and the REF~PR TDSP Rate Subclass segment in the NM1 loop per Change Control #2000-003. |
|  |  | - Remove codes NM1MA, NM1MR and NM1MX; add codes REF4P, REFIX, REFMT and REFPRT to REF~TD in the NM1 loop per Change Control #2000-009.. |
| December 12, 2000 Version 1.2 (Continued) |  | - Add REF~PRT Unmetered Service class in the NM1 loop per Change Control #2000-002. |
|  |  | - Add REF~TD code of DTM307 in the LIN loop to indicate Change of Pilot Eligibility Date per Change Control #2000-040. |
| February 15, 2001 Version 1.2a |  | The following changes were made: |
|  |  | - Add REF PHC (Energized/De-energized) to the LIN loop to indicate to ERCOT when a facility is energized or de-energized during conversion per Change Control #2001-044. |
|  |  | - Change the N106 element in the N1 (Transmission Distribution Service Provider) segment from required to dependent to facilitate when the TDSP is neither the sender or receiver (i.e. when ERCOT forwards the transaction to the CR) per Change Control #2001-045. |
| March 16, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Removed Scenario Names from Transaction Description page |
|  |  | - Corrected the How to Use this Implementation Guide page |
|  |  | - Changed RA/Clearinghouse to ERCOT |
|  |  | - Corrected the gray box example for Distribution Loss Factor Code (REF~AQ) |
|  |  | - Corrected the spelling of Southwest Power Pool on REF~SR |
|  |  | - Changed “Standard Interconnection Point” to “Station ID” on REF~TD~REFSPL |
|  |  | - Corrected examples on the NM1 segments |
|  |  | - Corrected gray box example on REF~46 Old Meter Number |
|  |  | - Added decimal example to gray box for REF~4P Meter Multiplier |
|  |  | - Corrected gray box example on REF~Q2 Start Meter Read |
|  |  | - Added the descriptions to REF~Q3 C04002 which were omitted prior |
|  |  | - Clarified requirements for meter additions, meter exchanges or meter removals in NM1 Loop REF segments |
|  |  | - Added REF~TD Reason for Change code REF~TD~REFPTC |
| August 3, 2001 Version 1.4 |  | The following changes were made: |
|  |  | - Per Change Control 2001-047, added codes to REF~PRT for unmetered service types: - “ED” - Electronic Device - “CU” -Cat Unit - “BS” -Bus Shelters - “TR” – Transceiver - “PO” -Phone Outlet - “PA” - Power Analog node - “PB” - Phone Booth - “AN” – Antenna “OT” - Other Unmetered |
|  |  | - Per Change Control 2001-109, delete the words “addition” & “retirement” on the transaction set page in the bullet from ERCOT to current CR, etc. as ERCOT does not forward the 814_20 to the CR in this situation |
|  |  | - Per Change Control 2001-114: - remove the REF segment Meter Service Voltage. - remove the Reason for Change “REF4L – Change Meter Service Voltage” code from the REF ~TD segment & the REF~4L in the example, change SE01 value. |
|  |  | - Per Change Control 2001-116: - Add sentence to REF and REF01 of Load Profile gray box: “The load profile is made up of the following codes, with mandatory underscores between each: Profile Group and Segment Code, Weather Zone Code, Meter Data Type Code, Weather Sensitivity Code, TOU Schedule Code. The value shall correspond to an ERCOT-defined Load Profile that is published on the ERCOT website.” - Change the example in the gray box of the REF~LO from REF~LO~GS to REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOU - Change the example in the gray box on the REF~SPL from REF~SPL~~MESA SUBSTATION to REF~SPL~~WEST - Change Example #1 load profile to REF~LO~RESLOWR_WEST_NIDR_NWS_TOU01 - Change Example #1 station id to REF~SPL~~WEST - Change Example #2 load profile to REF~LO~ RESHIWR_WEST_NIDR_NWS_TOU09 |
|  |  | - Per Change Control 2001-120: - Change explanation in REF*Q2 gray box from “Meter Addition: Required” to “Meter Addition: Required when adding meters. Not required when adding unmetered service devices.” - Change explanation in REF*Q3 gray box from “Meter Removal: Required” to “Meter Removal: Required when removing meters. Not required when removing unmetered service devices.” |
|  |  | - Per Change Control 2001-123 |
|  |  | - Add note to N1, N2 and N4: When submitting a change for the customer loop, service address, city and state and zip code must be provided. If the service address, city, state and zip data are not provided, they will be replaced with blanks in the ERCOT system. |
|  |  | - Per Change Control 2001-124 |
|  |  | - Gray box on NM1 loop clarifiying: When replacing two meters with one meter, send two NM1 loops: one NM1 loop removes one of the existing meters and another NM1 loop exchanges the other existing meter. - Add another example showing how to exchange two existing meters for one new meter - Renumber examples |
|  |  | - Per Change Control 2001-149 |
|  |  | - Delete the REF where REF01=KK (Wholesale Meter Point) segment. ERCOT does not permit electronic establishment or updating of Wholesale Delivery points. Paperwork must be submitted in order to establish these points. |
|  |  | - Remove the Reason for Change code REFKK from the REF~TD |
|  |  | - Modify gray box in REF~4P. Ref. 2001-118 |
|  |  | - Modified example in REF~IX. Changed from K1MON to KHMON |
|  |  | - Fixed example in gray box of REF~MT from “REF~IX~6.0~TU^51” to “REF~IX~6.0~KHMON~TU^51” |
|  |  | Added REDLINES from CHANGE CONTROLS: - 11/14/01 2002-252 02/22/02 - 02/22/02 2002-277 02/22/02 2001-219 – 02/27/02 – TEMP meter indicator in service address |
| June 17, 2002 Version 1.5 |  | The following changes were made: |
|  |  | - Remove gray box QTR Quarterly from REF02 Meter Type. Ref.2001-179 |
| 7/16/02 |  | - Added clarification to REF03 of the REF~PRT Ref: 2001-215 |
|  |  | - Update gray box for Example REF~TD~REF4P . Ref. 2001-245 |
|  |  | - Removed “9” as DUNs option for ERCOT. Ref: 2002-267 |
|  |  | - Add Special Needs REF~SUsegment, Example 6 of 7, and code in REF~TD for Special Needs. Ref.2002-258 |
|  |  | - Added examples for special needs indicator. Ref:2002.307 |
|  |  | - Applied Change Control 2002-322, Add Language to the grey boxes as shown in the below Redline 814_20 NM1 Meter Level Information Segment. |
| 8/5/02 |  | - Corrected Typo in REF~SU Ref: 2002-338 |
| 8/5/02 |  | Change Control 2002-353 – Corrected Implementation guide “Summary of Changes” by cleaning up previously missed or inaccurate additions to Summary of Changes: |
|  |  | - Change Control 2002-325 – Clean-up Change Control for the 2001-179. Modified language in Change Control Log to “Remove gray box QTR Quarterly from REF02 Meter Type. Ref.2001-179.” This was previously missed at time of update of implementation guide and is needed to reflect the approved change control. |
|  |  | - Change Control 2002-327 – Clean-up Change Control for the 2001-215. Added clarification to REF03 of the REF~PRT. This was previously missed at time of update of implementation guide and is needed to reflect the approved change control. |
|  |  | - Change Control 2002-353 – Updated Change Control Log to add approved Version 1.5 Clean-up Change Controls 2002-325 and 2002-327. These were previously missed at time of update of implementation guide and are needed to reflect the approved cleanup change controls. |
| 5/29/03 |  | - Change Control 2003-502 Make the Start Date and Eligibility Date both Optional from the TDSP |
| 5/29/03 |  | - Change Control 2003-505 Update the transaction flow in the 814_20 to reflect ERCOT’s system changes to only send one 814_20 when the New CR is also the Current CR |
| May 27th, 2003 Version 1.6 |  | The following changes were made: |
| 5/27/03 |  | Change Control 2002-386 – Removed D-U-N-S+4 from the N104 gray box in the N1~AY loop |
| 5/29/03 |  | Change Control 2002-427 Add note to the Meter Multiplier (REF~4P), Number of Dials (REF~IX) and the Meter Type (REF~MT) that the segments are not used for unmetered devices |
| 5/29/03 |  | Change Control 2002-442 Update example #5 of 7 |
| 5/29/03 |  | Change Control 2002-452 Remove references in the Document Flow that indicate the 814_20 is sent to and the 814_21 is sent from the CSA CR |
| 5/29/03 |  | Change Control 2002-467 Include how to use section for the 814_20 transaction |
| 5/29/03 |  | Change Control 2003-505 Update the transaction flow in the 814_20 to reflect ERCOT’s system changes to only send one 814_20 when the New CR is also the Current CR |
| 5/30/03 |  | Change Control 2003-513 The 814_20 would need a new REF segment for Competitive Meter Owner Type at the NM1 Meter level loop, and REF01 populated with the code “0P” Owner Type to indicate that the Customer, CR, Third Party or TDSP is the owner. |
| 1/29/04 |  | Change Control 2003-552 Verbiage contained in the REF~0P gray box indicates incorrectly that this segment is required any time a meter is added or exchanged on an ESI ID |
| July 21st, 2003 Version 2.0 |  |  |
| 8/15/03 |  | Change Control 2003-547 – Please include the Move-In/Move-Out Task Force (MIMO) High Level Requirement Specification as part of the Baseline for Version 2.0 |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O = Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |
|  | 060 | N3 | Address Information | O | 2 |  |  |  |  |  |  |  |  |
|  | 070 | N4 | Geographic Location | O | 1 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 040 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - NM1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 080 | NM1 | Individual or Organizational Name | O | 1 |  | n2 |  |  |  |  |  |  |
|  | 130 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 140 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

2.	The NM1 loop is used to identify the parties associated with the individual line item (LIN), such as an individual consumer in a consolidated third party Consumer Service Provider transaction.

Business Process Overview

| Overview |  | The 814_20 how to use guide contains the following sections: - Create ESI ID - Maintaining ESI ID - Reason for Change General Information - Settlement System Maintained Data at ERCOT - Service Address Maintain ESI ID - NM1 Loop Maintain ESI ID - Updating Information within the NM1 Loop - NM1 Reason for Change - Retire ESI ID - Out of Sequence 814_20 |
| --- | --- | --- |
| Create ESI ID |  | In addition to the ST, BGN, N1 (for TDSP and ERCOT), and the LIN segments, the following segments must also be sent on a Create ESI ID: - Customer (N1~8R – N1 loop for Service Address) - Address for Service Address (N3 and N4) - ASI = 021 (Create) - REF~AQ (Distribution Loss Factor Code) - REF~PTC (Premise Type) - REF~Q5 (ESI ID) - REF~SPL (Station ID) - REF~SR (Service Relationship) - DTM~196 (Start Date for ESI ID) - DTM~307 (Eligibility Date) - NM1~MQ (Meter Level Info) - REF~LO (Load Profile) - REF~TZ (Meter Reading Cycle – required if meter is read on cycle) - DTM~313 (Meter Cycle by Day of the Month – required if meter is read on a specific day of the month) NOTE: Either the REF~TZ or the DTM~313 may be sent, but not both. Since ERCOT does not store meter numbers or unmetered device information on the Create ESI ID transaction, the NM1 (Meter Level Information) must always be populated as: NM1~MQ~3~~~~~~93~ALL Note: An ESI ID value (REF~Q5) cannot be updated once it is established. If the TDSP should send an erroneous value for the ESI ID when they sent the Create ESI ID transaction, the TDSP will have to retire the ESI ID in error and submit a new Create ESI ID with the correct ESI ID value. |
|  |  | Recipients of Create ESI ID When a new ESI ID is created by a TDSP, the 814_20 will be sent to ERCOT. Since there is no Current CR or Pending CR, ERCOT will not forward the 814_20 to any CRs. CRs should note that for new construction, a TDSP may retire the temporary ESI ID when the construction is completed and create a new ESI ID for the permanent structure, though both ESI IDs could have the same physical service address. |
| Maintaining ESI ID |  | Updating Information Prior to Energizing the ESI ID for the first time. When an ESI ID is established, a TDSP may not have all of Metering information needed by the CR. The information may not be available at the time the TDSP responds to a move-in request with the 814_04. ERCOT recommends that prior to 867_04 being generated, key information should be sent to ERCOT for the ESI ID. This includes: - Meter information such as meter number, meter multiplier, and meter type - Unmetered information if the ESI ID has unmetered devices - TDSP Rate Class and TDSP Rate Subclass In addition, any fields that the TDSP initially defaulted and wants to change should be updated using the 814_20 Maintain prior to submitting the 867_04. ERCOT recommends that the TDSP send an Effective Date of Change on an 814_20 Maintain containing this information, of at least one day prior to the Actual Switch Date (DTM02) provided on the 867_04. Note: Some TDSPs may not have the information available prior to the time the 867_04 is generated. |
|  |  | Recipients of Maintain ESI ID Transactions When a TDSP submits an 814_20 Maintain ESI ID, ERCOT will process the transaction and load the applicable data into their systems. If the data is successfully loaded, ERCOT will respond back to the TDSP with an 814_21 Accept transaction. After sending the TDSP the Accept response on the 814_21, ERCOT will forward the 814_20 transaction to the Current CR and Pending CR if applicable. Current CR – The Current CR is the CR of Record for the ESI ID at the time the 814_20 is sent by ERCOT. Pending CR – is any CR who has a move-in, CSA move-in, or switch in process. In the case of the CSA move-in, the CSA CR may not have yet received the 814_22 transaction. Note: If the Date of Change is a date in the past, ERCOT does not do any validations to determine who was the CR of Record at that date or the time between the Effective Date of Change and the current date. A CR who is no longer the current CR or the pending CR will not receive the 814_20, though they may have been the CR of record on the effective date of change. |
|  |  | Rejecting the Maintain ESI ID If the CR sends an 814_21 Reject response to ERCOT, the 814_21 Reject response is not forwarded to the TDSP. It is incumbent on the CR to contact the TDSP and/or ERCOT to determine how they will correct the problem. Otherwise, the market participants are out of sync. ERCOT or the TDSP may need to resend a corrected 814_20. |
| Reason for Change General Information |  | Within the 814_20 there are two REF~TD (Reason for Change) segments. The first REF~TD segment is at the ESI ID level. This segment will be populated if any of the following segments are included in the Maintain ESI ID transaction: - N1~8R Loop (N1 – Customer, N3 Address Information and N4 Geographical Information) - REF~AQ (Distribution Loss Factor) - REF~PTC (Premise Type) - REF~SPL (Station ID) - REF~SR (Power Region) - REF~SU (Special Needs Indicator) The Reason for Change code DTM307 Change Pilot Eligibility Date IOU TDSPs but may be used for MC TDSPs, if the MCTDSP chooses to have a pilot prior to full retail open access. The second REF~TD segment will occur within the NM1 loop and is utilized to change meter level information. There may or may not be a REF~TD segment(s) if the NM101 is MA (Meter Addition) or MX (Meter Exchange). There will always be at least one REF~TD if the NM101 is MQ (Meter Information). The REF~TD is not used within the NM1 loop if the NM101 is MR (Meter Removal). For more information on the combination the Reason for Change codes, which can be used within the NM1 loop refer to the Error: Reference source not found section. |
| Settlement System Maintained Data at ERCOT |  | Updating an energized ESI ID (ERCOT TDSP) Settlement System maintained fields at ERCOT. For ERCOT TDSPs there are five fields on the 814_20 that can only be updated on a meter read date if the ESI ID is energized. These fields are the: - N403 Postal Code (ZIP Code) - Distribution Loss Factor Code (REF02 in the REF~AQ) - Station ID (REF02 in the REF~SPL) - Power Region (REF02 in the REF~SR) - Load Profile (REF02 in the REF~LO) The meter read date is based on the Service Period End Date provided on an 867_03 transaction. How the date is determined for IDR meters is different from Non IDR Meters and Unmetered Devices. For IDR ESI IDs the effective date of change is one day greater than the Service Period End Date. Since the IDR data ends at 23:59:59 the Effective Date of Change should begin on a full day, which would be 00:00. - For example, if the Service Period End Date and Time on the Monthly Usage867_03 is 20020915 23:59:59, the Effective Date of Change on the 814_20 should be 20020916. For Non-IDR and Unmetered ESI IDs the Effective Date of Change should match the Service Period End Date of the 867_03. - For example, if the Service Period End Date is 20020915, the Effective Date of Change on the 814_20 should be 20020915. These five fields can be updated at any time the ESI ID is de-energized. For an energized ESI ID, CRs need to be aware that because these fields can only be updated on a meter read date, there may be instances where the 867_03 meter exchange date does not match the Effective Date of Change on the 814_20 Meter Exchange transaction. The differences can occur if the meter exchange happened in the middle of the meter read period and the meter exchange resulted in a change to the Load Profile. Note: The requirement for matching the Effective Date of Change to a Meter Read Service Period End Date does not apply to non-ERCOT TDSPs. Since ERCOT does not store the Non-ERCOT TDSP’s meter reads they are unable to apply the validation rule to Non-ERCOT TDSP’s ESI IDs. If an ERCOT TDSP tries to update one, or all, of these fields on a start meter read date (867_04) and the ESI ID had been previously de-energized, ERCOT will reject the 814_20. The 867_04 is not loaded into the settlement system at ERCOT. The rejection will occur because the ESI ID is energized on the date of the initial meter read date, however, ERCOT does not have a meter read within the settlement system to validate against the 814_20. Document will need updated when SIR 7191 is implemented. |
| Service Address Maintain ESI ID |  | The Service Address (N1~8R) consists of three segments - N1 Customer - N3 Address Information - N4 Geographic Location For the N102 free-form name, the TDSP should use a default value of X, PREMISE, or CUSTOMER. The TDSP should not provide the actual customer name they have on record for the ESI ID since the 814_20 will be forwarded by ERCOT to the current CR and all pending CRs. This will prevent CRs from receiving customer information associated to another CR. If the TDSP is updating the N3 segment the information should be a complete replacement of the N3 information previously sent. - For example, if the TDSP wants to update the N301 field and keep the existing information in the N302 field the TDSP will need to send both the N301 and N302 even though the N302 is not changing. The need to send both fields is that ERCOT and the CR cannot determine if the TDSP intended to eliminate the N302 information or keep the existing information. If any field(s) is changing in the N1, N3, or N4 segment of the N1~8R loop, all three segments are required in the Maintain ESI ID transaction. |
| NM1 Loop Maintain ESI ID |  | Within a single 814_20 transaction there can be meter additions, meter exchanges and meter removals. Each will have a separate NM1 Loop. There are four codes available within the NM101. - MA – this code is to be used when adding either a new meter to an existing ESI ID or adding a new unmetered device type to an existing ESI ID. This code will not be used to adjust the quantity of an unmetered device type for an ESI ID. - MQ – for meters, this code is used for everything except adding, removing or exchanging a meter. For unmetered devices, this code is used for everything except adding or removing an unmetered device type. For example, changes to the quantity of unmetered device types should use the MQ code. - MR – this code is to be used when removing an existing meter or an unmetered device type from an ESI ID. For unmetered devices this code should only be used to remove the total quantity of an unmetered device type. - MX – this code is to be used when exchanging a meter. The code should not be used for removing some of a specific unmetered device type. The code can be used to remove the total count of a specific unmetered device type. If an unmetered device type needs to be corrected, i.e. the wrong device type was previously sent, there should be two unmetered NM1 loops. The first loop will remove the incorrect device type (MR code). The second loop will add the correct device type (MA code). |
| Updating Information within the NM1 Loop |  | Maintaining NM1 Information – Overview The 814_20 transaction does not support adding, changing, or removing detail information for a specific meter or for unmetered devices. - For example, if the original 814_20, which added a new meter had more than one Meter Multiplier (REF~4P) segments, the transaction does not provide a way to send a REF~4P segment to correct the one in error. Meter Multiplier and Number of Dials Due to the limitations of the transaction, if the TDSP needs to add, correct, or remove one segment they will have to send all occurrences of that segment even though some of the occurrence are not changing. The CRs upon receipt of the Meter Multiplier (REF~4P) and/or Number of Dials (REF~IX) segments will do a complete replace of the segment information based on the information provided by the TDSP. TDSP Rate Class and TDSP Rate Subclass The updates to TDSP Rate Class and TDSP Rate Subclass are similar to the Meter Multiplier and Number in that there is no clear method to determine if a TDSP Rate Class or TDSP Rate Subclass is being added or replaced. Nor is there a clear method for removing a TDSP Rate Class or TDSP Rate Subclass. A general assumption is a meter associated to an ESI ID or unmetered ESI ID will always have at least one TDSP Rate Class. Therefore, - When the TDSP sends the TDSP Rate Class and TDSP Rate Subclass the information provided will replace the information provided to the CR in an earlier transaction. - If the TDSP wants to remove the TDSP Rate Subclass data, they will send only the TDSP Rate Class information for the ESI ID. This will be the indication to the CR that the TDSP Rate Subclass information is being deleted for the meter or unmetered devices. Meter Reading Cycle and Meter Cycle by Day of the Month The Meter Reading Cycle (REF~TZ) and Meter Cycle by Day of the Month (DTM~313) are mutually exclusive for meters and unmetered devices. If the TDSP previously sent a DTM~313 segment and then sends a REF~TZ a meter number, the CR should update their system to identify the meter is being changed from reading on a specific day of the month to reading based on a meter reading cycle. For an unmetered ESI ID all devices must have the same Meter Reading Cycle or Meter Cycle by Day of the Month. For metered ESI IDs where there are multiple meters each meter could potentially have a different Meter Reading Cycle or Meter Cycle by Day of the Month. Load Profile An ESI ID may have only one active load profile. Though the transaction can support each meter having a different load profile, the market does not support having multiple load profiles for the ESI ID. Meter Type There will be only one occurrence of the Meter Type (REF~MT) segment for a meter number. If this segment is being updated the Meter Multiplier (REF~4P) and Number of Dials (REF~IX) segments will also be present on the transaction. The reasons why the Meter Multiplier and Number of Dials will be present are: - If the Meter Type is being changed from a specific type of consumption /interval period (i.e. KHMON) to “COMBO”, there are additional Meter Multiplier and Number of Dial segments that need to be included. - If the Meter Type is being changed from COMBO to a specific type of consumption/interval period, there are Meter Multipliers and Number of Dial segments that need to be removed. - If the Meter Type is being changed from one specific type of consumption/interval period the Meter Type previously provided to the CR needs to be replaced with the new Meter Type information in the Meter Multiplier and Number of Dials. Unmetered Service Type When the NM101 is equal to MQ (Meter Information) the Unmetered Service Type will be populated if the TDSP wants to change either the free form Description or the Number of Unmetered Devices associated to the Unmetered Device Type. The Number of Unmetered Devices should always represent the total number of devices. - For example, if an ESI ID had 100 and the TDSP was adding 2 more devices the value in C04002 of the REF~PRT should be 102. If two devices were being removed the value in C04002 of the REF~PRT should be 98. |
| NM1 Reason for Change |  | NM101 = MA (Meter Addition) For a metered device the following codes may be included in the REF~TD segment if the corresponding REF or DTM segment are also included in the NM1 Loop: - DTM313 - REFLO - REFNH - REFPR - REFTZ For an unmetered ESI ID the following codes may be included in the REF~TD segment if the corresponding REF or DTM segment are also included in the NM1 Loop: - DTM313 - REFLO - REFNH - REFPR - REFTZ For an unmetered ESI ID where the NM101 is populated the REFPRT code will always be populated. For meters and unmetered ESI IDs there should never be a both a DTM313 and REFTZ reason for change. NM101 = MX (Meter Exchange) For a metered device the following codes may be included in the REF~TD segment if the corresponding REF or DTM segment are also included in the NM1 Loop: - DTM313 - REFLO - REFNH - REFPR - REFTZ For meters and ESI IDs there should never be both a DTM313 and REFTZ reason for change. NM101 = MQ (Meter Information) For a metered device the following codes may be included in the REF~TD segment if the corresponding REF or DTM segment are also included in the NM1 Loop: - DTM313 - REF4P - REFIX - REFLO - REFMT - REFNH - REFPR - REFTZ Note if the REFMT reason for change is populated, the NM1 Loop should also have Reason for Change Codes REF4P and REFIX. For an unmetered ESI ID the following codes may be included in the REF~TD segment if the corresponding REF or DTM segment are also included in the NM1 Loop: - DTM313 - REFLO - REFNH - REFPR - REFTZ For an unmetered ESI ID where the NM101 is populated the REFPRT code will always be populated. For meters and unmetered ESI IDs there should never be both a DTM313 and REFTZ reason for change. In the case where the TDSP is deleting a TDSP Rate Subclass by only sending a TDSP Rate Class and not sending a new TDSP Rate Subclass the REFPR Reason for Change will not be provided on the transaction. When the TDSP is changing from a Meter Reading Cycle to Meter Read by Day of the Month, the DTM313 code will be provided. The REFTZ code will not be provided since the REF~TZ segment was not included in the NM1 Loop. Likewise, when the TDSP is changing from Meter Read by Day of the Month to Meter Reading Cycle, the REFTZ code will be provided. The DTM313 will not be provided since the DTM~313 segment was not included in the NM1 Loop. |
| Retire ESI ID |  | Generating Retire ESI ID A TDSP cannot retire an ESI ID if ERCOT has a Current CR of Record or Pending CR associated to the ESI ID. If the TDSP submits an 814_20 Retire ESI ID and there is a Current CR or Pending CR, ERCOT will send an 814_21 Reject Response back to the TDSP. A TDSP may retire an ESI ID without sending a Maintain ESI ID transaction to remove all metered or unmetered devices from the ESI ID. A TDSP may retire an ESI ID if there is a CSA associated to the ESI ID, provided the ESI ID is not currently energized. This can occur if the CSA was established after the ESI ID became de-energized. A TDSP can not retire an ESI ID prior to stop read date, unless the meter has never been energized at ERCOT. Note: There is the possibility that the TDSP does not know about the Pending CR, at the time they submit the 814_20 Retire ESI ID. This situation would occur if ERCOT received an 814_16 move-in request for a de-energized ESI ID and had not yet sent the 814_03 to the TDSP before the TDSP sent the 814_20. In these situations ERCOT will send an 814_21 Reject transaction to the TDSP. |
|  |  | Recipients of Retire ESI ID When an ESI ID is retired by a TDSP, the 814_20 will be sent to ERCOT. Since there is no Current CR or Pending CR, ERCOT will not forward the 814_20 to any CRs. |
| Out of Sequence 814_20 |  | There are some situations where a CR may receive an 814_20 prior to receipt information needed to validate whether the 814_20 should be accepted or rejected. In these situations the CR should respond back to ERCOT with an 814_21 accept response. The following outlines when this situation can occur. Receiving 814_20 prior to receipt of 814_05 Based on timing issues within the flow of transactions there is the possibility that a CR may receive an 814_20 Create/Maintain/Retire ESI ID Request transaction though they have not received the 814_05 accept response from ERCOT. When ERCOT receives the 814_01 (Enrollment Request) or an 814_16 (Move-In Request) from a CR, ERCOT will generate an 814_03 for the ESI ID and send it to the TDSP. At the point the 814_03 is generated by ERCOT, the CR becomes the pending CR. If ERCOT receives and processes an 814_20 for the ESI ID before they receive the 814_04 from the TDSP or before they send the 814_05 to the CR, ERCOT will forward the 814_20 to the CR since they are the pending CR. Note: The CR may not have enough information when they receive the 814_20 Create/Maintain/Retire ESI ID Request to validate the information in the transaction is accurate or inaccurate. One example is the CR receives a meter exchange and it has not received an 814_05 for the ESI ID. In this example, the CR cannot validate the old meter number on the exchange. If the CR received an 814_28 Completed Unexecuatable or Permit Required indicating a permit is required prior to the TDSP sending the 814_04 transaction the length of time when this situation occurs is increased. Receiving 814_20 prior to receipt of 814_22 Based on timing issues within the flow of transactions there is the possibility that a CSA CR may receive an 814_20 Create/Maintain/Retire ESI ID Request transaction though they are not currently serving the ESI ID. They have not yet been notified that the current customer is moving out of the ESI ID and that the CSA move-in will occur on a specific date via the 814_22 Continuous Service Agreement (CSA) Move-In Request process. When ERCOT receives the 814_24 Move Out Request from a CR and there is a CSA in effect for the ESI ID, ERCOT will generate an 814_03 CSA Move-In for the ESI ID and send it to the TDSP. At the point the 814_03 is generated by ERCOT, the CSA CR becomes the pending CR. If ERCOT receives and processes an 814_20 for the ESI ID before they receive the 814_04 from the TDSP or before they sent the 814_22 to the CSA CR, ERCOT will forward the 814_20 to the CSA CR since they are the pending CR. Note: The CSA CR may not have enough information when they receive the 814_20 Create/Maintain/Retire ESI ID Request to validate the information in the transaction is accurate or inaccurate. One example is where the CSA CR receives a meter exchange and it has not received an 814_22 for the ESI ID. In this example, the CSA CR cannot validate the old meter number on the exchange. |

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~13~200104011956531~20010401~~~~~20 BGN~13~200104021200719~20010402~~~200104011956531~~20 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Request |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Dep | BGN06 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Refers to the BGN02 of the 814_20 Create/Maintain/Retire ESI ID Request when ERCOT forwards the request. |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |
|  |  |  |  | 20 |  | Decree Recall |  |  |  |  |  |
|  |  |  |  |  |  | Indicates Texas SET Transaction 814_20 |  |  |  |  |  |

Segment:	N1 Name (Customer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used When submitting a change for the customer loop, service address, city and state and zip code must be provided. If the service address, city, state and zip data are not provided, ERCOT will reject the transaction. |
| --- | --- | --- |
|  |  | N1~8R~PREMISE N1~8R~X |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8R |  | Consumer Service Provider (CSP) Customer |  |  |  |  |  |
|  |  |  |  |  |  | Customer |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | This name field must be used in order to populate the service address in N3 and N4. The sender can determine what they put in this field. |  |  |  |  |  |  |  |  |

Segment:	N3 Address Information (Customer Service Address)

Position:	060

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify the location of the named party

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used When submitting a change for the customer loop, service address, city and state and zip code must be provided. If the service address, city, state and zip data are not provided, ERCOT will reject the transaction. |
| --- | --- | --- |
|  |  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW EX: For "temporary/construction service" TEMP will always be at the end of the address field segment and would be populated as follows. N3~123 N MAIN ST TEMP or N3~12345 WEST JOHN QUINCY ADAMS BOULEVARD NW BUILDING 17 S~UITE 164-A TEMP |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N301 | 166 | Address Information | M |  | AN 1/55 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address |  |  |  |  |
| Dep | N302 | 166 | Address Information | O |  | AN 1/55 |  |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address Overflow Condition: If there is an overflow, it must be sent. |  |  |  |  |

Segment:	N4 Geographic Location (Customer Service Address)

Position:	070

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used When submitting a change for the customer loop, service address, city and state and zip code must be provided. If the service address, city, state and zip data are not provided, ERCOT will reject the transaction. |
| --- | --- | --- |
|  |  | N4~ANYTOWN~TX~78111 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N401 | 19 | City Name | O |  | AN 2/30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form text for city name |  |  |  |  |
| Must Use | N402 | 156 | State or Province Code | O |  | ID 2/2 |  |
|  |  |  | Code (Standard State/Province) as defined by appropriate government agency |  |  |  |  |
| Must Use | N403 | 116 | Postal Code | O |  | ID 3/15 |  |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |
|  |  |  | Postal codes will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411~~41 (TDSP to ERCOT) N1~8S~TDSP COMPANY~1~007909411 (ERCOT to CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Dep | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  | TDSP to ERCOT: Required ERCOT to CR: Not Used |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~40 N1~AY~ERCOT~1~183529049~~41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |
|  |  |  |  |  |  | ERCOT |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT D-U-N-S Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required if CR is the receiver, otherwise not used. |
| --- | --- | --- |
|  |  | N1~SJ~CR NAME~1~007909422~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | LIN~1~SH~EL~SH~MP |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  | O |  | AN 1/20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  | M |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  | M |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  | Commodity |  |  |  |  |  |  |  |  |
|  |  |  |  | EL |  | Electric Service |  |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | IN |  | Initial Conversion or Opt-In |  |  |  |  |  |
|  |  |  |  |  |  | Submission of premise data to ERCOT for initial conversion or opt in. |  |  |  |  |  |
|  |  |  |  | MP |  | Maintain Premise |  |  |  |  |  |
|  |  |  |  |  |  | Indicates this transaction relates to the establishment, maintenance, and termination of premise data. |  |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~7~001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  | M |  | ID 1/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  |  | 7 |  | Request |  |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  | M |  | ID 3/3 |  |  |
|  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |  |
|  |  |  |  | 001 |  | Change |  |  |  |  |  |
|  |  |  |  |  |  | Change ESI ID Information |  |  |  |  |  |
|  |  |  |  | 002 |  | Delete |  |  |  |  |  |
|  |  |  |  |  |  | Retire ESI ID |  |  |  |  |  |
|  |  |  |  | 021 |  | Addition |  |  |  |  |  |
|  |  |  |  |  |  | Create ESI ID |  |  |  |  |  |

Segment:	REF Reference Identification (Distribution Loss Factor Code)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The Distribution Loss Factor Code is used to identify the related loss factor value that is used in settlement. The code must be contained in a loss factor table that the TDSP has provided to ERCOT. It is usually related to the delivery voltage or rate class. It is not the same as the transformer loss factor used when the TDSP meters service at a different point than it delivers service. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
|  |  | REF~AQ~T |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | AQ |  | Access Code |  |  |  |  |  |
|  |  |  |  |  |  | Distribution Loss Factor Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | With the exception of code "T", each Distribution Company will post what each of the next 5 codes represent and how they are applied. |  |  |  |  |  |  |  |  |
|  |  |  |  | A |  |  |  |  |  |  |  |
|  |  |  |  | B |  |  |  |  |  |  |  |
|  |  |  |  | C |  |  |  |  |  |  |  |
|  |  |  |  | D |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |
|  |  |  |  | T |  | Transmission Only |  |  |  |  |  |

Segment:	REF Reference Identification (Energized/De-Energized)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Conversion Request: Required Create ESI ID Request: Not Used Change ESI ID Information Request: Not Used Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | REF~PHC~E |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PHC |  | Process Handling Code |  |  |  |  |  |
|  |  |  |  |  |  | Indicates that the ESI ID is energized or de-energized |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | D |  | De-Energized |  |  |  |  |  |
|  |  |  |  | E |  | Energized |  |  |  |  |  |

Segment:	REF Reference Identification (Premise Type)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | REF~PTC~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PTC |  | Patent Type |  |  |  |  |  |
|  |  |  |  |  |  | Premise Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 01 |  | Residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a premise where a customer will reside or an umetered service located at such a premise (e.g. Outdoor Lights) provided it is not a master metered, multi-family facility, or a facility metered as a commercial facility. |  |  |  |  |  |
|  |  |  |  | 02 |  | Small Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand of 1 Megawatt or below (i.e. Where the demand for the previous 12 months is less than or equal to 1 megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |
|  |  |  |  | 03 |  | Large Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand greater than 1 Megawatt (i.e. Where demand for at least 1 month of the previous 12 months is greater than 1 Megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Station ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
|  |  | REF~SPL~~WEST |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SPL |  | Standard Point Location Code (SPLC) |  |  |  |  |  |
|  |  |  |  |  |  | Substation at which the TDSP's distribution system is connected to the transmission grid for the Service Delivery Point (SDP). The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Station ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Special Needs Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used A "Y" in this field means that the customer is either: A residential customer who qualifies through the Residential Critical Care Eligibility Determination Form, as issued by the PUCT. Or A customer for which electric service is considered crucial for the protection and maintenance of public safety pursuant to Subst. Rules 25.52 and 25.53. Or An industrial customer for whom an interruption or suspension of service will create a dangerous or life threatening condition at the customers premise. |
| --- | --- | --- |
|  |  | REF~SU~Y |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Special Processing Code |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Indicator |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs are not required |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs are required |  |  |  |  |  |

Segment:	REF Reference Identification (Power Region)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | REF~SR~ERCOT |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SR |  | Sales Responsibility |  |  |  |  |  |
|  |  |  |  |  |  | Service Relationship |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | ERCOT |  | Electric Reliability Council of Texas |  |  |  |  |  |
|  |  |  |  | SERC |  | Southeast Electric Reliability Council |  |  |  |  |  |
|  |  |  |  | SPP |  | Southwest Power Pool |  |  |  |  |  |
|  |  |  |  | WSCC |  | Western States Coordinating Council |  |  |  |  |  |

Segment:	REF Reference Identification (Reason for Change)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | This transaction allows multiple change request on a single 814. If any single change request is invalid, the transaction is rejected in its entirety with appropriate reject reasons. More than one REF~TD may be present. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required change is at an ESI ID level (Header or LIN Loop), otherwise not used. Retire ESI ID Request: Not Used |
|  |  | REF~TD~N18R |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | TD |  | Reason for Change |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  |  | DTM307 |  | Change Pilot Eligibility Date |  |  |  |  |
|  |  |  |  | N18R |  | Change Customer Name/Service Address |  |  |  |  |
|  |  |  |  | REFAQ |  | Change Distribution Loss Factor Code |  |  |  |  |
|  |  |  |  | REFPTC |  | Change Premise Type |  |  |  |  |
|  |  |  |  | REFSPL |  | Change Station ID |  |  |  |  |
|  |  |  |  | REFSR |  | Change Power Region |  |  |  |  |
|  |  |  |  | REFSU |  | Change Special Needs Indicator |  |  |  |  |

Segment:	DTM Date/Time Reference (Effective Date of Change)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | DTM~152~20010602 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 152 |  | Effective Date of Change |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Start Date for ESI ID)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Optional Conversion/Create ESI ID Request: Used if available. ERCOT defaults to a value of processing date minus 180 days if the TDSP chooses to leave the segment out of the transaction. TDSPs must populate the Create Date to effectuate a clean-up effort that is greater than 180 days in the past. ESI ID Information Request: Not Used Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | DTM~196~20010601 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 196 |  | Start |  |  |  |  |  |
|  |  |  |  |  |  | Start date for ESI ID |  |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (End Date for ESI ID)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Not Used Retire ESI ID Request: Required |
| --- | --- | --- |
|  |  | DTM~197~20010602 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 197 |  | End |  |  |  |  |  |
|  |  |  |  |  |  | End date for an ESI ID |  |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Eligibility Date)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Optional Conversion/Create ESI ID Request: only used for pilot projects and/or Opt In. Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | DTM~307~20010602 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 307 |  | Eligibility |  |  |  |  |  |
|  |  |  |  |  |  | Date the ESI ID is eligible for participation in the Market Pilot and/or for Opt In. |  |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |

Segment:	NM1 Individual or Organizational Name (Meter Level Information)

Position:	080

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To supply the full name of an individual or organizational entity

Syntax Notes:	1	If either NM108 or NM109 is present, then the other is required.

2	If NM111 is present, then NM110 is required.

Semantic Notes:	1	NM102 qualifies NM103.

Comments:	1	NM110 and NM111 further define the type of entity in NM101.

| Notes: |  | Conversion/Create ESI ID Request: Required with "ALL" in NM109 Change ESI ID Information Request: Required if changing is at a meter level Retire ESI ID Request: Not Used The code "All" in the NM109 can be used with a Change Transaction. When replacing two meters with one meter, send two NM1 loops: one NM1 loop removes one of the existing meters and another NM1 loop exchanges the other existing meter. |
| --- | --- | --- |
|  |  | NM1~MQ~3~~~~~~93~ALL (Conversion/Create Example) NM1~MX~3~~~~~~32~GE1203948 (Change Example) NM1~MQ~3~~~~~~93~ALL (Change Example) NM1~MQ~3~~~~~~93~UNMETERED (Change Example) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | NM101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | MA |  | Party for whom Item is Ultimately Intended |  |  |  |  |  |
|  |  |  |  |  |  | Meter Addition - use when adding a meter or unmetered devices to an existing ESI ID |  |  |  |  |  |
|  |  |  |  | MQ |  | Metering Location |  |  |  |  |  |
|  |  |  |  |  |  | Meter Information - this code is used for everything except adding, removing or exchanging a specific meter. MQ is used for Conversion/Opt-In. |  |  |  |  |  |
|  |  |  |  | MR |  | Medical Insurance Carrier |  |  |  |  |  |
|  |  |  |  |  |  | Meter Removal - use when removing a meter or unmetered devices from an existing ESI ID |  |  |  |  |  |
|  |  |  |  | MX |  | Juvenile Witness |  |  |  |  |  |
|  |  |  |  |  |  | Meter Exchange - use when exchanging a meter |  |  |  |  |  |
| Must Use | NM102 | 1065 | Entity Type Qualifier |  |  |  | M |  | ID 1/1 |  |  |
|  |  |  | Code qualifying the type of entity |  |  |  |  |  |  |  |  |
|  |  |  |  | 3 |  | Unknown |  |  |  |  |  |
| Must Use | NM108 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 32 |  | Assigned by Property Operator |  |  |  |  |  |
|  |  |  |  |  |  | Meter Number |  |  |  |  |  |
|  |  |  |  | 93 |  | Code assigned by the organization originating the transaction set |  |  |  |  |  |
|  |  |  |  |  |  | This code used when a Meter Number is NOT provided in NM109 (i.e., when NM109 contains "ALL" or "UNMETERED"). |  |  |  |  |  |
| Must Use | NM109 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | Meter Number or, UNMETERED - for Unmetered Services or, ALL - for ALL meters The code "All" can be used with a Change Transaction. For a Meter Addition, this field will contain the new meter number. For a Meter Removal, this field will contain the old meter number. For a Meter Exchange, this field will contain the new meter number and the old meter number will be provided in REF~46. Meter numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Old Meter Number)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required for a Meter Exchange Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Not Used NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Not Used |
| --- | --- | --- |
|  |  | REF~46~1298741GE |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | 46 |  | Old Meter Number |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Old Meter Number |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Multiplier)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~4P segment for each Meter Type, creating the potential for more than one REF~4P segment per NM1 Loop. For instance, if the meter has multiple meter types, and therefore the REF~MT segment is REF~MT~COMBO, there will be multiple REF~4P segments, one to indicate the meter multiplier for each type. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~4P~1200~K1015~TU^41 REF~4P~1~KH015~TU^41 Note: Format of the multiplier could be in the form of a fraction (e.g. 48.78432) Also note that since the Meter Multiplier is required for each Time of Use, this segment is also used to determine which Time of Use will be sent on the 867 transaction. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used This segment will not be provided in the NM1 loop for UNMETERED Services. NM101 = MA (Meter Addition): Required NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the meter multiplier |
|  |  | REF~4P~10~KHMON~TU^41 REF~4P~1~K1MON~TU^41 REF~4P~1.5~K1MON~TU^41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 4P |  | Affiliation Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Multiplier (Ending Reading - Beginning Reading) * Meter Multiplier = Billed Usage |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Meter Multiplier |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note this is a composite data element. Populate C04001 & C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Number of Dials)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~IX segment for each applicable Meter Type, creating the potential for more than one REF~IX segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~IX segments, one to indicate the number of dials for each type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~IX~6.0~KHMON~TU^51 REF~IX~5.1~KHMON~TU^51 |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used This segment will not be provided in the NM1 loop for UNMETERED services or for demand. NM101 = MA (Meter Addition): Required NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the number of dials |
|  |  | REF~IX~6.0~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | IX |  | Item Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of Dials on the Meter displayed as X.Y. The notation X.Y means that the meter has X dials to the left of the decimal point, and Y dials to the right. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of Dials |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Load Profile)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The load profile is made up of the following codes, with mandatory underscores between each: Profile Group and Segment Code, Weather Zone Code, Meter Data Type Code, Weather Sensitivity Code, TOU Schedule Code. The value shall correspond to an ERCOT-defined Load Profile that is published on the ERCOT website. Conversion/Create ESI ID Request: Required Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Optional NM101 = MX (Meter Exchange): Optional NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the Load Profile |
| --- | --- | --- |
|  |  | REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOUREF |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | LO |  | Load Planning Number |  |  |  |  |  |
|  |  |  |  |  |  | Load profile |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Load Profile |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will only be one REF~MT segment for each NM1 loop. If there are multiple meter types on this meter, only one REF~MT will be sent. It will contain the code COMBO. The specific meter types will be identified in REF03 for the attribute segments contained within this loop. For example, REF~IX~6.0~KHMON~TU^51. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used This segment will not be provided in the NM1 loop for UNMETERED services. NM101 = MA (Meter Addition): Required NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the Meter Type |
|  |  | REF~MT~KHMON REF~MT~COMBO |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MT |  | Meter Ticket Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | When REF01 is MT, the meter type is expressed as a five-character field. The first two characters are the type of consumption, the last three characters are the metering interval reported for billing purposes. "COMBO" is used for a meter that records more than one measurement. Valid values can be a combination of the following values: Type of Consumption K1 Kilowatt Demand (kW) K2 Kilovolt Amperes Reactive Demand (kVAR) K3 Kilovolt Amperes Reactive Hour (kVARH) K4 Kilovolt Amperes (kVA) KH Kilowatt Hour (kWh) Metering Interval Reported for Billing Purposes nnn Number of minutes from 001 to 999 DAY Daily MON Monthly For Example: KHMON Kilowatt Hours Per Month K1015 Kilowatt Demand per 15 minute interval Other Valid Codes COMBO This code is used to indicate that the meter has multiple measurements, e.g., one meter that measures both kWh and Demand. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Class)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Optional NM101 = MX (Meter Exchange): Optional NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the Rate Class |
| --- | --- | --- |
|  |  | REF~NH~RS1 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | NH |  | Rate Card Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP rate class or tariff |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Subclass)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Optional NM101 = MX (Meter Exchange): Optional NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the Rate Subclass |
| --- | --- | --- |
|  |  | REF~PR~123 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PR |  | Price Quote Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Subclass - Used to provide further classification of a rate. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Subclass |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Start Meter Read)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~Q2 segment for each applicable Meter Type, creating the potential for more than one REF~Q2 segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~Q2 segments, one to indicate the start read for each meter type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~Q2~12345~KHMON~TU^51 REF~Q2~12555~KHMON~TU^41 |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Required when adding meters. Not required when adding unmetered service devices. NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Not Used |
|  |  | REF~Q2~12345~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q2 |  | Starting Package Number |  |  |  |  |  |
|  |  |  |  |  |  | Start Meter Read |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Start Meter Read for new meter |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (End Meter Read)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~Q3 segment for each applicable Meter Type, creating the potential for more than one REF~Q3 segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~Q3 segments, one to indicate the end read for each meter type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~Q3~99990~KHMON~TU^51 REF~Q3~54681~KHMON~TU^41 |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Not Used NM101 = MX (Meter Exchange): Required NM101 = MR (Meter Removal): Required when removing meters. Not required when removing unmetered service devices. NM101 = MQ (Meter Information): Not Used |
|  |  | REF~Q3~56480~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q3 |  | Ending Package Number |  |  |  |  |  |
|  |  |  |  |  |  | End Meter Read |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | End Meter Read for old meter |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Owner Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | This transaction allows multiple change request on a single 814. If any single change request is invalid, the transaction is rejected in its entirety with appropriate reject reasons. NM101 = MA (Meter Addition): Required if changing meter ownership NM101 = MX (Meter Exchange): Required if changing meter ownership NM101 = MQ (Meter Information): Required if changing meter ownership NM101 = MR (Meter Removal): Not Used |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
|  |  | REF~0P~3 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 0P |  | Real Estate Owned Property Identifier |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Meter Ownership Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | Customer |  |  |  |  |  |
|  |  |  |  | 2 |  | CR |  |  |  |  |  |
|  |  |  |  | 3 |  | Third Party |  |  |  |  |  |
|  |  |  |  | 4 |  | TDSP |  |  |  |  |  |

Segment:	REF Reference Identification (Reason for Change)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | More than one REF~TD may be present. |
| --- | --- | --- |
|  |  | Create ESI ID Request: Not Used Change ESI ID Information Request: Required change is at a meter level (NM1 Loop) Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Required if including a change to Load Profile, TDSP Rate Class or Subclass, or Meter Cycle NM101 = MX (Meter Exchange): Required if including a change to Load Profile, TDSP Rate Class or Subclass, or Meter Cycle NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required |
|  |  | REF~TD~REF4P |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | TD |  | Reason for Change |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  |  | DTM313 |  | Change Meter Cycle by Day of the Month |  |  |  |  |
|  |  |  |  | REF4P |  | Change Meter Multiplier |  |  |  |  |
|  |  |  |  | REFIX |  | Change Number of Dials |  |  |  |  |
|  |  |  |  | REFLO |  | Change Load Profile |  |  |  |  |
|  |  |  |  | REFMT |  | Change Meter Type |  |  |  |  |
|  |  |  |  | REFNH |  | Change TDSP Rate Class |  |  |  |  |
|  |  |  |  | REF0P |  | Change Meter Owner |  |  |  |  |
|  |  |  |  | REFPR |  | Change TDSP Rate Subclass |  |  |  |  |
|  |  |  |  | REFPRT |  | Change Unmetered Service Type |  |  |  |  |
|  |  |  |  | REFTZ |  | Change Meter Cycle |  |  |  |  |

Segment:	REF Reference Identification (Unmetered Service Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~PRT segment for each applicable Unmetered Service Meter Type, creating the potential for more than one REF~PRT segment per NM1 Loop. This segment is used to provide additional information to the CR for the specified Unmetered Service. It describes the type of device which this measurement loop references and can include additional text information which may be useful to the CR (i.e., a specific wattage of a light, additional text information for further clarification, etc.) |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Unmetered Device Addition): Required NM101 = MX: Not Used NM101 = MR (Unmetered Device Removal): Required NM101 = MQ: Not Used |
|  |  | REF~PRT~MV~750~QQ^100 REF~PRT~SD~400 Company Owned~QQ^20 REF~PRT~MV~400 Customer Owned~QQ^15 REF~PRT~MV~Third party maintained facilities~QQ^100 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PRT |  | Product Type |  |  |  |  |  |
|  |  |  |  |  |  | Defined Unmetered Service Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | If a new code is needed, it must be requested via the Change Control process. |  |  |  |  |  |  |  |  |
|  |  |  |  | AN |  | Antenna |  |  |  |  |  |
|  |  |  |  | AR |  | Argon |  |  |  |  |  |
|  |  |  |  | BB |  | Bill Boards |  |  |  |  |  |
|  |  |  |  | BS |  | Bus Shelters |  |  |  |  |  |
|  |  |  |  | CU |  | Cat Unit |  |  |  |  |  |
|  |  |  |  | ED |  | Electronic Device |  |  |  |  |  |
|  |  |  |  | FL |  | Fluorescent |  |  |  |  |  |
|  |  |  |  | HA |  | Historical/Antique |  |  |  |  |  |
|  |  |  |  | IN |  | Incandescent |  |  |  |  |  |
|  |  |  |  | LV |  | Levys |  |  |  |  |  |
|  |  |  |  | MH |  | Metal Halide |  |  |  |  |  |
|  |  |  |  | MV |  | Mercury Vapor |  |  |  |  |  |
|  |  |  |  | OT |  | Other Unmetered |  |  |  |  |  |
|  |  |  |  | PA |  | Power Analog Node |  |  |  |  |  |
|  |  |  |  | PB |  | Phone Booth |  |  |  |  |  |
|  |  |  |  | PO |  | Phone Outlet |  |  |  |  |  |
|  |  |  |  | PS |  | Pump Station |  |  |  |  |  |
|  |  |  |  | RR |  | Rail Road Crossings |  |  |  |  |  |
|  |  |  |  | SD |  | Sodium |  |  |  |  |  |
|  |  |  |  | TL |  | Traffic Lights |  |  |  |  |  |
|  |  |  |  | TR |  | Transceiver |  |  |  |  |  |
|  |  |  |  | WM |  | Wallpacked Mercury Vapor |  |  |  |  |  |
|  |  |  |  | WS |  | Warning Sirens |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Used to provide additional clarification information to the CR for the Unmetered Service when necessary. When applicable, used to provide the specific wattage/lumens for a light. This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | QQ |  | Unit Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of devices |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of unmetered devices of this type. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Reading Cycle)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Meter cycles will be provided on the TDSP's website. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Required if meter is read on a cycle Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Optional NM101 = MX (Meter Exchange): Optional NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if only changing the Meter Reading Cycle |
|  |  | REF~TZ~15 REF~TZ~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TZ |  | Total Cycle Number |  |  |  |  |  |
|  |  |  |  |  |  | Cycle number when the meter will be read. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Metered: Meter Reading Cycle Unmetered: Billing Cycle Must be numeric and only two digits. |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Meter Cycle by Day of the Month)

Position:	140

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Conversion/Create ESI ID Request: Required if meter is read on a specific day of the month Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used NM101 = MA (Meter Addition): Optional NM101 = MX (Meter Exchange): Optional NM101 = MR (Meter Removal): Not Used NM101 = MQ (Meter Information): Required if changing the Meter Cycle by Day of the Month |
| --- | --- | --- |
|  |  | DTM~313~~~~DD~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 313 |  | Cycle |  |  |  |  |  |
|  |  |  |  |  |  | Meter Cycle by Day of the Month |  |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X |  | ID 2/3 |  |  |
|  |  |  | Code indicating the date format, time format, or date and time format |  |  |  |  |  |  |  |  |
|  |  |  |  | DD |  | Day of Month in Numeric Format |  |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X |  | AN 1/35 |  |  |
|  |  |  | Expression of a date, a time, or range of dates, times or dates and times |  |  |  |  |  |  |  |  |
|  |  |  | Day of month that the meter will be read, in DD format (e.g., 01 - 28). |  |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~32~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

814_20 Example #1 of  7

Create ESI ID Request – TDSP to ERCOT

| TDSP submits Create ESI ID Request to ERCOT |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060112301~20010601~~~~~20 |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name – Dummy by Inserting “PREMISE” |
|  | N3~123 MAIN STREET APT#2~BUILDING C |  | ESI ID Service Address |
|  | N4~ANYTOWN~TX~761110001 |  | ESI ID Service Address |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Creating a New ESI ID Request |
|  | ASI~7~021 |  | Request to Create a New ESI ID |
|  | REF~AQ~T |  | Distribution Loss Factor Code |
|  | REF~PTC~01 |  | Premise Type |
|  | REF~Q5~~101234500000000000000000000001000011 |  | ESI ID |
|  | REF~SPL~~ WEST |  | Station ID |
|  | REF~SR~ERCOT |  | Power Region |
|  | DTM~196~20010503 |  | Start Date for ESI ID |
|  | DTM~307~20010602 |  | Eligibility Date |
|  | NM1~MQ~3~~~~~~93~ALL |  | Beginning of Meter Loop, Meter Number |
|  |  | REF~LO~ RESLOWR_WEST_NIDR_NWS_TOU01 | Load Profile |
|  |  | REF~TZ~15 | Meter Reading Cycle |
| SE~20~000000001 |  |  | Number of Segments, Control Number |

814_20 Example #2 of  7

Maintain ESI ID Notification Request – ERCOT to Current CR

| ERCOT forwards meter exchange data the Current CR Example includes a change in Load Profile as a result of the Meter Exchange ERCOT stores the Load Profile change and forwards the all information to CR |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060123401~20010601~~~2001060112301~~20 |  |  | Request, Unique Transaction Number, Transaction Date, Original Transaction number of the Create ESI ID Request, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name |
|  | N3~123 MAIN STREET APT#2~BUILDING C |  | ESI ID Service Address |
|  | N4~ANYTOWN~TX~761110001 |  | ESI ID Service Address |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP Name and DUNS Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT and DUNS Number, Sender |
| N1~SJ~CR NAME~9~007909422CRN~~40 |  |  | Current CR Name and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Maintenance Request |
|  | ASI~7~001 |  | Request to Change Meter Information |
|  | REF~Q5~~101234500000000000000000000001000011 |  | ESI ID |
|  | DTM~152~20010703 |  | Effective Date of Change |
|  | NM1~MX~3~~~~~~32~GB1203948 |  | Beginning of Meter Loop, Meter Number |
|  |  | REF~46~137656MB | Old Meter Number |
|  |  | REF~4P~10~KHMON~TU^41 | Meter Multiplier |
|  |  | REF~IX~6.0~KHMON~TU^51 | Number of Dials |
|  |  | REF~LO~ RESHIWR_WEST_NIDR_NWS_TOU09 | Load Profile |
|  |  | REF~MT~KHMON | Meter Type |
|  |  | REF~Q2~12345~KHMON~TU^51 | Start Meter Read |
|  |  | REF~Q3~56480~KHMON~TU^51 | End Meter Read |
|  |  | REF~TD~REFLO | Reason for Change |
| SE~22~000000001 |  |  | Number of Segments, Control Number |

814_20 Example #3 of  7

Retire ESI ID Request – TDSP to ERCOT

| TDSP to ERCOT Retire an ESI ID |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~13~2001060112302~20010601~~~~~20 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  | Termination Request |
|  | ASI~7~002 | Request to Retire ESI ID |
|  | REF~Q5~~101234500000000000000000000001000021 | ESI ID |
|  | DTM~197~20010603 | End Date for ESI ID |
| SE~9~000000001 |  | Number of Segments, Control Number |

814_20 Example #4 of  7

Maintain ESI ID Request – TDSP to ERCOT

| TDSP to ERCOT Change ESI ID information – Read Cycle |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060112303~20010601~~~~~20 |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Maintenance Request |
|  | ASI~7~001 |  | Request to Change ESI ID Information |
|  | REF~Q5~~101234500000000000000000000001000031 |  | ESI ID |
|  | DTM~152~20010603 |  | Effective Date of Change |
|  | NM1~MQ~3~~~~~~32~GE1203948 |  | Meter Level Information |
|  |  | REF~TD~REFTZ | Reason Change, Change Meter Cycle |
|  |  | REF~TZ~17 | Meter Reading Cycle |
| SE~12~000000001 |  |  | Number of Segments, Control Number |

814_20 Example #5 of  7

Maintain ESI ID Notification Request – ERCOT to Current CR

| ERCOT to Current CR Change ESI ID information – Notification of a Change in Read Cycle ERCOT Forwards Change in Read Cycle to Current CR |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060123403~20010601~~~2001060112303~~20 |  |  | Request, Unique Transaction Number, Transaction Date, Original Transaction number of the Maintain ESI ID Notification Request, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP Name and DUNS Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT and DUNS Number, Sender |
| N1~SJ~CURRENT CR NAME~9~007909422CRC~~40 |  |  | CR Name and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Request to Change ESI ID Information |
|  | ASI~7~001 |  | Request Change, Change Meter Cycle |
|  | REF~Q5~~101234500000000000000000000001000031 |  | ESI ID |
|  | DTM~152~20010603 |  | Effective Date of Change |
|  | NM1~MQ~3~~~~~~32~GE1203948 |  | Meter Level Information |
|  |  | REF~TD~REFTZ | Reason Change, Change Meter Cycle |
|  |  | REF~TZ~17 | Meter Reading Cycle |
| SE~13~000000001 |  |  | Number of Segments, Control Number |

814_20 Example #6 of 7

Maintain ESI ID Request – TDSP to ERCOT

| TDSP to ERCOT Change ESI ID information – Special Needs Indicator |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~13~2001060112303~20010601~~~~~20 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  | Maintenance Request |
|  | ASI~7~001 | Request to Change ESI ID Information |
|  | REF~Q5~~101234500000000000000000000001000031 | ESI ID |
|  | REF~TD~REFSU | Reason Change, Change Special Needs Indicator |
|  | DTM~152~20010603 | Effective Date of Change |
|  | REF~SU~Y | Special Needs Indicator |
| SE~11~000000001 |  | Number of Segments, Control Number |

814_20 Example #6 of  7

A Temporary Meter exists in the N3; N301 – TDSP to ERCOT

| TDSP submits Create ESI ID Request to ERCOT for a temporary meter, no overflow address information. |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060112301~20010601~~~~~20 |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name – Dummy by Inserting “PREMISE” |
|  | N3~123 N MAIN ST TEMP |  | ESI ID Service Address when a Temporary Meter Exists |
|  | N4~ANYTOWN~TX~761110001 |  | ESI ID Service Address |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Creating a New ESI ID Request |
|  | ASI~7~021 |  | Request to Create a New ESI ID |
|  | REF~AQ~T |  | Distribution Loss Factor Code |
|  | REF~PTC~01 |  | Premise Type |
|  | REF~Q5~~101234500000000000000000000001000011 |  | ESI ID |
|  | REF~SPL~~ WEST |  | Station ID |
|  | REF~SR~ERCOT |  | Power Region |
|  | DTM~196~20010503 |  | Start Date for ESI ID |
|  | DTM~307~20010602 |  | Eligibility Date |
|  | NM1~MQ~3~~~~~~93~ALL |  | Beginning of Meter Loop, Meter Number |
|  |  | REF~LO~ RESLOWR_WEST_NIDR_NWS_TOU01 | Load Profile |
|  |  | REF~TZ~15 | Meter Reading Cycle |
| SE~20~000000001 |  |  | Number of Segments, Control Number |

814_20 Example #7 of 7

A Temporary Meter exists in the N3; N302 – TDSP to ERCOT

| TDSP submits Create ESI ID Request to ERCOT for a temporary meter with overflow address information. |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~2001060112301~20010601~~~~~20 |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name – Dummy by Inserting “PREMISE” |
|  | N3~12345 WEST JOHN QUINCY ADAMS BOULEVARD NW BUILDING 17 S~UITE 164-A TEMP |  | ESI ID Service Address when a Temporary Meter Exists |
|  | N4~ANYTOWN~TX~761110001 |  | ESI ID Service Address |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP Name and DUNS Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~MP |  |  | Creating a New ESI ID Request |
|  | ASI~7~021 |  | Request to Create a New ESI ID |
|  | REF~AQ~T |  | Distribution Loss Factor Code |
|  | REF~PTC~01 |  | Premise Type |
|  | REF~Q5~~101234500000000000000000000001000011 |  | ESI ID |
|  | REF~SPL~~ WEST |  | Station ID |
|  | REF~SR~ERCOT |  | Power Region |
|  | DTM~196~20010503 |  | Start Date for ESI ID |
|  | DTM~307~20010602 |  | Eligibility Date |
|  | NM1~MQ~3~~~~~~93~ALL |  | Beginning of Meter Loop, Meter Number |
|  |  | REF~LO~ RESLOWR_WEST_NIDR_NWS_TOU01 | Load Profile |
|  |  | REF~TZ~15 | Meter Reading Cycle |
| SE~20~000000001 |  |  | Number of Segments, Control Number |