Texas

Standard

Electronic

Transaction

814_PC:

##### Maintain Customer Information Request

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_PC:

MAINTAIN CUSTOMER INFORMATION REQUEST

This transaction set, from a CR to TDSP, is used for CR's who have chosen Options 2 and 3 concerning service orders and outages.   A Competitive Retailer choosing Option 2 or 3 shall be required to provide TDSP with the information needed to verify CR's customer's identity (name, address, and home phone number) for a particular Point of Delivery served by CR and to continually provide TDSP updates of such information.

Document Flow:

- CR to TDSP

|  |  | # Transaction Notes |
| --- | --- | --- |
| Uppercase Characters |  | All Texas SET EDI Transactions will use Uppercase Characters |
| Update/Replace Customer Information |  | Each time this document is received by the TDSP, the TDSP is expected to replace the original or previous customer's information for the premise with the information contained in this document. |
| Timing |  | The timing of this transaction has been a question for many market participants. Texas SET intends to make a proposal to RMS that the CR send the 814_PC to the TDSP after receiving the 867_04 Start Read for that ESI ID. |

|  |  |  |  | # Summary of Changes |
| --- | --- | --- | --- | --- |
| March 16, 2001 Version 1.3 |  |  |  | Initial Release |
| August 3, 2001 Version 1.4 |  |  |  | The following changes were made: |
|  |  |  | - Updated examples: changed FJ to 5J (CC#2001-080) |  |
|  |  |  | - Added gray box for PER segment – name format (CC# 2001-132) |  |
|  |  |  | - Added “Washington DC” to the REF for Driver’s License (CC#2001-080) |  |
| June 17th, 2002 Version 1.5 |  |  |  | The following changes were made: |
|  |  |  |  | - Clarify the use of the N1 segments Ref. 2001-188 |
|  |  |  |  | - Clarify the use of customer contact phone number Ref. 2002-255 |
|  |  |  |  | - Add Special Needs Indicator segment Ref. 2002-259 |
|  |  |  |  | - Add customer billing information Ref. 2002-269 |
| 8/5/02 |  |  |  | - Cleaned up examples with the N1~FJ. Ref. 2002-331 8/5/02 |
| 9/15/02 |  |  |  | - Change Control 2002-364 – Modified gray box of the N4.02 in the N1~BT loop. |
| 10/15/02 |  |  |  | - Change Control 2002-404 – Added REF segment (Bill Type Indicator) to LIN loop. |
|  |  |  |  | - Change Control 2002-411 – Added clarifying language to the N1~BT gray box |
| 2/10/03 |  |  |  | - Change Control 2003-477 – Removed Change Control 405 (PER.02). 405 was withdrawn and should not have been added to the Implementation Guide |
| May 29th, 2003 Version 1.6 |  |  |  | The following changes were made: |
|  |  |  |  | - Change Control 2003-476 Change the wording for the gray box that describes the condition under which State or Province Code (N402) of the N1~BT Loop (Customer Billing Address) is required. |
| 5/30/03 |  |  |  | - Change Control 2003-511 This change control is being requested due to the fact some TDSP's have been receiving TX SET transactions with telephone (TE) or communication numbers with punctuation and symbols in the PER04 and PER06 data elements of the PER Segment, which is incorrect. |
| September 29th, 2003 Version 2.0 |  |  |  | No Changes |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| # Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| # Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O = Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |
|  | 070 | N4 | Geographic Location | O | 1 |  |  |  |  |  |  |  |  |
|  | 080 | PER | Administrative Communications Contact | O | >1 |  |  |  |  |  |  |  |  |
|  | 090 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~13~200104011956531~20010401~~~~~PC |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Request |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |
|  |  |  |  | PC |  | Previously Converted |  |  |  |  |  |
|  |  |  |  |  |  | Indicates Texas SET Transaction 814_PC |  |  |  |  |  |

Segment:	N1 Name (First Contact)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |
| --- | --- | --- |
|  |  | Required |
|  |  | N1~FJ~SNOW, JOE RAY JR N1~FJ~MAINTENANCE DEPT |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |
|  |  |  |  | FJ |  | First Contact |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |
|  |  |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |  |  |  |  |  |  |  |

Segment:	PER Administrative Communications Contact (Contact Name/Number)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To identify a person or office to whom administrative communications should be directed

Syntax Notes:	1	If either PER03 or PER04 is present, then the other is required.

2	If either PER05 or PER06 is present, then the other is required.

3	If either PER07 or PER08 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | PER~IC~~TE~8005551212 PER~IC~~TE~8005551212~TE~8005552121 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M |  | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the major duty or responsibility of the person or group named |  |  |  |  |  |  |  |
|  |  |  |  | IC |  | Information Contact |  |  |  |  |
|  | PER03 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER04 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |
|  | PER05 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER06 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Driver's License)

Position:	090

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Driver's License numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |
| --- | --- | --- |
|  |  | Optional |
|  |  | REF~5J~21932332~TX |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | 5J |  | Driver's License |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Driver's License Number |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |
|  |  |  | State |  |  |  |  |  |  |  |
|  |  |  |  | AK |  | Alaska |  |  |  |  |
|  |  |  |  | AL |  | Alabama |  |  |  |  |
|  |  |  |  | AR |  | Arkansas |  |  |  |  |
|  |  |  |  | AZ |  | Arizona |  |  |  |  |
|  |  |  |  | CA |  | California |  |  |  |  |
|  |  |  |  | CO |  | Colorado |  |  |  |  |
|  |  |  |  | CT |  | Connecticut |  |  |  |  |
|  |  |  |  | DC |  | Washington DC |  |  |  |  |
|  |  |  |  | DE |  | Delaware |  |  |  |  |
|  |  |  |  | FL |  | Florida |  |  |  |  |
|  |  |  |  | GA |  | Georgia |  |  |  |  |
|  |  |  |  | HI |  | Hawaii |  |  |  |  |
|  |  |  |  | IA |  | Iowa |  |  |  |  |
|  |  |  |  | ID |  | Idaho |  |  |  |  |
|  |  |  |  | IL |  | Illinois |  |  |  |  |
|  |  |  |  | IN |  | Indiana |  |  |  |  |
|  |  |  |  | KS |  | Kansas |  |  |  |  |
|  |  |  |  | KY |  | Kentucky |  |  |  |  |
|  |  |  |  | LA |  | Louisiana |  |  |  |  |
|  |  |  |  | MA |  | Massachusetts |  |  |  |  |
|  |  |  |  | MD |  | Maryland |  |  |  |  |
|  |  |  |  | ME |  | Maine |  |  |  |  |
|  |  |  |  | MI |  | Michigan |  |  |  |  |
|  |  |  |  | MN |  | Minnesota |  |  |  |  |
|  |  |  |  | MO |  | Missouri |  |  |  |  |
|  |  |  |  | MS |  | Mississippi |  |  |  |  |
|  |  |  |  | MT |  | Montana |  |  |  |  |
|  |  |  |  | NC |  | North Carolina |  |  |  |  |
|  |  |  |  | ND |  | North Dakota |  |  |  |  |
|  |  |  |  | NE |  | Nebraska |  |  |  |  |
|  |  |  |  | NH |  | New Hampshire |  |  |  |  |
|  |  |  |  | NJ |  | New Jersey |  |  |  |  |
|  |  |  |  | NM |  | New Mexico |  |  |  |  |
|  |  |  |  | NV |  | Nevada |  |  |  |  |
|  |  |  |  | NY |  | New York |  |  |  |  |
|  |  |  |  | OH |  | Ohio |  |  |  |  |
|  |  |  |  | OK |  | Oklahoma |  |  |  |  |
|  |  |  |  | OR |  | Oregon |  |  |  |  |
|  |  |  |  | PA |  | Pennsylvania |  |  |  |  |
|  |  |  |  | RI |  | Rhode Island |  |  |  |  |
|  |  |  |  | SC |  | South Carolina |  |  |  |  |
|  |  |  |  | SD |  | South Dakota |  |  |  |  |
|  |  |  |  | TN |  | Tennessee |  |  |  |  |
|  |  |  |  | TX |  | Texas |  |  |  |  |
|  |  |  |  | UT |  | Utah |  |  |  |  |
|  |  |  |  | VA |  | Virginia |  |  |  |  |
|  |  |  |  | VT |  | Vermont |  |  |  |  |
|  |  |  |  | WA |  | Washington |  |  |  |  |
|  |  |  |  | WI |  | Wisconsin |  |  |  |  |
|  |  |  |  | WV |  | West Virginia |  |  |  |  |
|  |  |  |  | WY |  | Wyoming |  |  |  |  |

Segment:	REF Reference Identification (Social Security Number)

Position:	090

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Social Security numbers will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |
| --- | --- | --- |
|  |  | Optional |
|  |  | REF~SY~123456789 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | SY |  | Social Security Number |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Social Security Number |  |  |  |  |  |  |  |

Segment:	N1 Name (Customer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |
| --- | --- | --- |
|  |  | Required |
|  |  | N1~8R~SNOW, JOE RAY JR N1~8R~XYZ COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8R |  | Consumer Service Provider (CSP) Customer |  |  |  |  |  |
|  |  |  |  |  |  | Customer |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |  |  |  |  |  |  |  |  |

Segment:	N2 Additional Name Information (Doing Business As)

Position:	050

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify additional names or those longer than 35 characters in length

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Required if applicable |
| --- | --- | --- |
|  |  | N2~D/B/A OIL FIELD COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N201 | 93 | Name | M |  | AN 1/60 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form name |  |  |  |  |
|  | N202 | 93 | Name | O |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |

Segment:	N4 Geographic Location (Customer Zip Code)

Position:	070

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Zip Code (N403) is Required. The first 5 characters will be used for validation. |
| --- | --- | --- |
|  |  | N4~~~78111 N4~~~781110001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N403 | 116 | Postal Code | O |  | ID 3/15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |
|  |  |  | Postal codes will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |

Segment:	N1 Name (Customer Billing Name/Address)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required if ESIID is located in Muni/Co-op Area, otherwise not used. ERCOT will not do this validation. IOU TDSPs may reject the transaction if they receive this information. If the IOU TDSP accepts the transaction, the billing information will be discarded and the IOU TDSP will not be responsible for any billing expectations of the CR. |
| --- | --- | --- |
|  |  | Required |
|  |  | N1~BT~DOE, JOHN N1~BT~XYZ COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | BT |  | Bill-to-Party |  |  |  |  |  |
|  |  |  |  |  |  | CUSTOMER BILLING NAME / ADDRESS |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | The Customer Billing Name should be formatted as follows: Last Name, First Name. |  |  |  |  |  |  |  |  |
|  |  |  | Only one comma will be used per the following examples. |  |  |  |  |  |  |  |  |
|  |  |  | N1~BT~DOE, JOHN |  |  |  |  |  |  |  |  |

Segment:	N2 Additional Name Information (Customer Billing Name Overflow)

Position:	050

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify additional names or those longer than 35 characters in length

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Optional: Used if ESIID is located in Muni/Co-op Area, otherwise not used. ERCOT will not do this validation. IOU TDSPs may reject the transaction if they receive this information. If the IOU TDSP accepts the transaction, the billing information will be discarded and the IOU TDSP will not be responsible for any billing expectations of the CR. |
| --- | --- | --- |
|  |  | N2~D/B/A OIL FIELD COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N201 | 93 | Name | M |  | AN 1/60 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form name |  |  |  |  |
|  | N202 | 93 | Name | O |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |

Segment:	N3 Address Information (Customer Billing Address)

Position:	060

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify the location of the named party

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Required: |
| --- | --- | --- |
|  |  | Required if ESIID is located in Muni/Co-op Area, otherwise not used. ERCOT will not do this validation. IOU TDSPs may reject the transaction if they receive this information. If the IOU TDSP accepts the transaction, the billing information will be discarded and the IOU TDSP will not be responsible for any billing expectations of the CR. |
|  |  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N301 | 166 | Address Information | M |  | AN 1/55 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Billing Address |  |  |  |  |
| Dep | N302 | 166 | Address Information | O |  | AN 1/55 |  |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Billing Address Overflow Condition: If there is an overflow, it must be sent. |  |  |  |  |

Segment:	N4 Geographic Location (Customer Billing Address)

Position:	070

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Required if ESIID is located in Muni/Co-op Area, otherwise not used. ERCOT will not do this validation. IOU TDSPs may reject the transaction if they receive this information. If the IOU TDSP accepts the transaction, the billing information will be discarded and the IOU TDSP will not be responsible for any billing expectations of the CR. |
| --- | --- | --- |
|  |  | N4~ANYTOWN~TX~78111 N4~ANYTOWN~TX~781110001 N4~MISSISSAUGA~ON~L4W4E4~CA |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

|  | N401 | 19 | City Name |  |  |  | O |  | AN 2/30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form text for city name |  |  |  |  |  |  |  |
| Dep | N402 | 156 | State or Province Code |  |  |  | O |  | ID 2/2 |  |
|  |  |  | Code (Standard State/Province) as defined by appropriate government agency |  |  |  |  |  |  |  |
|  |  |  | Condition: Required if Customer Billing Address is in the United States, Canada, or any other country that has defined state or province as part of the mailing address. |  |  |  |  |  |  |  |
| Must Use | N403 | 116 | Postal Code |  |  |  | O |  | ID 3/15 |  |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |  |  |  |
|  |  |  | Postal codes will only contain upper case letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |
| Dep | N404 | 26 | Country Code |  |  |  | O |  | ID 2/3 |  |
|  |  |  | Code identifying the country |  |  |  |  |  |  |  |
|  |  |  | For country codes not listed, please refer to ISO 3166, a version of which can be found at http://treetops.commerce.net/research/stats/countrycodes.html. |  |  |  |  |  |  |  |
|  |  |  | Required if outside the United States. |  |  |  |  |  |  |  |
|  |  |  |  | CA |  | Canada |  |  |  |  |
|  |  |  |  | CH |  | Switzerland |  |  |  |  |
|  |  |  |  | DE |  | Germany |  |  |  |  |
|  |  |  |  | ES |  | Spain |  |  |  |  |
|  |  |  |  | FR |  | France |  |  |  |  |
|  |  |  |  | GB |  | United Kingdom |  |  |  |  |
|  |  |  |  | IT |  | Italy |  |  |  |  |
|  |  |  |  | JP |  | Japan |  |  |  |  |
|  |  |  |  | MX |  | Mexico |  |  |  |  |
|  |  |  |  | PR |  | Puerto Rico |  |  |  |  |
|  |  |  |  | SE |  | Sweden |  |  |  |  |

Segment:	N1 Name (TDSP)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | TDSP |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CR NAME~1~007909422~~41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Second Contact Person)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required if available |
| --- | --- | --- |
|  |  | N1~VA~SNOW, JOE RAY JR N1~VA~XYZ COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |
|  |  |  |  | VA |  | Second Contact |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |
|  |  |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |  |  |  |  |  |  |  |

Segment:	PER Administrative Communications Contact (Contact Name/Number)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To identify a person or office to whom administrative communications should be directed

Syntax Notes:	1	If either PER03 or PER04 is present, then the other is required.

2	If either PER05 or PER06 is present, then the other is required.

3	If either PER07 or PER08 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | PER~IC~~TE~8005551212 PER~IC~~TE~8005551212~TE~8005552121 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M |  | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the major duty or responsibility of the person or group named |  |  |  |  |  |  |  |
|  |  |  |  | IC |  | Information Contact |  |  |  |  |
|  | PER03 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER04 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |
|  | PER05 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER06 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Driver's License)

Position:	090

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Driver's License numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |
| --- | --- | --- |
|  |  | Optional |
|  |  | REF~5J~21932332~TX |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | 5J |  | Driver's License |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Driver's License Number |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |
|  |  |  | State |  |  |  |  |  |  |  |
|  |  |  |  | AK |  | Alaska |  |  |  |  |
|  |  |  |  | AL |  | Alabama |  |  |  |  |
|  |  |  |  | AR |  | Arkansas |  |  |  |  |
|  |  |  |  | AZ |  | Arizona |  |  |  |  |
|  |  |  |  | CA |  | California |  |  |  |  |
|  |  |  |  | CO |  | Colorado |  |  |  |  |
|  |  |  |  | CT |  | Connecticut |  |  |  |  |
|  |  |  |  | DC |  | Washington DC |  |  |  |  |
|  |  |  |  | DE |  | Delaware |  |  |  |  |
|  |  |  |  | FL |  | Florida |  |  |  |  |
|  |  |  |  | GA |  | Georgia |  |  |  |  |
|  |  |  |  | HI |  | Hawaii |  |  |  |  |
|  |  |  |  | IA |  | Iowa |  |  |  |  |
|  |  |  |  | ID |  | Idaho |  |  |  |  |
|  |  |  |  | IL |  | Illinois |  |  |  |  |
|  |  |  |  | IN |  | Indiana |  |  |  |  |
|  |  |  |  | KS |  | Kansas |  |  |  |  |
|  |  |  |  | KY |  | Kentucky |  |  |  |  |
|  |  |  |  | LA |  | Louisiana |  |  |  |  |
|  |  |  |  | MA |  | Massachusetts |  |  |  |  |
|  |  |  |  | MD |  | Maryland |  |  |  |  |
|  |  |  |  | ME |  | Maine |  |  |  |  |
|  |  |  |  | MI |  | Michigan |  |  |  |  |
|  |  |  |  | MN |  | Minnesota |  |  |  |  |
|  |  |  |  | MO |  | Missouri |  |  |  |  |
|  |  |  |  | MS |  | Mississippi |  |  |  |  |
|  |  |  |  | MT |  | Montana |  |  |  |  |
|  |  |  |  | NC |  | North Carolina |  |  |  |  |
|  |  |  |  | ND |  | North Dakota |  |  |  |  |
|  |  |  |  | NE |  | Nebraska |  |  |  |  |
|  |  |  |  | NH |  | New Hampshire |  |  |  |  |
|  |  |  |  | NJ |  | New Jersey |  |  |  |  |
|  |  |  |  | NM |  | New Mexico |  |  |  |  |
|  |  |  |  | NV |  | Nevada |  |  |  |  |
|  |  |  |  | NY |  | New York |  |  |  |  |
|  |  |  |  | OH |  | Ohio |  |  |  |  |
|  |  |  |  | OK |  | Oklahoma |  |  |  |  |
|  |  |  |  | OR |  | Oregon |  |  |  |  |
|  |  |  |  | PA |  | Pennsylvania |  |  |  |  |
|  |  |  |  | RI |  | Rhode Island |  |  |  |  |
|  |  |  |  | SC |  | South Carolina |  |  |  |  |
|  |  |  |  | SD |  | South Dakota |  |  |  |  |
|  |  |  |  | TN |  | Tennessee |  |  |  |  |
|  |  |  |  | TX |  | Texas |  |  |  |  |
|  |  |  |  | UT |  | Utah |  |  |  |  |
|  |  |  |  | VA |  | Virginia |  |  |  |  |
|  |  |  |  | VT |  | Vermont |  |  |  |  |
|  |  |  |  | WA |  | Washington |  |  |  |  |
|  |  |  |  | WI |  | Wisconsin |  |  |  |  |
|  |  |  |  | WV |  | West Virginia |  |  |  |  |
|  |  |  |  | WY |  | Wyoming |  |  |  |  |

Segment:	REF Reference Identification (Social Security Number)

Position:	090

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Social Security numbers will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |
| --- | --- | --- |
|  |  | Optional |
|  |  | REF~SY~123456789 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | SY |  | Social Security Number |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Social Security Number |  |  |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | LIN~1~SH~EL~SH~MCI |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  | O |  | AN 1/20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  | M |  | ID 2/2 |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  | M |  | AN 1/48 |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |
|  |  |  | Commodity |  |  |  |  |  |  |  |
|  |  |  |  | EL |  | Electric Service |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |
|  |  |  |  | MCI |  | Maintain Customer Information |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~7~001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  | M |  | ID 1/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |
|  |  |  |  | 7 |  | Request |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  | M |  | ID 3/3 |  |
|  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |
|  |  |  |  | 001 |  | Change |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Billing Type)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | IOU TDSPs may reject the transaction if they receive this information. If the IOU TDSP accepts the transaction, the billing information will be discarded and the IOU TDSP will not be responsible for any billing expectations of the CR. |
| --- | --- | --- |
|  |  | REF~BLT~LDC |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | BLT |  | Billing Type |  |  |  |  |  |
|  |  |  |  |  |  | Indicates whether the bill is consolidated by the TDSP or CR, or whether each party will render their own bill. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | DUAL |  | Dual Billing |  |  |  |  |  |
|  |  |  |  |  |  | Each party bills the customer for its portion |  |  |  |  |  |
|  |  |  |  | ESP |  | ESP Consolidated Billing |  |  |  |  |  |
|  |  |  |  |  |  | The CR bills the customer. |  |  |  |  |  |
|  |  |  |  | LDC |  | LDC Consolidated Billing |  |  |  |  |  |
|  |  |  |  |  |  | The TDSP bills the customer |  |  |  |  |  |

Segment:	REF Reference Identification (Special Needs Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required: |
| --- | --- | --- |
|  |  | A "Y" in this field means that the customer is either: · A residential customer who qualifies through the Residential Critical Care eligibility Determination Form, as issued by the PUCT. Or · A customer for which electric service is considered crucial for the protection and maintenance of public safety pursuant to Subst. Rules §§25.52 and 25.53. Or · An industrial customer for whom an interruption or suspension of service will create a dangerous or life threatening condition at the customer's premise |
|  |  | REF~SU~Y |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Special Processing Code |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Indicator |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs is not required |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Customer |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~18~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

814_PC Example #1 of 4

Maintain Customer Information Request – CR to TDSP

| CR submits Maintain Customer Information Request to the TDSP Residential Example with all potential contact information populated |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~200104011956531~20010401~~~~~PC |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~FJ~ DOE, JOHN P JR |  |  | First Contact Name |
|  |  | PER~IC~~TE~8005551212~TE~8005551213 | Contact Name and Telephone Number(s) |
|  |  | REF~5J~21932332~TX | Driver’s License Number |
|  |  | REF~SY~123456789 | Social Security Number |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  |  | CR Name and DUNS Number, Sender |
| N1~8S~TDSP NAME~1~007909411~~40 |  |  | TDSP Name and DUNS Number, Receiver |
| N1~8R~DOE, JOHN P JR |  |  | Customer Name |
|  |  | N4~~~781110001 | Zip Code |
| N1~VA~DOE, JANE A |  |  | Second Contact Name |
|  | PER~IC~.~TE~8005552222~TE~8005553333 |  | Contact Name and Telephone Number(s) |
|  | REF~5J~32632318~TX |  | Driver’s License Number |
|  | REF~SY~987654321 |  | Social Security Number |
| LIN~1~SH~EL~SH~MCI |  |  | Maintain Customer Information |
|  |  | ASI~7~001 | Request Change |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  |  | REF~SU~N | Special Needs Indicator |
| SE~19~000000001 |  |  | Number of Segments, Control Number |

814_PC Example #2 of 4

Maintain Customer Information Request – CR to TDSP

| CR submits Maintain Customer Information Request to the TDSP Commercial Example |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~13~200104011956531~20010401~~~~~PC |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  | CR Name and DUNS Number, Sender |
| N1~8S~TDSP NAME~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~8R~XYZ HOLDING COMPANY |  | Customer Name |
|  | N2~D/B/A PIZZA COMPANY | Doing Business As Name |
|  | N4~~~781110001 | Zip Code |
| N1~FJ~SMITH, JOHN |  | First Contact |
|  | PER~IC~~TE~8005551212 | Contact Name and Telephone Number(s) |
| N1~VA~MILLER, BILL |  | Second Contact |
|  | PER~IC~~TE~8005552222~TE~8005553333 | Contact Name and Telephone Number(s) |
| LIN~1~SH~EL~SH~MCI |  | Maintain Customer Information |
|  | ASI~7~001 | Request Change |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
| SE~16~000000001 |  | Number of Segments, Control Number |

814_PC Example #3 of 4

Maintain Customer Information Request – CR to TDSP

| CR submits Maintain Customer Information Request to the TDSP Industrial Example |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~13~200104011956531~20010401~~~~~PC |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  | CR Name and DUNS Number, Sender |
| N1~8S~TDSP NAME~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~8R~XYZ INDUSTRIES |  | Customer Name |
|  | N2~D/B/A OIL FIELD COMPANY | Doing Business As Name |
|  | N4~~~781110001 | Zip Code |
| N1~FJ~ELECTRICAL DEPARTMENT |  | First Contact |
|  | PER~IC~~TE~8005552222 | Contact Name and Telephone Number(s) |
| N1~VA~MAINTENANCE DEPARTMENT |  | Second Contact |
|  | PER~IC~~TE~8005553333 | Contact Name and Telephone Number(s) |
| LIN~1~SH~EL~SH~MCI |  | Maintain Customer Information |
|  | ASI~7~001 | Request Change |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
| SE~16~000000001 |  | Number of Segments, Control Number |

814_PC Example #4 of 4

Maintain Customer Information Request – CR to MC TDSP

| CR submits Maintain Customer Information Request to the MC TDSP Residential Example with all potential contact information populated and a change to billing type. |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~13~200104011956531~20010401~~~~~PC |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~FJ~ DOE, JOHN P JR |  |  | First Contact Name |
|  |  | PER~IC~~TE~8005551212~TE~8005551213 | Contact Name and Telephone Number(s) |
|  |  | REF~5J~21932332~TX | Driver’s License Number |
|  |  | REF~SY~123456789 | Social Security Number |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  |  | CR Name and DUNS Number, Sender |
| N1~8S~TDSP NAME~1~007909411~~40 |  |  | TDSP Name and DUNS Number, Receiver |
| N1~8R~DOE, JOHN P JR |  |  | Customer Name |
|  |  | N4~~~781110001 | Zip Code |
| N1~VA~DOE, JANE A |  |  | Second Contact Name |
|  | PER~IC~~TE~8005552222~TE~8005553333 |  | Contact Name and Telephone Number(s) |
|  | REF~5J~32632318~TX |  | Driver’s License Number |
|  | REF~SY~987654321 |  | Social Security Number |
| LIN~1~SH~EL~SH~MCI |  |  | Maintain Customer Information |
|  |  | ASI~7~001 | Request Change |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  |  | REF~SU~N | Special Needs Indicator |
|  |  | REF~BLT~LDC | Billing Type |
| SE~20~000000001 |  |  | Number of Segments, Control Number |