LCRA request for load exemption:

For 2004, LCRA asked for and was granted a load zone exemption for the SeaWay and Macidonio substations.  As a result of the exemption, these two buses were moved from the Houston zone to the South zone.  These are the only loads served by LCRA in the Houston zone and serve approximately 13 and 11 MW at summer peak respectively.

For 2005, LCRA requests that TAC approves its request to move the SeaWay (7670) and Macidonia (7671) buses from the Houston Zone to the South Zone contingent on a determination by ERCOT that the conditions of Section 7.2.1.1(5) (a) of the protocols are met.  LCRA warrants that requirements of Section 7.2.1.1 (5)(b) are met.

- (5)	At the time of the TAC review and approval process, a Non Opt-In Entity may request an exemption to its zonal placement by submitting a request to the TAC for consideration.  TAC will evaluate the merits of the request against the following criteria.  If TAC agrees with the request, TAC will recommend approval to the Board.

(a)	Exemptions shall not cause significant operational impact to the ERCOT System.  This will normally be satisfied if the request will not result in a change in the annual average Shift Factor for the original or requested Congestion Zone by more than five percent (5%).  After the proposed move, the Zonal Shift Factor in each of the original and requested Congestion Zones shall be no less than ninety-five percent (95%), and no more than one hundred and five percent (105%), of its original Zonal Shift Factor before the proposed move.

Since only busses with generation have an impact on the calculation of Zonal Shift Factors, if no generation exists at a bus that is part of an exemption request, a token generator of one (1) MW will be placed at that bus to test the impact of moving that bus from one (1) zone to another.  This will be done to all busses that make up a request for exemption.  For example, if an Entity is requesting an exemption for five (5) busses and only one (1) bus has generation On-line in the Load flow case being used to test the impact, one (1) MW of generation will be placed at the four (4) other busses and the impact of all five (5) busses on Zonal Shift Factors will be determined as a group as opposed to individually.  The Load flow case used to determine this impact will be the same Load flow case used to calculate annual TCR quantities.

(b)	Exemptions shall not have a commercial impact/gain for the requestor.  This will be measured by whether or not the scheduling which would otherwise result would be eligible for Pre-assigned Congestion Rights.