# Texas SET Change Control Form

Texas SET Change Request

Change Control #2002-401

This TX SET Change Request can be found on the ERCOT website at www.ercot.com  .

| Requester’s Name: Ed Skiba | Company Name: Entergy | Phone #: 504-576-5246 |
| --- | --- | --- |
| Date of Request: 08/29/2002 | Affected EDI Transaction Set #(s): 814_11 | E-Mail Address: edskiba@us.ibm.com |
| Emergency Priority* (Y/N): No | Requested Implementation Date: Future Implementation | Production Implementation Date: |
| Testing Required for this CC (Y/N): defer to test team | Testing Flight Number: (ready to test for this flight) | Status: |

Brief Explanation (This will be copied into the description in the Change Control Summary Spreadsheet):

Add the N1~8S (TDSP) segment to the 814_11 to provide information to the CR as to whether ERCOT rejected the 814_10 or the TDSP rejected the 814_03.

Reason for Request (Explain why this change is needed. For business or technical purposes?):

When the CR receives and 814_11 reject, there is no way for the CR to know that the rejection was from the TDSP and not ERCOT.  Adding the N1~8S segment will provide the CR with the information let them know whether ERCOT rejected the 814_10 or the TDSP rejected the 814_03.

Detail Explanation  (Exactly what change is required? To which TX SET Standards? Why?):

See Redline below.

For Change Control Manager Use Only:

| Date of TX SET Discussion: | Expected Implementation Date: |  |
| --- | --- | --- |

TX SET Discussion and Resolution:

Minor modifications made to the change control and approved on 9/6/02 for V1.6.

| *Emergency Priority | Used for Change Controls that require immediate implementation. |
| --- | --- |

Please submit this form via e-mail to txsetchangecontrol@ercot.com .

Your request will be evaluated and prioritized at an upcoming TX SET meeting or conference call.

814_11

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required when the 814_11 is a reject response resulting from an 814_04 reject response, otherwise not used. |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411~~OA |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | OA |  | Electronic Return Originator |  |  |  |  |  |
|  |  |  |  |  | Required when the TDSP sent an 814_04 reject response and the rejection is being forwarded to the CR that submitted the 814_10. This code is OH A (OA) and not Zero A (0A). |  |  |  |  |  |  |