## ERCOT Operations Report

## ROS & OWG

## December 2004

December Frequency control   59.9 > F > 60.1

|  |  |  | Reg | ERCOT | Schedule Change |
| --- | --- | --- | --- | --- | --- |
| Time | Frequency | Load | (Max) | SCE | (Largest QSE Change) |
| 12/3/2004 2:05 | 60.178 | 27361 | -518 | 1637 | -2060 (-2000) Incorrect generation schedule from 1 QSE resulted in deployment of excess UBES. |
| 12/13/2004 22:06 | 59.897 | 34897 | 342 | -1001 | -4950 (-2200) |
| 12/16/2004 2:04 | 60.106 | 28331 | -560 | 210 | -320 (-130) |
| 12/21/2004 17:59 | 60.116 | 33095 | -241 | 144 | 780 (500) |

Disturbances

|  |  | MW |
| --- | --- | --- |
| 12/3/2004 4:25 | 59.895 | 400 |
| 12/7/2004 8:27 | 59.764 | 840 |
| 12/16/2004 1:24 | 59.819 | 790 |
| 12/18/2004 0:09 | 59.823 | 715 |
| 12/21/2004 23:58 | 59.833 | 750 |
| 12/23/2004 1:14 | 59.899 | 520 (3 unit train) |
| 12/26/2004 19:58 | 59.821 | 780 |
| 12/27/2004 4:39 | 59.895 | 345 (2 unit train) |

Deployed Responsive (36) (Reported Above)

| 12/2/04 6:09:56 | 12/9/04 6:09:36 | 12/15/04 11:36:32 | 12/22/04 7:51:46 |
| --- | --- | --- | --- |
| 12/2/04 17:35:36 | 12/9/04 17:40:04 | 12/15/04 19:28:46 | 12/22/04 19:06:14 |
| 12/2/04 23:12:00 | 12/10/04 6:06:04 | 12/16/04 1:24:02 | 12/22/04 23:16:48 |
| 12/3/04 4:25:34 | 12/10/04 17:54:32 | 12/16/04 22:02:10 | 12/23/04 1:13:24 |
| 12/3/04 6:04:52 | 12/12/04 17:41:16 | 12/17/04 22:03:30 | 12/25/04 0:45:10 |
| 12/3/04 23:44:24 | 12/13/04 17:39:16 | 12/18/04 0:09:46 | 12/26/04 19:58:20 |
| 12/5/04 8:06:32 | 12/13/04 22:04:26 | 12/20/04 6:07:54 | 12/27/04 4:39:48 |
| 12/7/04 8:27:34 | 12/14/04 6:02:36 | 12/21/04 23:58:20 | 12/28/04 22:01:24 |
| 12/8/04 6:09:42 | 12/14/04 22:06:40 | 12/22/04 6:09:14 | 12/29/04 22:04:48 |

# Locations and Relative Activity of Congestion Management

| Historical OOMC/RMR | Dec '03 | Nov '04 | Dec '04 |
| --- | --- | --- | --- |
| DFW | 69 | 7 | 17 |
| E RIO | 40 | 57 | 47 |
| W RIO | 34 | 29 | 49 |
| Laredo | 34 | 40 | 43 |
| Bryan Area | 0 | 6 | 17 |
| Corpus | 31 | 30 | 38 |
| Abilene | 52 | 30 | 15 |
| San Angelo | 32 | 0 | 0 |
| Western | 18 | 0 | 14 |
| Houston | 22 | 0 | 0 |
| San Antonio | 0 | 0 | 1 |
|  | 332 | 199 | 242 |

OOME / Congestion Management Activity

| Contingency | Overload | Occurrences |
| --- | --- | --- |
| Heights Sub - Freeway Park | Heights - PH Robinson | 15 |
| Uvalde - Hamilton | Asherton - West Conoco | 12 |
|  | South - North CSC | 10 |
| Celanese Bishop - Barney Davis | Aransas Pass - Naval base | 7 |
| Austrop - Sandow | Austrop Tr | 7 |
| Midland East - Odessa | Odessa - Sw Moss | 7 |
| N Edinburg - Bates | Pharr MVEC - Polk | 5 |
| Rio Hondo Tr | Rio Hondo - La Palma | 4 |
| Calaveras- Skyline/Walzem Rd | Five Points - West Side | 3 |
| Cico - Helotes | Gilles - Wolf Creek | 3 |
| Military Highway T1 | Military Highway T2 | 3 |
| Dilley Switch - San Miguel | Bigfoot Tr | 2 |
| Hearne Tr | Booneville - Sutton | 2 |
| Sargent - Watermill | Cedar Hill - Watermill | 2 |
| Clear Springs - Marion/Zorn | Cico - Comfort | 2 |
| Zorn - Hayes | Comfort - Cico | 2 |
| Forney Elkton - Tricorner | Flynn - Fairmont | 2 |
| Morgan Creek - Odessa | General Tire - Rexall | 2 |
| La Palma - Rio Hondo | Haine Dr - La Palma | 2 |
| Holt Switch - Andrews North | Holt Sw T1 | 2 |
| Big Lake - Friend Ranch | Midway Lane - Ozona | 2 |
|  | Northeast - North CSC | 2 |
| Asherton - Eagle Pass | Uvalde - Asphalt Mines | 2 |
| Big Brown - Venus Switch | Watermill - Cedar Hill Switch | 2 |
| Austrop - Lost Pines | Winchester - Fayetteville | 2 |
| Menard Tr | Ballinger - Paint Rock | 1 |
|  | Bates Tr | 1 |
| Bates U1 | Bates Tr | 1 |
| STP - Skyline/Hill Country | Blessing - Lane City | 1 |
| Dansby - Hearne | Booneville - Sutton | 1 |
| Watermill Tr | Centerville Rd Sw Tr | 1 |
| Cico - Pipe Creek | Cico - Comfort | 1 |
| Gillespe Tr | Fort Mason - Mason | 1 |
| Morgan Creek - Odessa | General Tire - Olefin | 1 |
| Hill Top Lakes Tr | Hearne - Elliot | 1 |
| Lon Hill - Whitepoint | Nueces Bay - Whitepoint | 1 |
| N Edinburg - Bates | Polk - Hall Acres | 1 |
| B Davis U2 | Port Aransas - Mustang Island | 1 |
| Kingsville - Lon Hill | Port Aransas - Mustang Island | 1 |
| Sandow Tr. | Sandow - Salty | 1 |
|  | South - Houston CSC | 1 |
| Watermill Tr | Tricorner - Seagoville | 1 |
|  | West - North | 1 |

Indicates a Remedial action plan has been developed for the element

# New RAP/SPS

None

New Procedures/ Forms/ Operations Bulletins

| 12/21/2004 | ERCOT Market Operations Bulletin 21 | Shadow Price Calculations for 5 Zone, 6 CSC model for 2005 |
| --- | --- | --- |
| 12/20/2004 | ERCOT Market Operations Bulletin 20 | SPD Price Administration Process |
| 12/21/2004 | ERCOT Power Operations Bulletin 133 | OOME for CT’s |
| 12/09/2004 | ERCOT Power Operations Bulletin 132 | Verification of Dynamic Ratings TCN#186 |
| 12/09/2004 | ERCOT Power Operations Bulletin 131 | Managing Maintenance Level 2 and 3 outages TCN#172 |
| 12/09/2004 | ERCOT Power Operations Bulletin 130 | Granting VDE’s TCN#177 |
| 12/09/2004 | ERCOT Power Operations Bulletin 129 | Monitoring West Texas Generation TCN#183 |
| 12/09/2004 | ERCOT Power Operations Bulletin 128 | Extending TCN#138 TCN#178 |
| 12/09/2004 | ERCOT Power Operations Bulletin 127 | Managing Maintenance Level 2 and 3 outages TCN#176 |
| 12/09/2004 | ERCOT Power Operations Bulletin 126 | Update of QSE list. |
| 12/09/2004 | ERCOT Power Operations Bulletin 125 | Update of QSE list. |
| 12/09/2004 | ERCOT Power Operations Bulletin 124 | Failure to clear Day Ahead A/S Market |
| 12/09/2004 | ERCOT Power Operations Bulletin 123 | Procedures when DC Tie is over scheduled |
| 12/06/2004 | ERCOT Power Operations Bulletin 122 | Verbal Dispatch Exception |
| 12/06/2004 | ERCOT Power Operations Bulletin 121 | Verbal Dispatch Exception |

# Security Alert Stage/ Threatcon/ Related issues

None/Yellow

Major system voltage problems

None.

Blackout/Load Shed incidences

None

# Communication

12/1	01:45 – 2:00	Held interval.  OC1 inadvertently activated.

12/3	02:15 – 03:30	Held intervals.  One QSE experienced a Portal failure and scheduling issues.

Major Computer Problems, Enhancements & Fixes

None

ERCOT peak (Unofficial)

12/23		44,425

OCN, Advisory, Alert, Emergency Notice and Major Events

12/1	02:00 – 04:30	OCN issued.  Capacity shortage 08:00 – 10:00

04:00 – 04:30	Alert issued.  Capacity shortage 08:00 – 10:00

12/2	13:30 – 15:20	Transmission Alert.  Loss of Holt Switch – Permian Basin loads

Moss Switch – Amoco North Cowden Tap.

12/7      18:45 - 19:25	Transmission Alert.   Centerville Road – Garland Road exceeding first contingency

limit.

12/9       09:20 – 11:20	OCN issued.  Capacity shortage 18:00 – 20:00

14:40 – 20:00	OCN issued.  Capacity shortage 19:00 – 20:00

12/21       07:30 – 24:00	OCN issued.  Cold temperatures and freezing precipitation forecasted.

12/21	  10:00 – 17:00  Transmission Alert.  Loss of Holt Sw – N Andrews loads Holt Tr.

12/22       00:00 – 24:00	OCN issued.  Cold temperatures and freezing precipitation forecasted.

12/23	   19:00 – 22:45	 Transmission Alert.  Loss of Barney Davis – Celanese Bishop results in islanding in

South Corpus Christie area.

12/24       06:00 – 14:30  Transmission Alert. Loss of Uvalde – Hamilton loads Asherton – W Monaco.

12/24 08:15 – 12/25 15:00  Transmission Alert.  Loss of Barney Davis U2 loads Port Aransas – Mustang Island.

12/24       11:30		  Transmission Alert.  Loss of Menard T1 loads Ft. Mason – Mason.

12/24 00:00 – 12/25 15:00   Advisory issued for low temperatures and strong winds.

12/25        05:30 – 12:00   Transmission Alert.  Loss of Uvalde Hamilton loads Asherton – W Conoco.

12/25      08:15		  Transmission Alert.  Loss of Barney Davis U2 loads Port Aransas – Mustang Island.

12/26       06:45 – 12:30    Transmission Alert.  Loss of Bates U1 loads Bates T1.

12/27        03:15 – 05:00    OCN issued for 08:00 capacity shortage.

12/30       14:30 – 16:00    OCN issued for 19:00 capacity shortage.

12/31       16:30		    OCN issued for 1/1/05.  Insufficient Down Reg for HE 06,23, & 24.