| PRR Number | 565 | PRR Title | Calculation of Losses for Settlement |
| --- | --- | --- | --- |
| Date |  | 02/10/05 |  |
| Submitter’s Information |  |  |  |
| Name |  |  |  |
| E-mail Address |  |  |  |
| Company |  |  |  |
| Company Address |  |  |  |
| Phone Number |  |  |  |
| Fax Number |  |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT |  | Change in LodeStar for calculation for Distribution Losses for Settlement. | More accurate calculations of Distribution Losses for Settlement. |
| MARKET SEGMENT |  |  |  |
| Consumer |  |  |  |
| LSE: General, Including NOIE |  | Change to shadow settlement systems. | More accurate calculations of Distribution Losses for shadow Settlement. |
| LSE: CR & REP |  | Change to shadow settlement systems. | More accurate calculations of Distribution Losses for shadow Settlement. |
| QSE |  | Change to shadow settlement systems. | More accurate calculations of Distribution Losses for shadow Settlement. |
| Resource |  |  | More accurate calculations of Distribution Losses for shadow Settlement. |
| TDSP |  | Change to shadow settlement systems. | More accurate calculations of Distribution Losses for shadow Settlement. |

Comments

As PRS requested at its January 2005 meeting, ERCOT staff is providing comments and additional data to support the use of actual System Load to calculate Distribution Loss Factors for Settlement (Section 13.1.1.).

ERCOT staff notes that, since market open, no TDSPs have opted to use Method 2, described in Protocol Section 13.3.1.2, for filing Distribution Loss Factors and concurs with the PWG recommendation to delete this option from the Protocols.

PRR565 would not require any additional staffing efforts, however, a project would be required to enhance the ERCOT systems to calculate and post actual Distribution Loss Factors.  The project would involve modifying the Lodestar system to utilize actual losses in its calculations, as well as creating a new extract for the posting to the Texas Market Link of the actual Distribution Losses.  An estimate of the costs and timing of the project will be provided in ERCOT’s formal impact analysis.  There is not a feasible manual workaround while the system changes are being developed.

ERCOT staff concurs with the PWG observation that transmission loss calculations have been and continue to be based on actual load data, and that, for consistency the Distribution Loss calculations for Settlement should also be based on actual load data. Use of actual ERCOT System Load data will increase the accuracy of the Distribution Loss Factors by eliminating forecasting error from the calculation of the Loss Factors.  As the load comparison graph below indicates, forecasting error can be significant on any given Operating Day.

While the “true” value of Distribution Losses is not known, ERCOT staff has compared Distribution Loss differences that result when they are calculated using actual and forecasted load.  The following bar chart displays the frequency distribution of these differences over the October 2003 – September 2004 time period.

Loss differences range from -21 Mw to 74 Mw, with a mean of about 10 Mw.  About 99% of the intervals have loss differences in the -10 Mw to 40 Mw range, and about 58% of intervals have loss differences between 0 Mw to 10 Mw.

ERCOT also performed an interval level linear regression analysis of the ERCOT forecast and actual load versus LSEGUFE load by TDSP and Loss Code for the same time period.  The following chart shows the results of these analyses.

The R-Square values are greater using actual ERCOT load for every TDSP and loss code combination indicating that a closer correlation (better fit) to the TDSP load exists when using actual load as opposed to forecasted load.  Based on staff experience and judgment, true Distribution Loss Factors for an interval for a TDSP are essentially linearly related to the TDSP load during that interval.  Since ERCOT actual load has a higher correlation to TDSP load than ERCOT forecasted load has to TDSP load, using actual load will produce more accurate estimates of Distribution Loss Factors.

ERCOT staff further points out that in the Section 13.3 equation [shown below] used to calculate the settlement interval loss factor, the TDSP supplied ADLF and K parameters are calculated based on both ERCOT and the TDSP’s historical actual load, and the AAL parameter is based on ERCOT historical actual load.  Thus, using ERCOT actual load for SIELi is consistent with the development of the parameters of the formula itself.

SILFi = ADLF * [ K+(1-K)*(SIELi/AAL)]

ERCOT is providing clarification in the following sections to the language proposed in PRR 565 (Calculation of Losses for Settlements): Section 13.1.1- Responsibility for Transmission and Distribution Losses; 13.1.2- Calculation of Losses for Settlement;

13.3.1- Loss Factor Calculation; 13.3.1.1- Method 1 Settlement Interval Distribution Loss Factors Calculated by ERCOT; and 13.3.1.2- Method 2 Settlement Interval Distribution Loss Factors Submitted by Distribution Service Provider.

ERCOT has incorporated revisions developed in the TNT process primarily Section13.2.

Revised Proposed Protocol Language

13.1.1	Responsibility for Transmission and Distribution Losses

T&D Losses are the responsibility of the QSE representing the Competitive Retailer’s Load.  The QSE will schedule the necessary amount of energy to cover Competitive Retailer’s Load plus the applicable T&D Losses.  ERCOT shall allocate T&D Losses to Load at the appropriate aggregate level as part of the data aggregation process to calculate the Load obligation of QSEs for settlement purposes.

Transmission Loss Factors shall be forecasted by ERCOT, and posted to the MIS by 0600 of the Day Ahead period.  ERCOT shall forecast the ERCOT-wide Transmission Loss Factors expressed as a percent of Load for each settlement interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Transmission Loss Factors for each Settlement Interval using actual System Load for that Settlement Interval and will post the resulting deemed actual Transmission Loss Factors to the settlement system and the MIS.

Distribution Loss Factors shall be forecasted by ERCOT, and posted to the MIS by 0600 of the Day Ahead period.  ERCOT shall forecast the Distribution Loss Factors expressed as a percent of Load for each Settlement interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Distribution Loss Factors for each settlement interval using actual system Load for that Settlement Interval and post the resulting deemed actual Distribution Loss Factors to the settlement system and the MIS.

Distribution Loss Factors will be subject to audit by ERCOT for accurate and consistent application.  Non-Opt-in Entities (NOIE) with Interval Data Recorders at the settlement point of delivery are not required to provide Distribution Loss Factors.

In the special case where there are distribution Facilities upstream from a wholesale NOIE settlement IDR that settlement IDR will be compensated for line and transformer losses between the IDR and the ERCOT Transmission Grid to account for the Distribution Losses.  The NOIE will be then treated as a transmission level NOIE.  Calculations are subject to review by ERCOT.  Since loss compensation is included in the wholesale settlement IDR, the TDSP providing upstream wheeling facilities may need to offer wholesale wheeling tariffs excluding the losses that have already been compensated for.

13.1.2	Calculation of Losses for Settlement

ERCOT shall use the deemed actual Distribution Loss Factors applicable to each ESI ID and the deemed actual Transmission Loss Factors when adjusting aggregated Load for losses to determine the QSE total Load obligations.

13.2	Transmission Losses

13.2.1	Forecasted Transmission Loss Factors

The forecasted Transmission Loss Factor for each interval in the Operating Day shall be a linear interpolation or extrapolation using the on-peak and the off-peak Transmission Loss Factors and the corresponding forecast of ERCOT System Load during the same interval to calculate the loss factors .

At 0600 of the Day Ahead period, ERCOT shall forecast a Transmission Loss Factors for each Settlement Interval of the Operating Day and post the forecasted Transmission Loss Factors which correspond to the Operating Day forecast.  The source of the on-peak and off- peak losses are the ERCOT load flow base cases for the applicable season.

13.2.2	Deemed Actual Transmission Loss Factors

ERCOT will determine the deemed actual Transmission Loss Factor for each interval in the Operating Day, by use of a linear interpolation or extrapolation using the on-peak and the off-peak Transmission Loss Factors corresponding to the actual ERCOT System Load for the interval.

The day after the Operating Day, ERCOT shall calculate deemed actual Transmission Loss Factors for each Settlement Interval of the Operating Day andpost the Transmission Loss Factors to be used in settlement calculations.

ERCOT shall use the Transmission Loss Factors corresponding to the on-peak and off-peak base case ERCOT System Loads during the applicable seasons as the basis for the ERCOT-wide deemed actual Transmission Loss Factors.  ERCOT will post Transmission Loss Factors to the MIS by 0600 two days after the Operating Day.

13.2.3	Transmission Loss Factor Calculations

The following formulas shall be used to translate the seasonal on-peak and off-peak Transmission Loss factors into Settlement Interval Transmission Loss Factors.

TLFi	=	(SSC * SIELi) + SIC

Where:

i =	interval

TLFi = 	Transmission Loss factors for a settlement interval

SIELi =	Settlement Interval ERCOT Load (forecasted or actual)

SSC =	Seasonal Slope Coefficient

SIC =	Seasonal Intercept Coefficient

And,

SSC 	=	(SONLF – SOFFLF)/(SONL-SOFFL)

SIC	=	[(SOFFLF*SONL)-(SONLF*SOFFL)]/(SONL-SOFFL)

Where:

SONLF =	Seasonal on-peak percent loss factor

SOFFLF =	Seasonal off-peak percent loss factor

SONL =	Seasonal on-peak Load value

SOFFL =	Seasonal off-peak Load value

13.2.4	Seasonal Transmission Loss Factor Calculation

Seasonal on-peak and off-peak Transmission Loss Factors are derived from annually updated ERCOT on-peak and off-peak load flow base cases analysis by ERCOT.  Base cases reflect the most current data on the transmission system and Generation Resource dispatch.  The ERCOT Transmission Grid topology and related Generation Resource dispatch in the base cases are the critical factors in calculating losses.  Seasonal time periods are defined as follows:

- 1)	Spring   (March – May)

- 2)     	Summer (June – August)

- 3)	Fall (September – November)

- 4)	Winter (December – February)

ERCOT shall calculate Seasonal Transmission Loss Factors by dividing ERCOT seasonal case transmission losses (60 kV system and higher) by ERCOT seasonal base Load adjusted (reduced) for self serve Load modeled in the case.  The resulting loss factors are expressed as a percentage of Load.

ERCOT shall post to MIS Public Area the Seasonal Transmission Loss Factors thirty (30) days prior to the start of the season .

13.2.5	Loss Monitoring

ERCOT shall monitor Transmission Losses annually and will investigate abnormal loss factors.  ERCOT and Transmission Service Providers shall use the cost of losses as one criterion in evaluating the need for transmission additions.

13.3	Distribution Losses

By October 30th of each year for the next calendar year, each Distribution Service Provider (DSP), except NOIEs shall calculate and provide ERCOTth of each yearoribution Loss Factors to be applied to distribution voltage level Loads in its area of certification.  ERCOT shall review and approve the Distribution Loss Factor calculation methodology   used by each DSP prior to use of the los factors for settlement purposes.  If the Distribution loss Factor calculation methodology does not conform inth ERCOT’s interpretation of the pPotocol criteria in this subsection, ERCOT will work with the Distribution Service Provider to correct the deficiency.  Whiil deficiencies are resolveding deient approved DLFs and the calculation methodology will be posted, and the last approved DLFs shall be used for settlement.  A DSP may only submit a change to the DLF calculation methodology submitted by DSP may only be made on an annually basis.

The TDSP shall assign a Distribution Loss Factor code to each ESI ID.  A maximum of five Distribution Loss Factors may be submitted for each Distribution Service Provider based upon ERCOT approved parameters, such as service voltages or number of transformations.

The following coding standards will be used to identify the Distribution Loss

Factor applicable to each ESI ID:

- T = Transmission connected Customers (no Distribution Loss Factor applied)

- A through E = TDSP defined Customer segment(s)

The Distribution Service Providers, except NOIEs, are obligated to provide Distribution Loss Factors to ERCOT.  ERCOT will post the Distribution Loss Factor and calculation methodology, including any equations and constants, for each DSP.

Loss factor variables submitted by the Distribution Service Provider shall include:

(1)	The annual Distribution Loss Factor for each DLF code;

(2)	The methodology upon which the calculation was made;

(3)	The annual interval average Load for the entire DSP service area used to calculate the annual loss factor for each DLF code;

(4)	The portion of the loss factor that is not sensitive to Load for each DLF code;

13.3.1 Loss Factor Calculation

ERCOT shall use the DLFs submitted by the DSP to calculate the Settlement Interval Distribution Loss Factors.  Distribution Loss Factors will be calculated from the data provided by Distribution Service Providers as follows using the following equation:

SILFi = ADLF * [ K+(1-K)*(SIELi/AAL)]

Where:

i =	interval

SILFi =	Settlement Interval Distribution Loss Factor

ADLF =	Annual Distribution Loss factor

K =	a factor provided by DSP between 0.0 and 1.2

SIELi =	Settlement Interval ERCOT System Load (forecasted or actual)

AAL =	Annual Interval Average ERCOT System Load

And where:

AAL =	Annual Total System MWh / (Number of Settlement Intervals in the year)

ERCOT shall use the deemed actual Distribution Loss Factors calculated for each Settlement Interval of the Operating Day for settlement purposes.

13.3.2	Loss Monitoring

Distribution Loss Factors for all Distribution Service Providers, except for NOIEs, will be submitted to ERCOT and will be subject to audit for accuracy and consistency of application.

13.4	Special Loss Calculations for Settlement & Analysis

13.4.1	Deemed Actual Transmission Losses for NOIEs

All QSEs representing Load, including NOIEs, will be responsible for Transmission Losses allocated in the manner described in these Protocols.  Those Entities using transmission tie line meters to determine Load will adjust the net meter readings to remove calculated Transmission Losses behind the meter in order to determine the Load responsibility of the Entity.  ERCOT will provide to settlement the calculation of the losses behind the meters, for each interval, using actual system conditions for that interval.

The deemed actual transmission losses for NOIEs shall be a linear interpolation or extrapolation between the seasonal on-peak and the seasonal off-peak NOIE Transmission Loss Factors corresponding to the actual NOIE metered load in the interval.

ERCOT shall calculate seasonal NOIE Transmission Loss Factors corresponding to the on- peak and off- peak base case System Loads during each of the four seasons of the upcoming year as the basis for the NOIE Transmission Loss Factors.  NOIE seasonal loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT wide Transmission Loss Factors.

13.4.2	Deemed Actual Transmission Losses for UFE Analysis

ERCOT shall adjust Net Generation data used for UFE analysis zones that contain Transmission Facilities behind any metering points will be adjusted to the ERCOT wide Transmission Loss Factor.  This adjustment requires reducing the Net Generation by the calculated deemed actual MWh of Transmission Losses and adding back the ERCOT wide Transmission Loss Factor translated into a MWh value.  ERCOT shall provide the calculation of the deemed actual Transmission Losses behind the UFE zonal meters, for each interval, to settlement using actual system conditions for that interval.

The deemed actual Transmission Losses for UFE analysis zones shall be a linear interpolation or extrapolation between the seasonal on-peak and the seasonal off-peak UFE analysis zone Transmission Loss Factors corresponding to the actual UFE analysis zone metered load in the interval.

ERCOT shall calculate seasonal UFE analysis zones Transmission Loss Factors corresponding to the on- peak and off- peak base case System Loads during each of the four seasons of the upcoming year as the basis for the UFE analysis zone Transmission Loss Factors.  UFE analysis zone seasonal loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT wide Transmission Loss Factors.