| PRR Number | 564 | PRR Title | Clarification of OOME Definition |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.1.11 |  |
| Requested Resolution (Normal or Urgent) |  | Normal |  |
| Revision Description |  | Clarify the definition of OOME instruction to explicitly include the actuation of a special protection system (SPS). |  |
| Reason for Revision |  | While the protocols implicitly indicate that an SPS actuation as an OOME dispatch they do not explicitly include an SPS in its definition in section 6. Section 5 – Dispatch, of the protocols indicate that an SPS is an authorized dispatch instruction by ERCOT as stated below. 5.4.1 Control Area Operator Authority ERCOT, as Control Area Operator (CAO), is authorized to take the following actions for the limited purpose of securely operating the ERCOT Transmission Grid in conformance with the standards specified in Section 5.2, Standards and Practices, including, but not limited to: - Direct the physical operation of the ERCOT System, including circuit breakers, switches, voltage control equipment, protective relays, metering and Load shedding equipment; …….. As stated in OG section 7.2.2.13, Special Protection Systems (SPS) are protective relay systems designed to detect abnormal ERCOT System conditions and take pre-planned corrective action (other than the isolation of faulted elements) to provide acceptable ERCOT System performance. SPS actions include among others, changes in demand, generation, or system configuration to maintain system stability, acceptable voltages, or acceptable facility loadings. The SPS is used because neither the ERCOT Operating System or System Operators would be able to take corrective action in an acceptable timeframe. Therefore, as the above indicates, an SPS is a pre-planned corrective action dispatch approved by ERCOT which is implemented automatically when system conditions warrant. Therefore, an SPS is a legitimate ERCOT OOME dispatch instruction. |  |
| Credit Implications (Yes or No, and summary of impact) |  | No impact |  |
| Timeline |  |  |  |
| Date Posted |  | 12/17/04 |  |
| Please see the Master List on the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Robert Helton |
| E-mail Address | bhelton@anpower.com |
| Company | American National Power, Inc. |
| Company Address | 18509 Champions Circle, Point Venture TX. 78654 |
| Phone Number | 832-435-7815 |
| Fax Number | 512-267-3833 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | No impact as this is only a clarification | None, this is handled by the issuance of a VDI instruction upon SPS actuation. | Clarifies ERCOT’s actions |
| MARKET SEGMENT | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |
| Consumer | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |
| LSE: General, Including NOIE | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |
| LSE: CR & REP | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |
| QSE | No impact as this is only a clarification | No impact as this is only a clarification | Clarifies the QSE’s actions and eliminates the need for filing disputes for settlement |
| Resource | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |
| TDSP | No impact as this is only a clarification | No impact as this is only a clarification | No impact as this is only a clarification |

Proposed Protocol Language Revision

6.1.11	Out-Of-Merit Energy Service

As provided by ERCOT to QSEs: The deployment by ERCOT for the Settlement Interval of energy from Resources, that may or may not have provided Resource-specific premium bids, and used by ERCOT to provide Balancing Energy Service when no Market Solution exists for resolving Congestion, the actuation of an ERCOT approved Special Protection System or if required in declared emergencies as described in these Protocols.