How to Use this Implementation Guide

Segment:	REF Reference Identification (ESI ID)

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: This section is used to show the Texas Rules for implementation of this segment. |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

One or more examples.

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| # Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| # Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

| PTD Definition and Use |  | This section describes the purpose of each PTD Loop. The PTD Loops for Non-Interval Meters and Unmetered usage are used in pairs - The PTD Loops for Unmetered Services require one summary level Loop (PTD*SU) and at least one detail level Loop (PTD*BD). - The PTD Loops for Non-interval Meters require one summary level Loop (PTD*SU) and at least one detail level Loop (PTD*PL). Three PTD Loops are required for Interval Meters, at least one summarized consumption Loop for ERCOT settlement (PTD*PP), at least one summarized Loop per meter per measurement type (PTD*BO), and at least one detail level Loop by interval per meter per measurement type (PTD*PM) . ERCOT uses the PTD*SU (Unmetered and Non-interval Summary) Loop because netted consumption and coincidental peak are reported in these loops. ERCOT uses the PTD*PP (Interval Summary Across Meters) Loop for settlement because netted consumption is reported in this Loop. The PTD*PP is also used to report consumption to the CR and TDSP by ERCOT for ERCOT Polled Services (EPS) In addition, ERCOT uses the PTD*PP to report consumption to the CR and the TDSP for ERCOT Polled Services (EPS). This will be a separate 867_03 containing only the PTD*PP Loop, where BPT01 = to EX. |  |
| --- | --- | --- | --- |
| Non-Interval Meter Reporting |  | Metered Services Information (PTD01 = SU and PL): These Loops are used to convey the usage and demand for metered data, at both a detail level by meter by unit of measure (PTD01=PL) and for each unit of measure at a summary level for all meters (PTD01=SU). Non-Interval and Unmetered Usage Summary (PTD01=SU): All consumption and demand reported in each PTD*SU Loop must have the unit of measure in common. Consumption is summed to the ESI ID level by unit of measure. One PTD*SU Loop is provided (if measured) for coincidental peak demand (as outlined in ERCOT Protocols, Section 18: Load Profiling, Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement). Metered Services Detail (PTD01=PL): One or more PTD*PL is required, one for each unit of measure for each meter. For example, in the case of one meter reporting one unit of measure (kWh), the PTD01=PL will be the same as the PTD01=SU and both must be provided. Another example, if you have two meters and each meter measures kW and kWh, one PTD*SU Loop that sums the kWh is sent. Another PTD*SU Loop is sent for kW. The kWh consumption from Meter 1 and Meter 2 is summed and provided in one PTD*SU Loop. The coincidental peak demand from Meter 1 and Meter 2 is provided in one PTD*SU Loop. Four PTD*PL Loops are required for reporting this scenario at the detail level. One PTD*PL Loop for Meter 1 kWh, a second PTD*PL for Meter 1 kW, a third PTD*PL for Meter 2 kWh, and a fourth PTD*PL for Meter 2 kW. |  |
| Unmetered Reporting |  | Unmetered Services Information (PTD01 = SU and BD): These Loops are used to convey the usage for unmetered data, at both a detail level by unmetered device type (PTD01=BD) and for each device type, at a summary level for the ESI ID (PTD01=SU). Unmetered Usage Summary Information (PTD01 = SU): All consumption reported in each PTD*SU Loop must have the unit of measure in common, summing to the ESI ID level by unit of measure. This Loop is used to convey the total usage for all unmetered devices at a summary level for an ESI ID(PTD01=SU). The usage of all devices reported in the detail level PTD*BD Loop is summed and reported in on PTD*SU Loop. Unmetered Services Summary (PTD01=BD): One PTD per device type per ESI ID. Total Consumption for all unmetered services at the unmetered device detail level. Loop is required for each unmetered device type per each unique wattage and/or lumen reported. Even though some of the consumption may be estimated, the consumption is reported as actual quantity delivered for unmetered services. The detail is required for Unmetered Services. For example, in the case of one ESI ID reporting one device type with one wattage, the PTD01=BD will be the same as the PTD01=SU and both must be provided. Another example: 300 Mercury Vapor Lights (one device type) where 100 lights are 750 watts and 200 are 800 watts. One PTD*BD Loop is sent to provide the Mercury Vapor Device Type reporting 750 watts for the 100 lights, with one REF*PRT reporting the 750 watts and one QTY Loop showing the usage calculation for the 100 lights. Another PTD*BD Loop is sent to provide the Mercury Vapor Device Type reporting 800 watts for the 200 lights, with one REF*PRT reporting the 800 watts and one QTY Loop showing the usage calculation for the 200 lights. The PTD*SU Loop sums the usage of both the 750 watt lights and the 600 watt lights. Only one PTD*SU summing the usage of both the 750 and 800 watt lights at the ESI ID level is provided. |  |
| Interval Meter Reporting |  | Interval Metered Services Information – by Channel or Unit of Measure by Meter (PTD01 = PP, BO and PM): These Loops are used to convey the usage for interval meters at the detail level (PTD*PM) and at two different summarized levels (PTD*PP and PTD*BO). The PTD*BO reports monthly interval consumption at each meter. The ERCOT settlement Loop (PTD*PP) is used to report all consumption summarized across meters at each interval on the ESI ID. Demand is not reported for interval meters in the PTD*PP, PTD*PM, or PTD*BO Loops. Metered Services Detail (PTD01=BO) The PTD*BO Loops sum the intervals for the month by unit of measure for each meter. In the PTD*BO consumption across intervals and across the same unit of measure is summarized at the meter level. Individual intervals are not reported in the PTD01=BO Loop. The PTD01=BO Loop will never report kW or KVAR . Metered Services Summary (PTD01=PP)(Stored by ERCOT): For each PTD*PP Loop consumption is reported at the interval level summarized and netted across meters by unit of measure. This reporting is required by ERCOT systems for settlement. PTD01=PP sums consumption across all meters by interval by unit of measure (kWh and/or kVArh only) by channel. There must be one summary Loop by unit of measurement for all meters provided in the detail. The PTD01=PP provides control totals for the sum of all intervals in the PTD01=PM by unit of measure by channel. For example, if there are two meters on the ESI ID, where Meter 1 measures kW and kWh and Meter 2 measures kWh, there will be one PTD01=PP Loop summarizing kWh across Meter 1 and Meter 2. The kW readings are not reported in the PTD01=PP Loop. For the interval summary, two PTD01=BO Loops are provided: one for kWh on Meter 1 and one for kWh on Meter 2. The kW readings are not reported in the PTD01=BO Loop. For interval detail, two PTD01=PM Loops are provided: one for kWh on Meter 1, and one for kWh on Meter 2. The kW readings are not reported in the PTD01=PM Loop. Metered Services Detail (PTD01=PM): For each PTD01=PM Loop consumption is reported at the interval level by meter by unit of measure by channel. Demand is not reported in the PTD01=PM Loop. This is the lowest level of detail. |  |
| ERCOT Polled Services |  | ERCOT forwards ESI ID consumption to TDSPs and CRs at least once a day for ESI IDs polled by ERCOT. These ESI IDs represent netted load data from ERCOT Polled Settlement (EPS) Meters at Generation Facilities. Metered Services Summary (PTD01=PP)(Created by ERCOT): Only one PTD*PP Loop is reported at the interval level across meters for generation meters (auxiliary net) for kWh. In most instances, ERCOT systems will read multiple meters, perform netting, splitting, adjust for actual transmission losses and populate ESI ID consumption data to be used in settlement. Since ESI ID consumption data for EPS is not read directly from a meter, but is actually an output from the above, MPs have no other source for this data than from ERCOT. Each time the aggregation process takes place, an 867_03 is generated for each ESI ID that ERCOT populates. The 867_03 is then sent to the TDSP and CR. Additionally, the CR and TDSP can expect an 867_03 for each settlement run, so on a daily basis, the CR and TDSP may receive and 867_03 for each EPS-load ESI ID for initial, final and true up in any resettlement day if the values have changed from the last settlement run. The BPT01 in the 867_03 has a value of “EX” – Daily Consumption ERCOT polled services to identify the transaction. This transaction is sent daily by ERCOT. |  |
| TDSP Read Generation |  | TDSP may choose to provide interval energy production data to ERCOT for Distributed Generation (locations producing less than 10 MW that are connected at less than 60KV). This data will be submitted on at least a monthly basis: Metered Services Summary (PTD01=PP)(Created by TDSP): Only one PTD*PP Loop is reported at the interval level for TDSP generation meters 10 MW or less. The TDSP will populate the REF*Q5 with the Resource ID assigned by ERCOT instead of the ESI ID. The Meter Role segment (REF*JH=”S”) is used to indicate to ERCOT that this 867_03 is reporting generation volume. Since there is no CR associated with the RID, this transaction will not be forwarded to the CR. |  |
| Subtract Meter Usage Reporting |  | Reporting Subtract Meter Services Information – by Channel and Unit of Measure by Meter (PTD01 = PP, BO and IA ) when separate ESI IDs have been assigned to the Master Meter and the subtract meter(s): These Loops are used to convey the usage for interval meters when separate ESI IDs have been assigned to the Master Meter and the subtract meter(s). The PTD*BO Loops sum the intervals for the month by unit of measure. Three PTD*BO Loops may be provided: one PTD*BO Loop that provides the meter number for the Master Meter, one PTD*BO Loop to provide combined additive usage with no meter number provided and/or one PTD*BO Loop to provide combined subtractive usage with no meter number provided. The value reported in the ERCOT settlement Loop (PTD*PP) has already been adjusted for any additive/subtractive consumption. Interval Summary (PTD01=BO): For the Master Meter, the PTD*BO Loops sum the intervals for the month by unit of measure. The usage reported for the Master Meter in this loop is not netted. For subtractive meters, each PTD*BO Loop combines consumption reported for all subtractive meters. For the Master Meter, the PTD*BO Loops sum the intervals for the month by unit of measure. The usage reported for the Master Meter in this loop is not netted. For additive meters, each PTD*BO Loop combines consumption reported for all additive meters. Metered Services Summary (PTD01=PP)(Stored by ERCOT): For each PTD*PP Loop consumption is reported at the interval level summarized and netted across all meters by unit of measure. This reporting is required by ERCOT systems for settlement. PTD = IA (Net Interval Usage Summary): The IA loop will provide summarized net interval usage at the ESI ID level. This net usage loop applies only to interval meters. The PTD~IA Loop provides the net of the Interval summary (PTD~BO) loops and also provides a control total for the total of the intervals reported in the Interval Usage Summary Across Meters (PTD~PP) loop. |  |
| Estimated Consumption Adjustments |  | When reporting additional consumption for fast, slow or tampered meters when readings in the PTD*PL do not match the consumption in the MEA of the PTD*PL: Depending on TDSP practice, readings are not adjusted to compensate for missing or abundance of consumption. To provide the adjustment the missing or abundance of consumption is provided in a separate PTD*PL Loop with no meter number provided. The PTD*PL Loops must be netted to derive consumption that matches the meter readings. For estimated consumption adjustments, at least two PTD*PL Loops are provided. One PTD*PL Loop provides consumption that is reported at the meter level by unit of measure and one PTD*PL Loop provides the lump adjustment by meter by unit of measure, but no meter number is provided. Estimated consumption adjustments are included in the consumption reported PTD*SU Loop. Non-Interval and Unmetered Usage Summary (PTD01=SU): All consumption and demand reported in each PTD*SU Loop must have the unit of measure in common. Estimated consumption adjustments are included in the consumption at the ESI ID level by unit of measure. One PTD*SU Loop is provided (if measured) for coincidental peak demand. Metered Services Detail (PTD01=PL): One or more PTD*PL Loops is required, one for each unit of measure for each meter and at least one PTD*PL Loop providing the adjusted consumption. |  |
| Valid Loop Combinations |  | Combination # 1 – Non-Interval Reporting - Non-Interval and Unmetered Usage Summary (PTD01=SU) - Non-Interval Detail (PTD01=PL) Combination #2 – Unmetered Reporting - Non-Interval and Unmetered Usage Summary (PTD01=SU) - Unmetered Detail (PTD01=BD) Combination # 3 –Interval Reporting - Interval Summary (PTD01=BO) - Interval Usage Summary Across meters (PTD01=PP) - Interval Detail (PTD01=PM) Combination #4 – ERCOT Polled Services - Interval Detail (PTD01=PP) Combination #5 – TDSP Polled Services - Interval Detail (PTD01=PP) Combination #6 – Subtractive Usage – Separate ESI IDs – Non Interval - Non-Interval Detail (PTD01=PL) with Meter Number provided - Non-Interval Detail (PTD01=PL) without Meter Number provided that combines all subtract usage - Non-Interval and Unmetered Usage Summary (PTD01=SU) that nets metered usage and subtract usage Combination #7 – Subtract Meter – Separate ESI IDs – Interval - Interval Detail (PTD01=PM) with meter number - Interval Summary (PTD01=BO) with meter number - Interval Summary (PTD01=BO) without meter number that combines all subtract usage for a monthly total - Interval Summary (PTD01=BO) without meter number that combines all additive usage for a monthly total - Interval Usage Summary Across meters (PTD01=PP) that nets metered interval usage and subtract interval usage - Net Interval Usage Summary (PTD01=IA) |  |
| Order Loops are sent Muni/Co-op Estimated Consumption and Meter Readings |  | The PTD Loops may be sent in any order. The Terms and Conditions of Nueces Electric Cooperative and San Patricio Electric Cooperative require that all estimated reads be marked as estimated and that the reason and method for estimating be provided. All estimated reads and consumption will be identified. Nueces Electric Cooperative and San Patricio Electric Cooperative estimated for only one reason: A meter read was not received. Nueces Electric Cooperative and San Patricio Electric Cooperative use only one method for estimating consumption: Same Month Last Year. Estimated Consumption in all PTD Loops will be provided in the QTY segment. The QTY01 will be qualified with the value of “KA” to indicate the consumption is estimated, the reason for the estimation is that a meter read was not received, an the method for estimating was Same Month Last Year. All estimated Meter Reads will be identified in the MEA segment. The MEA01 will be qualified with an “AE” (Actual Beginning Read and Estimated End Read), “EA” Estimated Beginning Read and Actual End Read, or “EE” (Estimated Beginning and End Read) to indicate the one or both reads are estimated, the reason for the estimation is that a meter read was not received, an the method for estimating was Same Month Last Year. |  |

Gaps Overlaps of Usage dates in the 867.

The DTM*150 and DTM*151 is validated by ERCOT for gaps and overlaps. The DTM*151 (Service Period End Date) for Month 1 must equal the DTM*150 (Service Period Begin Date) for Month 2 for Non-IDR meters. IDR meters may have a one day difference for TDSPs who read their IDR meters at midnight. TDSPs are aware of the requirement because of their conversion experience.  CRs did not participate in conversion so it is necessary to alert them of the issue.

Date Ranges in DTM~150 and DTM~151 in the, , PTD*PL, , and PTD*SU must be sent as follows:

| Service Period Start Date | Service Period End Date |
| --- | --- |
| DTM~150~20010501 | DTM~151~20010601 |
| DTM~150~20010601 | DTM~151~20010701 |
| DTM~150~20010701 | DTM~151~20010801 |

Date Ranges in DTM~150 and DTM~151 in the PTD*BO and PTD*PM may have a one day gap as follows if the IDR meter is read at midnight.

| Service Period Start Date | Service Period End Date |
| --- | --- |
| DTM~150~20010501 | DTM~151~20010601 |
| DTM~150~20010602 | DTM~151~20010701 |
| DTM~150~20010702 | DTM~151~20010801 |

Date Ranges in DTM~150 and DTM~151 in the PTD*PP must be sent as follows if the IDR meter is read at midnight:

| Service Period Start Date | Service Period End Date |
| --- | --- |
| DTM~150~20010501~2359 | DTM~151~20010601~2359 |
| DTM~150~20010602~ 0000 or 20010602~0001 | DTM~151~20010701~2359 |
| DTM~150~20010702~ 0000 or 20010702~0001 | DTM~151~20010801~2359 |

Daylight Saving Time in the 867 for Interval Data.

No special designation is required in LodeStar to indicate the observation of Daylight Savings Time or the return to Standard Time.  LodeStar has no problem accepting the 23 hours of interval data in the Spring and the 25 hours of interval data in the Fall.  It is not necessary to modify the 867_02 and 867_03.

Direction on how to transmit 867 Cancels and Restatements for multiple months.

Assume data was sent previously for the following periods and that the usage data for 3/1/01-4/1/01 must be cancelled.

2/1/01 – 3/1/01

3/1/01 – 4/1/01  - Cancel

4/1/01 – 5/1/01

5/1/01 – 6/1/01

6/1/01 – 7/1/01

It is necessary to cancel usage for four periods (3/1/01-4/1/01, 4/1/01-5/1/01 and 5/1/01-6/1/01 and 6/1/0-7/1/01) in order to cancel the usage for 3/1/01-4/1/01.    All cancels must be received prior to restatement.

Cancel Order – Sent in any date order

| 2/1/01 – 3/1/01 |  |
| --- | --- |
| 3/1/01 – 4/1/01 - Cancel | Appears as the first cancellation in the 867 file, or if sending separate files, this is the first cancellation file. |
| 4/1/01 – 5/1/01 – Cancel | Appears as the second cancellation in the 867 file, or if sending separate files, this is the second cancellation file. |
| 5/1/01 – 6/1/01 – Cancel | Appears as the third cancellation in the 867 file, or if sending separate files, this is the third cancellation file. |
| 6/1/01 – 7/1/01 – Cancel | Appears as the fourth cancellation in the 867 file, or if sending separate files, this is the fourth cancellation file. |

It is necessary to restate usage for four periods (3/1/01-4/1/01, 4/1/01-5/1/01 and 5/1/01-6/1/01 and 6/1/01-7/1/01) in order to restate the usage for 3/1/01-4/1/01.  Usage must be restated in the specific order of Historic to Current.

Restatement Order – Historic to Current

| 2/1/01 – 3/1/01 |  |
| --- | --- |
| 3/1/01 – 4/1/01 - Restatement | Appears as the first restatement in the 867 file, or if sending separate files, this is the first restatement file. |
| 4/1/01 – 5/1/01 – Restatement | Appears as the second restatement in the 867 file, or if sending separate files, this is the second restatement file. |
| 5/1/01 – 6/1/01 – Restatement | Appears as the third restatement in the 867 file, or if sending separate files, this is the third restatement file. |
| 6/1/01 – 7/1/01 – Restatement | Appears as the fourth restatement in the 867 file, or if sending separate files, this is the fourth restatement file. |

ERCOT Validation

TX SET requires the original transaction number must be provided in the BPT09 of the 867 Cancellation.  ERCOT ensures that the BPT09 is populated in the 867 Cancellation, but does not validate that the value provided in the Cancellation matches the value provided in the original 867 usage provided.

ERCOT does validate that the period being cancelled matches the original period, e.g. canceling period 3/1/01-4/1/01 is valid, but canceling the period 3/1/01-4/15/01 is not valid and ERCOT would reject.  ERCOT cannot cancel the period 3/1/01-4/15/01 because that period was not originally defined in LodeStar.  ERCOT would also reject a cancellation for the period 3/1/01-6/1/01 because it spans several periods.

TX SET requires that an individual 867 transaction is required for each period that is cancelled and/or restated.  When canceling three months of data, three separate 867 transactions must be sent, one for each period.  When restating three months of data, three separate 867 transactions must be sent, one for each period.  ERCOT will reject a cancellation for the period 3/1/01-6/1/01 because it spans several periods.

If an MP attempts to cancel a period that is not in the database, ERCOT will generate an 824 EDI response with an error code of:  DDM (Dates Do Not Match).

ERCOT validates the date ranges on the 867 restatement.  In the example above, after the three periods were cancelled, the last period of original usage remaining in LodeStar is 2/1/01-3/1/01.  ERCOT ensures that the beginning date of the restatement period matches the end date of the original usage, e.g., 2/1/01-3/1/01 (end date of 3/1/01) 3/1/01-4/1/01 (beginning date of 3/1/01).

Compliant Restatement

| 2/1/01 – 3/1/01 | End Date 3/1/01 must equal |
| --- | --- |
| 3/1/01 – 4/1/01 - Restatement | Begin Date must 3/1/01= End date of previous period: 3/1/01 |
| 4/1/01 – 5/1/01 – Restatement | Begin Date must 4/1/01 = End date of previous period: 4/1/01 |
| 5/1/01 – 6/1/01 – Restatement | Begin Date must 5/1/01 = End date of previous period: 5/1/01 |
| 6/1/01 – 7/1/01 – Restatement | Begin Date must 6/1/01 = End date of previous period: 6/1/01 |

However, ERCOT cannot ensure that only one period is restated.  ERCOT would not reject the following, although it is not valid because it is not TX SET compliant.  Even though ERCOT would not reject the following, CRs would reject.

Noncompliant Restatement (cancellation spans several periods)

| 2/1/01 – 3/1/01 | End date 3/1/01 equals |
| --- | --- |
| 3/1/01 – 7/1/01 - Restatement | Beginning Date 3/1/01 – Not valid because the restatement is for more than one period. ERCOT does not reject, but restating more than one period is not TX SET compliant. CRs will reject. |

Adjustment to Initial Reads (867_04)

The start date of the 867_04 and the beginning date of the first 867_03 will always be the same.

The 867_04 provides initial read information.  The Texas SET EDI Implementation Guidelines do not support sending a corrected or adjusted Initial Meter Read Notification.  If the initial meter read requires a correction or adjustment, it will be done using the Measurements (Meter Reads) and/or Service Period Start information from the first 867_03.

If there is a discrepancy between the 867_04 and the first 867_03 after the switch or move-in has been completed, the CR should use the starting information from the 867_03 as the initial meter read information.