| PRR Number | 589 | PRR Title | CSC and Zone Determination |
| --- | --- | --- | --- |
| Date |  | April 20. 2005 |  |
| Submitter’s Information |  |  |  |
| Name |  | Beth Garza |  |
| E-mail Address |  | bgarza@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-6361 |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | No impact | No impact | Greater over authority over CSC determination process. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Unknown |
| LSE: General, Including NOIE | Unknown | Unknown | Unknown |
| LSE: CR & REP | Unknown | Unknown | Unknown |
| QSE | Unknown | Unknown | Unknown |
| Resource | Unknown | Unknown | Unknown |
| TDSP | None | None | None |

Comments

Based on input received at the April 13, 2005 Congestion Management Working Group meeting, ERCOT has revised the proposed language to this PRR to reflect the following comments:

- Actual or projected congestion cost is a factor that may be used to identify candidate CSCs, rather than being an unspecified threshold.

- ERCOT may identify CSC candidates based on their operational experience.

- The competitiveness test originally proposed in this PRR should be replaced with a description of the factors to be assessed.

- The two hard, but unspecified thresholds should be recast as criteria to be evaluated in determining Congestion zones.

- The proposed revisions to Section 7.2.1.3 should be deleted.  These proposed revisions have been included as comments to PRR 587 (Midyear Modification of CSC or CRE Definitions.

Revisions proposed as a result of these comments have been high-lighted in yellow.

Revised Proposed Protocol Language

7.2	CSC Zone Determination Process

By November 1 of each year, the appropriate ERCOT subcommittee will report to the TAC and ERCOT Board with recommended CSC designations, resulting Congestion Zone boundaries, CRE designations and associated Boundary Generation Resources for ERCOT Board review and approval.

Shortly after ERCOT Board approval, CSCs and resulting Congestion Zone boundaries and CRE designations will be posted on the MIS.  This posting will include a station and bus-by-bus identification of each Congestion Zone.

7.2.1	Process for Determining CSCs

ERCOT will reassess CSCs annually and will identify Congestion Zones based on the reassessed CSC’s by November 1 of each year.  ERCOT staff will use the following process to identify CSC’s:

(1)	By August 1, of each year, ERCOT staff will complete an analysis of Load flow data using the latest Steady State Working Group Data Set A summer peak case and will determine expected operating limits, candidate CSCs and associated constraints to be used in the designation of CSCs for the upcoming calendar year using the following steps:

(a) 	Determination of new CSC candidates:

    - Candidate CSCs may be determined using transfer analysis between Study Zones.  Study Zones are groupings of buses used to test transfer limits using power system simulation tools.  These Study Zones can be either of the following: (1) reasonable approximations of the final Congestion Zones; Weather Zones, SSWG Areas or Zones; or (2) any grouping of buses by geographic region or generation/Load pocket.  Candidate CSCs will often be the contingency that repeatedly causes multiple limiting elements to reach 100% of their emergency rating at high transfer levels.  If a transmission line is repeatedly the limiting element under multiple contingencies it will be considered a CSC candidate.

    - Candidate CSCs may also be identified through an evaluation of the actual annual Congestion costs from the prior year.  For this purpose, Congestion costs include the cost of any zonal Balancing Energy Service, RMR Service, OOMC Service, Replacement Reserve Service, OOME Service, or Resource-specific Dispatch Instructions used to resolve Congestion on that particular transmission path in that particular direction during the prior year.  ERCOT may also identify CSC candidates by considering projected annual Congestion costs for the CSC designation year.  Since this type of data reflects past system topology and generation, ERCOT will also consider generation and transmission system changes expected to be in service in the CSC designation year.

    - ERCOT may also identify candidate CSCs based on operational experience.

(b)	If no new candidate CSCs are considered other than the immediately preceding year’s CSCs, then ERCOT shall use the Congestion Zones from the immediately preceding year as Study Zones.

(c) 	If new candidate CSCs are considered, then ERCOT shall calculate Shift Factors and perform cluster analysis based on the candidate CSCs to create Study Zones.

(d)	ERCOT shall perform system simulation studies with transfers between the Study Zones and find constraints on the transmission system to determine if candidate CSCs are appropriate for designation as CSCs.

(e)	ERCOT shall determine whether the candidate CSCs qualify for CSC designation based, in part, on the criterion that there must be a sufficiently competitive market to resolve Congestion on the transmission path to be considered for CSC designation.    One factor that will be considered when assessing whether a sufficiently competitive market exists is the concentration of capacity ownership by Generation Entity, compared to the distribution of shift factor impacts.

(2)	CSC approval process:  The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the list of constraints should be recommended for approval to the TAC and the Board.

(3)	Thirty (30) days prior to TAC review, a Non Opt-In Entity may request an exemption to its zonal placement by submitting a request to ERCOT for consideration.  ERCOT staff will evaluate the merits of the request and authorize the exemption based on the following criteria:

(a)	Exemptions shall not cause significant operational impact to the ERCOT System.  This will normally be satisfied if the request will not result in a change in the annual average Shift Factor for the original or requested Congestion Zone by more than five percent (5%).   After the proposed move, the Zonal Shift Factor in each of the original and requested Congestion Zones shall be no less than ninety-five percent (95%), and no more than one hundred and five percent (105%)], of its original Zonal Shift Factor before the proposed move.

Because only buses with generation have an impact on the calculation of Zonal Shift Factors, if no generation exists at a bus that is part of an exemption request, a token generator of one (1) MW will be placed at that bus to test the impact of moving that bus from one (1) zone to another.  This will be done to all buses that make up a request for exemption.  For example, if an Entity is requesting an exemption for five (5) buses and only one (1) bus has generation On-line in the Load flow case used to test the impact, one (1) MW of generation will be placed at the four (4) other buses and the impact of all five (5) buses on Zonal Shift Factors will be determined as a group as opposed to individually.  The Load flow case used to determine this impact will be the same Load flow case used to calculate annual TCR quantities.

(b)	Exemptions shall not have a commercial impact/gain for the requestor.  This will be measured by whether or not the scheduling which would otherwise result would be eligible for Pre-assigned Congestion Rights.

7.2.2	Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT staff will take the following steps to determine Congestion Zones and Zonal Shift Factors:

(1)	Shift Factors are developed using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

(2)	A statistical clustering analysis will be used to aggregate transmission buses into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); and (ii) station IDs as provided by TDSPs in Protocol Section 15.1.2.5, Response from TDSP to Registration Notification Request, can only be assigned into one Congestion Zone.

The following measurements will be calculated for each combination of zones: (i) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone ; and (ii), the R2 (coefficient of determination) from the cluster analysis.

(3)	If the clustering process described in (2) results in the splitting of two or more buses within any single station into different zones, then the following process will be used to adjust the zone selection to place all buses within a single station in the same zone:

(i)	For Generation Resource plant stations, determine which zone contains the largest amount of generating capacity of the station and assign all buses of that station to that zone;

(ii)	For all other stations, determine which zone is assigned the largest total amount of Customer Load of the buses in the station and assign all buses of that station to that zone;

(iii)	If there is no Load or generation in the station, or if the amount of generation and/or Load assigned to individual station buses are equal, then the Load or generation shall be assigned to the zone containing the lowest bus number of the station.

(4)	Zonal Shift Factors for each CSC will be determined by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the Generation Resource megawatt levels on the bus in the Load flow base case used to determine CSC Limits.

The appropriate ERCOT TAC Subcommittee will determine if the above process was followed.   If it was followed, the resulting Congestion Zones will be recommended for approval to the TAC and the Board.

The revisions proposed for this section have been included in the proposed revisions presented in ERCOT’s comments to PRR 587 (Midyear Modification of CSC or CRE Definitions.