| PRR Number | 575 | PRR Title | Mandatory Down Balancing Ramp Rate |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Recommended Action |  | Approve |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 4.5.2 Receipt of QSE’s Balancing Energy Bid Curves |  |  |
| Proposed Effective Date |  |  |  |  |
| Priority and Rank Assigned |  |  |  |  |
| Revision Description |  | The proposed new subsection 4.5.2(3) would clarify that the specified ramp rate may not be less than the Market Participant minimum required down Balancing Energy bid divided by forty(40), and the Mandatory Down Balancing Bid amount should be deployable within one hour. |  |  |
| Overall Market Benefit |  | Increased ERCOT system reliability. |  |  |
| Overall Market Impact |  | None |  |  |
| Consumer Impact |  | None |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. ERCOT credit staff and the CWG have reviewed PRR575 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - PRR 575 and the Impact Analysis (IA) was posted on 2/14/05. - PRS considered this PRR on 3/17/05. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | PRS voted to recommend approval of PRR575 as amended by PRS and ERCOT. PRS passed the motion with two opposing votes from the Independent Power Market segments and six abstentions from the Independent Power Generator and IOU segments. All segments were present for the vote. |  |  |
| Summary of PRS discussion |  | PRS discussed the potential impact of the one-hour interval on QSEs with large portfolios, small generators, system stability, and ramp rates. ERCOT Staff stated that PRS could insert a deliverability requirement in the PRR and that ERCOT would be comfortable changing the interval to one hour. PUCT Staff commented that PRS must be certain that ERCOT will have enough down balancing energy, particularly as QSEs may be abusing the ability to select down ramp rates. PUCT Staff stressed the need for a balance between strong rules and compliance monitoring. |  |  |
| TAC Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  |  |  |  |
| Summary of TAC discussion |  |  |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | None | None | Increased system reliability |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Increased system reliability |
| LSE: General, Including NOIE | None | None | Increased system reliability |
| LSE: CR & REP | None | None | Increased system reliability |
| QSE | Not Known | Not known | Increased system reliability |
| Resource | Not Known | Not Known | Increased system reliability |
| TDSP | Not Known | Not Known | Increased system reliability |

| Original Sponsor |  |
| --- | --- |
| Name | Joel Firestone |
| Email | jfirestone@ercot.com |
| Company | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| LCRA 021505 | Objects to implementation of this PRR. |
| Texas Genco 031505 | Comments that the PRR may have negative impacts on frequency control. |
| TXU Energy 031505 | Objects to ERCOT’s description of the benefit of this PRR. |
| Air Liquide 032105 | States that it neither reasonable, nor practical to expect all resources to behave in the same manner. |

Proposed Protocol Language Revision

4.5.2	Receipt of QSE’s Balancing Energy Bid Curves

ERCOT will receive QSEs Balancing Energy bid curves by the end of each Adjustment Period for an Operating Hour.  QSEs may voluntarily submit Balancing Energy Service Up and Down bid curves to ERCOT for use in the Operating Period.

QSEs shall be required to submit mandatory Balancing Energy Service Down bid curves.  The QSEs will submit Balancing Energy Service Down bids based on the specifications set forth below.

(1)	For each Congestion Zone, the minimum bid shall be the lesser of the following:

(a)	ERCOT’s required percentage of Balancing Energy Service Down bids as posted at 0600 of the Day Ahead multiplied by the QSE’s net energy schedule for Balancing Energy Service requirements.

(b)	The QSE’s net energy schedule for Balancing Energy Service requirements less the QSE’s minimum sustainable capacity (except for OOMC units and RMR Units) represented in its Resource Plan.

MinimumDownBalanceBidByZone=Min(NetEneSch∗DBESReq%),(NetEneSch−uOLMinCap)

(2)	If the sum of the Balancing Energy Service Down bids for each Congestion Zone from (1) above is more than the QSE’s net energy schedule for Balancing Energy Service requirements less the QSE’s minimum sustainable capacity (except for OOMC units and RMR Units) represented in its Resource Plan, and less the QSE’s scheduled Regulation Down Service, then this amount shall be the minimum ERCOT-wide Balancing Energy requirement.  In this case, no zonal requirement shall exist as the requirement will be ERCOT-wide.

MinimumDownBalanceBidERCOTWide=Max0,zNetEneSch−uOLMinCap−SchRegDn

The net energy schedule for Balancing Energy Service requirements is the QSE’s scheduled energy per Congestion Management Zone less any QSE trades, energy scheduled by ERCOT from Resources under an RMR Agreement, and energy scheduled from Resources that received an OOMC Dispatch Instruction for the hour.

Where:

NetEneSch = 	the QSE’s scheduled energy per Congestion Management Zone less any QSE trades, energy scheduled by ERCOT from Resources under an RMR Agreement, and energy scheduled from Resources that received an OOMC Dispatch Instruction for the hour.

OLMinCap = 	Low Sustainable Limit for each On-line and available unit (except for RMR Units and OOMC units).

SchRegDn = 	Regulation Service Down scheduled by the QSE

DBESReq% = Balancing Energy Service Down percentage posted by ERCOT

Z= 	Congestion Management Zone

u = 	unit

Balancing bid curves will have the following components:

(a)	$/MWh

(b)	Quantity (MW)

(c)	Congestion Zone

(d)	Ramp Rate

3)	The specified ramp rate may not be less than the Market Participant minimum required down Balancing Energy bid divided by forty(40).  The calculated minimum required down Balancing Energy bid amount must be deployable within one hour.

[PIP 210: When block deployment for Loads acting as a Resource can be implemented, add the following paragraph:] A Load acting as a Resource has the option to request a Load bid to be deployed only as a complete block. The block deployment option shall be selected at the time of bid submittal.