| PRR Number | 565 | PRR Title | Calculation of Losses for Settlement |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Recommended Action |  | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 13.1.1 - Responsibility for Transmission and Distribution Losses 13.1.2 - Calculation of Losses for Settlement 13.3.1 - Loss Factor Calculation 13.3.1.1 - Method 1 Settlement Interval Distribution Loss Factors Calculated by ERCOT 13.3.1.2 - Method 2 Settlement Interval Distribution Loss Factors Submitted by Distribution Service Provider |  |  |
| Proposed Effective Date |  | Upon system implementation. |  |  |
| Priority and Rank Assigned |  | Priority 3.1 Rank 111.5 |  |  |
| Revision Description |  | Allow Distribution Losses to be calculated using actual rather than forecasted ERCOT load. Forecasted ERCOT load frequently differs from actual, the change will result in accurate loss calculations and improvement in UFE, also the basis for calculating distribution losses will be consistent with the basis used for transmission losses. |  |  |
| Overall Market Benefit |  | ERCOT will provide accurate calculations of Distribution Losses for Settlements. |  |  |
| Overall Market Impact |  | ERCOT will provide accurate calculations of Distribution Losses for Shadow Settlements. |  |  |
| Consumer Impact |  | None |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. ERCOT Credit staff and the CWG have reviewed PRR 565 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - RMS endorsed a draft of PRR 565 on 12/09/05. - PRR 565 was posted on 12/22/04. - PRS considered on 01/20/05. - ERCOT’s comments posted on 02/10/05. - PRS considered on 02/17/05. - Additional comments were submitted by ERCOT and posted on 03/08/05, no other party submitted comments. - On 03/17/05 PRS reviewed the impact analysis and prioritized PRR565. - On 04/07/05 TAC voted to recommend approval of PRR 565. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 02/17/2005 PRS unanimously voted to recommended approval of PRR565 as amended by ERCOT’s comments. All market segments were present. On 03/17/2005 PRS recommended approval for PRR565 as modified by ERCOT with one abstention. All market segments were present. Also on 03/17/2005 PRS voted to assign PRR565 a priority 3.1 and a rank of 111.5 with one abstention. All market segments were present. |  |  |
| Summary of PRS discussion |  | On 01/20/05 PRS stated that the language for PRR565, Calculation of Losses for Settlements, was not clear and there are discrepancies between the description and the actual revision. Other concerns were stated that a Market Participant could be advantaged by the use of actual rather than forecasted Load data for the Calculation of Losses. PRS requested that ERCOT staff provide additional data and revised language at its February meeting. On 02/17/2005 ERCOT staff provided data and clarification to the language for PRR565. On 03/17/2005 PRS discussed ERCOT comments and assigned a priority and rank to the PRR. |  |  |
| TAC Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | TAC voted unanimously to recommend approval of PRR 565 with all Market segments present. |  |  |
| Summary of TAC discussion |  | TAC was concerned with the term use of deemed transmission losses. ERCOT explained the use of defined terms in the Protocol and pledged to review PRR 565 to ensure proper use of terms. |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | None | Changes to how DLFs are calculated | More accurate application of distribution loss calculations and settlements |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | More accurate application of distribution loss calculations and settlements |
| LSE: General, Including NOIE | None | None | None |
| LSE: CR & REP | None | Changes to how DLFs are calculated in shadow settlement systems | More accurate application of distribution loss calculations and settlements |
| QSE | None | Changes to how DLFs are calculated in shadow settlement systems | More accurate application of distribution loss calculations and settlements |
| Resource | None | none | none |
| TDSP | Method 2 option eliminated | None | None |

| Original Sponsor |  |
| --- | --- |
| Name | On behalf of PWG by PWG Chair Ernie Podraza |
| Company | Reliant Energy |
| Segment |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| ERCOT (02/10/05) | Comments support this PRR. |
| ERCOT (03/08/05) | Comments support this PRR. |

Proposed Protocol Language Revision

13.1.1	Responsibility for Transmission and Distribution Losses

T&D Losses are the responsibility of the QSE representing the Competitive Retailer’s Load.  The QSE will schedule the necessary amount of energy to cover Competitive Retailer’s Load plus the applicable T&D Losses.  ERCOT shall allocate T&D Losses to Load at the appropriate aggregate level as part of the data aggregation process to calculate the Load obligation of QSEs for settlement purposes.

Transmission Loss Factors shall be forecasted by ERCOT, and posted to the MIS by 0600 of the Day Ahead period.  ERCOT shall forecast the ERCOT-wide Transmission Loss Factors expressed as a percent of Load for each settlement interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Transmission Loss Factors for each Settlement Interval using actual ERCOT System Load for that Settlement Interval and will post the resulting deemed actual Transmission Loss Factors to the settlement system and the MIS.

Distribution Loss Factors shall be forecasted by ERCOT, and posted to the MIS by 0600 of the Day Ahead period.  ERCOT shall forecast the Distribution Loss Factors expressed as a percent of Load for each Settlement interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Distribution Loss Factors for each settlement interval using actual SRCOT System Load for that Settlement Interval and post the resulting deemed actual Distribution Loss Factors to the settlement system and the MIS.

Distribution Loss Factors will be subject to audit by ERCOT for accurate and consistent application.  Non-Opt-in Entities (NOIE) with Interval Data Recorders at the settlement point of delivery are not required to provide Distribution Loss Factors.

In the special case where there are distribution Facilities upstream from a wholesale NOIE settlement IDR that settlement IDR will be compensated for line and transformer losses between the IDR and the ERCOT Transmission Grid to account for the Distribution Losses.  The NOIE will be then treated as a transmission level NOIE.  Calculations are subject to review by ERCOT.  Since loss compensation is included in the wholesale settlement IDR, the TDSP providing upstream wheeling facilities may need to offer wholesale wheeling tariffs excluding the losses that have already been compensated for.

13.1.2	Calculation of Losses for Settlement

ERCOT shall use the deemed actual Distribution Loss Factors applicable to each ESI ID and the deemed actual Transmission Loss Factors when adjusting aggregated Load for losses to determine the QSE total Load obligations.

13.2	  Transmission Losses

13.2.1		Forecasted Transmission Loss Factors

The forecasted Transmission Loss Factor for each interval in the Operating Day shall be a linear interpolation or extrapolation using the on-peak and the off-peak Transmission Loss Factors and the corresponding forecast of ERCOT System Load during the same interval to calculate the loss factors .

At 0600 of the Day Ahead period, ERCOT shall forecast a Transmission Loss Factors for each Settlement Interval of the Operating Day and post the forecasted Transmission Loss Factors which correspond to the Operating Day forecast.  The source of the on-peak and off- peak losses are the ERCOT load flow base cases for the applicable season.

13.2.2	Deemed Actual Transmission Loss Factors

ERCOT will determine the deemed actual Transmission Loss Factor for each interval in the Operating Day, by use of a linear interpolation or extrapolation using the on-peak and the off-peak Transmission Loss Factors corresponding to the actual ERCOT System Load for the interval.

The day after the Operating Day, ERCOT shall calculate deemed actual Transmission Loss Factors for each Settlement Interval of the Operating Day and post the Transmission Loss Factors to be used in settlement calculations.

ERCOT shall use the Transmission Loss Factors corresponding to the on-peak and off-peak base case ERCOT System Loads during the applicable seasons as the basis for the ERCOT-wide deemed actual Transmission Loss Factors.  ERCOT will post Transmission Loss Factors to the MIS by 0600 two (2) days after the Operating Day.

13.2.3	Transmission Loss Factor Calculations

The following formulas shall be used to translate the seasonal on-peak and off-peak Transmission Loss factors into Settlement Interval Transmission Loss Factors.

TLFi	=	(SSC * SIELi) + SIC

Where:

i =	interval

TLFi = 	Transmission Loss factors for a settlement interval

SIELi =	Settlement Interval ERCOT System Load (forecasted or actual)

SSC =	Seasonal Slope Coefficient

SIC =	Seasonal Intercept Coefficient

And,

SSC 	=	(SONLF – SOFFLF)/(SONL-SOFFL)

SIC	=	[(SOFFLF*SONL)-(SONLF*SOFFL)]/(SONL-SOFFL)

Where:

SONLF =	Seasonal on-peak percent loss factor

SOFFLF =	Seasonal off-peak percent loss factor

SONL =	Seasonal on-peak Load value

SOFFL =	Seasonal off-peak Load value

13.2.4	Seasonal Transmission Loss Factor Calculation

Seasonal on-peak and off-peak Transmission Loss Factors are derived from annually updated ERCOT on-peak and off-peak load flow base cases analysis by ERCOT.  Base cases reflect the most current data on the transmission system and Generation Resource dispatch.  The ERCOT Transmission Grid topology and related Generation Resource dispatch in the base cases are the critical factors in calculating losses.  Seasonal time periods are defined as follows:

- 1)	Spring   (March – May)

- 2)     	Summer (June – August)

- 3)	Fall (September – November)

- 4)	Winter (December – February)

ERCOT shall calculate seasonal Transmission Loss Factors by dividing ERCOT seasonal case transmission losses (60 kV system and higher) by ERCOT seasonal base Load adjusted (reduced) for self serve Load modeled in the case.  The resulting loss factors are expressed as a percentage of Load.

ERCOT shall post, to the MIS, seasonal Transmission Loss Factors thirty (30) days prior to the start of the season.

13.2.5	Loss Monitoring

ERCOT shall monitor Transmission Losses annually and will investigate abnormal loss factors.  ERCOT and Transmission Service Providers shall use the cost of losses as one criterion in evaluating the need for transmission additions.

13.3	    Distribution Losses

By October 30th of each year for the next calendar year, or two (2) months prior to the posting of any update to the approved Distribution Loss Factor codes and calculation each Distribution Service Provider (DSP), except NOIEs, shall calculate and provide ERCOTth of each yearoribution Loss Factors to be applied to distribution voltage level Loads in its area of certification.  ERCOT shall review and approve the Distribution Loss Factor calculation methodology   used by each DSP prior to use of the los factors for settlement purposes.  If the Distribution loss Factor calculation methodology does not conform inth ERCOT’s interpretation of the pPotocol criteria in this subsection, ERCOT will work with the Distribution Service Provider to correct the deficiency.  Whiil deficiencies are resolveding deient approved  Distribution Loss FactorDLFs and the calculation methodology will be posted, and the last aved DLFs shall be used for settlement.  A DSP may only submit a change to the DLF Distribution Loss Factor calculation methodology submitted by Day only be made on an annually basis or when a change in a DSP service area warrants an updator basehe DSP internal evaluation.

The TDSP shall assign a Distribution Loss Factor code to each ESI ID.  A maximum of five (5) Distribution Loss Factors may be submitted for each Distribution Service Provider based upon ERCOT approved parameters, such as service voltages or number of transformations.

The following coding standards will be used to identify the Distribution Loss

Factor applicable to each ESI ID:

- T = Transmission connected Customers (no Distribution Loss Factor applied)

- A through E = TDSP defined Customer segment(s)

The Distribution Service Providers, except NOIEs, are obligated to provide Distribution Loss Factors to ERCOT.  ERCOT will post the Distribution Loss Factor and calculation methodology, including any equations and constants, for each DSP.

Loss factor variables submitted by the Distribution Service Provider shall include:

(1)	The annual Distribution Loss Factor for each DLF code;

(2)	The methodology upon which the calculation was made;

(3)	The annual interval average Load for the entire DSP service area used to calculate the annual loss factor for each DLF code;

(4)	The portion of the loss factor that is not sensitive to Load for each DLF code;

13.3.1 Loss Factor Calculation

ERCOT shall use the DLFs submitted by the DSP to calculate the Settlement Interval Distribution Loss Factors.  Distribution Loss Factors will be calculated from the data provided by Distribution Service Providers as follows using the following equation:

SILFi = ADLF * [ K+(1-K)*(SIELi/AAL)]

Where:

i =	interval

SILFi =	Settlement Interval Distribution Loss Factor

ADLF =	Annual Distribution Loss factor

K =	a factor provided by DSP between 0.0 and 1.2

SIELi =	Settlement Interval ERCOT System Load (forecasted or actual)

AAL =	Annual Interval Average ERCOT System Load

And where:

AAL =	Annual Total System MWh / (Number of Settlement Intervals in the year)

ERCOT shall use the deemed actual Distribution Loss Factors calculated for each Settlement Interval of the Operating Day for settlement purposes.

13.3.2	Loss Monitoring

Distribution Loss Factors for all Distribution Service Providers, except for NOIEs, will be submitted to ERCOT and will be subject to audit for accuracy and consistency of application.

13.4	Special Loss Calculations for Settlement & Analysis

13.4.1	Deemed Actual Transmission Losses for NOIEs

All QSEs representing Load, including NOIEs, will be responsible for Transmission Losses allocated in the manner described in these Protocols.  Those Entities using transmission tie line meters to determine Load will adjust the net meter readings to remove calculated Transmission Losses behind the meter in order to determine the Load responsibility of the Entity.  ERCOT will provide to settlement the calculation of the losses behind the meters, for each interval, using actual system conditions for that interval.

The deemed actual Transmission Losses for NOIEs shall be a linear interpolation or extrapolation between the seasonal on-peak and the seasonal off-peak NOIE Transmission Loss Factors corresponding to the actual NOIE system load in the interval.

ERCOT shall calculate seasonal NOIE Transmission Loss Factors corresponding to the on- peak and off- peak base case NOIE system loads during each of the four (4) seasons of the upcoming year as the basis for the NOIE Transmission Loss Factors.  NOIE seasonal loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT wide Transmission Loss Factors.

13.4.2	Deemed Actual Transmission Losses for UFE Analysis

ERCOT shall adjust Net Generation data used for UFE analysis zones that contain Transmission Facilities behind any metering points  to the ERCOT wide Transmission Loss Factor.  This adjustment requires reducing the Net Generation by the calculated deemed actual MWh of Transmission Losses and adding back the ERCOT wide Transmission Loss Factor translated into a MWh value.  ERCOT shall provide the calculation of the deemed actual Transmission Losses behind the UFE zonal meters, for each interval, to settlement using actual system conditions for that interval.

The deemed actual Transmission Losses for UFE analysis zones shall be a linear interpolation or extrapolation between the seasonal on-peak and the seasonal off-peak UFE analysis zone Transmission Loss Factors corresponding to the actual UFE analysis zone metered load in the interval.

ERCOT shall calculate seasonal UFE analysis zones Transmission Loss Factors corresponding to the on- peak and off- peak base case UFE zone system loads during each of the four (4) seasons of the upcoming year as the basis for the UFE analysis zone Transmission Loss Factors.  UFE analysis zone seasonal loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT wide Transmission Loss Factors.