| ## Review Weather Information in the POS |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | This procedure is performed prior to the 0600 system conditions forecast posting. |
| 1 | Bring up the following web page on Internet Explorer: http://mosdtd.ercot.com/wstatus/weather_status.cfm The Weather Bank phone number is (405) 330-0919. - The date and time of last successful database load should be less than one (1) hour old; if it is greater than one (1) hour old, NOTIFY the Shift Supervisor and/or the Help Desk to notify the Web Portal On Call person. |
| 2 | Determine if 2-Day Ahead scheduling is needed, based upon the two day out weather forecast. |
| NOTE: | Until the 2-Day Ahead scheduling system is made operational, review the 2-day out weather forecast and determine whether a 2-Day Ahead weather event is imminent. The trigger will be a severe cold front, which along with other operational concerns, may cause gas supply problems; discuss with Shift Supervisor. |
| 3 | Periodically during the day, repeat step one to ensure the weather updates are running. |
| 4 | LOG any significant problems including any action taken. |

| ### 2.5.2 Issue an Advisory |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Send to TDSPs and QSEs for information purposes; however ERCOT may increase ancillary service requirements and/or increase the Day Ahead market to two days ahead. Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Advisory per applicable text in Appendix 4 IF: System Advisories: a. When ERCOT recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability. b. When weather or ERCOT System conditions require more lead-time than the normal Day Ahead market allows. - Transmission system conditions indicate ERCOT is likely to exceed established limits that would ensure reliability in the occurrence of a single event (ERCOT is likely to exceed first Contingency Criteria). (Appendix 1: First Contingency Criteria) d. There is a hurricane in the Gulf with expected landfall in Texas. e. A cold front is approaching with temperatures anticipated to be in mid to low 20o F range and maximum temperature expected to remain near or below freezing impacting 50% or more of major metropolitan areas. f. There is the probability for icing conditions to affect major transmission systems. g. A situation is emerging that has the reasonable expectation of leading ERCOT into an insecure state. - Loss of communications or control condition is anticipated or significantly limited. - If ERCOT is unable to comply with any of the market timing deadlines in the Day Ahead or Adjustment Period, or omits one or more procedures in the scheduling process (Protocols 4.8). j. Longer-range (approx. 48 hours) notifications are needed to ensure the secure operation of the ERCOT System. The notifications are to alert Market Participants (MPs) to a potential threat to reliable operations. This would include, among other operational concerns, the lead-time necessary for dual fuel generating units to prepare for fuel supply problems in the event of anticipated severe cold weather. |
| 2 | USE the Emergency Notification System AND ISSUE the Advisory. (Short version – 32 characters). |
| 3 | POST the long version of the Operating Condition Notice on the Market Information System (MIS) using Appendix 4 as a typical guide and initiate a hotline call to all Transmission Operators and QSEs (or see that another operator does so). Make a printout of the hotline log and save in the daily log file. |