| PRR Number | 592 | PRR Title | Modify Shift Factor Calculation to Exclude Fixed Output Generators |
| --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Urgent | Recommended Action | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | - Overview of ERCOT Congestion Management 7.2.1.2 Congestion Zone & Zonal Shift Factor Determination Methodology 7.2.1.4 Determining Generation Resources Deemed Likely to Vary Their Output [new] 7.3.4.1 Balancing Energy CSC Congestion Charge |  |
| Proposed Effective Date |  | November 1, 2005 |  |
| Priority and Rank Assigned |  | n/a |  |
| Revision Description |  | Modifies the calculation of the monthly zonal Shift Factors used to manage and settle zonal congestion. This PRR would address Recommendation No. 3 of Potomac Economics’ recommendations to improve ERCOT’s wholesale market design. |  |
| Overall Market Benefit |  | May be impacted financially either negatively or positively. |  |
| Overall Market Impact |  | May be impacted financially either negatively or positively. |  |
| Consumer Impact |  | May be impacted financially either negatively or positively. |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. ERCOT credit staff and the CWG have reviewed PRR592 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| Procedural History |  | - PRR592 was posted on 4/5/05. - ERCOT posted comments on 4/13/05. - On 4/18/05, PRS denied the request for urgency status by email vote. - On 4/21/05, PRS voted to grant urgency status and considered this PRR. - On 5/19/05, PRS again considered this PRR. |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 4/21/05, PRS voted to defer action to its regularly scheduled May meeting; PRS voted unanimously in favor of the motion. All segments were present for the vote. On 5/19/05, PRS voted to recommend approval of this PRR as amended by ERCOT. The motion passed with two abstentions from the Independent Generator and IPM segments. All market segments were present for the vote. |  |
| Summary of PRS discussion |  | At the 4/21/05 meeting, the participants discussed that additional comments from the CMWG had not been completed in time for posting for this meeting. Deferral would not delay implementation because this PRR will not be effective until November, 2005. On 5/19/05, the participants echoed CMWG’s concern that this PRR may not have an impact (negative or positive) on the market, other than implementing Potomac Recommendation No. 3. PRS noted that there was no change to the ERCOT impact analysis for PRR592 and that no project prioritization was required. |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Under Option 2: Additional manual calculations to be performed every month to determine which generators to exclude from the calculation of zonal shift factors. | No impact expected | Zonal shift factors will only include the effects of generators expected to change their output level. |
| MARKET SEGMENT |  |  |  |
| Consumer | No impact expected | No impact expected | May be impacted financially either negatively or positively. |
| LSE: General, Including NOIE | No impact expected | No impact expected | May be impacted financially either negatively or positively |
| LSE: CR & REP | No impact expected | No impact expected | May be impacted financially either negatively or positively |
| QSE | No impact expected | No impact expected | May be impacted financially either negatively or positively |
| Resource | No impact expected | No impact expected | May be impacted financially either negatively or positively |
| TDSP | No impact expected | No impact expected | May be impacted financially either negatively or positively |

| Original Sponsor |  |
| --- | --- |
| Name | Beth Garza |
| Company | ERCOT |
| Segment | n/a |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| ERCOT 041305 | Offers revision to Option 2 such that data will be based on the same month in the immediately preceding year. |
| LCRA 042605 | Comments on the averaging effect of zonal shift factors, CSC usage that results from base load units, and the role of hydro Resources. |
| CMWG 051105 | Suggested that the change in Shift Factor calculation be effective for 2006 Annual TCRs; expressed concern with changing the process to calculate Zonal Shift Factors mid-year; and expressed preference for Option 1 |
| ERCOT 051105 | Offered further revisions to the determination of available TCR quantities described in sections 7.5.2.1 and 7.5.2.2. |

Proposed Protocol Language Revision

7.1	Overview of ERCOT Congestion Management

ERCOT employs a Zonal Congestion management scheme that is flow-based, whereby the ERCOT Transmission Grid, including attached Generation Resources and Load, will be divided into a predetermined number of Congestion Zones.  Each Congestion Zone is defined such that each Generation Resource or Load within the Congestion Zone boundaries has a similar effect on the loading (Shift Factor) of Transmission Facilities between Congestion Zones.  For purposes of solving Zonal Congestion the Shift Factor will be assumed the same for: (i) all Generation Resources deemed likely to vary their output and (ii) Loads within a Congestion Zone.  Therefore any imbalance between Loads and Generation Resources in a Congestion Zone will be deemed to have the same impact on a given loading between Congestion Zones.

This Congestion management scheme applies zonal Shift Factors, determined by ERCOT, to predict potential Congestion on CSCs under the known topology of the ERCOT System.  The zonal Shift Factors determined by ERCOT should most closely represent the effect of Generation Resources deemed likely to vary their output and Loads in the Congestion Zone on the CSCs with the current topology of ERCOT System.  This scheme is used in the Day Ahead and Adjustment Periods to evaluate potential Congestion and notify Market Participants accordingly.  ERCOT also uses this scheme, along with other factors, to determine if it should procure Replacement Reserve Service in a Congestion Zone to provide additional Balancing Energy Service to solve expected Congestion.  ERCOT will use monthly zonal Shift Factors to post and analyze the Schedule impact on CSCs until it implements calculations of interval Shift Factors  in the system.

ERCOT will manage transmission Congestion and categorize the cost of Congestion management as either Zonal Congestion management costs or Local Congestion management Costs.  Zonal Congestion management costs are those costs attributable to managing Congestion on CSCs or predefined Closely Related Elements (CRE).  The costs of managing Zonal Congestion will be directly assigned to QSEs based on the impact of each QSE’s portfolio  on CSCs.  All other Congestion management costs are considered Local Congestion management costs.

ERCOT will use the Zonal Congestion management model with Shift Factors of Generation Resources and Loads on CSCs and zonal Balancing Energy Service MCPEs to determine the Shadow Prices of energy across corresponding CSCs.   In addition, ERCOT will directly assign to QSEs the costs of any Replacement Reserve Service procured on a zonal basis for Congestion management purposes.  ERCOT will use these Shadow Prices in settlement to directly assign to QSEs the cost of managing Zonal Congestion to QSEs.

The Local Congestion management scheme relies on a more detailed Operational Model to determine how each particular Resource or Load impacts the transmission system.  This model does not use portfolios and uses the current known topology of the transmission system.

ERCOT will uplift the cost to solve Local Congestion pro rata to each QSE based on its Load Ratio Share.

ERCOT will manage Congestion by:

(1)	Evaluating the levels of Zonal Congestion and any other Congestion during the Day Ahead, the Adjustment Period and the Operating Period using appropriate models of the ERCOT Transmission Grid;

(2)	Examining the impacts of QSE energy schedules on CSCs;

(3)	Posting on the MIS, the total megawatt quantity impacts on every CSC, and allowing QSEs to adjust schedules to mitigate potential Congestion on the CSCs;

(4)	Procuring during the Adjustment Period, as needed, Replacement Reserve Service (RPRS) to use with other Resources for which QSEs have submitted Balancing Energy bids to provide sufficient capacity for Balancing Energy flows in the Operating Hour while respecting operational limits of the ERCOT Transmission Grid;

(5)	Determining settlement for QSEs providing RPRS procured to manage Congestion; and

(6)	Determining settlement for QSEs providing Balancing Energy associated with resolving Zonal Congestion.

ERCOT will carry out these steps in accordance with this Section and the Scheduling and Ancillary Service Scheduling and Selection requirements in Sections 4 and 6, respectively.

7.2	CSC Zone Determination

7.2.1	Principles for Determination and Modification and/or Elimination of CSC Congestion Zones

7.2.1.1	Process for Determining CSCs [no change]

7.2.1.2	Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will determine Congestion Zones and Zonal Shift Factors by:

(1)	Developing Shift Factors using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

(2)	Using statistical clustering to aggregate transmission busses into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); (ii) the R2 (coefficient of determination) from the cluster analysis should exceed a threshold level set by the appropriate subcommittee; (iii) station IDs as provided by TDSPs in Protocol Section 15.1.2.5, Response from TDSP to Registration Notification Request, can be assigned only into one Congestion Zone; and (iv) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone should not exceed a limit set by the same subcommittee.  The limits will be set to balance the competing goals of minimizing the number of Congestion Zones and maximizing the accuracy of the Commercial Model’s representation of the ERCOT System.

(3)	If the clustering process described in (2), above,  results in the splitting of two or more busses within any single station into different zones, then the following process will be used to adjust the zone selection to place all busses within a single station in the same zone:

(i)	For Generation Resource stations, determine which zone contains the largest amount of generating capacity of the station and assign all busses of that station to that zone;

(ii)	For all other stations, determine which zone is assigned the largest total amount of Customer Load of the busses in the station and assign all busses of that station to that zone;

(iii)	If there is no Load or generating capacity in the station, or if the amount of generating capacity and/or Load assigned to individual station busses are equal, then the Load or generating capacity shall be assigned to the zone containing the lowest bus number of the station.

(4)	Creating Zonal Shift Factors for each CSC by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the megawatt levels on the bus in the Load flow base case used to determine CSC Limits from Generation Resources deemed likely to vary their output.

(5)	Congestion Zone approval process: The appropriate ERCOT TAC Subcommittee will review the process followed above to determine if the Congestion Zones should be recommended for approval to the TAC and the Board.

7.2.1.3	Determining Closely Related Elements (CREs) [no change]

7.2.1.4	Determining Generation Resources Deemed Likely to Vary Their Output

At the beginning of each calendar year, ERCOT will compile a list of all Generation Resources fueled by nuclear fuel, coal or lignite.  ERCOT will exclude such units from the calculation of Zonal Shift factors for the rest of the calendar year.  All other Generation Resources will be deemed likely to vary their output.

7.3	Congestion Management for CSCs/Zonal Congestion

7.3.1	Determination of Zonal Congestion

ERCOT will analyze energy schedules to determine the existence and extent of Zonal Congestion as part of the Day Ahead Scheduling Process.

7.3.2	Resolution of Zonal Congestion [no change]

7.3.3	[Expired 2/15/2002]  [no change]

7.3.4	Settlement of Zonal Congestion

7.3.4.1	Balancing Energy CSC Congestion Charge

The calculation for Zonal Congestion Management Charge is as follows:

ICSCiq	=	Σ ((QSSiqz - SOiqz) * SFzcsc )z

If ICSCiq > 0 then,

CSCBECSCiq	=	SPCSCi	* MAX (0, ICSCiq)

Else (counterflow)

CSCBECSCiq	=	SPCSCi	* ICSCiq

End

CSCBEiq	=	SUM (CSCBECSCiq)CSC

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCBECSCiq 	CSC Energy Related Congestion Charge, per interval, per CSC, per QSE

CSCBEiq 	CSC Energy Related Congestion Charge, per interval of that QSE

SPCSCi    	Shadow Price per CSC, per interval

ICSCiq    	Scheduled MW Impact per CSC, per interval, per QSE

SFzcsc	Commercial Model Shift Factor of Generation Resources deemed likely to vary their output per CSC, per zone

QSSiqz	QSE Supply Schedule per interval, per zone, per QSE

SOiqz	Scheduled Obligation (prior to Real Time) per interval, per zone, per QSE

ERCOT will compute the Zonal Shift Factors for settlement purposes using  Shift Factors of Generation Resources within the zone deemed likely to vary their output, weighted by the generation level in the monthly base case.  The monthly base case will be the most recent base case used by ERCOT but no less current than the base case used by ERCOT to determine the number of TCRs available for sale for the corresponding month.

7.5.2.1	Annual TCR Quantities

Annually determined summer ERCOT peak flow limits for CSCs will provide the starting basis for determining the number of TCRs to be auctioned.  The flow limit established for each CSC on an annual basis may vary from time to time based on changes in system conditions.  The ultimate number of TCRs to be auctioned for the CSCs will take into account forecasted changes in system conditions.

ERCOT will determine the total annual quantity of TCRs for the annual distribution by adjusting the annual summer peak base case CSC Limits by the following amount for each CSC: the sum over all nodes of the product of the expected level of Load at each node from the annual summer peak base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and a modification of the Zonal Shift Factor calculated using all Generation Resources.

7.5.2.2	Monthly TCR Quantities

ERCOT will determine the total quantity of TCRs for the monthly distribution by adjusting the corresponding modified seasonal base case CSC Limits by the following amount for each CSC: the sum over all nodes of the product of the expected level of Load at each node from the modified seasonal base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and a modification of the Zonal Shift Factor calculated using all Generation Resources.  The modified seasonal base case for a particular month will be based on the most limiting situation within the corresponding month.  This analysis takes into account all transmission outages of significant duration and impact with the objective of collecting sufficient revenues from direct assigned Congestion costs to pay all TCR holders.

ERCOT may distribute TCRs in amounts less than the total quantities based on expectations of the frequency and magnitude of potential CSC de-rates that could occur during any particular month.