| PRR Number | 608 | PRR Title | Improve Ancillary Service Performance Conditions |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.10.6 Ancillary Service Performance Conditions |  |
| Requested Resolution (Normal or Urgent) |  | Normal |  |
| Revision Description |  | Revise section 6.10.6 of the protocols to include the impact of different ramp rates on the calculation of a QSE’s SCE Performance Metric. |  |
| Reason for Revision |  | Some combined cycle plants have duct burners, which have different ramp rates and heat rates. Since ERCOT’s existing systems only allow QSEs to submit one ramp rate, combined cycle plants with duct burners cannot provide accurate ramp rate information and will have an SCE when moving into and operating in the duct burners. One option is to change ERCOT’s system to allow QSEs to submit multiple ramps. Given that the cost to make this change is not likely to be reasonable, Coral recommends that Section 6.10.6 Ancillary Service Performance Conditions be changed to include the impact of different ramp rates on a QSE’s Performance Metric. |  |
| Credit Implications (Yes or No, and summary of impact) |  | There should be no impact on credit. |  |
| Timeline |  |  |  |
| Date Posted |  | 6/1/2005 |  |
| Please see the Master List on the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Jeff Brown |
| E-mail Address | jbrown@coral-energy.com |
| Company | Coral Power, LLC |
| Company Address | 909 Fannin, Houston, TX 77010 |
| Phone Number | 713-767-5317 |
| Fax Number | 713- 265-5317 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Provides better information to ERCOT. | Should have minimal impact on ERCOT’s systems. | Should provide ERCOT with a better understanding of the capabilities of combined cycle plants with duct burners. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Consumers benefit because efficient combined cycle plants will not be discouraged from participating in the market. |
| LSE: General, Including NOIE | None | None | LSEs benefit because efficient combined cycle plants will not be discouraged from participating in the market. |
| LSE: CR & REP | None | None | LSEs benefit because efficient combined cycle plants will not be discouraged from participating in the market. |
| QSE | Allows combined cycle plants with duct burners to be treated fairly when determining their SCE performance score. | None | A QSE’s SCE performance metric will not be calculated based on ramp rate limitations in ERCOT’s system. |
| Resource | Allows combined cycle plants with duct burners to be treated fairly when determining their SCE performance score. | None | A resource’s SCE performance metric will not be calculated based on ramp rate limitations in ERCOT’s system. |
| TDSP | None | None | None |

Proposed Protocol Language Revision

6.10.6	Ancillary Service Deployment Performance Conditions

ERCOT shall determine the performance of providers of QSE SCE Performance under normal operating conditions.  ERCOT shall remove from consideration of average performance of a QSE any period during which any of the following events has occurred and which does not have a passing score.

(1)	The two (2) hour period after the QSE has experienced a Forced Outage;

(2)	The entire Settlement Intervals for all QSEs in which ERCOT has deployed or recalled Balancing Energy in response to an Unusual Event;

(3)	The entire Settlement Intervals in which ERCOT has issued a verbal Dispatch Instruction;

(4)	The period where ERCOT issues instructions to any QSE to be deployed at ramp rates in excess of bid ramp rates such as the ten (10) minute time period where a QSE is ramping up due to a RRS deployment;

(5)	The entire Settlement Interval(s) in which a QSE is ramping into (from time of notification to time of deployment) or out of (from end of deployment to thirty (30) minutes later) a NSRS deployment; and

PIP112: Add item #6 below to the list and renumber the existing (6) to (7) when BUL is implemented: (6) If requested by a QSE, up to three (3) scheduling hours following an instruction issued by ERCOT to recall previously deployed Responsive Reserve, Non-Spin or Balancing Energy Up Services provided from Replacement Capacity that is using qualified Load as the QSE’s Resource; and

- The entire Settlement Interval(s) in which a combined cycle plant is ramping into or operating in the duct burners.

- Certain other periods of abnormal operations as determined by ERCOT.