| PRR Number | 607 | PRR Title | One-Minute Ramp Schedules |
| --- | --- | --- | --- |
| Date |  | 6/16/05 |  |
| Submitter’s Information |  |  |  |
| Name |  | John Dumas |  |
| E-mail Address |  | jdumas@ercot.com |  |
| Company |  | ERCOT |  |
| Company Address |  | 2705 West Lake Drive |  |
| Phone Number |  | 512 248 3195 |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Not known | Significant system changes to accommodate one-minute ramp schedules. | PRR may not produce intended benefits (see comments below) |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | None |
| LSE: General, Including NOIE | None | None | None |
| LSE: CR & REP | None | None | None |
| QSE | Not known | Not known | PRR may not produce intended benefits (see comments below) |
| Resource | Not known | Not known | PRR may not produce intended benefits (see comments below) |
| TDSP | None | None | None |

Comments

Although one-minute ramp schedules could offer more granular data to monitor the behavior of portfolios during ramping, balancing energy will still only be procured for the 15-minute aggregate.  As such, ERCOT does not agree that providing one-minute schedules will decrease the amount of Regulation Service required to correct frequency because Regulation Service does not rely on schedule information to correct frequency.  Regulation Service corrects frequency by sending instructions to Regulation Service providers every four seconds that matches real-time generation to real-time load.  .  , Therefore, one-minute scheduling would not help reduce Regulation deployments unless ERCOT also could deploy balancing energy every minute which is not feasible without significant system changes, if at all.

A Qualified Scheduling Entity (QSE) may wish to explore entering into a Responsibility Transfer agreement as a possible option to address the ramping constraint.

In order to properly analyze this PRR, ERCOT also needs more details on the mechanics of this proposal with regards to the settlement formulas and scheduling processes (e.g., whether the QSE would submit daily one-minute ramp schedules or a ramp profile that would be added to the energy schedule).

Revised Proposed Protocol Language

None