# Model for Interval Data Recorder Segment (BusIDRrq)

This section provides a summary of the estimated profile model for the Interval Data Recorder Required segment.  The profile type for this segment is BusIDRrq, and includes customers with peak demand levels over 1,000 kW.  The BusIDRrq profile will only be used in those rare instances when no customer-specific historic interval data are available for settlements, e.g., for a new ESI ID that has a demand level greater than 1,000 kW.

Large customer data were available for three utilities (TXU, CPL, and WTU) at the rate class level.  Data were included in the analysis when the class average peak values exceeded 1,000 kW.  The resulting sums for each utility do not correspond exactly to the definition of IDR customers used for ERCOT, since some customers under 1,000 kW may be included in the rate class totals, and some customers over 1,000 kW are known to be in classes that were not interval metered on a census basis.

Figure 10-1 shows a scatter plot of daily energy versus average temperature for all days (about 1,300) in the database.  In this plot, the data for each utility are scaled to adjust for differences in average customer size.  Days are color coded, with solid blue circles for weekdays, orange diamonds for Saturdays, red triangles for Sundays, and green squares for holidays.  The chart shows some seasonality, with loads about 10% higher in the warm months than in the cold months.  However there is not a strong indication of significant daily weather sensitivity.

Figure 10-2 and Figure 10-3 show loads at hour 7 (the hour ending at 7 a.m.) and hour 16 (the hour ending 4 p.m.) plotted against average temperature.  Both show the same pattern of mild seasonality, moderate day-type effects, and absence of daily weather sensitivity.

Figure 10-1:  Energy vs. Dry Bulb – All Utility Data – BusIDRrq

Figure 10-2:  Hour 7 Load vs. Dry Bulb – BusIDRrq

Figure 10-3: Hour 16 Load vs. Dry Bulb – BusIDRrq

The groups of variables that appear in the daily energy model are as follows:

--	Day of the week variables

--	Month/Day-type variables

--	Holiday variables

--	Season variables

--	Solar and Daylight Saving variables

The month/day-type variables are unique to the BusIDRrq profile type, and do not appear in any of the other profile models.  These variables are binary (0,1) variables that are the product of monthly binaries and the weekend and weekday binaries.  For example, the variable JulyWkDay is set to 1 for all days in July that are weekdays (and not holidays) and 0 for all other days.

The estimated coefficients from the daily energy equation are presented in Table 10-1.  The fit statistics for the equation include an R Square value of 0.63 and a mean absolute percentage error (MAPE) of 4.9%.  Most of the coefficients are statistically significant.  Key observations include the following:

	Month and day type effects (e.g., JanuaryWkDay) are the main explanatory variables.  As expected, the constants on weekdays are larger than on weekend days.  Also, the constant shifts for summer months are larger than the constant shifts for winter months.

	Holiday effects are all negative and statistically significant for the major holidays.

The estimated coefficients for the hourly fraction models are presented in Appendix D.  The hourly fraction models use a reduced specification with season/day-type constant shifts rather than month/day-type shifts.  This reflects the fact that large customer load shapes are very stable within each season, although the levels change slightly from month to month.

Table 10-1:  Daily Energy Equation – Estimated Coefficients – BusIDRrq

Figure 10-4 provides a plot of the predicted values for the eight weather zones for the coldest week in January of 1999.  Figure 10-5 provides a comparable plot for the hottest week in August of 1999.  The predicted values are almost identical, reflecting the fact that all inputs (except hours of light) are the same for all zones.  Zone-specific variables were not included in the model because IDR data were not provided by all utilities and because the models were intentionally kept as simple as possible, given their limited uses.

Table 10-2 provides a summary of mean absolute percentage error (MAPE) values for the estimated hourly models.  The first set of values, labeled Daily MAPE, show the average values computed from individual hourly errors.  These values average to 5.8% and range from 4.1% to 7.6%, depending on the hour.  These averages are computed from the absolute values of the differences between the data and modeled loads in each hour divided by the sample load in that hour.

In contrast to the results presented in previous sections, the data for the BusIDRrq segment represent a total of class loads, rather than a sample estimate.  As a result, the error statistics do provide a measure of the accuracy of the model relative to the true population loads for these classes for the three utilities that provided the data.  (Note, as discussed above, the large customer class totals do not correspond exactly to the population of IDR customers as defined by ERCOT).  Because the data span three service territories and zone-specific variables are not included in the BusIDRrq model, part of the model error reflects the fact that the IDR profiles differ across the different data sets while the model predictions do not.

The second set of MAPE values is computed from the differences between the average sample value in each month and the average modeled value in each month.  In this comparison, the predicted values for each month are calibrated to have the same total as the actual data values for that month prior to computing the error statistics.

The final graph, Figure 10-7, shows a comparison of the average sample value across all data sets for the months of January and July, and the average predicted value across all data sets for these same months.  As expected, at this high level of aggregation, the model results provide an excellent approximation to the average value for the underlying data.

Figure 10-4: Simulated Profiles – 1999 Cold Winter Week -- BusIDRrq

Figure 10-5: Simulated Profiles – 1999 Hot Summer Week -- BusIDRrq

Table 10-2:  Daily and Monthly MAPE Values for BusIDRrq

Figure 10-6:  MAPE Values (Daily and Monthly Average) – BusIDRrq

Figure 10-7:  BusIDRrq Average Shapes – Actual and Predicted