# Model for Business High Load Factor (BusHiLF)

This section provides a summary of the estimated profile model for the Business High Load Factor segment.  The profile type for this segment is BusHiLF.  This segment includes business customers with demand meters and with average load factors greater than 0.60.

Figure 9-1 shows a scatter plot of daily energy versus average temperature for all days in the database, representing over 3,000 days of data in different years and in different utility service areas.  In this plot, the data for each utility are scaled to adjust for differences in average customer size.  Days are color-coded, with solid blue circles for weekdays, orange diamonds for Saturdays, red triangles for Sundays, and green squares for holidays.  The chart shows significant seasonality or weather sensitivity on the cooling side, and shows no evidence of weather sensitivity on the heating side.  There is a significant overlap of energy use on weekdays and weekend days.

Figure 9-2 provides a second version of this chart, with the observations for a specific utility highlighted.  The purpose of this chart is to illustrate that separate slopes are estimated for each weather zone, and differences in these slopes will reflect the differences between weather relationships for the data points that represent each zone.  Weather slopes in the North Central zone are determined by the TXU East and TNMP North data.  Slopes in the South Central zone are determined by the Austin Energy and CPS data.  Slopes in the Coast zone are determined by Reliant Energy and TNMP South data.  Slopes in the West zone are determined by WTU and TXU West data.

Figure 9-3 and Figure 9-4 show loads at hour 7 (the hour ending at 7 a.m.) and hour 16 (the hour ending 4 p.m.) plotted against average daily temperature.  These plots include data for all contributing utilities.  The hour 7 plot shows a weak upward sloping relationship between temperature and loads.  A stronger relationship is evident in the hour 16 plot.  These relationships may indicate a combination of seasonality (higher loads in the summer season) and well as daily weather sensitivity (higher loads on the hot days in each season).

Figure 9-1:  Energy vs. Dry Bulb – All Utility Data – BusHiLF

Figure 9-2:  Energy vs. Dry Bulb -- Individual Utility Data – BusHiLF

Figure 9-3:  Hour 7 Load vs. Dry Bulb – BusHiLF

Figure 9-4: Hour 16 Load vs. Dry Bulb – BusHiLF

The groups of variables that appear in the daily energy model are as follows:

--	Day of the week variables

--	Holiday variables

--	Season variables

--	Solar and Daylight Saving variables

--	Dry bulb temperature slopes by zone (Hot, Med, Cold)

--	Dry bulb slope extensions (XXHot, XHot, XCold, WkEnd interactions)

--	Temperature buildup variables

Because of the relatively weak relationship with weather variables, dew point, cloud cover and wind speed variables were not included in the Business High Load Factor model.  The estimated coefficients from the daily energy equation are presented in Table 9-1.  The fit statistics for the equation include an R Square value of 0.80 and a mean absolute percentage error (MAPE) of 4.2%.  Most of the coefficients are statistically significant.  Key observations include the following:

	Calendar effects are as expected.  Weekdays have higher loads relative to Sundays (which are represented by the zone-specific constant terms), as indicated by the weekday coefficient estimates, which range between 97.56 and 135.57 kWh.  Holiday loads show significant negative effects (relative to Sundays) in most cases.  These day-type effects are small compared to the regional constant terms, all of which are over 2,000 kWh.

	The hot-side slopes vary across the regions, ranging from 9.63 kWh per degree to 37.37 kWh per degree.  These slopes are significantly different on weekend days.  These slopes are amplified above 80 degrees by an additive slope of 7.11 kWh per degree, but are muted beyond 85 degrees by an additive slope of –24.47 kWh per degree, effectively canceling weather response beyond this level.

	Middle slopes for temperatures between 60 and 70 degrees are also strong positive values, ranging from 3.36 kWh per degree to 34.15 kWh per degree.

	Cold-side slopes for temperatures below 60 degrees are weak negative values, indicating evidence of additional cooling effects rather than heating effects between 50 and 60 degrees.  Below 50 degrees an additive slope of 14.08 kWh per degree provides counteracts the negative effects, implying little or no weather response on the cold side in most zones.

	Temperature buildup effects on the hot side are as expected, with a slope of 5.80 kWh per degree on hot days and a slope of 2.26 kWh per degree on mild days.  The cold day effect is negative and insignificant.

Table 9-1:  Daily Energy Equation – Estimated Coefficients – BusHiLF

Figure 9-5 provides a plot of the predicted values for the eight weather zones for the coldest week in January of 1999.  Figure 9-6 provides a comparable plot for the hottest week in August of 1999.  Differences in these plots result from differences in the timing and severity of weather in the eight zones, as well as the differences in the zone-specific weather slopes mentioned above.

Table 9-2 provides a summary of mean absolute percentage error (MAPE) values for the estimated hourly models.  The first set of values, labeled Daily MAPE, show the average values computed from individual hourly errors.  These values average to 4.8% and range from 4.5% to 5.5%, depending on the hour.  These averages are computed from the absolute values of the differences between the sample and modeled loads in each hour divided by the sample load in that hour.  These should not be considered to be measures of “accuracy,” since the true population loads are not known.  To report accuracy, it would be necessary to compute the true residual (Actual – Predicted) in each hour, but the actual population values are unknown.  Further, with the number of sample cases in five of the seven samples falling below 50 cases, there is significant variation in the sample average values on a daily basis that is random noise (including both sampling error and daily variations in individual customer behavior).

The second set of MAPE values is computed from the differences between the average sample value in each month and the average modeled value in each month.  In this comparison, the predicted values are calibrated to have the same total as the sample values for each month (as they would when used in settlements calculations).  These values, which average to 1.3%, eliminate daily noise from the sample through the averaging process, and may provide a better indication of the types of errors that can be expected at the population level in the settlements process.

The two sets of MAPE values are plotted out by hour of the day in Figure 9-7.

The final graph, Figure 9-8, shows a comparison of the average sample value across all data sets for the months of January and July, and the average predicted value across all data sets for these same months.  As expected, at this high level of aggregation, the model results provide an excellent approximation to the average value for the underlying sample data.

Figure 9-5: Simulated Profiles – 1999 Cold Winter Week – BusHiLF

Figure 9-6: Simulated Profiles – 1999 Hot Summer Week – BusHiLF

Table 9-2:  Daily and Monthly MAPE Values for BusHiLF

Figure 9-7:  MAPE Values (Daily and Monthly Average) – BusHiLF

Figure 9-8:  BusHiLF Average Shapes – Actual and Predicted