# Model for Business Low Load Factor (BusLoLF)

This section provides a summary of the estimated profile model for the Business Low Load Factor segment.  The profile type for this segment is BusLoLF.  This segment includes business customers with demand meters and with average load factors below 0.40.

Figure 7-1 shows a scatter plot of daily energy versus average temperature for all days in the database, representing over 3,000 days of data in different years and in different utility service areas.  In this plot, the data for each utility are scaled to adjust for differences in average customer size.  Days are color-coded, with solid blue circles for weekdays, orange diamonds for Saturdays, red triangles for Sundays, and green squares for holidays.  The chart shows significant weather sensitivity on both the heating and cooling sides, and also shows strong separation of the weekdays from the weekend days.

Figure 7-2 provides a second version of this chart, with the observations for a specific utility highlighted.  The purpose of this chart is to illustrate that separate slopes are estimated by zone, and differences in these slopes will reflect the differences between weather relationships for the data points that represent each zone.  Weather slopes in the North Central zone are determined by the TXU East and TNMP North data.  Slopes in the South Central zone are determined by the Austin Energy and CPS data.  Slopes in the Coast zone are determined by Reliant Energy and TNMP South data.  Slopes in the West zone are determined by WTU and TXU West data.

Figure 7-3 and Figure 7-4 show loads at hour 7 (the hour ending at 7 a.m.) and hour 16 (the hour ending 4 p.m.) plotted against average daily temperature.  These plots include data for all contributing utilities.  The hour 7 plot shows a relatively weak relationship between temperature and loads on both the heating and cooling sides of the curve.  A slightly stronger relationship on the cooling side of the curve is evident in the hour 16 plot, but the data variation remains large relative to the weather slopes.

Figure 7-1:  Energy vs. Dry Bulb – All Utility Data – BusLoLF

Figure 7-2:  Energy vs. Dry Bulb -- Individual Utility Data – BusLoLF

Figure 7-3:  Hour 7 Load vs. Dry Bulb -- BusLoLF

Figure 7-4: Hour 16 Load vs. Dry Bulb – BusLoLF

The groups of variables that appear in the daily energy model are as follows:

--	Day of the week variables

--	Holiday variables

--	Season variables

--	Solar and Daylight Saving variables

--	Dry bulb temperature slopes by zone (Hot, Med, Cold)

--	Dry bulb slope extensions (XXHot, XHot, XCold, WkEnd interactions)

--	Temperature buildup variables

Because of the relatively weak relationship with weather variables, dew point, cloud cover and wind speed variables were not included in the Business Non-Demand model.  The estimated coefficients from the daily energy equation are presented in Table 7-1.  The fit statistics for the equation include an R Square value of 0.629 and a mean absolute percentage error (MAPE) of 12.5%.  Most of the coefficients are statistically significant.  Key observations include the following:

	Calendar effects are as expected.  Weekdays have significantly higher loads relative to Sundays (which are represented by the zone-specific constant terms), as indicated by the weekday coefficient estimates, which range between 110.87 and 120.10 kWh.  Holiday loads show strong and significant negative effects (relative to Sundays).

	The hot-side slopes vary across the regions, ranging from 3.63 kWh per degree to 11.99 kWh per degree.  These slopes are significantly reduced on weekend days by a shift of –2.05 kWh per degree.  These slopes are amplified above 80 degrees by an additive slope of 2.41 kWh per degree, but are muted beyond 85 degrees by an additive slope of -2.15 kWh per degree.

	Middle slopes for temperatures between 60 and 70 degrees are somewhat weaker than the hot-side slopes, with values ranging from 2.25 kWh per degree to 6.83 kWh per degree.

	Cold-side slopes for temperatures below 60 degrees of mixed sign, ranging from

–1.46 kWh per degree to 6.56.  These slopes are amplified below 50 degrees by an additive slope of 3.34, implying that loads in all areas rise once temperatures drop below this level.

	Temperature buildup effects are statistically significant and have the expected signs, with a slope of 2.13 kWh per degree on the hot side, and a slope of –1.13 kWh per degree on the cold side.

Table 7-1:  Daily Energy Equation – Estimated Coefficients – BusLoLF

Figure 7-5 provides a plot of the predicted values for the eight weather zones for the coldest week in January of 1999.  Figure 7-6 provides a comparable plot for the hottest week in August of 1999.  Differences in these plots result from differences in the timing and severity of weather in the eight zones, as well as the significant differences in the zone-specific weather slopes mentioned above.

Table 7-2 provides a summary of mean absolute percentage error (MAPE) values for the estimated hourly models.  The first set of values, labeled Daily MAPE, show the average values computed from individual hourly errors.  These values average to 14.6% and range from 12.7% to 17.5%, depending on the hour.  These averages are computed from the absolute values of the differences between the sample and modeled loads in each hour divided by the sample load in that hour.  These should not be considered to be measures of “accuracy,” since the true population loads are not known.  To report accuracy, it would be necessary to compute the true residual (Actual – Predicted) in each hour, but the actual population values are unknown.  Further, with the number of sample cases in four of the seven samples falling below 50 cases, there is significant daily variation in the sample values basis that is random noise (including both sampling error and daily variations in individual customer behavior).

The second set of MAPE values is computed from the differences between the average sample value in each month and the average modeled value in each month.  In this comparison, the predicted values are calibrated to have the same total as the sample values for each month (as they would when used in settlements calculations).  These values, which average to 4.7%, eliminate daily noise from the sample through the averaging process, and may provide a better indication of the types of errors that can be expected at the population level in the settlements process.

The two sets of MAPE values are plotted out by hour of the day in Figure 7-7.

The final graph, Figure 7-8, shows a comparison of the average sample value across all data sets for the months of January and July, and the average predicted value across all data sets for these same months.  As expected, at this high level of aggregation, the model results provide an excellent approximation to the average value for the underlying sample data.

Figure 7-5: Simulated Profiles – 1999 Cold Winter Week – BusLoLF

Figure 7-6: Simulated Profiles – 1999 Hot Summer Week – BusLoLF

Table 7-2:  Daily and Monthly MAPE Values for BusLoLF

Figure 7-7:  MAPE Values (Daily and Monthly Average) – BusLoLF

Figure 7-8:  BusLoLF Average Shapes – Actual and Predicted