| PRR Number | 356 | PRR Title | SCE Performance requirement |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Action |  | Rejected |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.10.5 QSE Real Power Performance Criteria |  |  |
| Revision Description |  | Provide additional incentive to QSEs to closely follow SCE, and to reduce the burden of Ancillary Service capacity costs on QSEs that do closely follow SCE. |  |  |
| Overall Market Benefit |  | - Provides incentive to improve SCE performance without the need to precisely define and enforce an acceptable SCE. - Compensates QSEs that make efforts to improve unit response and otherwise minimize SCE. - Allows each QSE to perform its own analysis of the benefits and costs of approaching zero SCE, and/or committing to deliver energy and services at ramp rates it cannot perform. - Provides the benefits outlined above with enough uncertainty to minimize gaming. - Reduces the demand on ERCOT to police SCE. |  |  |
| Overall Market Impact |  | Not known |  |  |
| Consumer Impact |  | Not known |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | No. |  |  |
| Procedural History |  | - PRR356 was posted on 8/24/02 - On 9/16/02, ERCOT posted the Impact Analysis (IA). - On 9/25/03, PRS remanded this PRR to the WMS and ROS Task Force. - PRS considered this PRR on 8/24/05. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | PRS voted to support rejection of this PRR. The motion carried with one opposing vote from the Coop segment. All market segments were present. |  |  |
| Summary of PRS discussion |  | PRS discussed WMS’ determination that this PRR has either been replaced by subsequent PRRs or has become obsolete. |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Not known (IA is 5 years old) | Not known (IA is 5 years old) | Improved SCE and potential for improved reliability. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Improved SCE and potential for improved reliability. |
| LSE: General, Including NOIE | Not known | Not known | Improved SCE and potential for improved reliability. |
| LSE: CR & REP | Not known | Not known | Improved SCE and potential for improved reliability. |
| QSE | Not known | Not known | Improved SCE and potential for improved reliability. |
| Resource | Not known | Not known | Improved SCE and potential for improved reliability. |
| TDSP | None | None | None |

| Original Sponsor |  |
| --- | --- |
| Name | Paul S. Breitzman |
| Company | City of Garland |
| Segment | Municipally Owned Utility |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| Garland 092303 | Incorporates comments made by the PUCT. |
| PUCT 091202 | Explains what the effect of the PRR would be. |

Proposed Protocol Language Revision

### 6.10.5	QSE Real Power Performance Criteria

#### 6.10.5.1	Ancillary Service Demand Factor

For each Settlement Interval ERCOT shall calculate an Ancillary Service Demand Factor for each QSE in accordance with the following:

ASDFqi = (ABSCEqi / ABSCEti )

Where:	  ABSCEqi = the 15 minute integration of the absolute value of the SCE of the QSE    during that interval, multiplied by ( 1 – ( 0.5 * ( UNCRESqi / TOTRESqi ) ) )

ABSCEti = Sum of ABSCEqi for all QSEs during the interval

UNCRESqi = Net capability of online generation of the QSE in that interval that is deemed by ERCOT to be uncontrollable

TOTRESqi = Net Capability of all online generation of the QSE in that interval.

- 6.10.5.2	Ancillary Service Cost Reallocation

For each Settlement Interval ERCOT shall calculate an Ancillary Service Cost Reallocation for each QSE in accordance with the following:

ASCRqi = ( 1 – (ELFPFi / ( ELFPFi + ABSCEti ) ) ) * IECASi * ( ASDFqi – LRSqi )

Where:	  LRSqi = Load Ratio Share for Ancillary Service allocation for the QSE, adjusted for bilateral transactions and bid awards, as used to deploy Ancillary Services

ELFPFi = the absolute value of the difference between i) ERCOT’s load forecast made to determine Balancing Energy deployments for the interval, and ii) ERCOT integrated actual load for the interval.

IECASi = The Interval Equivalent Cost of Ancillary Services, determined by ERCOT to be 25% of the cost of Regulation Service plus Non-Spin Reserve Service plus Responsive Reserve Service for the hour containing the interval, based on the capacity clearing prices for each Service for the hour, and based on the total amount of these Services allocated to all QSEs: provided; that if ABSCEti < 100 MW in any interval, then IECASi is set to zero for that interval.

In the event ASCRqi for any QSE for any Settlement Interval is positive, the QSE will be charged that amount.  In the eventASCRqi for any QSE for any Settlement Interval is negative, the QSE will be paid that amount.

PIP112: Add the following two paragraphs when BUL is implemented:

On deployment of BULs, the QSE shall control its Loads to the difference between its Scheduled power and the BUL instructed amount.  ERCOT, at a time established at their discretion, shall determine the Base Load of the sum of all Loads qualified by the QSE as BUL Resources by obtaining actual meter data from IDR meters or directly from the TDSP who has agreed to supply the data.  While IDRs must normally be present on all Loads providing BUL Resources, IDRs may be installed and read on a sample of the Loads qualified by a QSE as a BUL Resource, provided the Loads are sampled in accordance with Section 18.7.2.  The Base Load is determined by the finding the average actual metered energy during the same interval as the instruction to deploy BUL from the previous 10 days (in the case of weekday deployment) or six days (in the case of weekend or holiday deployment).  Days in which BUL instructions occurred during the same interval are ignored.  If the instruction to deploy occurs on a weekday, only weekdays are included in the average.  If the instruction to deploy occurs on a weekend, only weekend days and ERCOT-defined holidays are included in the average. For small customer load management programs, Base Load for each interval is determined from the aggregated appropriate baseline load profile for participating customers assuming the load management program is not in operation.

ERCOT shall also integrate for each Settlement Interval the signal provided by the QSE representing the amount of BUL power deployed. The expected total reduction of Load shall be determined by subtracting the integrated signal for each interval from the Base Load as determined above.  For small customer load management programs during each interval, the expected total reduction of Load shall be determined by subtracting the aggregated participating customer load during operation of the load management program, which is calculated from the appropriate load profile, from the Base Load for these customers as calculated above.  Control performance of the QSE providing BUL shall be deemed satisfactory when during the first hour in which BUL is deployed in an Operating Day, the actual metered Load for any Settlement Interval during the hour is equal to or less than the amount of energy expected.  Failure of the QSE to provide satisfactory control performance during the first hour in which BUL is deployed in an Operating Day will cause ERCOT to withhold payments for any capacity provided as BUL for the Operating Day.  Failure to provide at least 90% of the expected total Load during the first hour of deployment will also disqualify those specific Loads and will reduce the amount the QSE is qualified to bid accordingly.  QSEs may apply for re-qualification of a specific load according to Section 6.10.2. Repeated failures of the same Load to perform in three Operating Days in a Calendar Year will result in a 90 day waiting period after which the QSE may apply for re-qualification of the specific Load according to Section 6.10.2.

#### 6.10.5.3	Effect on SCE of ERCOT Instructions

Operational instructions by ERCOT should be included in SCE to the greatest extent possible.  In the event of operational instructions by ERCOT that directly cause any QSE to increase or decrease its SCE, appropriate adjustments to ABSCEqi shall be made in settlement when performing the calculations in 6.10.5.1.  The extent of the need to make such adjustments shall be reported periodically to the appropriate subcommittee.

- 6.10.5.4	ERCOT Load Forecast Error

ERCOT shall post on its website the interval values of ELFPFi.  ERCOT shall make periodic reports to the appropriate subcommittee showing the trend of ELFPFi and its impact on the amount of Ancillary Services that ERCOT determines is necessary

- 6.10.5.5	SCE Obligation

Ancillary Service Cost Reallocation under 6.10.5.2 is not intended to substitute for a requirement that all QSEs make a continuous good faith effort to operate at or near a zero SCE.  ERCOT shall post on its website daily averages of the absolute value of every QSE SCE as a percentage of the daily average respective QSE load.  When such data shows average absolute SCE to be more than five percent of average load for any QSE on any day, ERCOT will report that QSE to the appropriate regulatory authority.