Emergency Load Program

To be used during:

- 1 in 10 year event

  - Severe Summer conditions

- Severe winter conditions

  - Fuel supply shortages

Requirements:

- No telemetry requirements

- IDR or statistical equivalent of IDR

- Dispatched by ERCOT action (See Conditions for Deployment)

- Registration of ESI-IDs

- 1 test per year

Subscription:

- Subscription shall be annual

- Amount to be established by ERCOT. For discussion purposes assume 1200MWs for our discussion

- First come first serve up to participation limit

- 10% reserved for residential (only if Non-IDR can participate)

Payment for Services:

- Energy settled as L.I. (or R.I. if net generation private networks are allowed to participate)

- Pay for performance

  - 15 minute average

  - Scaled penalty factor for non-compliance?

  - Performance based on Portfolio, not each ESI-ID

- Payment could be any of the following:

(This is probably the most significant issue to resolve. Any load today can participate voluntarily and depending on the arrangement with their REP can receive an energy payment.  Since this is to be a reliability tool for ERCOT it will need to be within the control of the ISO.  This will require some payment to incent loads to register their ESI-IDs and participate in this program. If the payment is not sufficient enough, the program will have a poor subscription rate and fail to be of any value. The key question then becomes; how much is the market willing to pay to reduce the odds of firm load curtailment?)

Cost Recovery Mechanism:

    - Charged back to QSE’s on a LRS

    - There has been a suggestion we try to incorporate an option similar to what is currently proposed in the Resource Adequacy Strawman. See section (j)(5).

Contract:

- Annual or 10 year contract period

- Notification of possible deployment prior to adjustment period

- Limited number of hours of interruption per year (suggest 17 hours maximum, includes 1 hour annual test)

- ESI-IDs not participating as a resource or any standard offer programs

Conditions for Deployment

- After ERCOT has exhausted all options to obtain additional resources if that would help remedy the situation

- After step 2 of EECP but prior to firm load deployment in step 3