| PRR Number | 352PRR | PRR Title | Extension of Proxy Day Determination |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Proposed Effective Date |  | Pending project priority and start date. |  |
| Priority Assigned |  | 2.1 [Rank 18] |  |
| Summary of Impact Analysis |  | Impacts the ERCOT computer system. |  |
| Benefit |  | The requested revision minimizes the use of the IDR Default Profile in settlement calculations by extending the period of historical usage that is available for proxy day selection. This means that ESI ID specific information is more likely to be used in settlement calculations than the IDR Default Profile. |  |
| PRR Background |  | ## This PRR changes the Non-Weather Sensitive (NWSIDR) Proxy Day determination method to use the most recent historical ESI-ID interval data from the last 12 months. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval as submitted. |  |
| ###### TAC Recommendation |  | TAC recommends approval as recommended by PRS. |  |

Recommended Language

#### 11.3.3.3	Non-Weather Sensitive (NWSIDR) Proxy Day Method

For ESI-IDs designated as NWSIDR, ERCOT will use a method for proxy day determination.  This method incorporates the following.

- Use the most recent proxy day for which data is available as the estimate for the target Operating Day.  From historical ESI-ID-specific interval data, choose the most recent occurrence of the appropriate day of the week (Su, M, T, W, Th, F, Sa) corresponding to the day of the week of the Operating Day (Holidays are treated as Sundays) within the most recent 12 months of the trade day;

- If there is no historic interval data available according to (1) above, the IDR data will be estimated using the default profile assigned to the ESI ID for the trade day.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |