| PRR Number | 363PRR | PRR Title |  | IDR Data Start-Stop Times |
| --- | --- | --- | --- | --- |
| ###### Status |  | Posted |  |  |
| Protocol Section Requiring Revision |  | Section 10 Metering Sub Section 10.9.2 DSP Metered Entities |  |  |
| Requested Resolution |  | ## Urgent |  |  |
| Revision Description |  | Add language to require TDSPs (and future Meter Reading Entities) to send only whole days of data for all IDR ESI IDs |  |  |
| ###### Reason for Revision |  | To establish a Protocol mandate for providing IDR data in a fashion to support allowing ERCOT to remove the gap validation from the IDR data loading process. The result will be more IDR data loaded and available for settlement. This requirement also facilitates any “backdating” associated to IDR ESI IDs. |  |  |
| ## Timeline |  |  |  |  |
| Date Received |  |  | 09/20/2002 |  |
| Date Posted |  |  | 09/20/2002 |  |
| Comments Due |  |  | 09/23/2002 |  |
| PRS Review Date |  |  | 09/24/2002 |  |
| PRS Recommendation |  |  | 09/26/2002 |  |
| TAC Consideration |  |  | 10/03/2002 |  |
| Board Consideration |  |  | 10/15/2002 |  |

Proposed Protocol Language Revision

### 10.9.2 TDSP Metered Entities

TDSPs will be required to ensure that the meters for which they are responsible comply with the standards for metering Facilities of the relevant Governmental Authority.  In addition, IDRs used for settlement shall conform to the following:

- IDRs must be able to capture energy consumption in increments consistent with, or in fractions of, ERCOT defined settlement time interval (currently fifteen (15) minutes);

- Regardless of data retrieval method, interval data shall be provided on a schedule that supports the requirements of final settlement;

- IDRs must have battery or other energy-storage back-up to maintain time during power outages;

- IDRs must have time synchronization capability;

- Meter clocks used within ERCOT shall be maintained on a time reference that enables the TDSP to submit data on the Central Prevailing Time.  The meter clock shall be synchronized to within at least +/- 5% of the settlement interval when compared to the National Institute of Standards and Technology (NIST) Atomic Clock;

- IDRs may record at increments less than ERCOT defined settlement and be aggregated to the appropriate settlement interval time block by the TDSP prior to the data being sent to ERCOT;

- New and replacement IDRs used for settlement must be able to capture energy in increments of five (5) minutes (excluding memory allocation).  At market open they will be programmed to read at 15-minute intervals; and

- Settlement intervals will divide each hour in the following standard:

XX:00:01 through XX:15:00

XX:15:01 through XX:30:00

XX:30:01 through XX:45:00

XX:45:01 through XX:00:00

- IDR data submitted to ERCOT for Operating Days November 01, 2002 or later will contain only whole days with start times beginning at 00:00:00 and stop times ending at 23:59:59.

| ## Sponsor |  |
| --- | --- |
| Name | James Cohea |
| E-mail Address | jcohea@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive – Taylor, TX 76574 |
| Phone Number | (512) 248-3872 |
| Fax Number | (512) 248-3886 |
| ## ERCOT Impact Analysis |  |
| Date | 09-20-2002 |
| Financial Impact | No impact on ERCOT |
| Analysis | Implementation will not require code modification at ERCOT. However TNMP and AEP will need to make system/process modifications to adhere to this proposed requirement. If this PRR is approved, ERCOT will add time-stamp validation as part of the “Lodestar Data Loading Improvement” project that has already been approved and prioritized by the Market. |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |