| PRR Number | 371PRR | PRR Title | Define OOME as Instructed Deviation and Portfolio Out Of Merit Energy Service |
| --- | --- | --- | --- |
| ###### Date |  | November 11, 2002 |  |
| ## Submitter’s Information |  |  |  |
| Name |  | Jun Yu |  |
| E-mail Address |  | jyu@ercot.com |  |
| Company |  | ERCOT |  |
| Company Address |  | 2705 West Lake Dr. |  |
| Phone Number |  | 512-248-3969 |  |
| Fax Number |  | 512-248-3082 |  |

Comments

Under current settlement Payment of OOMC in any is clamped by by 16 heat rate.  Given that the hourly charge as a function of MCPE is proposed, I’m suggesting an alternative method for clamping the payment.

#### 6.8.2.1 Resource Category Generic Costs

In order to properly assign local congestion costs, it is necessary to establish certain generic costs associated with resources that will be used to calculate production costs incurred when the resource provides Out-of-Merit Service.  These generic resource costs include: generic fuel cost, generic startup costs, and generic operating costs.

- Each ERCOT generation resource will be assigned to one of the following Resource Categories for the purpose of determining generic fuel costs:

Nuclear

Hydro

Coal and Lignite

Combined Cycle

Gas Steam

Simple Cycle

Diesel (and all other diesel or gas-fired resources)

Renewable (i.e., non-Hydro renewable resources)

Block Load Transfer

The category of each resource will be determined by the Resource Owner and reported to ERCOT.

(2)	Each ERCOT generation resource will be assigned to one of the following Resource Categories for the purpose of determining generic startup and generic operating costs:

Baseload

Gas-Intermediate

Gas-Cyclic

Gas-Peaking

Renewable (Including Hydro)

Block Load Transfer

Except for the Renewable category, the above categories are based on the categories defined for the Capacity Auction.  Therefore, for those resources that were included in the Capacity Auction, their category shall be the same as that used for the Capacity Auction.  For those resources not included in the Capacity Auction, the category will be determined by the Resource Owner and reported to ERCOT.

(3)	Resource Category Generic Fuel Costs

Each ERCOT generation resource will be assigned a Resource Category Generic Fuel Cost (RCGFC) based on the resource category to which it is assigned.  For nuclear, hydro and lignite and coal resources, the RCGFC will be a fixed dollar/MWh amount as shown below.  For the remaining resource categories (except Renewable), the RCGFC will be the product of a heat rate (based on the heat rates used for the Capacity Auction) and a Fuel Index Price (FIP).  The RCGFC for Renewable resources will be $0/MWh.

The FIP shall be the Midpoint price expressed in $/MMBTU, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day of OOME deployment.  The FIP for Saturdays, Sundays, holidays and other days for which there is no FIP published in Gas Daily, shall be the next published FIP after the day of OOME deployment.  In the event that the FIP is not published for more than two (2) days, the previous day published FIP will be used for initial settlement and the next day published FIP will be used for true up of the final settlement statement.

The RCGFC for each type of resource for upward instructions will be:

Nuclear = $15.00/MWh

Hydro = $10.00/MWh

Coal and Lignite = $18.00/MWh

Combined Cycle = FIP * 10 MMBtu/MWh

Gas-Steam = FIP * 14.5 MMBtu/MWh

Simple Cycle = FIP * 15 MMBtu/MWh

Diesel = FIP * 16 MMBtu/MWh

Block Load Transfer = FIP * 18 MMBtu/MWh

Renewable = $0/MWh

The RCGFC for each type of resource for downward instructions will be:

Nuclear = $0.00/MWh

Hydro = $0.00/MWh

Coal and Lignite = $3.00/MWh

Combined Cycle = FIP * 5 MMBtu/MWh

Gas-Steam = FIP * 7.5 MMBtu/MWh

Simple Cycle = FIP * 10.5 MMBtu/MWh

Diesel = FIP * 12 MMBtu/MWh

Block Load Transfer = Not Applicable

Renewable = $0/MWh

The RCGFC for each type of resource when it operates at the unit Minimum Sustainable Limit will be:

Nuclear =

Hydro =

Coal and Lignite =

Combined Cycle =

Gas-Steam =

Simple Cycle =

Diesel =

Block Load Transfer =

Renewable =

(4)	Resource Category Generic Startup Costs

Resource Category Generic Startup Costs (RCGSC) represents the startup cost of capacity used for Replacement Reserve Service.   The RCGSC for each type of resource will be:

Baseload	=	$2,310 + (FIP * 6.6 MMBtu/MW * RMCU )

Gas-Intermediate	=	$4,800 + (FIP * 16.5 MMBtu/MW * RMCU)

Gas-Cyclic 	=	$3,000 + (FIP * 9.0 MMBtu/MW * RMCU)

Gas-Peaking	=	$2,310 + (FIP * 1.15 MMBtu/MW * RMCU)

Renewable	=	$0

Where:

RMC = Resource Maximum Capacity (in MW)

u = unit

#### 6.8.2.2 Capacity Payments

- OOMC Service may be used by ERCOT as a procured Replacement Reserve Resource in the Adjustment Period where necessary to support emergency operations and provide voltage support, stability or to manage localized transmission limitations All Resources that are available and plan to be off-line during the interval for which ancillary services are being procured are eligible to be selected to provide OOMC Service.  OOMC Service may be used only if there is no resource available through the Specific Process for Replacement Reserve Service procurement defined in section 6.6.3.2.1 or through contracted RMR Service.

- Resources selected to provide OOMC Service that actually reconnect to the ERCOT transmission grid and start the unit in order to provide the OOMC service will be paid both the Resource Category Generic Capacity Cost for starting the unit as well as the Resource Category Generic Operational Cost for operating during the instructed interval(s).

- Resources that are connected to the ERCOT transmission grid when instructed to provide OOMC Service will be paid the Resource Category Generic Operational Cost for operating during the instructed interval(s).

- If a Resource is called to provide OOMC service and the payment for OOMC service is insufficient to cover all costs of providing the Service and provide a premium, then that Resource will be paid all verifiable, costs in excess of the OOMC payment that are directly attributable to the OOMC Service plus a premium.  These costs may include the cost of exceeding swing gas contract limits, additional gas demand costs set by fuel supply or transportation contracts, and any additional costs to purchase emissions credits or other costs incurred due to environmental regulations.    The premium to be provided shall be the product of the costs of providing the service times 10%.    Verification of these costs must be submitted to ERCOT to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  QSEs requesting such cost recovery shall perform the following:

(a)	Submit a settlement dispute in accordance with the dispute process outlined in Section 9.5.  In addition to the standard information required on the entry form on the ERCOT Portal, the dispute should clearly indicate:

(i)	The deployment instruction received from ERCOT to provide services.

(ii)	The payment received for providing the service.

(iii)	The actual cost of providing the service.

(iv)	A reference to the documentation to be provided in writing as indicated in section 6.8.2.2(4)(b).

(b)	Requests for additional compensation shall provide documentation in writing to allow for verification of the claimed amounts.  Documentation requirements for each cost type are as follows:

(i)	Fuel cost for providing the service shall be determined by taking the product of the fuel consumption (MMBtu) at the normal minimum operating level of the resource and it’s associated fuel cost ($/MMBtu).

(ii)	Fuel consumption at the normal minimum operating level shall be based upon a heat rate curve for the resource from the most recently conducted heat rate tests.  Test data shall be provide in sufficient detail to allow for the validation of the heat rate curve provided.

(iii)	Fuel cost, including transportation cost, will require supporting documentation of sufficient detail to allow for the verification of the cost of fuel consumed at the deployed resource.  Documentation may include contracts, invoices or other documents all of which will be treated as confidential.  All copies of such documentation will be returned upon completion of the review.  For gas fired resources, such documentation will not be required if the requested incremental fuel cost is less than 115% of the Fuel Index Price.

(iv)	Compensation for additional types of cost, not outlined above, must be reviewed and approved by the ERCOT Board of Directors during an Executive Session of their next regularly scheduled meeting.  Requests must be presented in person by a representative of the company submitting the request and must also include language suitable to be included in the Protocols to define the documentation requirements for future request of a similar nature.  Subsequent to the approval of such costs, the requesting company shall submit a Protocol Revision Request, in accordance with Protocol 21, incorporating the necessary documentation standards provided to the ERCOT Board.

- The calculation for capacity payments of Out –of Merit Service is as follows:

PCOOMRPqi	=	SUM (PCOOMRPqui)u

Given:

If BPRpqui exists,

Then:

PCOOMRPqui	 =	-1* MIN [BPRpqui * COOMRPui, MAX [0, (PSui + POui)]]

Else:

PCOOMRPqui	 =	-1* MAX [0, (PSui + POui)]

If the unit is deemed to be online as described in 6.8.2.2(3)

Then:

PSui	=   0

If the unit is deemed to be offline as described in 6.8.2.2(2)

Then:

PSui	=	RCGCCc

POui	=	SUM [(RCGFCc - MCPEjz) * MINCAPui]j / 4

The equation below will be used to determine the Total OOM Capacity Payments to be allocated to each QSE as described in Section 6.9.7.1, OOM Capacity Charge.

PCOOMRPqi	 = 	SUM (PCOOMRPqui)q

Where:

c:	Resource Category

i:	hourly interval

j:	Settlement intervals within the hourly interval i

u:	single Resource

z:		zone

q:		QSE

PCOOMRPqui: 	OOM Replacement Reserve Capacity Payments by single Resource by interval for that QSE

PCOOMRPqi:	Total OOM Replacement Reserve Capacity Payment ($) by interval for that QSE (All OOM single Resources added together for that QSE)

BPRPui:	Bid Price for Replacement Reserve ($/MW) of the unit per interval

COOMRPui: 	Out of Merit Replacement Reserve Capacity awarded capacity (MW) per single Resource per interval

PCOOMRPi:	Summation of OOM Replacement Capacity Payment ($) per interval for all QSEs in the market

RCGCCc:	Resource Category Generic Capacity Cost for a specific category of generation unit.  RCGCCc = RCGSC/(number of hours).

PSui:	Price for starting a unit that is selected out of merit order to provideReplacement Reserve service.

POui:	Price for operating a unit that is selected out of merit order to provideReplacement Reserve service.

RCui:		Revenue credit for unit deployed

MINCAPu	Generating unit Minimum Sustainable Limit as reported in the Resource Plan.

RCGFCc:	Resource Category Generic Fuel Cost for a specific category at its Minimum Sustainable Limit level

MCPEiz	Market Clearing Price for Energy per interval for the zone where resource u is located.

- The calculation for additional compensation of Out –of Merit Service is as follows:

PCAOOMRPqi	=	SUM (PCAOOMRPqui)u

Given:

If BPRpqui exists,

Then:

PCAOOMRPqui	 =	-1* MIN [BPRpqui * COOMRPui,

(PSVui + POVui+PAVui) *110%]- PCOOMRPqui

Else:

PCOOMRPqui  =	-1* (PSVui + POVui+PAVui) *110%- PCOOMRPqui

If the unit is deemed to be online as described in 6.8.2.2(3)

Then:

PSVui	=   0

If the unit is deemed to be offline as described in 6.8.2.2(2)

Then:

PSVui	=	HVRSCu

POVui	=	SUM [(HRu * FRu- MCPEjz) * MINCAPui]j / 4

Where:

c:	Resource Category

i:	hourly interval

j:	Settlement intervals within the hourly interval i

u:	single Resource

z:		zone

q:		QSE

PCOOMRPqui: 	OOM Replacement Reserve Capacity Payments by single Resource by interval for that QSE

PCAOOMRPqui: 	Additional compensation for OOM Replacement Reserve Capacity service by single Resource by interval for that QSE

PCAOOMRPqi:	Total additional compensation for OOM Replacement Reserve Capacity service ($) by interval for that QSE (All OOM single Resources added together for that QSE)

BPRPui:	Bid Price for Replacement Reserve ($/MW) of the unit per interval

TVRSCu:	Verifiable start up cost of unit u approved by ERCOT Board of Directors.  Generic Start-Up Costs will be used before the verifiable cost is approved by ERCOT Board of Directors.

HVRSCu:	Hourly verifiable start up cost of unit u

(HVRSCu = TVRSCu / (Number of hours)

HRu:	Heat Rate of unit u at the Minimum Sustainable Limit level

FRu:	Fuel Cost of unit u which includes transportation cost per MMBTU

PSVui:	Verifiable cost for starting a unit that is selected out of merit order to provide Replacement Reserve service

POVui:	Verifiable cost for operating a unit that is selected out of merit order to provide Replacement Reserve service.

PAVui:	Other verifiable cost to provide Replacement Reserve service approved by ERCOT Board of Directors.

MINCAPu	Generating unit Minimum Sustainable Limit as reported in the Resource Plan.