| PRR Number | 371PRR | PRR Title | Payment for Minimum Energy when OOMC Issued |
| --- | --- | --- | --- |
| ###### Status |  |  |  |
| Protocol Section Requiring Revision |  | 1.3.1.1, Items Considered Protected Information [revised (6)] 6.5.10, Out-Of-Merit Capacity and Out-of-Merit Energy Services [added (9)] 6.8.2, Capacity and Energy Payments for Out-of Merit Service 6.8.2.1, Resource Category Generic Costs 6.8.2.2, Capacity Payments and Minimum Energy Payments [revised title] |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | Language implements a mandatory minimum OOME instruction for the same intervals that OOMC instructions are issued to a unit. |  |
| ###### Reason for Revision |  | Units that are issued an OOMC instruction are dispatched based on reasons other than economic market conditions. The OOMC instruction ensures that notwithstanding economics, energy from an instructed generator is available for instantaneous dispatch by ERCOT for system reliability. However, to provide instantaneous dispatch, a QSE must bring the instructed plant on-line and generate at least the minimum amount of energy required by plant design. In the absence of an OOME instruction for at least this amount of energy, the QSE solely bears the cost of the difference between market pricing and unit production cost. For most gas-fired units subject to OOMC, the heat rate at minimum generating levels is significantly higher than heat rate at higher load levels. As a result, the QSE must sell the minimum energy take into the balancing energy market as a price taker or back down a more efficient unit to make room for minimum energy on for OOMC purposes. The Protocols provide for QSEs to be paid an OOMC payment based on the unit type. However, the revenue provided in the form of an OOMC payment and the balancing market will frequently not cover the QSE’s cost to generate at the highest point on a unit’s heat rate curve and/or back down a more efficient unit in the QSE’s fleet. In effect, QSE’s issued OOMC instructions, but not corresponding OOME instruction, are subsidizing ERCOT’s need for generation to meet system reliability. The following example illustrates the financial impact of this mismatch between OOMC and OOME: A Gas Cyclic Unit with a minimum load of 120 MW is issued an OOMC instruction to be available for 20 hours of a day. The QSE will receive a maximum payment of $58,000 under the Protocols’ OOMC calculation, and assuming a UBES average price of $23.5/MWh, receives an additional $56,400 in UBES payments yielding total revenue of $ 114,400. On the cost side, the unit heat rate at minimum load is 15,000 BTU/kWh, or $60/MWh at a gas price of $4.00/MMBTu. In addition, the QSE incurs a unit start cost of $8,000 and a gas transportation charge of $10,000, making the unit’s total cost to run at minimum load for the day $ 162,000. In total, the QSE in this example loses $ 47,600. The Protocols do provide for a dispute process in the situation described and can recover actual cost plus 10%. Under this process, the QSE provides documentation of actual costs and once approved will receive payment covering the cost shortfall. In essence, therefore, a QSE will ultimately be paid an amount approximately equal to the amount it would have been paid had it received an OOME instruction. Accordingly, the existing process arrives, after additional time and administrative cost to ERCOT and the QSE at the same result that would occur if ERCOT is required to issue an OOME Up instruction equal to minimum unit load for any unit that it also issues an OOMC instruction. The proposed revision ensures this objective. |  |
| ###### Known Benefit for Revision |  |  |  |
| ## Timeline |  |  |  |
| Date Received |  | 10/28/02 |  |
| Date Posted |  | 10/28/02 |  |
| Comments Due |  | 11/18/02 |  |
| PRS Review Date |  | 05/05/03 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  |  |  |
| Board Consideration |  |  |  |

Proposed Protocol Language Revision

#### 1.3.1.1  Items Considered Protected Information

“Protected Information” is information containing or revealing:

- Schedule Control Error, as calculated by ERCOT, identified to a specific QSE, with the exception of real-time SCE data that may be viewable on-site at ERCOT facilities;

- Bids or pricing information identifiable to a specific QSE, except as provided in 1.3.3(2)(b);

- Status of Resources including but not limited to outages or limitations or scheduled or metered Resource data;

- Resource Plans;

- Energy and Ancillary Service Schedules identifiable to a specific QSE;

- ERCOT Dispatch Instructions identifiable to a specific QSE, except for OOMC deployments as provided in 6.5.10(9);

### 6.5.10	Out-Of-Merit Capacity and Out-of-Merit Energy Services

- ERCOT will use OOMC and OOME Services to procure additional capacity and energy required to provide reliable ERCOT System operation, as determined by ERCOT.

- Any Generation Resource or Load acting as a Resource may be called upon by ERCOT to provide OOMC or OOME Service, in any time frame that the Resource or Load is listed as available in the Resource Plan.

[PRR369: Add (3) under Section 6.5.10 upon system implementation & notice to the Market.]

(3)	For combined cycle plants, or other plants that involve tandem operation of units, a Generation Entity may request ERCOT approval to treat the plant at which these units are located as an Aggregated Unit.

- OOMC is used by ERCOT only when necessary to provide ERCOT System security and capacity adequacy. ERCOT will call on OOMC and OOME Service in such a way as to minimize the use of this service as much as practicable.

- OOME will be settled in accordance with Balancing Energy settlement provisions in Section 6.8.2.2, Energy Payments.

- Bids from Loads acting as Resources may also be used by ERCOT under the OOME instructions and pricing structure.

- The QSE associated with the Generation Resource or Load acting as a Resource that receives a Dispatch Instruction to provide OOMC and/or OOME Service(s) must use all commercially reasonable efforts to provide the requested service(s).  If the QSE declines the Dispatch Instruction according to provisions of Section 5.4.4, Compliance with Dispatch Instructions, to provide OOMC and/or OOME Service(s), ERCOT will post such declines on the MIS.

- In the event that OOMC is ordered to sustain reliable ERCOT System operation, the QSE must bid into the Balancing Energy Up and Down market quantities which sum greater or equal to the High Sustainable Limit (if unavailable, the High Operating Limit) minus the Low Sustainable Limit (if unavailable, the Low Operating Limit) of the unit receiving the OOMC instruction, in the zone of the OOMC unit.

- Each day, ERCOT will post on the MIS for the previous Operating Day:  (1) each unit receiving an OOMC Dispatch Instruction; (2) intervals for which a unit received an OOMC Dispatch Instruction and minimum load for that instruction; and (3) intervals for which and quantity of energy deployed by ERCOT for each unit receiving an OOMC Dispatch Instruction.

6.8.2	Capacity and Energy Payments for Out-of-Merit Service

6.8.2.1	Resource Category Generic Costs

To properly assign Local Congestion costs, it is necessary to establish certain generic costs associated with Resources that will be used to calculate production costs incurred when the Resource provides Out-of-Merit Service.  These generic resource costs include: generic fuel cost and generic startup costs.

(1)	Each ERCOT Generation Resource will be assigned to one of the following Resource Categories for the purpose of determining generic fuel costs:

Nuclear

Hydro

Coal and Lignite

Combined Cycle greater than 90 MW*

Combined Cycle less than or equal to 90 MW*

Gas Steam Supercritical Boiler

Gas Steam Reheat Boiler

Gas Steam Non-reheat or boiler without air-preheater

Simple Cycle greater than 90 MW

Simple Cycle less than or equal to 90 MW

Diesel (and all other diesel or gas-fired resources)

Renewable (i.e., non-Hydro renewable resources)

*Determined by capacity of largest simple-cycle combustion turbine in the train

The category of each Resource will be reported to ERCOT by the Generation Entity.  Each Generation Entity shall ensure that each of its Resources is in the correct Resource category.

(2)	Resource Category Generic Fuel Costs

Each ERCOT Generation Resource will be assigned a Resource Category Generic Fuel Cost (RCGFC) based on the Resource Category to which it is assigned.  For Nuclear, Hydro, and Coal and Lignite Resources, the RCGFC will be a fixed dollar/MWh amount as shown below.  For the remaining resource categories (except Renewable), the RCGFC will be the product of a heat rate (based on the heat rates used for the Capacity Auction) and a Fuel Index Price (FIP).  The RCGFC for Renewable Resources will be $0/MWh.

The FIP shall be the Midpoint price, expressed in $/MMBTU, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day of [the OOMC or] OOME deployment.  The FIP for Saturdays, Sundays, holidays and other days for which there is no FIP published in Gas Daily, shall be the next published FIP after the day of [the OOMC or] OOME deployment.  In the event that the FIP is not published for more than two (2) days, the previous day published FIP will be used for initial settlement and the next day published FIP will be used for true up of the final settlement statement.

The RCGFC for each type of Resource for upward instructions will be:

Nuclear = $15.00/MWh

Hydro = $10.00/MWh

Coal and Lignite = $18.00/MWh

Combined Cycle greater than 90 MW* = FIP * 10 MMBtu/MWh

Combined Cycle less than or equal to 90 MW* = FIP * 10 MMBtu/MWh

Gas-Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh

Gas Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh

Gas Steam Non-reheat or boiler without air-preheater = FIP * 14.5 MMBtu/MWh

Simple Cycle greater than 90 MW = FIP * 15 MMBtu/MWh

Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh

Diesel = FIP * 16 MMBtu/MWh

Renewable = $0/MWh

*Determined by capacity of largest simple-cycle combustion turbine in the train

The RCGFC for each type of Resource for downward instructions will be:

Nuclear = $0.00/MWh

Hydro = $0.00/MWh

Coal and Lignite = $3.00/MWh

Combined Cycle greater than 90 MW* = FIP * 5 MMBtu/MWh

Combined Cycle less than or equal to 90 MW* = FIP * 5 MMBtu/MWh

Gas Steam Supercritical Boiler = FIP * 7.5 MMBtu/MWh

Gas Steam Reheat Boiler = FIP * 9.5 MMBtu/MWh

Gas-Steam Non-reheat or boiler without air-preheater = FIP * 10.5 MMBtu/MWh

Simple Cycle greater than 90 MW = FIP * 10.5 MMBtu/MWh

Simple Cycle less than or equal to 90 MW = FIP * 10.5 MMBtu/MWh

Diesel = FIP * 12 MMBtu/MWh

Renewable = $0/MWh

*Determined by capacity of largest simple-cycle combustion turbine in the train

(3)	Resource Category Generic Startup Costs

Resource Category Generic Startup Costs (RCGSC) represents the startup cost of capacity used for Replacement Reserve Service.  The RCGSC for each type of resource will be:

Option 1:

Combined Cycle greater than 90 MW* = $5,000 + (FIP * (To be determined)MMBtu/MW * RMCU)

Combined Cycle less than or equal to 90 MW* = $2,310 + (FIP * (To be determined) MMBtu/MW * RMCU)

Option 2:

For four hours or more between shutdown and startup for an OOMC instruction:

Combined Cycle greater than 90 MW* = $6,810 + (FIP * 2,200 MMBtu)

Combined Cycle less than or equal to 90 MW* = $5,310 + (FIP * 1,200 MMBtu)

For less than four hours between shutdown and startup for an OOMC instruction:

Combined Cycle greater than 90 MW* = $6,810 + (FIP * 1,100 MMBtu)

Combined Cycle less than or equal to 90 MW* = $5,310 + (FIP * 600 MMBtu)

Gas Steam Supercritical Boiler = $4,800 + (FIP * 16.5 MMBtu/MW * RMCU)

Gas Steam Reheat Boiler = $3,000 + (FIP * 9.0 MMBtu/MW * RMCU)

Gas Steam Non-reheat or boiler without air-preheater = $2,310 + (FIP * 2.30 MMBtu/MW *  RMCU)

Simple Cycle greater than 90 MW = $5,000 + (FIP * (To be determined)MMBtu/MW * RMCU)

Simple Cycle less than or equal to 90 MW = $2,300 + (FIP * (To be determined)MMBtu/MW * RMCU)

Renewable	=	$0

*Determined by capacity of largest simple-cycle combustion turbine in the train

Where:

RMC = Resource Maximum Capacity (in MW)

u = unit

- Resource Category Generic Minimum Energy Cost

Resource Category Generic Minimum Energy Cost (RCGMEC) is the heat rate of a unit in this category at either its minimum stable load or minimum environmental operating point when the Resource is selected to provide Out-of-Merit Service multiplied by the Fuel Index Price as defined in Section 6.8.2.2(2).  The RCGMEC for each type of resource will be:

Combined Cycle greater than 90 MW* = 10 MMBtu/MWh * FIP

Combined Cycle less than or equal to 90 MW* = 10 MMBtu/MWh * FIP

Gas Steam Supercritical Boiler = 16.5 MMBtu/MWh * FIP

Gas Steam Reheat Boiler = 17.0 MMBtu/MWh * FIP

Gas Steam Non-reheat or boiler without air-preheater = 19.0 MMBtu/MWh * FIP

Simple Cycle greater than 90 MW = 15.0  MMBtu/MWh * FIP

Simple Cycle less than or equal to 90 MW = 15.0 MMBtu/MWh * FIP

*Determined by capacity of largest simple-cycle combustion turbine in the train

6.8.2.2	Capacity and Minimum Energy Payments

(1)	OOMC Service may be used by ERCOT as a procured Replacement Reserve Resource in the Adjustment Period where necessary to support emergency operations and provide voltage support, stability or to manage localized transmission limitations.  All Resources that are available and plan to be off-line during the interval for which Ancillary Services are being procured are eligible to be selected to provide OOMC Service.  OOMC Service may be used only if there is no Resource available through the Specific Process for Replacement Reserve Service procurement defined in section 6.6.3.2.1 or through contracted RMR Service.  ERCOT shall not issue an OOME Up instruction for the energy associated with the Minimum Sustainable Limit of a Resource (i.e. the higher of the mechanical or environmental minimum load) for which it has issued an OOMC instruction.  However, for settlement purposes, ERCOT shall treat a resource selected to provide OOMC Service as if that resource was issued an OOME Up instruction for the energy associated with the Minimum Sustainable Limit of the Resource.

(2)	Resources selected to provide OOMC Service that actually reconnect to the ERCOT Transmission Grid and start the unit in order to provide the OOMC Service will be paid both the Resource Category Generic Capacity Cost for starting the unit as well as the Resource Category Generic Minimum Energy Cost (for upward instructions) less the MCPE  for operating at the Minimum Sustainable Limit of the Resource during the instructed interval(s).

- Resources that are connected to the ERCOT transmission grid when instructed to provide OOMC Service will be paid the Resource Category Generic Minimum Energy Cost (for upward instructions) less the MCPE for operating at the Minimum Sustainable Limit of the Resource during the instructed interval(s).

- When the Resource Specific OOMC instruction is issued, the instructed deviation resulting from the OOMC instruction will create no Re-Balancing Requirement or Uninstructed Resource Charge during the instructed interval(s), including during the ramp of a Resource selected to provide OOMC Service that must reconnect to the ERCOT Transmission Grid.

- For instruction intervals in which a Resource has been called upon by ERCOT to provide OOMC Service, for all energy supplied to the Balancing Energy Market by the Resource in excess of the Minimum Sustainable Limit that is not produced pursuant to an OOME instruction, the Resource shall be paid the MCPE.

During the ramp of a Resource which has been called upon by ERCOT to provide OOMC Service, (i) for intervals during the ramp when the MCPE is positive the MCPE payment to the resource ramping up to provide OOMC Service will be credited back to ERCOT; and (2) for intervals during the ramp when the MCPE is negative the resource ramping up to provide OOMC Service will not be charged the MCPE.

(6)	If a Resource is called to provide OOMC Service and the payment for OOMC Service is insufficient to cover all costs of providing the service and provide a premium, then that Resource will be paid all verifiable costs in excess of the OOMC payment that are directly attributable to the OOMC Service plus a premium.  These costs may include the cost of exceeding swing gas contract limits, additional gas demand costs set by fuel supply or transportation contracts, and any additional costs to purchase emissions credits or other costs incurred due to environmental regulations.  The premium to be provided shall be the product of the costs of providing the service times 10%.  Verification of these costs must be submitted to ERCOT to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  QSEs requesting such cost recovery shall perform the following:

(a)	Submit a settlement dispute in accordance with the dispute process outlined in Section 9.5.  In addition to the standard information required on the entry form on the ERCOT Portal, the dispute should clearly indicate:

(i)	The deployment instruction received from ERCOT to provide services.

(ii)	The payment received for providing the service.

(iii)	The actual cost of providing the service.

(iv)	A reference to the documentation to be provided in writing as indicated in section 6.8.2.2(6)(b).

(b)	Requests for additional compensation shall provide documentation in writing to allow for verification of the claimed amounts for each cost type listed below.  Except for Resource Fuel Costs for OOMC, compensation for cost types shall be reviewed and approved by the ERCOT Board of Directors during an Executive Session of their next regularly scheduled meeting.  Documentation requirements for each cost type are as follows:

- Resource Startup Cost for OOMC:  Resources can substantiate starting fuel costs, including fuel for ramp to Minimum Sustainable Limit of a Resource, based on actual plant data consisting of either meter readings or invoices.  Starting maintenance costs will be substantiated by a service agreement from the Resource’s maintenance provider.  If a Resource does not have in effect a service agreement, an estimate of starting maintenance costs prepared by an unaffiliated maintenance service provider will suffice.

(ii)	Resource Fuel Costs for OOMC:

Fuel cost will require supporting documentation of sufficient detail to allow for the verification of the cost of fuel consumed at the deployed resource. Documentation may include contracts, invoices or other documents all of which will be treated as confidential.  All copies of such documentation will be returned upon completion of the review.  For gas fired resources, such documentation will not be required if the requested incremental fuel cost is less than 115% of the Fuel Index Price.

Resources can substantiate fuel consumption using heat rate curves.  Anytime ERCOT’s OOMC deployment is generator specific, the heat rate curve of the associated energy production device will be used determine fuel consumption, i.e., when the generator of a combustion turbine receives an OOMC instruction, the heat rate will be the heat rate of that unit when it is not in combined cycle configuration (emergency heat rate curve).  If the deployment is coordinated and allows for combined cycle operation, and the facility is capable of combined cycle operation, the heat rate will be for that mode of operation (normal operation).  However, for a Combined Cycle Resource that produces Simple Cycle energy prior to achieving Combined Cycle operation, such Resource will be compensated at its Simple Cycle heat rate for the energy produced when in Simple Cycle mode until the steam turbine starts producing energy, at which time the compensation will switch to the Combined Cycle rate.  Each Resource will submit heat rate curves and costs for all modes of operation.

Unit total heat rate curves (by season, due to weather’s effect on the heat rate) will be based on design modified by actual unit test data.  Actual unit test data should be resubmitted by the Resource annually.  The actual heat rate calculation will be input fuel MMBtu/net MWh.  Net MWh will be determined at the facility revenue meter(s).  Resource performance data will be provided to support the heat rate curve when available.  When performance data is not available the unit’s own fuel flow metering device(s) will be used as support for the heat rate curve.

(iii)	Fuel Transportation Cost:  Resources can substantiate claims for Fuel Transportation Cost by submitting to ERCOT copies of transportation contracts with applicable invoices.  All copies of such documentation will be returned upon completion of the review.  For gas-fired Resources, such documentation will not be required if the requested incremental fuel cost is less than 115% of the Fuel Index Price.

  - Swing Costs:  When calculating the payment for fuel, if there is an additional charge for gas purchased on the same gas day or late in the gas day, the additional charge per unit of fuel delivered should be included.  Appropriate supporting documentation for the fuel delivery cost includes third party invoices and contracts.  Invoices need to be filed monthly, with a month lag, and contracts would stay on file with ERCOT until expiration.

  - Emission credit purchases:  When calculating the payment for environmental costs, the Resource shall be paid any additional costs to purchased emissions credits or other costs incurred due to the environmental regulations.  Appropriate supporting documentation includes invoices and contract for such purchased emission credits or such other costs.

  - Chemical Costs:  When calculating the payment for chemicals, the sum of the prior year chemical usage divided by the MWh the Resource produced is to be used as the price per MWh.  Prior year usage should be substantiated with inventory records and general ledger detail.  Due to many Resources using a LIFO or FIFO method for chemical expense, monthly invoices are not an acceptable means of support.

(vii)	Water Costs:  When calculating the payment for water costs, fixed charges for water should not be included in the cost of the water.  Variable charges for water, supported by an invoice or contract from the provider of the water, should be prorated over the number of MWh produced during the month and reimbursed.  Contracts with the water provider should be initially filed with ERCOT and remain on file until expiration.  Invoices should be filed monthly with ERCOT, with a one-month lag.

(c)	Resource Premium

The premium to be provided to a Resource shall be the sum of the costs outlined above times 10%.

(d)	Any and all information or documentation supplied to ERCOT for verification of the costs claimed in Section 6.8.2.2(6)(b) will be kept strictly confidential by ERCOT and not provided to any party, person, or entity other than those individuals on ERCOT Staff who require the information in order to perform their duties.

Any contract supplied to ERCOT as supporting documentation will remain on file with ERCOT and be used for settlement purposes until all instructions in the periods covered under the contract have been settled, at which time the contract shall be returned to the Generation Entity or destroyed.  Any invoice filed monthly, with a one-month lag, will be used for settlement purposes until all instructions in the period covered under the invoice have been settled, at which time the invoice shall be returned to the Generation Entity or destroyed.  ERCOT will settle and pay all costs for OOMC Service provision in accordance with the settlements timeline in the protocols using filings that are valid for an extended period of time (i.e., heat rate curves, contracts with a greater than one month period).  Any costs that change daily requiring invoices to be filed with a one month lag will be settled on the final statement for the last day of the month in question (i.e., for the month of January, all fuel invoices would have to be filed by the end of February, and then all fuel costs for January would be settled and paid on the final statement for January 31, which would be 59 days after January 31).

(7)	The calculation for capacity payments and minimum energy of Out-of-Merit Service is as follows:

PCOOMRPqi	=	SUM (PCOOMRPqui)u

Given:

If BPRpqui exists,

Then:

PCOOMRPqui	 =	-1* MIN [(BPRpqui * COOMRPui), (PSui + POui)]

Else:

PCOOMRPqui	 =	-1* (PSui + POui)

If the unit is deemed to be online as described in 6.8.2.1(3)

Then:

PSui	=   0

If the unit is deemed to be offline as described in 6.8.2.1(2)

Then:

PSui	=	RCGCCc

POui	=	SUM [(RCGMECc – MCPEjz) * MIN (MINCAPu, SCADAuj)i]/4

The equation below will be used to determine the Total OOM Capacity Payments to be allocated to each QSE as described in Section 6.9.7.1, OOM Capacity Charge.

PCOOMRPqi	 = 	SUM (PCOOMRPqui)q

Where:

c:	Resource Category

i:	hourly interval

j:	Settlement intervals within the hourly interval i

u:	single Resource

z:	zone

q:	QSE

PCOOMRPqui: 	OOM Replacement Reserve Capacity Payments by single Resource by interval for that QSE

PCOOMRPqi:	Total OOM Replacement Reserve Capacity Payment ($) by interval for that QSE (All OOM single Resources added together for that QSE)

BPRPui:	Bid Price for Replacement Reserve ($/MW) of the unit per interval

COOMRPui: 	Out of Merit Replacement Reserve Capacity awarded capacity (MW) per single Resource per interval

PCOOMRPi:	Summation of OOM Replacement Capacity Payment ($) per interval for all QSEs in the market

RCGCCc:	Resource Category Generic Capacity Cost for a specific category of generation unit.  RCGCCc = RCGSC/(number of instructed hours).

RCGMECc:	Resource Category Generic Minimum Energy Cost for a specific category of generation unit for an upward instruction.

PSui:	Price for starting a unit that is selected out of merit order to provide balancing energy.

POui:	Price for operating a unit that is selected out of merit order to provide balancing energy.

MINCAPu:	Generating unit Minimum Sustainable Limit as reported in the Resource Plan.

SCADAuj:	Average actual MW output of a Resource over the interval.

| ## Sponsor |  |
| --- | --- |
| Name | Mike Cunningham |
| E-mail Address | mcunningham@pwrteam.com |
| Company | Exelon |
| Company Address | 300 Exelon Way, Kennett Square, Pa. 19348 |
| Phone Number | 610-765-6545 |
| Fax Number | 610-765-6595 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |