| PRR Number | 373PRR | PRR Title | Define OOME as Instructed Deviation and Portfolio Out Of Merit Energy Service |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | Sections 6.7.7 |  |
| Requested Resolution |  |  |  |
| Revision Description |  | This PRR provides a method to define Out Of Merit Energy Service as Instructed Deviation. # Define OOME as Instructed Deviation–October 7, 2002 |  |
| ###### Reason for Revision |  | This protocol revision will provide a more consistent unit specific and portfolio deployment mechanism when manual OOME instructions are issued. All market participants, especially those participants with small generation portfolios, can benefit from this PRR in following the unit specific instructions, maintaining the Schedule Control Error while not being struck by Uninstructed Resource Charge. |  |
| ## Timeline |  |  |  |
| Date Received |  | 10/29/2002 |  |
| Date Posted |  | 10/31/2002 |  |
| Comments Due |  | 11/21/2002 |  |
| PRS Review Date |  | 11/26/2002 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  | 12/05/2002 |  |
| Board Consideration |  | 12/17/2002 |  |

Proposed Protocol Language Revision

### 6.7.7	Deployment of Out-of-Merit Energy Service

Deployment of units for OOME Service will follow Balancing Energy Service deployment guidelines as specified in Section 5, Dispatch of these Protocols.

      - When the Resource Specific OOME instruction is issued before the Balancing Energy Market clearing, the instructed deviation resulted from the OOME instruction will be defined as the difference between the instructed output level and the Resource Plan output level at the close of the Adjustment Period for the target service interval and the instructed deviation will be balanced using the balancing energy bid stack.  OOME instructions will be sent to QSEs concurrent with Balancing Energy instructions for the target interval.  The Balancing Energy deployment will be limited by the constraints imposed by the OOME instructions.  The instructed deviations for these OOME instructions will be included in the Balancing Energy instructions for that QSE for the target interval.

      - Resource Specific OOME instructions issued after the Balancing Energy Market clearing will not be instructed deviations.  ERCOT will endeavor to minimize these instructions.

| ## Sponsor |  |
| --- | --- |
| Name | Larry Gurley |
| E-mail Address | Lgurley1@txu.com |
| Company | TXU Energy |
| Company Address | 1717 Main St., Dallas, TX, 75201 |
| Phone Number | 214-875-9020 |
| Fax Number | 214-875-9356 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |