| PRR Number | 373PRR | PRR Title | Define OOM as Instructed Deviation |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Proposed Effective Date |  | Pending analysis and project start. |  |
| Priority & Rank Assigned |  | 2.1 |  |
| Summary of Impact Analysis |  | Impacts the ERCOT computer system. |  |
| Benefit |  | This protocol revision will provide a more consistent unit specific and portfolio deployment mechanism when manual OOME instructions are issued. All market participants, especially those participants with small generation portfolios, can benefit from this PRR in following the unit specific instructions, maintaining the Schedule Control Error while not being struck by Uninstructed Resource Charge. |  |
| PRR Background |  | ## This PRR provides a method to define Out Of Merit Energy Service as Instructed Deviation. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval as amended by the WMS comments. |  |

Recommended Language

WMW [sic, WMS] reviewed this PRR along with comments and recommends the following.

### 6.7.7	Deployment of Out-of-Merit Energy Service

Deployment of units for OOME Service will follow Balancing Energy Service deployment guidelines as specified in Section 5, Dispatch of these Protocols.

      - When the Resource Specific OOME instruction is issued before the Balancing Energy Market clearing, the instructed deviation resulted from the OOME instruction will be defined as the difference between the instructed output level and the Resource Plan output level at the close of the Adjustment Period for the target service interval and the instructed deviation will be balanced using the balancing energy bid stack.  OOME instructions will be sent to QSEs concurrent with Balancing Energy instructions for the target interval.  The Balancing Energy deployment will be limited by the constraints imposed by the OOME instructions.  The instructed deviations for these OOME instructions will be included in the Balancing Energy instructions for that QSE for the target interval.  ERCOT must honor the unit ramp rates as indicated by the QSE’s Resource Plan.

      - Resource Specific OOME instructions issued after the Balancing Energy Market clearing will not be instructed deviations.  ERCOT will use best efforts to minimize instructions issued after the Balancing Energy Market has cleared.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| TXU | Revised language. |
| Frontera | Expressed support for revision. |
| Garland | Revised language. |