| PRR Number | 618 | PRR Title | Balancing Energy Up from a Specific LaaR Resource |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Recommended Action |  | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 7.4.3.1, Balancing Energy Up from a Specific Resource 7.4.3.2, Balancing Energy Down from a Specific Resource |  |  |
| Proposed Effective Date |  | December 1, 2005 |  |  |
| Priority and Rank Assigned |  | n/a |  |  |
| Revision Description |  | These proposed revisions provide an equation that describes how a QSE representing a Load Acting as Resource (LaaR) receives compensation when a LaaR it represents provides Balancing Energy Up, and clarifies that only Generation Resources will be paid for Balancing Energy Down to resolve Local Congestion. |  |  |
| Overall Market Benefit |  | Clarification regarding LaaRs’ ability to provide service to resolve Local Congestion. |  |  |
| Overall Market Impact |  | None |  |  |
| Consumer Impact |  | None |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. ERCOT credit staff and the CWG have reviewed PRR611 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - ERCOT posted PRR618 and its Impact Analysis (IA) on 7/28/05. - PRS considered this PRR on 8/24/05. - On 9/29/05, PRS reviewed the impacts that may result from this PRR. - On 10/6/05, TAC considered this PRR. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 8/24/05, PRS voted to approve this PRR as amended by ERCOT at PRS. The motion passed with four abstentions from the Consumer, Independent Generator, and Independent Power Marketer segments. All market segments were present. |  |  |
| Summary of PRS discussion |  | On 8/24/05, ERCOT Staff explained that although it appears to be the policy to pay LaaRs that have been selected to resolve congestion, there is no language in the Protocols that lays out how these LaaRs are to be paid. This PRR proposes to settle such LaaR deployments at the generic rate. ERCOT Staff also introduced additional language to the PRR. On 9/29/05, PRS noted that there was no impact as a result of this PRR and, therefore, does not require a system change. |  |  |
| TAC Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | TAC unanimously voted to recommend approval of PRR618 with all market segments present. |  |  |
| Summary of TAC discussion |  | No discussion. |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | None | None | Clarification regarding LaaRs’ ability to provide service to resolve Local Congestion. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | NA |
| LSE: General, Including NOIE | None | None | NA |
| LSE: CR & REP | None | None | NA |
| QSE | Change in ability to bid for LaaRs to solve Local Congestion | Not known | Clarification regarding LaaRs’ ability to provide service to resolve Local Congestion. |
| Resource | None | None | Clarification regarding LaaRs’ ability to provide service to resolve Local Congestion. |
| TDSP | None | None | NA |

| Original Sponsor |  |
| --- | --- |
| Name | Inocencio Gonzalez |
| Company | igonzalez@ercot.com |
| Segment | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None |  |

Proposed Protocol Language Revision

7.4.3.1	Balancing Energy Up from a Specific Resource

(1)	In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever a specific Resource is deployed incrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Resource is located and the incremental bid premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

[PRR485: Replace the paragraph above with the following once the system change is implemented:] In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed incrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Resource is located and the incremental bid premium specified. Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

The calculation of the payment for Balancing Energy Up from a specific Resource is as follows:

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLqiuz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqivz, NETUEQivq)) * LBEAGRivq)

PMqiuz	=	Max (BPMqiuz, MCPEiz )

PMqivz	=	Max (BPMqivz, MCPEiz )

BPMqivz 	= 	Min (BPMqiuz)

NETUEQivq	=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq =	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 	=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSUquiz	Payment to QSE for providing Balancing Energy Up from Specific unit to solve Local Congestion per QSE per interval per zone

LPCRSUqviz	Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone

PMqiuz	Incremental Premium Specified for that specific unit per interval

PMqivz	Incremental Premium Specified for that specific Aggregated Unit per interval

BPMqiuz	Incremental bid premium for that specific unit per interval

BPMqivz	Incremental bid premium for that specific Aggregated Unit per interval

OLqiuz	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

IOLqiuz	The Instructed output level for the unit given the unit-specific instruction

IOOMUPqui	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNqui	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval

MRqivz	Meter reading per Aggregated Unit per interval

MCPEiz	Market Clearing Price for Energy per interval per zone

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value

NETLBEDQivq	The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

LBEAGRivq	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market

[PRR569: Replace the formulas above with the following once the system change is implemented:] LPCRSUquiz = -1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLqiuz, IOLqiuz - OLqiuz))) LPCRSUqviz = -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqivz, NETUEQivq)) * LBEAGRivq) For gas-fired units: FBPMqiuz = (BPMqiuz / FIPd-1) * FIPd PMqiuz = Max (FBPMqiuz, MCPEiz ) PMqivz = Min(PMqiuz) For all other Resource categories: PMqiuz = Max (BPMqiuz, MCPEiz ) PMqivz = Min(PMqiuz) NETUEQivq = Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))] NETOOMUEQivq = Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u)) NETLBEUQivq = Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)) NETOOMDEQivq = Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)) NETLBEDQivq = Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)) LBEAGRivq = [(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] Where: d Operating Day of interval i i interval q QSE u individual generation unit (including units within an Aggregated Unit) v Aggregated Unit z zone BPMqiuz Incremental bid premium for that specific unit per interval FBPMqiuz Incremental bid premium, adjusted for fuel price, for that specific unit per interval FIPd Fuel Index Price for Operating Day of interval i FIPd-1 Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i IOLqiuz The Instructed output level for the unit given the unit-specific instruction IOOMDNqui OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval) IOOMUPqui OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval) LPCRSUquiz Payment to QSE for providing Balancing Energy Up from Specific unit to solve Local Congestion per QSE per interval per zone LPCRSUqviz Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone LBEAGRivq The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market LBEDQqui Local Balancing Energy Down Instructions for that interval per specific unit LBEUQqui Local Balancing Energy Up Instructions for that interval per specific unit MCPEiz Market Clearing Price for Energy per interval per zone MRqiuz Meter reading per unit per interval MRqivz Meter reading per Aggregated Unit per interval NETLBEDQivq The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value NETLBEUQivq The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value NETUEQivq The overall net direction per the instructions given to the Aggregated Unit results in an up direction OLqiuz Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval OLqivz Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval PMqiuz Incremental Premium Specified for that specific unit per interval PMqivz Incremental Premium Specified for that specific Aggregated Unit per interval

(2)	Whenever a LaaR with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified as defined below:

LPCLAARquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (OLqiuz -LAARTOTqiuz, OLqiuz - IOLqiuz)))

PMqiuz	= Max (FBPMqiuz, MCPEiz )

FBPMqiuz	= (BPMqiuz  / FIPd-1) * FIPd

Where:

i	interval

q	QSE

u	individual LAAR unit

z	zone

BPMqiuz	Incremental bid premium for that specific LaaR unit per interval

FBPMqiuz	Incremental bid premium, adjusted for fuel price, for that specific unit per interval

IOLqiuz	The Instructed output level for the LaaR unit given the unit-specific instruction

LAARTOTqiuz	Actual ESI ID usage per unit per interval

LPCLAARquiz	Payment to QSE for providing Balancing Energy Up from specific LaaR unit to solve Local Congestion per QSE per interval per zone

MCPEiz	Market Clearing Price for Energy per interval per zone

OLqiuz	Resource Plan output level submitted by the QSE for the specific LaaR unit given the unit-specific instruction for that interval

PMqiuz	Incremental Premium specified for that specific LaaR unit per interval

7.4.3.2	Balancing Energy Down from a Specific Resource

In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever a specific Generation Resource is deployed decrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Generation Resource is located and the decremental bid premium specified.

[PRR485: Replace the paragraph above with the following once the system change is implemented:] In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Generation Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Down, is deployed decrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Generation Resource is located, and the decremental bid premium specified.

The calculation of the payment for Balancing Energy Down from a specific Resource is as follows:

LPCRSDquiz 	= 	-1 * Σ((MCPEiz - BPMqiuz ) * Max(0, Min(OLqiuz -MRqiuz , OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ((MCPEiz - BPMqivz ) * Max(0, Min(OLqivz -MRqivz, NETDEQivq)))v * LBEAGRivq

[PRR485: Replace the formulas above with the following once the system change is implemented:] LPCRSDquiz = -1 * Σ(MAX(0,(MCPEiz - BPMqiuz )) * Max(0, Min(OLqiuz -MRqiuz, OLqiuz - IOLqiuz)))u LPCRSDqviz = -1 * Σ(MAX(0,(MCPEiz - BPMqivz )) * Max(0, Min(OLqivz -MRqivz, NETDEQivq)))v * LBEAGRivq

BPMqivz 	= 	Max (BPMqiuz)

NETDEQivq	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq	=		Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=	Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq	=		Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSDquiz	Payment to QSE for providing Balancing Energy Down from Specific unit to solve Local Congestion per QSE per interval per zone

LPCRSDqviz	Payment to QSE for providing Balancing Energy Down from Aggregated Units to solve Local Congestion per QSE per interval per zone

BPMqiuz	Decremental bid premium specified for that specific unit per interval

BPMqivz	Decremental bid premium specified for that specific Aggregated Unit per interval

OLqiuz	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

IOLqiuz	The instructed output level for the unit given a unit-specific instruction

IOOMDNqui	OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPqui	OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

MCPEiz	Market Clearing Price of Energy per interval per zone

LBEUQqui	Local Balancing Energy Up instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down instructions for that interval per specific unit

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq	The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

LBEAGRivq	The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market.

[PRR569: Replace the formulas above with the following once the system change is implemented:] LPCRSDquiz = -1 * Σ(MAX(0,(MCPEiz - PMqiuz )) * Max(0, Min(OLqiuz -MRqiuz, OLqiuz - IOLqiuz)))u LPCRSDqviz = -1 * Σ(MAX(0,(MCPEiz - PMqivz )) * Max(0, Min(OLqivz -MRqivz NETDEQivq)))v * LBEAGRivq For gas-fired units: FBPMqiuz = (BPMqiuz / FIPd-1) * FIPd PMqiuz = FBPMqiuz PMqivz = Max(PMqiuz) For all other Resource categories: PMqiuz = BPMqiuz PMqivz = Max(PMqiuz) NETDEQivq = Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))] NETOOMUEQivq = Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)] NETLBEUQivq = Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)] NETOOMDEQivq = Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)] NETLBEDQivq = Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)] LBEAGRivq = [SUM(LBEUQqui)u + SUM(LBEDQqui)u] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] Where: d Operating Day of interval i i interval q QSE u individual generation unit (including units within an Aggregated Unit) v Aggregated Unit z zone BPMqiuz Decremental bid premium specified for that specific unit per interval FBPMqiuz Decremental bid premium, adjusted for fuel price, for that specific unit per interval FIPd Fuel Index Price for Operating Day of interval i FIPd-1 Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i IOLqiuz The instructed output level for the unit given a unit-specific instruction IOOMDNqui OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval) IOOMUPqui OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval) LBEAGRivq The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market. LBEDQqui Local Balancing Energy Down instructions for that interval per specific unit LBEUQqui Local Balancing Energy Up instructions for that interval per specific unit LPCRSDquiz Payment to QSE for providing Balancing Energy Down from Specific unit to solve Local Congestion per QSE per interval per zone LPCRSDqviz Payment to QSE for providing Balancing Energy Down from Aggregated Units to solve Local Congestion per QSE per interval per zone MCPEiz Market Clearing Price of Energy per interval per zone MRqiuz Meter reading per unit per interval. MRqivz Meter reading per Aggregated Unit per interval. NETDEQivq The overall net direction per the instructions given to the Aggregated Unit results in a down direction NETLBEDQivq The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value. NETLBEUQivq The net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value. NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value. NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value. OLqiuz Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval OLqivz Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval PMqiuz Decremental Premium for that specific unit per interval PMqivz Decremental Premium for that specific Aggregated Unit per interval