| PRR Number | 543 | PRR Title | Schedules and Emergency Assistance Over ERCOT DC Ties |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Recommended Action |  | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 2.1 Definitions 4.4.18.1 Control Area Operations 5.6.6.1 Emergency Electric Curtailment Plan (EECP) 5.6.7 EECP STEPS 5.6.8, Emergency Compensation 5.7 Block Load Transfers between ERCOT and Non-ERCOT Control Areas 5.7.3 Scheduling & Operation of BLTs 5.8 Back to Back Ties and BLTs with the CFE Control Area (New); and 6.8.2.1 Resource Category Generic Costs |  |  |
| Proposed Effective Date |  | March 1, 2006 |  |  |
| Priority and Rank Assigned |  | n/a |  |  |
| Revision Description |  | Incorporates the use of ties with Comision Federal de Electricidad (CFE) for emergency services. Clarify Protocols for the use of Back to Back Ties and Block Load Transfers (BLTs) with CFE during emergencies. |  |  |
| Overall Market Benefit |  | Greater reliability in Emergency Conditions. |  |  |
| Overall Market Impact |  | None |  |  |
| Consumer Impact |  | None |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. The ERCOT Credit Working Group has reviewed PRR 543 and believes that this PRR does not have any credit implications. Comision Federal de Electricidad will utilize a QSE in the ERCOT Region. As a result, any and all credit obligations will be calculated per the ERCOT Protocols. |  |  |
| Procedural History |  | - PRR 543 was posted on 9/14/04. - On 10/19/04, the Credit Working Group (CWG) posted comments. - On 10/19/04, ERCOT posted an Impact Analysis (IA) report. - On 11/10/05, ERCOT posted comments to PRR543. - On 11/14/05, ERCOT posted a revised IA that reflected ERCOT’s comments. - PRS considered this PRR on 11/17/05. - On 12/16/05, PRS reviewed the IA and the PRS Recommendation Report. - On 1/12/06, TAC considered this PRR. - On 1/16/06, CWG posted revised comments. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | PRS voted to recommend approval of this PRR as revised by ERCOT comments and PRS. The motion passed unanimously with all market segments present for the vote. |  |  |
| Summary of PRS discussion |  | ERCOT staff reviewed the revisions proposed by ERCOT Staff and explained the need for this PRR. PRS discussed whether the proposed provisions are legally tight; the treatment of Non Opt-in Entities (NOIEs) and impact on Unaccounted for Energy (UFE); and metering of BLT in relation to Transmission and Distribution Service Providers (TDSPs). PRS participants also ascertained that the QSE representing the Direct Current (DC) tie will receive the Dispatch Instruction and whether (in relation with subsection 5.7, Block Load Transfers between ERCOT and Non-ERCOT Control Areas) the BLT will be paid for even though ERCOT did not technically provide a Dispatch Instruction. |  |  |
| TAC Recommendation |  | TAC voted to recommend approval of this PRR as submitted by PRS. The motion passed unanimously with all market segments present. |  |  |
| Summary of TAC discussion |  | No discussion. |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | None | None | Greater reliability in emergency situations. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Greater reliability in emergency situations. |
| LSE: General, Including NOIE | None | None | Greater reliability in emergency situations. |
| LSE: CR & REP | None | None | Greater reliability in emergency situations. |
| QSE | Not known | Not known | Greater reliability in emergency situations. |
| Resource | Not known | Not known | Greater reliability in emergency situations. |
| TDSP | Not known | Not known | Greater reliability in emergency situations. |

| Original Sponsor |  |
| --- | --- |
| Name | Bill Bojorguez |
| Company | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| ERCOT 111005 | Proposed to limit these transactions to emergency purposes only and offered revised language. |
| CWG 011606 | Noted that because all transaction would be through a QSE that there would be no credit implications. |

Proposed Protocol Language Revision

2.1	Definitions

Back to Back (BtB) Tie

A Direct Current or other asynchronous tie between two different synchronous systems where all the equipment necessary to interconnect the two systems is contained within a single building or substation.

Comision Federal de Electricidad (CFE)

The state-owned federal commission of electricity of Mexico. The government agency in Mexico charged with the responsibility of operating the Mexican national electricity grid (outside Mexico City).

4.4.18.1	Control Area Operations

Control Area schedules between the ERCOT Control Area and interconnected non-ERCOT Control Area(s), through the use of schedules over Direct Current Tie(s), will be implemented in accordance with these Protocols, North American Electric Reliability Council (NERC) scheduling protocols, and in compliance with NERC operating policies.  Scheduling must also be in accordance with any applicable Federal Energy Regulatory Commission tariffs.

ERCOT will perform schedule confirmation with the applicable interconnected non-ERCOT Control Area(s) and, if appropriate, will coordinate the approval process for the NERC tags as both the ERCOT Control Area and on behalf of ERCOT TSPs.

5.6.6.1	Emergency Electric Curtailment Plan (EECP)

At times it may be necessary to reduce electrical demand because of a temporary decrease in available electricity supply.  To provide orderly, predetermined procedures for curtailing demand during such emergencies, ERCOT will initiate and coordinate the implementation of the Emergency Electric Curtailment Plan following the EECP Steps set forth below in subsection 5.6.7.1, Restoration of Market Operations.

The objective of the EECP is to provide for maximum possible continuity of service while maintaining the integrity of the ERCOT Transmission Grid in order to reduce the chance of cascading outages.

ERCOT’s operating procedures shall meet the following goals while continuing to respect the confidentiality of market sensitive data:

(1)	Use of the market to the fullest extent practicable without jeopardizing the reliability of the ERCOT system;

(2)	Use of Responsive Reserve Services and other Ancillary Services to the extent permitted by ERCOT System conditions;

(3)	Maximum use of ERCOT System capability;

(4)	Maintenance of station service for nuclear Generation Resource Facilities;

(5)	Securing of startup power for Generation Resources;

(6)	Operation of power Generation Resources during loss of communication with ERCOT;

(7)	Restoration of service to critical Loads in the manner defined in the Operating Guides; and

(8)	Restoration of service to all customers following major system disturbances, giving priority to the larger groups of customers.

ERCOT shall be responsible for coordinating with QSEs and TDSPs to monitor system conditions, initiating the EECP steps set forth in this Section, notifying all QSEs, and coordinating the implementation of the EECP Steps while maintaining transmission security limits.

During the EECP, ERCOT has the authority to obtain energy from BtB Ties or Block Load Transfers (BLTs) from non-ERCOT Control Areas when capacity is available.

5.6.7	EECP STEPS

Step 1 — Maintain 2300 MW of ERCOT’s Ancillary Service Resources comprised of: (1) undeployed Responsive Reserve Service; and (2) that amount of Non-Spinning Reserve Service which has been deployed but has not ramped to its specified output level.  ERCOT will:

(1)	Use those available Ancillary Services that can be deployed to increase Responsive Reserves.

(2)	Provide Dispatch Instructions to QSEs to start all Resources that are available in the time frame of the emergency (OOMC).  Similarly, ERCOT will provide Dispatch Instructions to QSEs to suspend any ongoing Generation Resource performance testing and maximize Resource deployment (OOME) to increase Responsive Reserve levels on other Resources.  QSEs intending to provide OOMC and OOME will comply as soon as practicable

(3)	Use available BtB  import capacity that is not already being used and inquire about availability of Block Load Transfers.

(4)	Start RMR Units available in the time frame of the Emergency.  RMR Units should be loaded to full capability.

5.6.8	Emergency Compensation

Deployment and compensation for OOMC and OOME Resources that ERCOT instructs for deployment due to a supply-related Emergency will be according to Section 6.8.2, Capacity and Energy Payments for Out of Merit Services.

There will be no compensation other than normal settlement methods to Loads in the event of Load shedding by the TDSP during EECP.

When energy is delivered to the ERCOT Control Area from a non-ERCOT Control Area through emergency instructions over a BtB Tie, the BtB Tie will be treated as a Resource receiving an OOME instruction via its QSE in the Congestion Zone where it is located.  For settlement purposes, the Resource Plan of the BtB Resource shall be set to zero.

When energy is delivered from the ERCOT Control Area through an emergency Dispatch Instruction over a BtB Tie, the BtB Tie will be treated as Load in the Congestion Zone in which the BtB Tie is located.  The QSE(s) representing the BtB Tie is responsible for any applicable fees and charges as described in Section 4.4.18.4

5.7	Block Load Transfers between ERCOT and Non-ERCOT Control Areas

Under Alert,  EECP, or other Emergency Conditions, it may be necessary for ERCOT to request the implementation of Block Load Transfer (BLT) schemes which will transfer Loads normally located in the ERCOT Control Area to a Non-ERCOT Control Area.  Similarly, when Non-ERCOT Control Areas experience certain transmission contingency or short supply conditions; it may be necessary for ERCOT to agree to the implementation of BLT schemes which will transfer Loads normally located in a Non-ERCOT Control Area to the ERCOT Control Area.  BLTs will be restricted to the following conditions:

(1)	BLTs shall occur only under a specific Dispatch Instruction from ERCOT.

(2)	BLTs that are looped systems may be tied to the other Control Area’s electrical system through multiple interconnection points at the same time.  Transfers of looped configurations will be permitted only if all interconnection points are netted under a single ESI ID.  Individual meter points for looped system BLTs must be within the same Congestion Zone.

(3)	BLTs of Load to the ERCOT Control Area will be: (i) treated as non-competitive wholesale Load in the ERCOT settlement system, (ii) registered in a manner similar to that of NOIEs, and (iii) responsible for UFE allocations and Transmission Losses consistent with similarly situated NOIE metering points.

(4)	BLTs of Load to the ERCOT Control Area will be permitted only if such transfers will not jeopardize the reliability of the ERCOT System.  Under an ERCOT Emergency Notice, BLTs that have been implemented may be curtailed or terminated by ERCOT in order to maintain the reliability of the ERCOT System.

(5)	BLTs of Load from the ERCOT Control Area will be treated as a Resource in the ERCOT settlement system.

(6)	BLTs of Load from the ERCOT Control Area will be sent Dispatch Instructions only with the permission of the affected Non-ERCOT Control Area.  Under emergency conditions, BLTs that have been implemented may be curtailed or terminated by the Non-ERCOT Control Area in order to maintain the reliability of the Non-ERCOT system.

(7)	Restoration of service to outage customers using BLTs will be accomplished, as quickly as possible, if the transfers will not jeopardize the reliability of the ERCOT System and only under specific Dispatch Instruction from ERCOT.

(8)	The necessary Market Participant agreements, metering, and ERCOT settlement systems are in place prior to the implementation of any BLT.

(9)	Transmission BLT metering points utilized five (5) or more time per year, as monitored by the TDSP, will conform to ERCOT Polled Settlement (EPS) Metering requirements as defined in Section 10 of these Protocols and the Settlement Metering Operating Guides.  BLT metering points that are Distribution Systems or are transmission BLT metering points used less than five (5) times per year will be Revenue Quality Meters, Four (4) channel bi-directional kWh/kVARh, fifteen (15) minute Interval Data Recording (IDR) metering with remote interrogation.  ERCOT may impose additional metering requirements as deemed necessary to ensure ERCOT System reliability and integrity.

5.7.3	Scheduling & Operation of BLTs

BLTs will be treated as a Resource of the  QSE  associated with a BLT metering point   However, the  QSE will not be required to provide the real time data to ERCOT normally provided for Resources.  ERCOT shall confirm its availability with the Non-ERCOT Control Area or the TDSP prior to issuing any Dispatch Instructions to the QSE and the TDSP.  Any energy delivered under such a Dispatch Instruction shall be treated as an OOME Dispatch Instruction to the QSE.  For settlement purposes, the Dispatch Instruction will be treated as an instructed deviation, and the Resource Plan of the BLT Resource shall be set to zero.

Generation Resources connected to the ERCOT Region included in the BLT are under no obligation to provide Ancillary Services or any other services while that transmission area is connected to a non-ERCOT Control Area other than through a BtBTie.

The QSE of the LSE assigned to the ESI ID associated with a BLT point will include that Load in its Balanced Schedules.

5.8	BtB Ties and BLTs  with the CFE Control Area [new]

Under Alert, EECP or other Emergency  Condition which may negatively affect the reliability of the ERCOT System, it may be necessary for ERCOT to request energy to be delivered from BtB Ties and BLTs  with the CFE Control Area in accordance with an agreement between CFE and ERCOT.

Similarly, when the CFE Control Area experiences certain reliability  contingency or short supply conditions, it may be necessary for ERCOT to allow energy to be delivered to the CFE Control Area.

Deliveries to and from the CFE Control Area over the CFE/ERCOT BtB Ties or in the form of Block Load Transfers  shall be restricted to the following conditions:

- Deliveries of energy from the ERCOT Control Area to the CFE Control Area will be permitted only if such transfers do not jeopardize the reliability of the ERCOT System.  Under an Advisory, Alert, EECP or other Emergency  Condition, ERCOT may curtail or terminate deliveries to CFE that have been scheduled or implemented in order to maintain the reliability of the ERCOT System.

- Under Emergency Conditions, CFE may curtail deliveries to the ERCOT Control Area in order to maintain the reliability of the CFE system.

- 5.9	Frequency Response Requirements and Monitoring

[Renumber existing Section 5.8 and subsections to 5.9]

6.8.2	Capacity and Energy Payments for Out-of-Merit or Zonal OOME Service

6.8.2.1	Resource Category Generic Costs

To properly calculate Local Congestion costs, it is necessary to establish certain generic costs associated with Resources that will be used to calculate production costs incurred when the Resource(s) provides Out of Merit or Zonal OOME Service.  These generic Resource costs include generic fuel costs and generic startup costs.

(1)	Each ERCOT Generation Resource will be assigned to one of the following Resource Categories for the purpose of determining generic fuel costs:

Nuclear

Hydro

Coal and Lignite

Combined Cycle greater than 90 MW**

Combined Cycle less than or equal to 90 MW**

Gas-Steam Supercritical Boiler

Gas-Steam Reheat Boiler

Gas-Steam Non-reheat or boiler without air-preheater

Simple Cycle greater than 90 MW

Simple Cycle less than or equal to 90 MW

Diesel (and all other diesel or gas-fired Resources)

Renewable (i.e., non-Hydro renewable Resources)

Block Load Transfer

BtB Tie with non-ERCOT Control Area

**	Determined by capacity of largest simple cycle combustion turbine in the train

The category of each Resource will be reported to ERCOT by the Generation Entity.  Each Generation Entity shall ensure that each of its Resources is in the correct Resource category.

(2)	The Fuel Index Price (FIP) shall be the Midpoint price, expressed in $/MMBtu, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day of the OOMC or OOME deployment.  The FIP for Saturdays, Sundays, holidays and other days for which there is no FIP published in Gas Daily, shall be the next published FIP after the day of the OOMC or OOME deployment.  In the event that the FIP is not published for more than two (2) days, the previous day published FIP will be used for Initial Settlement and the next day published FIP will be used for the Final Settlement Statement.

(3)	Resource Category Generic Fuel Costs

Each ERCOT Generation Resource will be assigned a Resource Category Generic Fuel Cost (RCGFC) based on the Resource Category to which it is assigned.  For Nuclear, Hydro, Coal and Lignite Resources, the RCGFC will be a fixed dollar/MWh amount as shown below.  For the remaining Resource categories (except Renewable), the RCGFC will be the product of a heat rate (based on the heat rates used for the Capacity Auction) and a Fuel Index Price (FIP).  The RCGFC for Renewable Resources will be $0/MWh.

The RCGFC for each type of Resource for upward instructions will be:

Nuclear = $15.00/MWh

Hydro = $10.00/MWh

Coal and Lignite = $18.00/MWh

Combined Cycle greater than 90 MW** = FIP * 9 MMBtu/MWh

Combined Cycle less than or equal to 90 MW** = FIP * 10 MMBtu/MWh

Gas-Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh

Gas-Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh

Gas-Steam Non-reheat or boiler without air-preheater = FIP * 14.5 MMBtu/MWh

Simple Cycle greater than 90 MW = FIP * 14 MMBtu/MWh

Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh

Diesel = FIP * 16 MMBtu/MWh

Block Load Transfer = FIP * 18 MMBtu/MWh

Back to Back  Tie with non-ERCOT Control Area = FIP * 18 MMBtu/MWh

Renewable = $0/MWh

LaaR = FIP * 18 MMBtu/MWh

**	Determined by capacity of largest simple cycle combustion turbine in the train

The RCGFC for each type of Resource for downward instructions will be:

Nuclear = $0.00/MWh

Hydro = $0.00/MWh

Coal and Lignite = $3.00/MWh

Combined Cycle greater than 90 MW** = FIP * 5 MMBtu/MWh

Combined Cycle less than or equal to 90 MW** = FIP * 6.5 MMBtu/MWh

Gas-Steam Supercritical Boiler = FIP * 7.5 MMBtu/MWh

Gas-Steam Reheat Boiler = FIP * 9.5 MMBtu/MWh

Gas-Steam Non-reheat or boiler without air-preheater = FIP * 10.5 MMBtu/MWh

Simple Cycle greater than 90 MW = FIP * 10.5 MMBtu/MWh

Simple Cycle less than or equal to 90 MW = FIP * 12 MMBtu/MWh

Diesel = FIP * 12 MMBtu/MWh

Block Load Transfer = Not Applicable

BtB Tie with non-ERCOT Control Area = Not Applicable

Renewable = $0/MWh

**	Determined by capacity of largest simple cycle combustion turbine in the train.