The purpose of this document is to describe the editor and data management requirements for the Network Model Management System.

PR50142 NMMS Network Model Management System Requirements

The intended target audiences of this document include the Transition Plan Task Force, the Network Model Group at ERCOT, IT, Business Process Owners, Business Subject Matter Experts that distribute information to the Market and the development groups that deploy information to the Market.

Document Revisions

| Version | Date | Contributors | Comments |
| --- | --- | --- | --- |
| 1.0 | 1/25/06 | L. Caylor, C. Crews, K. Dye, J. Koepke, J. Weatherly, D. Evans, J. Adams, J. Dondeti, & C. Rutten | Original compiled by C. Crews. Reformatted and recompiled using J. Dondeti suggestion. Additions by contributors. |
| 1.1 | 2/8/06 | L. Caylor, C. Crews, K. Dye, J. Koepke, J. Weatherly, D. Evans, J. Adams, J. Dondeti, & C. Rutten | Added IT comments. Added Planning comments. Changed TO to TSP Additions from contributors. |
| 1.2 | 2/10/06 |  | Added Protocol Source referencing |
| 1.2a | 2/20/06 |  | Only change involved moving the diagrams to a new appendix document so that the file size of the requirements document is smaller |

®

TABLE OF CONTENTS

1	VISION	8

2	BACKGROUND	9

3	REQUIREMENTS FOR AN ENHANCED MODEL UPDATE PROCESS	11

3.1.1	Protocol Labeling (Requirements that can be specifically association to a particular Protocol)	12

3.1.2	Non-Protocol Labeling (Requirements that are needed but cannot be specifically associated to a particular Protocol)	12

3.2	General Requirements	12

3.2.1	Requirements	12

3.3	Security	14

3.3.1	General Statement of the Security	14

3.3.2	Requirements for the Security	14

3.4	Data Archival	15

3.4.1	General Statement of the Data Archival	15

3.4.2	Requirements for the Data Archival	15

3.5	Data Submittal Process	16

3.5.1	General Statement of the Data Submittal Process	16

3.5.2	Requirements for the Data Submittal Process	16

3.6	Project Build and Test Process	20

3.6.1	General Statement of the Project Build and Test Process	20

3.6.2	Requirements for the Project Build and Test Process	20

3.7	Model Database	25

3.7.1	General Statement of the Model Database	25

3.7.2	Requirements for the Model Database	25

3.8	Update Network Operations Model Process	29

3.8.1	General Statement of the Update Network Operations Model Process	29

3.8.2	Requirements for the Update Network Operations Model Process	29

3.9	Build Day-ahead Model Process	31

3.9.1	General Statement of the Build Day-ahead Model Process	31

3.9.2	Requirements for the Build Day-ahead Model Process	31

3.10	Internal Data Editor and Comparison Tool	33

3.10.1	General Statement of the Internal Data Editor and Comparison Tool	33

3.10.2	Requirements for the Internal Data Editor and Comparison Tool	33

3.11	Application Software Data Process	34

3.11.1	General Statement of the Application Software Data Process	35

3.11.2	Requirements for the Application Software Data Process	35

3.12	Timeline Database Process	36

3.12.1	General Statement of the Timeline Database Process	36

3.12.2	Requirements for the Timeline Database Process	36

3.13	Study Case Builder Process	37

3.13.1	General Statement of the Study Case Builder Process	37

3.13.2	Requirements for the Study Case Builder Process	37

3.14	CRR Auction Case Builder Process	38

3.14.1	General Statement of the CRR Auction Case Builder Process	38

3.14.2	Requirements for the CRR Auction Case Builder Process	38

3.15	Planning Case Builder Process	39

3.15.1	General Statement of the Planning Case Builder Process	39

3.15.2	Requirements	39

3.16	Posting and Notification Capabilities	40

3.16.1	General Statement of the Posting and Notification Capabilities	40

3.16.2	Requirements for the Posting and Notification Capabilities	40

4	SIGN OFFS	42

TABLE OF APPENDICES

Appendix 1: Conceptual Overview of Model Database and General Models Generation Process	11

Appendix 2: Data Submittal Process	16

Appendix 3: Project Build and Test Process	20

Appendix 4: Transition from Planning Project to Operations Project	24

Appendix 5: Model Database	25

Appendix 6: Update Network Operations Model Process	29

Appendix 7: Build Day-ahead Model Process	31

Appendix 8: Internal Data Editor and Comparison Tool	33

Appendix 9: Application Software Data Process	34

Appendix 10: Timeline Database Process	36

Appendix 11: Posted Timeline Database	36

Appendix 12: Study Case Builder Process	37

Appendix 13: CRR Auction Case Builder Process	38

Appendix 14: Planning Case Builder Process	39

Appendix 15: Posting and Notification Capabilities	40

TABLE OF TABLES

Table 1: Acronyms and Terms	5

Table 2: Dates for Data Submittals, Model Testing, and Model Posting	41

Table 1: Acronyms and Terms

| Acronym | Meaning |
| --- | --- |
| AOGM | Automated One-line Generator Manager software suite |
| CIM | Common Information Model |
| CIP | Critical Infrastructure Protection |
| CRR | Congestion Revenue Rights |
| DID | The unique identifier for a Device in the Model Database. The Electric Power Network consists of devices connected to form a network. |
| EMS | Energy Management System |
| GUI | Graphical User Interface |
| ICCP | Inter-Control Center Communications Protocol |
| ISO | Independent System Operator |
| LMP | Locational-Based Marginal Pricing |
| MIS | Market Information System |
| MP | Market Participant |
| NERC | North American Electric Reliability Council |
| NMG | Network Modeling Group |
| NMMS | Network Model Management System |
| NOMCR | Network Operations Model Change Request |
| OTS | Operator Training Simulator |
| PID | The unique identifier for a Project. |
| PTI | Power Technologies Institute software (PSSE). Now owned by Siemens |
| RTNET | Real Time Study Software (Currently AREVA) |
| RTO | Reliability Transmission Operator |
| RTU | Remote Terminal Units |
| SCADA | Supervisory Control and Data Acquisition |
| SME | Subject Matter Expert |
| SR | Service Request |
| STNET | Load Flow Software Package (Currently AREVA) |
| TML | Texas Market Link |
| TSP | Transmission Service Providers |
| UI | User Interface |

| Term | Definition |
| --- | --- |
| Annual Planning Model | The data set within the Model Database that is used by Market Participants who participate in producing rolling five year planned additions to the Electrical Power Network. Defined in Nodal Protocol Sections 3.10.2 and 3.11. This is a bus-to-bus oriented model. |
| CRR Network Model | The data set within the Model Database that is used by Market Participants who participate in the CRR Auction. Defined in Nodal Protocol Section 3.10.3 and Section 7 |
| CIM/XML | CIM/XML format as described by ERCOT. Schema will be posted to Market Participants. |
| Common Information Model | EPRIs definition: A conceptual information model for describing management that is not bound to a particular implementation. There are two parts to CIM: The CIM Specification and the CIM Schema. The CIM Specification describes the language, naming, Meta Schema, and mapping techniques to other management models, such as: SNMP MIBs and DMTF MIFs, etc. The Meta Schema is a formal definition of the model. It defines the terms used to express the model and their usage and semantics. The elements of the Meta Schema are Classes, Properties, and Methods. The Meta Schema also supports Indications and Associations as types of Classes and References as types of Properties. The CIM Schema provides the actual model descriptions. The CIM Schema supplies a set of classes with properties and associations that provide a well-understood conceptual framework where it is possible to organize the available information about the managed environment. The CIM Schema itself is structured into three distinct layers: - The Core Schema is an information model that captures notions that are applicable to all areas of management - Common Schemas are information models that capture notions that are common to particular management areas, but independent of a particular technology or implementation. The common areas are systems, devices, networks, applications, metrics, databases, the physical environment, event definition and handling, management of a CIM infrastructure (the Interoperability Model), users and security, policy and trouble ticketing/ knowledge exchange (the Support Model). These models define classes addressing each of the management areas in a vendor-neutral manner. - Extension Schemas represent organizational or vendor-specific extensions of the Common Schema. These schemas can be specific to environments, such as operating systems (for example, UNIX® or Microsoft Windows®). Extension Schema fall into two categories, Technology-Specific areas such UNIX98 or Product-Specific areas that are unique to a particular product, such as: Windows. |
| Device | The item representing field equipment that a Market Participant is responsible for that can be added to, changed within, or retired from the Electrical Power Network and subsequently the Model Database. |
| Electrical Power Network | The electrical power generators, the stations that house various devices, the transmission lines, the towers and poles that carry the transmission lines, the switching units, the transformers, and other directly related devices that constitute the interconnected power system that is under the operational jurisdiction of ERCOT. |
| Generation Subsystem | This is the data set representing the generation assets within the Electrical Power Network. |
| Inter-Control Center Communications Protocol | The Inter-Control Center Communications Protocol (ICCP) is the primary protocol used to communicate information between energy control centers that operate SCADA/EMS/DCS systems, and between control center and power generators. In recent years, ICCP has also been used for communications between control centers and remote terminal units (RTU) or substations. |
| In-Service date | Time a device is placed into service in the field |
| Market | The Market is the aggregate of the Market Segments. The Market Segments as defined in Article 2 of the ERCOT Bylaws segments are: - Independent Retail Electric Providers (REP)s, - Independent Generators, - Independent Power Marketers, - Investor Owned Utilities, - Municipals, - Cooperatives, and Consumers. |
| Market Data | - Data sent to the Market. |
| Market Participants | An Entity that engages in any activity that is in whole or in part the subject of these Protocols, regardless of whether such Entity has executed an Agreement with ERCOT. |
| Model | A data set from the Model Database representing the topology of the Electrical Power Network. |
| Model Database | The set of databases that contain the Annual Planning, CRR Network, and Network Operations Model(s) and all of the pending Projects and other data needed to create the needed models for the Electrical Power Network. OTS and Dynamic Simulation Models are contained in the Model Database. |
| Network Operations Model | The released data set from the Model Database as the most accurate representation at the moment of release of the Electrical Power Network. This is a breaker oriented model. |
| Planning Model | The data set within the Model Database that is used by the Planning Group of ERCOT and Market Participants. |
| Project | A packet of information defining a Market Participant’s projected addition, change, or deletion of Devices within the Electrical Power Network that is to go into service at some specific date in the future. There may be Operation and Planning Projects. Subsets of Operations and Planning Projects may exist. |
| SCADA Communications | The data set representing the types of field monitoring equipment, facilities, and systems. |
| SCADA systems | Supervisory Control and Data Acquisition (SCADA) networks utilize the Inter-control Center Communications Protocol (ICCP) protocol to run key U.S. utilities. |
| Test Model | The model being used in the Model Test System to test submitted Projects. |
| Station | A physical footprint represented within the Electrical Power Network that contains Devices. |

# VISION

The purpose of this document is to describe the editor and data management requirements for the Network Model Management System (NMMS). This document will provide an overview of the current capabilities of the existing model editor. Furthermore the document will provide the business requirements of the NMMS to satisfy the future ERCOT Nodal Market (Nodal Market). The NMMS shall provide a platform to promote and ensure consistency between the Annual Planning, CRR (Congestion Revenue Rights), and Network Operations Model data sets. In the current Zonal Market there are no defined rules to correlate and maintain consistency between the different model data sets. The Nodal Market rules defined multiple inter-relationships that must be maintained per the ERCOT Nodal Market Nodal Protocols (Nodal Protocols). The NMMS shall provide a single point of entry for model data within ERCOT. The functions that the various models support should only change due to the nature of the Nodal Market requirements. In other words, the NMMS Planning Model shall be used as the building block for the Annual Planning Model(s) requirements as specified by the Nodal Protocols. One distinction needs emphasized and that is the NMMS Planning Model shall use the current and near future configuration parameters as the base building block of the Annual Planning Model(s). The intended target audiences of this document include the Transition Plan Task Force, the Network Model Group (NMG) at ERCOT, IT, Business Process Owners, Business Subject Matter Experts that distribute information to the Market and the development groups that deploy information to the Market. The document will be used as a template of requirements to be met by various groups including IT and eventually a vendor.

ERCOT®

Prepared by:

Craig J. Rutten

The NMMS project is intended to provide data management system tailored to the needs and responsibilities of an Independent System Operator (ISO) and ERCOT. Independent System Operators have transmission analysis needs unique to the industry that are not served by software tools traditionally designed to serve vertically integrated utilities.

A short list of the problems encountered by ISOs and ERCOT, in particular, include:

- An ISO does not own simulated or modeled equipment and is dependent upon 3rd parties to provide data.

- ISO transmission modeling has a direct economic impact upon Market Participants, and the ISO has a fiduciary responsibility to those participants.

- Due to the size of ISO modeled systems, the number of network changes is larger than most vertically integrated utilities; and due to the problem listed above, timeliness is more critical.

- ISO members have imposed market rules upon the ISO that are must be observed to preserve market integrity.

- Audit trails for accountability are needed for network changes that were not required for traditional utilities.

- A financial impact of future year network models has been created, e.g. a CRR, making the creation of auditable network models a requirement.

- A need that did not exist for vertically integrated utilities for Market Participants (MP) to gain access to power flow models that have been created.

- Transparency of process and results is required that did not exist for vertically integrated utilities.

To solve these problems, a new tool is required. This tool will require changes in existing network modeling applications to take full advantage of the data sets available from NMMS. ERCOT currently uses AREVA™ software for its real-time operations, and PTI™ software for planning studies.

# BACKGROUND

The Operations Model Database is a combination of databases at ERCOT describing the electrical power network within the jurisdiction of ERCOT [Electrical Power Network]. This data is generally referred to as the “Network Operations Model” or “Operations Model”. The current update process typically occurs every other week.

The ERCOT Operations Model provides data and topology for several software applications within ERCOT

These include, but are not limited to:

- Frequency control.

- Congestion management.

- Outage Scheduler.

- State Estimator.

- STNET (general term used for study cases within ERCOT).

- Contingency analysis.

- Market analysis.

- Other power system study software including, but not limited to:

  - PowerWorld

  - PTI/PSSE

  - DSA

The Operations Model is converted to a PTI format, and then posted for Market Participants to use.

The current model editing software permits changes to the Operations Model. This model then is tested in an off-line environment. The Operations Model is then loaded into the Production System.

This is a labor-intensive process involving the maintenance of many separate databases that makes implementing model updates very time-consuming and, thus, difficult to perform, on a more frequent basis than every other week (the current schedule).

The current software and processes used for model updates have the following major drawbacks:

- No reasonable and practical method for providing variants of the Operations Model to study known or anticipated future Electrical Power Network configurations.

- A capability is needed to create variations of Operations Model that should be limited only by the ability of the data submitters. These variations would provide the future configuration changes to the Electrical Power Network that can be used to study the Electrical Power Network and analyze future conditions based upon changes being introduced into the Operations Model and/or the Annual Planning Model.

- Other applications requiring look-ahead studies, including, but not limited to: the Outage Scheduler, would benefit from variations on the Operations Model containing future Projects and/or updates.

- Does not meet the need for keeping data consistent between the Annual Planning and Operations Models.

- The model change package used now, i.e. Service Request (SR), submitted by a Market Participant (MP) is communicated over a private connection by the data submitters. The current data link software does not provide the needed flexibility to properly track and report the status of a submitted model change package. Although manual audit trails have been created, they are time consuming and difficult to maintain.

- User defined validation/semantic rules are not supported by the present editor.

- The ability to insert quantities of related data, subject to data validation criteria and auditable log file tracking, needs to exist (e.g. if NMG is provided with a change in ownership of devices in the Electrical Power Network, all devices owned need to be selected and changed to reflect the new ownership in a single update versus changing attributes to the devices individually).

- With the model editor being used currently, the following database elements cannot be changed within the Operations Model without negatively impacting all of the associated database links, pointers, and indices:

- Company names.

- Substation Id.

- Weather zones.

- Device ownerships.

- Device types.

- Plant controller.

- Other database elements.

- This lack of functionality severely impacts the following activities, applications, and data bases:

- Outage Scheduler.

- PI Historian.

- PowerWorld Map.

- Contingency definitions.

- One-line diagram displays.

- Various external databases including, but not limited to:

  - ICCP

  - Dynamic Ratings.

- The current model editor does not provide an integrated link with SCADA (ICCP and RTU) data. Managing these links and the constructing of the required data points in the various models are necessary, but difficult due to the large quantity of data flowing into the NMG.

- The Operations Model and the Outage Scheduler application are presently linked by numerous manual processes. Changes in the Operations Model may cause previously defined outages to no longer be correctly defined in study cases.

- The current model editor has extremely limited data export capability.

- There is no permanent unique identifier for each Device within the Model Database.

- There is no ability to queue or manage effectively SRs that did not meet their current in-service date.

- The Dynamic Elements model that contains the dynamic rating element data is not an integrated part of the Model Database.

- Special handling and error checking is required when the Operations Model is prepared and moved to the Production System.

- The Dynamic Simulation Models, including, but not limited: to the Generator, FACTS, and other dynamic simulation model devices and associated attributes used for power system dynamic simulations, are not an integrated part of the Model Database.

# REQUIREMENTS FOR AN ENHANCED MODEL UPDATE PROCESS

Appendix 1: Conceptual Overview of Model Database and General Models Generation Process

## [See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

## Legend for Requirements Source

### Protocol Labeling (Requirements that can be specifically association to a particular Protocol)

#### Protocol (section #) - Location in the Protocols that is the source of a requirement.  Ex: Protocol 3.10.1 label means that the source for the requirement can be found in Section 3.10.1 of the Protocols.

### Non-Protocol Labeling (Requirements that are needed but cannot be specifically associated to a particular Protocol)

BP - Stands for Business Process.  This means that an internal ERCOT Business Process is the source of a requirement.  Ex: System shall email the user when the file is posted to the web.  This may be needed due to the Business user’s procedures that require him/her to validate the accuracy of a posting to the web but this process is not required by the Protocols.

SR - Stands for System or Security Requirement.  This means that a System or Security Requirement is the source of a requirement.  Ex: System shall allow remote VPN access.  Browser button shall be configured so that the user can navigate back and forth between screens.  Both of these examples are necessary for a developer to develop a system but neither can be directly associated to a particular Protocol.

## General Requirements

### Requirements

- Training and Documentation

  - Training shall be for provided for all areas of the NMMS system.

This training shall be divided into the following sections:

    - Data submission interface. Protocol 3.10.1

    - Project preparation and test. Protocol 3.10.1

    - The Model Database including the editor and comparison tool. Protocol 3.10.4 (3)

    - The Case builder. BP

    - The Timeline Database. Protocol 3.10 (7), (8) Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (5)

    - Posting and Notification. Protocol 3.10 (11)

    - Network Operations and Day-ahead Models Update process including the Model Builder. BP

    - Interface to application software. BP

  - Documentation shall be provided to cover each sub-system within the NMMS package. BP

  - This documentation shall be sufficient to guide trained personnel through the process. BP

- Model Editor Interface

  - The Model Editor shall permit editing all levels of database parameters. SR

  - It shall be user friendly and offer MS Windows™ type editing capabilities. SR

  - There shall be a GUI interface to edit database parameters plus a “flat file” type editor to facilitate editing large amounts of data quickly. SR

- System Operator and Data Submittal Interface

  - The System Operator interface shall provide an easily accessible menu to select the listed projects for injection into the Network Operations Model. BP, SR

  - All actions shall be time-stamped and recorded in a log to indicate the time and date for all actions taken. BP, SR

  - Actions taken from the System Operators display for each Project shall be available for posting to the MIS. The format of this posting shall be edited through a template accessible by a NMG GUI. Protocol 3.10 (8)

- The NMMS shall support security controlled UI interactive editing for format of all reports, postings, data templates, validation rules, import/export formats, and other interfaces to the system. This GUI interface shall not require server access for the user. BP

- The NMMS shall operate on a fully redundant platform with a service level agreement equal to the ERCOT EMMS. BP, SR

- There shall be no requirement to limit or reduce access of users during validations, querying, file transfers, or other routine activities. BP, SR

- Source code shall be made available to ERCOT for all modules of the NMMS. BP, SR

- Response time of all validations, report output, display build and other functions shall meet normal response times expected for an EMMS. BP, SR

## Security

### General Statement of the Security

Security shall be provided for the NMMS system to prevent unauthorized entry and use.

### Requirements for the Security

- Provide sufficient access privilege levels sufficient to protect against unauthorized data entry and modification. BP, SR

- Provide read-only capabilities for the model editor so that database data reviews are allowed without granting modification privileges. BP

- Meet all ERCOT policies for security. BP, SR

- ERCOT shall follow applicable policies regarding release of Critical Energy Infrastructure Information (CEII). Protocol 3.10 (8), Protocol 3.10.2 (3), Protocol 3.10.3 (5)

## Data Archival

### General Statement of the Data Archival

Much of the data managed by the NMMS System will serve as documentation to modifications made to the electric system topology. The changes made to the database must be maintained as a traceable history.

### Requirements for the Data Archival

- The audit trail with, at least, the current and last two years of all model and display update activities and information shall be maintained. BP, SR

- The Projects merged into the Production Database must be maintained for at least 5 years. The 1st year should be retrievable within 5 minutes.  Subsequent years shall take less than 2 minutes per year to retrieve. BP

- Each day a system topology snapshot shall be saved and maintained for a period of 10 years. BP

## Data Submittal Process

Appendix 2: Data Submittal Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Data Submittal Process

The data submittal process is an important link in providing traceable records of the information passing between market participants and the Planning and Network Model Group at ERCOT. This process will accumulate data to develop metrics for improvement of the data submittal process. The process for updating the Network Operations Model referenced in the Nodal Protocols is the Network Operations Model Change Request (NOMCR) process.  The NMMS shall act as the single point of entry for ERCOT model data.  This indicates that other ERCOT specified processes should exist in addition to the NOMCR process.  The NMMS design accepts updates outside of the NOMCR process as described in Nodal Protocol 3.10.1 to accommodate Annual Planning and CRR Network Model update timeframes.  Model updates are generically referenced as Projects involving data submittal.  Devices include, but are not limited to, Transmission Elements and generation related equipment.

### Requirements for the Data Submittal Process

To support the Project Verification process, the following capabilities shall be available:

- Acknowledge, track, and verify, as needed; Projects and associated data as the Projects are processed through the Model Test System. Protocol 3.10 (8)

- Serve as a feedback loop for the Market Participants that provided the data. Protocol 3.10 (8)

- Serve as the conduit for information requests. Protocol 3.10 (8)

The following requirements shall be fulfilled:

- Automatic receipt acknowledgement provided to the submitting Market Participants for each Project received. Protocol 3.10 (8)

- The notification and/or audit application(s) to transfer Project status back to the submitting entity, owner(s), and operator(s).As a minimum, the status notifications shall include:

  - Received at NMG or Planning. Protocol 3.10 (8)

  - Complete as submitted to NMG or Planning. Protocol 3.10 (8)

  - Incomplete as submitted to NMG or Planning. Protocol 3.10 (8)

  - Requires additional data with problem report included. Protocol 3.10 (8), (10)

  - Ready for test. Protocol 3.10 (8)

  - Planned in-service date into the Network Operations Model. Protocol 3.10 (8)

  - Approved for energizing on scheduled date. Protocol 3.10 (8), (10)

  - In-service. Protocol 3.10 (8), (10)

  - Closed. Protocol 3.10 (8)

  - Project requires Operations update. Protocol 3.10 (8)

  - Other status as required by NMG or Planning. Protocol 3.10 (8)

- Status notifications, classification, and sub-classifications shall be rule based. Protocol 3.10

- The rules referred to in § 3.4.2. - 6 must be easily editable by the NMG. BP, SR

- Automated Replies:

  - After validation, automatically send an acceptance notification or a problem reports to the Market Participants for their processed Projects. Protocol 3.10 (8), (10), (11)

  - Automatically create a problem report for corrective action and notify the Market Participant when correction(s) are due based on the current update cycle. Protocol 3.10 (11)

  - These schedule parameters to be easily edited by the NMG. BP, SR

- Reports:

  - Project status reports identifying Projects by station, by Device type, and a comparison of all model changes between two points in time to be supported. Protocol 3.10 (11), Protocol 3.10.4 (3)

  - As a minimum, all reports to be equipped with export capability to transfer user-selected audit trail data in CSV and/or Excel format. BP, SR

- Report Formatting:

  - All reports to have a configurable report printout capability. BP, SR

  - The format of the printout to depend on the contents of the report. BP, SR

  - Formats of reports to be definable by the NMG. BP, SR

- Time-dated ERCOT defined Project summaries that identify the related database changes for the following internal and external interfaces within ERCOT Operations to be prepared and sent:

  - ICCP/SCADA. BP, SR

  - PI Historian. BP, SR

  - Outage Scheduler. BP, SR

  - Network Contingency Definitions. BP, SR

  - PowerWorld Map. BP, SR

  - CRR Network Models. BP, SR

  - Annual Planning Models. BP, SR

  - Dynamic Simulation Model. BP, SR

- Automatically alert MP and other entities of pending Planning Projects. Protocol 3.10.4 (5)

- Data Entry

  - Errors occurring during the Project data entry process to be captured. Protocol 3.10 (6), Protocol 3.10.4 (4)

  - Reporting on errors to be made available. Protocol 3.10 (6), Protocol 3.10.4 (4)

    - Note: A rudimentary data entry validation can consist of Id and Device verifications to provide an immediate indication as to the completeness of the submitted information.

- Incomplete and/or invalid data entries to be noted, and then an error message to be generated describing the data entry problems. Protocol 3.10 (6), Protocol 3.10.4 (4)

- Completeness criteria to be defined, maintained, and edited as needed by the NMG. BP, SR

- All data entries associated with an incomplete Project to be saved for future modification and submittal utilizing the original unique Project Identification (PID) number. BP, SR

- A Project to be submitted via a Data Submittal interface that will support the following requirements:

  - Time Related (Format for all times referenced shall be mm/dd/yyyy hh:mm:ss using military time-based methodology). BP, SR

  - All changes within a Project and the Project itself to be automatically time-stamped per the submittal time. BP, SR

  - Include the in-service date and time for the new Device(s). Protocol 3.10.1

  - Include the out-of-service date and time for the retired Device(s). Protocol 3.10.1

  - Market Participants can submit Projects for planned construction (e.g. transmission, generation, resource, and distribution, as required) for periods as specified by the Nodal Protocols prior to the in-service date. Protocol 3.10.1 (1)

  - Archive all status notification information and time stamps in such a way to provide a detailed audit trail report of all updates. BP, SR

  - Data forms to be contained within the Data Submittal Interface.  These forms will be editable by the Network Model Group and ERCOT Planning Group. BP, SR

- A Project to be submitted using the pre-defined formats for each Device type.

  - These formats will be available from the Data Submittal Interface. Protocol 3.10.7

  - The NMG and ERCOT Planning Group shall have access to modify these formats and add additional format documents. BP, SR

- As a minimum, each Project to include the following information:

  - Generated unique PID. BP, SR

  - Submitter. Protocol 3.10. (11)

  - Submitter classification (Planning, Operations, and others as approved by ERCOT) Protocol 3.10. (11)

  - Sub-classification:

    - ERCOT Board Approved Project. Protocol 3.10. (11)

    - Regional Planning Group Approved Project. Protocol 3.10. (11)

    - Unapproved Planning Project. Protocol 3.10. (11)

    - Emergency Project. Protocol 3.10. (11)

    - SPS Project. Protocol 3.10. (11)

    - RMR Project. Protocol 3.10. (11)

    - Pending PUCT CCN Approval Project. Protocol 3.10. (11)

    - Pending Project. Protocol 3.10. (11)

    - TSP Budgeted Project. Protocol 3.10. (11)

    - Long Range Study Project. Protocol 3.10. (11)

    - Other Protocol 3.10. (11)

  - Planned Device In-Service/Out-of-Service date. Protocol 3.10.1 (1)

  - Appropriate data forms to accommodate introduction of Project. BP, SR

  - PTI bus number for each changed or new Device. Protocol 3.10 (9)

  - All notes associated with the Project. Protocol 3.10 (11)

  - Project revision number. Protocol 3.10 (9)

  - Project Status. BP, SR

  - Prerequisite Projects. Protocol 3.10 (6)

  - Outages associated with Project. Protocol 3.3.2.1

  - One-line diagrams associated with the Project before and after changes. Protocol 3.3.2.1

  - CIM/XML format, as used by ERCOT. Protocol 3.10 (7), (8), Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (5)

  - GPS or GIS coordinates, where appropriate (e.g. new stations). BP, SR

  - Actual In-Service/Out-of-Service date(s) per Device Protocol 3.10 (8)

- The capability for multiple, simultaneous users to enter data either using the web-based GUI and/or a flat file. BP, SR

- The capability to flag and notify the NMG and ERCOT Planning pre-defined Market Participant(s) responsible for Project corrections. Protocol 3.10 (11)

- Automatically notify NMG for resubmitted Projects. Protocol 3.10 (11)

- Ensure that Device Identification (DID) uniqueness, specifically, but not limited to Device names, is maintained within the Model Database with business rules for uniqueness as defined by NMG is incorporated. Protocol 3.10.7.1 (2)

- NMG and Planning Group defined validation criteria to be used to verify consistency within the Model(s) and earlier submitted dependent Projects. Protocol 3.10 (6)

- NMG and Planning Group defined validation criteria to be used to verify consistency within the following models:

  - Network Operations. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Day-ahead. BP

  - CRR Network. Protocol 3.5.1, Protocol 3.10 (5), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Annual Planning. Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (3)(b)

  - Operator Training Simulator (OTS) BP, SR

  - Dynamic Simulation BP

- Criteria shall include:

  - Topology. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Equipment. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Names. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Ratings. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Impedances. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

  - Other relational attributes. Protocol 3.5.1, Protocol 3.10 (5), (9), 3.10.2 (1), 3.10.3 (1), 3.10.3 (2), 3.10.8 (1), 3.10.8.4 (2), Protocol 7.5.1 (2), Protocol 7.5.1 (3)(b), Protocol 7.5.5.3 (3), Protocol 7.5.5.4 (1)

- The ability to pre-define Project and Device validation criteria and checks shall exist.  The validation checks shall be easily added, deleted, and edited by the NMG when adding new requirements or deleting existing ones. BP, SR

- Various ERCOT groups and/or departments shall be able to import Projects that can be specialized for their needs, e.g. the Planning Group can import Planning Projects to generate an Annual Planning Model. Protocol 3.10. (5)

- An acceptance response and other defined responses to be sent to the submitting MP with the generated PID and the associated DID – the PID is unique to this Project, the DID is based on the Device(s) in place and is unique to the Device(s) not just the Project. Protocol 3.10. (8)

- Version tracking shall be available for each Project to permit tracking of updates. Protocol 3.10. (8)

## Project Build and Test Process

Appendix 3: Project Build and Test Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Project Build and Test Process

The environment to build and test Projects is critical to the success of this function. The Projects are the building blocks of the model update process. Network Model personnel will utilize this interface to build and manage these building blocks.  The Model Test System and Model Testing Environment are described in the Nodal Protocols (Section 3.10.4 (3)) as the State Estimator test facility, State Estimation test facility, LMP Test Facility, and test system.

### Requirements for the Project Build and Test Process

- The NMMS shall provide capability to manage time–based (i.e. time relational data changes) data models.

The model editing software shall:

  - Allow the building of a time-dated model change Project based on the projected in-service date of the Device(s) in that Project. Protocol 3.10 (8), Protocol 3.10.1 (1),(2),(3)

  - Provide the tools to audit the scheduled and actual implementation dates for each Project per Nodal Protocol requirements. Protocol 3.10 (8), Protocol 3.10.1 (1),(2),(3)

- The Projects shall be:

  - Processed by the either the NMG or the Planning Group as determined by the submitter classification. Protocol 3.10 (8)

    - Sub-classifications shall exist with defined rules to manage the Projects based on the sub-classifications. For example, there may be a need to test or review ERCOT Board of Director Approved Projects in a specified timeframe.

Sub-classifications: Protocol 3.10 (8)

      - Regional Planning Group Approved Project. Protocol 3.10 (8)

      - ERCOT Board of Director Approved Project. Protocol 3.10 (8)

      - Unapproved Planning Project. Protocol 3.10 (8)

      - Emergency Project. Protocol 3.10 (8)

      - SPS Project. Protocol 3.10 (8)

      - RMR Project. Protocol 3.10 (8)

  - Inserted into the Model Database for propagation into the appropriate models or cases based on the scheduled in-service date (the “activation date”) or the defined needs of the model.

Examples of models or cases:

    - Network Operations. Protocol 3.10 (8)

    - Dynamic Simulation BP

    - Annual Planning. Protocol 3.10.2, Protocol 3.11

    - CRR Network. Protocol 3.10.3, Protocol 7.5

    - Timeline Database. BP

    - OTS BP

    - Day-Ahead BP

    - Others. BP , Protocol 3.10.

  - Inserted and processed for testing and Case Building as determined by the submitter classification. Protocol 3.10 (8)

  - Identified by a created unique PID for any period in between the current date and the end of the model update period. Protocol 3.10 (8)

  - Supplied to the MPs in the CIM/XML format used by ERCOT. Protocol 3.10.2 (3), 3.10.3 (5), 3.10.4 (5)

  - Verified for sequence validity of all Projects impacted after any move in the scheduled in-service date of a Project. Protocol 3.3.2.1, Protocol 3.10 (7)

This includes, but is not limited to:

    - Checking for sequence validity assuming that all existing Projects scheduled for implementation prior to the schedule date of a projected Project take place. Protocol 3.3.2.1, Protocol 3.10 (7)

    - Verifying that the interconnection points of a Project exist. Protocol 3.3.2.1, Protocol 3.10 (7)

    - Verifying that valid breaker/switch devices exist, will be created, or will be deleted in the Project under validity verification. Protocol 3.3.2.1, Protocol 3.10 (7)

    - Verifying that no Device is left one-ended without a specific approval of such a condition. Protocol 3.3.2.1, Protocol 3.10 (7)

    - Flagging and reporting of Projects which have become invalid due to a sequence change. BP

- Subject to ERCOT Planning validation, allow the creation of Projects from a conversion of IDEV/Python format files to permit Planning Group input. BP, SR

- Allow MP Operations personnel to modify existing associated MP Planning Projects to convert existing data into an Operations Project. BP, SR

- Allow for Projects to be converted into an IDEV/Python format file. Protocol 3.10 (6)

- The ability to designate any node as an Electrical Bus and apply the name provided by the Market Participant to that node. Protocol 3.10.7.1.2

- Provide a Model Test System for the testing of Projects. Protocol 3.10.1, Protocol 3.10.4 (3)

- The Model Testing Environment shall include:

  - The Automated One-line Generator Manager (AOGM) software suites. Protocol 3.10 (7),(8) Protocol 3.10.3 (3)(a)

  - The State Estimator. Protocol 3.10 (7), Protocol 3.10.4 (1), Protocol 3.10.4 (3)

  - LMP calculator software with the capability to load test data for running the LMP calculator. Protocol 3.10 (7), Protocol 3.10.4 (1), Protocol 3.10.4 (3)

  - Loadflow Software. Protocol 3.10 (7), Protocol 3.10.4 (3)

  - SCADA (input from SE). Protocol 3.10 (7), Protocol 3.10.4 (1), Protocol 3.10.4 (3)

  - Raw SCADA/ICCP BP

  - Contingency Analysis. Protocol 3.10 (7), Protocol 3.10.4 (3)

  - Contingency files and editor package to build and modify the contingency files. Protocol 7.5.1 (2)

- The Model Testing Environment shall have a suitable environment to verify and approve the Projects where:

  - Selectable Projects have integrated with the current Network Operations Model to test future configurations. Protocol 3.10.1, Protocol 3.10.4 (3)

  - The Test Model can be tested in a simulated Production Database environment. Protocol 3.10.1, Protocol 3.10.4 (3)

- The capability to transfer Production Database data to the Model Test System shall be required. This capability will enable each time-sensitive model changes to undergo extensive testing with various applications to verify the accuracy of the Projects. An example of the Production Database data required is actual generation schedules. Protocol 3.10.1, 3.10.4 (3)

- The NMG requires the capability to simulate a Production Database environment and run both network security and market applications using the Test Model and the Projects. This will permit the testing of the Test Model and each Project at a user-specified point in time. Protocol 3.10.1, 3.10.4 (3)

- The Model Test System shall support the following capabilities in addition to individual software suite requirements:

  - Simulate the Production Database environment for testing time-sensitive Operations and Market applications including their external interfaces using network security and market applications. Protocol 3.10.1, Protocol 3.10.4 (3)

  - Provide Production Database ICCP and SCADA communication interfaces to test the State Estimator results using Production Database data uploads. Protocol 3.10.1, Protocol 3.10.4 (3)

  - Execute the State Estimator and LMP calculator and provide archival of results with the Test Model dataset before the Projects are made available for loading into the Network Operations or Day-ahead Models. Protocol 3.10.1, Protocol 3.10.4 (3)

  - Perform Load Flow and Contingency Analysis to test the time-dependent aspects of the Test Model and impact on network security. Protocol 3.10.1, Protocol 3.10.4 (3)

  - Validate Projects each time a Project is included or each time the inclusion schedules for the Projects are re-arranged. This is necessary to assure that the remaining Projects are acting on Devices that have remained in the Model Database. Protocol 3.10. (7)

  - Archive all status notification information and time stamps to provide a detailed audit trail of all updates to the Network Operations and Day-ahead Models. Protocol 3.10. (7)

  - Calculate performance statistics within specified periods, as defined by the NMG, for the Model Updating process. Protocol 3.10.4 (3),(4)

Statistics to include, but not be limited to:

    - Number of Projects. Protocol 3.10.4 (3),(4)

    - Status of Projects. Protocol 3.10.4 (3),(4)

    - Number of definable and editable list of attributes or Devices, such as:

      - Ratings. Protocol 3.10.4 (3),(4)

      - Impedances. Protocol 3.10.4 (3),(4)

      - ICCP points. Protocol 3.10.4 (3),(4)

      - Lines. Protocol 3.10.4 (3),(4)

      - Loads. Protocol 3.10.4 (3),(4)

      - Load schedules. Protocol 3.10.4 (3),(4)

      - Breakers. Protocol 3.10.4 (3),(4)

      - Transmission Elements Protocol 3.10.4 (3),(4)

      - Etc. Protocol 3.10.4 (3),(4)

- Pre-defined validation criteria shall exist and be able to be modified by adding new requirements or deleting existing ones. BP, SR

- SCADA links between the defined Devices and the SCADA database shall be able to be revised using various modeling methods, including:

  - Flat file. BP, SR

  - GUI. BP, SR

- A linking tool between the Model Database and ICCP data shall include searching tools to identify potential ICCP point links using filters such as:

  - PID. BP, SR

  - Station. BP, SR

  - kV level. BP, SR

  - Company. BP, SR

  - Device type. BP, SR

  - Any “filterable” combination of ICCP data and associated equipment attributes to provide granularity. BP, SR

- Contingencies shall be able to be extracted for use in other applications including, but not limited to:

  - PTI/PSSE. BP, SR

  - PowerWorld. BP, SR

- The unique PID will be assigned to each Project for status monitoring and audit trail purposes. Protocol 3.10 (8)

- A comprehensive audit trail of all model update activities and the resultant changes; e.g. Device added, modified, or deleted; both to the Model Database, and to all of the associated databases and external interfaces shall be automatically created. Protocol 3.10 (8), Protocol 3.10.4 (3)

- Reporting tools shall be included that allow for custom-defined queries and reports based on the information residing in the audit trail and the Model Database. Protocol 3.10 (8), Protocol 3.10.4 (3)

- In order to preserve and promote the accuracy of the data creating the Network Operations Model an Automated One-line Generator Manager (AOGM) shall be provided. Protocol 3.10 (7),(8) Protocol 3.10.3 (3)(a)

The AOGM shall be able to support the following:

- The AOGM shall automatically generate substation and generating plant one-line diagrams based on time-based model data. Protocol 3.10 (7),(8)

  - Only a single drawing shall be required for all applications to utilize. BP, SR

  - It will be used by all power application software packages, such as, but not limited to the following packages: BP, SR

    - RTNET. BP, SR

    - STNET. BP, SR

    - SCADA. BP, SR

- The AOGM shall provide for the conversion of the existing ERCOT One-line diagrams.

  - The AOGM shall automatically convert the existing ERCOT one-line diagrams. BP, SR

  - During the automated conversion process, the model editing software shall maintain the position of the Device(s) within the station and the existing ERCOT one-line diagrams. BP, SR

- The AOGM shall have editable and easily definable parameters for building displays. BP, SR

- The AOGM shall create a System One-line diagram for any grouping of:

  - The entire Network Operations Model. Protocol 3.10 (7),(8) Protocol 3.10.3 (3)(a)

  - Company or Companies. BP, SR

  - Weather Zone(s). BP, SR

  - Voltage class(es). BP, SR

  - Cost-Allocation Load Zone. BP, SR

  - Black-start resources and transmission paths. BP, SR

  - Hubs Protocol 3.10.3 (3)(a)

  - Selectable defined regions BP

  - Definable, via UI, lists of interconnected substations BP

  - Annual Planning Model BP

  - CRR Network Model Protocol 3.10.3 (3)(a)

  - Definable groups of Electrical Busses BP

- The AOGM shall use a definable easily editable Device library. BP, SR

- The AOGM shall have the capability to create single or multiple one-line diagrams associated with a Project or series of Projects. Protocol 3.10 (7)

- The AOGM shall automatically update the System One-line diagram when an individual one-line diagram has been updated or created. Protocol 3.10 (7), (8), Protocol 3.10.3 (3)(a)

- The AOGM build a complete one-line diagram with all Settlement Points (indicating the Settlement Point that the Electrical Bus is a part of) and including all Hub Buses used to calculated Hub prices (if applicable). This one-line shall be included in the CRR Network Model build. Protocol 3.10.3 (3)(a)

Appendix 4: Transition from Planning Project to Operations Project

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

## Model Database

Appendix 5: Model Database

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Model Database

The Model Database is the storehouse for the data required to build Operator Training Simulator (OTS), Dynamic Simulation, CRR Network, Annual Planning and Network Operations Models. The tools available to manage this data will ultimately determine how well the system functions. These tools must be well designed and user friendly.

### Requirements for the Model Database

- Classification of data.

The following data sets shall be managed within the NMMS:

  - SCADA Communication (ICCP/RTU). Protocol 3.3.2.1(f), Protocol 3.10.

  - SCADA. Protocol 3.3.2.1(f), Protocol 3.10.

  - Generation Subsystem. Protocol 3.3.2.1, Protocol 3.10.

  - Network Operations. Protocol 3.3.2.1, Protocol 3.10

  - Contingency. Protocol 3.10.3 (3)

  - Remedial Action (SPS/RAP). Protocol 3.10.7.3 (2)

  - Dynamic Ratings. Protocol 3.10.8 (1),(2),(3),(4), Protocol 3.10.8.4.1(a), Protocol 3.10.8.4 (2), Protocol 3.10.8.3

  - Annual Planning. Protocol 3.10 (5), (9), Protocol 3.10.2 (1), Protocol 3.10.8 (1)

  - OTS. Protocol 3.10 (5)

  - Test Protocol 3.10.4 (3)

- The Model Database is the set of databases that stores:

  - Projects. Protocol 3.10 (8)

  - Annual Planning Model. Protocol 3.10 (9), Protocol 3.10.2

  - CRR Network Model. Protocol 7.5.1, Protocol 3.10.3

  - Dynamic Simulation Model. Protocol 3.10 (5)

    - Note: The Dynamic Simulation Models include but are not limited to the Generator, FACTS, and other dynamic simulation model devices and associated attributes used for power system dynamic simulations.

  - Network Operations Model. Protocol 3.10

  - OTS Model BP

- A unique identifier:

  - Create a unique identifier (e.g. Device Id or DID) for each new Device within the Project. Protocol 3.10 (11), Protocol 3.10.7.1 (2)

  - That DID will be the primary identifier for each Device in the Model Database. Protocol 3.10 (10), (11), Protocol 3.10.7.1 (2)

  - The DID shall remain the same for the life of that Device. Protocol 3.10 (10), (11), Protocol 3.10.7.1 (2)

  - Confirm that the name listed for each existing Device is unique with respect to all of the other Devices in the Model Database. Protocol 3.10 (10), (11), Protocol 3.10.7.1 (2)

  - Based on the listed name of the Device, find or create the DID for each Device within a Project. Protocol 3.10 (10), (11), Protocol 3.10.7.1 (2)

  - Add DIDs to the description of Devices. BP, SR

- The Model Database editor and data exchange application(s) will meet NMG defined Performance Metrics and calculate the following NMG defined Key Performance indicators:

  - Number of Projects Protocol 3.10 (8)

  - Project In-service date metrics. Protocol 3.10 (8), Protocol 3.10.4 (4)

  - Device In-service date metrics. Protocol 3.10 (8), Protocol 3.10.4 (4)

  - Project In-service date moves. Protocol 3.10 (8), Protocol 3.10.4 (4)

  - Project Revisions. Protocol 3.10 (8), Protocol 3.10.4 (4)

  - Project Status. Protocol 3.10 (8), Protocol 3.10.4 (4)

  - Others as defined by ERCOT. Protocol 3.10 (8)

- Allow the Network Operations Model conversion into the most current PTI PSS/E raw format. BP, SR

- On request, the generation of the entire system overview or the individual project one-line diagrams. Protocol 3.10 (7), Protocol 3.10.3 (3)

- The Network Operations Model and the Outage Scheduler application should be directly linked to allow accurate and automatic flow of data between them. Protocol 3.3.2.1 (d)

- The Generator, FACTS, and other devices used in the Dynamic Simulation Models shall be exportable in an industry standard format. Protocol 3.10 (6)

- To support the Project verification process, the Model Database will include the current Network Operations Model and all completed Projects. The description of the database structure and associated linkages shall be provided. Protocol 3.10.

- All Devices in the Model Database shall include the following fields in addition to the current existing fields:

  - The DID. Protocol 3.10.7

  - Long name. Protocol 3.10.7

  - Name abbreviation. BP, SR

  - Description. BP, SR

  - Congestion management zone(s). Protocol 3.10.

  - Weather zone(s). Protocol 3.10.

  - Physical address. BP

  - Owner(s). Protocol 3.10.

  - Operator(s). Protocol 3.10.

  - Contact phone number for owner(s) and operator(s). BP

  - PTI bus number(s). BP

  - Dynamic Rating set(s) where applicable as required by device type. Protocol 3.10.8

  - Weather Station(s) where applicable as required to accommodate Dynamic Rating system. Protocol 3.10.8

  - Device In-service/Out-of-Service Date(s). Protocol 3.10.

  - Metered end of Device (where applicable). Protocol 3.10.2 Protocol 3.11

  - Load Zone (where applicable). Protocol 3.10.

  - GPS or GIS coordinates of related substations. Protocol 3.10 (7)

  - County name where Device is located. Protocol 3.10 (7), 3.10.2. 3.11

  - Ten additional group designations to be determined by NMG. BP

  - Generator reactive schedules based on area load. BP

- Each Model Database attribute name or description field shall be definable up to 128 characters in length. BP, SR

- Device definitions shall be provided for Transmission Elements and generation related equipment including , but not limited to:

  - STATCOMs. BP

  - Variable Frequency transformer. BP

  - Static Var Compensator. BP

  - Transmission Elements defined in the Nodal Protocols (Section 3.10.7) Protocol 3.10.7

- New element definitions shall be provided.

- Such as, but not limited to:

  - Electrical Bus. Protocol 3.10.7

  - Cost-Allocation Load Zones. BP

  - Hubs. Protocol 3.5.2

- A GUI shall be provided which allows movement of devices within one-lines.  The software redefines the device and associated linkages. All Devices within a station shall be movable, including the ability to move the Device between zones, companies, or stations, without deleting or redefining the Device and the associated database linkages. BP

- All Test Model data shall be able to be revised using various modeling methods, including:

  - Flat file. BP

  - GUI. BP

- The Model Database shall provide a Temperature Based Dynamic Load Schedule capability and functionality based on defined Weather Zones which generates bus load schedules using temperature profile inputs.

  - Functionality flexibility shall include the following selection criteria:

    - Weather Zone. BP

  - Geographically defined area, examples:

    - GPS/GIS coordinates. BP

    - County names(s). BP

    - Substation. BP

    - Annual Planning Model Load Schedules shall allow the use of ERCOT coincident peak load data and Transmission Service Providers (TSP) coincident peak load data. BP

- The Model Database shall accept incremental data as well as batch file changes to rebuild the entire Network Operations Model or to build Projects. Both editing methods shall be subject to validation and logging. Protocol 3.10 (7)

- The Model Database shall be exportable to a flat file.  Flat file import capabilities shall be provided so that extensive modifications can be made to the Network Operations Model. Protocol 3.10

- Selected data, as specified by ERCOT, shall be capable of being exported from the Model Database in the following formats:

  - CIM/XML (as used by ERCOT). Protocol 3.10 (7), 3.10.2 (3), 3.10.3 (5). 3.10.4 (5)

  - Microsoft Access. BP, SR

  - Microsoft Excel. BP, SR

  - PSS/E RAW format (selectable versions including most recent available). BP, SR

  - PSS/E DYRE format for the Generator, FACTS, and other device Dynamic Simulation data (selectable versions including most recent available). BP, SR

  - Text files. Protocol 3.10.4 (4), Protocol 3.10.7.4.1

- The following data transfers and formats shall be supported:

  - The importing and exporting of modeling data using CIM/XML format, as used by ERCOT. Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (5).

  - At a minimum, other data formats shall include:

    - CSV (Guidelines for user friendly CSV file formats will be provided during detailed design). BP, SR

    - PTI PSS/E (.raw, IDEV, Python, DYRE). BP, SR

    - IEEE Standard. BP, SR

- It shall be possible to extract, using SQL or an industry standard query language, selected data from the Model Database using any of the following criteria:

  - PID(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Date Project is submitted. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Market Participant(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - DID(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Device Type(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Device In-service/Out-of-Service Date(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Long name. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Name abbreviation. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Description. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Congestion management zone(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Weather zone(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Physical address. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Owner(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Operator(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - PTI bus number(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - Operator(s) phone number(s). Protocol 3.10 (8), 3.10.4 (3), (4))

  - Owner(s) phone number(s). Protocol 3.10 (8), 3.10.4 (3), (4)

  - kV level. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Metered end of Device. Protocol 3.10 (8), 3.10.4 (3), (4)

  - GPS or GIS coordinates. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Dynamic Rating sets where applicable as required by device type. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Weather Station(s) where applicable as required to accommodate Dynamic Rating system. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Load Zone where applicable. Protocol 3.10 (8), 3.10.4 (3), (4)

  - Areas within, or specific sets, of GPS or GIS coordinates. Protocol 3.10 (8), 3.10.4 (3), (4)

  - County name Protocol 3.10 (8), 3.10.4 (3), (4)

  - Any “filterable” combination of Model Database parameters and associated equipment attributes to provide granularity Protocol 3.10 (8), 3.10.4 (3), (4)

## Update Network Operations Model Process

Appendix 6: Update Network Operations Model Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Update Network Operations Model Process

The update of the Production Database, specifically the Network Operations Model provides the topology to operate the electric system in a reliable manner and provides data required to operate the Nodal Market. Fast, accurate updates are necessary for proper operation of the NMMS. The following requirements are defined to ensure this process will meet the needs of the Nodal Market.

### Requirements for the Update Network Operations Model Process

- Update the Network Operations Model with minimal impact on the execution time and availability. BP, SR

- A comprehensive audit trail of all model update activities and the resultant changes; e.g. Device added, modified, or retired; both to the Network Operations Model, and to all of the associated databases and external interfaces shall be automatically created and available for review. Protocol 3.10 (8)

- At the completion of each model update cycle, report all of the Network Operations Model changes to the System Operators at ERCOT in an easily definable summary form. Protocol 3.10 (8)

- The NMMS shall allow the reversal of a Project or Projects inclusion if inadvertently introduced into the Production Database. BP, SR

- An additional time stamp shall be added to a Project to reflect when the System Operator activated the model update Projects.  The notification and/or audit application(s) shall record the in-service time stamp, the initiating System Operator, and notify the Project submitter through the submitter interface. Protocol 3.10 (8)

- The Model Database Editor interface shall be used to define scheduled implementation into the Network Operations Model with, at least, four options being available.

  - The first option is to be “Schedule”. In this option, time-dated and approved model update Projects are defined for inclusion in the list displayed to the System Operator. The insertion into the Network Operations Model is always selected manually by the System Operator. Protocol 3.10 (7), Protocol 3.10.1 (4)

  - The second option is to be “Immediate”.  In this option, the approved Projects marked for activation would be displayed on the list for System Operator review. After approval by the System Operator the Project would be inserted into the Network Operations Model at the next pre-scheduled update time. If the pre-scheduled time is each hour, the change selected for “Immediate” would be effective at the start of the next hour Protocol 3.10 (7), Protocol 3.10.7.1 (3)

  - The third option is to be the “Event Triggered”.  In this option, the approved Projects marked for activation would be placed displayed on the list for System Operator review. After approval by the System Operator the changes defined by the Project would be inserted into the Network Operations Model by an “Event”. The Event and condition of the event would be specified at Project build time. Typical events may be events, such as: “SCADA flow detected”. Protocol 3.10 (7)

  - The forth option is to be “System Operator Inserted”. If approved by an authorized System Operator, the change would be effective at the start of the next hour or next update period. The System Operator would insert his approval into the list displayed to him. This list will report all scheduled Projects to be placed in service. This report will indicate the trigger method, scheduled time, Project status, and other information about the Project. Protocol 3.10 (7)

## Build Day-ahead Model Process

Appendix 7: Build Day-ahead Model Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Build Day-ahead Model Process

The Day-ahead Model is changed as required by the market application software. This model built by the Day-ahead process is a complete replacement of the previous Day-ahead Model. This model will provide the best estimate of the electric system topology for the Day-ahead.

### Requirements for the Build Day-ahead Model Process

- This model built by the Day-ahead process is a complete replacement of the previous Day-ahead model. BP

- The Day-ahead Model will be built each day as required by Market applications. BP, SR

- The Day-ahead model is the Network Operations Model at the time the build starts plus any remaining approved scheduled (dated) Projects for that day plus the scheduled and approved Projects for the next day plus the approved outages for the next day. BP, SR

- The NMMS shall update the Day-ahead Model for the above mentioned business functions with minimal impact on the execution time and availability. BP, SR

- At the completion of each model update cycle, report all of the Day-Ahead Model changes to the System Operators at ERCOT in an easily definable summary form.  BP

- The Day-ahead Model shall be posted on the MIS Secure Area. BP

- The NMMS shall allow the reversal of a Project or Projects inclusion if inadvertently introduced into the Production Database. BP

- The Model Database Editor interface shall be used to define scheduled implementation into the Day-ahead Model with, at least, four options being available.

  - The first option is to be “Schedule”. In this option, time-dated and approved model update Projects are defined for inclusion in the list displayed to the System Operator. The insertion into the Day-ahead Model is always selected manually by the System Operator. BP

  - The second option is to be “Immediate”.  In this option, the approved Projects marked for activation would be displayed on the list for System Operator review. After approval by the System Operator the Project would be inserted into the Day-ahead Model at the next pre-scheduled update time. If the pre-scheduled time is each hour, the change selected for “Immediate” would be effective at the start of the next hour. Protocol 3.10.8 (2)(a)

  - The third option is to be the “Event Triggered”.  In this option, the approved Projects marked for activation would be placed displayed on the list for System Operator review. After approval by the System Operator the changes defined by the Project would be inserted into the Day-ahead Model by an “Event”. The Event and condition of the event would be specified at Project build time. Typical events may be events, such as: “SCADA flow detected”. BP

  - The forth option is to be “System Operator Inserted”. If approved by an authorized System Operator, the change would be effective at the start of the next hour or next update period. The System Operator would insert his approval into the list displayed to him. This list will report all scheduled Projects to be placed in service. This report will indicate the trigger method, scheduled time, Project status, and other information about the Project. Protocol 3.10 (7)

## Internal Data Editor and Comparison Tool

Appendix 8: Internal Data Editor and Comparison Tool

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Internal Data Editor and Comparison Tool

The Data Editor and Comparison tool is required to properly maintain the NMMS. This tool routinely both manually and automatically as defined by the NMG, checks and compares the data in the Production Database and the Model Database. The data editor will be used to make large changes in the data when required.

### Requirements for the Internal Data Editor and Comparison Tool

- Consistent with the testing objectives of the user’s operating group, each user should be able to extract from the Model Database up-to-date and accurate data selectively by or any combination of:

  - Date. Protocol 3.10.4 (3)

  - Station. Protocol 3.10.4 (3)

  - Voltage class. Protocol 3.10.4 (3)

  - Market Participant. Protocol 3.10.4 (3)

  - Submitter classification. Protocol 3.10.4 (3)

  - Weather Zone. Protocol 3.10.4 (3)

  - Hub. Protocol 3.10.4 (3)

  - Load Zone. Protocol 3.10.4 (3)

- Comparisons of the data in the Annual Planning, Day-ahead, and Network Operations Models must be easily accomplished. Differences may be output to displays and printable on demand.  Protocol 3.10.4 (3)

- Comparisons of data between any model and a similar version of the same model must be easily accomplished.  Differences may be output to displays and printable on demand.  (ex.  The ability to compare the April Network Operations Model to the March Network Operations Model must exist). Protocol 3.10.4 (3)

- It shall be possible to extract, using SQL or an industry standard query language, selected data from the Model Database as described in Section 3.6.2.20 (Requirements for the Model Database). BP

## Application Software Data Process

Appendix 9: Application Software Data Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Application Software Data Process

All data being managed by the NMMS must be available for various application software packages to use. This availability is a critical function for all study software as well as Production Database based Market applications.

The following applications will be impacted by NMMS:

- State Estimator.

- STNET (Study Network Application).

- Outage Scheduler.

- Contingency Analysis.

- Dynamic Study Software.

- MIS (Market Information System).

- OTS

Known impacts at this time are:

- Increased length of name and description fields.

- Topology will be available for periods into the future rather than being limited to the present configuration.

- Large increase in posted data.

### Requirements for the Application Software Data Process

- All data being managed by the NMMS must be available for various application software packages to use. BP

- The various databases shall store the required data until an application request for a defined data set. The NMMS will provide a defined interface for the transfer of the required data.  BP

## Timeline Database Process

Appendix 10: Timeline Database Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Timeline Database Process

The Timeline Database Process provides a CIM model for posting as defined in the time-line in section 3.10of the Nodal Protocols. This process will also provide output to meet other requirements in the NMMS.  The Timeline Database Process shall provide CIM/XML models of the Annual Planning, CRR Network, and Network Operations Model for posting or other use per the Nodal Protocols.

### Requirements for the Timeline Database Process

- The Timeline Database Process will provide a model that includes all Projects up to the desired start date. This model forms the starting point for the CIM-based model of interest. Protocol 3.10.1, Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (3), (4), (5)

- Each Project from the start date to the end date is attached to the build. Protocol 3.10.1, Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (3), (4), (5)

- With the resulting Model Database, a model may be constructed for any period between the start date and the end date. Protocol 3.10.1, Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (3), (4), (5)

- A selection may be made to include or exclude outages in the constructed model. Protocol 3.10.4 (3), Protocol 3.12

- Load and generation will be loaded with default data. BP

- The CIM files will be exported in CIM/XML format, as defined by ERCOT, and posted to the MIS Secure Area according to the table time-line in Section 3.10 of the Nodal Protocols.  Other Nodal Protocol posting requirements for the CRR Network, Annual Planning, OTS, Dynamic Simulation, and other models will use the Timeline Database Process. Protocol 3.10.3 (5), Protocol 3.10.4 (5)

Appendix 11: Posted Timeline Database

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

## Study Case Builder Process

Appendix 12: Study Case Builder Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Study Case Builder Process

Study Case Builder process provides OTS, Outage Scheduling, and other study cases that may be any allowed combination of the Network Operations Model and future projects including Planning Projects.  The Study Case Builder is envisioned as the tool used for providing cases for the use of Outage Schedule personnel, Planning personnel, Operations Planning personnel or other ERCOT personnel describing the future Transmission Grid configurations outside the near Production Database time frame.  Studies of the time period covered by the Network Operations Model shall use the Network Operations Model (i.e. outage studies in the shorter time frames).

### Requirements for the Study Case Builder Process

- The Study Case Builder shall implement a case from the approved Projects and the selected Network Operations or Annual Planning Model. Protocol 3.10.1, Protocol 3.10.2, Protocol 3.10.3, Protocol 3.12

- The start and end date selected will determine the Projects to be included in the Study or OTS case. Protocol 3.10.1, Protocol 3.10.4 (3), Protocol 3.12

- Outages may be imported from the Outage Scheduler for the period specified by the start and end dates. Protocol 3.10.4 (3)

- The Study Case Builder interface will provide detailed selection criteria for the basis of the case. BP, SR,

- The selection criteria shall include, but not be limited to, the following:

  - Load Outages for specified date. BP, SR,

  - Review outage table and delete selected outages. BP, SR,

  - Load selected generation plan. BP, SR,

  - Set system load. BP, SR,

## CRR Auction Case Builder Process

Appendix 13: CRR Auction Case Builder Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the CRR Auction Case Builder Process

CRR Auction Case Builder process provides a case prepared from the CRR Network Model.  The CRR Network Model is based on the Network Operations Model, selected outages, and selected future projects as defined by the Nodal Protocols.

### Requirements for the CRR Auction Case Builder Process

- The CRR Case Builder shall implement a case from the approved Projects and the selected Operations or Planning Model. Protocol 3.10.3, Protocol 7.5.1, Protocol 7.5.3

- The start and end date selected will determine the Projects to be included in the CRR case. Protocol 3.10.3, Protocol 7.5.1, Protocol 7.5.3

- Outages may be imported from the Outage Scheduler for period specified by the start and end dates. Protocol 3.10.3, Protocol 7.5.1 (2)

- The CRR Case Builder interface will provide detailed selection criteria for the basis of the case. BP, SR

- The selection criteria shall include the following:

  - Load Outages for specified date(s) according to the definitions provided in the Nodal Protocols. Protocol 7.5.1 (2)

  - Review outage table and delete selected outages. Protocol 7.5.1 (2)

  - Load selected generation plan. Protocol 7.5.1 (2)(b), Protocol 7.5.1 (3)

  - Set system load. Protocol 3.12, Protocol 7.5.1 (2)(b), Protocol 7.5.1 (3),

- Each CRR case must include the same attributes as the CRR Network Model:

  - A complete one-line diagram with all Settlement Points (indicating the Settlement Point that the Electrical Bus is a part of) and including all Hub Buses used to calculate Hub prices (if applicable). Protocol 3.10.3 (3)(a)

  - Generation Resource locations. Protocol 3.10.3 (3)(b)

  - Transmission Elements. Protocol 3.10.3 (3)(c)

  - Transmission impedances. Protocol 3.10.3 (3)(d)

  - Transmission ratings. Protocol 3.10.3 (3)(e)

  - Contingency lists. Protocol 3.10.3 (3)(f)

  - Data inputs used in the calculation of Dynamic Ratings. Protocol 3.10.3 (3)(g)

  - Other relevant assumptions and inputs used for the CRR Network Model.  Protocol 3.10.3 (3)(h)

## Planning Case Builder Process

Appendix 14: Planning Case Builder Process

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Planning Case Builder Process

Planning Case Builder process is based on a sub-classification selection of approved projects, preliminary projects, or other designations.  The Planning Case Builder shall use the Network Operations Model as the primary building block.

### Requirements

- The Planning Case Builder shall implement a case from the approved Projects and the selected Network Operations or Annual Planning Model. Protocol 3.10.2, Protocol 3.11

- The start and end date selected will determine the Projects to be included in the specific case. Protocol 3.10.2

- The importation of outages may be selected for a specified period. Protocol 3.10.2

- The Planning Case Builder interface will provide detailed selection criteria for the basis of the case. Protocol 3.10.2

- The Planning Case Builder shall, upon demand, output a PTI PSS/E RAW format file and CIM/XML format. BP

- The Planning Case Builder shall, upon demand, output the PTI PSS/E DYRE file and CIM/XML format. BP

- Annual Planning Model Load Schedules which allow the use of ERCOT coincident peak load data or TSP coincident peak load data shall be used by the Planning Case Builder. BP

- The selection criteria shall include the following:

  - Load Outages for specified date. Protocol 3.10.2

  - Review outage table and delete selected outages. Protocol 3.10.2

  - Load selected generation plan. Protocol 3.10.2

  - Set system load. Protocol 3.10.2, Protocol 3.12

## Posting and Notification Capabilities

Appendix 15: Posting and Notification Capabilities

[See Referenced Appendix in PR50142 NMMS Network Model Management System Requirements Appendix v1.2a]

### General Statement of the Posting and Notification Capabilities

The Posting and Notification capabilities are considered to be an important part of keeping the Market Participants informed as soon as practicable of the activities occurring in the Model Database. Notifications will occur between ERCOT Planning, ERCOT Operations, and between ERCOT and Market Participants to ensure each area is advised of the most current relevant information. In general, the Posting and Notification system is expected to provide the opportunity for full awareness of Model Database change activities.

### Requirements for the Posting and Notification Capabilities

- All approved Projects shall be posted on the secure area of the Market Information System (MIS Secure Area) website of ERCOT. Protocol 3.10 (1),(2),(3),(4),(8)

- The description of the Project and a CIM model of the Project will be posted automatically when the Project is approved by either the ERCOT Network Model Group or the Planning Group (i.e. moves into the Model Database). Protocol 3.10 (1), (8), (11), Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (5)

- The data posting will be using the industry data exchange format of CIM/XML, as used by ERCOT. Protocol 3.10 (8)

- Each schedule or other change to a Project will be posted to the MIS Secure Area. Protocol 3.10 (10), (11)

- A CIM-based representation of the Annual Planning, CRR Network, and Network Operations Model(s) and incremental updates will be posted according to Nodal Protocols. This CIM-based representation is outlined in the section Timeline Database Process. Protocol 3.10.1, Protocol 3.10.2 (3), Protocol 3.10.3 (5), Protocol 3.10.4 (5)

- When a scheduled Project is moved into the Network Operations Model, an MIS Secure Area posting with time and date shall confirm the change. Protocol 3.10 (3)

- If a Project is not energized within the allocated scheduled time, then a MIS Secure Area posting shall advise Market Participants with all necessary information to identify the Project. Protocol 3.10 (8)

- Non-scheduled Projects, such as Dynamic Rating updates, that make changes to the Network Operations Model will be posted to the MIS Secure Area as soon as practicable. These Projects shall contain the reason for the change and shall follow Protocol requirements for notification. Protocol 3.10 (8)

| Deadline to submit Projects to the NMG | Model complete and available for Test | Updated Operations Model testing complete | Updated Operations Model sent to Production Environment | Target physical equipment in-service date |
| --- | --- | --- | --- | --- |
| January 1 | February 15 | March 15 | April 1 | Month of April |
| February 1 | March 15 | April 15 | May 1 | Month of May |
| March 1 | April 15 | May 15 | June 1 | Month of June-August Summer Model |
| June 1 | July 15 | August 15 | September 1 | Month of September |
| July 1 | August 15 | September 15 | October 1 | Month of October |
| August 1 | September 15 | October 15 | November 1 | Month of November |
| September 1 | October 15 | November 15 | December 1 | Month of December |
| October 1 | November 15 | December 15 | January 1 of next year | Month of January of next year |
| November 1 | December 15 | January 15 of next year | February 1 of next year | Month of February of next year |
| December 1 | January 15 of next year | February 15 of next year | March 1 of next year | Month of March of next year |

Notes:

1.	Transmission and Resource data submissions complete per ERCOT Network Operations Model Change Request process for inclusion in next update period.

2.	Network Operations Model data changes and preliminary fidelity test complete.  The test version of the Network Operations Model will be available for market review and test.

3.	Testing of the Network Operations Model by Market Participants is complete.

4.	Updates include changes starting at this date and ending within the same month.

Table 2: Dates for Data Submittals, Model Testing, and Model Posting

# SIGN OFFS

PMO Review by: 	Date

Business Manager: 	Date

Business Owner: 	Date

Business Owner: 	Date

IT Delivery Manager: 	Date

IT Manager, Computer Application Services: 	Date

PMO Project Manager: 	Date

Business Project Manager: 	Date

IT System Architect: 	Date

Project Owner: 	Date

Date

Date

Date

Transmission and Resource data submissions complete per ERCOT Network Operations Model

Network Operations Model data changes and preliminary fidelity test is complete. The version of the Network Operations Model will be available for Market review and test.

Testing of the Network Operations Model by Market Participants is complete.

Updates include changes starting at this date and ending within the same month.