| PRR Number | 650 | PRR Title | Balancing Energy Price Adjustment Due to Non-Spinning Reserve Service Energy Deployment |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.7.4 – Deployment of Non-Spinning Reserve Service 6.8.1.12 – Payments for Balancing Energy Provided from Ancillary Services During the Operating Period 6.9.5.1 – Balancing Energy Clearing Price |  |
| Requested Resolution (Normal or Urgent) |  | Normal |  |
| Revision Description |  | Adjusts the Market Clearing Price of Energy (MCPE) when the deployment of Non-Spinning Reserve Service (NSRS) occurs to provide correct price signals via a post-deployment adjustment to separate the pricing solutions from the deployment on NSRS. This adjustment will still be subject to MCSM, if the Balancing Energy Service Up bids are depleted and no zonal congestion occurs, as detailed in Protocol Section 6.9.5.1(2). ERCOT will provide a notice to all market participants that highlights the interval beginning the NSRS deployment and another notice when NSRS deployment is being discontinued. The adjusted MCPE price signal would remove NSRS deployments and manually re-run SPD, sending accurate price signals in an energy-only market design. |  |
| Reason for Revision |  | The foremost concern is the reliability issues that arise every time NSRS is deployed for depletion of balancing energy bid stack. The original intent of NSRS is deployment in response to loss-of-Resource contingencies on the ERCOT system or for the depletion of the responsive reserve stack. In the relatively few intervals when ERCOT deploys NSRS, the resulting MCPE is improperly depressed. This causes an incorrect price signal to the market during a time when more, not less, generation is needed. This creates a greater reliability concern. Currently, when the Bid Stack for BES Up is estimated by ERCOT to reach 95% deployment, ERCOT deploys NSRS in accordance with Protocol Section 6.7.4 (1) to replenish the BES Up stack. The resulting MCPE applied to all Balancing Energy is improperly distorted due to the fact that a large block of price-taking NSRS is suddenly deployed. The result is a distorted MCPE price that signals to the market that less capacity should be committed and carried online, when the economically correct price signal should be that the market is capacity short and additional capacity that can cost effectively be brought into the market is needed. |  |
| Credit Implications (Yes or No, and summary of impact) |  | None because credit requirements use the adjusted MCPE |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | Yes, but will be limited |  |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  |  |  |
| Timeline |  |  |  |
| Date Posted |  | 2/24/06 |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Randa Stephenson |
| E-mail Address | randa.stephenson@txu.com |
| Company | TXU |
| Company Address | 1717 Main Street Dallas, TX 75201 |
| Phone Number | 214-875-8156 |
| Fax Number | 214-875-9042 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | Manual after the fact price adjustment, which includes modify case-input file to remove NSRS deployments, manually re-run SPD and post the prices in BES report (like MCSM) |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | None |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Additional generation online from accurate price signals of depleted UBES stacks | Improves reliability. The true market benefits are that generators will receive the proper pricing signals and therefore respond appropriately. Loads could end up paying a higher price, but this higher price should incite the appropriate new build/unmothball decisions and commitment to come online day ahead. Additionally, for every hour that NSRS is deployed for BES bid stack replenishment it is not available for the other reasons NSRS was procured for. The current procedures create a death spiral. Loads lean on the BES market over peak, instead of contracting bilaterally. Some generators anticipate the need and bid into BES others do not. BES stack is depleted, therefore ERCOT deploys NSRS to replenish BES bid stack which results in lower MCPE’s and less BES deployments. Generators will stop bidding into BES. Loads see the lower prices and lean on the BES market ever harder which leads to ERCOT having to use NSRS more often. |
|  | 2 | Adjusted MCPE subject to mitigation by MCSM when stack is depleted | Minor impact due to the infrequency of Non-Spin deployments |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Protocol Language Revision

6.7.4	Deployment of Non-Spinning Reserve Service

(1)	ERCOT shall deploy Non-Spinning Reserve Service when it predicts more than 95% of the Balancing Energy Service Up available for the Operating Hour will be deployed.

(2)	ERCOT may, in its sole judgment, deploy Non-Spinning Reserve Service as necessary.

(3)	Deployment of Non-Spinning Reserve Service Resources will be proportioned among suppliers.

(4)	NSRS deployment or recall instructions by ERCOT are not constrained by any ramp rate.  However, the QSE is expected to deploy or recall those instructions at a ramp rate that would comply to the instruction in thirty (30) minutes.  During a period of NSRS deployment, all energy provided by the QSE responding to the NSRS deployment will be considered instructed.

(5)	Energy from Non-Spinning Reserve capacity may be deployed in a Congestion Zone by ERCOT if, in its judgment, not enough Balancing Energy Service Up is available to satisfy reliability needs.

(6)	ERCOT will provide Notice via the Messaging System to QSEs of their obligations for NSRS energy as the QSE's Resources are selected.  Providers will be required to respond with manual or electronic acknowledgement. ERCOT will provide Notice via the Messaging System to all QSEs at the beginning of the interval when NSRS is deployed and at the end of the interval when NSRS is deployed.

(7)	All providers of Non-Spinning Reserve Resources will provide Notification to ERCOT of their availability and level of deployment.

(8)	Once deployed, the Obligation to deliver energy will remain until ordered to stop providing by ERCOT (after not less than one hour), but not longer than the period of the service is scheduled.

(9)	NSRS may be deployed at any time in a Settlement Interval.

(10)	The QSEs providing Non-Spinning Reserve Service shall meet the deployment performance requirements specified in Section 6.10.4, Ancillary Services Deployment Performance Measures.

[PRR496: Add (11) to Section 6.7.4 as follows when system change implemented.] (11) Non-Spinning Reserve Service procured from a Load Acting as a Resource Block Bid shall be deployed as a block.

6.8.1.12	Payments for Balancing Energy Provided from Ancillary Services During the Operating Period

All Balancing Energy deployed by a Dispatch Instruction and delivered by a generating Resource from Balancing Energy Service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.8.1.13, Resource Imbalance, except when Non-Spinning Reserve Service is deployed in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service, paragraph (1) or (5).  In that case, the MCPE shall be the MCPE that would have occurred if the deployments of Non-Spinning Reserve Service did not occur.  All Balancing Energy deployed by a Dispatch Instruction and delivered by a Load acting as Balancing Energy Service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.9.5.2, Settlement for Balancing Energy for Load Imbalance, except when Non-Spinning Reserve Service is deployed in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service, paragraph (1) or (5).  In that case, the MCPE shall be the MCPE that would have occurred if the deployments of Non-Spinning Reserve Service did not occur.  Settlement for any energy provided without an authorized Dispatch Instruction will be settled according to Section 6.8.1.15, Payments for Balancing Energy Provided from Uninstructed Deviation.

Deployed Balancing Energy Up on qualified Balancing Up Loads will be paid a capacity payment for the first Settlement Interval that it is deployed equal to the MCPC of Non-Spinning Reserve Service for the hour in which the deployment occurs.  A continuous deployment of Balancing Energy Up on qualified Loads for over sixty (60) minutes will be paid a capacity payment for each subsequent Settlement Interval which it is deployed equal to the MCPC of Non-Spinning Reserve Service for that hour divided by four (4).

6.9.5.1	Balancing Energy Clearing Price

(1)	A Market Clearing Price for Energy (MCPE) will be calculated for each Settlement Interval by Congestion Zone as a product of the mathematical optimization model.  The MCPE will be used to pay or charge each QSE, for each Settlement Interval for Balancing Energy Service.

(2)	In the event that Non-Spinning Reserve Service is deployed for an interval in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service, paragraph (1) or (5), ERCOT shall adjust MCPE by disregarding the deployments of Non-Spinning Reserve Service and procuring the Balancing Energy Service that would have been necessary. Afterwards, a Market Clearing Price for Energy (MCPE) will be calculated for each Settlement Interval by Congestion Zone as a product of the mathematical optimization model and in accordance with 6.9.5.1 (3).  The MCPE will be used to pay or charge each QSE, for each Settlement Interval for Balancing Energy Service.

(3)	In the event there is no Zonal Congestion and all available Balancing Energy Service Up bids are deployed for an interval, ERCOT shall adjust the MCPE as follows:

MCPEa	=	Min (MCPE, 95% price *1.5)

Where:

MCPEa	the adjusted MCPE

MCPE	the unadjusted MCPE

95% price	the MCPE that would result from deploying 95% of the available MW in the Bid Stack

During a Settlement Interval for which an adjusted MCPE is calculated, quantities of Balancing Energy Service Up bids above MCPEa shall be paid as bid, with the sum of incremental payments above the MCPEa charged to each QSE that was charged for Resource Imbalance and/or Load Imbalance during the Settlement Interval, as follows:

IRSiq	=	∑[MAX(0, RIizq) + MAX(0, LIizq)]z/ ∑[(MAX(0, RIizq) + MAX(0, LIizq)]zq

Where:

i	interval being calculated

z	zone being settled

q	QSE

IRSiq	Imbalance Ratio Share per interval per QSE

RIizq	Resource Imbalance ($) per interval per zone by QSE

LIizq	Load Imbalance ($) per interval per zone by QSE

The allocation of charges to QSEs with positive Resource and/or Load Imbalance is:

QPAMiq	=	PAMi * IRSiq

Where:

i	interval being calculated

q	QSE

IRSiq	Imbalance Ratio Share per interval per QSE

QPAMiq	QSE share of incremental payments above the MCPEa

PAMi	Incremental Payments above MCPEa paid out by ERCOT

Incremental payments above MCPEa are uplifted to QSEs that were charged for Load Imbalance and Resource Imbalance for intervals in which an adjusted MCPE is calculated.

The incremental amount paid above MCPEa will be calculated manually by truncating the aggregate Bid Stack at the MCPEa level and determining how many MW were accepted above this amount.  The integrated MWh under the curve will be paid as bid.  The total incremental amount will be uplifted to QSEs through the method described above.