Principles of Consistency

Engineering Planning, CRR Auction, and Network Operations Models

- CRRs Auctions will occur up to 2 ½ years before actual operations.  Potential bidders must use the Annual Planning Models to determine where and how much to bid of individual CRR financial instruments (Note these are sold Electrical Bus to other Electrical Buses or to HUBS and/or Zones).

  - These models must be consistent to provide market participates with clear and accurate information.  Consistent in naming convention so that the information is clear as to which CRR is being purchased. And consistent in modeling the system components so that accurate results are reflected in the economics of the CRR market.

- Consistency between the Engineering Planning, CRR Auction and Network Operations Models means:

  - The name of existing Transmission Elements including Electrical Buses, lines, transformers, generators, loads, breakers, switches, capacitors, reactors, phase shifters, or other similar devices, if modeled as part of the ERCOT Transmission Grid, must be identical without using abbreviations or acronyms (Protocols Section 3.10.7.1)

  - New and not yet named future Transmission Elements should use the same naming convention to minimize name changes in the future

  - All existing transmission line’s and transformer’s impedances (or equivalent branch circuit impedance) and ratings (Normal, Emergency and 15-Minute) shall be identical and use Dynamic Ratings where implemented according to the nodal protocols

  - ERCOT’s Engineering Planning models should start from the posted Annual Planning Model

  - If all normally closed breakers and switches are closed and normally open breakers and switches are open in the CRR Auction and Network Operations Model, the line calculated line flows between substations in the Engineering Planning model shall be consistent (very close)

  - TSP’s models for short circuit analysis, voltage stability, and frequency stability may use terminology and modeling used in standard engineering practices but should be consistent with ERCOT terminology if the TDU expects to provide the model to ERCOT or other Market Participants