| PRR Number | 662 | PRR Title | Modify Ancillary Service Deployment Performance Conditions |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.10.6 Ancillary Service Deployment Performance Conditions |  |
| Requested Resolution (Normal or Urgent) |  | Urgent |  |
| Revision Description |  | Clarifies current language exempting the intervals following a Forced Outage from the Schedule Control Error compliance measure and adds additional exemptions to address infeasible portfolio instructions, Responsive Reserve Service ramping, unit testing, and the scheduling of Uncontrollable Renewable Resources. |  |
| Reason for Revision |  | A Schedule Control Error performance measure stringent enough to meaningfully contribute to satisfactory system frequency control and efficient procurement and deployment of Regulation Service cannot, in most instances, be met by QSEs experiencing one or more of the exempted events addressed in this proposed revision. |  |
| Credit Implications (Yes or No, and summary of impact) |  | None. |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | Unknown. |  |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Unknown. |  |
| Timeline |  |  |  |
| Date Posted |  | 4/10/06 |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Mark J. Bruce (on behalf of WMS Frequency Task Force) |
| E-mail Address | mark_j_bruce@fpl.com |
| Company | FPL Energy |
| Company Address | 816 Congress Ave., Suite 1100 Austin, TX 78701 |
| Phone Number | 512-685-1860 |
| Fax Number | 512-685-1901 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | Amendments to Section 6.10.6 do not require ERCOT system changes. Qualifying intervals can be manually excluded from the affected QSEs’ SCE performance calculation. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | None. | None. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Avoids improper disqualification of QSEs from providing Ancillary Services. | Avoids unneccessary depletion of Ancillary Service bid stacks and the resulting higher prices expected from tighter supply. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Protocol Language Revision

6.10.6	Ancillary Service Deployment Performance Conditions

ERCOT shall determine the performance of providers of QSE SCE Performance under normal operating conditions.  ERCOT shall remove from consideration of average performance of a QSE any period during which any of the following events has occurred and which does not have a passing score.

(1)	The two (2) hour period after the QSE has experienced a Forced Outage and the QSE is under-generating;

(2)	The entire Settlement Intervals for all QSEs in which ERCOT has deployed or recalled Balancing Energy in response to an Unusual Event;

(3)	The entire Settlement Intervals in which ERCOT has issued a verbal Dispatch Instruction;

(4)	The period where ERCOT issues instructions to any QSE to be deployed at ramp rates in excess of the unit’s capabilities or QSE’s capabilities;

(5)	The ten (10) minute period in which a QSE is ramping up due a RRS deployment;

(6)	The entire Settlement Interval(s) in which a QSE is ramping into (from time of notification to time of deployment) or out of (from end of deployment to thirty (30) minutes later) a NSRS deployment;

(7)	If requested by a QSE, the entire Settlement Interval(s) in which a QSE is performing ERCOT-required testing, limited to X hours per month; {'X' to be determined}

(8)	The entire Settlement Interval(s) in which a QSE’s Resource Plan shows only Uncontrollable Renewable Resources On-line; and

PIP112: Add item #9 below to the list and renumber the existing (9) to (10) when BUL is implemented: (9) If requested by a QSE, up to three (3) scheduling hours following an instruction issued by ERCOT to recall previously deployed Responsive Reserve, Non-Spin or Balancing Energy Up Services provided from Replacement Capacity that is using qualified Load as the QSE’s Resource; and

(9)	Certain other periods of abnormal operations as determined by ERCOT.