| PRR Number | 661 | PRR Title | SCE Performance Enforcement Criteria |
| --- | --- | --- | --- |
| Date |  | 4/20/06 |  |
| Submitter’s Information |  |  |  |
| Name |  | John Dumas |  |
| E-mail Address |  | jdumas@ercot.com |  |
| Company |  | ERCOT |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 |  |  |
| Additional Qualitative Information | 1 |  |  |
| Other | 1 |  |  |

Comments

ERCOT would not be able to perform monitoring on a 100% deployment rate as proposed by this PRR.  Currently, a participant’s awarded regulation share in MWs determines the maximum MW level that the Automatic Generation Control (AGC) can deploy.  The 125% level determines the maximum ramp rate at which a participant can be deployed by AGC on a four-second cycle basis.  There is currently no way to determine from history what percentage of a participant’s ramp rate was deployed by AGC.  ERCOT can only determine the integrated MW amount of deployment over a given time range.

This PRR does not propose a change to the existing compliance monitoring criteria.  ERCOT Compliance will follow the same process of analyzing SCE as it does today under Section 6.10.5.3, SCE Monitoring Criteria.  Therefore, PRR661 as submitted should not adversely affect ERCOT’s ability to pass the NERC performance criteria.  This PRR does, however, introduce payment formulas based on QSE performance.  ERCOT will need clarification of the formulas and exemption clauses in order to evaluate their effectiveness and system impacts.

Revised Proposed Protocol Language

6.10.5.3	SCE Monitoring Criteria

SCE Monitoring Criteria will be reviewed by the appropriate ERCOT TAC subcommittee and submitted into these Protocols upon approval.

Each QSE shall control its Resources to operate to the final Resource bilateral schedules as converted to a base power function plus the equivalent power requirement of any instructed Ancillary Services and other SCE obligation terms including governor response.  ERCOT shall calculate one (1) and ten (10) minute averages of each QSE’s SCE.  ERCOT shall also calculate each QSE’s participation factor as the ratio of the QSE’s generation scheduled change in the measurement period (1 or 10 minute) to the total ERCOT generation scheduled change in the same measurement period.  ERCOT shall limit the deployment of RGS Service to QSEs for each control cycle equal to one hundred twenty five percent (125%) of the total amount of RGS Service in ERCOT divided by the number of control cycles in ten (10) minutes.  Intervals where a QSE’s generation is less than one (1) MW in the measurement period (1 or 10 minute) will not be included in the calculation of the SCE Monitoring Criteria.  Satisfactory control performance of the QSE shall be deemed acceptable when:

(1)	The one (1) minute averages of the QSE’s SCE meet the following criteria over the calendar month (commonly referred to as SCPS1), and

AVG(month)SCE1ParticipationFactor∗ΔF1(10∗Bias1)∗ε21≤1

(2)	The ten (10) minute averages of the QSE’s SCE meet the following criteria for ninety percent (90%) of the ten (10) minute periods over the calendar month (commonly referred to as SCPS2).

SCE10≤L10∗KParticipationFactor

Where:

SCE1 	is the one minute average of SCE.

SCE10	is the ten minute average of SCE.

Bias1 	is the one minute average of the ERCOT total bias used in the ACE calculation.

F1 	is the one minute average of frequency deviation from schedule.

Participation Factor	is determined by the ratio of the QSE’s generation scheduled change for the measurement period (1 or 10 minute) to total ERCOT generation schedule change for the measurement period (1 or 10 minute).  Generation schedule change per interval is defined as below:

{Absolute Value

[     (ResourceSchedule – ResourceSchedulePreviousInterval)

+ (BalancingDeployment – BalancingDeploymentPreviousInterval)  ]

+ RegulationUpSchedule

+ RegulationDownSchedule}

If this Participation Factor Calculation results in a value of less than 1%, then 1% will be used.

 	is a constant derived from the targeted frequency bound.  It is the targeted root-mean score of one (1) minute average frequency error from a schedule based on frequency performance over a given year as established according to NERC Performance Requirements by ERCOT and the appropriate ERCOT Subcommittee as assigned by TAC.

L10 	is a limit to recognize the desired performance of frequency for ERCOT as established according to NERC Performance Requirements by the appropriate ERCOT Subcommittee assigned by TAC.  As of July 2003, L10 is defined as (1.65 * E10 * 10 * Bias10) where E10 is 0.01315 Hz and Bias10 is the ten (10) minute average of the ERCOT total bias used in the ACE calculation.

K	is a constant currently set to .81 which is established by the appropriate ERCOT Subcommittee as assigned by TAC.  K should initially be set to .81 to provide an ERCOT wide L10 equivalent to the ERCOT wide L10 currently used by Control Areas in ERCOT.  This constant can be adjusted to ensure correlation between passing the NERC CPS2 criteria and passing the SCE ten (10) minute control limit.

6.10.12	Non-Compliance Actions of ERCOT

ERCOT may revoke any or all Ancillary Service qualification of any QSE providing an Ancillary Service(s) for continued under-performance.

Failure to deliver energy resulting from a valid Dispatch Instruction is cause for ERCOT, at ERCOT’s option, to withhold payment for any Ancillary Services purchased and not delivered.

ERCOT will make information relative to each QSE’s performance as well as ERCOT’s performance, available to the marketplace on a monthly basis, subject to the provisions of Section 1.3.1, Restrictions on Protected Information.

6.10.12.1	SCE Enforcement Criteria

If a QSE falls below the acceptable compliance performance level for the month being analyzed, ERCOT will limit the QSE’s ability to sell Regulation Services for all hours of the day of the next whole month to its daily average schedule determined from the QSE’s Regulation Service schedules submitted to ERCOT for the month being analyzed.  In addition, ERCOT will allocate Enforcement Regulation Service (ERS) for the next whole month during the hours ending 0500, 0600, and 2200 in a quantity as described below.  Such allocated ERS may not be self-provided from the QSE’s own Generation Resources.  The Regulation Services limitation and the ERS allocation will remain in place for two (2) consecutive months.  ERCOT will communicate such Regulation Services limitations and ERS allocation to the QSE by certified mail.  It will be the responsibility of the QSE to ensure compliance with such limitation and allocation.

The ERS quantity will be determined as follows:

ERS = the greater of: 	(i)	(90% - QSE Performance level) * 25 MW; and

(ii)	(90% - QSE Performance level)/2 * (its daily average determined from the QSE’s Regulation Service schedules submitted to ERCOT).

Where:

ERS		Enforcement Regulation Service

90%		The minimum acceptable SCE compliance performance level.

If a QSE falls below the acceptable SCE performance level during the first month of the enforcement period, ERCOT will maintain the existing Regulation Services limitation and ERS, but increase the ERS amount by an incremental ten percent (10%) or five (5) MW, whichever is larger, for the next month.

If a QSE falls above the acceptable SCE performance level during the first month of the enforcement period, ERCOT will decrease the ERS level by a factor of fifty percent (50%).  If a QSE falls above the acceptable SCE performance level during the second month of the enforcement period, ERCOT will remove all Regulation Services limitations and ERS allocations.

A QSE may request that certain hours, up to a maximum of three (3) hours, be classified as exempted hours for purposes of SCE performance analysis.  The requesting QSE agrees to provide ERCOT incremental Regulation Service during the exempted hours in a quantity as determined by RSME.

RSME = 50% (SCE excess hourly average)

Where:

RSME	The incremental Regulation Service provided for exempted hours

SCE excess hourly average	The total MWh outside of the allowable bandwidth for the month for the exempted hours

ERCOT will reduce the ERCOT Load obligation for hours ending 0500, 0600, and 2200 by an amount equal to the sum of all ERS allocated to all QSEs.