A

## Accumulated Inadvertent

The accumulation over time of inadvertent energy.  The Accumulated Inadvertent account of a Control Area represents the amount of excess energy a Control Area has either supplied to the Interconnection or absorbed from the Interconnection.  Control Areas monitor their Accumulated Inadvertent to ensure the accounts do not grow to large.  If an Accumulated Inadvertent account becomes too large, a Control Area will arrange for a payback.

## ACSR

Acronym for Aluminum Conductor Steel Reinforced.  ACSR is a common type of transmission line conductor that comes in many different sizes and designs.

## Active Power

The component of the complex power that performs the work.  The common unit of Active Power is the MW and the symbol is “P”.

## Actual Net Interchange

The sum of the actual active power flows on all of a Control Area’s tie-lines.

## Adjusted Metered Load

Retail Load usage data that has been adjusted for UFE and transmission and/or distribution losses.

## Adjustment Period

The Adjustment Period for any given Operating Hour is the time period following the close of the Day-Ahead market and extending up to each Operating Period.

## AGC Pulses

The Automatic Generation Control (AGC) system in ERCOT will send signals to Qualified Scheduling Entities (QSEs), whose AGC will send signals to selected generators to adjust their set-points.  These signals are sent by telecommunication lines.  The signals are often called Pulses.

## Advisory

The second of four possible levels of communication issued by ERCOT in anticipation of a possible emergency condition detailed in Protocols.

## Affiliate

(a)	An Entity who directly or indirectly owns or holds at least five percent of the voting securities of another Entity; or

(b)	An Entity in a chain of successive ownership of at least five percent of the voting securities of another Entity; or

(c)	An Entity that has at least five percent of its voting securities owned or controlled, directly or indirectly, by another Entity; or

(d)	An Entity that has at least five percent of its voting securities owned or controlled, directly or indirectly, by an Entity who directly or indirectly owns or controls at least five percent of the voting securities of another Entity or an Entity in a chain of successive ownership of at least five percent of the voting securities of another Entity; or

(e)	A person who is an officer or director of another entity or of a corporation in a chain of successive ownership of at least five percent of the voting securities of an Entity; or

(f)	An Entity that actually exercises substantial influence or control over the policies and actions of another Entity; or

(g)	Any other Entity determined by the PUCT to be an Affiliate.

## Aggregated Retail Load Data

Adjusted Metered Load that has been aggregated as defined in Protocols Section on Aggregate Retail Load Data.

## Aggregated Unit

A single plant or facility containing two or more individual generation units that require tandem operation for optimal performance (e.g. a combined cycle plant) which ERCOT has agreed to treat as a single unit for settlement purposes only as set forth in the Protocols Section on Aggregating Units.

## Agreement

Standard form Agreement executed between ERCOT and Market Participants.

## Air-Gap

The air space between the rotor and stator of a motor or generator.

## Alert

The third of four possible levels of communication issued by ERCOT in anticipation of a possible emergency condition detailed in Section 5.6, Emergency and Short Supply Operation.

## Alternating Current (AC)

In an AC system the current and voltage magnitudes constantly vary or alternate.  Current and voltage magnitudes do not alternate in a DC system.

## Alternative Dispute Resolution

Procedures outlined in the Protocols for settling disputes by means other than litigation.

## Alternator

A rotating machine in which the output is an alternating voltage and current.

## Ampere

The unit of measurement for current flow.

## Amplitude

The value or magnitude that a waveform has at a specific point in time.

## Analog Electronics

Electronic circuitry in which the magnitudes of quantities are used in a continuous manner to perform functions.  For example, input current values may be amplified and used to perform work in an analog circuit.  Analog is different than digital in the sense that Analog circuits use voltage and current magnitudes continuously while digital circuits use voltage and current to determine the state (on or off) of elements.

## Ancillary Service

Those services necessary to support the transmission of energy from resources to loads while maintaining reliable operation of the transmission provider’s transmission systems in accordance with good utility practice.

## Ancillary Services Plan

ERCOT produced plan that identifies the types and amount of Ancillary Services necessary for each hour of the next day, or next two (2) days, to operate the ERCOT Transmission Grid reliably, and which includes the allocation of types and amounts of the Ancillary Services Obligations for each QSE.

## Annual Transmission Planning Report

A report prepared by ERCOT, as required by the PUCT rules, regarding the status of the ERCOT System including identification of ERCOT System existing and potential Congestion, which includes identification of current and recommended construction of Transmission Facilities.

## Arc

The discharge of current through the air or in a gas.

## Arcing Horns

An enhancement to a disconnect switch to increase the switches current interrupting capability.

## Area Control Error (ACE)

ACE is the acronym for the Area Control Error.  ACE is the generation error of a Control Area that is synchronously tied to adjacent Control Areas.  ACE is the instantaneous difference between the actual and scheduled interchange biased by the effects of the difference between the actual and scheduled frequency.

## Armature

The component of a machine in which the voltage is induced.  In a synchronous generator, the armature is always the stator.  The Armature winding is wrapped about the armature.

## Asynchronous

To be out-of-step with a reference.  An induction generator is an Asynchronous generator as it does not rotate in synchronism with the power system.

## Automatic Generation Control (AGC)

ERCOT as the Control Area and applicable QSEs have an AGC system.  ERCOT’s AGC assesses system reliability as a function of frequency and electronically sends megawatt values to each involved QSE as if it were a single generating unit.  The QSE’s AGC then performs an economic assessment of available units in their portfolio and pulses those units to adjust their set point, thus supporting the reliability requirements of the system and also addressing the economic requirements of the QSE.

## Automatic Voltage Regulator (AVR)

A device used on Generation Resources to automatically maintain a voltage set point.

## Autotransformer

A transformer with a single coil.  The entire coil acts as the primary winding while a portion of the same coil acts as the secondary winding.

## Auxiliary Relay

A relay whose function is to supplement the actions of other types of relays.  For example, a lockout relay is an auxiliary relay with numerous contacts that each can perform an action.

## Available Transfer Capability (ATC)

A measure of the transfer capability remaining in the transmission system for further commercial activity.  The ATC between two areas provides an indication of the amount of additional power that can be transferred from one area to another for a specified time for a specific set of conditions.

## Availability Plan

An hourly representation of availability of RMR, Synchronous Condenser and/or Black Start Resources submitted to ERCOT by Entities with RMR, Synchronous Condenser and/or Black Start Resources by 0600 in the Day Ahead Period.

## Average Daily Usage

The ratio of the total consumption divided by the number of days the consumption covered. (ADU = Monthly kWh/# of days).

B

## Back EMF

When AC current flows through a conductor, an alternating magnetic field is created.  This alternating magnetic field induces a voltage in the conductor.  The induced voltage is called the conductor’s Back Electro-motive Force (EMF).

## Back-to-Back HVDC

A High Voltage Direct Current (HVDC) system in which Alternating Current (AC) is converted to Direct Current (DC) and then immediately back to AC.  The HVDC transmission path is very short, normally consisting of a short section of bus work.

## Balanced Schedule

An Energy and Ancillary Service schedule submitted to ERCOT by a Qualified Scheduling Entity that consists of projected interval Obligations and projected interval Supply, and that includes Qualified Scheduling Entity Obligations for Transmission and Distribution Losses.  A Balanced Schedule must have projected aggregate Supply equal to projected aggregate Obligations, by Settlement Interval.

## Balancing Authority

An Entity that integrates Resource Plans ahead of time and maintains load-interchange-generation balance within its metered boundary and supports frequency real time.

## Balancing Energy

Balancing Energy represents the change in zonal energy output or demand determined by ERCOT to be needed to ensure secure operation of ERCOT Transmission Grid, and supplied by the ERCOT through deployment of bid Resources to meet Load variations not covered by Regulation service.

## Bandwidth

The acceptable range of a quantity.  For example, if it is desired to hold the voltage at a 345 KV bus between 355 and 360 KV then the bandwidth is 355 - 360 KV or five (5)KV.

## Bankrupt

The condition of an Entity when such Entity (i) files a petition or otherwise commences a proceeding under any bankruptcy, insolvency, reorganization or similar law, or has any such petition filed or commenced against it, (ii) makes an assignment or any general arrangement for the benefit of creditors, (iii) has a liquidator, administrator, receiver, trustee, conservator or similar official appointed with respect to it or any substantial portion of its property or assets, or (iv) is generally unable to pay its debts as they fall due.

## Baseload Generating Units

Generators that normally run continuously to serve load.

## Bid Stack

Bids received for Ancillary Services organized from lowest price to highest price bid for the same service and time interval.

## Black Start Plan

A written set of operating procedures and guidelines for restoring the power system following widespread outages.  The ERCOT Control Area and each TDSP have Black Start Plans.

## Black Start Service

A contracted Ancillary Service acquired by ERCOT for the benefit of all Loads, provided by Resource capable of starting without support of the ERCOT Transmission Grid.

## Black Start Resource

A single Resource capable of providing Black Start Service.

## Blade

The component of a steam turbine upon which the steam impacts.  Blades are also called “buckets”.

## Blocked Governor

A governor control system that has somehow been blocked or prevented from responding to a frequency deviation.  There are many ways to block a governor.

## Block Load Transfer

A transfer scheme which isolates a group of loads from the Control Area in which they normally are served and subsequently interconnects them with an adjacent Control Area.  Such transfer schemes involve either transferring loads normally in ERCOT to a Non-ERCOT Control Area or transferring loads normally in Non-ERCOT Control Areas to the ERCOT Control Area.  Block Load Transfers specifically exclude transfers of load between ERCOT and Non-ERCOT Control Areas that occur behind a retail settlement meter.

## Boiler

The component of a steam power plant in which the steam is created.

## Boiler Follow

A mode of operation of a fossil unit’s turbine/boiler control system in which the unit’s turbine is allowed to immediately respond to a load (MW) change request while the boiler response “follows” with the resulting temperature and pressure swings.

## Boundary Generation Resources

Those Resources, identified prior to ERCOT’s annual Transmission Congestion Rights (TCR) auction, on a unit-specific basis, that are related to a combination of Commercially Significant Constraint (CSC) and Closely Related Element (CRE), and whose impacts on the CRE are opposite to those on the related CSC.

## Brush

A sliding contact, usually made of carbon, located between the excitation current source and the rotor field winding leads of a synchronous generator.

## Bulk Electric System

The aggregate of electric generating plants, transmission lines, and related equipment.  The term may refer to those facilities within on electric utility, or within a group of utilities in which the transmission facilities are interconnected.

## Business Day

Monday through Friday, excluding ERCOT observed holidays listed below:

(1)	New Year’s Day

(2)	Memorial Day

(3)	Independence Day

(4)	Labor Day

(5)	Thanksgiving Thursday and Friday

(6)	Two (2) days at Christmas, as designated by the ERCOT CEO

## Bank Business Day

Any day during which the United States Federal Reserve Bank of New York is open for normal business activity.

## Retail Business Day

Same as above except in the case of retail transactions processed by a Transmission and/or Distribution Service Provider (TDSP), Competitive Retailer (CRs) shall substitute TDSP holidays for ERCOT holidays when determining the time available to the TDSP to process the transaction.   For additional, important information related to Retail Business Days, please refer to the Retail Market Guide.

## Business Hours

8:00 A.M. to 5:00 P.M. Central Prevailing Time on Business Days.

C

## Capacitance

The property of an electrical circuit that opposes voltage changes by storing energy in its electric field.  The symbol for capacitance is “C” and the unit is the Farad (symbol “f”).  All energized equipment has a natural capacitance.

## Capacitive Load

A load that supplies lagging reactive power to the system.

## Capacitive Reactance

The opposition that capacitance provides to AC current.  The capacitive reactance (XC) in a 60 HZ circuit is:

## Capacitively Coupled Voltage Transformer (CCVT)

An instrument transformer that is similar in function to a potential transformer (PT).  A capacitive voltage divider circuit is used in a CCVT to reduce power system voltage magnitudes to usable (110 Volts) levels.  Capacitive Voltage Transformers (CVTs), Coupling Capacitor Potential Devices (CCPDs), and Bushing Potential Devices (BPDs) are similar in design and function to a CCVT.

## Capacitor

A device intentionally designed to act as a capacitor and store energy in its electric field.  A shunt Capacitor acts as a source of reactive power to the system.  Series Capacitors are rarely seen devices that are used to reduce the inductive reactance of a transmission path.

## Capacity

The rated continuous load carrying ability (expressed in MW or MVA) of generating equipment or other electrical apparatus.

## Capital Asset

An asset expected to have a useful life of at least twelve (12) months that is not bought or sold in the normal course of business.

## Capital Expenditure

An outlay of money to acquire a Capital Asset or extend the useful life of an existing Capital Asset for at least twelve (12) months.

## Central Prevailing Time

Either Central Standard Time or Central daylight time as established by national time standards.

## Certificate of Compliance

A certificate issued by ERCOT stating that the Metering Facilities referred to in the certificate satisfy the certification criteria for Metering Facilities contained in these Protocols.

## Centrifugal Ballhead Governor

A mechanical governor that uses rotating flyweights to sense shaft speed.  A very common governor due to its durability and accuracy.

## Certification

To endorse authoritatively as being of a standard quality.

## Charge

An atomic force.  An electron has a negative charge.  A proton has a positive Charge.  Like Charges repel one another wile opposite Charges attract one another.  Current is the flow of Charge.

## Charging Current

The leading current that flows into the natural capacitance of a transmission line when it is first energized.

## Check Meter

A redundant revenue quality meter, which produces equal or better accuracy than the primary revenue quality meter, connected at the same metering point, which must be certified in accordance with the ERCOT Protocols.

## Circadian Rhythms

The name given to the roughly 24-hour cycles our bodies go through managing  physiological processes.  Circadian rhythms are important in determining sleep and eating patterns.

## Circuit Breaker (CB)

A piece of power system equipment that is used to disconnect other equipment from the power system.  Circuit Breakers are grouped according to their insulating medium.  For example, Oil Circuit Breakers (OCB), Air Circuit Breaker (ACB), etc.

## Circuit Switcher (CS)

A device that is similar in function to a Circuit Breaker.  A Circuit Switcher will contain an interrupting device (typically gas based) to enhance its current interrupting ability.

## Closely Related Elements (CREs)

Those transmission facilities that have shift factor impacts similar to those associated with a particular Commercially Significant Constraint (CSC), and for which there exists a limited amount of Boundary Generation Resources between it and the particular CSC, so that the zonal deployment of Balancing Energy Service is effective in mitigating Zonal Congestion.

## Cold Load Pick-Up

The increase in a load’s magnitude during the initial period (approximately 30 minutes) after it is energized.

## Commercial Model

Transmission model developed by ERCOT that arranges groups of Generation Resource and Load busses into Congestion Zones that have similar impacts on Commercially Significant Constraints.

## Commercially Significant Constraint (CSC)

A constraint in the ERCOT Transmission Grid that is found, through the process described in Section 7, to result in Congestion which limits the free flow of energy within the ERCOT market to a commercially significant degree.

## Commercially Significant Constraint (CSC) Limit

The maximum power flow across a CSC allowed to maintain reliable operation.

## Competitive Retailer (CR)

Municipally Owned Utility or an Electric Cooperative that offers customer choice and sells electric energy at retail in the restructured electric power market in Texas; or a Retail Electric Provider (REP) as defined in 25.5 of the PUCT Substantive rules.

## Competitive Retailer (CR) of Record

The Competitive Retailer assigned to the ESI ID in ERCOT's database.  There can be no more than one CR of Record assigned to an ESI ID for any given time period.

## Complex Power

The vector sum of the active and reactive power.  The common unit for Complex Power is MVA and the symbol is “S”.

## Compliance Monitor

An Entity that performs the functions of reviewing and ensuring compliance with the NERC Reliability Policies and Standards and administers sanctions or penalties for noncompliance to Standards.

## Compressor

A machine that increases the pressure of a gas (typically air) or vapor.

## Condenser

The component of a thermal power plant in which the steam is cooled to water after exiting the turbine.

## Conductor

A material with a low impedance to current flow.  A conductor is the opposite of an insulator.

## Congestion

The situation that exists when requests for power transfers across a Transmission Facility element or set of elements, when netted, exceed the transfer capability of such elements.

## Congestion Zone

A grouping of busses that create a similar Shift Factor on CSCs.

## Constant Frequency Control

An operating mode of an AGC system.  While in Constant Frequency Control, an AGC system will monitor only the frequency error to determine Resource adjustments needed to balance sources and obligations.

## Constant Net Interchange Control

An operating mode of an AGC system.  While in Constant Net Interchange Control an AGC system will determine the ACE value by considering only the interchange error.

## Contingency

An event that is possible, but uncertain as to whether it will occur.  For example, the loss of a transmission line or the loss of a generator is a Contingency.  First Contingency refers to the loss of one unique element.  Second Contingency refers to the loss of a subsequent unique element.  A power system may be designed to withstand all possible first Contingencies and not collapse or harm neighboring systems.

## Contingency Reserve

A portion of the Operating Reserves that is available following the loss of Resources to meet the Disturbance Control Standard.

## Continuous Service Agreement (CSA)

An arrangement between the owner or controller of a leased Premise and a CR wherein the CR provides service to the leased Premise between tenants so that the Premise does not experience discontinuation of electric service during vacancy.

## Control Area

An electrical system, bound by interconnect (tie-line) metering and telemetry, which continuously regulates, through automatic Resource control, its Resource(s) and interchange schedules to match its system load, regulates frequency, and meets all applicable Control Area requirements.

## Control Area Operator

An individual or set of individuals responsible for monitoring and control operation of the Control Area.

## Control Center

A location that is manned 24 hours a day, 7 days a week, which regulates, through automatic or manual intervention, system frequency and voltage, and the servicing of system loads.  Regulation may be through management of resources, loads, or lines and equipment.

## Control System

A collection of electrical and mechanical components designed to produce a series of outputs based on a series of measured inputs.  Common Control Systems related to power are excitation control and governor control systems.

## Coordinated Control

A mode of operation of a fossil unit’s turbine/boiler control system in which the operation of the boiler and the turbine systems are coordinated.  The coordination balances the pressure and temperature limits of the boiler system with the desire for a turbine to immediately respond to load (MW) change requests.

## Core

The material used within a transformer.  A transformer’s Core is formed of a magnetic material such as iron.  The purpose of the Core is to confine the magnetic field to a target area.

## Corona

A condition that occurs on energized equipment when the surface potential (voltage) is so large that the dielectric strength of the surrounding air breaks down (ionization occurs).  Symptoms of Corona are a visible ring of light and a hissing sound.  Corona is undesirable, as energy losses are a consequence.  Corona rings are used to reduce the gradient of the electric field and reduce the likelihood of Corona occurring.

## Cosine

The Cosine of either of the unknown angles of a right triangle is the ratio of the side adjacent to the unknown angle to the hypotenuse.

## Current

The rate of flow of electrical charge through a conducting path.

## Current System Conditions

The Real-Time status of the ERCOT System, which may affect ERCOT’s operational decisions.

## Current Transformer (CT)

A low power transformer (an instrument transformer) used to reduce normally high power system current magnitudes to lower values (about 5 amps).  A CT has a low number of turns on the primary winding and a high number of turns on the secondary winding.  The low magnitude secondary currents of a CT are typically input to protective relays, meters, etc.

## Curtailment of Service

The interruption of transmission service in an emergency situation when necessary to preserve the stability of the transmission network and service to customers.

## Customer

An Entity that purchases electricity for its own consumption.

## Customer Choice

The freedom of a retail Customer to purchase electric services, either individually or on an aggregated basis with other retail Customers, from the provider or providers of the Customer’s choice and to choose among various fuel types, energy efficiency programs, and renewable power suppliers.

## Customer Registration Database

The database maintained by the Registration Agent containing information identifying each Premise, including current and previous Competitive Retailers serving the Premise.

## Cycle

A complete sequence of a repeating waveform.  For example, the magnitude of a sine wave changes as the degrees progress from 0 to 360.  After 360 the sinewave begins to repeat itself.  One Cycle of a sinewave is therefore 360.

D

## Data Aggregation

The process of netting, grouping and summing Load consumption data, applying appropriate profiles, Transmission Loss Factors, and Distribution Loss Factors and calculating and allocating UFE to determine each QSE and/or Load Serving Entities responsibility by Settlement Interval by Congestion Zone and by other prescribed aggregation determinants.

## Data Aggregation System

The database and communication system that will collect meter data from TDSPs and directly polled meters in ERCOT. The system will perform aggregation functions to the Load data in order to satisfy certain objectives such as providing TDSPs with Load share data to use in billing Competitive Retailers, assigning QSE Load responsibility, and assisting Competitive Retailers and QSEs in their settlement responsibilities. The data will also be compiled along Congestion and weather zones.

## Data Archive

An integrated normalized data structure of all the target source systems' transactions.  The population of the data archive will be an extraction of data from the transaction systems without transforming the data. The Data Archive will be used to populate the Data Warehouse.

## Data Warehouse

De-normalized data stored in a schema, physically optimized to handle high volumes of data and concurrent user access, and is generally lightly indexed.

## Day Ahead

The twenty-four (24) hour period prior to the beginning of the Operating Day.

## Deadband

The distance about a target value within which a control system will not respond.  For example, a governor control system may have a Deadband of 0.03 HZ.  Unless the frequency deviation exceeds 0.03 HZ, the governor will not respond.

## Delivery Plan

A plan by ERCOT containing the hours and levels of operation that an RMR Unit, including Synchronous Condenser Unit, is instructed to operate.

## Delta Connection

A 3 electrical connection in which the three phases are connected in series with one another.  The three phases form a closed triangle.  Transformer windings are often connected in a Delta configuration.

## Demand

Instantaneous or integrated power consumption

## Deviation

The variance from the scheduled value.  For example, if the scheduled value of voltage is 142 KV and the actual value is 138 KV then the voltage Deviation is -4 KV.

## Dielectric

Insulating material used to separate and insulate.  Capacitors use a Dielectric between the two conducting plates.

## Differential Relay

A protective relay that responds to the difference between the currents entering and leaving the protected zone of the Relay.  Differential Relays are typically applied to transformers, substation buses, and generators.

## Digital Electronics

Electronic circuitry that operates on data in the form of digits.  Most Digital circuits use the binary system in which digital components are either turned on (a data value of “1”) or off (a data value of “0”) based on circuit voltage magnitudes.  Digital electronics have replaced analog electronics in most, but not all, applications.

## Direct Current (DC)

In a DC system, the current and voltage magnitudes are constant.  Current and voltage magnitudes alternate in an AC system.

## Direct Current Tie, DC Tie

Any non-synchronous transmission interconnections between ERCOT and non-ERCOT electric power systems.

## Direct Load Control

Controlling select end-use equipment (e.g. A/C, water heaters) for purposes of reducing or increasing energy consumption during select periods.

## Disconnect Switch

A mechanical device that is used to isolate equipment from energized parts of the power system.  Disconnect Switches are not rated for interrupting current unless additional arc interrupting components are added to the disconnect.

## Dispatch

The act of issuing Dispatch Instructions.

## Dispatch Instruction(s)

Specific command(s) issued by ERCOT to QSEs or TDSPs during the course of operating the ERCOT System.

## Distance Relay

A protective relay that activates if the ratio of the measured voltage divided by the current (the impedance) falls below a pickup point.  A Distance Relay is also commonly called an Impedance Relay.

## Distribution Losses

The difference between the energy delivered to the Distribution System and the energy consumed by Loads connected to the Distribution System.

## Distribution Loss Factor

The ratio of the Distribution Service Provider’s estimated Distribution Losses to the total amount of energy deemed consumed (IDR plus profiled consumption) on the Distribution Service Provider’s system.

## Distribution Service Provider

An Entity that owns and maintains a Distribution System for the delivery of energy from the ERCOT Transmission Grid to the Customer.

## Distribution System

That portion of an electric delivery system operating at under 60 kilovolts (kV) that provides electric service to customers or wholesale customers.

## Disturbance Control Standard (DCS)

A means to measure whether the responsible party, i.e. Control Area or Reserve Sharing Group, is maintaining sufficient contingency reserves so as not to jeopardize the reliability of the Interconnection following a “Reportable Disturbance”.

## Droop

A characteristic of a governor control system that requires a decrease in generator shaft speed to produce an increase in the generator’s MW output.  There are two types of Droop: Permanent, and Transient.

## Permanent Droop

Permanent Droop is the droop used to enable all generators (with active governors) to share in frequency regulation and to ensure a MW response in proportion to unit size.  Desired values of Permanent Droop are in the neighborhood of 5%.  The 5% Droop means that a 5% frequency change will result in the unit’s governor moving the fuel (steam, water, gas, etc.) valves across their full range.

## Transient Droop

A feature implemented in governor control systems to ensure generators do not enter into power oscillations following load changes.  Power oscillations may occur due to the inherent time delay between a request for a load change by the governor and the ability of a generator to actually deliver the load change.  This natural time delay could lead to excessive control action by the governor.  A condition of oscillation called “hunting” could result.  If a generator is on isochronous control, transient droop is a necessity.  The Transient Droop function or “compensation” dampens a governor’s initial response following a speed change.  The effect is temporary as opposed to a Permanent Droop function which is permanent.

## Droop Curve

A graphical method of representing the performance of a governor.  The horizontal axis is typically generator output while the vertical axis is system frequency.  When a governor with a % droop is plotted on such a curve the plot droops from left to right with increasing generator output.

## DUNS Number

A unique nine-digit common company identifier used in electronic commerce transactions.

## Dynamic Schedule

A real time telemetered signal to ERCOT derived from an actual metered Load that represents an energy obligation and Resource in a QSE schedule.

E

## ECAR

Acronym for the East Central Area Reliability Coordination Agreement.  ECAR is one of the ten NERC Regional Reliability Councils.

## Economic Dispatch

The process for reducing the production cost of the generation supply to the lowest possible value.

## Electric Cooperative

(a)	A corporation organized under Chapter 161, Texas Utilities Code, or a predecessor statute to Chapter 161 and operating under that chapter;

(b)	A corporation organized as an electric cooperative in a state other than Texas that has obtained a certificate of authority to conduct affairs in the State of Texas; or

(c)	A successor to an electronic cooperative created before June 1, 1999, in accordance with a conversion plan approved by a vote of the members of the electric cooperative, regardless of whether the successor later purchases, acquires, merges with, or consolidates with other electric cooperatives.

## Electric Field

The invisible lines of force that surround an energized piece of equipment.  An Electric Field is created when a conductor is energized by a voltage.  Energy is stored in an Electric Field.

## Electric Reliability Council of Texas, Inc. (ERCOT)

A Texas nonprofit corporation that has been certified by the PUCT as the Independent Organization, as defined in §39.151 of PURA, for the ERCOT Region.

## Electric Service Identifier (ESI ID)

The basic identifier assigned to each Service Delivery Point used in the registration and settlement systems managed by ERCOT or another Independent Organization

## Electrical Circuit

An Electrical Circuit consists of a voltage source, a conducting path to a load, a load, and a return path from the load to the voltage source.  All of these ingredients are necessary for current to flow in the Electrical Circuit.

## Electrical Distance

The impedance of a transmission line is a measure of the Electrical Distance.  For example, if a line has a 100  impedance then 50  is ½ the line’s electrical length.  Impedance relays are often called distance relays in reference to the use of impedance as a measure of Electrical Distance.

## Electrical Energy

Energy of an electrical nature.  Generators convert the mechanical energy of a rotating turbine to the Electrical Energy that is transmitted across the power system.

## Electro-Hydraulic Control (EHC)

A form of a control system in which electrical devices are typically used to measure a quantity magnitude and hydraulics used to perform a control action.  For example, a governor control system may consist of a simple electrical machine that measures the shaft’s speed of rotation (frequency) and a hydraulic system that changes the positions of fuel valves.

## Electromagnet

Temporary magnet created by passing electric current through a coil.  The coil is typically wound about a magnetic core.

## Electromagnetic Induction

The creation of a voltage in a conductor due to a relative movement between the conductor and a magnetic field.  Electromagnetic Induction is the basic principle of operation of transformers and generators.

## Electromotive Force (EMF)

The voltage produced by a generator is called an Electromotive Force or EMF.  The symbol “E” is often used to indicate an EMF.

## Eligible Transmission Service Customer

A Transmission and/or Distribution Service Provider (for all uses of its Transmission System) or any electric utility, municipally owned utility, Electric Cooperative, Power Generation Company, Competitive Retailer, Retail Electric Provider, federal power marketing agency, exempt wholesale generator, qualifying facility, power marketer, or other person whom the Public Utility Commission of Texas has determined to be an Eligible Transmission Service Customer.

## Emergency Condition

That operating condition where the safety or reliability of the ERCOT System is compromised or threatened, as determined by ERCOT.

## Emergency Electric Curtailment Plan (EECP)

A plan which provides an orderly, predetermined procedure for maximizing use of available Resources and, only if necessary, curtailing demand during electric system emergencies while providing the maximum possible continuity of service and maintaining the integrity of the ERCOT System.

## Emergency Notice

The fourth of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition.

## Emergency Short Supply

The condition wherein ERCOT experiences an insufficient amount of bids in any Ancillary Services market.

## EMS

The acronym for Energy Management System.  An EMS system is a computer system used by a Control Area and QSEs in the generation control process.

## Energy

The power used over a period of time.  Electrical Energy is measured in watt-hours, kilowatt-hours (kWh), or Megawatt-hours (MWh).

## Energy Conversion

The conversion of energy from one form to another.  For example, a coal fired steam turbine/generator converts the coal’s chemical energy to thermal energy by burning the coal.  The thermal energy is then converted to mechanical energy by heating water and turning the turbine with steam.  The mechanical energy of the turbine is then converted to electrical energy via electromagnetic induction.

## Energy Ratio Share

A QSE’s ratio of Adjusted Metered Load plus Settlement Meter energy output from Uncontrollable Renewable Resources electing to utilize Renewable Production Potential for URC and OOME to total ERCOT Adjusted Metered Load plus Settlement Meter energy output from Uncontrollable Renewable Resources electing to utilize Renewable Production Potential for URC and OOME related to the appropriate interval.

## Engineering Studies

Studies performed by ERCOT for the purpose of studying, evaluating, or planning of the ERCOT System.

## Entity

Any natural person, partnership, municipal corporation, cooperative corporation, association, governmental subdivision, or public or private organization.

## ERCOT Board

The Board of Directors of the Electric Reliability Council of Texas.

## ERCOT CEO

ERCOT Chief Executive Officer.

## ERCOT Member

Any member of ERCOT that is a member in good standing in accordance with the ERCOT Bylaws.

## ERCOT Metered Entity

Any one of the following entities that meets the requirements of Section 10.2.3, ERCOT Polled Settlement Meters:

(1)	Any Generation connected directly to the transmission system;

(2)	Any Generation equal to or over 10MW;

(3)	Any Generation participating in any ancillary service market;

(4)	Non-opt-in Cooperatives and Municipality points of delivery over 10MW; or

(5)	Direct-Current ties (or interchanges with other control areas outside of ERCOT).

Additionally ERCOT will directly poll any generator or non-opt-in utility metering point at the request of the entity if the entity meets all requirements and certifications associated with EPS metering.

## ERCOT Polled Settlement (EPS) Meter

Any meter polled by ERCOT as defined in Section 10 for use in the financial settlement of the Market.

## ERCOT Region

An acronym for the Electrical Reliability Council of Texas.  The geographic area under the jurisdiction of the PUCT that is served by TDSPs that are not synchronously interconnected with electric utilities outside the state of Texas.

## ERCOT Operating Guides

Guidelines approved by the ERCOT Board describing the reliability standards for ERCOT.

## ERCOT Service Fee Schedule

Schedule of fees charged by ERCOT for various services provided to designated Entities, in accordance with these Protocols and/or as approved by the ERCOT Board, and as posted on the MIS.

## ERCOT System

The interconnected combination of generation, transmission, and distribution components in the ERCOT Region.

## ERCOT System Load

The sum of all HVDC interconnections and Generation Resources metered at the point of its interconnection with the ERCOT System at any given point in time.

## ERCOT Transmission Grid

All of those Transmission Facilities which are within the ERCOT Region.

## Excitation System

A generator control system used to control the production of reactive power.  The Excitation System’s main components are the voltage regulator and the exciter.

## Exciter

The DC power source for an excitation system.

## Exciting Current

The current drawn by a transformer to magnetize its core and supply the core losses.  The exciting current is typically 1-2% of the transformers full load current.

F

## Facilities

Equipment situated for the purpose of conducting service and/or business through use of the ERCOT System.

## Farad

The unit of capacitance.  The symbol for the Farad is “f”.

## Fault

An unintentional short-circuit in the power system.  A Fault may occur between two phases, between three phases, or between any number of phases and ground.

## FERC

Acronym for the Federal Energy Regulatory Commission.  The Federal government agency responsible for developing regulations that apply to the energy industry.

## Ferranti Rise Effect

Phenomena in which a transmission line—with one end closed and one end open—is exposed to its highest voltage magnitude at the open-end of the line.  The Ferranti Rise Effect is due to the absorption of a leading charging current when a transmission line is energized but open-ended.

## Final Day-Ahead Schedule

Those schedules ERCOT deems valid following the close of the Day Ahead period.

## Financing Persons

The lenders, security holders, investors, partners, multilateral institutions, and others providing financing or refinancing for the business of Entity, including development, construction, ownership, operation and/or maintenance of a Facility or any portion thereof, or any trustee or agent acting on behalf of any of the foregoing.

## Field Winding

The winding wrapped about the rotor of a synchronous machine.  DC excitation current is fed to the Field Winding to produce the rotor’s magnetic field.

## FRCC

The acronym for the Florida Reliability Coordinating Council.  The FRCC is one of the ten NERC Regional Reliability Councils.

## Flat Frequency Control

An alternative name for constant frequency control.

## Flat Tie-Line Control

An alternative name for constant net interchange control.

## Flyweights

The portion of a centrifugal ballhead governor that rotates.

## Force Majeure Event

Any event beyond the reasonable control of, and that occurs without the fault or negligence of, the Entity whose performance is prevented by the occurrence of such event.  Examples of such a Force Majeure Event include, but are not limited, to: an act of God, labor disturbance, act of the public enemy, war, insurrection, riot, fire, storm or flood, explosion, breakage or accident to machinery or equipment, or a curtailment, order, regulation or restriction imposed by governmental, military, or lawfully established civilian authorities.

## Forced Outage

A component failure or other condition that requires that the equipment be removed from service immediately or up to and including the next weekend.

## Frequency

The rate at which a repeating waveform repeats itself.  Frequency is measured in cycles per second or in Hertz (HZ).

## Frequency Bias ()

A value entered into the AGC system of a Control Area.  The Frequency Bias or “” (Greek letter “beta”) value approximates the frequency response characteristic (FRC) of the Control Area.

## Frequency Bias of Portfolio

A positive (+) value, in megawatts per 0.1Hz, to represent response of a QSE’s Resources to a deviation in frequency from scheduled frequency.

## Frequency Response Characteristic (FRC)

The MW response of the power system (or a section of the power system) to a frequency deviation.  The FRC is typically stated in terms of MW per 0.1 HZ.  For example, a Control Area may have a FRC of 200 MW/0.1 HZ.  This value of FRC indicates that for a frequency deviation of 0.1 HZ this Control Area would respond with 200 MW.  The FRC of a system will vary with changing system conditions.

## Fuel Index

An electronically-published index that reflects the price of fuel as determined by a fuel industry organization using available market information.

## Fuel Index Price

The FIP is the Midpoint price expressed in $/MMBTU, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day of OOME deployment.  The FIP for Saturdays, Sundays, holidays and other days for which there is no FIP published in Gas Daily, shall be the next published FIP after the day of OOME deployment.  In the event that the FIP is not published for more than two (2) days, the previous day published FIP will be used for initial settlement and the next day published FIP will be used for true up of the final settlement statement.

## Functional Model

Identifies and defines the functions Control Areas currently perform.  This does not depend on how organizations are, or will be, structured or how functions will be implemented in the future.  Consists of functions that ensure reliability and meet the needs of the marketplace.  In their purist or root level state, these functions are provided by Generators, Transmission Service Providers, Transmission Owners, Transmission Operators, Distribution Providers, Load Serving Entities, Purchasing-Selling Entities, Reliability Authorities, Planning Authorities, Balancing Authorities, Interchange Authorities, and the Compliance Monitor.

G

## Generation Capability for Daily Planning

This term is used with respect to individual generators or for a Control Area or QSE as a whole.  For a generating unit, the term refers to the net capability of the unit as verified in accordance with ERCOT Operating Guides.  For a Control Area or QSE, the term is the sum of the net capabilities of the generating units in the Control Area or QSE that are planned to be on-line for a given time or day.

## Generation Control

The process by which the generation supply is adjusted to both maintain system frequency and keep a close match between the actual tie-line flows and the scheduled tie-line flows.

## Generation Entity

Owner or controller of a Generation Resource used for generating electricity and electrically connected to the ERCOT System.

## Generation Re-Dispatch

The adjustment of generation source magnitude and/or location to achieve a specific goal.  For example, generation location and/or magnitude may to adjusted to relieve a transmission line overload.

## Generation Resources

Facilities that produce energy and that are owned or operated by a Generation Entity.

## Generator

Owns and operates generation unit(s) or runs a Market for generation products that performs the functions of supplying power and Interconnected Operations (Ancillary) Services.

## Good Utility Practice

Any of the practices, methods, and acts engaged in or approved by a significant portion of the electric industry during the relevant time period, or any of the practices, methods, and acts that, in the exercise of reasonable judgment in light of the facts known at the time the decision was made, could have been expected to accomplish the desired result at a reasonable cost consistent with good business practices, reliability, safety, and expedition. Good utility practice is not intended to be limited to the optimum practice, method, or act, to the exclusion of all others, but rather is intended to include acceptable practices, methods, and acts generally accepted in the region.

## Governmental Authority

Any Federal, state, local or municipal body having jurisdiction over a Market Participant or ERCOT; provided, however, a Governmental Authority who is also a Market Participant shall not exercise its jurisdiction in any matter that involves the interests of that Market Participant where that matter also involves the interests or responsibilities of any other Market Participant or ERCOT, unless the matter is one in which the Market Participant has exclusive jurisdiction.

## Governor Characteristic Curve

A graphical method of representing the performance of a governor.  The horizontal axis is typically generator output while the vertical axis is system frequency.  When a governor with a % droop is plotted on such a curve the plot droops from left to right with increasing generator output.

## Governor Control System

A control system for a generator that is used to control the speed of the generator’s rotating shaft.  In a steam-turbine generator, the Governor Control System controls the amount of steam striking the turbine blades.  In a hydroelectric turbine generator, the Governor Control System controls the amount of water striking the turbine blades.  Governor Control Systems are a key ingredient in maintaining a scheduled Interconnection frequency.

## Gross Generation

The output power (in MW) at the stator terminals of a generator.

H

## Heat Rate

An expression for the efficiency of a thermal power plant.  The Heat Rate is the amount of heat (measured in British Thermal Units or BTU) that is required to produce a kWh or electrical output.  The lower the Heat Rate the more efficient the power plant.

## Henry

The unit of inductance.  The symbol for a Henry is “H”.

## High Operating Limit (HOL)

The maximum net dependable capability of a Resource that may be delivered for a period of up to one hour.

## High Side Winding

The high voltage winding of the transformer.

## High Sustainable Limit (HSL)

The maximum net capability of a Resource that may be delivered for an indefinite period.

## Hour-Ahead Period

The Hour-Ahead Period occurs during the hour prior to the operating hour.

## HVDC

Acronym for High Voltage Direct Current.  The term HVDC is commonly used when the DC voltage is above 100 KV.

## Hydraulics

The use of fluid forces to perform work.  For example, hydraulics are often used in governor control systems to develop the large forces required to move steam or water valves.

## Hydro Responsive Reserve

The MW response capability of a hydroelectric unit that is operating in synchronous condenser fast response mode.  The MW capability must be available within 10 seconds following a system frequency decay.

## Hypotenuse

The side of a right triangle which is opposite the 90 angle.

I

## IDR Data Threshold

The percentage of Interval Data Recorder (IDR) data by a Meter Reading Entity (MRE) that must be available before ERCOT will calculate a True-Up Settlement.

## IDR Optional Removal Threshold

The kW (kVA) level at which an interval data recorder may be removed.

## IDR Mandatory Installation Threshold

The kW (kVA) level at which the installation of Interval Data Recorders are required for settlement purposes.

## IEEE

Acronym for the Institute of Electrical and Electronic Engineers.  The IEEE is an international standards organization that publishes guidelines for, among other areas, power systems.

## Impedance

The total opposition to the current flow in an electrical circuit.  The symbol for the Impedance is “Z”.  The Impedance includes the resistance (R), capacitance (C), and the inductance (L).

## Impedance Relay

A protective relay that activates if the ratio of the measured voltage divided by the current (the impedance) falls below a pickup point.  An Impedance Relay is also commonly called a Distance Relay.

## Impulse Turbine

A water turbine in which high velocity water is directed through nozzles at the Turbine buckets.  A Pelton Wheel is an example of an Impulse Turbine.

## Inadvertent Energy

Unscheduled energy flow between adjacent Control Areas.  When inadvertent interchange exists for a period of time, it will be accumulated.

## Inadvertent Energy Account

An account maintained by ERCOT to track any differences between deemed meter readings at the DC Ties, based on schedules, and the actual metered values at the DC Tie meters.

## Inadvertent Energy Payback

When the inadvertent energy that a Control Area accumulates exceeds a specified value, the Control Area must arrange for an Inadvertent Payback.  The ISO coordinates Inadvertent Energy Payback.

## Inadvertent Net Interchange

The difference between deemed meter readings at the DC Ties, based on schedules, and the actual metered values at the DC Tie meters.

## Incremental Cost

The cost associated with producing an additional MWh of energy from a generating unit.  Incremental Cost is typically stated in $/MWh or Mills/kWh.

## Incremental Losses

The increase in losses due to an increase in power flow.  For example, assume the power flow on a transmission line is initially 100 MW.  If the power flow is increased to 101 MW there will be Incremental energy Losses associated with the 1 MW increase in power flow.  The percentage of the Incremental Loss increases with increasing levels of power flow.

## Independent System Operator (ISO)

The designated coordinating authority within ERCOT.  The ISO is used by ERCOT member entities to request and receive information required to continually monitor the operating conditions of ERCOT.  The ISO will request that entity’s make changes which will ensure the total ERCOT system’s security and facilitate the efficient use of ERCOT transmission facilities.  The ISO is also a forum for coordinated system planning in ERCOT.

## Inductance

The property of an electrical circuit that opposes a change in current flow.  The symbol for Inductance is the letter “L” and the unit is the Henry (symbol “H”).

## Induction Machine

An AC machine that can be operated as a generator or as a motor.  When operated as a generator the Induction Machine’s rotor is driven at a speed greater than synchronous speed.  When operated as a motor the Induction Machine’s rotor is driven at a speed less than synchronous speed.  Induction generators are rarely used by large scale power generators.  Induction motors are the most common type of AC motor.  Induction Machines absorb reactive power (always a lagging load) and cannot be used to produce reactive power as a synchronous machine can.

## Inductive Load

A load that absorbs lagging reactive power from the system.

## Inductive Reactance

The opposition that inductance provides to AC current.  The Inductive Reactance (XL) in a 60 HZ circuit is:

## Inertia

The property of an object that resists changes to the motion of the object.  For example, the Inertia of a rotating object resists changes to the object’s speed of rotation.  The Inertia of a rotating object is a function of its mass, diameter, and speed of rotation.

## Inertial Energy

Energy stored within a rotating mass.  For example, a spinning generator contains Inertial Energy.  The terms Inertial Energy, stored energy, and rotational energy or often used interchangeably to refer to the energy stored in the rotating elements (motors and generators) of the power system.

## In-Rush Current

The sudden rush of current when a transformer or a motor is first energized.  The peak magnitudes of the In-Rush current last only a few cycles but can reach levels 8-10 times the device’s full load current.

## Instability

The inability to control grid response, with the threat of collapse.  When a power system loses stability, it enters a period of Instability.

## Installed Reserve

The difference between expected annual peak MW generation capability and annual peak MW load.  Installed Reserves are a rough approximation of spare or reserve generation.

## Instrument Transformers

A low power transformer classification.  Instrument Transformers include current transformers (CTs), potential transformers (PTs) and capacitive devices.

## Insulator

A material with a high impedance to current flow.  An Insulator is the opposite of a conductor.

## Inter-QSE Trade

Energy or Ancillary Services scheduled to or from other QSEs or ERCOT.

## Interchange

Energy either delivered or received by a Control Area.  Interchange schedules and/or flows are always made between Control Areas.

## Interchange Authority

An Entity that authorizes valid transactions and ensures balanced Interchange Schedules.

## Interconnect

When two power systems tie together with AC transmission it is referred to as an Interconnect.

## Interconnect Control Error (ICE)

ICE indicates the mismatch between generation and demand in an electrically isolated Control Area.  ICE is the biased effect of the difference between the actual and scheduled frequency.

## Interconnected Operations Services

Services necessary for the receipt, transfer, and delivery of a transaction, also known as Ancillary Services.

## Interconnection

A group of power systems connected together with AC transmission lines.  There are four major Interconnections in NERC; Eastern, Western, ERCOT and Quebec.  In addition, there are many other smaller Interconnections including Alaska and several in Hawaii.

## Interconnection Agreement

An agreement that sets forth requirements for physical connection between an Eligible Transmission Service Customer and Transmission and/or Distribution Service Providers.

## Interruptible Load

Customer load that can be interrupted by the supplying system in accordance with contractual provisions.

## Interruptible Responsive Reserve

Interruptible load controlled by high-set (59.7 HZ and above) underfrequency relaying.

## Interval Data Recorder (IDR)

Metering Device that is capable of recording Load usage in each Settlement Interval in accordance with Section 9, Settlement and Section 10, Metering.

## Island

An electrically isolated portion of an Interconnection.  An Islanded system maintains its own frequency.  Islands are frequently formed after major disturbances or during restoration following a major disturbance.

## Isochronous Governor Control

A governor operated with a 0% droop.  When in Isochronous Control, a governor will try to maintain 60 HZ.  Isochronous Control may be used during a system restoration.

## Isolated

To be electrically separated from the remainder of the Interconnection.  An Isolated system does not have transmission ties to the rest of the Interconnection.  An Isolated system is an electrical island.

## Invoice

## Settlement Invoice

A notice for payment or credit due rendered by ERCOT based on data contained in Initial, Final, True-Up or any Resettlement Statements.

## TCR Invoice

An invoice issued to a successful bidder based on a final round of a TCR auction.

## Invoice Recipient

Market Participants that receive an Invoice from ERCOT.

J

## K

## Kirchhoff’s Current Law

A basic electrical law that states that the sum of all the currents at any point in an electrical circuit equal zero amps.

## Kirchhoff’s Voltage Law

A basic electrical law that states that the sum of all the voltages around any closed electrical circuit equal zero volts.

L

## Lagging

Term used when comparing voltage and current waves.  The wave that is heading positive and crosses zero last is the Lagging wave.  In a Lagging load the current wave lags the voltage wave.

## Lambda

The incremental cost of generation.  Lambda is commonly expressed in $/MWh or Mils/kWh.  The symbol for Lambda is “” (the Greek letter lambda).

## Late Payment

Payments due to ERCOT by Invoice Recipients that are not received by the due date and time.

## Leading

Term used when comparing voltage and current waves.  The wave that is heading positive and crosses zero first is the Leading wave.  In a Leading load, the current wave leads the voltage wave.

## Load

The electric power delivered at any specified point or points on a system.

## Load Profile

A representation of the energy usage of a group of Customers, showing the demand variation on an hourly or sub-hourly basis.

## Load Profile Type

A classification of a group of Customers having similar energy usage patterns and that are assigned the same Load Profile.

## Load Profiling

The set of processes used for the development and creation of Load Profiles.

## Load Profiling Methodology

The fundamental basis on which Load Profiles are created. The implementation of a Load Profiling Methodology may require statistical sampling, engineering methods, econometric modeling, or other approaches.

## Load Ratio Share

A QSE’s ratio of Adjusted Metered Load to total ERCOT Adjusted Metered Load related to the appropriate interval.

## Load Reference Set-Point

In governor control systems, this setting determines the position of the controlled valve when the frequency is at the scheduled value.  From a system operations perspective, the Load Reference Set-Point is the MW a generator will produce when the frequency is 60 HZ.

## Load Serving Entity

An Entity that provides electric service to Customers and Wholesale Customers.  Load Serving Entities include Retail Electric Providers, Competitive Retailers, and Non-Opt-In Entities that serve load.

## Load Tap Changer (LTC)

A tap changer that is designed to change the turns ratio when the transformer has current flow across its windings.  The term Under Load Tap Changer (ULTC) is also used to refer to this type tap changer.

## Load/Frequency Relationship

The relationship between frequency deviations and the load magnitude.  In general, the load magnitude varies with the frequency.  If the frequency rises the load magnitude rises and vice versa.

## Local Congestion

Any Congestion that cannot be resolved by deployment of Balancing Energy Service by Congestion Zone.  Local Congestion will include those actions, and related costs, associated with mitigating overloads on CSCs or CREs beyond that which can be accomplished by the zonal deployment of BES.

## Location Code

The code representing the physical location of a Premise.

## Loss of Synchronism

The loss of the magnetic bond between elements of a power system.  Loss of Synchronism and out-of-step refer to the same concept.

## Losses

The energy losses in the power system.  The total system Losses consist of the transmission, transformation, and distribution system Losses.

## Low Operating Limit (LOL)

The minimum net capability of a Resource that may be delivered for a period of up to one hour.

## Low Side Winding

The low voltage winding of the transformer.

## Low Sustainable Limit (LSL)

The minimum net capability of a Resource that may be delivered for an indefinite period.

M

## MAAC

Acronym for the Mid-Atlantic Area Council.  MAAC is one of the ten NERC Regional Reliability Councils.

## Magnetic Field

The invisible lines of force between the north and south poles of a magnet.  A Magnetic Field is created when current flows through a conductor.  Energy is stored in a Magnetic Field.

## Magnetism

A property of matter associated with moving charges.  A material may be a permanent magnet or it may acquire magnetic characteristics due to current flow through the material.

## MAIN

Acronym for the Mid-America Interconnected Network.  MAIN is one of the ten NERC Regional Reliability Councils.

## Maintenance Outage

Removing transmission facility or Resource facility equipment from service to perform work on specific facility components that could be postponed past the very next weekend.  Such work is required to prevent a potential Forced Outage and that could not be postponed until the next Planned Outage.

## MAPP

Acronym for the Mid-Continent Area Power Pool.  MAPP is one of the ten NERC Regional Reliability Councils.

## Market Clearing Price for Capacity

The highest price associated with a Congestion Zone for a Settlement Interval for Ancillary Service capacity awarded in each Ancillary Services capacity procurement run by ERCOT. There will be a separate Market Clearing Price for Capacity for each Ancillary Services capacity market.

## Market Clearing Price for Energy

The highest price associated with a Congestion Zone for a Settlement Interval for Balancing Energy deployed during the Settlement Interval.

## Market Implementation Plan

Plan developed by ERCOT that addresses training, testing, qualification, and registration of Market Participants for participation in the Customer Choice Pilot, as well as the conversion to single Control Area operations.

## Market Information System (MIS)

An electronic communications interface established and maintained by ERCOT that provides a communications link to Market Participants, including secure access by and communications to a Market Participant regarding information linked to each individual Market Participant.

## Market Participant

An Entity that engages in an activity that is in whole or in part the subject of the Protocols, regardless of whether such Entity has executed an Agreement with ERCOT.

## Market Segment

The Segments defined in Article 2 of the ERCOT Bylaws.  The segments are:

(1)	Independent REPs,

(2)	Independent Generators,

(3)	Independent Power Marketers,

(4)	Investor Owned Utilities,

(5)	Municipals,

(6)	Cooperatives, and

(7)	Consumers.

## Market Solution

A Market Solution exists when at least three unaffiliated Resources, with capacity available, submit bids to ERCOT that can solve a circumstance of local Congestion and no one bidder is essential to solving the Congestion.

## Mass Drop

The immediate cessation of service by a CR to all ESI IDs served by the CR.

## Mechanical Energy

Energy of a mechanical nature,. e.g., a rotating mass possesses Mechanical Energy.

## Mechanical Input Power

Power input of a mechanical nature.  For example, a steam or water turbine input Mechanical Power to the rotor of a generator.

## Merit Order

The ranking of Resources as a direct function of the monetary bid from those resources.

## Messaging System

The ERCOT-to-QSE communications system used to send Real-Time notices and Dispatch Instructions to the QSEs.

## Meter Data Acquisition System

The system to obtain revenue quality meter data from ERCOT Polled Settlement meters and Settlement Quality Meter Data from the TDSP for settlement and to populate the Meter Data Aggregation System and ERCOT Data Archive.

## Meter Data Exchange Format

The format for submitting meter data to, or receiving data from, ERCOT Settlement Agent.

## Meter Data Request Format

The format for requesting Settlement Quality Meter Data from the ERCOT Settlement Agent.

## Meter Reading Entity (MRE)

An Entity that is responsible for providing ERCOT with ESI ID level consumption data as defined in Texas SET and Protocols Section 19.  This Entity must be a TDSP that is registered with ERCOT as prescribed in Protocols Section 10.  In the case of an ERCOT Polled Settlement (EPS) Meter or ERCOT populated ESI ID data (such as generation site load), ERCOT will be identified as the MRE in ERCOT systems.

## Metering

A device for measuring a quantity.  For example Meters are used to measure power flows, voltages, current, frequency, etc.

## Metering Facilities

Revenue Quality Meters, instrument transformers, secondary circuitry, secondary devices, meter data servers, related communication Facilities and other related local equipment intended to supply ERCOT settlement quality data.

## Microprocessor

An arithmetic, logic, and control unit all contained on one integrated circuit chip.  One Microprocessor will contain thousands of transistors.

## Mill

A unit of currency equal to 1/10 of a cent.

## Mismatched Schedule Processing Fee

The fee charged to a QSE that fails to correct a mismatched schedule in a timely manner.

## Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced retirement of the Generation Resource.

## Motor Load

A simplified grouping of all spinning type load.  Motor Load includes air conditioner compressors, motor drives, etc.  Total load is composed of Motor Load plus non-motor load.

## Municipally Owned Utility

A utility owned, operated, and controlled by a municipality or by a nonprofit corporation, the directors of which are appointed by one or more municipalities.

## Must-Run Alternative (MRA) Agreement

The contractual arrangement between ERCOT and a MRA Resource

## Must-Run Alternative (MRA) Resource

A Resource that was selected through the planning process, Exit Strategy from an RMR Agreement to provide steady-state or dynamic voltage support, stability or management of localized transmission constraints under first contingency criteria, as described in the Operating Guides, at a lower cost than an RMR Agreement.

N

## Natural Frequency

Every mechanical device has a natural frequency of oscillation.  For example, when a force is applied to a bridge the bridge will oscillate at its natural frequency.  Electrical circuits also have a natural frequency of oscillation.  An electrical circuits natural frequency is determined by its resistance, inductance, and capacitance.

## NERC

An acronym for the North American Electric Reliability Council.  The national organization that is responsible for establishing standard and policies for reliable electric system operations and planning, or its successor.

## NERC Control Performance Standards

The Control Performance Standards (CPS) are a set of generation control performance standards developed by NERC to ensure adequate generation control within the NERC systems.  The CPS is composed of CPS1, CPS2 and the Disturbance Control Standard (DCS).

## Net Dependable Capability

Maximum sustainable capability of a Generation Resource as demonstrated by performance testing.

## Net Generation

Gross generation minus station auxiliaries or other internal unit power requirements metered at or adjusted to the point of interconnection at the common switchyard.

## Net Interchange

The sum or net of all the interchange of a Control Area.  The term “net tie-line flow” is often used in place of Net Interchange.

## New Renewable Facilities

Renewable energy generators placed in service on or after September 1, 1999. A New Facility includes the incremental capacity and associated energy from an existing Renewable Facility through repowering activities undertaken on or after September 1, 1999.

## No Load Tap Changer (NLTC)

A tap changer that is designed to change the turns ratio only when the transformer has no current flow across its windings.  The term Off Load Tap Changer (OLTC) is also used to refer to this type tap changer.

## Nominal

The design or rated value.  Not necessarily the value that is intended or that occurs.  For example, the Nominal voltage for a piece of equipment would be the design or rated voltage but in operation, the equipment may be operated at a different value of voltage.

## Nominal Load

The rated or nameplate load.  For example, 100 MW of customer load may be fed from a utility bus.  This load will draw 100 MW if the voltage and frequency at the bus are at nominal values.  If voltage or frequency should vary then the actual load will be different from the Nominal Load.

## Non-Metered Load

Load that is not required to be metered by applicable distribution or transmission tariff.

## Non-Motor Load

A simplified grouping of all non-spinning type load.  Non-Motor Load includes resistive heaters, lighting, etc.  Total load is composed of motor load plus Non-Motor type Load.

## Non-Opt-In Entity (NOIE)

An Electric Cooperative or Municipally Owned Utility that does not offer Customer Choice.

## Non-Spinning Reserve

Off-line generation capacity capable of being synchronized and ramped to a specified output level within thirty (30) minutes (or Load that is capable of being interrupted within thirty (30) minutes) and that is capable of running (or being interrupted) at a specified output level for at least one (1) hour.  May also be from unloaded on-line capacity that meets the above response requirements and that is not participating in any other activity, including ERCOT markets, self-generation, and other energy transactions.

## Non-Spinning Reserve Service (NSRS)

An Ancillary Service that is provided through utilization of Non-Spinning Reserve.

## Normal Excitation

A condition in which the generator’s excitation system is supplying exactly the excitation current needed to maintain the magnetic field of the generator.  A Normally Excited generator is neither supplying nor absorbing reactive power from the system.

## North American Electric Reliability Council

The national organization that is responsible for establishing standards and policies for reliable electric system operations and planning, or its successor.

## Notice or Notification

Sending of information by an Entity to Market Participants, ERCOT, or others, as called for in the Protocols.  Notice or notification may be sent by electronic mail, facsimile transmission, or U.S. mail.

## NPCC

Acronym for the Northeast Power Coordinating Council.  NPCC is one of the ten NERC Regional Reliability Councils.

O

## Obligation

Total Obligations scheduled by a QSE that are comprised of Energy Obligations and Ancillary Services Obligations where:

## Energy Obligations =

Load + Losses + Energy sales + Energy exports; and

## Ancillary Services Obligations =

ERCOT allocated Ancillary Services Obligations (which may be self-arranged) + Ancillary Services sales (to ERCOT or to other QSEs)

## Off-line

The status of Resources that are not synchronously interconnected to the ERCOT System.

## Ohm

The unit of impedance.  The symbol for the Ohm is “” (Greek upper case letter Omega.)

## Ohm’s Law

A basic electrical law that relates the voltage, current, and impedance (V=IxZ).

## On-line

The status of Resources that are synchronously interconnected to the ERCOT System.

## Operating Condition Notice

The first of four possible levels of communication issued by ERCOT in anticipation of a possible emergency condition.

## Operating Day

The actual day, including hours ending 0100 to 2400, during which energy is flowing.

## Operating Guides

Guidelines approved by the ERCOT Board describing the reliability standards for ERCOT.

## Operating Hour

The current clock hour.

## Operating Period

A two-hour period comprised of the Operating Hour and the hour preceding the Operating Hour.

## Operating Reserve

The MW capability above system demand required to provide for frequency regulation, load forecasting error, and equipment forced outages.  Operating Reserve consists of spinning and non-spinning reserves.

## Operating Plan

A plan developed by ERCOT to operate the ERCOT System in Real Time.

## Operational Constraint

Anticipated or actual security violation or overload of a transmission element, based on actual network topology.

## Operational Model

The transmission model based on actual network topology of the ERCOT System.

## Oscillations

Cyclic variations in voltage, current, frequency, or power flows.  The voltage and current of the power system naturally oscillates at 60 HZ.  The term Oscillations is typically used to refer to low frequency (a few HZ) Oscillations that may occur.

## Out of Merit Capacity

Capacity provided by a Resource selected by ERCOT outside the bidding process to resolve a reliability or security event when no Market Solution exists.

## Out of Merit Energy

Energy provided by a Resource selected by ERCOT outside the bidding process to resolve a reliability or security event when no Market Solution exists.

## Out of Merit Order (OOM)

The selection of Resources for Ancillary Services that would otherwise not be selected to operate because of their place (or absence) in the bidding process for that service.

## Out-of-Step

To lose synchronism.  Out-of-Step is best viewed in terms of rotating magnetic fields.  When a generator is Out-of-Step with the power system, the generator’s rotating magnetic field is no longer in-step or in synchronism with the rotating magnetic field of the system.

## Outage

Removal of a Facility from service to perform maintenance, construction or repair on the Facility for a specified duration.

## Forced Outage

An Outage initiated by protective relay, or manually in response to an observation by personnel or the System Operator that the condition of equipment could lead to an event, or potential event, that poses a threat to people, equipment, or public safety.

## Maintenance Outage

Removing Transmission Facility or Resource Facility equipment from service to perform work on specific Facility components that could be postponed briefly.  Such work is required to prevent a potential Forced Outage and cannot be postponed until the next Planned Outage.  Such Maintenance Outages are classified as follows:

(1)	Level 1 Maintenance Outage – Equipment that must be removed from service within twenty-four (24) hours to prevent a potential Forced Outage;

(2)	Level II Maintenance Outage – Equipment that must be removed from service within seven (7) days to prevent a potential Forced Outage; and

(3)	Level III Maintenance Outage – Equipment that must be removed from service within thirty (30) days to prevent a potential Forced Outage.

## Planned Outage

Any major or minor Transmission Facility or Resource Facility equipment Outage, other than a defined Maintenance Outage, that is planned and scheduled in advance as submitted to ERCOT.

## Overcurrent Relay

A protective relay that activates in response to a high current magnitude.  Overcurrent Relays can be either timed or instantaneous and directional or non-directional.

## Overexcited

A generator is overexcited when the applied excitation is greater than that needed to support the generator’s magnetic field.  The excess reactive power produced by the overexcitation condition flows to the system.  An Overexcited generator supplies reactive power to the system.  The terms “lagging” and “boosting” are also used to refer to an Overexcited generator.

A transformer may also be Overexcited.  Overexcitation of a transformer results from either applying two high a voltage magnitude or two low of a frequency.

P

## Parallel Circuit

An electrical circuit in which all the positive terminals are connected to a common point.  All the negative terminals are connected to a second point.  The voltage drop is the same across each full element of the parallel circuit.

## Peaking Generating Units

Generators that are normally only run during the peak load periods of the load cycle.

## Penstock

A water pipe or conduit that carries water from the upper water reservoir to the turbine at a hydroelectric unit.

## Period

The time for one complete cycle of a repeating wave.  For example, a 60 HZ current steps through 60 cycles per second.  The Period of one complete cycle is therefore 1/60th of a second.

## Permanent Magnet Generator (PMG)

A simple generator that uses a permanent magnet (a material that retains its magnetism) to provide the required magnetic field.  PMGs are often part of governor control systems.  The PMG is used to produce an output voltage whose magnitude is representative of a generator’s shaft speed.

## Per-Unit

A system for reporting quantity magnitudes.  A Per-Unit number is stated in terms of a base quantity.  For example if the base voltage is 345 KV then a voltage of 359 KV is 1.04 Per-Unit (359/345=1.04).

## Phase

AC power systems use three conductors to efficiently generate and transmit large amounts of power.  Each of the conductors is called a Phase.  The Phases are each assigned a letter designation; “A”, “B”, and “C”.  Customer load can be connected as single-phase (1), two-phase (2), or three-phase (3).

## Phase Angle

The angle by which one waveform leads or lags another waveform.  A Phase Angle can exist between two voltages, two currents, or between a current and a voltage.

## Phasors

Similar to a vector but also includes information about the frequency of the quantity.  A Phasor diagram is a collection of lines that, like vectors, illustrate a quantity’s magnitude and direction.  However, Phasor diagrams must also specify the frequency of the quantity.  AC voltages and currents can be represented by Phasors.

## Pick-up Point

An operating setting for a protective relay that determines at what point the relay will activate.

## Pilot Relay

A protective relay system typically used to protect high voltage transmission lines.  Pilot Relays use telecommunication systems to communicate between the terminals of the transmission line.

## Planned Outage

Any major or minor transmission facility or Resource facility equipment maintenance that is planned and scheduled in advance as submitted to ERCOT.

## Planning Authority

Ensures a plan is available for adequate Resources and Transmission Facilities within its area of responsibility.  Integrates and assesses the plans from transmission planning functions and resource planning functions within its area of responsibility to ensure they meet NERC Reliability Standards and develop recommendations and solutions to plans that do not meet NERC Reliability Standards.

## Poles

The electrical circuits formed by the field winding on the rotor of a synchronous generator.  When DC current is passed through the field winding the Poles become magnetic north or south poles.

## Potential Difference

A difference in voltage magnitudes between two locations.  Current can flow along a closed path if a Potential Difference exists across the path.

## Potential Transformers (PT)

A low power transformer (an instrument transformer) used to reduce normally high power system voltages to low values (about 110 Volt).  A PT has a high number of turns on the primary winding and a low number of turns on the secondary winding.  The low magnitude secondary voltages of a PT are typically input to meters, relays, etc.

## Postage Stamp Allocation

The pro rata allocation of charges (or payments), which spreads to designated, Entities based on a pro rata share (of actual or estimated consumption).

## Power

The rate at which energy is expended to do work.  Power is measured in watts (W), kilowatts (kW), or Megawatts (MW).

## Power Angle

The phase angle between two voltage waveforms.  A Power Angle is the same as a voltage angle.  The Power Angle is a major factor in determining the amount of active power flow between two locations.

## Power Generation Company

An Entity registered by the PUCT that:  (1) generates electricity that is intended to be sold at wholesale;  (2) does not own a transmission or distribution Facility in this state other than an essential interconnecting Facility, a Facility not dedicated to public use, or a Facility otherwise excluded from the PURA definition of “electric utility;” and (3) does not have a certificated service area, although its affiliated electric utility or transmission and distribution utility may have a certificated service area.

## Power Factor

The ratio of the active power (MW) to the complex power (MVA).  The cosine of the phase angle between a load’s voltage and current is the Power Factor of the load.  A unity Power Factor load draws no reactive power, just active power.

## Power Marketer

An entity that:

- Becomes an owner or controller of electric energy in this state for the purpose of buying and selling the electric energy at wholesale;

- Does not own generation, transmission, of distribution facilities in this state;

- Does not have a certificated service area; and

- Has been granted authority by the Federal Energy Regulatory Commission to sell electric energy at market-based rates or has registered as a power marketer.

## Power System

The collective name given to the elements of the electrical system.  The Power System includes the generation, transmission, distribution, substations, etc.  The term Power System may refer to one section of a large interconnected system or to the entire interconnected system.

## Power-Angle Curve

A plot of the first 180 of the power transfer equation.  A Power-Angle curve is a useful tool for studying the angle stability of a simple power system.

## Pre-assigned Congestion Rights (PCRs)

Congestion Rights allocated prior to the annual TCR auctions to MOUs and ECs which own or have a long-term (greater than five years) contractual commitments, entered into prior to September 1, 1999, for annual capacity and energy from a specific remote Generation Resource.

## Premise

A Service Delivery Point or combination of Service Delivery Points that are assigned a single Electric Service Identifier (ESI ID) for purposes of settlement and registration.

## Price-to-Beat

The bundled rate a Retail Electric Provider that is affiliated with an Entity required to unbundled its electric services, and offer Customer Choice, must charge to residential and small commercial Customers upon initiation of Customer Choice, as further described in Section 39.202 of PURA and PUCT rules.

## Primary Meter

The ERCOT approved, revenue-quality meter connected at an ERCOT approved interconnection point.

## Primary Winding

The winding of a transformer that is connected to the power input or source end of the transformer.

## Prime Mover

A mechanism that converts thermal or hydraulic energy into mechanical power.  For example, a coal-fired boiler with a steam turbine is a Prime Mover as it converts the thermal energy of coal into the mechanical power to turn the turbine.

## Prior Agreement

Any previous agreement between an Entity, its Affiliate(s) or its predecessor(s) in interest and ERCOT regarding performance under the ERCOT Protocols.

## Production Cost

The costs associated with starting, operating, and stopping generating units.

## Program Administrator

The entity approved by the PUCT that is responsible for carrying out the administrative responsibilities related to the Renewable Energy Credit Program as set forth in PUCT Substantive Rule 25.173(g).

## Proposal for Installation

A written proposal submitted by an Entity to ERCOT describing a proposal for the installation of additional Metering Facilities.

## Proprietary Customer Information

Any information compiled by a Market Participant on a Customer in the normal course of Market Participant’s business that makes possible the identification of any individual Customer by matching such information with the Customer’s name, address, account number, type of classification service, historical electricity usage, expected patterns of use, types of Facilities used in providing service, individual contract terms and conditions, price, current charges, billing records, or any other information that a Customer has expressly requested not be disclosed.  Information that is redacted or organized in such a way as to make it impossible to identify the Customer to whom the information relates does not constitute Proprietary Customer Information.

## Protected Information

That information protected from disclosure as described in Protocols, Section 1, Overview.

## Protective Relay

A mechanical or electronic device used to sense power system disturbances and respond to limit the possible damage.

## Protocol Implementation Plan

Plan developed by ERCOT that identifies any known differences between the ERCOT market operations system, power operations system, and settlement systems and these Protocols, and specifies a plan to conform such systems to these Protocols.

## Provider of Last Resort (POLR)

The designated Competitive Retailer as defined in the PUCT Substantive Rules for default Customer service, and as further described in Section 15.1, Customer Switch of Competitive Retailer.

## Purchasing – Selling Entity (PSE)

An Entity that is eligible to purchase or sell energy or capacity and Ancillary Services.

## Pythagorean Theorem

A mathematical relationship which states that in a right triangle the square of the hypotenuse length is equal to the sum of the squares of the lengths of the remaining two sides.

Q

## QSE Operator

The person designated by the QSE to communicate with ERCOT on a twenty-four (24)-hour basis.

## Qualified Scheduling Entity

A Market Participant that is qualified by ERCOT in accordance with Section 16, Registration and Qualification of Market Participants, to submit Balanced Schedules and Ancillary Services bids and settle payments with ERCOT.

## Qualifying Facility

A qualifying cogenerator or qualifying small power producer according to regulatory qualification criteria as defined in PURA.

## Quick Start Unit

Generation units that are not On-line, but are capable of producing energy within the next Settlement Interval.

R

## Ramp Window

The time period, symmetrical across the top of a clock hour, used for adjusting generation to implement a schedule.  A typical Ramp Window is from 10 minutes to the hour till 10 minutes after the hour.  When all QSEs involved in a schedule change use the same Ramp Window, frequency deviations will be minimized.

## Rate-of-Change (Protective Relay)

A type of protective relay that monitors the rate at which a quantity changes.  For example, a Rate-of-Change relay may monitor the rate at which the MW flow along a transmission line varies.  The relay could be set to trip the line if the rate of MW flow change exceeds a specified value.

## Reach

The extent of protection that an impedance relay provides to a transmission line.  The Reach is typically defined in terms of the impedance of the line.  For example, a zone 1 impedance relay may Reach 90% into the protected line.

## Reaction Turbine

A water turbine in which the pressure differences across the turbine blades causes the blades to turn.  A Francis Turbine is an example of a Reaction Turbine.

## Reactive Capability Curve

A graphical method of illustrating the complex power output limits of a synchronous generators.  The Reactive Capability Curve is sometimes called a “D-Curve” as it is typically shaped like the letter “D”.

## Reactive Power

The product of voltage and the out-of-phase component of alternating current.  Reactive Power, usually measured in megavolt-amperes reactive, is produced by capacitors, overexcited generators, and other capacitive devices and is absorbed by reactors, underexcited generators, and other inductive devices.

## Reactive Reserve

That reactive capability required to meet the sudden loss of generation, Load, or transmission capacity and maintain voltage within desired limits.

## Reactor

A device intentionally designed to act as an inductor and store energy in its magnetic field.  A shunt Reactor acts as a sink (absorbs) of reactive power to the system.  Series Reactors are rarely seen devices that are used to increase the inductive reactance of a transmission path.

## Ready Reserve

Spinning Reserve in excess of that required to satisfy Responsive Reserve obligations or to back-up interruptible purchases.

## Real Time

The current instant in time.

## REC Program

The Renewable Energy Credit trading program, as described in Section 14, Renewable Energy Credit Trading Program, and PUCT Subst. R. 25.173.

## Reclosing Relay

A relay that automatically (after a few cycles or seconds) recloses a transmission line following a fault.

## Registered Market Participant

An Entity that is registered with ERCOT to participate in the competitive market administered by ERCOT within the ERCOT Region.  Registered Market Participants include those using statewide systems administered by ERCOT and may be non-ERCOT participants.

## Registration Processing Period

Minimum amount of time the ERCOT registration system requires to process transactions.  This period begins when ERCOT receives a registration transaction request and continues until the completion of the transaction.

## Regulating Reserve

Responsive Spinning Reserve and/or Ready Reserve that is responsive to AGC commands.  Regulating Reserve must be available to AGC at a sufficient rate for effective frequency regulation.

## Regulation Service

A service that is used to control the power output of Resources in response to a change in system frequency so as to maintain the target system frequency within predetermined limits.

## Regulating Unit

A generator used for the regulation of system frequency.  To serve as a Regulating Unit the generator must have available spinning reserve.

## Regulation

The ability to control the power output of Resources in response to a change in system frequency so as to maintain the target system frequency within predetermined limits.

## Relay

An electrical or mechanical device that responds to a measured input with a user determined output.  Types of Relays include auxiliary relays, monitoring relays, regulating relays, and protective relays.

## Reliability Authority

An Entity that ensures the reliability of the bulk power transmission system within its Reliability Authority Area.

## Reliability Councils

The geographical regions of NERC.  NERC is divided into ten Regional Reliability Councils.  ERCOT is one of these councils.

## Reliability Must Run (RMR) Service

The provision of generation capacity and/or energy resources from Reliability Must Run Unit or a Synchronous Condenser Unit.

## Reliability Must Run (RMR) Unit

A Generation Resource unit operated under the terms of an Agreement with ERCOT that would not otherwise be operated except that they are necessary to provide voltage support, stability or management of localized transmission constraints under first contingency criteria where Market Solutions do not exist.

## Remedial Action Plan

Predetermined operator actions to maintain ERCOT Transmission Grid reliability during a defined adverse operating condition.

## Renewable Production Potential

The maximum generation in MWh/interval from an Uncontrollable Renewable Resource that could be generated from all available units of such Resource.  The Renewable Production Potential depends on the renewable energy that can be generated from the available units (wind or solar radiation) and the energy conversion characteristics of each unit. The Renewable Production Potential will be determined from data submitted in accordance with procedures established by ERCOT.

## Replacement Reserve Service

A service that is procured from Generation Resource units planned to be off-line and Load acting as a Resource that are available for interruption during the period of requirement.

## Representative Interval Data Recorder

The technique for profiling premises participating in special pricing programs which consists of implementing a statistically representative Load research sample on the program population.  The sample data is then used to develop the representative IDR (RIDR) for profiling these premises.

## Resistance

The property of a material that opposes or resists current flow by converting electric energy to heat.  The symbol for Resistance is the letter “R”.

## Resistivity

A measure of the ability of a material to conduct electricity.  The greater the Resistivity, the greater the opposition to current flow.  An insulator has a high Resistivity.

## Resource

Facilities capable of providing electrical energy or Load capable of reducing, or increasing the need for electrical energy or providing Ancillary Services to the ERCOT System,.  This includes Generation Resources and Loads acting as Resources.

## Resource Category Generic Fuel Cost (RCGFC)

A standard $/MWh cost for fuel specific to one of eight resource categories (Nuclear, Hydro, Coal and Lignite, Combined Cycle, Simple Cycle, Gas Steam, Diesel and Non-Hydro Renewable).

## Resource Category Generic Startup Cost

A fixed price for starting a unit that is selected out of merit order to provide balancing energy.  The RCGSC is defined by the generation unit category (Base-load, Gas Intermediate, Gas Cyclic, Gas Peaking and Renewable).

## Resource Category Generic Operational Cost

A standard $/MWh price for running a unit selected out of merit order to provide balancing energy.  The RCGOC is defined by the generation unit category (Base-load, Gas Intermediate, Gas Cyclic, Gas Peaking and Renewable).

## Resource Entity

A Market Participant that owns or controls a Resource.

## Resource ID

A unique identifier assigned to each Resource used in the registration and settlements systems managed by ERCOT.

## Resource Minimum Down Time

The minimum time from shutdown of a Resource required until that Resource can be restarted and available to the ERCOT market.

## Resource Plan

A plan provided by a QSE to ERCOT indicating the forecast state of Generation Resources or individual Loads each acting as a Resource, including information on availability, limits and forecast generation or Load of each Resource.

## Responsibility Transfer

The controlled and orderly transfer of eligible resources from one QSE to another in accordance with Section 4, Scheduling.

## Responsive Reserve Service

Responsive Reserve consists of the daily operating reserves that are intended to help restore the frequency of the interconnected transmission system within the first few minutes of an event that causes a significant deviation from the standard frequency.

## Responsive Spinning Reserve

That portion of spinning reserve resulting from a generator’s normal governor action or from an HVDC line’s MW response that is designed to stop frequency decay.

## Restrike

The re-ignition of an electric arc.

## Retail Electric Provider

A person that sells electric Energy to retail Customers in this state.  As provided in PURA 31.002(17), a Retail Electric Provider may not own or operate generation assets.  As provided in PURA 39.353(b), a Retail Electric Provider is not an Aggregator.

## Revenue Quality Meter

For ERCOT Metered Entities, a meter that is in compliance with the Protocols and the Operating Guides. For TDSP Metered Entities, a meter that is in compliance with Local Regulatory Authority approved meter standards or the Protocols and the Operating Guides.

## Right Triangle

A triangle in which one of the three internal angles is equal to 90.

## Root Mean Square (RMS)

The effective value of an AC voltage or current.  The RMS value of an AC quantity would produce the same amount of heat in a DC resistive circuit.  For example if an AC circuit has a RMS current of 10 amps, then 10 amps of DC current would have the same heating effect.  Most AC meters read RMS values.

## Rotational Energy

Energy stored within a rotating mass.  For example, a spinning generator contains rotational energy.  The terms inertial energy, stored energy, and Rotational Energy are often used interchangeably to refer to the energy stored in the rotating elements (motors and generators) of the power system.

## Rotor

The rotating component of a motor or generator.

S

## Sample Design

The processes by which ERCOT determines the appropriate requirements for a sample of Customer Premises which requirements will be used to create a Load Profile.

## Sample Size

The number of data points (i.e. Customer Premises) in a particular sample.

## Sampling

The process of selecting a subset of a population of Customers that statistically represents the entire population.

## Saturation

After a magnetic field reaches certain strength, any further increase in the current that caused the magnetic field will not result in any increase in the strength of the magnetic field.  The magnetic material is saturated at this point.  When a transformer becomes saturated, the core’s magnetic field will expand and link portions of the transformer not designed for exposure to an alternating magnetic field.  Transformer thermal problems may result.

## SCADA

Acronym for Supervisory Control and Data Acquisition.  A SCADA system is used to remotely gather data and control power system elements.

## Schedule Control Error

The difference in the QSE’s actual Resource output and its base power schedule plus instructed Ancillary Services.

## Scheduled

The desired or target value.  For example, for a 345 KV bus, the Scheduled value of voltage may be 358 KV.  System Operators would try to hold this bus voltage at 358 KV.

## Scheduled Net Interchange

The sum of the intended (scheduled) active power flows between two Control Areas.

## Scheduling Process

The process through which schedules for energy and Ancillary Services are submitted by QSEs to ERCOT as further described in Section 4, Scheduling.

## Season

Winter months are December, January, and February; Spring months are March, April, and May; Summer months are June, July, and August; Fall months are September, October, and November.

## Secondary Winding

The winding of a transformer that is connected to the power output or load end of the transformer.

## Segmentation

The process of dividing a population into a number of sub-sets, according to certain parameters, for the purpose of creating Load Profiles for sub-sets of the population.

## Segmentation Parameter

The parameter chosen as the basis for Segmentation.

## Self-Arranged Ancillary Service

Resources used for Ancillary Services designated by a QSE for use by ERCOT for meeting the ERCOT allocated portion of the Ancillary Services Obligations of a QSE.   These Resources may not be included in the ERCOT Ancillary Services market.

## SERC

Acronym for the Southeastern Electric Reliability Council.  SERC is one of the ten NERC Regional Reliability Councils.

## Series Circuit

Electrical circuit in which elements are connected end to end.  The same current flows through each element of a Series Circuit.

## Service Address

The street address associated with an ESI-ID as recorded in the Registration Database.  This address shall conform to United States Postal Service Publication 28.

## Service Delivery Point

The specific point on the TDSP’s system where electricity flows from the TDSP to a Load.

## Service Fee Schedule

A listing of ERCOT fees and charges to Market Participants, posted on the Market Information System.

## Service Filing

A filing by a QSE to ERCOT as part of the QSE’s certification process, as defined in Section 16, Registration and Qualification of Market Participants.

## Servomotor

A device used to create a force based on a control signal input.  For example, in a governor control system a control signal is first developed using a control valve.  The control signal is input to a Servomotor.  The Servomotor then drives steam valves, wicket gates, etc.  The Servomotor may be an electric motor, an hydraulic piston, or any other means of developing a force.

## Settlement Interval

The time period for which a market service is deployed and financially settled.

## Settlement Quality Meter Data

Data that has been edited, validated, and is appropriate for ERCOT Settlement Agent to use for settlement and billing purposes.

## Settlement Statement

A statement issued by ERCOT reflecting a breakdown of administrative, miscellaneous, and market charges for the applicable Market Services.

## Initial Statement

The first iteration of a Settlement Statement issued for a particular Operating Day.

## Final Statement

The statement issued at the end of the fifty-ninth (59th) calendar day following the Operating Day.

## Resettlement Statement

The statement using corrected settlement data.

## True-Up Statement

The statement issued six (6) months following the Operating Day, as further described in Section 9.2.6, True-Up Statement.

## Shadow Price

The cost of an operation to effect a one (1) MW change in a constraint.

## Shield Wires

Conductors strung across the top of transmission lines that are designed to protect the transmission line from lightning strikes.

## Shift Factor

A measure of the flow of a unit injection of the power on the transmission element from a particular bus to a fixed reference bus.

## Site Specific Shift Factor

The actual Shift Factor for a particular bus.

## Zonal Shift Factor

The average Shift Factor for all busses in a particular zone.

## Short Circuit

The introduction of a low impedance path between conductors energized at different voltages.  A Short Circuit is the same as a fault.

## Short Supply Advisory

An ERCOT power system condition in which the summation of all the Control Area’s forecasted loads and responsive reserve obligations are greater than their forecasted available generating capacity.

## Sign/Direction Terminology Conventions for Reactive Power

## Generator

Lagging power factor operating condition is when volt-ampere reactive (VAR) flow is out of the Generation Resource unit (overexcited generator) and into the transmission system and is considered to be positive (+) flow, i.e., in the same direction as megawatt power flow.  The generator is producing megavolt-amperes reactive.

Leading power factor operating condition means that VAR flow is into the Generation Resource unit (underexcited generator) and out of the transmission system and is considered to be negative (–) flow, i.e., in the opposite direction as megawatt power flow.  The generator is absorbing megavolt-amperes reactive.

## Transmission Line Terminal

VAR flow out of the bus and into the line is considered to be positive (+) flow.  VAR flow into the bus and out of the line is considered to be negative (–) flow.

## Capacitor

Produces reactive power (VAR source) for voltage control and causes the system power factor to move towards a leading condition.

## Reactor

Absorbs reactive power (VAR sink) for voltage control and causes the system power factor to move towards a lagging condition.

## Sine

The Sine of either of the unknown angles of a right triangle is the ratio of the side directly opposite the unknown angle to the hypotenuse.

## Sinewave

A graphical representation of the sine function.  For example, AC voltage and current are Sinewaves.  The wave shape for the cosine function is the same as the sine function but with a 90 phase angle difference.

## Sliding Pressure

A method of operation available in some thermal units.  Normally a unit’s steam valves are adjusted to control the amount of steam sent to the turbine stages.  In a Sliding Pressure mode of operation, the steam valves are operated wide open to minimize valve throttling losses.  The boiler pressure is then varied to adjust the MW output of the unit.

## Slip

The difference between the synchronous speed of an induction motor and the speed at which its rotor actually rotates.

## Slip-Rings

Metal rings attached to the shaft of a synchronous machine.  The Rotor’s field winding terminates on the inner surface of the Slip-Rings while the Brushes ride on the smooth outer surface of the Slip-Rings.

## Software

A series of instructions written to enable computer hardware to perform useful tasks.

## Solid State Relay

A relay that uses semiconductor components.

## Spinning Reserve

Net generating capability that is synchronized to the system and available to carry load but not currently carrying load.  Spinning Reserve also includes the capability of HVDC lines that can be utilized within 10 minutes.

## SPP

Acronym for the Southwest Power Pool.  SPP is one of the ten NERC Regional Reliability Councils.

## Stability

A stable power system is one in which all elements of the system are bound together by magnetic forces.  For example, a stable generator’s internal magnetic field rotates in synchronism (in-step) with the magnetic field of the 3 power system to which it connects.

## Stator

The stationary component of a motor or generator.

## Stored Energy

Energy stored within a rotating mass.  For example, a spinning generator contains Stored Energy.  The terms inertial energy, Stored Energy, and rotational energy are often used interchangeably to refer to the energy stored in the rotating elements (motors and generators) of the power system.

## Substation

An element of the power system that contains circuit breakers, disconnect switches, transformers, reactors, capacitors, and other equipment.  A central control house is often provided to house control and protective equipment.

## Supercritical Boiler

A boiler system that is operated at a much higher temperature and pressure than a conventional drum type boiler.  Supercritical Boilers are also called “once through” type boilers.  A Supercritical Boiler does not have any significant steam storage that effects its response to governor commands.

## Supply

Total supply scheduled by a QSE that is comprised of Energy Supply and Ancillary Services Supply where:

## Energy Supply =

Resources + Energy purchases + Energy imports; and

## Ancillary Services Supply =

Resources + Ancillary Services purchases (including purchases through ERCOT) + Ancillary Services imports

## Surge Impedance Loading (SIL)

The MW loading on a transmission line at which the line’s natural reactive power production equals its reactive power usage.

## Switchable Resource

A Generation Resource that can be connected to either the ERCOT Transmission Grid or a grid outside the ERCOT Region.

## Switching Order

A sequence of steps to accomplish a desired Switching Process.

## Switching Process

The process by which the status (open, closed, etc.) of power system equipment (switches, circuit breakers, etc.) is adjusted to perform maintenance or enhance operations.

## Synch-Check Relay

A protective relay that will not allow a circuit breaker to be closed unless the frequency difference, voltage magnitude difference, and voltage angle across the open circuit breaker are within acceptable limits.

## Synchronous

To be in-step with a reference.  A Synchronous generator rotates in synchronism with the power system.

## Synchronous Condenser

A synchronous machine operated under the terms of an annual agreement with ERCOT that is only capable of supplying Volt Amperes Reactive (VAR) that would not otherwise be operated except that it is necessary to provide voltage support under first contingency criteria where Market solutions do not exist.

## Synchronous Speed

The speed at which a synchronous generator must rotate in order to stay in synchronism with the rotating magnetic field of the system.  The Synchronous Speed is determined by the frequency of the power system and the number of rotor magnetic poles.

## Synchroscope

A device for comparing the frequency difference and voltage angle across an open circuit breaker.

## System Benefit Fund

The fund established by the PUCT to provide funding for Customer education programs, programs to assist low-income electric Customers; and the property tax replacement mechanism provided by Section 39.601 of PURA.

## System Congestion Fund

ERCOT’s accounting fund from which payments for resolving Congestion are disbursed and to which ERCOT credits Congestion-related receipts from QSE’s representing Loads.

## System Load

The amount of electric power delivered at any specified point or points on a system.

## System Operator

An Entity supervising the collective transmission facilities of a power region that is charged with coordination of market transactions, system-wide transmission planning, and network reliability.  A System Operator is authorized to operate or supervise the operation of the bulk electric system and has the responsibility and authority to implement real time actions that ensure the stable and reliable operation of the bulk electric system.

T

## Taps

Fixed electrical contacts at different positions on a transformer’s winding.  Taps are adjusted to change the voltage ratio of a transformer.

## Target

An indicator on a relay that is displayed when the relay operates.

## TCR Interface

The CSC for which Transmission Congestion Rights are auctioned and awarded.

## TDSP Metered Entity

Any Entity that meets the requirements of Protocols, TDSP Metered Entities.

## Technical Advisory Committee

A subcommittee in the ERCOT governance structure reporting to the Board of Directors as defined by the ERCOT bylaws.

## Telemetry

Equipment for measuring a quantity (amps, volts, MW, etc.) and transmitting the result over a telecommunication system (radio, microwave, etc.) to a remote location for indication or recording.

## Tertiary Winding

An additional winding added to a power transformer.  The Tertiary Winding may be used to connect a reactor or capacitor, or to provide station service.

## Texas SET

Texas Standard Electronic Transaction procedures, set forth in Section 19, Texas SET, used to transmit information pertaining to the Customer Registration Database. Record and Data Element Definitions are provided in the data dictionary in Protocols Section 19.

## Tie-Line

A transmission line that connects two Control Areas.  A Tie-Line is sometimes referred to as an Interconnection.

## Tie-Line Bias Control

An operating mode of an AGC system.  While in Tie-Line Bias Control an AGC system will determine the ACE value by considering both the interchange and frequency errors.

## Tie-Line Telemetry

Telemetry equipment used to measure power flow data on a tie-line connecting Control Areas.  The power flow data is then transmitted to both Control Areas.

## Time Error

An accumulated time difference between the ERCOT system time and a time standard.  Time Error is caused by a difference between the ERCOT system’s actual frequency and 60 HZ.

## Time of Use Metering

A programmable electronic device capable of measuring and recording electric energy in pre-specified time periods.  For Load Profiling purposes Time of Use Metering does not include IDRs.

## Time of Use Schedule

A schedule identifying the Time of Use period associated with each Settlement Interval.  These schedules may include on-peak, off-peak, and shoulder periods.

## Torque

The Torque is a force that produces a rotating or twisting action.

## Torque Angle

The angle by which the rotating magnetic field of synchronous machine leads or lags the rotating magnetic field of the system to which it connects.  A generator has a positive Torque Angle while a motor has a negative Torque Angle.  The symbol for the Torque Angle is the letter “” (Greek lower case letter delta).

## Total Energy Obligation

The total energy Obligation for a Qualified Scheduling Entity during a Settlement Interval, including the energy from the Balanced Schedule and integrated energy of instructed Ancillary Services.

## Total Load

The summation of motor and non-motor load.

## Total Transmission Capacity

The maximum power that may be transferred across a transmission corridor while maintaining reliability of the ERCOT System.

## Transaction

An interchange event.  When ever an interchange of capacity and/or energy is arranged it is called a Transaction.  ERCOT has divided transactions into several types.  See the ERCOT Operating Guides for the transaction type descriptions.

## Transaction Clearinghouse

A batch, transactional interface intended to provide reliable exchange of high volume, standardized transactions between Market Participants and ERCOT using the Texas SET procedures.

## Transformer

An electrical device with two or more windings used to transform AC system voltage and current levels.  A Transformer works by the principle of electromagnetic induction where a voltage is induced in one winding by an alternating magnetic field in the other winding.  Power transformers are used to transform large amounts of power while instrument transformers are used to produce low voltages and currents.

## Translation Factor

The monthly ratio of the aggregate Competitive Retailer’s four (4) coincident peak demand over the monthly aggregate Competitive Retailer’s coincident peak demand, as further described in Section 9.8, Transmission Billing Determinant Calculation.

## Transmission Access Service

Use of the TDSP’s Transmission Facilities for which the TDSP is allowed to charge for the use through tariff rates approved by the PUCT.

## Transmission and/or Distribution Service Provider (TDSP)

An Entity that owns or operates for compensation in this state equipment of Facilities to transmit and/or distribute electricity, and whose rates for Transmission Service, distribution service, or both is set by a Governmental Authority.

## Transmission Billing Determinants

Key parameters and formula components required by a TDSP in determining the billing charges for the use of its Transmission Facilities and/or distribution Facilities.

## Transmission Congestion Right (TCR)

A financial hedge against the cost of 1 MW flowing across a particular Commercially Significant Constraint, in a single direction, for 1 hour.

## Transmission Congestion Right Account Holder (TCR Account Holder)

A Registered Market Participant that is qualified to bid for and own TCRs in ERCOT’s annual or monthly auctions, or that has acquired such rights in secondary markets for purposes of participating in ERCOT’s financial settlements for Congestion Credits.  [A TCR Account Holder is a Registered Market Participant whose record of ownership appears in ERCOT’s databases of auction results or financial settlements for Zonal Congestion Credits].

## Transmission Facilities

The following Facilities are deemed to be Transmission Facilities:

- Power lines, substation, and associated Facilities, operated at 60 kV or above, including radial lines operated at or above 60 kV.

- Substation Facilities on the high side of the transformer, in a substation where power is transformed from a voltage higher than 60 kV to a voltage lower than 60 kV or is transformed from a voltage lower than 60 kV to a voltage higher than 60 kV.

- The direct current interconnections with the Southwest Power Pool (SPP), Western Electric Coordinating Council (WECC), Comision Federal de Electricidad, or other interconnections.

## Transmission Lines

Transmission Lines in ERCOT are classified as power lines operated at 60 kV or above, including radial lines operated at or above 60 kV.

## Transmission Loss Factors

The fraction of ERCOT Load (forecast or actual) that is considered to constitute the ERCOT Transmission Grid losses in the Settlement Interval.  Transmission Loss Factors are computed by ERCOT and are based on a linear interpolation (extrapolation) of the calculated losses in the off-peak and on-peak seasonal ERCOT base cases.

## Transmission Losses

Difference between energy input into the ERCOT Transmission Grid and the energy taken out of the ERCOT Transmission Grid.

## Transmission Operator

An Entity charged to carry out dispatch instructions directly or on behalf of represented TDSPs.  Specific duties include:

- Operate/Manage transmission facilities between energy sources and points of delivery.

- Monitor the loading of transmission facilities.

- Notify ERCOT of the status of transmission facilities.

- Maintain operational metering.

- Implement Black Start.

## Transmission Owner

An owner of transmission facilities who develops agreements with Transmission Service Providers and Transmission Operators to provide for the establishment of ratings of facilities for the purpose of transporting energy transfers.

## Transmission Service

Commercial use of transmission facilities.

## Transmission Service Provider

An Entity under the jurisdiction of the PUCT that owns or operates Transmission Facilities used for the transmission of electricity and provides transmission service in the ERCOT Transmission Grid.

## Trigonometry

A branch of mathematics that deals with the relationships between the lengths of sides and the angles between the sides of triangles.

## Turbine

A rotating mechanical device.  A Turbine is rotated by the force of a working fluid.  The working fluid is typically steam, water, or combustion gas.

## Turbine Follow

A mode of operation of a fossil units turbine/boiler control system in which the unit’s response to a load (MW) change request is delayed if the boiler’s temperature and/or pressure moves outside set boundaries.  The MW response of the unit’s turbine “follows” the response of the boiler.

## Turns Ratio

The ratio of the number of turns in the primary winding of a transformer to the number of turns in the secondary winding.

## Two-Day Ahead

The twenty-four (24) hour period beginning the instant after 2400 forty-eight (48) hours before the Operating Day.

U

## Unaccounted for Energy (UFE)

The difference between total metered Load each Settlement Period, adjusted for applicable Distribution Losses and Transmission Losses, and total ERCOT System net generation.

## Uncontrollable Renewable Resources

Generators that can only produce an amount of energy directly related to their supply of variable energy from their respective renewable resources, such as wind or solar.

## Underexcited

A generator is Underexcited when the applied excitation is less than that needed to support the generator’s magnetic field.  The deficiency in the reactive power needs of the generator is supplied by the system.  An Underexcited generator absorbs reactive power from the system.  The terms “leading” and “bucking” are also used to refer to an Underexcited generator.

## Underfrequency Load Shedding (UFLS)

The tripping of customer load based on magnitudes of system frequency.  For example, a utility may interrupt 5% of their connected load if frequency falls below 59.3 HZ, an additional 10% if frequency falls below 58.9 HZ, and a final 10% if frequency falls below 58.5 HZ.  The purpose of UFLS is a last ditch effort to arrest a frequency decline.

## Uninstructed Deviation

A condition occurring whenever the total metered resources of a QSE for a Settlement Interval are different from the total of the scheduled resources plus any Resource deployments instructed by ERCOT.

## Uninstructed Factor

A factor used to reduce the total payments made to a Resource for Uninstructed Deviations. The Uninstructed Factor could change by interval, in accordance with Section 6.8.1.14.2, Determining the Uninstructed Factor.

## Unit Commitment

The process of selecting which generating units will be placed on line to serve the load and reserve requirements.

## Unusual Event

Specifics events, as defined in these Protocols Section 4, Scheduling, that allow ERCOT to deploy Balancing Energy Service outside of the normal deployment notification.

## Uplift

The process of allocating costs to QSEs based on Loads and exports within the ERCOT Region.

V

## Vectors

Line segments that are used to represent the magnitude and direction of physical quantities.  DC voltages and currents can be represented by Vectors.

## Virtual QSE

An LSE whose QSE has provided notice of its intent to terminate its relationship with the LSE and who then fails to meet ERCOT’s creditworthiness requirements to become an Emergency QSE, as set forth in Section 16.2.13.1, Designation as an Emergency QSE or Virtual QSE.

## Voltage

The electrical force (a separation of charge) that causes current to flow.

## Voltage Angle

The phase angle between two voltages.  The Voltage Angle is the same as the power angle.  (In a generator, the Voltage Angle between the internal generator voltage and the stator terminal voltage is the equivalent of the torque angle.)

## Voltage Profile

A predetermined distribution of desired nominal voltage set points across the ERCOT System.

## Voltage Ratio

The ratio between the primary (VP) and secondary (VS) voltages of a transformer.  The transformer’s voltages are related to the transformer’s turns ratio (NP/NS) according to the formula.

## Voltage Regulator (Generator)

A component of an excitation system.  The Voltage Regulator monitors the generator’s output voltage and causes an adjustment in excitation current when required.  Voltage regulators can be operated in either a manual or automatic mode.

## Voltage Relay

A protective relay that activates in response to either a high or low voltage.

## Voltage Source

A device capable of producing a voltage.  Generators and batteries are Voltage Sources.

## Voltage Support Service

A service that is required to maintain transmission and distribution voltages on the ERCOT Transmission Grid within acceptable limits.

W

## Weather Zone

A geographic region in which climatological characteristics are similar for all areas within such region.

## WECC

Acronym for the Western Electric Coordinating Council.  WECC is one of the ten NERC Regional Reliability Councils.

## Wholesale Customers

Non-Opt-In Entities receiving service at wholesale points of delivery from an LSE other than themselves.

## Wicket Gate

A device used to control water input to a hydro turbine.  Wicket Gates function as water valves.  Wicket Gates can be very large depending on the amount of water controlled.

## Wye Connection

A 3 electrical connection in which one end of each of the three phases is connected to a common point.  The common point is often grounded.  Transformer windings are often connected in a Wye configuration.

X

Y

Z

## Zonal Congestion

Congestion on CSCs or CREs that can be resolved by zonal deployment of Balancing Energy Services and RPRS.

## Zonal Congestion Credits

Payments equal to the directly assigned costs of BES and RPRS that were incurred in managing Zonal Congestion.  Zonal Congestion Credits are paid to TCR Account Holders, appearing in ERCOT’s database, as the owner of record for the hour of relevant Settlement Intervals.

## Zones of Protection

The zone or area within which a protective relay can sense disturbances.

## Acronyms

AAA	American Arbitration Association

ADR	Alternative Dispute Resolution

ADU	Adjusted Daily Usage

AEIC	Association of Edison Illuminating Companies

AGC	Automatic Generation Control

AML	Adjusted Metered Load

AMR	Adjusted Metered Resource

AP	Adjustment Period

API	Automated Programmatic Interface

ARR	Adjusted REC Requirement

AREP	Affiliate Retail Electric Provider

AS	Ancillary Service

ATC	Available Transmission Capability

AVR	Automatic Voltage Regulator

BLT	Block Load Transfer

BSS	Black Start Service

CAO	Control Area Operator

CCF	Capacity Conversion Factor

CFC	Constant Frequency Control

COP	Current Operating Plan

CPT	Central Prevailing Time

CR	Competitive Retailer

CRR	Congestion Revenue Right

CRE	Closely Related Elements

CSA	Continuous Service Agreement

CSC	Commercially Significant Constraint

CSV	Comma Separated Values

CT	Current Transformer

DA	Day Ahead

DAM	Day Ahead Market

DAS	Data Aggregation System

DC	Direct Current

DLC	Direct Load Control

DLF	Distribution Loss Factor

DOE	Department of Energy

DSA	Dynamic Security Analysis

DSG	Dynamically Scheduled Generation

DSL	Dynamically Scheduled Load

DSP	Distribution Service Provider

DUNS #	DUNS Number

EC	Electric Cooperative

EDI	Electronic Data Interchange

EECP	Emergency Electric Curtailment Plan

EMS	Energy Management System

EPS	ERCOT Polled Settlement Meter

EPRI	Electric Power Research Institute

ERCOT	Electric Reliability Council of Texas

ESC	Engineering Subcommittee

ESI ID	Electric Service Identifier

ETIN	Electronic Transmission Information Network

EWG	Exempt Wholesale Generators

FERC	Federal Energy Regulatory Commission

FPA	Federal Power Act

FPC	Federal Power Commission

FRR	Final REC Requirement

FTC	Federal Trade Commission

FTP	File Transfer Protocol

GR	Generation Resources

HA	Hour Ahead

HO	Historical Output

HOL	High Operating Limit

HSL	High Sustainable Limit

HVDC	High Voltage Direct Current

IDR	Interval Data Recorder

IOU	Investor Owned Utilities

IPP	Independent Power Producers

KWH	Kilowatt Hour

LMP	Locational Marginal Pricing/Locational Marginal Price

LOL	Low Operating Limit

LPA	Load Profiling Agent

LSE	Load Serving Entity

LSL	Low Sustainable Limit

MAD	Mean Absolute Deviation

MAPE	Mean Absolute Percentage Error

MCP	Market Clearing Price

MCPC	Market Clearing Price for Capacity

MCPE	Market Clearing Price for Energy

MDAS	Meter Data Acquisition System

MIS	Market Information System

MOS	Market Operating System

MOU	Municipally Owned Utility

MP	Market Participant

MRA	Must-Run Alternative

MRE	Meter Reading Entity

MVA	Megavolt Ampere

MVAR	Megavolt Ampere Reactive

NBS	National Bureau of Standards

NERC	North American Electric Reliability Council

NOIE	Non Opt-In Entity

NSRS	Non-Spinning Reserve Service

NWSIDR	Non-Weather-Sensitive Interval Data Recorder

OC	Operational Congestion

OCN	Operating Condition Notice

OOM	Out of Merit Order

OOME	Out of Merit Energy

OOMC	Out of Merit Capacity

OTC	Operating Transmission Capacity

PCR	Pre-assigned Congestion Right

PM	Power Marketer

POLR	Provider of Last Resort

POS	Power Operating System

PRR	Preliminary REC Requirement

PTB	Price to Beat

PTP	Point to Point

PUCT	Public Utility Commission of Texas

PURA	Public Utility Regulatory Act, Title II, Texas Utility Code

PURPA	Public Utility Regulatory Policy Act

QSE	Qualified Scheduling Entity

RA	Registration Agent

RAP	Remedial Action Plan

RAPP	Registration Agent Processing Period

RCAG	Remote Control Area Generator

RCAL	Remote Control Area Load

REC	Renewable Energy Credit

REP	Retail Electric Provider

RGS	Regulation Service

RGSD	Regulation Service Down

RGSU	Regulation Service Up

RID	Resource ID

RIDR	Representative IDR

RMR	Reliability Must-Run

RPP	Registration Processing Period

RPRS	Replacement Reserve Service

RRO	Responsive Reserve Obligation

RRS	Responsive Reserve Service

RRWF	Rapid Response Wind Farm

RspT	Responsibility Transfer

RSS	Reliability and Security Subcommittee

RT	Real Time

RUC	Reliability Unit Commitment

SASM	Supplemental Ancillary Service Market

SCADA	Supervisory Control And Data Acquisition

SCE	Schedule Control Error

SCED	Security Constrained Economic Dispatch

SCF	System Congestion Fund

SCUC	Security Constrained Unit Commitment

SDP	Service Delivery Point

SET	Standard Electronic Transaction

SFT	Simultaneous Feasibility Test

SGIA	Standard Generation Interconnection Agreement

SIS	Security Information System

SPP	Settlement Point Price

SPP	Southwest Power Pool

SRR	Statewide REC Requirement

SRWF	Slow Response Wind Farm

TAC	Technical Advisory Committee

TCOS	Transmission Cost of Service

TCR	Transmission Congestion Right

T&D Losses	Transmission Losses & Distribution Losses

TDSP	Transmission and/or Distribution Service Provider

TGR	Tradeable Generation Right

TLF	Transmission Loss Factor

TMOS	Transmission Market Operations Subcommittee

TNM	Texas NODAL Market

TOU	Time Of Use

TOUS	Time Of Use Schedule

TSP	Transmission Service Provider

TTC	Total Transfer Capability

TUOS	Transmission Use of Service

TX SET	Texas Standard Electronic Transaction

UFE	Unaccounted For Energy

URL	Unit Reactive Limit

USTR	Uninitiated Service Termination Request

Va.m.M	Vector Absolute Megawatt-Mile

Va.m.O	Vector Absolute Megawatt Ohm

VEE	Validation, Editing and Estimation of meter data

VSA	Voltage Security Analysis

VSS	Voltage Support Service

VT	Voltage Transformer

WPGR	Wind Powered Generation Resource

WSCC	Western System Coordinating Council

WSIDR	Weather-Sensitive Interval Data Recorder

XML	Extensible Markup Language