| PRR Number | 661 | PRR Title | SCE Performance Enforcement Criteria |
| --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Urgent | Recommended Action | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.10.5.3, Regulation Services Monitoring Criteria 6.10.12.1, SCE Enforcement Criteria (new) |  |
| Proposed Effective Date |  | July 1, 2006 |  |
| Priority and Rank Assigned |  | TBD |  |
| Revision Description |  | This revision creates the criteria for Schedule Control Error (SCE) Performance Enforcement. |  |
| Overall Market Benefit |  | If a large QSE is eliminated from the market, the Market Clearing Price (MCPE) will spike to a higher clearing price. This PRR eliminates price shock if a large QSE is eliminated from the Ancillary Services market. |  |
| Overall Market Impact |  | None |  |
| Consumer Impact |  | Not known |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  | Yes. ERCOT credit staff and the Credit Working Group have reviewed PRR661 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| Procedural History |  | - PRR661 was posted on 4/7/06 with a request for Urgent Status. - Urgent Status was granted by e-mail vote on 4/12/06. - On 4/18/06, PUC WMO posted comments. - On 4/19/06, ERCOT posted an Impact Analysis (IA). - On 4/20/06, ERCOT posted comments. - On 4/21/06, Calpine and WMS posted comments. - On 4/21/06, PRS tabled this PRR. - On 5/18/06, PRS considered this PRR. - On5/22/06, PRS again considered this PRR. - On 5/26/06, PUCT Staff posted comments. - On 5/30/06, ERCOT posted comments. - On 5/31/06, ERCOT posted an Impact Analysis (IA) and Cost Benefit Analysis (CBA). - On 6/1/06, TAC considered this PRR. |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 4/21/06, PRS voted to table consideration of PRR661, along with PRRs 586, 649, 656, and 662, until the May 18, 2006 PRS Meeting. The motion passed with two abstentions from the Independent Power Marketer and Independent Generator market segments. All market segments were present for the vote. On 5/18/06, PRS voted to recommend approval of PRR661 as recommended by WMS and revised by PRS. The motion passed with three abstentions from the Municipal (1) and the Independent REP (2) Market Segments. All Market Segments were present for the vote. On 5/22/06, PRS voted to recommend that TAC request a 30-day extension of the disqualification option in enforcing Protocol Section, 10.5.3, Regulation Services Monitoring Criteria, in the event that ERCOT is unable to finalize the IA and Cost Benefit Analysis (CBA) due to the short timeline prior to TAC. The motion passed unanimously with all Market Segments present for the vote. |  |
| Summary of PRS Discussion |  | On 4/21/06, the Frequency Control Task Force (FCTF) Chair, Mark Bruce, reported that FCTF has reviewed various options to address the SCE. The options were presented to the Reliability Operations Subcommittee (ROS) and the Wholesale Market Subcommittee (WMS). The ROS concluded that the measures implemented by PRR525, SCE Performance and Monitoring appear to be working and voted to endorse PRR525. According to Mr. Bruce, at this time the WMS is focused on developing an SCE solution that uses the existing SCE performance measurement (the PRR 525 methodology) and that requires minimal system changes. WMS expects to adopt a final recommendation for a market structure to properly align the commercial incentives and/or penalties needed to ensure QSEs maintain appropriate SCE at its May 17, 2006 meeting for consideration by the PRS in May. This recommendation will likely take the form of comments or amendments to PRR 661. Therefore, WMS recommends PRS table consideration of PRR586, 649, 656, 661, and 662 until the May 18, 2006 PRS Meeting. On 5/18/06, PRR661 and PRR662, Modify Ancillary Service Deployment Performance Conditions, were presented as a package. Participants discussed that PRR661 retains the performance criteria in Protocol Section 6.10.5.3, Regulation Services Monitoring Criteria, as recommended by ROS. PRR661 creates a performance charge for bad performance and a performance credit for good performance. QSEs that perform badly for three months will be limited from participating in the regulation market. Commission Staff emphasized that if the penalty is insufficient, the PUCT will not support this PRR. Generation Resources responded that it is difficult to assess what the proper threshold is for penalties and that most failing QSEs are wind Generation Resources. Commission Staff will review the penalty values to determine whether they are sufficient. On 5/22/06, the ERCOT Project Management reported that ERCOT is still in the process of evaluating how this PRR would be implemented. Issues being evaluated are the level of automation; the impact on Lodestar; the issue of shadow settlements; whether the Enterprise Data Warehouse is capable of supporting 10-minute data; the separation of duties between ERCOT Settlements and Compliance, particularly as it relates to the SCE calculation and subsequent settlements and billing. ERCOT may also need feedback on the types of data to be made available to the market and how, as well as the expected timeline of settling the penalties and credits. ERCOT Compliance expressed concern over the length of time needed to conduct a manual review of exemptions and the potential for error, especially if there is money involved. ERCOT Compliance urged that this process be automated the maximum extent possible. |  |
| TAC Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 6/1/06, TAC voted on a motion to recommend approval of PRR662 as amended by TAC, including the revisions and implementation assumptions numbered 1-15 presented in ERCOT’s comments. TAC also directed WMS to provide a methodology to establish the appropriate level for the scaling factor used in the SCE Performance Charge calculation defined by PRR661. WMS is to provide this methodology by the August TAC meeting. The motion passed with one opposing vote from an Independent REP and one abstention from an IOU. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | Mr. Bruce presented an overview of the history of the FCTF work related to SCE. ERCOT staff summarized ERCOT’s comments. ERCOT staff noted that this PRR may prevent ERCOT from disqualifying QSEs for continuous nonperformance because it is not explicit as to at which point a QSE may risk disqualification. ERCOT offered triggering events at which disqualification may be enforced. The proposal was rejected by the group, stating that this PRR does not revise the introductory paragraph to Section 6.10.12, Non-Compliance Actions of ERCOT. ERCOT further noted that it will continue posting of the names of underperforming QSEs. Participants concurred. ERCOT also stated that it agreed with the PUCT staff comments. Parviz Adib summarized PUCT Staff comments on PRRs, stating that limitations to providing Regulation Service should be more demanding and then proposed a scaling factor or multiplier. Mr. Adib also noted that the 90% score already allows for a fairly high level of nonperformance. Various Market Participants (OPC, Consumers, and MOU) supported PUCT’s comments. Participants discussed the feasibility of a weighted regulation share for the distribution of credits and the possibility of the scaling factor. Participants noted the short time frame given to TAC to properly analyze this PRR. Some stakeholders and PUCT Staff suggested remanding this PRR for further discussion and requesting an extension on disqualification enforcement at the June ERCOT Board meeting. The Independent Generators objected, stating that there would be a risk that the Board may not favor such a resolution and that the FCTF discussed these issues over an 18-month period. Participants agreed to direct the WMS to further explore the feasibility of a scaling factor. ERCOT staff emphasized that the IA is based on a set of assumptions and that any deviation from these assumption may change the IA (i.e. making the manual implementation of this PRR infeasible). ERCOT encouraged the group to develop a scaling factor for the entire month to minimize the calculation impacts. |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | Present Market Clearing Price for Regulation = $20/MWh |  |
| --- | --- | --- | --- |
|  | 2 | If large QSE eliminated from market, Market Clearing Price will spike to a higher clearing price |  |
|  | 3 | Large QSE probably sells 200MW per hour of Regulation Services 24 hours a day |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Resource QSEs should already be performing their obligations. | $0 |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Eliminates Price Shock to market when a large QSE is eliminated from the Ancillary Services market. |  |
| Additional Qualitative Information | 1 | $10 to $30 differential * 200MW * 24hours * 365days/year =$17.5 MM to $52.5 MM per year. |  |
| Other | 1 | Minimal incremental work from the work presently being done. |  |

| Original Sponsor |  |
| --- | --- |
| Name | Rafael Lozano |
| Company | PSEG Texgen I |
| Segment | Independent Generator |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| PUC WMO 041806 | Commented that the approach of this proposal does not meet the recommendation made by Potomac Economics because it targets only very bad performers and does not relate charges in proportion to the costs of procuring regulation. |
| ERCOT 042006 | Commented that ERCOT would not be able to perform monitoring on a 100% deployment rate as proposed by this PRR and requested clarification of the formulas and exemption clauses in order to evaluate their effectiveness and system impacts. |
| Calpine 042106 | Commented that by all accounts it appears that PRR 525 is beginning to have a beneficial impact on the control performance behaviors of many fleets. Recommended that the stakeholders better define the enforcement mechanism to accompany PRR 525. |
| WMS 042106 | Recommended that PRS table PRR 661 at its April 21, 2006 meeting. WMS expects to adopt a final recommendation for a market structure to properly align the commercial incentives and/or penalties needed to ensure QSEs maintain appropriate SCE at its 5/17/06 meeting for consideration by the PRS in May. This recommendation will likely take the form of comments or amendments to PRR 661. |
| WMS 051806 | Proposed revisions to implement penalties and credits, and limits on participation of bad performers. |
| PUC Staff 052606 | Argued that limitations of providing Regulation Service should be more demanding and proposed a scaling factor. |
| ERCOT 053006 | Supported the PUC Staff comments, offered disqualification options, announced intent to continue posting of underperforming QSEs, and stated assumptions for implementation. |

Proposed Protocol Language Revision

6.10.5.3	SCE Monitoring Criteria

SCE Monitoring Criteria will be reviewed by the appropriate ERCOT TAC subcommittee and submitted into these Protocols upon approval.

Each QSE shall control its Resources to operate to the final Resource bilateral schedules as converted to a base power function plus the equivalent power requirement of any instructed Ancillary Services and other SCE obligation terms including governor response.  ERCOT shall calculate one (1) and ten (10) minute averages of each QSE’s SCE.  ERCOT shall also calculate each QSE’s participation factor as the ratio of the QSE’s Generation Resource scheduled change in the measurement period (1 or 10 minute) to the total ERCOT Generation Resource scheduled change in the same measurement period.  ERCOT shall limit the deployment of RGS Service to QSEs for each control cycle equal to one hundred twenty five percent (125%) of the total amount of RGS Service in ERCOT divided by the number of control cycles in ten (10) minutes.  Intervals where a QSE’s generation level is less than one (1) MW in the measurement period (1 or 10 minute) will not be included in the calculation of the SCE Monitoring Criteria.  Satisfactory control performance of the QSE shall be deemed acceptable when:

(1)	The one (1) minute averages of the QSE’s SCE meet the following criteria over the calendar month (commonly referred to as SCPS1), and

AVG(month)SCE1ParticipationFactor∗ΔF1(10∗Bias1)∗ε21≤1

(2)	The ten (10) minute averages of the QSE’s SCE meet the following criteria for ninety percent (90%) of the ten (10) minute periods over the calendar month (commonly referred to as SCPS2).

SCE10≤L10∗KParticipationFactor

Where:

SCE1 	is the one minute average of SCE.

SCE10	is the ten minute average of SCE.

Bias1 	is the one minute average of the ERCOT total bias used in the ACE calculation.

F1 	is the one minute average of frequency deviation from schedule.

Participation Factor	is determined by the ratio of the QSE’s generation scheduled change for the measurement period (1 or 10 minute) to total ERCOT generation schedule change for the measurement period (1 or 10 minute).  Generation schedule change per interval is defined as below:

{Absolute Value

[     (ResourceSchedule – ResourceSchedulePreviousInterval)

+ (BalancingDeployment – BalancingDeploymentPreviousInterval)  ]

+ RegulationUpSchedule

+ RegulationDownSchedule}

If this Participation Factor Calculation results in a value of less than 1%, then 1% will be used.

 	is a constant derived from the targeted frequency bound.  It is the targeted root-mean score of one (1) minute average frequency error from a schedule based on frequency performance over a given year as established according to NERC Performance Requirements by ERCOT and the appropriate ERCOT Subcommittee as assigned by TAC.

L10 	is a limit to recognize the desired performance of frequency for ERCOT as established according to NERC Performance Requirements by the appropriate ERCOT Subcommittee assigned by TAC.  As of July 2003, L10 is defined as (1.65 * E10 * 10 * Bias10) where E10 is 0.01315 Hz and Bias10 is the ten (10) minute average of the ERCOT total bias used in the ACE calculation.

K	is a constant currently set to .81 which is established by the appropriate ERCOT Subcommittee as assigned by TAC.  K should initially be set to .81 to provide an ERCOT wide L10 equivalent to the ERCOT wide L10 currently used by Control Areas in ERCOT.  This constant can be adjusted to ensure correlation between passing the NERC CPS2 criteria and passing the SCE ten (10) minute control limit.

6.10.12	Non-Compliance Actions of ERCOT

ERCOT may revoke any or all Ancillary Service qualifications of any QSE providing an Ancillary Service(s) for continued failure to comply with required performance standards.

Failure to deliver energy resulting from a valid Dispatch Instruction is cause for ERCOT, at ERCOT’s option, to withhold payment for any Ancillary Services purchased and not delivered.

ERCOT will make information relative to each QSE’s performance as well as ERCOT’s performance, available to Market Participants on a monthly basis, subject to the provisions of Section 1.3.1, Restrictions on Protected Information.

6.10.12.1	SCE Performance Charge

For the month being analyzed, a QSE at or above the target level for the ten (10)-minute criteria for SCE performance, the SCE Performance Charge will be zero.

If a QSE falls below the target level for the ten (10)-minute criteria for SCE performance for the month being analyzed, ERCOT shall calculate the number of additional 10-minute periods that the QSE needed to have achieved the target level.  For each ten (10)-minute period below the target level, ERCOT shall calculate the potential performance charge as the simple average MCPC of URS and DRS for that period multiplied by the absolute value of average SCE during that period.  If the MCPC for URS or DRS is negative, ERCOT will set that price equal to zero (0) in calculating the SCE charge.  The QSE’s monthly Performance Charge shall be calculated as the sum of the highest cost ten (10)-minute periods which were below the target level with the number of periods equal to those required to have met the target level.

AINTqm 		=	90% * MINTqm – PINTqm

For each ten (10)-minute interval below target level:

INTCHGiq	=	SAMCPCi * ABS(SCEiq)*SF/6

SAMCPCi = Σ (MCPCRGUP + MCPCRGDN)/2

PCHSCEqhm 	= 	SUM of AINTq Greatest 	INTCHGiq

Where:

q		QSE

m		Month interval

i		10-minute interval below target level

h		Hour interval

MINTqm		10-minute intervals measured for compliance

PINTqm		10-minute intervals meeting compliance measure

SF 		Scale Factor set to 1

AINTqm		Additional 10-minute intervals needed to achieve target

INTCHGqi	Potential 10-minute interval performance charge

SAMCPCi	Simple average of URS and DRS MCPC for the hour

SCEi		Average SCE for the interval

PCHSCEqhm	SCE Performance charge

6.10.12.2 SCE Performance Credit

Receipts for SCE Performance Charges shall be allocated, as a credit, to QSEs with an SCE Compliance Score above the target level for the month being analyzed.  The SCE Performance Credit shall be calculated as follows:

PCRSCEqhm	=	-1 * SUM(PCHSCEqhm )

Where:

PCRSCEqhm	SCE Performance credit

PCHSCEqhm	SCE Performance charge, by QSE

The Performance Charges collected by ERCOT shall be allocated to all QSEs that pass the SCE Performance measurement (ninety percent (90%) or greater score) for the month. ERCOT shall distribute the payments on a pro-rata share of total ‘Regulation supplied’ basis, calculated over the set of QSEs that passed the performance measure for the month. To calculate the total Regulation Service schedule for each QSE passing the performance measure, ERCOT shall sum the schedules over the month for Regulation Service-Up and Regulation Service-Down.

ERCOT shall allocate the performance charges collected for each month to QSEs that pass the performance criteria for the month, as follows:

PCRSCEphm	=	SUM(PCHSCEqhm  )*RGPRph

Where:

PCRSCEphm	SCE Performance credit, by QSE, for QSEs passing the performance measure

PCHSCEhm	SCE Performance charge

RGPRqm		Regulation market participation ratio

q		QSE

p		QSEs passing performance measure

For each QSE that passes the performance measure, the QSE Participation Ratio is determined on the basis of its actual participation in the scheduled supply of Regulation Service-Up and Regulation Service-Down. The participation ratio shall be calculated as follows:

(RGSuphm+RGSdownhm)p

RGPRphm	=

SUM(RGSuphm+RGSdownhm)qp

Where:

RGPRqhm		Regulation market participation ratio

p		individual QSE that passed monthly performance measure

qp		QSEs passing performance measure

RGSuphm		Total Regulation Service-Up MW scheduled for the hour per month

RGSdownhm	Total Regulation Service-Down MW scheduled for the hour per month

If a QSE falls below a compliance performance level of eighty percent (80%) for any three months in a consecutive six month period, ERCOT shall limit the QSE’s right to provide Regulation Service for all hours of the day, prospectively, for a three month period. If, after three consecutive months, the QSE has not yet attained three monthly scores of 90% or greater, the limitation shall remain in place for the number of months that it takes for the QSE to successfully achieve an SCE performance of eighty percent (80%) as evaluated under Section 6.10.5.3, for three consecutive months.  During this period, ERCOT shall limit the QSE’s ability to supply Regulation Service to the QSE’s average schedule for the previous three months for each Regulation Service, as determined from the QSE’s Regulation Service schedules submitted to ERCOT for the months being analyzed.  ERCOT shall communicate, and the QSE shall comply with, the Regulation Services limit.

ERCOT shall calculate the Regulation Service limit by taking the total number of MWh of Regulation Service-Up scheduled and submitted to ERCOT for the month and dividing such quantity by the number of hours in which the QSE submitted Regulation Service-Up Schedules to ERCOT.

(RGSup)mq

RSLup	=

HrsRGSU

Where:

RSL		Regulation Service schedule limitation

RGSup		Total Regulation Service-Up MWh scheduled and submitted to ERCOT

q		QSE

m		monthly total

HrsRGSU	Total number of hours in which Regulation Service-Up schedules were submitted to ERCOT over the previous three months.

6.10.12.3 Settlement Timeline and Dispute Process

ERCOT shall process the SCE Performance Charges and Payments for a month within ten (10) Business Days after the ERCOT Compliance Dept. issues final scores.  ERCOT will apply Debits and Credits onto Settlement Statements on a scheduled Final Settlement Statement of its choosing and in accordance with Section 9.2, Settlement Statements.

ERCOT may issue a Resettlement Statement for the monthly SCE Performance charges and payments if required due to approved disputes or corrections.  Any such disputes must be submitted and approved thirty (30) days prior to the scheduled date for issuance of the True-Up Settlement Statement for the last day of the month in question. ERCOT may utilize a manual entry to process any Resettlement Statements necessary on a scheduled True-Up Settlement Statement for an Operating Day of the month being analyzed.

Market Participants disputing their SCE scores pursuant to Section 6.10.5, QSE Real Power Performance Criteria, and Section 6.10.6, Ancillary Service Deployment Performance Conditions, will make disputes known to the ERCOT Compliance Dept. prior to the date on which ERCOT issues the initial monthly settlement.  After the initial SCE performance monthly settlement, Market Participants will submit disputes relating to SCE performance calculations in accordance with Section 9.5, Settlement and Billing Dispute Process.