| PRR Number | 607 | PRR Title | One-Minute Ramp Schedules |
| --- | --- | --- | --- |
| Action |  | Referred to WMS |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 4.4.15 QSE Resource Plans 4.10.4 Resource Low Sustainable Limit as a Percentage of High Sustainable Limit Measure 4.10.7 Down Bid & Obligation Measure |  |
| Revision Description |  | Proposed revision would allow QSEs representing Resources to submit one-minute schedules for the ramping period associated with unit start up and shut down. ERCOT would be required to accept these one-minute ramp schedules for operations, compliance, and settlement purposes. |  |
| Overall Market Benefit |  | One-minute ramp schedules would provide ERCOT with the most accurate generator ramp profiles available. Better ramp information should lead to more accurate Balancing Energy deployment and decrease the amount of Regulation Service required to correct frequency. |  |
| Overall Market Impact |  | Not known |  |
| Consumer Impact |  | None |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  |  |  |
| Procedural History |  | - PRR 607 was posted on 5/27/05. - ERCOT posted comments on 6/16/05. - On 6/23/05, PRS considered this PRR. |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | PRS voted to refer PRR607 to WMS for review at the appropriate task force. PRS passed the motion with one abstention form the Municipal segment. All market segments were present. |  |
| Summary of PRS discussion |  | The Sponsor of this PRR indicated that FPL submitted PRR607 as a placeholder for discussion of frequency issues. Participants inquired whether one-minute ramp schedules were feasible. ERCOT Staff responded that they are feasible, but there are physical limitations. |  |

ERCOT/Market Segment Impacts and Benefits

Instructions: To allow for comprehensive PRR consideration, please fill out each block below completely.  Wherever possible, please include reasons, explanations, and quantitative cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Not known | Significant system changes to accommodate one-minute ramp schedules. | PRR may not produce intended benefits. |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | None |
| LSE: General, Including NOIE | None | None | None |
| LSE: CR & REP | None | None | None |
| QSE | QSEs representing resources submitting 1-minute ramp schedules will likely improve their SCE performance | QSEs choosing to submit 1-minute ramp schedules may also choose to make minor systems modifications to avoid manual entry into resource plans | Intended benefit is better SCE performance, more rational deployment instructions. PRR may not produce intended benefits. |
| Resource | Providing more detailed ramp rate information to ERCOT should lead to less operator intervention | None | Intended benefit is better SCE performance, more rational deployment instructions. PRR may not produce intended benefits. |
| TDSP | None | None | None |

| Original Sponsor |  |
| --- | --- |
| Name | Mark Bruce |
| Company | Florida Power and Light |
| Segment | Independent Generator |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| ERCOT 061605 | Explains the system limitations in implementing this PRR and requests additional details. |
| Air Liquide 062005 | Expresses concern of PRR impact on QFs’ ability to utilize PURPA Put. |

Proposed Protocol Language Revision

4.4.15	QSE Resource Plans

Each QSE that represents a Resource will present a Resource Plan to ERCOT at 1600.  These Resources may be specific Generation Resources and/or LaaRs.  The Resource Plan capacity should be sufficient to accommodate the combined quantity of energy and Ancillary Services scheduled by that QSE from the Resources that the QSE represents.  The Resource Plan shall indicate the availability of the Resources represented by the QSE, including a lead-time status code, and the planned operating level of each Resource, for each hour of the Operating Day.  The Resource Plan shall indicate the HOL and LOL, and HSL and LSL by Resource.  A Resource may be listed as unavailable to ERCOT if the Resource’s capacity has been committed to markets in regions outside of ERCOT.  QSEs representing Resources may submit one-minute ramp schedules reflecting the ramp profile of those Resources during startups and shut downs.  The one-minute ramp schedules will be used for compliance purposes and the integrated values of the ramp values within a particular interval will be used for settlement purposes for that interval.

QSEs shall use best efforts, consistent with Good Utility Practice, to continually update their Resource Plans to reflect the current and anticipated operating conditions of the Resources.  ERCOT will monitor the performance of QSEs with respect to the submission of accurate Resource Plans in accordance with the measures established in Section 4.10, Resource Plan Performance Metrics.  ERCOT will work with individual QSEs as necessary to improve the individual QSE performance.

[PRR423: Add the following sentence to the end of the first paragraph of 4.4.15, QSE Resource Plans, when the system change is implemented.] ERCOT shall use other Resource Dispatch options to maintain system reliability prior to Dispatching a Generation Resource below its LOL. ERCOT shall request Qualifying Facilities (QF), hydro units, and/or nuclear to operate below their LOL only after other Resource Dispatch options have been exhausted.

4.10.4	Resource Low Sustainable Limit as a Percent of High Sustainable Limit Measure

The “Resource LSL/HSL Measure” compares the range between the LSL and HSL submitted in the Resource Plan using the last Resource Plan submitted by the QSE before the start of the Operating Hour but after the end of the Adjustment Period.  Only hours when the Resource Plan HSL of a Resource is greater than zero (0) MW and the Resource Plan status for the same Resource is On-line are included in the calculation of this measure.  ERCOT-approved Aggregated Units are treated as single units for the purposes of calculating the score for this measure.  LaaRs, Generation Resources undergoing required testing, Generation Resources during ramp intervals, and Generation Resources with a Resource Category Generic Fuel Cost of “Renewable” or “Hydro” pursuant to Section 6.8.2.1, Resource Category Generic Costs, item 1, are excluded from this measure.

To determine whether an Occurrence is recorded, the Resource Plan HSL is multiplied by the percentage corresponding to the Resource category as specified in Section 6.8.2.1, item 1, for a particular Resource.  The Resource Plan LSL should not exceed the percentage of the Resource Plan HSL in the table below for a given Generation Resource; such an exceedance shall be recorded as an Occurrence.  The Resource category for each Resource is based on the Resource category designated by the Resource Entity for the Resource

| Resource Category Generic Fuel Cost | LSL Percent of HSL |
| --- | --- |
| Qualifying Facilities | As approved by ERCOT |
| Nuclear | 70 |
| Hydro | N/A |
| Coal and Lignite | 60 |
| Combined Cycle greater than 90 MW | 85 |
| Combined Cycle less than or equal to 90 MW | 85 |
| Gas Steam Supercritical Boiler | 40 |
| Gas Steam Reheat Boiler | 40 |
| Gas Steam Non-reheat or boiler without air-preheater | 40 |
| Simple Cycle greater than 90 MW | 90 |
| Simple Cycle less than or equal to 90 MW | 90 |
| Diesel (and all other diesel or gas-fired Resources) | 90 |
| Renewable (i.e., non-Hydro renewable Resources) | N/A |
| Block Load Transfer | N/A |

If the Resource Plan LSL is greater than the resulting value, then an Occurrence is recorded for that Resource for that hour.  Only one (1) Occurrence can be recorded per Resource per hour.  To determine the QSE Measure Score for the Resource LSL/HSL measure, Occurrences are summed for all Resources for every hour in a given month, and divided by the total number of entries submitted in the Resource Plan by a QSE for all Resources where the HSL is greater than zero (0) MW and the Resource Plan Status is On-line for every hour in that month.

Generation Resources may request (with appropriate supporting documentation) an alternate percentage, subject to approval by ERCOT.

4.10.7	Down Bid & Obligation Measure

The “Down Bid & Obligation Measure” reviews the consistency between the minimum mandatory Balancing Energy Service Down bid, the Regulation Service Down schedule, and the Resource Plan sustainable limits.  Only hours where the energy schedule is greater than zero (0) MW and Generation Resources are not in ramp mode are included in the calculation of this measure.

An Occurrence is recorded if the QSE’s minimum mandatory Balancing Energy Service Down bid exceeds the QSE’s actual zonal aggregated Balancing Energy Service Down bid by more than one (1) MW.  The minimum mandatory Balancing Energy Service Down bids are calculated in accordance with the requirement established in Section 4.5.2, Receipt of QSE’s Balancing Energy Bid Curves.  An Occurrence may also be recorded if the QSE’s Resource Plan aggregated LSL exceeds the sum of the QSE’s energy schedule, Regulation Service Down schedule, and minimum mandatory Balancing Energy Service Down bid by more than one (1) MW.  Only one (1) Occurrence can be recorded per QSE per hour.

To determine the QSE Measure Score for the Down Bid & Obligation Measure, Occurrences are summed for every hour in a given month and divided by the number of hours in a given month when the QSE’s energy schedule is greater than zero (0) MW.