| NPRR Number |  | NPRR Title | Draft Net Metering |
| --- | --- | --- | --- |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 6.6.3.1 Real-Time Energy Imbalance Payment or Charge at a Resource Node Section 10.3.2.3 Generation Netting for ERCOT Polled Settlement Meters Section 10.13 Meter Identification (renamed to EPS Meter Identification and Modeling)) |  |
| Revision Description |  | This revision adds the language and formulas that specify how to calculate the “adjusted RTSPP” which is to be used when calculating the imbalance payment or charge at a Resource Node for resources whose output is determined with a net metering configuration with more than one settlement meter. |  |
| Reason for Revision |  | At the 04-24-06 TPTF meeting, the option that applied a price adjustment factor was selected. This NPRR along with the NPRR with the changes in section 10 provide the protocol revisions necessary to implement this option. |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): |  |
| TPTF Review (Yes or No, and summary of conclusion) |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  |  |  |
| Timeline |  |  |  |
| Date Posted |  |  |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company |  |
| Company Address |  |
| Phone Number |  |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive NPRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the NPRR.

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Nodal Protocol Language Revision

6.6.3.1	Real-Time Energy Imbalance Payment or Charge at a Resource Node

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Resource Node Settlement Point:

(a) 	the energy produced by all its Generation Resource s at the Settlement Point; plus

(b) 	the amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c) 	the amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(d) 	the amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e) 	the amount of its Self-Schedules with source specified at the Settlement Point; minus

(f) 	the amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(g) 	the amount of its Energy Trades at the Settlement Point where the QSE is the seller.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

RTEIAMTq, p	=	(-1) * [RTSPP p * {Σr(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)}+ (k * RTSPP p  * ΣrRTSPLG q, p, r )]

Where,

k=me(RTSPPme,b∗GENLOADme)r(RTSPPr,b∗RTSPLGr)

For each bus with a settlement meter:

RTSPP me,b	=	Σy (RTLMP me ,b  y * SMWF me ,b y)

For each bus with a settlement meter:

SMWF  y 	=	(SEIE  y * TLMP y) / [Σy (SEIE  y * TLMP y)]

And

RTSPLGr=max(0,(meGENLOADme))∗GENSCADArrGENSCADAr

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s Energy Offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPLG q, p, r | MWh | Real-Time Split Generation per QSE per Settlement Point per Resource—The Real-Time energy produced and allocated to the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. This is the amount allocated to the Generation Resource using the methodology as specified in 10.3.2.3. |
| k | none | Adjustment Factor – The calculated multiplier used to adjust the RTSPP due to net metering. |
| RTSPP me,b | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price for the 15-minute Settlement Interval at bus with a settlement meter. |
| GENLOAD | MWh | Generation Output or LoadThe gross generation, net generation, export, import, or load for the 15 minute Settlement Interval: a positive value represents generation/export and a negative one represents load/import. |
| GENSCADA | MWh | SCADA Generation – The amount of generation output of a Generation Resource shown by SCADA for the 15 minute Settlement Interval. |
| RTLMP me, b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP at Electrical Bus b, for the SCED interval y. |
| SEIE b, y | MW | State Estimator Import or Export at bus per intervalThe import or export values from State Estimator for Electrical Bus b in the net metering configuration, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y within the Settlement Interval. |
| me | none | Metera settlement meter, which measures gross generation, net generation, export, import, or load at the electrical bus. |
| SMWF y | none | Settlement Meter Weighting Factor per bus per intervalThe weight used in the Settlement Point Price calculation for Electrical Bus b, for the portion of the SCED interval y within the 15-minute Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |

(3)	The total net payments and charges to each QSE for Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	ΣpRTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance at all Resource Node Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |