| NPRR Number |  | NPRR Title | Section 2, Zonal PRR Synchronization and ERCOT Staff Clarifications |
| --- | --- | --- | --- |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 2, Definitions and Acronyms |  |
| Revision Description |  | This NPRR incorporates the following: (1) relevant language from the following PRRs that were approved by the Board between April 2004 and February 2006: - {{proposed in separate PRR}} - PRR 518, Clarification of Requirements Related to Retail Transactions - PRR 522, Collateral Requirements and Credit Changes - PRR543, Schedules and Emergency Assistance Over CFE-ERCOT DC Ties - PRR 573, Mothballed Generation Resource Definition and Time to Service Updates - PRR 591, Switchable Unit Declaration - PRR625, Clarification of Emergency QSE Language - PRR630, Private Use Networks (2) TPTF determinations regarding ERCOT Staff clarification questions as discussed by TPTF and documented in the ERCOT Clarification Matrix for Section 8 on and discussed at the TPTF meeting on 3/6/06. (3) Incorporation of revisions from the testimony of Shams Siddiqi as required by the PUCT in its Order in Docket No. 31540, Proceeding to Consider Protocols to Implement a Nodal Market in the Electric Reliability Council of Texas Pursuant to Subst. R. §25.501 (4/5/06). |  |
| Reason for Revision |  | To incorporate into the Draft Nodal Protocols provisions approved by the Board for the Zonal market and determined by the ERCOT stakeholders to be relevant to the Nodal market. |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): |  |
| TPTF Review (Yes or No, and summary of conclusion) |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  | N/A |  |
| Timeline |  |  |  |
| Date Posted |  |  |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Trip Doggett |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West lake Dr., Taylor, TX 76574 |
| Phone Number | 512-248-6360 |
| Fax Number | (512) 248-3992 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive NPRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the NPRR.

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | Inclusion of these revisions provides for a Nodal market design that more completely reflects the intentions of the ERCOT stakeholders. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Nodal Protocol Language Revision

ERCOT Nodal Protocols

Section 2:  Definitions and Acronyms

May 17, 2006

____________________________________________________________________________________________

PRR307, Load Providing Regulation Services (Controllable Resources)

PRR 518, Clarification of Requirements Related to Retail Transactions

PRR 522, Collateral Requirements and Credit Changes

PRR543, Schedules and Emergency Assistance Over CFE-ERCOT DC Ties

PRR 573, Mothballed Generation Resource Definition and Time to Service Updates

PRR 591, Switchable Unit Declaration

PRR625, Clarification of Emergency QSE Language

PRR630, Private Use Networks

ERCOT Clarifications - CC

ERCOT Clarifications - BTW

ERCOT Clarifications – KR

Shams Siddiqi

TPTF-032806

TPTF 041006

Market Rules

Conform Style

TPTF-072406

2	DEFINITIONS AND ACRONYMS	2-1

2.1	DEFINITIONS	2-1

Adjusted Metered Load (AML)	2-1

Adjustment Period	2-1

Advisory	2-1

Affiliate	2-1

Agreement	2-2

Alert	2-2

All-Inclusive Generation Resource (see Resource)	2-2

All-Inclusive Resource (see Resource)	2-2

Alternative Dispute Resolution (ADR)	2-2

Ancillary Service	2-2

Ancillary Service Capacity Monitor	2-2

Ancillary Service Obligation	2-2

Ancillary Service Offer	2-3

Ancillary Service Schedule	2-3

Ancillary Service Plan	2-3

Ancillary Service Supply Responsibility	2-3

Ancillary Service Trade	2-3

Area Control Error (ACE)	2-3

Automatic Voltage Regulator	2-3

Availability Plan	2-3

Bank Business Day	2-4

Bankrupt	2-4

Base Point	2-4

Black Start Resource	2-4

Black Start Service	2-4

Block Load Transfer (BLT)	2-4

Bus Load Forecast	2-5

Business Day	2-5

Business Hours	2-5

Capacity Trade	2-5

Central Prevailing Time (CPT)	2-5

Combined-Cycle Configuration	2-5

Common Information Model (CIM)	2-6

Competitive Constraint	2-6

Competitive Retailer (CR)	2-6

Congestion Revenue Right (CRR)	2-6

Congestion Revenue Rights Account Holder (CRR Account Holder)	2-6

Continuous Service Agreement (CSA)	2-6

Control Area	2-6

Control Area Operator (CAO)	2-7

Cost Allocation Zone	2-7

Critical Energy Infrastructure Information (CEII)	2-7

CRR Auction	2-7

CRR Network Model	2-7

CRR Owner	2-7

Current Operating Plan (COP)	2-7

COP and Trades Snapshot	2-8

Customer	2-8

Customer Choice	2-8

Customer Registration Database	2-8

DAM-Committed Interval	2-8

DAM Energy Bid	2-8

Data Aggregation	2-8

Data Aggregation System (DAS)	2-9

Data Archive	2-9

Data Warehouse	2-9

Day-Ahead	2-9

Day-Ahead Market (DAM)	2-9

Day-Ahead Operations	2-9

Day-Ahead Reliability Unit Commitment (DRUC)	2-9

DC Tie Load	2-9

DC Tie Resource	2-10

DC Tie Schedule	2-10

Delivery Plan	2-10

Demand	2-10

Direct Current Tie (DC Tie)	2-10

Direct Load Control (DLC)	2-10

Dispatch	2-10

Dispatch Instruction	2-10

Distribution Loss Factor (DLF)	2-11

Distribution Losses	2-11

Distribution Service Provider (DSP)	2-11

Distribution System	2-11

DSR Loads	2-11

DUNS Number	2-11

Dynamic Rating	2-11

Dynamic Rating Processor	2-11

Dynamically Scheduled Resource (DSR)	2-12

Electric Cooperative	2-12

Electric Reliability Council of Texas, Inc. (ERCOT)	2-12

Electric Service Identifier (ESI ID)	2-12

Electrical Bus	2-12

Eligible Transmission Service Customer	2-13

Emergency Base Point	2-13

Emergency Condition	2-13

Emergency Electric Curtailment Plan (EECP)	2-13

Emergency Ramp Rate	2-13

Emergency Rating (see Ratings)	2-13

Energy Offer Curve	2-13

Energy Trade	2-14

Entity	2-14

ERCOT Board	2-14

ERCOT CEO	2-14

ERCOT Member	2-14

ERCOT Operator	2-14

ERCOT-Polled Settlement (EPS) Meter	2-14

ERCOT Region	2-14

ERCOT System	2-14

ERCOT System Demand	2-15

ERCOT Transmission Grid	2-15

15-Minute Rating (see Ratings)	2-15

Financing Persons	2-15

Force Majeure Event	2-15

Forced Outage (see Outage)	2-15

Fuel Index Price (FIP)	2-15

Fuel Oil Price (FOP)	2-16

Generation Entity	2-16

Generation Resource (See Resource)	2-16

Good Utility Practice	2-16

Governmental Authority	2-16

High Ancillary Service Limit (HASL)	2-17

High Emergency Limit (HEL)	2-17

High Sustained Limit (HSL for a Generation Resource)	2-17

High Sustained Limit (HSL for a Load Resource)	2-17

Hourly Reliability Unit Commitment (HRUC)	2-17

Hub	2-17

Hub Bus	2-17

Intermittent Renewable Resource (IRR)	2-18

Interval Data Recorder (IDR)	2-18

Invoice	2-18

Level I Maintenance Outage (See Outage)	2-18

Level II Maintenance Outage (See Outage)	2-18

Level III Maintenance Outage (See Outage)	2-18

Load	2-18

Load Frequency Control (LFC)	2-18

Load Profile	2-19

Load Profile Type	2-19

Load Profiling	2-19

Load Ratio Share	2-19

Load Resource (See Resource)	2-19

Load Serving Entity	2-19

Load Zone	2-19

Locational Marginal Price (LMP)	2-19

Low Ancillary Service Limit (LASL)	2-19

Low Emergency Limit (LEL)	2-20

Low Power Consumption (LPC for a Load Resource)	2-20

Low Sustained Limit (LSL for a Load Resource)	2-20

Low Sustained Limit (LSL for a Generation Resource)	2-20

Maintenance Outage (See Outage)	2-20

Make-Whole Payment	2-20

Make-Whole Charge	2-20

Market Clearing Price for Capacity (MCPC)	2-20

Market Information System (MIS)	2-21

MIS Public Area	2-21

MIS Secure Area	2-21

MIS Certified Area	2-21

Market Participant	2-21

Mass Drop	2-21

Master QSE	2-21

Maximum Power Consumption (MPC for a Load Resource)	2-21

Messaging System	2-21

Meter Data Acquisition System (MDAS)	2-22

Meter Reading Entity (MRE)	2-22

Minimum-Energy Offer	2-22

Minimum Reservation Price	2-22

Mitigated Offer Caps	2-22

Mitigated Offer Floor	2-22

Municipally Owned Utility (MOU)	2-22

Net Dependable Capability	2-22

Net Generation	2-23

Network Operations Model	2-23

Network Security Analysis	2-23

Non-Competitive Constraint	2-23

Non-Opt-In Entity (NOIE)	2-23

Non-Opt-In Entity (NOIE) Load Zone	2-23

Non-Spinning Reserve (Non-Spin)	2-23

Normal Ramp Rate	2-23

Normal Rating (see Ratings)	2-24

Off-Line	2-24

Off-Peak Hours	2-24

Oklaunion Exemption	2-24

On-Line	2-24

On-Peak Hours	2-24

Operating Day	2-24

Operating Guides	2-24

Operating Hour	2-25

Operating Period	2-25

Opportunity Outage (see Outage)	2-25

Outage	2-25

Forced Outage	2-25

Maintenance Outage	2-25

Opportunity Outage	2-25

Planned Outage	2-26

Simple Transmission Outage	2-26

Outage Scheduler	2-26

Output Schedule	2-26

Planned Outage (See Outage)	2-26

Power System Stabilizer	2-26

Premise	2-26

Prior Agreement	2-26

Program Administrator	2-27

Protected Information	2-27

Provider of Last Resort (POLR)	2-27

QSE Clawback Interval	2-27

QSE-Committed Interval	2-27

Qualified Scheduling Entity (QSE)	2-27

Qualifying Facility (QF)	2-28

Ratings	2-28

Emergency Rating	2-28

15-Minute Rating	2-28

Normal Rating	2-28

Reactive Power	2-28

Real-Time	2-28

REC Program	2-28

Regulation Down Service (Reg-Down)	2-29

Regulation Service	2-29

Regulation Up Service (Reg-Up)	2-29

Reliability Must-Run (RMR) Service	2-29

Reliability Must-Run (RMR) Unit	2-29

Reliability Unit Commitment (RUC)	2-29

Remedial Action Plan (RAP)	2-30

Renewable Production Potential (RPP)	2-30

Resource	2-30

All-Inclusive Generation Resource	2-30

All-Inclusive Resource	2-30

Generation Resource	2-30

Load Resource	2-30

Non-Modeled Generator	2-30

Resource Entity	2-31

Resource ID (RID)	2-31

Resource Node	2-31

Resource Parameter	2-31

Resource Status	2-31

Responsive Reserve	2-31

Retail Electric Provider (REP)	2-32

Revenue Quality Meter	2-32

RUC-Committed Hour	2-32

RUC-Committed Interval	2-32

RUC Study Period	2-32

Security-Constrained Economic Dispatch (SCED)	2-32

Self-Arranged Ancillary Service Quantity	2-33

Service Delivery Point	2-33

Settlement	2-33

Settlement Interval	2-33

Settlement Meter	2-33

Settlement Point	2-33

Settlement Point Price	2-33

Settlement Quality Meter Data	2-33

Shadow Price	2-33

Shift Factor	2-34

Short-Term Wind Power Forecast	2-34

Simple Outage (See Outage)	2-34

Special Protection Systems (SPS)	2-34

Split Generation Resource	2-34

Startup Cost	2-34

Startup Offer	2-34

State Estimator (SE)	2-34

System Operator	2-35

TDSP Metered Entity	2-35

Technical Advisory Committee (TAC)	2-35

Texas Nodal Market Implementation Date	2-35

Texas SET	2-35

Three-Part Supply Offer	2-35

Transmission Access Service	2-35

Transmission and/or Distribution Service Provider (TDSP)	2-35

Transmission Element	2-36

Transmission Facilities	2-36

Transmission Loss Factors	2-36

Transmission Losses	2-36

Transmission Service	2-36

Transmission Service Provider (TSP)	2-36

Unaccounted for Energy (UFE)	2-37

Unit Reactive Limit	2-37

Updated Network Model	2-37

Usage Profile (See Load Profile)	2-37

USD	2-37

Verbal Dispatch Instruction (VDI)	2-37

Voltage Support Service	2-37

Weather Zone	2-38

Wholesale Customer	2-38

Wholesale Electric Market Monitor (WEMM)	2-38

2.2	ACRONYMS AND ABBREVIATIONS	2-38

# DEFINITIONS AND ACRONYMS

The list of acronyms is at the end of this Definitions Section.

## DEFINITIONS

Definitions are supplied for terms used in more than one Section of the Protocols.  If a term is used in only one Section, it is defined there at its earliest usage.

A

Adjusted Metered Load (AML)

Retail Load usage data that has been adjusted for Unaccounted for Energy (UFE), Transmission Losses, Distribution Losses, and DC Tie exports (except for the Oklaunion Exemption).

Adjustment Period

For each Operating Hour, the time between 1800 in the Day-Ahead up to the start of the hour before that Operating Hour.

Advisory

The second of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition, detailed in Section 6.5.9, Emergency Operations.

Affiliate

(a)	An Entity that directly or indirectly owns or holds at least five percent of the voting securities of another Entity; or

(b)	An Entity in a chain of successive ownership of at least five percent of the voting securities of another Entity; or

(c)	An Entity that has at least five percent of its voting securities owned or controlled, directly or indirectly, by another Entity; or

(d)	An Entity that has at least five percent of its voting securities owned or controlled, directly or indirectly, by an Entity who directly or indirectly owns or controls at least five percent  of the voting securities of another Entity or an Entity in a chain of successive ownership of at least five percent  of the voting securities of another Entity; or

(e)	A person who is an officer or director of another entity or of a corporation in a chain of successive ownership of at least five percent of the voting securities of an Entity; or

(f)	 Any other Entity determined by the PUCT to be an Affiliate.

Agreement

A signed written agreement between ERCOT and a Market Participant using one of the standard form agreements in Section 22 of these Protocols, including those agreements containing changes to the standard form, which changes have been approved by the ERCOT Board.

Alert

The third of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition, detailed in Section 6.5.9, Emergency Operations.

All-Inclusive Generation Resource (see Resource)

All-Inclusive Resource (see Resource)

Alternative Dispute Resolution (ADR)

Procedures, outlined in Section 20, Alternative Dispute Resolution Procedure, for settling disputes by means other than litigation.

Ancillary Service

A service necessary to support the transmission of energy to Loads while maintaining reliable operation of the Transmission Service Provider’s transmission system using Good Utility Practice.

Ancillary Service Capacity Monitor

A set of processes described in Section 8.1.2.3, QSE Ancillary Service Capacity Compliance Monitoring Criteria, to determine the real-time capability of Resources to provide Ancillary Service.

Ancillary Service Obligation

For each Ancillary Service, a QSE’s ERCOT-allocated share of total ERCOT System needs for that Ancillary Service.

Ancillary Service Offer

An offer to supply Ancillary Service capacity in the Day-Ahead Market or a Supplemental Ancillary Service Market.

Ancillary Service Schedule

The MW of each Ancillary Service that each Resource is providing in Real-Time and the MW of each Ancillary Service for each Resource for each hour in the Current Operating Plan.

Ancillary Service Plan

A plan produced by ERCOT, as described in Section 4.2.1, Ancillary Service Plan and Ancillary Service Obligation, which identifies the types and amount of Ancillary Service necessary for each hour of the Operating Day.

Ancillary Service Supply Responsibility

The net amount of Ancillary Service capacity that a QSE is obligated to deliver to ERCOT, by hour and service type, from Resources represented by the QSE.

Ancillary Service Trade

A QSE-to-QSE transaction that transfers an obligation to provide Ancillary Service capacity between a buyer and a seller.

Area Control Error (ACE)

A calculation of the MW correction needed to control the actual system frequency to the scheduled system frequency.

Automatic Voltage Regulator

A device on a Generation Resource used to automatically control the Generation Resource’s voltage to an established set point.

Authorized Representative

The person designated by an Entity through the registration process under Section 16, Registration and Qualification of Market Participants, who is responsible for administrative communications between ERCOT and the Entity the person represents, and who has enough authority to commit and bind the Entity the person represents.

Availability Plan

An hourly representation of availability of Reliability Must-Run (RMR) Units, Synchronous Condenser Units, or Black Start Resources submitted to ERCOT by 0600 in the Day Ahead by QSEs representing RMR Units or Black Start Resources.

B

Bank Business Day (see Business Day)

Bankrupt

The condition of an Entity that:

(1)	files a petition or otherwise commences a proceeding under any bankruptcy, insolvency, reorganization or similar law, or has any such petition filed or commenced against it;

(2)	makes an assignment or any general arrangement for the benefit of creditors;

(3)	has a liquidator, administrator, receiver, trustee, conservator, or similar official appointed with respect to it or any substantial portion of its property or assets; or

(4)	is generally unable to pay its debts as they fall due.

Base Point

The MW output level for a Resource produced by the SCED process.

Black Start Resource

A Generation Resource under contract with ERCOT to provide Black Start Service.

Black Start Service

An Ancillary Service provided by a Resource able to start without support of the ERCOT Transmission Grid.

Block Load Transfer (BLT)

A transfer system that isolates a group of Loads from the Control Area in which they normally are served and then connects them to an adjacent Control Area. Such transfer systems involve either transferring Loads normally in ERCOT to a Non-ERCOT Control Area or transferring Loads normally in Non-ERCOT Control Areas to the ERCOT Control Area.

Bus Load Forecast

A set of processes used by ERCOT to determine a forecast of the Load at each Electrical Bus in the ERCOT Transmission Grid.

Business Day

Monday through Friday, excluding observed holidays listed below:

(1)	New Year’s Day;

(2)	Memorial Day;

(3)	Independence Day;

(4)	Labor Day;

(5)	Thanksgiving Thursday and Friday; and

(6)	Two days at Christmas, as designated from time to time by the ERCOT CEO.

Bank Business Day

Any day during which the United States Federal Reserve Bank of New York is open for normal business activity.

Retail Business Day

- Same as a Business Day, except in the case of retail transactions processed by a TSP or DSP, CR’s shall substitute the TSP or DSP holidays for ERCOT holidays when determining the time available to the TSP or DSP to process the transaction.   For additional important information related to Retail Business Days, please refer to the Retail Market Guide.

Business Hours

8:00 A.M. to 5:00 P.M. Central Prevailing Time on Business Days.

C

Capacity Trade

A QSE-to-QSE financial transaction that transfers responsibility to supply capacity between a buyer and a seller at a Settlement Point.

Central Prevailing Time (CPT)

Either Central Standard Time or Central Daylight Time, in effect in Austin, Texas.

Combined-Cycle Configuration

Any combination in which a combined-cycle power block can be operated as a separate Resource. Each possible configuration operated as a separate Resource has a distinct set of operating parameters, physical constraints, and Energy Offer Curve.

Comision Federal de Electricidad (CFE)

The state-owned federal commission of electricity of Mexico. The government agency in Mexico charged with the responsibility of operating the Mexican national electricity grid (outside Mexico City).

Common Information Model (CIM)

A standard way to communicate information about a transmission system. CIM is used to describe the ERCOT transmission system topology consisting of Transmission Elements, including all the parameters needed to describe the Transmission Elements and how they interrelate to one another. The CIM that ERCOT and the TSP use must conform to NERC and EPRI standards for CIM.

Competitive Constraint

A contingency and limiting Transmission Element pair that is determined to be competitive by an appropriate TAC Subcommittee.

Competitive Retailer (CR)

Municipally Owned Utility or an Electric Cooperative that offers Customer Choice and sells electric energy at retail in the restructured electric power market in Texas; or a Retail Electric Provider (REP) as defined in P.U.C. Subst. R. 25.5, Definitions.

Congestion Revenue Right (CRR)

A financial instrument that entitles the holder to be charged or to receive compensation (i.e., congestion rent), depending on the instrument, when the ERCOT Transmission Grid is congested in the Day-Ahead Market or in Real Time.

Flowgate Right (FGR)

A Flowgate Right is a type of CRR that entitles the holder to receive compensation and is evaluated in each CRR Auction and DAM as the positive power flows represented by the quantity of the CRR bid or offer (MW) on a flowgate (i.e., predefined directional network element or a predefined bundle of directional network elements).

Point-to-Point (PTP) Obligation

A PTP Obligation is a type of CRR that entitles the holder to be charged or to receive compensation and is evaluated in each CRR Auction and DAM as the positive and negative power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points of the quantity represented by the CRR bid or offer (MW).

Point-to-Point (PTP) Option

A PTP Option is a type of CRR that is evaluated in each CRR Auction and DAM as the positive power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points in the quantity represented by the CRR bid or offer (MW), excluding all negative flows on all directional network elements.  A PTP Option entitles the holder to receive compensation equal to the positive energy price difference between the sink and the source settlement point prices.  A PTP Option with Refund is evaluated in the same manner and compensated as described in Section 7.4.2, PCRR Allocation Terms and Conditions.

Continuous Service Agreement (CSA)

An arrangement between the owner or controller of a leased Premise and a Competitive Retailer (CR) wherein the CR provides service to the leased Premise between tenants so that the Premise does not experience discontinuation of electric service during vacancy.

Control Area

An electrical system, bound by interconnect (tie line) metering and telemetry, that continuously regulates, through automatic Resource control, its Resource(s) and interchange schedules to match its system Load and frequency schedule.

Control Area Operator (CAO)

An individual or set of individuals responsible for monitoring and controlling operation of a Control Area.

Cost Allocation Zone

One of the four zones in effect during the 2003 ERCOT market as they are changed pursuant to Section 3.4.2, Load Zone Modifications.  A Cost Allocation Zone may be used by ERCOT to uplift certain costs to a QSE’s Load regardless of NOIE Load Zone.

CR of Record

The Competitive Retailer assigned to the ESI ID in ERCOT’s database.  There can be no more than one CR of Record assigned to an ESI ID for any given time period.

Critical Energy Infrastructure Information (CEII)

Information concerning proposed or existing critical infrastructure (physical or virtual) that:

(1)	Relates to the production, generation, transmission or distribution of energy;

(2)	Could be useful to a person planning an attack on critical infrastructure;

(3)	Is exempt from mandatory disclosure under the Freedom of Information Act, 5 U.S.C. §522; and

(4)	Gives strategic information beyond the location of the critical infrastructure.

CRR Account Holder

An Entity that is qualified to become the owner of record of CRRs and is registered as a CRR Account Holder with ERCOT.

CRR Auction

A periodic auction by ERCOT that allows eligible CRR Account Holders to buy and sell CRRs.

CRR Network Model

A model of ERCOT network topology to be used in conducting a CRR Auction. It must be based on, but is not the same as, the Updated Network Model, as detailed in Section 3.10.3, CRR Network Model.

CRR Owner

The CRR Account Holder that owns one or more CRRs.

Current Operating Plan (COP)

A plan by a QSE reflecting anticipated operating conditions for each of the Resources that it represents for each hour in the next seven Operating Days, including Resource operational data, Resource Status, and Ancillary Service Schedule.

COP and Trades Snapshot

A record of a QSE’s Capacity Trades, Energy Trades, and most recent COP.

Customer

An Entity that purchases electricity for its consumption.

Customer Choice

The freedom of a retail Customer to purchase electric services, either individually or on an aggregated basis with other retail Customers, from the provider or providers of the Customer’s choice and to choose among various fuel types, energy efficiency programs, and renewable power suppliers.

Customer Registration Database

The database maintained by the registration agent containing information identifying each Premise, including current and previous Competitive Retailers serving the Premise.

D

DAM-Committed Interval

A Settlement Interval for which the Resource has been committed due to a Day-Ahead Market award.

DAM Energy Bid

A proposal to buy energy in the Day-Ahead Market at a Settlement Point at a monotonically decreasing price with increasing quantity.

Data Aggregation

The process of netting, grouping, and summing Load consumption data, applying appropriate profiles, Transmission Loss Factors, and Distribution Loss Factors and calculating and allocating UFE to determine each QSE and/or Load Serving Entity’s responsibility by Settlement Interval by Load Zone and by other prescribed aggregation determinants.

Data Aggregation System (DAS)

The database and communication system that collects meter data from TSPs, DSPs and ERCOT Polled Settlement Meters. The system performs aggregation functions to the Load data in order to satisfy certain objectives, such as providing TSPs with Load share data to use in billing Competitive Retailers, assigning QSE Load responsibility, and assisting Competitive Retailers and QSEs in their settlement responsibilities. The data is also compiled along Load and Weather Zones.

Data Archive

An integrated normalized data structure of all the target source systems' transactions. The population of the data archive is an extraction of data from the transaction systems without altering the data. The Data Archive is used to populate the Data Warehouse.

Data Warehouse

De-normalized data stored in a schema, physically optimized to handle high volumes of data and concurrent user access, and generally lightly indexed.

Day-Ahead

The 24-hour period before the start of the Operating Day.

Day-Ahead Market (DAM)

A daily, co-optimized market in the Day-Ahead for Ancillary Service capacity, certain Congestion Revenue Rights, and forward financial energy transactions.

Day-Ahead Operations

The Day-Ahead process consisting of the Day-Ahead Market (DAM), and Day-Ahead Reliability Unit Commitment (DRUC).

Day-Ahead Reliability Unit Commitment (DRUC)

A Reliability Unit Commitment process performed for the next Operating Day.

DC Tie Load

A Load used to represent the withdrawal of power from the ERCOT System to a DC Tie.

DC Tie Resource

A Resource used to represent the injection of power into the ERCOT System from a DC Tie.

DC Tie Schedule

The information for a physical transaction between a buyer and a seller, one of which is in ERCOT and the other of which is in a Non-ERCOT Control Area, for energy at a Settlement Point that is a DC Tie.

Delivery Plan

A plan by ERCOT containing the hours and levels of operation that a Reliability Must-Run Unit, including a synchronous condenser unit, is instructed to operate.

Demand

The amount of instantaneous electric power in MW delivered at any specified point or points on a system.

Direct Current Tie (DC Tie)

Any non-synchronous transmission interconnections between ERCOT and non-ERCOT electric power systems.

Direct Load Control (DLC)

Controlling end-use equipment (e.g., air conditioning equipment, water heaters) to reduce or increase energy consumption during select periods.

Dispatch

The act of issuing Dispatch Instructions.

Dispatch Instruction

A specific command issued by ERCOT to a QSE, TSP or DSP in the operation of the ERCOT System.

Distribution Loss Factor (DLF)

The ratio of a Distribution Service Provider’s estimated Distribution Losses to the total amount of energy deemed consumed (Interval Data Recorder plus profiled consumption) on the Distribution Service Provider’s system.

Distribution Losses

The difference between the energy delivered to the Distribution System and the energy consumed by Customers connected to the Distribution System.

Distribution Service Provider (DSP)

An Entity that owns or operates a Distribution System for the delivery of energy from the ERCOT Transmission Grid to Customers.

Distribution System

That portion of an electric delivery system operating under 60 kilovolts (kV) that provides electric service to Customers or Wholesale Customers.

DSR Loads

A Load that a QSE designates to be followed by a Dynamically Scheduled Resource.

DUNS Number

A unique nine-digit common company identifier used in electronic commerce transactions, supplied by the Data Universal Numbering System (DUNS).

Dynamic Rating

The current-carrying capability of a Transmission Element adjusted to take into account the effect of ambient weather conditions.

Dynamic Rating Processor

A process used to establish ERCOT Transmission Element limits based upon factors such as ambient temperature and wind speed.

Dynamically Scheduled Resource (DSR)

A Resource that has been designated by the QSE, and approved by ERCOT, as a DSR status-type and follows a DSR Load.

E

Electric Cooperative (EC)

(a)	A corporation organized under Chapter 161  of the Electric Cooperative Corporation Act, Tex. Util. Code Ann. (1997);

(b)	A corporation organized as an electric cooperative in a state other than Texas that has obtained a certificate of authority to conduct business in Texas; or

(c)	A successor to an electric cooperative created before June 1, 1999, under a conversion plan approved by a vote of the members of the electric cooperative, regardless of whether the successor later purchases, acquires, merges with, or consolidates with other electric cooperatives.

Electric Reliability Council of Texas, Inc. (ERCOT)

A Texas nonprofit corporation that has been certified by the PUCT as the Independent Organization, as defined in the Public Utility Regulatory Act, Tex. Util. Code Ann. §39.151 (Vernon 1998 & Supp. 2005)(PURA), for the ERCOT Region.

Electric Service Identifier (ESI ID)

The basic identifier assigned to each Service Delivery Point used in the registration and settlement systems managed by ERCOT or another Independent Organization.

Electrical Bus

A physical transmission element that connects, using breakers and switches, one or more Loads, lines, transformers, generators, capacitors, reactors, and phase shifters to the ERCOT Transmission Grid where there is negligible impedance between the connected transmission elements.  Electrical Buses may be connected to other Electrical Buses using breakers and associated switches.  There are no breakers or switches within an Electrical Bus.  The breakers and associated switches that are used to interconnect Electrical Buses are not themselves an Electrical Bus.  All Electrical Buses are designated by ERCOT and TSPs for modeling the electrical topology of the ERCOT Transmission Grid.

Eligible Transmission Service Customer

A Transmission and/or Distribution Service Provider (for all uses of its transmission system), or any electric utility, Municipally Owned Utility, Electric Cooperative, power generation company, Competitive Retailer, Retail Electric Provider, federal power marketing agency, exempt wholesale generator, Qualifying Facility, power marketer, or other Entity that the PUCT has determined to be an Eligible Transmission Service Customer.

Emergency Base Point

The target MW output level for a Resource that is selected by ERCOT during an Emergency Condition.

Emergency Condition

Either:

(1)	An operating condition in which the safety or reliability of the ERCOT System is compromised or threatened, as determined by ERCOT; or

(2) 	The failure of the SCED process.

Emergency Electric Curtailment Plan (EECP)

A plan that provides an orderly, predetermined procedure for maximizing use of available Resources and, only if necessary, curtailing load during an Emergency Condition while providing for the maximum possible continuity of service and maintaining the integrity of the ERCOT System.

Emergency Ramp Rate

The maximum rate of change in MW per minute of a Resource to provide Responsive Reserve that is deployed by ERCOT and that is provided to ERCOT in up to ten segments, each represented by a single MW per minute value (across the capacity of the Resource), which describes the available rate of change in output for the given range (between HSL and LSL) of the output of a Resource.

Emergency Rating (see Ratings)

Energy Imbalance Service

An Ancillary Service that is provided when a difference occurs between the scheduled and the actual delivery of energy in Real-Time.

Energy Offer Curve

A proposal to sell energy at a Settlement Point at a monotonically increasing price with increasing quantity.

Energy Trade

A QSE-to-QSE financial transaction that transfers responsibility for energy between a buyer and a seller at a Settlement Point.

Entity

Any natural person, partnership, municipal corporation, cooperative corporation, association, governmental subdivision, or public or private organization.

ERCOT Board

The Board of Directors of the Electric Reliability Council of Texas, Inc.

ERCOT CEO

The Chief Executive Officer of ERCOT.

ERCOT Member

Any member of ERCOT in good standing under the ERCOT Bylaws.

ERCOT Operator

An employee of ERCOT responsible for operating the ERCOT Transmission Grid

ERCOT-Polled Settlement (EPS) Meter

Any meter polled directly by ERCOT for use in the financial settlement of the market.

ERCOT Region

The power region represented by the ERCOT Control Area.

ERCOT System

The interconnected power system that is under the jurisdiction of the PUCT and that is not synchronously interconnected with either the Eastern Interconnection or the Western Electricity Coordinating Council.

ERCOT System Demand

The sum of all power flows, in MW, on the DC Ties and from Generation Resources metered at the point of its interconnection with the ERCOT System at any given time.

ERCOT Transmission Grid

All Transmission Facilities that are part of the ERCOT System.

F

15-Minute Rating (see Ratings)

Financing Persons

The lenders, security holders, investors, partners, multilateral institutions, and other Entities providing financing or refinancing for the business of another Entity, including development, construction, ownership, operation and/or maintenance of a facility or any portion thereof, or any trustee or agent acting on behalf of any of the foregoing.

Flowgate Right (FGR) (see Congestion Revenue Right (CRR))

Force Majeure Event

Any event beyond the reasonable control of, and that occurs without the fault or negligence of, an Entity whose performance is prevented by the occurrence of such event.  Examples of such a Force Majeure Event may include the following, subject to the limitations of the above sentence:  an act of God, labor disturbance, act of the public enemy, war, insurrection, riot, fire, storm or flood, explosion, breakage or accident to machinery or equipment, or a curtailment, order, regulation or restriction imposed by governmental, military, or lawfully established civilian authorities.

Forced Outage (see Outage)

Fuel Index Price (FIP)

The midpoint price expressed in dollars per million British thermal units ($/MMBtu), published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel.”  The Gas Daily indicates which flow dates the prices are effective.  For Saturdays, Sundays, holidays, and other days for which Gas Daily does not publish an effective price, the effective price shall be the effective price for the Operating Day following the holiday or day without a published price.  In the event, at the time of settlement or calculation of generic costs, that the described midpoint price for a particular Operating Day is not available, the effective price for the most recent preceding Operating Day shall be used.

Fuel Oil Price (FOP)

The sum of five cents plus the average of the Platts Oilgram Price Report for U.S. Gulf Coast, pipeline  No. 2 oil, converted to dollars per million British thermal units ($/MMBtu).  The conversion is 0.1385 MMBtu per gallon.   The Platts Oilgram Price Report indicates which Operating Days the prices are effective.    For Saturdays, Sundays, holidays, and other days for which Platts Oilgram Price Report does not publish an effective price, the effective price shall be the effective price for the Operating Day  following the holiday or day without a published price.    In the event, at the time of settlement or calculation of generic costs, that the effective price for a particular Operating Day is not available, the effective price for the most recent preceding Operating Day shall be used.

G

Generation Entity

Owner of an All-Inclusive Generation Resource.

Generation Resource (See Resource)

Good Utility Practice

Any of the practices, methods, and acts engaged in, or approved by, a significant portion of the electric utility industry during the relevant time period, or any of the practices, methods, and acts that, in the exercise of reasonable judgment in light of the facts known at the time the decision was made, could have been expected to accomplish the desired result at a reasonable cost consistent with good business practices, reliability, safety, and expedition. Good Utility Practice is not intended to be limited to the optimum practice, method, or act, to the exclusion of all others, but rather is intended to include acceptable practices, methods, and acts generally accepted in the region.

Governmental Authority

Any federal, state, local, or municipal body having jurisdiction over a Market Participant or ERCOT. But a Governmental Authority that is also a Market Participant may not exercise its jurisdiction in any matter that involves the interests of that Market Participant where that matter also involves the interests or responsibilities of any other Market Participant or ERCOT, unless the matter is one in which the Market Participant has exclusive jurisdiction.

H

High Ancillary Service Limit (HASL)

A dynamically calculated MW upper limit on a Resource to reserve the part of the Resource’s capacity committed for Ancillary Service, calculated as described in Section 6.5.7.2, Resource Limit Calculator.

High Emergency Limit (HEL)

Limit established by the QSE describing the maximum temporary unsustainable energy production capability of the Resource. This limit must be achievable for a time stated by the QSE, but not less than 30 minutes.

High Sustained Limit (HSL for a Generation Resource)

Limit established by the QSE, continuously updated in Real Time, that describes the maximum sustained energy production capability of the Resource.

High Sustained Limit (HSL for a Load Resource)

Limit calculated by ERCOT, using the QSE-established Low Power Consumption.

Hourly Reliability Unit Commitment (HRUC)

Any Reliability Unit Commitment executed after the Day-Ahead RUC.

Hub

A designated Settlement Point consisting of a group of Resource Nodes and other Electrical Buses, including transmission junction points, and the settlement price calculation methodology that is set out in the definition of the Hub in Section 3.5.2, Hub Definitions. Hubs may only be created by an amendment to Section 3.5.2, Hub Definitions.   The list of Resource Nodes and other Electrical Buses and the settlement price calculation methodology that define a Hub can never be modified and a Hub, once defined, exists in perpetuity.

Hub Bus

An Electrical Bus or group of Electrical Buses in one substation that is a single element of a Hub definition. The LMP of the Hub Bus is the simple average of the LMPs assigned to each Electrical Bus in the Hub Bus.  This is used solely for calculating the prices of existing Hub Buses defined in Section 3.5.2, Hub Definitions.

I

Independent Organization

An independent organization as defined in the Public Utility Regulatory Act, Tex. Util. Code Ann. §39.151 (Vernon 1998 & Supp. 2005).

Intermittent Renewable Resource (IRR)

Generation Resources that can only produce energy from variable, uncontrollable Resources such as wind, solar, or run-of-the-river-hydro.

Interval Data Recorder (IDR)

Metering device that is capable of recording Load in each Settlement Interval under Section 9, Settlement and Billing, and Section 10, Metering.

Invoice

A notice for payment or credit due rendered by ERCOT.

J

K

L

Level I Maintenance Outage (See Outage)

Level II Maintenance Outage (See Outage)

Level III Maintenance Outage (See Outage)

Load

The amount of energy in MWh delivered at any specified point or points on a system.

Load Frequency Control (LFC)

Deployment of those Generation Resources that are providing Regulation Service to ensure that system frequency is maintained within predetermined limits and deployment of those Generation Resources that are providing Responsive Reserve Service when necessary as backup regulation. LFC does not include the deployment of Responsive Reserve by Load Resources when deployed as a block under Emergency Electric Curtailment Plan (EECP) procedures.

Load Profile

A representation of the energy usage of a group of Customers, showing the Demand variation on an hourly or sub-hourly basis.

Load Profile Type

A classification of a group of Customers having similar energy usage patterns and that are assigned the same Load Profile.

Load Profiling

The set of processes used to develop and create Load Profiles.

Load Ratio Share

The ratio of an Entity’s Adjusted Metered Load to total ERCOT Adjusted Metered Load for an interval.

Load Resource (See Resource)

Load Serving Entity

An Entity that sells energy to Customers or Wholesale Customers and that has registered as an LSE with ERCOT. Load Serving Entities include Competitive Retailers (which includes Retail Electric Providers) and Non-Opt-In Entities that serve Load.

Load Zone

A group of Electrical Buses assigned to the same zone under Section 3.4, Load Zones.  Every Electrical Bus in ERCOT with a Load must be assigned to a Load Zone for settlement purposes. A NOIE Load Zone is a type of Load Zone.

Locational Marginal Price (LMP)

The offer-based marginal cost of serving the next increment of Load at an Electrical Bus, which marginal cost is produced by the DAM process or by the SCED process.

Low Ancillary Service Limit (LASL)

A dynamically calculated MW lower limit on a Resource to maintain the ability of the Resource to provide committed Ancillary Service.

Low Emergency Limit (LEL)

Limit established by the QSE describing the minimum temporary unsustainable energy production capability of the Resource. This limit must be achievable for a period of time indicated by the QSE but not less than 30 minutes.

Low Power Consumption (LPC for a Load Resource)

Limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained power consumption of the Load Resource.

Low Sustained Limit (LSL for a Load Resource)

Limit calculated by ERCOT, using the QSE-established Maximum Power Consumption.

Low Sustained Limit (LSL for a Generation Resource)

Limit established by the QSE, continuously updatable in Real-Time, that describes the minimum sustained energy production capability of the Resource.

M

Maintenance Outage (See Outage)

Make-Whole Payment

A payment made by ERCOT to the QSE for a Resource to reimburse the QSE for allowable startup and minimum energy costs of the Resource not recovered in energy revenue when the Resource is committed by the DAM or by a RUC.

Make-Whole Charge

A charge made by ERCOT to a QSE for a Resource to recapture all or part of the revenues received by the QSE that exceed the Make-Whole Payment for the Resource.

Market Clearing Price for Capacity (MCPC)

The hourly price for Ancillary Service capacity awarded in the Day-Ahead Market or a Supplemental Ancillary Service Market.

Market Information System (MIS)

An electronic communications interface established and maintained by ERCOT that provides a communications link to the public and to Market Participants, as a group or individually.

MIS Public Area

The portion of the MIS that is available to the public.

MIS Secure Area

The portion of the MIS that is available only to registered users.

MIS Certified Area

The portion of the MIS that is available only to a specific Market Participant.

Market Participant

An Entity that engages in any activity that is in whole or in part the subject of these Protocols, regardless of whether that Entity has signed an Agreement with ERCOT.

Mass Drop

The immediate cessation of service by a Competitive Retailer (CR) to all ESI IDs served by the CR.

Master QSE

The QSE that manages the meter-splitting telemetry and the verifiable cost submission for split Generation Resource meters.

Maximum Power Consumption (MPC for a Load Resource)

Limit established by the QSE, continuously updated in Real-Time, that describes the maximum sustained power consumption of the Load Resource.

Messaging System

The ERCOT-to-QSE communications system used to send Real-Time notices and Dispatch Instructions to QSEs.

Meter Data Acquisition System (MDAS)

The system to obtain revenue quality meter data from ERCOT Polled Settlement meters and Settlement Quality Meter Data from the TSPs and DSPs for settlement and to populate the Data Aggregation System and Data Archive.

Meter Reading Entity (MRE)

A TSP or DSP that is responsible for providing ERCOT with ESI ID level consumption data as defined in Section 19, Texas Standard Electronic Transaction (SET).  In the case of an ERCOT-Polled Settlement (“EPS”) Meter or ERCOT populated ESI ID data (such as Generation Resource site load), ERCOT will be identified as the MRE in ERCOT systems.

Minimum-Energy Offer

Represents an offer for the costs incurred by a Resource in producing energy at the Resource’s LSL expressed in dollars per MWh.

Minimum Reservation Price

The lowest price that a seller is willing to accept.

Mitigated Offer Caps

An upper limit on the price of an offer as detailed in Section 4.4.9.4.1, Mitigated Offer Cap.

Mitigated Offer Floor

A lower limit on the price of an offer as detailed in Section 4.4.9.4.2, Mitigated Offer Floor.

Mothballed Generation Resource (see Resource)

Municipally Owned Utility (MOU)

A utility owned, operated, and controlled by a nonprofit corporation, the directors of which are appointed by one or more municipalities, or a utility owned, operated, or controlled by a municipality.

N

Net Dependable Capability

The maximum sustained capability of a Resource as demonstrated by performance testing.

Net Generation

Gross generation less station auxiliary Load or other internal unit power requirements metered at or adjusted to the point of interconnection with the ERCOT Transmission Grid at the common switchyard.

Network Operations Model

A representation of the ERCOT System providing the complete physical network definition, characteristics, ratings, and operational limits of all elements of the ERCOT Transmission Grid and other information from TSPs, Resource Entities, and QSEs.

Network Security Analysis

A processor used by ERCOT to monitor Transmission Elements in the ERCOT Transmission Grid for limit violations and to verify Electrical Bus voltage limits to be within a percentage tolerance as outlined in the Operating Guides.

Non-Competitive Constraint

Any Transmission Element that is not a Competitive Constraint.

Non-Opt-In Entity (NOIE)

An Electric Cooperative or Municipally Owned Utility that does not offer Customer Choice.

Non-Opt-In Entity (NOIE) Load Zone

A Load Zone established by a NOIE or a group of NOIEs using a one-time NOIE election.

Non-Spinning Reserve (Non-Spin)

An Ancillary Service that is provided through use of the part of Off-Line Generation Resources that can be synchronized and ramped to a specified output level within 30 minutes (or Load Resources that can be interrupted within 30 minutes) and that can operate (or Load Resources that can be interrupted) at a specified output level for at least one hour. Non-Spinning Reserve Service (Non-Spin) may also be provided from unloaded On-Line capacity that meets the 30-minute response requirements and that is reserved exclusively for use for this service.

Normal Ramp Rate

The rate of change in megawatts (MW) per minute of a Resource, which is specified by the QSE to ERCOT by up to ten segments; each segment represents a single MW per minute value (across the capacity of the Resource) that describe the available rate of change in output for the given range (between HSL and LSL) of output of a Resource.

Normal Rating (see Ratings)

Notice or Notification

Sending of information by an Entity to Market Participants, ERCOT, or others, as called for in these Protocols.  Notice or notification may be sent by electronic mail, facsimile transmission, or U.S. mail.

- O

Off-Line

The status of a Resource that is not synchronously interconnected to the ERCOT System.

Off-Peak Hours

Hours that are not On-Peak Hours.

Oklaunion Exemption

The export schedules from the Public Service Company of Oklahoma, the Oklahoma Municipal Power Authority, and the AEP Texas North Company for their share of the Oklaunion Resource over the North DC Tie are not treated as Load connected at transmission voltage, are not subject to any of the fees described in Section 4.4.4 DC Tie Schedules, and are limited to the actual net output of the Oklaunion Resource.

On-Line

The status of a Resource that is synchronously interconnected to the ERCOT System.

On-Peak Hours

Hours ending in 0700 to 2200 Central Prevailing Time from Monday through Friday excluding NERC holidays.

Operating Day

The day, including hours ending 0100 to 2400, during which energy flows.

Operating Guides

Guidelines approved by the ERCOT Board describing the reliability standards for ERCOT.

Operating Hour

A full clock hour during which energy flows.

Operating Period

A two-hour period comprised of the Operating Hour and the clock hour preceding the Operating Hour.

Opportunity Outage (see Outage)

Outage

The condition of a facility that has been removed from service to perform maintenance, construction, or repair on the facility.

Forced Outage

An Outage initiated by protective relay, or manually in response to an observation by personnel that the condition of equipment could lead to an event, or potential event, that poses a threat to people, equipment, or public safety.

Maintenance Outage

An Outage initiated manually to remove equipment from service to perform work on components that could be postponed briefly but that is required to prevent a potential Forced Outage and that cannot be postponed until the next Planned Outage.  Maintenance Outages are classified as follows:

(1)	Level 1 Maintenance Outage – Equipment that must be removed from service within 24 hours to prevent a potential Forced Outage;

(2)	Level II Maintenance Outage – Equipment that must be removed from service within seven days to prevent a potential Forced Outage; and

(3)	Level III Maintenance Outage – Equipment that must be removed from service within 30 days to prevent a potential Forced Outage.

Opportunity Outage

An Outage that may be accepted by ERCOT when a specific Resource has been forced Off-Line due to a Forced Outage and the Resource has been previously accepted for a Planned Outage during the next eight days, or that may be accepted by ERCOT when a specific Resource has been forced Off-Line due to a Forced Outage and a TSP has been previously accepted for a Planned Outage during the next eight days.

Planned Outage

An Outage that is planned and scheduled in advance with ERCOT, other than a Maintenance Outage or Opportunity Outage.

Simple Transmission Outage

A Planned Outage or Maintenance Outage of any Transmission Element in the Network Operations Model such that when the Transmission Element is removed from its normal service, absent a Forced Outage of other Transmission Elements, the Outage does not cause a topology change in the LMP calculation and thus cannot cause any LMPs to change with or without the Transmission Element that is suffering the Outage.

Outage Scheduler

The application that TSPs or QSEs use to submit notification of Outages or requests for Outages to ERCOT for approval, acceptance, or rejection.

Output Schedule

The self-scheduled output for every five-minute interval of a Resource provided by a QSE before the execution of SCED.

P

Planned Outage (See Outage)

Power System Stabilizer

A device that is installed on Generation Resources to maintain synchronous operation of the ERCOT System under transient conditions.

Point-to-Point (PTP) Obligation (see Congestion Revenue Right (CRR))

Point-to-Point (PTP) Option (see Congestion Revenue Right (CRR))

Premise

A Service Delivery Point or combination of Service Delivery Points that is assigned a single Electric Service Identifier (ESI ID) for settlement and registration.

Prior Agreement

Any previous agreement between an Entity, its Affiliate, or its predecessor in interest and ERCOT about performance under the ERCOT Protocols.

Program Administrator

The Entity approved by the PUCT that is responsible for carrying out the administrative responsibilities for the Renewable Energy Credit Program as set forth in subsection (g) of P.U.C. Subst. R. 25.173, Goal for Renewable Energy. .

Protected Information

Information protected from disclosure as described in Section 1, Overview.

Provider of Last Resort (POLR)

The designated Competitive Retailer as defined in the P.U.C. Substantive Rules for default Customer service, and as further described in Section 15.1.1, Customer Switch of Competitive Retailer.

Q

QSE Clawback Interval

Any QSE-Committed Interval that is part of a contiguous block that includes at least one RUC-Committed Hour unless it is:

(1)	QSE-committed before the first RUC instruction for any RUC-Committed Hour in that contiguous block; or

(2)	Part of a contiguous block of a QSE-Committed Intervals, at least one of which was committed by the QSE before the RUC instruction described in paragraph (1) above.

QSE-Committed Interval

A Settlement Interval for which the QSE for a Resource has committed the Resource without a RUC instruction to commit it.

Qualified Scheduling Entity (QSE)

A Market Participant that is qualified by ERCOT in accordance with Section 16, Registration and Qualification of Market Participants, for communication with ERCOT for Resource Entities and Load Serving Entities and for settling payments and charges with ERCOT.

Qualifying Facility (QF)

A qualifying cogeneration facility or qualifying small power production facility under regulatory qualification criteria as defined in PURPA, 16 USC §§ 796(18)(B) and 796(17)(C).

R

Ratings

Emergency Rating

Two-hour MVA rating of a Transmission Element.

15-Minute Rating

The 15-Minute MVA rating of a Transmission Element.

Normal Rating

The rating at which a Transmission Element can operate without reducing its normal life expectancy.

Reactive Power

The product of voltage and the out-of-phase component of alternating current. Reactive Power, usually measured in megavolt-ampere reactive, is produced by capacitors, overexcited generators and other capacitive devices and is absorbed by reactors, underexcited generators and other inductive devices.

Real-Time

The current instant in time.

REC Program

The Renewable Energy Credit trading program, as described in Section 14, State of Texas Renewable Energy Credit Trading Program, and P.U.C. Subst. R. 25.173, Goal for Renewable Energy.

Regulation Down Service (Reg-Down)

An Ancillary Service that provides capacity that can respond to signals from ERCOT within three to five seconds to respond to changes in system frequency.  Such capacity is the amount available below any Base Point but above the Low Sustained Limit of a Generation Resource and may be called on to change output as necessary throughout the range of capacity available to maintain proper system frequency. A Load Resource providing Reg-Down must be able to increase and decrease Load as deployed within its Ancillary Service Schedule for Reg-Down below the Load Resource’s Maximum Power Consumption limit.

Regulation Service

Consists of either Regulation Up Service  or Regulation Down Service.

Regulation Up Service (Reg-Up)

An Ancillary Service that provides capacity that can respond to signals from ERCOT within three to five seconds to respond to changes in system frequency.  Such capacity is the amount available above any Base Point but below the High Sustained Limit of a Generation Resource and may be called on to change output as necessary throughout the range of capacity available to maintain proper system frequency.  A Load Resource providing Reg-Up must be able to increase and decrease Load as deployed within its Ancillary Service Schedule for Reg-Up above the Low Power Consumption limit.

Reliability Must-Run (RMR) Service

An Ancillary Service provided from a Reliability Must-Run Unit under an Agreement with ERCOT.

Reliability Must-Run (RMR) Unit

A Generation Resource operated under the terms of an Agreement with ERCOT that would not otherwise be operated except that it is necessary to provide voltage support, stability or management of localized transmission constraints under first contingency criteria where market solutions do not exist.

Reliability Unit Commitment (RUC)

Process to ensure that there is adequate Resource capacity and Ancillary Service capacity committed in the proper locations to serve ERCOT forecasted Load.

Remedial Action Plan (RAP)

A set of pre-defined actions to be taken to relieve transmission security violations (normally post-contingency overloads or voltage violations) that are sufficiently dependable to assume they can be executed without loss of reliability to the interconnected network.  These plans may be relied upon in allowing additional market use of the transmission system.  RAPs may include controllable Load shedding by dispatcher or ERCOT action.

Renewable Production Potential (RPP)

The maximum generation in MWh per interval from an Intermittent Renewable Resource that could be generated from all available units of that Resource.  The Renewable Production Potential depends on the renewable energy that can be generated from the available units (wind, solar radiation, or run-of-river water supply), current environmental conditions and the energy conversion characteristics of each unit.

Resource

The term is used to refer to both a Generation Resource and a Load Resource.  “Resource” used by itself in these Protocols does not include a Non-Modeled Generator.

All-Inclusive Generation Resource

A term used to refer to both a Generation Resource and a Non-Modeled Generator.

All-Inclusive Resource

A term used to refer to a Generation Resource, Load Resource and a Non-Modeled Generator.

Generation Resource

A generator that is capable of providing energy or Ancillary Service to the ERCOT System and is registered with ERCOT as a Generation Resource. “Generation Resource” used by itself in these Protocols does not include a Non-Modeled Generator.

Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced retirement of the Generation Resource.

Switchable Generation Resource

A Generation Resource that can be connected to either the ERCOT Transmission Grid or a non-ERCOT Control Area.

Wind Power Generation Resource (WGR)

A Generation Resource that is powered by wind.

Load Resource

A load that is capable of providing Ancillary Service to the ERCOT System and is registered with ERCOT as a Load Resource.

Non-Modeled Generator

A generator that is:

(1)	Capable of providing net output of energy to the ERCOT System;

(2)	10 MW or less in size; and

(3)	Registered with ERCOT as a Non-Modeled Generator, which means that the generator may not participate in the Ancillary Service or energy markets, RUC, or SCED.

- Resource Entity

An Entity that owns or controls an All-Inclusive Resource and is registered with ERCOT as a Resource Entity.

Resource ID (RID)

A unique identifier assigned to each Resource used in the registration and settlements systems managed by ERCOT.

Resource Node

The Electrical Bus defined in the Network Operations Model at which a Resource's measured output is settled. For a Generation Resource that is connected to the ERCOT Transmission Grid only by one or more radial transmission lines that all originate at the Generation Resource and terminate in a single substation switchyard, the Resource Node is an Electrical Bus in that substation. For all other Generation Resources, the Resource Node is the Generation Resource's side of the Electrical Bus at which the Generation Resource is connected to the ERCOT Transmission Grid

Resource Parameter

Static Resource-specific operational parameters submitted by a QSE to ERCOT.

Resource Status

The operational state of a Resource as provided in Section 3.9, Current Operating Plan (COP).

Responsive Reserve

An Ancillary Service that provides operating reserves that is intended to:

(1) 	Arrest frequency decay within the first few seconds of a significant frequency deviation on the ERCOT Transmission Grid using governor response and interruptible Load;

(2)	After the first few seconds of a significant frequency deviation, help restore frequency to its scheduled value to return the system to normal;

(3)	Provide energy or continued Load interruption during the implementation of the Emergency Electric Curtailment Plan (EECP); and

(4)	Provide backup regulation.

Retail Business Day (See Business Day)

Retail Electric Provider (REP)

As defined in P.U.C. Subst. R. 25.5, Definitions, an Entity that sells electric energy to retail Customers in Texas but does not own or operate generation assets and is not an MOU or EC.

Revenue Quality Meter

For ERCOT-Polled Settlement Meters, a meter that complies with the Protocols and the Settlement Metering Operating Guides. For TSP or DSP metered Entities, a meter that complies with Governmental Authority-approved meter standards, or the Protocols and the Operating Guides.

RUC-Committed Hour

An Operating Hour for which a RUC has committed a Resource to be On-Line.

RUC-Committed Interval

A Settlement Interval for which there is a RUC instruction to commit a Resource.

RUC Study Period

As defined under Section 5.1, Introduction.

S

Season

Winter months are December, January, and February; Spring months are March, April, and May; Summer months are June, July, and August; Fall months are September, October, and November.

Security-Constrained Economic Dispatch (SCED)

The determination of desirable Generation Resource output levels using Energy Offer Curves while considering State Estimator output for Load at transmission-level Electrical Buses, Generation Resource limits, and transmission limits to provide the least offer-based cost dispatch of the ERCOT System.

Self-Arranged Ancillary Service Quantity

The portion of its Ancillary Service Obligation that a QSE secures for itself using Resources represented by that QSE and Ancillary Service Trades.

Self-Schedule

Information for Real-Time Settlement purposes that specifies the amount of energy supply at a specified source Settlement Point used to meet an energy obligation at a specified sink Settlement Point for the QSE submitting the Self-Schedule.

Service Delivery Point

The specific point on the system where electricity flows from the TSP or DSP to a Customer.

Settlement

The process used to resolve financial obligations between a Market Participant and ERCOT.

Settlement Calendar

The Settlement Calendar provides information on when Settlement Statements and Invoices shall be posted, payment due dates and dispute deadlines.  Additional information is provided in Section 9.1.2, Settlement Calendar.

Settlement Interval

The time period for which markets are settled.

Settlement Meter

Generation and end-use consumption meters used for allocation of ERCOT charges and wholesale and retail Settlements.

Settlement Point

A Resource Node, Load Zone, or Hub.

Settlement Point Price

A price calculated for a Settlement Point for each Settlement Interval using LMP data and the formulas detailed in Sections 4.6, DAM Settlement and 6.6, Settlement Calculations for the Real-Time Energy Operations.

Settlement Quality Meter Data

Data that has been edited, validated, and is appropriate for the ERCOT settlement agent to use for settlement and billing purposes.

Shadow Price

A price for a commodity that measures the marginal value of this commodity, that is, the rate at which system costs could be decreased or increased by slightly increasing or decreasing, respectively, the amount of the commodity being made available.

Shift Factor

A measure of the flow on a particular Transmission Element due to a unit injection of power from a particular Electrical Bus to a fixed reference Electrical Bus.

Short-Term Wind Power Forecast

An ERCOT produced hourly forecast of wind-power energy production potential for each WGR.

Simple Transmission Outage (See Outage)

Special Protection Systems (SPS)

A set of automatic actions to be taken to relieve transmission security violations (normally post-contingency overloads or voltage violations) that are sufficiently dependable to assume they can be executed without loss of reliability to the interconnected network.

Split Generation Resource

A Generation Resource that has been split to function as two or more independent Generation Resources in accordance with Section 10.3.2.1, Generation Meter Splitting and Section 3.10.7.2, Modeling of Resources and Transmission Loads.

Startup Cost

All costs incurred by a Generation Resource in starting up and reaching breaker close in dollars per start.

Startup Offer

Represents an offer for all costs incurred by a Generation Resource in starting up and reaching breaker close in dollars per start.

State Estimator (SE)

A computational algorithm that uses Real-Time inputs from the network's Supervisory Control and Data Acquisition (SCADA) system that measure the network's electrical parameters, including its topology, voltage, power flows, etc., to estimate electrical parameters (such as line flows and Electrical Bus voltages and loads) in the ERCOT Transmission Grid.  The SE’s output is a description of the network and all of the values (topology, voltage, power flow, etc.) to describe each Electrical Bus and line included in the system model.

System Operator

An Entity supervising the collective Transmission Facilities of a power region, which Entity is charged with coordination of market transactions, system-wide transmission planning, and network reliability.

T

TDSP Metered Entity

Any Entity that meets the requirements of Section 10.2.2, TDSP Metered Entities.

Technical Advisory Committee (TAC)

A subcommittee in the ERCOT governance structure reporting to the Board of Directors as defined by the ERCOT Bylaws.

Texas Nodal Market Implementation Date

The date on which ERCOT starts operation of the Texas nodal market design in compliance with the rules and orders of the PUCT.  Once this date is determined, ERCOT shall post it on the ERCOT website and maintain it on either the ERCOT website or the MIS Public Area thereafter.

Texas SET

Texas Standard Electronic Transaction procedures, set forth in Section 19, Texas SET, used to transmit information pertaining to the Customer Registration Database. Record and data element definitions are provided in the data dictionary in Protocols Section 19.

Three-Part Supply Offer

An offer made by a QSE for a Generation Resource that it represents containing three components: a Startup Offer, a Minimum-Energy Offer, and an Energy Offer Curve.

Transmission Access Service

Use of a TSP’s Transmission Facilities for which the TSP is allowed to charge through tariff rates approved by the PUCT.

Transmission and/or Distribution Service Provider (TDSP)

An Entity that is either a Transmission Service Provider or a Distribution Service Provider.

Transmission Element

A physical transmission facility that is either a Electrical Bus, line, transformer, generator, load, breaker, switch, capacitor, reactor, phase shifter, or other similar device that is part of the ERCOT Transmission Grid and defined in the ERCOT Network Operations Model.

Transmission Facilities

(1)	Power lines, substations, and associated facilities, operated at 60 kV or above, including radial lines operated at or above 60 kV;

(2)	Substation facilities on the high voltage side of the transformer, in a substation where power is transformed from a voltage higher than 60 kV to a voltage lower than 60 kV or is transformed from a voltage lower than 60 kV to a voltage higher than 60 kV; and

(3)	The direct current interconnections between ERCOT and the Southwest Power Pool (SPP) or Comision Federal de Electricidad.

Transmission Loss Factors

The fraction of ERCOT Load (forecast or actual) that is considered to constitute the ERCOT Transmission Grid losses in the Settlement Interval, based on a linear interpolation (or extrapolation) of the calculated losses in the off-peak and on-peak seasonal ERCOT base cases.

Transmission Losses

Difference between energy put into the ERCOT Transmission Grid and energy taken out of the ERCOT Transmission Grid.

Transmission Service

Commercial use of Transmission Facilities.

Transmission Service Provider (TSP)

An Entity under the jurisdiction of the PUCT that owns or operates Transmission Facilities used for the transmission of electricity and provides Transmission Service in the ERCOT Transmission Grid.

U

Unaccounted for Energy (UFE)

The difference between total metered Load for each Settlement Interval, adjusted for applicable Distribution Losses and Transmission Losses, and total ERCOT System Net Generation.

Unit Reactive Limit

The maximum quantity of Reactive Power that a Generation Resource is capable of providing at a 0.95 power factor at its maximum real power capability.

Updated Network Model

A computerized representation of the ERCOT physical network topology, including some Resource Parameters, all of which replicates the forecasted or current network topology of the ERCOT System needed by ERCOT to perform its functions.

Usage Profile (See Load Profile)

USD

U.S. dollar.

V

Verbal Dispatch Instruction (VDI)

A Dispatch Instruction issued orally.

Virtual QSE

An LSE whose QSE has provided notice of its intent to terminate its relationship with the LSE and who then fails to meet ERCOT’s creditworthiness requirements to become an Emergency QSE, as set forth in Section 16.2.13.1, Designation as an Emergency QSE or Virtual QSE.

Voltage Profile

The normally desired predetermined distribution of desired nominal voltage set points across the ERCOT System.

Voltage Support Service

An Ancillary Service that is required to maintain transmission and distribution voltages on the ERCOT Transmission Grid within acceptable limits.

W

Weather Zone

A geographic region designated by ERCOT in which climatological characteristics are similar for all areas within such region.

Wholesale Customer

A Non-Opt-In Entity receiving service at wholesale points of delivery from an LSE other than itself.

Wholesale Electric Market Monitor (WEMM)

The Entity that the PUCT selects and that ERCOT enters into a contract with to monitor the wholesale electric market under the Public Utility Regulatory Act, Tex. Util. Code Ann. §39.151 (Vernon 1998 & Supp. 2005).

Wind Power Generation Resource (see Resource)

X

Y

Z

## ACRONYMS AND ABBREVIATIONS

ACE	Area Control Error

AML	Adjusted Metered Load

ADR	Alternative Dispute Resolution

AVR	Automatic Voltage Regulator

AREP	Affiliated Retail Electric Provider

BLT	Block Load Transfer

CAO	Control Area Operator

CEO	Chief Executive Officer

CIM	Common Information Model

COP	Current Operating Plan

CPT	Central Prevailing Time

CR	Competitive Retailer

CRR	Congestion Revenue Right

CSA	Continuous Service Agreement

DAM	Day-Ahead Market

DRUC	Day-Ahead Reliability Unit Commitment

DC Tie	Direct Current Tie

DLC	Direct Load Control

DLF	Distribution Loss Factor

DSP	Distribution Service Provider

DSR	Dynamically Scheduled Resource

DUNS	Data Universal Numbering System

DUNS #	DUNS Number

ECI	Element Competitiveness Index

EECP	Emergency Electric Curtailment Plan

EPRI	Electric Power Research Institute

EPS	ERCOT-Polled Settlement Meter

ERCOT	Electric Reliability Council of Texas, Inc.

ESI ID	Electric Service Identifier

FGR	Flowgate Right

FIP	Fuel Index Price

FOP	Fuel Oil Price

HASL	High Ancillary Service Limit

HDL	High Dispatch Limit

HE	Hour Ending

HEL	High Emergency Limit

HSL	High Sustained Limit

Hz	Hertz

IDR	Interval Data Recorder

IRR	Intermittent Renewable Resources

kV	Kilovolt

LASL	Low Ancillary Service Limit

LDL	Low Dispatch Limit

LEL	Low Emergency Limit

LFC	Load Frequency Control

LMP	Locational Marginal Price

LPC	Low Power Consumption

LSL	Low Sustained Limit

LTLF	Long-Term Load Forecast

LSE	Load Serving Entity

MAP	Mitigation Action Plan

MCFRI	McCamey Flowgate Right

MCPC	Market Clearing Price for Capacity

MMBtu	Million British Thermal Units

MOU	Municipally Owned Utility

MPC	Maximum Power Consumption

MRE	Meter Reading Entity

MTLF	Mid-Term Load Forecast

MVAr	Mega Volt-Amperes reactive

MW	Megawatt

MWh	Megawatt Hour

NERC	North American Electric Reliability Council

NOIE	Non-Opt-In Entity

Non-Spin	Non-Spinning Reserve

OCN	Operating Condition Notice

PCRR	Pre-Assigned Congestion Revenue Right

POLR	Provider of Last Resort

POS	Power Operating System

PRR	Protocol Revision Request

PTB	Price-to-Beat

PTP	Point-to-Point

QF	Qualifying Facility

QSE	Qualified Scheduling Entity

RAP	Remedial Action Plan

Reg-Down	Regulation Down

Reg-Up	Regulation Up

REP	Retail Electric Provider

RID	Resource ID

RIDR	Representative IDR

RMR	Reliability Must-Run

RPP	Renewable Production Potential

RRS	Responsive Reserve

SCED	Security-Constrained Economic Dispatch

SCUC	Security-Constrained Unit Commitment

SE	State Estimator

STLF	Short-Term Load Forecast

TAC	Technical Advisory Committee

TDSP	Transmission and/or Distribution Service Provider

TSP	Transmission Service Provider

UFE	Unaccounted For Energy

VAr	Volt-Ampere reactive

WEMM	Wholesale Electric Market Monitor

WGR	Wind Generation Resource